3GPP TSG RAN WG1 #114			R1-230XXXX

Toulouse, France, August 21st – August 25th, 2023

Agenda item:	7.1

Source: 	Moderator (Qualcomm Incorporated)

Title: 	Summary for x7899 (Correction on RRC parameter name for SRS carrier switching)

Document for:	Discussion and Decision

# Proposed change

#### 6.2.1.3	UE sounding procedure between component carriers

For a carrier of a serving cell c1 with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, denote c2 as the corresponding carrier of a serving cell whose UL transmissions are temporarily suspended as signalled by higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier. Define the set Sc2={c2, s1(c2)…sN-1(c2)} as the set of carriers of serving cells that each carrier meets one of the following conditions:

-	si(c2) is in the same band and same TAG as c2;

-	si(c2) is a carrier of inter-band CA with c2 and si(c2) is indicated through the capability signalling srs-SwitchingAffectedBandsListNR-r17 to be affected by the SRS switch from c2 to c1;

where 1≤i≤N-1.

R1-2307899 proposes the following change to TS 38.214 to align with the correct RRC parameter name:

Companies are encouraged to provide comments on the above change:

| Company | Comment |
| --- | --- |