3GPP TSG RAN WG1 #115          													                     R1-2312293

Chicago, USA, November 13th – November 17th, 2023

Source: 	Intel Corporation

Title:	Remaining issues on resource allocation for SL positioning

Agenda item:	8.3.1.3

Document for:	Discussion and Decision

# Introduction

At the RAN1#114bis meeting, the following agreements and conclusions were made regarding resource allocation for sidelink positioning [1].

Agreement

In scheme 1, with regards to distinguishing between DCI format 3_0 and 3_2:

- New RNTIs, i.e., SL-PRS-RNTI & SL-PRS-CS-RNTI, are introduced.

- Support DCI size alignment between DCI format 3_0, 3_1 and 3_2.

Agreement

Sidelink PRS Received Signal Strength Indicator (SL PRS-RSSI) is defined as the linear average of the total received power (in [W]) observed in:

- the SL-PRS resource and the associated PSCCH in OFDM symbols of slots configured for PSCCH and in OFDM symbols of slots configured for SL-PRS.

  - Introduce larger values for congestion control processing time capability than legacy SL

Agreement

The working assumption is confirmed with the following revision with regards to the number of padding bits:

- the padding bits, if any, are such that the size of the SCI format 2-D is the same as if the larger of SCI format 2-A or 2-B is embedded

Working assumption The number of bits in the embedded SCI format field of SCI format 2-D is 2 bits - If the “Embedded SCI format” field is set to 00, the SCI 2-A fields are included with necessary padding - If the “Embedded SCI format” field is set to 01, the SCI 2-B fields are included - If the “Embedded SCI format” field is set to 10, “size of SCI 2-B” number of reserved bits are included - If the “Embedded SCI format” field is set to 11, “size of SCI 2-B” number of reserved bits are included - Note: the size of SCI format 2-D is the same regardless of the value of the embedded SCI format field

Agreement

In Scheme 2, with regards to the SCI-based triggering of SL-PRS, the following WA is confirmed:

Working assumption In Scheme 2, with regards to the triggering of SL-PRS, for the SCI-based triggering, the SL-PRS request, in either SCI-1B or SCI-2D, is an explicit field - If (pre-)configured per resource pool, then 1 bit is used, otherwise, it is 0 bits

Agreement

- Regarding the following text in brackets in Section 8.4.4 of TS38.214:

[ If the '[SL PRS request]' field in the SCI associated with the received SL PRS is set to 1 then the UE shall report this request for SL PRS transmission to higher layers.]

- Keep the text and remove the brackets with the change shown below:

  - '[SL PRS request]' field in the SCI associated with the received SL PRS is set to 1 then the UE shall report this request for SL PRS transmission is reported to higher layers.

Conclusion

In scheme 1, with regards to an explicit indication of SL-PRS specific information in DCI format 3_0:

- Indication of SL-PRS specific information is not explicitly included in DCI

Agreement

With regards to the bitwidth of the field “Resource ID indication” when the value of the higher layer parameter sl-MaxNumPerReserveSL-PRS is configured to 3:

- Ceil(2*log2(Number of SL-PRS resources in (pre-)configuration)) bits should be used

Further discuss at TP for the above at RAN1#114bis.

Conclusion

In a dedicated resource pool, with regards to the PSCCH, do not introduce additional values for the subchannel (pre-)configuration.

Agreement

The following TP is endorsed for clause 16.4A of TS 38.213:

- Reason for change: to provide information regarding the starting PRB of PSCCH.

- Summary of change: include the information that the PSCCH starts from the lowest PRB of the sub-channel determined according to the index of the associated SL PRS resource

- The consequence if not approved is: the UE will not be able to determine which resource to use for PSCCH transmission

---------------------------- Start of Text Proposal for TS 38.213 ----------------------------- < Unchanged parts are omitted > ## 16.4A UE procedure for transmitting PSCCH in dedicated resource pool for SL PRS For a resource pool dedicated for SL PRS transmissions, a UE can be provided a number of symbols in the resource pool, by sl-TimeResourcePSCCH, starting from a second symbol that is available for SL transmissions in a slot, and a number of PRBs in the resource pool, by sl-FreqResourcePSCCH, starting from the lowest PRB of the sub-channel determined according to the index of the associated SL PRS resource for a PSCCH transmission with a SCI format 1-B. A UE that transmits a PSCCH with SCI format 1-B using SL PRS resource allocation scheme 2 [6, TS 38.214] sets < Unchanged parts are omitted > ---------------------------- End of Text Proposal for TS 38.213 -----------------------------

Agreement

Confirm the working assumption related to the TB size determination from RAN1 #114, and endorse the following TP:

| Reason for change: | Corrections on TBS in a shared resource pool |
| --- | --- |
| Summary of change: | In clause 8.1.3.2 of TS 38.214, complement the value of NsymbSL-PRS under different conditions. |
| Consequences if not approved: | The TBS procedure in a shared resoruce pool is incomplete. |

- ----------------------------------------- Start of text proposal to TS 38.214 v18.0.0------------------------------------------- 8.1.3.2 Transport block size determination <<< UNCHANGED PARTS OMITTED >>> The UE shall first determine the number of REs (NRE) within the slot. - A UE first determines the number of REs allocated for PSSCH within a PRB (NRE') by NRE'=NscRBNsymbsh-NsymbPSFCH-NsymbSL-PRS-NohPRB-NREDMRS, where - NscRB=12 is the number of subcarriers in a physical resource block, - NsymbshNsymbslot = sl-LengthSymbols -2, where sl-LengthSymbols is the number of sidelink symbols within the slot provided by higher layers, - NsymbPSFCH = 3 if 'PSFCH overhead indication' field of SCI format 1-A indicates "1", and NsymbPSFCH = 0 otherwise, if higher layer parameter sl-PSFCH-Period is 2 or 4. If higher layer parameter sl-PSFCH-Period is 0, NsymbPSFCH=0. If higher layer parameter sl-PSFCH-Period is 1, NsymbPSFCH=3. - NsymbSL-PRS is the number of OFDM symbols used for SL PRS in the slot as indicated by the ‘SL PRS resource ID’ in SCI format 2-D if the 2nd-stage SCI is SCI format 2-D, and NsymbSL PRS = 0 otherwise， - NohPRB is the overhead given by higher layer parameter sl-X-Overhead, - NREDMRS is given by Table 8.1.3.2-1 according to higher layer parameter sl-PSSCH-DMRS-TimePatternList. <<< UNCHANGED PARTS OMITTED >>> - ----------------------------------------- End of text proposal to TS 38.214 v18.0.0-------------------------------------------

Conclusion

For a dedicated resource pool, no more discussion on potential restriction by SL PRS-CBR and priority for the following SL PRS transmission parameters:

- Maximum Number of SL PRS resources in a slot

- Maximum comb-size of a SL PRS resource in a slot

- Maximum Number of OFDM symbols of a SL PRS resource in a slot

Agreement

With regards to the dedicated resource pool for positioning, suggest to the editors to align the terminology used as:

- “Dedicated SL PRS resource pool” defined in 38.214 as shown below:

  - A sidelink resource pool which can be used for transmission of SL PRS and cannot be used for transmission of PSSCH will be referred to as dedicated SL PRS resource pool.

Conclusion

From RAN1 perspective, there is no need to introduce an association between a dedicated resource pool for positioning and a shared resource pool, or between a dedicated resource pool for positioning and a sidelink communication resource pool.

Agreement

- With regards to the SL PRS Channel Occupancy Ratio (SL PRS CR):

  - Sidelink PRS Channel Occupancy Ratio (SL PRS CR) evaluated at slot n is defined as the total number of SL PRS resources in the dedicated SL PRS resource pool used for its transmissions in slots [n-a, n-1] and granted in slots [n, n+b] divided by the total number of configured SL PRS resources in the transmission pool over [n-a, n+b].

- With regards to the SL PRS Channel Busy Ratio (SL PRS CBR):

  - SL PRS Channel Busy Ratio (SL PRS CBR) measured in slot n is defined as the number of SL PRS resources in the dedicated SL PRS resource pool whose SL PRS RSSI measured by the UE exceed a (pre-)configured threshold sensed over a SL PRS-CBR measurement window [n-a, n-1], wherein a is equal to 100 or 100·2µ slots, according to [sl-TimeWindowSizeCBR-positioning] divided by the total number of the configured sidelink PRS resources in the transmission pool over [n-a, n-1].

Working assumption

Endorse the following TP for clause 8.2.4.2 of TS 38.214:

---------------------------- Start of Text Proposal for TS 38.214 ----------------------------- < Unchanged parts are omitted > #### 8.2.4.2 UE procedure for determining slots and SL PRS resource(s) associated with an SCI format 1-B in a dedicated resource pool The set of slots and SL PRS resources for SL PRS transmission is determined by the PSCCH containing the associated SCI format 1-B, and fields '[SL-PRS resource ID (s))', '[Time resource assignment]' of the associated SCI format 1-B as described below. The set of slots is determined as in clause 8.1.5, with the following modifications: - "SCI format 1-A" is replaced by "SCI format 1-B", - [ potential parameter name changes]. The first SL PRS resource is determined according to the sub-channel used for the PSCCH transmission containing the associated SCI format 1-B, where the index of the sub-channel in the resource pool is identical to the index of the SL PRS resource provided by [higher layer parameter]. The second SL-PRS and third SL PRS resource, if reserved by SCI format 1-B, are determined from " Resource ID indication" which is equal to a PRS Resource ID value (PRIV) where, If [sl-MaxNumPerReserve] is 2 then PRIV=r1 If [sl-MaxNumPerReserve] is 3 then PFRIV=r2*NSL-PRS+r1 where - r1 denotes the SL PRS resource ID for the second resource - r2 denotes the SL PRS resource ID for the third resource - NSL-PRS is the number of SL-PRS resources (pre-)configured in a slot of a resource pool. ---------------------------- End of Text Proposal for TS 38.214 ----------------------------- < Unchanged parts are omitted >

Agreement

For activation and deactivation of configured grant type 2 for SL PRS for DCI 3-2, use a dedicated field of size 1 bit.

Agreement

From RAN1 perspective, whether to support or not reporting of CBR measurements to LMF or another UE, is left up to other WGs.

Agreement

With regards to the shared resource pool for positioning, suggest to the editors to align the terminology used as:

- “shared SL PRS resource pool” defined in 38.214 as shown below:

A sidelink resource pool which can be used for transmission of both SL PRS and PSSCH will be referred to as shared SL PRS resource pool.

Agreement

Endorse the TP below for clause 8.5.2.3 of TS 38.214

| Reason for change: | Corrections on description associated with SCI format 2-D in a shared resource pool for the CSI reference resource definition |
| --- | --- |
| Summary of change: | In clause 8.5.2.3 of TS 38.214, SCI format 2-D is captured. |
| Consequences if not approved: | The description associated with SCI format 2-D in CSI reference resource definition is missing |

- 8.5.2.3 CSI reference resource definition <<< UNCHANGED PARTS OMITTED >>> If configured to report CQI index and RI index, in the CSI reference resource, the UE shall assume the following for the purpose of deriving the CQI index and RI index: - The reference resource uses the CP length and subcarrier spacing configured for the SL BWP. - Redundancy Version 0. - PSCCH occupies 2 OFDM symbols. - The number of PSSCH and DM-RS symbols is equal to sl-LengthSymbols‒2. - Assume no REs allocated for sidelink CSI-RS. - Assume no REs allocated SCI format 2-A, SCI format 2-B, or SCI format 2-C or SCI format 2-D. <<< UNCHANGED PARTS OMITTED >>> - ----------------------------------------- End of text proposal to TS 38.214 v18.0.0-------------------------------------------

Agreement

Step 5 for the resource selection procedure in Section 8.2.4.2 of 38.214 is modified as follows:

- In step 5, the second condition is modified as follows: for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList and any SL PRS resource ID in the set of SL PRS resource ID(s) provided by the higher layer, and a hypothetical SCI format 1-B received in slot t'mSL with 'Resource reservation period' field set to that periodicity value and indicating that SL-PRS resource ID, condition c in step 6 would be met.

Agreement

Confirm the following Working Assumption made in RAN1#114bis:

Working assumption Endorse the following TP for clause 8.2.4.2 of TS 38.214: ---------------------------- Start of Text Proposal for TS 38.214 ----------------------------- < Unchanged parts are omitted > #### 8.2.4.2 UE procedure for determining slots and SL PRS resource(s) associated with an SCI format 1-B in a dedicated resource pool The set of slots and SL PRS resources for SL PRS transmission is determined by the PSCCH containing the associated SCI format 1-B, and fields '[SL-PRS resource ID (s))', '[Time resource assignment]' of the associated SCI format 1-B as described below. The set of slots is determined as in clause 8.1.5, with the following modifications: - "SCI format 1-A" is replaced by "SCI format 1-B", - [ potential parameter name changes]. The first SL PRS resource is determined according to the sub-channel used for the PSCCH transmission containing the associated SCI format 1-B, where the index of the sub-channel in the resource pool is identical to the index of the SL PRS resource provided by [higher layer parameter]. The second SL-PRS and third SL PRS resource, if reserved by SCI format 1-B, are determined from " Resource ID indication" which is equal to a PRS Resource ID value (PRIV) where, If [sl-MaxNumPerReserve] is 2 then PRIV=r1 If [sl-MaxNumPerReserve] is 3 then PFRIV=r2*NSL-PRS+r1 where - r1 denotes the SL PRS resource ID for the second resource - r2 denotes the SL PRS resource ID for the third resource - NSL-PRS is the number of SL-PRS resources (pre-)configured in a slot of a resource pool. ---------------------------- End of Text Proposal for TS 38.214 ----------------------------- < Unchanged parts are omitted >.

In this contribution, we present our views on remaining issues on resource allocation for sidelink positioning reference signal (SL PRS). Our views on remaining issues on measurements and reporting for SL positioning are described in our companion contributions [2].

- Remaining issues on multiplexing of SL PRS

In this section, we present our view on the remaining issues for multiplexing of SL PRS and other SL channels/signals in both dedicated resource pool and shared resource pool.

## Multiplexing in a dedicated resource pool

At the RAN1#114 meeting, the following agreement was made regarding the collision handling between SL PRS and PSFCH in both shared and dedicated resource pool.

Agreement Support the following for SL-PRS multiplexing/collision with the following channels: - A SL-PRS resource and PSFCH (including the preceding gap symbol) are not mapped on the same symbols

However, this restriction for SL PRS transmission was not properly captured in the editor CR for TS38.214 [3]. To address this issue and include the high layer parameter for SL PRS resource, the following TP is proposed for the restrictions on SL PRS transmission in a dedicated resource pool.

------------------------------ TP#1: TS 38.214 ----------------------------------- 8.2.4.1.1 Resource allocation in time domain < Unchanged text omitted > For a dedicated SL PRS resource pool, the UE transmits SL PRS subject to the following restrictions: - the UE shall not transmit SL PRS and associated PSCCH in the same symbol; - the number of contiguous symbols and the starting symbol for SL PRS transmission shall correspond to one of the SL PRS resources in parameter sl-PrsResources-Dedicated-SL-PRS-RP; - a SL-PRS resource and PSFCH (including the preceding gap symbol) are not mapped on the same symbols. < Unchanged text omitted >

Agree on TP#1 for the restrictions on SL PRS transmission in a dedicated resource pool.

## Multiplexing in a shared resource pool

At the RAN1#112b-e meeting, it was agreed that for shared resource pools, a UE does not map SL PRS and PSSCH DMRS in the same OFDM symbol(s) [4]. Further, it was agreed at the RAN1#114bis meeting that in a shared resource pool, a UE shall not transmit SL PRS and SL CSI-RS in the same symbol, and transmission of SL PT-RS is cancelled in OFDM symbols with SL PRS [1].

However, the restrictions for SL PRS transmission in a shared resource pool was not properly captured in the editor CR for TS38.214 [3]. To address this issue and include the higher layer parameter for SL PRS resource, the following TP is proposed for the restrictions on SL PRS transmission in a shared resource pool.

------------------------------ TP#2: TS 38.214 ----------------------------------- 8.2.4.1.1 Resource allocation in time domain < Unchanged text omitted > For a shared SL PRS resource pool, the UE transmits the SL PRS in the symbols determined for PSSCH transmission according to clause 8.1.2.1, with the following restrictions: - the number of contiguous symbols for SL PRS transmission, LSL-PRS, shall correspond to one of the SL PRS resources in parameter sl-PrsResources-Shared-SL-PRS-RP. - the UE shall not transmit SL PRS in symbols where associated PSCCH is transmitted. - the UE shall not transmit SL PRS and PSSCH DMRS in the same symbol. - the UE shall not transmit SL PRS and SL CSI-RS in the same symbol. - the UE shall transmit SL PRS on contiguous symbols either in between or after symbols where PSSCH DMRS is transmitted. - the UE shall transmit SL PRS only after the last symbol with second stage SCI. - the UE shall not transmit PSSCH and SL PRS in the same symbol. - a SL-PRS resource and PSFCH (including the preceding gap symbol) are not mapped on the same symbols. - For a given value of LSL-PRS , SL PRS resource is mapped to the last consecutive LSL-PRS SL symbols in the slot that meet all the other restrictions < Unchanged text omitted >

Agree on TP#2 for the restrictions on SL PRS transmission in a shared resource pool.

- Remaining issues on congestion control

During RAN1#114bis [1], different options to restrict the SL PRS transmission parameters were discussed. It was agreed to limit the transmission power, the number of transmissions, and the CR. It is still open to introduce a restriction of the periodicity on the SL PRS transmission parameters beyond the already agreed ones. Note that this was not present for sidelink communication as for communication on the CR, an MCS range, an allocation size range (in subchannels), the number of transmissions, and the Tx power could be restricted depending on the CBR (see information element SL-PSSCH-TxParameters-r16 in 38.331 [5]). As it is already agreed that similar to SL communication the CR can be restricted depending on the CBR we think a further restriction of the periodicity is redundant as restricting the CR can already restrict how of transmissions can occur.

No further restrictions of the SL PRS transmissions parameters are defined for congestion control.

- Remaining issues on multiple resource reservation

RAN1 agreed not to support feedback-based retransmissions of SL-PRS. RAN1 also did not make any explicit agreement on “blind” retransmission of SL-PRS, although SCI design supports reservation of 1 or 2 future SL-PRS transmissions. In this situation, there is no agreement on how the multiple reservations signaled in SCI are generated by higher layer, and therefore no common understanding between RAN1 and RAN2 on implementation of these reservation mechanisms.

Agreement For SL-PRS transmission, at least support the following - SL-PRS transmissions with periodic reservation: SL-PRS transmissions which are being reserved with a similar mechanism as the SL periodic resource reservation for another TB in legacy SL communication - FFS: whether/what changes are needed - SL-PRS transmissions without periodic reservation: SL-PRS transmissions in which the SL-PRS is transmitted at least once without periodic reservation, with a similar mechanism as in legacy SL communication with SL resource without periodic reservation. - FFS: Maximum number of reservations and transmissions after triggering Conclusion Do not support ACK/NACK feedback for SL-PRS or lower-layer feedback-based retransmissions in Release 18.

Furthermore, to implement SL-PRS transmission on multiple resource without periodic reservation, RAN2/MAC has two existing references: Periodic reservation or reservation of retransmission resources without periodic reservation. In this context, SL-PRS transmission on multiple resources is neither of these two and therefore requires specific consideration.

Additionally, there was an agreement as part of Congestion Control to limit the maximum number of retransmissions:

Agreement In Scheme 2, - For a dedicated resource pool for positioning, - congestion control can restrict at least the following range of parameters for SL PRS configuration per resource pool by CBR and priority: - Maximum SL PRS transmission power - Maximum Number of SL PRS (re-)transmissions - Discuss further the following four SL PRS transmission parameters: - Minimum Periodicity of SL PRS - Maximum Number of SL PRS resources in a slot - Maximum comb-size of a SL PRS resource in a slot - Maximum Number of OFDM symbols of a SL PRS resource in a slot - For congestion control similar to legacy, the CR limits are (pre)-configured per priority in a resource pool - Note: Similar to SL communication how to achieve the CR limit is left to UE implementation. - For a shared resource pool for positioning, the SL PRS can share the same restriction of PSSCH without specific enhancement in addition to what is already specified.

It is noted, that although congestion control introduces this parameter, there seems to be no procedure or other basic agreement which can refer to these parameters neither in PHY nor in MAC specification.

Finally, according to current version of running CR for TS 38.321 [6], there is indeed a confusion on how the multiple reservations are supposed to be supported other than by retransmissions.

As a result, it is suggested to at least clarify within RAN1 how the multiple reservations are utilized by PHY, MAC, or higher layers.

Proposed Conclusion: RAN1 assumes that higher layers may provide more than one resource within a period or in case of reservation without periodicity, and the multiple resources may be utilized for transmission of multiple SL-PRS in different slots as part of one positioning session.

It is up to RAN2 to specify a mechanism for selection of multiple resources for SL-PRS so that MAC layer provides multiple resources to PHY layer for the purpose of reservation by SCI 1-B if 1 or 2 future reservations are configured.

Send an LS to RAN2 informing them of the above conclusion.

In addition, to resolve potential confusion with the maximum number (re-)transmissions, it is suggested to clarify that this parameter is also applicable outside of congestion control framework. Accordingly, this information would need to be included as part of the default Tx configuration for a dedicated SL PRS resource pool as is suggested in [7].

Proposed Conclusion: “Maximum Number of SL PRS (re-)transmissions” parameter is also applicable when congestion control is not used.

# Conclusions

In this contribution, we discussed the remaining issues for resource allocation for SL PRS. Further, we summarize the proposals as follows:

Agree on TP#1 for the restrictions on SL PRS transmission in a dedicated resource pool.

Agree on TP#2 for the restrictions on SL PRS transmission in a shared resource pool.

No further restrictions of the SL PRS transmissions parameters are defined for congestion control.

Proposed Conclusion: RAN1 assumes that higher layers may provide more than one resource within a period or in case of reservation without periodicity, and the multiple resources may be utilized for transmission of multiple SL-PRS in different slots as part of one positioning session.

It is up to RAN2 to specify a mechanism for selection of multiple resources for SL-PRS so that MAC layer provides multiple resources to PHY layer for the purpose of reservation by SCI 1-B if 1 or 2 future reservations are configured.

Send an LS to RAN2 informing them of the above conclusion.

Proposed Conclusion: “Maximum Number of SL PRS (re-)transmissions” parameter is also applicable when congestion control is not used.

# References

- Chairman’s notes, RAN1#114bis Meeting, October 2023.

- R1-2311142, “On Measurements and Reporting for SL Positioning”, Intel Corporation, November 2023.

- R1-2310764, “Corrections on the support for Expanded and Improved NR Positioning”, October 2023.

- Chairman’s notes, RAN1#112b-e Meeting, April 2023.

- 38.331, “NR; Radio Resource Control (RRC); Protocol specification”, v17.6.0, September 2023.

- [Post123bis][409][POS] Rel-18 positioning MAC CRs (Huawei)

- R1-2311141, “Higher layer parameters for Rel-18 Expanded and Improved NR Positioning,” Rapporteur (Intel Corporation), RAN1 #115, November 2023.