3GPP TSG RAN WG1 #116		R1-24xxxxx

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	7.1

Source:	Google

Title:	Summary on PUSCH Rate Matching

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide a summary on PUSCH rate matching based on draft CR R1-2400387 as follows.

| Title: | Draft CR on Rate Matching for PUSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT_Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently, the buffer size for LBRM is defined based on the UE reported maximum number of layers if the UE is not provided the configuration of the maximum number of layers. However, if the UE supports non-codebook based transmission, it can report two UE capabilities: maxNumberMIMO-LayersCB-PUSCH for codebook based transmission scheme and maxNumberMIMO-LayersNonCB-PUSCH for non-codebook based transmission scheme. Then how to determine the buffer size for LBRM could be one problem. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the maximum number of layers for buffer size determination for LBRM should be determined based on the UE capability for the configured transmission scheme. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The LBRM buffer size when the maximum number of layers is not configured is unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: No impact as this is common understanding. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

Proposed change

#### 5.4.2.1 Bit selection The bit sequence after encoding from Clause 5.3.2 is written into a circular buffer of length for the -th coded block, where is defined in Clause 5.3.2. For the -th code block, let if and otherwise, where, , is determined according to Clause 6.1.4.2 in [6, TS 38.214] for UL-SCH and Clause 5.1.3.2 in [6, TS 38.214] for DL-SCH/PCH, assuming the following: - maximum number of layers for one TB for UL-SCH is given by X, where - if the higher layer parameter maxMIMO-Layers of PUSCH-ServingCellConfig of the serving cell is configured, X is given by that parameter - elseif the higher layer parameter maxRank of pusch-Config of the serving cell is configured, X is given by the maximum value of maxRank across all BWPs of the serving cell - otherwise, X is given by maxNumberMIMO-LayersNonCB-PUSCH for the serving cell if txConfig is set to ‘nonCodebook’, and X is given by maxNumberMIMO-LayersCB-PUSCH for the serving cell otherwise

# Discussion

Based on the first round of online discussion, there seems to be the following understandings on the default value of X (maximum number of layers for one TB for UL-SCH).

- Option 1 (Proposal in the draft CR): Default value of X is maxNumberMIMO-LayersNonCB-PUSCH for the serving cell if txConfig is set to ‘nonCodebook’, and X is given by maxNumberMIMO-LayersCB-PUSCH for the serving cell otherwise

- Option 2: Default value of X is given by the maximum value of maxNumberMIMO-LayersNonCB-PUSCH and maxNumberMIMO-LayersCB-PUSCH for the serving cell

- Option 3: Default value of X is given by maxNumberMIMO-LayersCB-PUSCH for the serving cell

- Option 4: Default value of X is undefined

  - The gNB should always configure the maxMIMO-Layers or maxRank

- Other option (please specify)

Companies’ view

| Company | Which option is your understanding | Comment |
| --- | --- | --- |

# Conclusion

TBD