| 3GPP TSG RAN WG1 Meeting #118 | R1-2406083 |
| --- | --- |
| Maastricht, Netherlands, August 19th – 23th, 2024 |  |

| Agenda Item: | 9.6.1 |
| --- | --- |
| Source: | EURECOM |
| Title: | Discussion on LP-WUS and LP-SS Design |
| Document for: | Discussion and Decision |

# Introduction

In 3GPP wake-up signals (WUS) are specified in both LTE-M/NB-IoT as well as NR-Rel-17 under the name of paging early indication (PEI). The key difference between those WUS and a low-power (LP)-WUS is that the LP-WUS is received with a low-power wake-up receiver (WUR) independent of the main radio, i.e. the main radio can be turned off.
A SI [1] on low-power (LP) wake-up signal (WUS) and receiver for NR has been carried out in Rel-18 with the report available in [2]. In Rel-19, a WI [3] was agreed including the following objective related to LP-WUS and LP-SS design:

Objectives:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

  - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

    - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information.

  - At least duty-cycled monitoring of LP-WUS is supported

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

    - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI.

    - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS.

    - Y will be decided within WI. 320ms is the start point.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

This contribution focuses on LP-WUS design in RRC IDLE mode.

# OOK Waveform Designs

Regarding the LP-WUS waveform design the following has been agreed in RAN#116-bis:

Agreement:

For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS.

- FFS whether value of M depends on SCS

- FFS M=1 for OOK-4

In this section, we review the OOK modulation and the 2 agreed OOK schemes, OOK-1 and OOK-4.

We denote N PRBs as the bandwidth of the LP-WUS including potential guard bands (GB). The actual number of used PRBs for the WUS and GB are referred to as NWUS and NGB, respectively, such that N=NWUS+NGB.

Moreover, we refer to the ON-signal a as the sequence of complex symbols allocated to the OOK symbol if the corresponding bit is one.

The overall DL transmission block diagram is depicted in Figure 1. The B information bits b=[b0,b1,…,bB-1] are encoded and the resulting C coded bits c=[c0,c1,…,cC-1] are modulated onto L consecutive OFDM symbols each carrying M bits. That is, M is the number of coded bits per OFDM symbol or the number of OOK symbols. Subsequently, the signal in frequency-domain Sm of message m=0,1,…,2B-1 is mapped to the overall resources Xm of N sub-carriers and OFDM-modulated resulting in the time-domain signal xm(t).

Figure 1: Overall transmission block-diagram per OFDM symbol.

Subsequently, we review OOK modulation as well as the specific OOK-1 and OOK-4 modulations in the WI objectives.

## OOK Modulation

Denote the OOK modulated signal sm in time-domain for message m=0,1,…,2C-1 as

sm=[s0,s1,…,sC-1]

where C is the number of coded bits transmitted per OFDM symbol l and si=[si0,si1,…,siNB'-1] with NB'=N'/C the number of samples for sequence si, i=0,1,…,C-1. The OOK modulation consists of mapping an ON-signal or ON-sequence a=[a0,a1,…,aNB'-1] to si whenever the corresponding bit is one, i.e.

si=0,  &ci=0a,  &ci=1

Hereby, the ON-signal a can be any complex sequence and can be different for every si.

In case of multiple ON-sequence, consider Q ON-sequences aq able to encode B2= log2Q bits.

## OOK-1: Single-bit in 1 OFDM symbol

The simplest OOK scheme allocates the ON-signal a of length NWUS to the corresponding WUS SCs if the bit is one and zeros otherwise (in baseband).

A block-diagram is shown in Figure 2.

Figure 2: Block-diagram for OOK-1.

This transmission scheme can only transmit a single bit per OFDM symbol. Thus, the only means to increase the data rate is to shorten the OFDM symbol length by increasing the sub-carrier spacing (SCS).

## OOK-4: Transform M-bit OOK in time domain

This scheme uses DFT-precoding to convert M OOK symbols in time-domain to frequency domain for allocation to NWUS PRBs.

An example block-diagram is depicted in Figure 3. From the M (coded or physical) bits, a signal of length N' is generated where each bit is mapped to a sequence si of length NB', the sequence is 0 if the corresponding bit is zero and si=a otherwise. The resulting sequence is DFT precoded and the output is mapped to the overall transmission bandwidth.

Figure 3: OOK-4, Block-diagram with M=4.

## Waveform Generation

For the discussion on LP-WUS waveform generation, we borrow the block-diagram from the feature lead summary in Figure 4.

Figure 4: Block-diagram for LP-WUS waveform generation.

### Pulse Shaping at S1

Pulse shaping should be considered together with the ON-sequence design. It has been shown that shorter pulses, or ON-sequences that concentrate the energy in the middle of the pulse, are more robust to timing errors. However, short pulses will capture less multi-path diversity and it is therefore necessary to ensure that the duration of the pulse is sufficiently long.

Moreover, a potential preamble and/or a receiver that applies time-windowing (sliding window), will mitigate the impact of timing inaccuracy. Hence, shortening the pulse might not be necessary/beneficial after all.

Proposal 1: Consider if pulse-shaping is required after sequence design and potential preamble are agreed.

### DFT-Shift for Signal at S1

Given the generation of the OFDM waveform, a DFT-shift might be required to ensure that the LP-WUS spectrum integrates seamlessly into the overall NR spectrum.

For an energy detector, the DFT-shift has no impact on the detection performance. However, a coherent receiver needs to know if a DFT-shift has been applied or not. In general, the DFT shift can be applied at the gNB or at the receiver. We don’t have a strong preference but since the standard MR OFDM-based receiver compensates for shift, the same could be assumed for the LR.

Proposal 2: The DFT-shift is compensated at the LR.

### Mapping of WUS at S2 to existing NR signals

It has been proposed to map the frequency-domain WUS to existing NR signals in order to reduce gNB complexity and to improve robustness to frequency errors. In general, quantizing or mapping, the frequency-domain WUS will change its properties in time-domain. From a standard perspective, such a mapping is not necessary since arbitrary values are allowed in frequency domain, e.g. consider precoded signals for MU-MIMO which surely are different from QAM constellation or existing sequences.

Moreover, for OOK-4 such mapping is difficult and needs to ensure that the performance of the energy detection receiver is not impaired. Hence all supporters should evaluate the performance of both OFDM sequence detection and energy detection.

Proposal 3: Do not consider mapping/quantizing WUS in frequency-domain.

### LP-WUS Multiplexing Before or After IFFT

It has been proposed to multiplex LP-WUS with NR in time-domain to reduce interference with legacy NR transmission. We do not see any advantage in time-multiplexing the two transmissions. The resource overhead due to the additional CP will be approximately the same as the required guard-bands when the two transmissions are multiplexed in frequency-domain. Simulations have shown that with sufficient GBs, the impact of NR interference is negligible. On the other hand, TDM requires two processing chains at the gNB which increases complexity.

Proposal 4: Multiplexing NR and WUS in frequency-domain is the base line.

## Specifying Signal in Time or Frequency Domain

For OOK-1, the signal can be either specified in time-domain or frequency-domain. The specification impact is small and the gNB complexity is identical since the frequency-domain signal will be precomputed and stored in gNB memory. However, for a harmonized framework, OOK-1 and OOK-4 signal generation should follow the same procedure.

Concerning OOK-4, we think a generation in time-domain is the easiest and the most comprehensible approach. In the “worst” case, i.e. for M=4, there are 4 different signals per OFDM symbol (assuming Manchester Coding) which can easily be pre-stored at the gNB. A time-domain approach would follow three simple steps, encoding, sequence mapping and DFT precoding, which is very clear for anyone reading the specification.

Proposal 5: Specify OOK-1 and OOK-4 signal generation in time-domain.

# Overlaid OFDM Sequences

The following agreements have been made during last meeting RAN1#116-bis

Agreement:

For evaluation purpose on LP-WUS, companies report the overlaid OFDM sequence(s), including:

- Sequence(s) generation and how sequence(s) map in time or frequency domain (including any details with multiplexing and IFFT).

- Number of candidate overlaid OFDM sequences used for information conveying

  - Including details on whether the number of candidate overlaid sequences is per OFDM symbol or per OOK symbol

- How the proposed sequence design is processed by OFDM-based LP-WUR, e.g., in time domain or in frequency domain or in both time and frequency domain.

## How to carry Information by OFDM sequences

The relevant agreement from last meeting RAN1#116-bis states:

Agreement:

Regarding the overlaid OFDM sequence(s) of LP-WUS, consider the following options:

- Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol or OFDM symbol duration. OFDM-based LP-WUR can obtain the whole information bits by the presence of the overlaid sequence.

- Option 1-2: The overlaid OFDM sequence is pre-determined from multiple sequences. This sequence carry NO information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the OOK ON/OFF pattern.

- Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol or OFDM symbol duration, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options.

- Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK symbols.

- Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s)

- Option 3: One sequence is selected from multiple candidates overlaid OFDM sequences on one or more OOK ‘ON’ symbols, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s).

- Option 4: Use of modulated overlay sequence with constellation point: overlay sequence acting as a spreading sequence and constellation point carrying information for OFDM-based LP-WUR.

Other options are not precluded.

Option 1:
This is the straight-forward base-line option. A single ON-sequence is defined and hence no information is carried by the sequence.

Option 1-2:
This is a small variation of Option 1 in the sense that the single ON-sequence is somehow configured by the gNB among a certain set of possible ON-sequences. The purpose is not quite clear to us. Different cells should choose different sequences with good cross-correlation properties in the case the time-frequency resources of those cells overlap and would cause interference. But this will only solve the inter-cell interference problem for correlation-based LR. Energy-detectors will still receive energy from different cells, in which case it is desirable that the WUS resource of different cells do not overlap.

The LLS in Figure 5 evaluates Option 1 for three types of receivers (cf. Appendix). As expected, for Manchester Coding R=1/2, there is a ~2dB and ~6dB gain over the ED-WUR for COR-WUR-OOK and COR-WUR, respectively. The larger gain of the COR-WUR is due to the processing gain by carrying out correlations over the entire WUS which requires a more complex receiver.

Applying Pulse Position Coding (PPC) (see Section 5.2) yields a gain of 3dB, 2.8dB and 1.8dB over Manchester Coding for ED-WUR, COR-WUR-OOK and COR-WUR, respectively. The gain for COR-WUR is smaller because the gain does not come from the increased SNR per OOK symbol, but from reduced cross-correlation among the possible transmit signals.

Figure 5: WUR performance in TDL-C with 𝑀=4, 𝐵=8 and single ON-sequence (Q=1).

Observation 1: Correlation receiver achieves significant gain over energy detection.

Observation 2: For M=4, Pulse Position Coding achieves significant performance gain for all receiver types.

Option 2:
A straight-forward extension of Option 1 to multiple sequences. In our opinion this should be the base-line option for multiple sequences. There are 2 variations of Option 2 depending on whether all information is carried in the sequences (Option 2-2) or not (Option 2-1). In our understanding, both options make sense depending on the use case. For instance, consider IDLE mode with an 8-bit WUS payload to signal groups independently. The UE knows which bit corresponds to its assigned group. Therefore, it will only decode this position/sequence. Option 2-1 would carry groups 1-4 in OOK ON-positions and groups 5-8 in the sequence overlay. Good, the UEs with sequence detector can decode their groups after 8 OOK symbols. But what sequence overlay to use for the remaining 8 OOK symbols? Naturally, the sequence-overlay should continue and groups 1-4 should be carried in those remaining OOK symbols. The mapping should be such that combining is possible to increase coverage for the correlation-based receivers. Hence, this would be Option 2-2.

Option 3:
This is an extension of Option 2 where the sequence can span multiple OOK symbols, for one OOK symbol it is identical to Option 2. One issue might be if the sequence spans more ON-symbols than fit in an OFDM symbol, because a partial sequence may have undesirable spectral properties. There is also the issue of the pulse shape in time-domain. The shape might be different for consecutive pulse because they correspond to different parts of the sequence which in turn may result in the case that some pulses (and hence bits) are better decodable than others. Moreover, the sequence length will then depend on M which requires specification of sequences for different M. Another disadvantage concerns inter-cell interference since neighboring cells may not have the same configuration of M. Thus, different interfering ON-symbols may have bad cross-correlation properties resulting in interference. 
The advantages of longer sequences are that they have better cross-correlation properties than shorter sequences. However, this assumes that the correlation-receiver performs correlation across those longer sequence and hence across multiple ON-symbols which benefits from the inherent processing gain resulting from longer correlations. This can also be done with shorter sequences spanning only a single ON-symbol and hence is not an inherent advantage of Option 3.

Option 4:
This is a variation of Option 2, where multiple sequences are generated by multiplying the base ON-sequence with a complex (unit-power) modulation symbol, e.g. BPSK, QPSK, 8-PSK, effectively rotating the sequence by a payload dependent phase. We think that this is not an alternative to the other Options since it describes how the sequences are generated as opposed to how the information is carried. This option should be treated in the sequence design discussion.

Proposal 6: Only Option 1 and Option 2 should be further considered.

### Option 2: Multiple Sequences

The objective states that multiple ON-sequences aq can be defined, with q=1,2,…,Q, and that they can encode the same information b.

Recall the agreement on Option 2:

Agreement:

- Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol or OFDM symbol duration, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options.

- Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK symbols.

- Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s)

To better understand the issue, consider the example in Figure 6, where the payload b=[11011000] is transmitted with M=4 over L=4 OFDM symbols.

Figure 6: Example of OOK-4 WUS, Manchester coding and 8 groups, M=4, B=8 b = [11011000] and Q=2 sequences.

Denote bS the bit sequence carried by the overlaid OFDM sequences. If Q=2 sequences are available, one bit can be carried per ON-symbol. One option (Option A) is to transmit bS=bS0=b, i.e. the same bit sequence that is carried via the OOK modulation. This option would require the COR-WUR to receive all 4 OFDM symbols to decode the entire payload. Another option (Option B) is to transmit bS=bS1=[b4, b5,b6,b7,b0,b1,b2,b3], or any permutation in the first 4 bits and the associated last 4 bits. As with Option A, the COR-WUR needs to receive all OFDM symbols in order to decode the entire payload. However, it (i.e. a per-OOK symbol correlation receiver) can combine the correlation results of the first 2 OFDM symbols with the last 2 OFDM symbols to increase performance. For a COR-WUR that correlates over the entire WUS this mapping effectively reduces cross-correlation and improves performance.

Since it has been agreed that the same information is transmitted to both receivers, the base station has to transmit all 4 OFDM symbols anyway. Hence it can overlay the OFDM sequences for the entire transmission. On the receiver side, it makes a difference how the OFDM sequences are overlaid. With Option A, the COR-WUR has to receive the whole transmission. With Option B, it can turn off after 2 OFDM symbols or can receive the entire transmission with increased redundancy and performance.

Therefore, Option 2-1 does not really make sense unless we decide that once the COR-WUR has received all the WUS information, we stop transmitting multiple overlaid OFDM sequences and move back to Option 1 for the rest of the OOK WUS transmission. It is much more sensible to continue transmitting multiple overlaid OFDM sequences and improve the coverage of COR-WUR receivers.

Observation: Support Option 2-2, utilizing multiple overlaid OFDM sequences for the entire WUS transmission.

The exact bit sequence carried by the overlaid OFDM sequences can be further studied. One option was presented above as Option B and will further detailed in the next section.

Proposal 7: Evaluate how the information bits are mapped to multiple overlaid OFDM sequences.

### Option 2: Mapping of WUS Information to Sequences

The objective states that the same information b is transmitted irrespectively of the WUR-type, i.e. the information encoded with Q available sequences is part of b.

For better understanding, consider the example in Figure 7, where groups 1,2,4 and 5 are signaled to wake up using an OOK-4 waveform with M=4 and Manchester coding, i.e. 2 groups can be signaled in a single OFDM symbol. It can be observed, that each group is encoded in OOK-symbols at a specific time. Thus, the UEs are not required to decode the entire OOK-waveform, but may only decode the portion containing their group ID. If repetition is required, it may be advantageous to repeat a group ID on consecutive OOK symbols so that the ED-WUR may go back to sleep faster and save power.

Figure 7: Example of OOK-4 WUS, with M=4, Manchester coding and 8 groups, b = [11011000].

Concerning the COR-WUR, the information is decoded by correlating with a known ON-sequence(s) aq. In case of a single ON-sequence, the information can be decoded by correlating each OOK symbol with the known sequence and comparing the absolute value of adjacent OOK symbol (in case of Manchester coding). With this strategy (COR-WUR-OOK), the OOK waveform of the sequence-based waveform are of the same duration because the same information is transmitted to both receiver types.

If multiple ON-sequences are used to encode information, the question is which groups should be signaled via the sequences because the groups encoded with OOK are fixed.

Figure 8 show a joint encoding with Q=2 sequences, the number in the ON-pulse denotes the group ID that is signaled with sequence encoding. Therefore, the COR-WUR can decode the payload B by only receiving the first 2 OFDM symbols instead of all 4 OFDM symbols. To improve detection performance of the COR-WUR, the payload can be partially repeated on the last 2 OFDM symbols. To allow for combining at the COR-WUR, the same groups have to be encoded in one coded OOK symbol (2 consecutive symbols in this example). More precisely, the first 2 OOK symbols encode group 1 through position of the ON pulse and group 5 through the ON-sequences. When group 5 is signaled with OOK encoding in the first two pulses in the second OFDM symbol, the ON-sequence encodes group ID 1. Hence, the correlation values of both cases can be combined since they signal the same groups.

Figure 8: Example of OOK-4 WUS, Manchester coding and 8 groups, M=4, B=8 b = [11011000] and Q=2 sequences.

Figure 9 shows an example with pulse position coding (PPC) and otherwise the same parameters as in the previous example in Figure 7. It can be observed that the position of the ON-sequence within an OFDM symbol encodes 2 group IDs.

Figure 9: Example of OOK-4 WUS, pulse position coding and 8 groups 𝑀=4, 𝐵=8, b = [11011000], 𝐿=4 𝑂𝐹𝐷𝑀 𝑠𝑦𝑚𝑏𝑜𝑙𝑠 and 4 sequences.

Since the power is concentrated in a single OOK-symbol instead of 2 OOK symbols, there is a 3 dB gain for both ED-WUR and COR-WUR-OOK. However, in case of multiple ON-sequences, less bits can be encoded than in the example in Figure 8 because there are fewer ON-pulses. For instance, 2 ON-sequences can encode 4 bits over the entire OOK-4 waveform, as opposed to 8 bits in Figure 8. However, the power in the ON-pulse is larger which results in an overall performance gain.

Figure 10 provides link-level simulation results for multiple overlaid OFDM sequences in various settings. The results labeled “no encoding” use the same bit sequence for the overlaid OFDM sequences as carried by the OOK waveform, e.g. bS0 in Figure 6. On the other hand, the results labeled “encoding” use a different bit sequence for the overlaid OFDM sequences, e.g. bS1 in Figure 6. It can be observed that COR-WUR achieves a performance gain of ~4.4dB and ~3.3dB over COR-WUR-OOK without encoding and with encoding, respectively, due to the processing gain. For COR-WUR-OOK and COR-WUR encoding yields a 3.3dB and 2dB gain compared to no encoding, respectively.

Figure 10: WUR performance in TDL-C with 𝑀=4, 𝐵=8 and multiple ON-sequences.

With PPC and Q=4 sequences, we observe a 2.8dB and 2.4dB gain for COR-WUR-OOK compared to Manchester coding without and with encoding, respectively. In case of COR-WUR those gains are 2dB and 1dB, respectively.

In summary the following observations are made:

Observation 3:

- COR-WUR performs better than COR-WUR-OOK due to the processing gain of carrying out longer correlations.

- Transmitting the same payload as the OOK waveform with the overlaid OFDM sequences but in a different bit sequence yields a significant performance gain.

- Using Pulse Position Coding and increasing the number of sequences results in a significant performance gain

Proposal 8: For multiple ON-Sequences, jointly encode the payload with OOK and sequence encoding.

### Non-Coherent Time-Overlay Code

In this section, we propose to generate multiple sequences by multiplying base sequences with a modulated symbol. The modulated symbols are obtained by mapping encoded bits to a (complex) constellation point. This method has similarities with the method proposed in Option 4, with the difference that the modulated symbols are the result of a non-coherent encoding procedure.

More precisely, consider L ON-symbols in the WUS transmission which transmits a payload of B bits b=[b0,b1,…,bB-1]. The goal is to encode b using Q OFDM overlay sequences such that the decoding performance is maximized. To this end, we encode b by using a linear non-coherent code G such that

c=dG

where d, c  and G are the input bits, coded bits c=[c0,c1,…,cC-1] of length C and the generator matrix, respectively. Subsequently, the modulated N-PSK symbol wml of message m, m=0,1,…,2B-1 and ON-symbol l is obtained by

wml=ei2πcn/N

where cn is the nth entry of c. Finally, the transmitted sequence sml is given by

sml=sl∙wml

where sl is the base sequence transmitted on OOK ON-symbol l.

As an example, consider N=4 (e.g. QPSK), B=8 with R=1/2 Manchester Coding, i.e. L=8, a good generator matrix in GF4 is given by

G= 10003333010012330010213300011213

The gNB will have pre-stored the codebook of all possible WUS transmissions sm and the receiver correlates the received signal with all possible transmit messages as detailed in the COR-WUR receiver in Section 8.1.2.2. Note that, the receiver requires joint correlation across the time-overlay duration which also assumes that the channel is coherent during that period.

In order for the code to minimize cross-correlation between all possible messages sm, at least 2 orthogonal base sequences are required.

Figure 11 shows the performance of different encoding schemes for Q=2 base sequences. As in the simulation in Figure 10, the mapping of the bits for sequence encoding has a significant performance impact, about 2dB difference between “encoding” and “no encoding”. By additionally applying the non-coherent code “encoding + time-overlay” we observe an additional 1dB gain. This gain is due to the fact that the code improves cross-correlation between the transmitted codewords. There is no additional computational cost at the transmitter or receiver. Moreover, the time-domain overlay does not impact the performance of the ED-WUR.

Figure 11: WUR performance in TDL-C with 𝑀=4, 𝐵=8 and non-coherent encoding scheme.

Observation 4: A time-domain overlay code can significantly improve performance of the overlaid OFDM sequence transmission.

## Overlaid OFDM Sequence Design

The relevant agreement from last meeting RAN1#117 states:

Agreement:                                                                                                                                            RAN1#117

Support overlaid OFDM sequence based on existing NR sequence type for LP-WUS

- Down select among gold sequence, m sequence and ZC sequence

  - FFS the overlaid OFDM sequence is time or frequency domain sequence.

  - FFS how to reuse the existing sequences

  - Note: Strive to minimize the impact on OOK detection performance

- If overlaid OFDM sequence is supported for LP-SS, the same sequence type is used for both LP-SS and LP-WUS

Zadoff-Chu sequences are a natural candidate for multiple reasons:

- A sequence design is already available in the specification

- Different roots provide good cross-correlation properties to mitigate inter-cell interference for correlation-based receivers

- Cyclic-shifts provide easy way to generate multiple sequence with good cross-correlation properties

- Good spectral properties

Proposal 9: Consider Zadoff-Chu sequences as base line.

Concerning the specification of the overlaid OFDM sequence, the following agreement has been reached in RAN1#117

Agreement:                                                                                                                                            RAN1#117

For overlaid OFDM sequence(s) for LP-WUS in time or frequency domain, down-selection from the following:

- Option 1-1: overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing

  - The length of overlaid sequence(s) depends on the number of REs used for LP-WUS and the value of M

- Option 1-2: overlaid sequence(s) are the sequence(s) of an OFDM symbol before DFT/LS processing

  - The length of overlaid sequence(s) depends on the number of REs used for LP-WUS

- Option 2: overlaid sequence(s) are the sequence(s) of an OFDM symbol before IFFT processing

  - The length of overlaid sequence(s) depends on the number of REs used for LP-WUS

- Option 3: overlaid sequence(s) are the sequence(s) of an OOK on symbol in time domain after IFFT processing

  - The length of overlaid sequence(s) depends on IFFT size and the value of M

FFS: same or different options are applied for OOK-1 and OOK-4 M>1.

We support Option 1-1, where the overlaid OFDM sequence(s) are defined as the time-domain OOK ON-sequence. Hence, for a given BW, the length of the ON sequence depends on M.

Proposal 10: Support Option 1-1: Overlaid sequences are the OOK ON-sequences.

# How to Carry Information Bits by OOK symbols

## Encoding or Sequence Selection

It has been discussed if the information bits should be (1) encoded and mapped directly to OOK symbols or (2) jointly encoded via sequences spanning multiple OOK symbols.

We are in favor of the first approach because the second approach has many practical drawbacks including:

- Decoding requires to receive the entire sequence, i.e. no early-termination or just decoding the bit of interest.

- The number of ON-sequences will vary per OFDM symbol for M>1 resulting in different PSD per OFDM symbol.

- It is unclear how multiple OFDM sequences can be overlaid since the number of ON-symbols is supposedly different for every sequence.

- Higher specification impact since those sequence need to be defined

- The sequences will be very long for larger payloads

Interestingly, Manchester Coding could be thought of as a special case of OOK sequence encoding where the encoded sequences are orthogonal.

Hence, Manchester Coding + per bit encoding is sort of a hybrid approach of the two options.

Proposal 11: Encode information per bit and not jointly via non-orthogonal sequences.

## How to carry LP-WUS information

### RRC IDLE/INACTIVE

The relevant agreement from last meeting RAN1#117 states:

Agreement:                                                                                                                                            RAN1#117

For RRC idle/inactive state, down-select among the following options for at least indicating subgroup information using LP-WUS:

- Option 1: A LP-WUS indicates a bitmap with each bit corresponding to one subgroup of N subgroups for part of, one or more PO(s), e.g., N is 8~16, 24

  - Number of information bits for a LP-WUS is at least N, single LP-WUS to wake up one or more subgroups

- Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs, e.g., N is 8~256

  - Number of information bits for a LP-WUS is at least ceil (log2(X1)), where X1 is the number of codepoints indicating one or more subgroups. X1 is reported by companies, X1 could be smaller, equal to or larger than N.

- Option 3: A LP-WUS indicates multiple codepoint values with each corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs, e.g., N is 8~256

  - Number of information bits for a LP-WUS is at least K*ceil (log2(X2)), where X2 is the number of codepoints indicating one or more subgroups. X2 is reported by companies, X2 could be smaller, equal to or larger than N.

  - K is the number of multiple codepoint values in a LP-WUS where K is larger than 1

- How to satisfy FAR is reported by companies, e.g., FEC/CRC

- Note: multiple TDMed LP-WUSs can be used to support more subgroups for each option.

- Note: Y% effective paging rate per PO is reported by companies

- The followings are considered when down-select among options:

  - The number of supported UE subgroups per PO: M

  - Average network overhead to indicate the number of UE subgroups M per PO

  - False wake up rate due to subgroup-based indication, which will impact the power saving gain

  - Paging latency

  - Note: Coverage target shall be met under 1%BLER, 1% FAR (for false alarm from noise)

Option 1 should be adopted as the baseline option.

As eluded to in the agreement, these options are somewhat associated to the encoding of the WUS, i.e. encoded bits or sequence selection. Option 1 works well with encoded bits and Options 2 and 3 are catering to sequence selection. From our perspective, and as motivated in the previous section, the WUS information should be carried by encoded bits and hence Option 1 is a straight-forward approach.

Proposal 12: LP-WUS information payload is encoded sequentially where, for instance, every bit corresponds to a sub-group, Option 1.

### RRC CONNECTED

The relevant agreement from meeting RAN1#116-bis states:

Agreement:                                                                                                                                      RAN1#116-bis

Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs, at least consider the following：

- Option 1: A bitmap with each bit corresponding to [one or more] UEs

- Option 2: A codepoint value corresponding to one or part of UE identity, e.g., C-RNTI

- Option 3: A codepoint value corresponding to [one or more] UEs

- Option 4: Multiple codepoint values with each corresponding to [one or more] UE(s)

- Option 5: Multiple bit blocks with each corresponding to [one or more] UE(s)

- Combination of above options are not precluded.

- FFS how to carry LP-WUS information, e.g, by encoded bits (with/without CRC) and/or by OOK sequence selection for ‘ON-OFF’ pattern for OOK symbols of LP-WUS.

- FFS how to carry LP-WUS information by overlaid OFDM sequences.

  - It doesn’t preclude considering the configuration where a single candidate overlaid OFDM sequence is used

- FFS details of LP-WUS information to trigger PDCCH monitoring (e.g. whether above is applicable to one or more serving cells)

The same discussion as in the previous section applies here as well. As a RAN1 group we should focus on how the payload is transmitted and not what the payload is.

The only difference to IDLE/INACTIVE mode is that the payload size may be larger because of the use case that one or multiple UEs are addressed directly. Consequently, OOK sequence selection can be ruled out as an encoding technique since, among other drawbacks, the resource overhead is too high for larger payloads, e.g. a 16-bit UE ID.

# Coding

The study on LP-WUS [2] suggests to utilize Manchester Coding, where one input bit is mapped to two coded bits, i.e. 0->{0,1} and 1-> {1,0}. This coding technique is especially adapted to OOK with ED since it does not require threshold detection but a simple energy comparison of the two OOK symbols is sufficient for reliable decoding.

## Discussion on Manchester Coding

The discussion on coding in the previous meeting RAN1#117 resulted in a rather high-level Proposal that was brought up in the online-session but there was not enough time to discuss:

Firstly, it has been suggested that MC might not be necessary at all if there is a preamble which can be used to estimate the optimal threshold for OOK detection. For AWGN, the optimal threshold b is known and can be well approximated by

b=σ1+γ4

with SNR γ and noise variance σ2. Since the preamble is known, noise and SNR can be estimated. However, the assumption is then that these values do not change during the WUS data transmission part. This is a strong assumption especially because the WUS duration is rather long. Secondly, and more importantly, the optimal threshold computation is unknown for fading channels and thus a threshold estimation is difficult and will almost surely result in performance degradation. Lastly, threshold estimation requires additional, and likely complex, computations at the receiver which consumes power and goes against the principle design requirement of a low-power receiver.

Observation 5: Manchester coding is required to avoid complex threshold estimation for low-power receivers.

Moreover, we suggest to make a potential proposal more detailed by including options on how to specify the mapping:

- 0 [0 1] and 1[1 0] or

- 0 [1 0] and 1[0 1]

As discussed in Ambient IoT, the IEEE 802.3 convention is Option 2 because the encoded signal will just be an XOR of the clock and the data. As a result, the following agreement has been reached in A-IoT:

Therefore, we suggest to following this MC mapping.

Proposal 13: Specify Manchester Coding as 0 [1 0] and 1[0 1].

Another important point to be included in a potential agreement is how to handle the case of M=4. Following the Manchester coding scheme, two of the four available OOK symbols per OFDM symbol will be ON. Hence, the available transmit power per OFDM symbol is divided among the two ON-symbols.

In the next section we discuss an alternative approach where two input bits are encoded in the position of a single ON symbol.

## Pulse Position Coding for M=4

The detection performance of Manchester Coding can be improved by jointly encoding multiple bits. More precisely, B input bits b= b0,b1,…,bB-1 are encoded to C output bits c=c0,c1,…,cC-1 according to

c=2m

where m=0,1,…,2B-1 is the message and c is the decimal representation of codeword c. For B=1, we obtain the conventional Manchester code of rate R=1/2 presented in Table 1.

| Input Bits | Coded Bits |
| --- | --- |
| 0 | 01 |
| 1 | 10 |

Table 1: Encoding for B=1 input bits and C=2 coded bits, rate R = ½.

Encoding B=2 bits into C=4 coded bits is given by Table 2.

| Input Bits | Coded Bits |
| --- | --- |
| 00 | 0001 |
| 01 | 0010 |
| 10 | 0100 |
| 11 | 1000 |

Table 2: Encoding for B=2 input bits and C=4 coded bits, rate R = ½.

As can be seen from Table 2, the input bits are encoded in the position of the one-bit, hence the name Pulse Position Coding (PPC), or Pulse Position Modulation.

Figure 12 shows an example of the transmitted waveform for the two encoding schemes without any co-scheduled transmissions. It can be observed that the pulse position coded data results in higher signal amplitudes, i.e. a power boost, compared to Manchester coding.

Figure 12: Transmit Signal for payload b=[1 1 0 1 1 0 0 1], M=4, 4 OFDM symbols, no ACI.

The main advantage of using B=2 compared to B=1, is that the total WUS transmit power per OFDM symbol is concentrated into a single ON-symbol and not divided among two ON-symbols. Hence, there is a 3dB SNR gain.

As eluded to in Section 4.1, PPC and mapping to OOK symbols can be viewed as OOK sequence selection, where the bits are jointly encoded into orthogonal sequences.

Proposal 14: For M=4, consider jointly encoding multiple bits into ON pulse position to increase SNR by 3dB.

### Decoding at the Receiver

At the receiver, the Manchester codeword is decoded by accumulating the energy of the two corresponding OOK symbols and comparing their energy. The same strategy applies to PPC where the energy is computed for four OOK symbols and the symbol with the maximum energy determines the information bits.

Discussion of Self-Clocking Aspect

Manchester coding has the advantage the there is always a transition from zero to one or one to zero in the codeword which allows for timing alignment, also referred to as self-clocking. One simple implementation to maintain timing alignment during the decoding process is an Early-Late architecture, where two parallel branches accumulate energy with one branch starting accumulation a few samples earlier (Early branch) and the other a few samples later (Late branch). By comparing the results of the two branches, one can determine if it is necessary to realign the OOK symbol timing.

With PPC, there is only one transition in the codeword but the same principle applies. The only difference is that the timing alignment is evaluated at the end of the codeword which takes four OOK symbols instead of two. However, unless the timing changes significantly within 4 OOK symbols, there is no impact on decoding performance.

Also note that timing determination, i.e. determining the rising or falling edges of the pulse, is improved in PPC since the SNR of the Pulse is higher.

### Discussion on PAPR

It has been pointed out that PPC increases the PAPR of the resulting OFDM time-domain waveform which impacts existing base-station implementations, e.g. they may fail RAN4 requirements.

In this section, we analyze the impact of PPC on Peak-to-Average Power Ratio (PAPR) of the transmitted waveform. Unless stated otherwise, the simulation assumption in Table 6 are used. The transmission power of the WUS per OFDM symbol is kept constant. For an ON-sequence a=[a0a1…aNB'-1] of length NB', a cyclically extended Zadoff-Chu sequence is used which is generate as

an= e-jπun(n+1)NZC

where n=0,1,…,NB'-1 and NZC=NP(NB') with NP(x) denoting the next prime greater or equal to x. In the simulations we use root u=1.

From the encoding, it is expected that the PAPR of PPC is 3dB higher than with MC. However, the increase in PAPR is lower, for instance in Figure 12, the PAPR is 6.8dB and 9.3dB for MC and PPC, respectively. Hence the increase is only 2.5dB because of the CP. More precisely, if the last OOK pulse within the OFDM symbol is ON, the last samples of that symbol will be copied to the CP which increases the mean power of the signal and thus decreases the PAPR. The reduction in PAPR due to the CP is especially large in PPC because of the larger power in the ON symbol which results in a much lower PAPR for those payloads, overall resulting in a reduced average PAPR

With co-scheduled 64-QAM transmissions, the transmit signal looks quite different as depicted in Figure 13. The PAPR increases for MC and PPC to 9.3dB and 11.2dB, respectively.

Figure 13: Transmit Signal for payload b=[11011001], M=4, 4 OFDM symbols, with 64-QAM ACI.

Figure 14 shows the CCDF of the PAPR for both coding schemes in the absence of ACI, i.e. LP-WUS is the only transmission in the 20 MHz BW. It can be observed that the mean PAPR is 6.74 dB and 9.37 dB for MC and PPC, respectively, i.e. PPC increase the mean PAPR by about 2.63 dB.

In both schemes, the difference between average PAPR and 1% outage PAPR is about 0.3dB. This means that only 1% of the time, the PAPR exceeds the average PAPR by 0.3dB.

Figure 14: CCDF of PAPR, no ACI, 20MHz, 30kHz SCS, M=4, 8 bit payload, 4 OFDM symbols, average over 10k realizations.

Figure 15 shows the CCDF with co-scheduled 64-QAM transmissions. The mean PAPR is 9.52dB and 10dB for MC and PPC, respectively, i.e. a difference of about 0.5dB. As expected, the 64-QAM transmissions increase the average PAPR as well as the 1% outage PAPR. The transmission exceeds the average PAPR by about 1.6 dB for both schemes, 1% of the time.

Figure 15: CCDF of PAPR, 64-QAM ACI, 1PRB GB, 20MHz, 30kHz SCS, M=4, 8 bit payload, 4 OFDM symbols, average over 10k realizations.

In summary, the impact of PPC on PAPR is manageable by the transmitter. With co-scheduled transmissions the average and 1% outage PAPR are only 0.5dB larger with PPC than MC. Without co-scheduled transmissions, the PAPR is lower for MC but still well within the acceptable range for PPC.

Table 5 provides simulation results for PAPR of various transmission schemes and system configurations.

| Scheme | 20MHz, SCS 30, 51PRBs, FFT=1024 | 20MHz, SCS 15, 106 PRBs, FFT=2048 | 10MHz, SCS 15, 52 PRBs, FFT=1024 | 100MHz, SCS 30, 273 PRBs, FFT=4096 | 20MHz, SCS 15, 106 PRBs, FFT=2048, WUS BW=2.5MHz |
| --- | --- | --- | --- | --- | --- |
| 64QAM | 9.5/11.0 | 9.8/11.3 | 9.5/11.1 | 10.1/11.5 | 9.8/11.3 |
| OOK-1, R=1/2 | 9.6/11.3 | 9.9/11.3 | 9.4/11.0 | 10.1/11.6 | 9.8/11.4 |
| OOK-4, M=1, R=1/2 | 9.5/11.1 | 9.5/11.1 | 9.5/11.0 | 10.1/11.5 | 9.8/11.4 |
| OOK-4, M=2, R=1/2 | 9.7/11.4 | 9.9/11.3 | 9.5/11.0 | 10.1/11.5 | 9.9/11.4 |
| OOK-4, M=4, R=1/2 | 9.5/11.1 | 10.0/11.6 | 9.9/11.5 | 10.1/11.5 | 9.8/11.4 |
| OOK-4, M=4, PPC R=1/2 | 10.0/11.6 | 10.7/12.2 | 11.3/12.7 | 10.2/11.6 | 10/11.6 |
| OOK-4, M=4, PPC R=1/2, 64-QAM quantization | 10.0/11.6 | 10.6/12.2 | 11.2/12.7 | 10.2/11.6 | 10/11.6 |

Table 5: Average PAPR [dB] / 1%Outage PAPR [dB], Simulation results for different encoding schemes and system configurations. WUS BW either 11 PRBs or 22 PRBs

The baseline “64QAM” assumes random 64-QAM symbols on all sub-carriers, i.e. no WUS. It can be observed that the impact on PAPR increases with larger WUS BW w.r.t. to the system BW. Moreover, up to M=2, using OOK-1 or OOK-4 does not increase PAPR w.r.t. to the baseline. For M=4 with Manchester coding R=1/2 , there is only a very small increase of PAPR (0.4 dB) compared to the baseline.

For M=4, the PAPR increase caused by PPC is about 0.5 dB and 0.7dB compared to MC if the WUS BW constitutes about 25% of the overall BW (5MHz out of 20MHz) for SCS 30kHz and 15kHz, respectively. For a 10MHz channel and 5MHz WUS the PAPR increase is about 1.4 dB. On the other hand, for a 100MHz channel and 5MHz WUS there is no increase in PAPR. Similarly, for a 20MHz channel with a 2.5MHz WUS the PAPR increase is only 0.2dB. In addition, quantizing the frequency-domain signal to 64-QAM constellation points does not impact the PAPR.

Observation 6: PAPR increase of Pulse Position Coding for M=4 compared to Manchester encoding depends on the ratio of channel BW to WUS BW and is minor (~0.1dB) for many system configurations.

Figure 16 shows the performance for M=4 (and for comparison M=2 with 4 bit payload) with Manchester coding (MC) and Pulse Position Coding (PPC) for a 20MHz BW channel and a WUS BW of 5.04 MHz (24 PRB WUS + 2 PRB GB on each side), solid lines, and 2.52 MHz (12 PRB WUS + 1 PRB GB on each side), dashed lines. It can be observed that there is a 3dB SNR gain of PPC vs. MC in both system configurations. Moreover, in this setting (depending on RX filter design etc.), reducing the WUS BW by half, results in an SNR loss of about 3dB. The performance of MC with M=2 is about the same as M=4 with PPC but only delivering half the bit rate.

Figure 16: BLER vs. SNR, ED-WUR, 20 MHz (106 PRBs), SCS=15 kHz, M=4, 8 bit payload.

Therefore, with the same performance, a network operator can save 2.52 MHz of BW by configuring a WUS BW of 2.52 MHz with PPC as opposed to a WUS BW of 5.04 MHz with Manchester coding. The difference in PAPR is 0dB (10dB in both configurations) which is only 0.2dB above the baseline of 9.8dB.

Alternatively, the operator can use M=2 but with half the bit rate.

Proposal 15: Allow configuration of Pulse Position Coding for M=4.

# Conclusion

In this contribution, the following proposals and observations have been made:

Proposal 1: Consider if pulse-shaping is required after sequence design and potential preamble are agreed.

Proposal 2: The DFT-shift is compensated at the LR.

Proposal 3: Do not consider mapping/quantizing WUS in frequency-domain.

Proposal 4: Multiplexing NR and WUS in frequency-domain is the base line.

Proposal 5: Specify OOK-1 and OOK-4 signal generation in time-domain.

Observation 1: Correlation receiver achieves significant gain over energy detection.

Observation 2: For M=4, Pulse Position Coding achieves significant performance gain for all receiver types.

Proposal 6: Only Option 1 and Option 2 should be further considered.

Proposal 7: Evaluate how the information bits are mapped to multiple overlaid OFDM sequences.

Observation 3:

- COR-WUR performs better than COR-WUR-OOK due to the processing gain of carrying out longer correlations.

- Transmitting the same payload as the OOK waveform with the overlaid OFDM sequences but in a different bit sequence yields a significant performance gain.

- Using Pulse Position Coding and increasing the number of sequences results in a significant performance gain

Proposal 8: For multiple ON-Sequences, jointly encode the payload with OOK and sequence encoding.

Observation 4: A time-domain overlay code can significantly improve performance of the overlaid OFDM sequence transmission.

Proposal 9: Consider Zadoff-Chu sequences as base line.

Proposal 10: Support Option 1-1: Overlaid sequences are the OOK ON-sequences.

Proposal 11: Encode information per bit and not jointly via non-orthogonal sequences.

Proposal 12: LP-WUS information payload is encoded sequentially where, for instance, every bit corresponds to a sub-group, Option 1.

Observation 5: Manchester coding is required to avoid complex threshold estimation for low-power receivers.

Proposal 13: Specify Manchester Coding as 0 [1 0] and 1[0 1].

Proposal 14: For M=4, consider jointly encoding multiple bits into ON pulse position to increase SNR by 3dB.

Observation 6: PAPR increase of Pulse Position Coding for M=4 compared to Manchester encoding depends on the ratio of channel BW to WUS BW and is minor (~0.1dB) for many system configurations.

Proposal 15: Allow configuration of Pulse Position Coding for M=4.

# References

[1] RP-213645, “New SID: Study on low-power Wake-up Signal and Receiver for NR”, vivo, RAN#94e

[2] TR-38.869, “Study on low-power Wake-up Signal and Receiver for NR”, V.18.0.0, 3GPP, 2023

[3] RP-234056, “New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, CMCC (moderator, RAN VC), RAN#102, Edinburgh, GB, December 11-15, 2023

[4] R1-2405708, “Summary # 5 of discussions on LP-WUS and LP-SS design”, Moderator (vivo), RAN1#117

# Appendix

## Receiver Algorithms

### Envelop/Energy Detection in Base-band

The received signal y(t) is first passed through a band-pass filter to extract the WUS frequencies. Subsequently, y(t) is converted into base-band using a sampling frequency fS,WUS resulting in a sampling rate reduction of r=fS,OFDM/fS,WUS. Therefore, with K denoting the FFT size of the OFDM transmission, we obtain N=K/r WUS samples per OFDM symbol in time-domain at the receiver base-band. Denote yl∈CN the discrete received signal in base-band of OFDM symbol l. With M the number of OOK symbols per OFDM symbol we have yl=[yl,1, yl,2…yl,M] with yl,i∈CNM the NM=N/M samples per OOK symbol. The energy ei,l of OOK symbol i in OFDM symbol l is then given by

ei,l=n=1NMyl,i,n2=yl,iHyl,i

The energy is then compared to a threshold or if Manchester coding is used, the energy values are compared, e.g. for Manchester code R=1/2

bj,l=0,  &if ei,l<ei+1,l1,  &if ei,l≥ei+1,l

Where bj,l is bit j encoded in 2 consecutive OOK symbols i and i+1 of OFDM symbol l.

### Correlation detection in Base-band in Time-Domain

If the ON-sequence a is known the receiver can carry out correlations with all possible transmitted sequences sm=[sm1sm2…smL]∈CNL. After OFDM demodulation, the WUS signal (sub-carriers) is transformed back into time-domain via Inverse-DFT precoding. We assume that channel estimates are not available, hence a we use a RAKE demodulator for square-law combination of orthogonal signals [Proakis, Figure 14-5-7].

#### Correlation per OOK Symbol

If the correlation is carried out per OOK symbol (referred to as COR-WUR-OOK), the resulting time-domain signal yl,i of the ith OOK symbol is correlated with a and the NP peaks of the correlator output are combined to exploit frequency-diversity in fading channels i.e.

ei,l=n=1NPmaxNPyl,i⋆a[n]2

where maxNPfx2 returns the NP largest absolute values squared of f(x) and the linear cross-correlation function f⋆g[n]is defined as

f⋆gn=m=-NNf*[m]g[m+n]

#### Correlation per WUS

If the correlation is carried out over the entire WUS to achieve the largest processing gain (referred to as COR-WUR), the message  m, m=0,1,…,2B-1 , for payload of B bits WUS is decoded as

m=argmaxmn=1NPmaxNPy⋆sm[n]2

where y is the time-domain WUS and sm is the known WUS for message m.

## Simulation Assumptions

Unless otherwise stated, the link-level simulation assumption in Table 6 are used.

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 2.6 GHz (FDD) |
| Waveform | OOK-4 |
| Channel Structure | Option 3: Payload only (no CRC) |
| SCS | 30 kHz |
| WUS payload | 8 bits |
| Configuration of LP-WUS Signal | OOK-4: M=4 ON-Sequence = Zadoff-Chu Multiple Sequences are obtained via different cyclic shifts |
| WUS Duration | 4 OFDM symbols |
| Code Scheme | Manchester Code R=1/2 with transmission of encoded bits |
| Channel BW | 20MHz (51 PRBs @ 30kHz SCS) |
| LP-WUS BW | 5 MHz (14 PRBs = 168 SCs) 148 SCs for WUS (4.44 MHz) + 10 SCs GB on each side |
| Filter | 3rd order Butterworth with 4.32 MHz BW |
| Adjacent Sub-carrier Interference (ACI) | Random 64-QAM symbols with 0dB |
| WUS Sampling Rate | 7.68 MHz |
| ADC bit-width | inf |
| Channel Model | TDL-C, 300ns Delay Spread |
| Timing Error | 0 |
| Frequency Error | 0 |
| Antenna configuration | 1Tx, 1Rx |
| UE speed | 0 km/h |
| Receiver | Energy Detector, Correlation Detector (Energy of 5 highest peaks is combined) |

Table 6: Link-level simulation assumptions.