3GPP TSG RAN WG1 #120		R1-2500771

Athens, Greece, February 17th – 21st, 2025

Source: 	Apple Inc.

Title:                     Remaining issues for UE-initiated beam management

Agenda item:	9.2.1

Document for:	Discussion and Decision

# 1 Introduction

One of the objectives in the approved work item on NR MIMO Phase 5 is shown below [1]:

RAN1:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management

  - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching

  - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

In RAN1 119 meeting [2], good progross was made on a few controversial aspects related to UE-initiated beam management. In this contribution, we discuss remaining details to enable the UE-initiated beam management feature in Release 19 and fulfill the relevant objective above and design goals.

# 2. Discussions

## 2.1 Measurment resources

In RAN1 118, the following was agreed for the RS for new beams [3].

Agreement

Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported

  - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration

  - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set.

  - FFS: The RS in the RS resource set can be updated by MAC-CE

In the unified TCI framework, the NW uses RRC signaling to configure a set of TCI-states. A MAC-CE is then used to select and active one or multiple TCI-state out of these RRC-configured TCI-states based on the recent L1-RSRP reports to handle UE mobility. Event-2 is defined to trigger a beam report when the L1-RSRP of at least one candidate beam is offset better than the current beam. Since the current beam can be updated by MAC-CE or even DCI format, using the RRC signal to reconfigure RS resoures for candidate beam is too slow to handle UE mobility. Therefore, it is imporant to support MAC-CE based updates for the candidate beams to ensure RS resources promptly when current beam changes due to UE mobility, thereby maintaining the efficiency of UEIBR report.

Therefore, we propose:

Proposal 1:

- The RS in the RS resource set for candidate beam in Event-2 and Event-7 can be updated by MAC-CE.

2.2 Event evaluation for UEIBR procedure

The following was agreed in RAN1 117 meeting:

Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.

During the RAN1 118bis meeting, the necessity of resetting a event evaluation procedure was briefly discussed without reaching a concensus. Triggering Event-2 relies on L1-RSRPs measured on two RS sets: one for the current beam and the other for the new beam(s). It is nature to reset the counter and re-evaluate if any of these measurment resources are reconfigured by RRC or updated by DCI, ensuring accurate event triggering decision based on the newly-configured resouces. Therefore, the evaluation counter should be reset to start a new procedure.

Figure 1: Counter resetting for Event evaluation procedure

Proposal 2: The event evaluation counter is reset if any of the following conditions is met:

- Condition #1: A configuration for a corresponding measurement RS list for new beam is provided by higher layers.

- Condition #2: The indicated TCI-state is changed.

- Condition #3: The threshold value is changed.

Another FFS aspect related to event evaluation procedure involves determining the timing for event evaluation after measurement resources for the current beam and new beams are configured by NW. There are two alternatives:

- Alt.1: Mandating NW to configure a same periodicity of measurement resources for current beam and new beams.

- Alt.2: No restriction on the periodicity of measurement resources for the current beam and new beams.

Alt.1 simplifies standardization and testing efforts, as there is no need to handle cases where periodicity of measurement resources for current beam and new beam(s) are different. However, it imposes restriction on the resources used for current beam and new beam measurement in the UEIBR procedure. Note that, to minimize RS overhead, CSI-RS resources may be shared for multiple functionalities, rather than dedicated resources for a single function. The requirement posed by Alt.1 increases RS overhead and lowers spectrum efficiency, which is not preferrable. Therefore, we propose adopting Alt.2, which provides better scheduler flexibility in terms of measurement resource configuation for the UEIBR procedure.

When different periodicities are configured for the measurement resources of current beam and new beams, we prefer to use the minimum value between them. This approach ensures timely beam reporting once the event criteria is met based on L1-RSRP results measured on RS with a shorter period. Simliar to RLM measurement defined in Rel-15, event evalaution instances should balance beam report latency and UE power consumption.

- When DRX is NOT configured, a minimum of X=10 ms is hard-encoded in specification to limit the RLM measurement rate.

- When DRX is configured, a scaling factor determines a measurement period for RLM based on the DRX cycle length.

We proposed to use the same mechanisms for determining event evaluation instances.

Proposal 3:

- Denoting Tcurr,Tnew,TEEI as periodicities of RS for current beam, new beam(s) and event evaluation Instance

- When DRX is not configured, TEEI=max⁡(X,minTcurr,Tnew) where X=10ms.

- When DRX is configured,

  - If TDRX≤320ms, TEEI=S*Max(minTcurr,Tnew,TDRX), where S=1.5.

  - If TDRX>320ms, TEEI=TDRX.

## 2.2 UEIBR Content

In the RAN1 118 meeting, the following was agreed for report content of UEIBR procedure [2]:

Agreement

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the following differential L1-RSRP report format is supported.

CRI or SSBRI #1

CRI or SSBRI #2

…

CRI or SSBRI #N

L1-RSRP #1

Differential L1-RSRP #2

…

Differential L1-RSRP #N

Differential L1-RSRP for current beam, if report mode that current beam is always reported is enabled by RRC

Note: Other contents are not precluded

.

    - Differential L1-RSRP #2~#N/current beam is determined based on the difference between measured L1-RSRP corresponding to the CRI/SSBRI #2~#N/current beam and the measured L1-RSRP corresponding to CRI/SSBRI #1

      - L1-RSRP#1 is the largest measured RSRP among reported ones, and an absolute L1-RSRP.

      - FFS: range and step size of differential L1-RSRP

    - FFS: Whether/how to report additional indication of which CRI/SSBRI(s) satisfy the condition of Event-2.

    - FFS: Additional report content(s) (e.g., reporting configuration ID, indication for synchronization state, event ID, or cell ID).

One issue remain FFS aspect is Whether/how to report additional indication of which CRI/SSBRI(s) satisfy the condition of Event-2. In RAN1 117 meeting, the following was agreed to deal with the L1-RSRP variation and mitigate the ping-pong effect of UEIBR procedure:

Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

Based on earlier agreement, once event-2 is triggered and at least one beam satisfies the triggering condition, ‘N’ beams are always reported in a instance and. However, the current L1-RSRP report format does not indicate which of the reported beams satisfies the triggering condition.

It was agreed that the L1-RSRP report for current beam can be enabled/disabled by RRC signal. Accordingly, two modes need to be consdiered on the need of addition report content:

- Mode #1: reporting for current beam is not included based on RRC signal.

- Mode #2: reporting for current beam is included based on RRC signal.

For Mode#1, the additional content is needed to indicate whether the reported beam meets the event triggering condition. Without this information, the NW can not properly utilize the UE-initiated beam reporting to update the indicated TCI-state, rendering the mode without current beam reporting unworkable. The arguemnt against this indication is that NW must enable L1-RSRP for current beam in the report in order to address this problem. Clearly, this restriction is undirable as it significantly limits the use case of Mode #1. Additionally, we believe the problem persists even if the L1-RSRP of the current beam is reported (i.e., Mode-2), as elabrated below.

In Mode #2, the RRC signal configures the UE to include the L1-RSRP for current beam. Given that a fixed number of reports in an occasin is agreed, some reported L1-RSRPs in a instance may not meet the event criteria. Specifically, the reported L1-RSRP of a new beam maybe higher than that of current beam, which is not fulfil the criteria. Without this indication field, the NW can not determine which of these candidate beams with higher L1-RSRP values meets the event triggering criteria and thus is a quanfitied beam for TCI-state update.

Figure 2: Measurement results variation for different beams in a evaluation window

Figure 2 illustrates this issue in Mode 2 with the following assumptions:

- The event-2 evaluation window length is set to 4.

- The condition of event-2 triggering is set as ‘M=3’

- NW sets ‘N=2’ i.e., two candidate beams in a report.

Within the ‘length-4’ event-2 evaluation window, the numbers of instances that fulfil the conditons varies for beam 1 and beam 2. Event-2 is triggered because beam 1 has 4 valid instances, which exceeds the threshold ‘M=3’. However, beam 2 has only ‘2’ valid instance, which does not meet the event-2 triggering criteria. If the exsiting beam report format is fully reused for UEIBR report (i.e., only L1-RSRP), NW cannot know which of these two reported beams and L1-RSRPs mets the event-triggered criteria and therefore can not be utilized for subsequent TCI-state update. For example, assuming beam 1 is overloaded, NW may mistakenly assume that Beam 2 with the 2nd higher L1-RSRP still meets the criteria, leading to update of the TCI-state to Beam 2. Consequently, TCI-state update may not accurately reflect the beam quality, undermining the objective of ‘latency reduction’ for beam management.

It is important to note that, for a given serving cell, a total number of RRC-configured candidate beams for L1-RSRP measurement is quite limited and depends on the UE capability report. Assuming the number of measurement beams is approximately 4 to 8, the reported beams and measurement beams are almost the same. With this assumption, this problem cannot be addressed by UE implementation since the UE cannot arbitrarily report L1-RSRP. Instead, these values should come from the actual measurement on the configured DL beams.

Therefore, we have the following observation and propose:

Observation 1: Fully reuse the existing beam report content causes the mismatch between NW and UE on the beam(s) that triggers event-2 report and results in inapproprate TCI-state update.

The following was agreed during RAN1 119 meeting [2] to enhance the beam report, aiming to indicat the qualitified beam indexes in the report:

Agreement On UE-initiated/event-driven beam reporting, at least one of the following is supported as an extension on L1-RSRP report format depending on Event-2. - Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2. - The presence of this field is enabled by RRC with subjective to UE capability. - The presence of this field is only for the case that N > 1 and the time window and M are configured - Option-2: For each of reported CRI/SSBRI, to introduce additional indication of the number of Event-2 instances for the CRI/SSBRI(s) within a time window. - The presence of this field is enabled by RRC with subjective to UE capability. - The presence of this field is only for the case that N > 1 and the time window and M are configured. - Option-3: No further enhancement.

We believe one of option 1 and 2 should be selected to address the ‘beam reporting’ mismatch problem above, and Opt.3 should no longer be discussed anymore. Opt.1 and Opt.2 differ in how they handle information for a reported beam that does not meet the event criteria. Specifically, Opt.1 provides binary information with the minimized overhead, while Opt.2 intends to report the number of qualified L1-RSRP within a time window, even if this beam does not meet the event criteria. It is obvious that Opt.2 leads to an increased CSI report size, which is crucial for UL tranmission due to limited transmission power at UE. Moreover, the performance benefit of Opt.2 over Opt.1 is unclear since the number of ‘qualified’ instances in a time window does not offer more useful information for scheduler as long as the beam does not meet the event criteria.

We therefore recommend choosing Opt.1, as it balances the UL report overhead and the beam mangement reliability and efficiency:

Proposal 4: On L1-RSRP report for Event-2, support Option-1, i.e., for each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2

Another open issue is determining whether any additional report content is needed for Event-7 beyond what was agreed for Event-2. The purpose of Event-7 is to faciliate the update of the activated TCI-State by using the reported candidate beams that are threshold better than the activated TCI-state with the M-th best L1-RSRP value. To achieve the design goal, the index of activated TCI-State with the M-th best L1-RSRP needs to be included in the report. Additionally, the NW needs to identify which of the activated TCI-States should be updated by the reported candiate TCI-States. For this purpose, a bitmap indicator is required to indicate which of these activated TCI-States has lower L1-RSRPs comparerd to the ‘activated TCI-State with the M-th best L1-RSRP’.

Proposal 5: On Event-7 report, consider the following additional report content

- The TCI-State ID of the ‘activated TCI-State with the ‘M-th’ best quality’.

- A bitmap to indicate which activated TCI-States can be updated by the qualified L1-RSRP reports

## 2.3 UL Resource for UEIBR Transmission

### 2.3.2 Retransmission of first PUCCH channel

It remains open regarding the necessity of a timer to control the first PUCCH retransmission after Event-2 is triggered in Mode-A. The intended function of the first PUCCH channel in Mode-A is similar as SR transmission for BSR reports. Typically, the periodicity of the first PUCCH channel in Mode-A or SR in legacy system is short to ensure a fast reporting once the corresponding event is triggered. In BSR report, to control the amount of SR re-transmission and to avoid flooding the uplink with an excessive number of SR transmission, a prohabit timer is specified. With this timer, the SR can not be transmitted more often than once per timer interval as determined by timer for BSR report.

For Mode-A, there are two use cases to be considered for the first PUCCH channel transmission:

- Scenario 1: To control UL overhead and avoid any first PUCCH re-transmission during a timer interval.

- Scenario 2: To ensure first PUCCH channel high reliabilty by allowing a first PUCCH channel re-transmission until reception of UL grant.

Figure 2: Prohabit timer for first PUCCH transmission in Mode-A

(Scenario 1: Top, Scenario 2: Bottom)

In scenario 1, the function mirrors the SR prohabit timer defined for BSR use case. Therefore, it naturally starts the prohabit timer once the first PUCCH channel is transmitted. For scenario 2, however, the timer should  only start after receiving the corresponding UL grant. Essentially, the use the prohabit timer balances the resource overhead and the reliability of the first PUCCH channel, which depends on the UE link quality. A decision best left to NW scheduler. For example, it is more practical to apply scenario 1 for UEs located in the cell center or those with good link quality, while scenario #2 should be considered for cell-edge UE to ensure the reliabilty of the first channel.

Proposal 6: Introduce a prohabit timer to control the first PUCCH retransmission for Mode-A.

- Configuration #1: The timer starts after the first PUCCH channel is transmitted.

- Configuration #2: The timer starts after receiving the second PUSCH channel.

- It is up to NW to select one configuation for Event-2.

- A UE shall not transmit the first PUCCH channel when the prohabit timer is running.

### 2.3.2 UCI multiplexing for ‘New UCI Type’

Another important aspect is the UL resource allocation for UEIBR content transmission. The following was agreed in RAN1 119 meeting for the first channel transmisson [2]:

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, Alt-2 is supported for both mode-A and mode-B: the first PUCCH channel is a new UCI type - Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration. - It is RAN1’s understanding that the RRC parameter is NOT associated with SchedulingRequestId. - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. - Reuse multiplexing/dropping rule(s) of SR as baseline - FFS: overlapping with SR/LRR or PUSCH Note: Further details on first PUCCH retransmission (if supported) for mode A and mode B will be continue to be separately discussed in RAN1.

It remains open regarding how to handle the new UCI type carried in the first PUCCH channel of UE-Initiated Beam reporting procedure when it is overlapped with SR/LRR or PUSCH.

- Case 1: The first PUCCH channel is overlapped with PUSCH.

The new 1-bit UCI can be multiplexed on the PUSCH. Specifically, to avoid hypothetical detection at the NW, a 1-bit UCI is always transmitted, regardless of the corresponding event is triggered. In addition, to ensure the reliability of new UCI detection, the βoffsetHARQ-ACK is used to determine the RE numbers for new UCI transmission.

Proposal 7: The new UCI type is piggyback on PUSCH when it is overlapped with PUSCH.

- Case 2: The first PUCCH channel is overlapped with SR/LRR transmission on PUCCH.

When the PUCCH channel can not transmit all UCI information, the multiplexing needs to prioritize among the overlapped UCIs based predefined rules. The priority rule should be defined based on the significant impacts of each UCI type. Dropping Link recovery request (LRR) leads to beam failure on the SCell, which can severly compromise the UE throughput due to SCell beam failure. This type of failure disrupts the communication link on SCell entirely, resulting in a complete loss of serving for the affected beam. On ther other hand, while dropping the new UCI type may result in degraded serving beam throughput, the connection remains intact, allowing for continued, albeit reduced, data transmission. Given the critical nature of maintaining a stable and reliable SCell, it is evident that prioritizing link recovery requests is paramount to ensure continuous connectivity and prevent the more severe consequence of beam failure on the SCell. Additionally, dropping normal SR only has impacts on the latency performance of uplink transmission and no impact on link quality.

We therefore propose the following priority order for Case 2:

Proposal 7: When a new UCI type on first PUCCH channel overlaps with either LRR or normal SR, follow the prioritization order for UCI dropping: LRR > new UCI type > normal SR, where LRR has the highest priority.

2.3.3 UE initiated beam reporting for Multi-Events and Multi-CCs Case

For both Mode A and Mode B, RAN1 agreed to support a 1-bit new UCI type on the first PUCCH format 0/1, at least for single CC case [2]. The payload size of the new UCI type and the first PUCCH format, when multiple events are configured on a single CC or over multiple CCs in CA scenario, remains open. Various options are as follows:

- Alt.1: Reuse the single CC scheme, i.e., a 1-bit new UCI is transmitted on the first PUCCH format 0/1 resource if at least one of the associated events is triggered.

  - A single second PUSCH is used.

- Alt.2: Support multi-bits new UCI on PUCCH format 2/3/4, where each bit is associated with a pair of <Event, serving cell>.

  - Mode A: A single second PUSCH is used to carry all triggered CSI reports.

  - Mode B: A set of CG-PUSCH resources may first be configured by RRC. The UE selects the CG-PUSCH resource with the smallest number of REs that can still carry all triggered CSI reports.

Alt.1 is simper and offers a unified 1-bit new UCI type solution for both single-event/single-CC and multiple-event/multiple-CCs use cases. It minimizes the efforts rquired standarddization, implementation and testing. However, a drawback of Alt.1 is that it increases CSI report overhead, as the NW is unware of the exact number of triggered event. Consequently, it is likely that the second PUSCH is deminsioned based on the worst case (i.e., all associated events are triggered) to avoid dropping the measured CSI report. Alt.2 is mainly motivated to address the PUSCH overhead issue by using PUCCH format 2/3/4 in the first step to transmit multiple bits. This can be leverage by NW for 2nd PUSCH resource. Compared to Alt.1, the resource efficiency is improved in Mode A, as the UL resource is allocated based on the triggered events indicated by the first PUCCH format. In case of Mode B, the NW can re-allocate unused CG-PUSCH to other UEs, also minimizing resource waste.

The UCI multiplexing/dropping rule defined in 5G NR is quite complex, involving various UCI types and dynamic PUCCH resource selection. Therefore, a single solution should be pursed for the first PUCCH channel to ensure timely comipletion of this WI. Consequently, we prefer to adopt Alt.1 for UE-Initiated beam reporting for multi-event/multi-CC use case, same as single CC case. In many practical instances, these events are not triggered simutaneously, which can help to mitigate the resource efficiency issue. A number of reported events in a 2nd PUSCH occasion (e.g., N≥1) can be configured using RRC signal based on the gNB’s estimation of concurent events. Additionally, a simple priority rule can be defined for the CSI reports of events to select ‘N’ reports when more events are triggered for a given report instance.

Hence, we propose the following:

Proposal 8: Reuse the first channel design of single CC for multi-events/multi-CCs use case, i.e., 1 bit UCI on PUCCH format 0/1.

Proposal 9: For multi-events/multi-CCs use case, total number of CSI reports in a 2nd PUSCH is configurable by RRC signal.

Proposal 10: Introduce a priority rule for CSI reports selection when the triggered event is larger than the  total number of reports in a single 2nd PUSCH occasion configured by RRC.

# 3. Conclusion

In this contribution, we have presented our views on how to support UE-initiated/Event-Triggered beam reporting and propose the following:

Proposal 1:

- The RS in the RS resource set for candidate beam in Event-2 and Event-7 can be updated by MAC-CE.

Proposal 2: The event evaluation counter is reset if any of the following conditions is met:

- Condition #1: A configuration for a corresponding measurement RS list for new beam is provided by higher layers.

- Condition #2: The indicated TCI-state is changed.

- Condition #3: The threshold value is changed.

Proposal 3:

- Denoting Tcurr,Tnew,TEEI as periodicities of RS for current beam, new beam(s) and event evaluation Instance

- When DRX is not configured, TEEI=max⁡(X,minTcurr,Tnew) where X=10ms.

- When DRX is configured,

  - If TDRX≤320ms, TEEI=S*Max(minTcurr,Tnew,TDRX), where S=1.5.

  - If TDRX>320ms, TEEI=TDRX.

Proposal 4: On L1-RSRP report for Event-2, support Option-1, i.e., for each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2

Proposal 5: On Event-7 report, consider the following additional report content

- The TCI-State ID of the ‘activated TCI-State with the ‘M-th’ best quality’.

- A bitmap to indicate which activated TCI-States can be updated by the qualified L1-RSRP reports

Proposal 6: Introduce a prohabit timer to control the first PUCCH retransmission for Mode-A.

- Configuration #1: The timer starts after the first PUCCH channel is transmitted.

- Configuration #2: The timer starts after receiving the second PUSCH channel.

- It is up to NW to select one configuation for Event-2.

- A UE shall not transmit the first PUCCH channel when the prohabit timer is running.

Proposal 7: When a new UCI type on first PUCCH channel overlaps with either LRR or normal SR, follow the prioritization order for UCI dropping: LRR > new UCI type > normal SR, where LRR has the highest priority.

Proposal 8: Reuse the first channel design of single CC for multi-events/multi-CCs use case, i.e., 1 bit UCI on PUCCH format 0/1.

Proposal 9: For multi-events/multi-CCs use case, total number of CSI reports in a 2nd PUSCH is configurable by RRC signal.

Proposal 10: Introduce a priority rule for CSI reports selection when the triggered event is larger than the  total number of reports in a single 2nd PUSCH occasion configured by RRC.

# References

RP-234007, New WID: NR MIMO Phase 5, Samsung (Moderator), RAN#102, December 2023

RAN1 119 meeting Chairman notes

RAN1 118 meeting Chairman notes