3GPP TSG RAN WG1 #120 	R1- 2501003

Athens, Greece, February 17 – February 21, 2025

Agenda item:		9.3.1

Source:				Nokia, Nokia Shanghai Bell

Title:					On SBFD TX/RX/measurement procedures

Document for:		Discussion and Decision

# Introduction

A revised version of the Work Item (WI) on Evolution of NR duplex operation: Subband full duplex (SBFD) was approved in RAN#104 [RP-241614]. This contribution presents our views regarding SBFD signaling and UE Transmission/Reception behaviors taking into account the latest guidance from the RAN1 Chairman in R1-2408102 regarding RRC parameter list collection:

- Rel-19 higher layer signaling - Target: Provide initial input to RAN2 (and RAN3 if needed) after RAN1#120 (Feb, 2025) - RRC parameter list: Sorour (Ericsson) will coordinate for all Rel-19 WIs - Other higher layer signaling (including MAC CE): To be separately handled for each Rel-19 WI

# SBFD configuration in time and frequency domain

This section focuses on the configuration of the SBFD subbands in time and frequency domain as per the below objectives:

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in /RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded

## Cell-common signalling or UE specific signalling

The following agreement was reached in RAN1#116 regarding SBFD signaling:

Agreement For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier. - FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands

The following was furthermore agreed on SBFD random access operation in RAN1 agenda item 9.3.2 during RAN1#117 meeting:

Agreement Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode.

Motivated by the above agreements, RAN2 has agreed in RAN2#127 [R2-2407572] to support cell-specific SBFD time/frequency configuration provided in SIB1:

Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

Thus the working assumption in RAN1 #116 agreement for cell specific configuration of frequency location of SBFD subband should be confirmed.

The working assumption in RAN1 #116 agreement for cell specific configuration of frequency location of SBFD subband should be confirmed.

Based on these agreements, in our view, the only issue open from RAN1 perspective is the FFS above on whether UE-specific configuration should be supported or not. We are open to having the additional possibility of providing UE-specific configuration on time and/or frequency locations of SBFD subbands. This can be used to allow some UEs to operate with larger guardband than the guardband from gNB e.g. due to poorer filtering capabilities compared to other UEs, or different UE-to-UE CLI conditions. Note that the UE-specific signalling should be still subject to some restrictions, e.g. RBs indicated as guardband in the cell-specific configuration should not be re-configured for DL or UL direction, while the opposite operation should be possible. Besides, additional RRC configuration might be needed, while the RRC configuration can also be used for example for supporting other scenarios such as dual connectivity, EN-DC or NR-DC, or carrier aggregation.

In RAN1 #117 meeting, there is discussion on sending LS to RAN4 on the UE-specific guard band related issue. To make progress, we think RAN1 should finalize the LS and send to RAN4 in RAN1 #118 meeting.

In addition to cell-specific configuration of DL and UL subbands, signaling of UE-specific configuration can be supported, e.g. to configure some UEs to operate with larger guardband due to poorer filtering capabilities or different UE-UE CLI conditions compared guardband of gNB.

Signaling of the SBFD time and frequency configuration via RRC dedicated signaling can be considered, additionally to signaling in SIB, to support dual connectivity or carrier aggregation with SBFD carriers.

RAN1 should finalize the LS on UE-specific guardband and send to RAN4.

## RRC Parameters for time and frequency domain configuration

The following agreements have been reached regarding SBFD time-domain indication:

RAN1#116: Agreement: For RRC connected mode UEs, SBFD subband time locations are configured within a period. At least when only one TDD-UL-DL pattern is configured, the period is down-selected from one of the following options. - Option 1: The period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. - Option 2: The period is integer multiple of TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. - FFS: Further details FFS: Details when two TDD-UL-DL patterns are configured Agreement A slot can consist of SBFD symbols and non-SBFD symbols. Agreement A slot can consist of SBFD symbols and non-SBFD symbols. For semi-static indication of SBFD subband time location, - When only one TDD-UL-DL pattern is configured, SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for only one of the patterns, SBFD symbols are configured in consecutive manner within the TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for both patterns, SBFD symbols are configured in consecutive manner within each TDD-UL-DL pattern period. - SBFD symbols are configured in DL and/or flexible symbols configured in TDD-UL-DL-ConfigCommon - The configured SBFD symbols can start from any symbol within a slot and can end in any symbol within a slot. - referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as reference SCS. - FFS details RAN1#116bis: Agreement A symbol configured as SBFD symbol via cell-specific configuration cannot be reverted to a non-SBFD symbol via any UE-specific configuration or group-common signaling. A symbol not configured as SBFD symbol via cell-specific configuration cannot be reverted to an SBFD symbol via any UE-specific configuration or group-common signaling. RAN1#117: Agreement For RRC connected mode UEs, SBFD subband time period: - When only one TDD-UL-DL pattern is configured, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. - When two TDD-UL-DL patterns are configured, the period is the same as the sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. RAN1#118: Agreement For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are supported - A starting slot index - A starting symbol index within the starting slot - An ending slot index - An ending symbol index within the ending slot

Based on the above agreements, our preference is to introduce a new IE SbfdSymbols inside existing TDD-UL-DL-Pattern IE which contain new parameters that provide the start and end of SBFD symbols within the corresponding TDD-UL-DL-Pattern. SBFD end slot and end symbol could be regarded as optional in which case the SBFD symbols last until the end of the TDD pattern or until the start of a UL-only symbol.

-- ASN1START

-- TAG-TDD-UL-DL-CONFIGCOMMON-START

TDD-UL-DL-ConfigCommon ::=          SEQUENCE {

referenceSubcarrierSpacing          SubcarrierSpacing,

pattern1                            TDD-UL-DL-Pattern,

pattern2                            TDD-UL-DL-Pattern                                                       OPTIONAL, -- Need R

...

}

TDD-UL-DL-Pattern ::=               SEQUENCE {

dl-UL-TransmissionPeriodicity       ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10},

nrofDownlinkSlots                   INTEGER (0..maxNrofSlots),

nrofDownlinkSymbols                 INTEGER (0..maxNrofSymbols-1),

nrofUplinkSlots                     INTEGER (0..maxNrofSlots),

nrofUplinkSymbols                   INTEGER (0..maxNrofSymbols-1),

...,

sbfdSymbols                         SbfdSymbols         OPTIONAL

}

SbfdSymbols::=                      SEQUENCE {

sbfdStartSlot                       INTEGER (0..maxNrofSlots),

sbfdStartSymbol                     INTEGER (0.. maxNrofSymbols-1),

sbfdEndSlot							INTEGER (0..maxNrofSlots)     OPTIONAL,

sbfdEndSymbol						INTEGER (0.. maxNrofSymbols-1)    OPTIONAL,

}

-- TAG-TDD-UL-DL-CONFIGCOMMON-STOP

-- ASN1STOP

For the frequency-domain configuration, RAN1 has agreed on the following:

Agreement Confirm the following working assumption made in RAN1#118. Working Assumption For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. - One or two SBFD DL subbands can be configured

In our view, this can be captured in RRC specifications by defining a new SbfdSubbandLocation IE containing the RIV-based location of the subband and the associated subcarrier spacing. Then, one or up to two SbfdSubbandLocation (for UL and DL, respectively) are provided for each SCS in DL-specific and UL-specific IEs, e.g. FrequencyInfoUL and FrequencyInfoDL, or alternatively, DownlinkConfigCommon and UplinkConfigCommon, where the former option is shown below. Note that similar content should be also provided in the ‘-SIB’ variant of the DL and UL parameters since RAN2 has agreed that cell-specific SBFD time/frequency configuration is provided by SIB1.

FrequencyInfoUL ::=                 SEQUENCE {

frequencyBandList                   MultiFrequencyBandListNR                                OPTIONAL,   -- Cond FDD-OrSUL

absoluteFrequencyPointA             ARFCN-ValueNR                                           OPTIONAL,   -- Cond FDD-OrSUL

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

additionalSpectrumEmission          AdditionalSpectrumEmission                              OPTIONAL,   -- Need S

p-Max                               P-Max                                                   OPTIONAL,   -- Need S

frequencyShift7p5khz                ENUMERATED {true}                                       OPTIONAL,   -- Cond FDD-TDD-OrSUL-Optional

...

sbfdULsubbandLocationAndBandwidth  SEQUENCE (SIZE (1..maxSCSs) OF SbfdSubbandLocation    OPTIONAL

}

...

FrequencyInfoDL ::=                 SEQUENCE {

absoluteFrequencySSB                ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SpCellAdd

frequencyBandList                   MultiFrequencyBandListNR,

absoluteFrequencyPointA             ARFCN-ValueNR,

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

...

sbfdDLsubbandLocationAndBandwidth1  SEQUENCE (SIZE (1..maxSCSs) OF SbfdSubbandLocation    OPTIONAL,

sbfdDLsubbandLocationAndBandwidth2  SEQUENCE (SIZE (1..maxSCSs) OF SbfdSubbandLocation   OPTIONAL    -- Cond sbfdDLsubbandLocationAndBandwidth1

}

SbfdSubbandLocation::=             SEQUENCE {

subcarrierSpacing                   SubcarrierSpacing,

locationAndBandwidth                INTEGER (0..37949)

}

## Other issues

One open topic is related to the transmission of some periodic wideband signals, e.g. CSI-RS for tracking. These signals have typically a large periodicity, e.g, 160 ms, while the periodicity of SBFD configuration (part of TDD UL-DL pattern) is much smaller, e.g. 2 to 5 ms. To allow the transmission of these periodic wideband signals, the time-domain configuration would need to accommodate DL-only symbols or slots in every TDD UL-DL pattern period, even though the wideband DL transmission happens very seldom. To address this, we think that the existing periodic UL-DL pattern can be complemented with additional cell-specific signaling that can change the configuration for one or more slots, e.g. from SBFD to non-SBFD. One example is illustrated in Figure 1 where the format of some ‘sparse’ slots are modified from SBFD to non-SBFD symbol. This is used by the gNB to e.g. allow the transmission of wideband DL signals like CSI-RS for tracking, or reduce SSB (e.g. for cell edge coverage) interference caused by SBFD.

The TDD UL-DL pattern (provided via TDD-UL-DL-ConfigCommon) has typically a short periodicity of 1..5 ms, while some reference signals (e.g. SSB or CSI-RS for tracking) are transmitted with much larger periodicity (e.g. 20 .. 160 ms). The SBFD time-domain configuration shall allow SBFD operation in the majority of the time slots, while still allow some ‘sparse’ slots to operate as legacy/DL-only manner.

The periodic UL-DL pattern configuration can be complemented with additional cell-specific signaling that can change the configuration or format for one or some slots.

Figure 1: Cell-specific signaling to change the format of some slots or symbols

# Collision handling and link direction indication

## Link direction indication

UE link direction determination in SBFD symbols was discussed in RAN1#116 as part of the UE Transmission and reception behaviours topic, where the following was agreed:

Agreement For SBFD-aware UE transmission and reception in an SBFD symbol, consider the following options to determine link direction, i.e. whether to transmit or to receive in the SBFD symbol. - Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any). - Option 2: link direction is indicated by gNB explicitly. Other options are not precluded.

The main challenge here comes from the fact that the UE is half-duplex while the gNB is full duplex, therefore some companies raised potential link direction ambiguities or need of extra means for the UE to know whether to prioritize DL reception or UL transmission in the SBFD symbols.

Option 1 in the agreement above entails the use of collision handling rules. These are based on existing rules for UL/DL collision handling on flexible symbols as specified in TS 38.213, Clause 11.1 with potentially some SBFD-specific optimizations which are explained later in this section. Option 2, in our understanding, entails the use of additional or new signalling to explicitly indicate the UE whether the UE shall regard some SBFD symbols as uplink or downlink, which can be e.g. used to ignore transmissions/receptions in the opposite link direction.

In our view, to minimize specification effort, we think existing rules for UL/DL collision handling on flexible symbols should be the baseline for determining the link direction on SBFD symbols. In other words, as baseline, we should target to rely as little as possible on additional signalling or avoid having a separate/alternative mechanism to perform UL/DL link direction determination as this would complicate the scheduling of legacy and SBFD-aware UEs in the cell (since legacy UEs would not support new rules specified in Option 2). Note furthermore that Option 2 may i) limit the UL/DL scheduling flexibility or peak UL/DL throughput for the UE, e.g. in case the additional signalling is provided by RRC, or ii) prone to errors and additional overhead, in case PHY or MAC signalling is used. Last but not least, if the link direction is simply to tell the UE to always consider an SBFD slot as either UL or DL, then the benefit of link direction indication is unclear and gNB should not configure the slot as SBFD slot in the first place. In contrast, if the link direction is to tell the UE link direction only in case of UL/DL collision, then using only Option 1 should be sufficient for that purpose. In other words, we disagree with Option 2 in the sense that it assigns a pre-defined link direction (e.g. per slot) to the UE, while we support Option 1 with potentially additional indications that allows the UE to determine UL/DL priorities; these priorities might be taken into account for specific UL/DL collision cases.

To further exemplify the problem, in Figure 2 we compare Option 2 on link direction indication (Figure 2A) with an alternative approach of providing additional information to aid the collision handling procedure (Figure 2B). In Figure 2A, the link direction information may be used to cancel or invalidate semi-static signals configured on the opposite link direction; however, the fact that the link direction is fixed, this approach would also prevent e.g. receiving a PDCCH that schedules a dynamic DL transmission in a slot configured as ‘U’ by the link direction indication, even though there are no UL transmissions configured or scheduled in the slot. In contrast, in Figure 2B, the link direction is used to indicate priority between DL and UL in case of conflicting allocations, while a slot may still be used for transmissions/receptions in the opposite link direction if there are no planned transmissions/receptions for the configured link direction.

Figure 2: Comparison of link direction indication (i.e., Option 2, left) with an alternative approach of providing additional information to aid the collision handling procedure (Figure 2, right).

For link direction determination, Option 2 (link direction is indicated by gNB explicitly) assigns a pre-defined link direction (e.g. per slot) to the UE which diminishes the benefits of SBFD.

In contrast, Option 1 (based on collision handling rules) is more flexible and better aligned with legacy UE handling. Option 1 could benefit of additional indications that allows the UE to determine UL/DL priorities e.g. for some (but not all) UL/DL collision cases.

Based on the above, our preference is to support Option 1 as baseline. Note that this does not preclude that existing collision handling rules should not be modified for SBFD operation.

For the link direction determination, support Option 1 from RAN1#116 as baseline:

  - Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any).

- Note: Option 1 should not preclude enhancements to existing collision handling rules for SBFD operation.

If option 2 should be further discussed based on Option 1, option 2 should not fix the link direction in advance resulting diminishing the benefit of SBFD. For instance, existing collision handling rules may be supplemented with additional indications to determine UL/DL priorities for upcoming slot(s) (potentially limited to specific UL/DL collision cases) but without fixing the link direction in advance.

## Additional signaling for UL/DL collision handling:

Now, in the following, we discuss what types of additional signalling may be beneficial for the purposes of UL/DL collision handling. Current agreements regarding collision handling rules are listed below:

RAN1#116bis: Agreement If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UE does not receive DL channel/signal. - The above does not imply link direction indication is supported - FFS on dynamically scheduled UL transmission with repetition Agreement If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 1 (dynamically scheduled DL reception vs. semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles and timeline in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UL transmission is cancelled if cancellation timeline is met. - The above does not imply link direction indication is supported - FFS on dynamically scheduled DL reception with repetition RAN1#117: Agreement If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 4 (dynamically scheduled DL reception vs. dynamically scheduled UL transmission) in a SBFD symbol for SBFD-aware UEs, it is considered as an error case if a dynamically scheduled DL reception in DL subband(s) without repetitions overlaps with a dynamically scheduled UL transmission in UL subband without repetitions. - FFS DL reception with repetitions and/or UL transmission with repetitions Agreement If link direction indication is not supported nor provided for a SBFD symbol, collision Case 3 (semi-statically configured DL reception vs. semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs is an error case. - An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and dedicated higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol - An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and cell specific higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol - Cell-specifically configured DL reception refers to PDCCH in Type-0/0A/1/2 CSS set - FFS: Additional cases Agreement Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission - FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. Agreement For collision Case 1/2/4 with DL receptions and/or UL transmissions with repetitions, the same collision handling rules and timeline for DL receptions and UL transmissions without repetitions are applied for each repetition. RAN1#118bis: Agreement If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 6 (Dynamic or semi-static DL vs. valid RO), reuse the existing collision handling rules for HD-FDD RedCap UEs. - UE does not expect collision between PRACH triggered by PDCCH order and dynamically scheduled DL reception. - For collision between PRACH triggered by PDCCH order and semi-statically configured DL reception, UE does not receive DL channel/signal. - For collision between PRACH triggered by higher layer and DL channel/signal, leave it to UE implementation whether to receive the DL or transmit PRACH.

As noted above, RAN1 has agreed to apply the existing/legacy behavior for majority of the collision cases conditioned that a ‘link direction indication’ is not provided to the UE. Overall, we would like to note that existing TDD collision handling rules were specified mainly under the assumption that both the gNB and UE are half-duplex in a TDD carrier. This means that, from operation point of view, it does not make sense for a legacy system to configure e.g. both a semi-static/dynamic DL and semi-static/dynamic UL transmission that overlap in time since the gNB can only receive or transmit at a time in a TDD carrier. To fully attain the benefits of SBFD operation, we therefore support the provisioning of a ‘link direction priority indication’ to the UE. The main use case for this indication should be to allow semi-static UL and DL transmissions to overlap in time, while the link direction priority indication is used by the UE to determine whether to prioritize one or another signal at a particular time. E.g. RAN1 should e.g. allow CG-PUSCH to overlap with semi-static DL (e.g. PDCCH), where one or the other could be prioritized. This makes the configuration of semi-static PDCCH, PUCCH, PUSCH and other signals less cumbersome for the gNB, since otherwise the gNB need to meticulously ensure that the overlapping of semi-static UL and DL signals never happens at UE side. This indication could be provided for each slot (dynamically or semi-static), and/or specific to a channel or signal. The following is therefore proposed:

For collision Case 3 (semi-statically configured DL reception vs. semi-statically configured UL transmission), if link direction priority indication is provided to the UE, semi-static UL and DL transmissions can overlap in time and the link direction priority indication is used to prioritize one or another signal.

For link direction priority indication, RAN1 to consider at least the following for SBFD:

  - RRC level-signaling can indicate whether to prioritize semi-static UL or semi-static DL in certain slots in case of overlapping between semi-static UL and semi-static DL signals.

  - gNB indicates dynamically whether the UE should prioritize UL or DL transmissions in upcoming slot(s).

## Collision between configured SSB vs. dynamically scheduled or configured UL transmission (Case 5):

The following two agreements have been reached regarding this collision case:

Agreement (RAN1#117) Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission - FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. Working Assumption (RAN1#118bis) For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband

Two options for handling SSB reception were discussed in RAN1#118bis: In Option 1, the SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs in the SSB symbols. In Option 2, a slot consisting of SSB symbols is considered as a full DL slot meaning that there is no restriction on the location of the SSB in frequency domain.

Option 1 has been agreed as a working assumption in RAN1#118bis. In our views, Option 1 has significant drawbacks which may prevent deployment of SBFD in the field. Those drawbacks described in Table 1 are explained in the following:

- Placement of SSB in frequency domain: This is the most significant drawback of Option 1 as it forces the SSBs to be placed within the DL subband (as noted in the working assumption).

  - For FR2 spectrum, adopting Option 1 would prevent SBFD operation in carriers of 100 MHz considering limited number of PRBs. Moreover, in some cases, SBFD DUD operation in 200 MHz carrier bandwidths may be problematic as well, considering that the raster size for configuring the SSB location is as high as 17.28 MHz and there may be no good choice for SSB location in DL subband. Similar for FR1 with limited bandwidth, e.g. 20/30/40/MHz.

  - Even in FR1, full flexibility for placing the SSB in frequency-domain is desired in order to align the SSB position according to operator-specific preferences.

  - In both FR1 and FR2, for the mode where CORESET0 is aside of SSB in the same symbols, where more DL PRBs will be needed, Option 1 will result in more restriction on the placement of SSB and CORESET0 in DL PRBs.

- Resource waste: In Option 1, the resources in UL subband overlapping with SSB cannot be used for neither UL nor DL. The longer the SSB burst, e.g. SSBs for multiple beams of the cell, the more the waste. In absolute terms, the waste can be significant considering the UL subband bandwidth is typically 20 MHz in FR1 in all the symbols containing SSBs for multiple beams. In contrast, Option 2 allows all the RBs in all the SSBs symbols of the slots to be used for DL transmission if needed.

- Last but not least, in RAN1#118bis, one of the main drawbacks discussed for Option 2 was about the higher complexity and potential resource waste from having transitions between SBFD and non-SBFD operation. From our point of view, this aspect should not be the difference driving the decision between Option 1 and Option 2 considering that:

  - From UE perspective, both option 1 and option 2 require the UE to operate in legacy full ‘DL reception’ mode during the SSB symbols (e.g. one transition from SBFD to non-SBFD (legacy full DL) is needed prior to the SSB transmission occasion in case the UE was scheduled assuming slot/symbol as SBFD type in the previous). Actually, we think what was agreed in the working assumption is not correct in the sense that SSB symbols are already not SBFD symbols but legacy TDD symbols, instead.

  - From gNB perspective, for both option 1 and option 2, it is desirable to transmit the SSB using all the antenna elements of the gNB antenna panel as SSB is for both legacy UE (not supporting SBFD) and SBFD aware UE. This allows the SSB signals to reach the cell-edge and keep the SSB transmission unchanged as compared to legacy TDD operation. Note that transmitting the SSB using all the antenna elements is feasible in Option 1 (and not just in Option 2) since no UL transmissions are expected to take place at the same time as the SSB reception.

Table 1: Comparison of Option 1 vs Option 2 for handling SSB transmissions in SBFD symbols

|  | Option 1 (SSB symbols are SBFD symbols) | Option 2 (slot with SSB is DL-only) |
| --- | --- | --- |
| Placement of SSB in frequency domain | -- SSB restricted to DL subbands, e.g. upper or lower part of band in DUD operation. For some carrier bandwidths in FR1 or FR2, SBFD cannot be feasible considering required resources for SSB/CORESET0. | ++ SSB can be placed anywhere in frequency. This flexibility is crucial to align its position with CORESET0 location, and/or to deal with operator-specific preferences |
| Resource waste | -- Resources in UL subband overlapping with SSB cannot be used for neither UL nor DL. The longer the SSB burst, the more the waste, considering the multiple SSBs with different beam. Still possible to have some UL transmissions in symbols not overlapping with SSB, although it is agreed to not allow UL. | ++ All RBs can be used for DL transmission in the SSB slot (although no ‘SBFD UL improvement’ in the slot SSB slot, which could compromise UL coverage and/or latency). |
| UE/gNB complexity | -- Additional transitions needed between SBFD - and non-SBFD (or SSB symbols) operation | -- Additional transitions needed between SBFD- and non-SBFD operation |

As a compromise, in order to deal with the potential increase of transitions between SBFD and non-SBFD symbols in Option 2, Option 1 or Option 2 could be selected depending on whether the number of transition points within a TDD UL/DL pattern period would get larger than two as also suggested by other companies, e.g. R1-2408642. For instance, assuming DXXXU time-domain configuration, Option 2 could be applied for a SSB burst that starts in the D slot or first X slot, while Option 1 is applied for a SSB burst that starts in the second or third X slot of the TDD pattern.

Based on the above discussion, we make the following observations and proposals:

On collisions between configured SSB and UL transmission, limiting the SSB block/CORESET0 to be within DL subband in Option 1 have significant drawbacks, e.g. it prevents SBFD operation in FR2 carriers of 100 MHz bandwidth, or even 200 MHz considering raster size of 17.28 MHz, or FR1 with limited bandwidth as 20/30/40MHz. Also, it limits the flexibility for placing the SSB according to operator-specific preferences.

On collisions between configured SSB and UL transmission, transitions between SBFD and non-SBFD operation are not only for Option 2, but also apply to Option 1 considering that it is desirable to transmit the SSBs using all the antenna elements of the gNB antenna panel. This allows the SSB signals to reach the cell-edge and keep the SSB transmissions unchanged as compared to legacy TDD operation.

On collisions between configured SSB and UL transmission, RAN1 to revert the working assumption reached RAN1#118bis and adopt Option 2 instead (a slot consisting of SSB symbols is considered as a full DL slot)

RAN1 can also discuss some compromise solution, e.g. on which case Option 1 is selected and on which case Option 2 is selected.

## Collision handling order

The order for handling collision between transmission and receptions was discussed in RAN1#119 but without reaching any agreement.

Proposed Agreement: Collision handling in SBFD symbols follows the following steps: - Step 1: Resolve collision between transmissions/receptions and unusable time/frequency resources - E.g. transmissions/receptions in invalid symbol type for Configuration 1 etc. - Step 2: Resolve collision between transmissions with same transmission direction - Step 3: Resolve collision between transmissions and receptions

In addition to collision cases 1-6 treated in previous meetings, some companies raised whether the collision of more than two overlapping channels should also be treated. As an example, the following collision scenario was raised by Samsung in R1-2409593:

Figure 3: Collision handling scenario from R1-2409593

We would like to note that the above depicted collision scenario is not a new case specific to SBFD. In fact, the overlapping of more than two channels in different (UL/DL) directions is possible in Rel-15/16 specs, with the only difference that the collision would happen in a ‘flexible’ symbol rather than in a ‘SBFD’ symbol. In those cases, the UE should first handle channels in same direction (e.g. PUCCH and PUSCH in figure above), before handling the remaining DL and UL channels, e.g. as specified in TS 38.213:

# 9 UE procedure for reporting control information <text omited> In the remaining of this clause, a UE multiplexes UCIs with same priority index in a PUCCH or a PUSCH before considering limitations for UE transmission due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15 and 17.2. A PUCCH or a PUSCH is assumed to have a same priority index as a priority index of UCIs a UE multiplexes in the PUCCH or the PUSCH. <text omitted> ## 11.1 Slot configuration <text omitted> For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then - If the UE does not indicate the capability of partialCancellationPUCCH-PUSCH-PRACH-TX, the UE does not expect to cancel the transmission of the PUCCH or PUSCH or PRACH in the set of symbols if the first symbol in the set occurs within Tproc,2 relative to a last symbol of a PDCCH reception where the UE detects the DCI format; otherwise, the UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], determined from clauses 9, 9.2.5 and 9.2.6 or clause 6.1 of [6, TS 38.214], or the PRACH transmission in the set of symbols. <text omitted>

Since Rel-19 SBFD has so far agreed to reuse legacy collision handling principles for most of the collision cases, we also think that legacy procedures should be used for handling the overlapping of more than two channels.

The overlapping of more than two channels in UL/DL directions is possible in Rel-15/16 specs, with the only difference that the collision would happen in a ‘flexible’ symbol rather than in a ‘SBFD’ symbol.

Nevertheless, there is still one SBFD-specific aspect which is related to transmission/receptions on unusable resources e.g. in case configuration 1 is provided for UL or DL channels. In this case, we think it is reasonable that such transmission/receptions are excluded as the very first step before handling the collision; however, we think that this is very much an obvious behaviour which could be potentially handled in maintenance phase in case it is not clear from the first version of RAN1 specifications.

Transmission/receptions on unusable resources, e.g. when using configuration 1, should be excluded before handling collisions between channels and signals. Remaining handling of the collision between two or more DL/UL channels or signals follows existing/legacy procedure.

# Other UE Transmission and reception behaviours on SBFD subbands

## Interaction with RRC TDD-UL-DL-ConfigDedicated Configuration

The following agreement was reached in RAN1#116 regarding this issue:

Agreement For SBFD-aware UE transmission and reception in the SBFD symbols configured in DL and/or flexible in TDD-UL-DL-ConfigCommon, - UL transmissions within UL usable PRBs are allowed - FFS SSB symbols - DL receptions within DL usable PRBs are allowed - UL transmissions outside UL usable PRBs are not allowed - DL receptions outside DL usable PRBs are not allowed - This restriction is not applicable for CLI measurement CLI measurement behaviours for SBFD-aware UE are discussed in agenda item 9.3.3. RAN1 to discuss SBFD aware UE behaviors in SBFD symbols with interaction with legacy TDD slot configuration indications via TDD-UL-DL-ConfigDedicated and SFI in DCI format 2_0 - DCI format 2_0 cannot be used to revert SBFD symbol to non-SBFD symbol

In addition, the following agreement from RAN1#116bis is also relevant for this issue:

Agreement A symbol configured as SBFD symbol via cell-specific configuration cannot be reverted to a non-SBFD symbol via any UE-specific configuration or group-common signaling. A symbol not configured as SBFD symbol via cell-specific configuration cannot be reverted to an SBFD symbol via any UE-specific configuration or group-common signaling.

The NR TDD configuration framework has a hierarchical structure consisting of, first, cell-common TDD pattern provided via RRC/SIB signaling (TDD-UL-DL-ConfigCommon); second, UE specific TDD pattern via RRC signalling in TDD-UL-DL-ConfigDedicated and, third, dynamic (lower-layer) indication provided via a DCI scheduling an UL or DL transmission or providing Slot Format Indication (SFI) for one or multiple radio slots. Each of these signalling messages allows to indicate whether a symbol is either UL, DL or flexible, and currently only those symbols defined as ‘flexible’ may be later overridden to be either UL or DL following the specified signalling hierarchy.

If SBFD operation is configured on symbols configured as DL and/or flexible in TDD-UL-DL-ConfigCommon, RAN1#119 has agreed that SFI in DCI format 2_0 is only applicable to non-SBFD symbols but not applicable to SBFD symbols:

Agreement For a serving cell configured with SBFD subband time and frequency location, for an SBFD aware UE, SFI in DCI format 2_0 is only applicable to non-SBFD symbols but not applicable to SBFD symbols - Note: no additional specification efforts are expected to support joint operation of SBFD and dynamic SFI.

The interaction between SBFD cell-specific configuration and TDD-UL-DL-ConfigDedicated is discussed in the following. Figure 4 shows one example operation where SBFD operation is configured over a set of symbols configured as flexible in TDD-UL-DL-ConfigCommon, resulting in a TDD frame structure consisting of DL-only, UL-only, flexible and SBFD symbols. Looking at e.g. slot index 0 in the example in Figure 4, it seems reasonable to keep legacy behaviour in which flexible symbols in that slot can be converted to DL or UL symbols via dedicated signalling. For the second slot in Figure 4 (slot index 1), consisting of flexible and SBFD symbols, we think that it should not be allowed to convert the SBFD symbols to UL or DL only symbols (similarly as agreed for DCI format 2_0 in RAN1#116) since the subbands are cell-specific and apply commonly to all the UEs in the cell. The existing TDD-UL-DL-ConfigDedicated signalling allows to provide a pattern (allDownlink, allUplink, or explicit nrofDownlinkSymbols and nrofUplinkSymbols) for one or more specific slots in the TDD UL-DL pattern. Taking this into account, the following two options seem reasonable for handling TDD-UL-DL-ConfigDedicated information when there is a mix of flexible symbols and SBFD symbols:

- Option 1: The UE does not expect dedicated TDD UL-DL configuration on SBFD symbols, e.g. dedicated configuration can be provided for the flexible symbols in slot index 1, but no UL/DL link direction indication is expected on symbols configured with SBFD operation in the slot.

- Option 2: The UE ignores dedicated TDD UL-DL configuration overlapping with SBFD symbols, i.e. the signalling only applies for flexible symbols in the slot while any UL/DL link direction indication on SBFD symbols is ignored.

Note that Option 2 is similar to what was agreed in RAN1#119 for SFI handling, while option 1 puts a restriction on what can be configured by the gNB.

For a flexible symbol which is not configured with SBFD subbands, TDD-UL-DL-ConfigDedicated can override the flexible symbol into an UL symbol or a DL symbol.  For symbols configured with SBFD subbands, downselect between:

  - Option 1: The UE does not expect dedicated TDD UL-DL configuration on SBFD symbols

  - Option 2: The UE ignores dedicated TDD UL-DL configuration overlapping with SBFD symbols

Figure 4: Interaction of SBFD cell-specific configuration with RRC TDD-UL-DL-ConfigDedicated configuration.

## Tx/Rx transitions from UE perspective

During SBFD symbols, the UE is expected to switch between Tx and Rx mode according to the configured/scheduled transmissions/receptions for the UE (and/or potential link direction information, if supported). TS 38.211, Clause 4.3.2 specify the requirement for UE Tx-Rx transition time in the form of an error case as follows: “A UE not capable of full-duplex communication is not expected to transmit {receive} in the uplink {downlink} earlier than 𝑁Rx-Tx𝑇c {𝑁Tx-Rx𝑇c } after the end of the last received downlink {uplink} symbol”.

The gNB scheduler (and corresponding semi-static configuration) should therefore ensure that the specified requirement is respected for each Tx-Rx and Rx-Tx transition. Note this has nothing to do with the transitions between SBFD- and non-SBFD operation at gNB side; in other words, even though the SBFD time-domain configuration indicates e.g. 2 transitions between SBFD- and non-SBFD symbols within a TDD UL-DL pattern, from UE point of view there could be one, two, three or more transitions within the TDD pattern according to the gNB scheduling decisions on that particular set of symbols. One example is illustrated in Figure 5 which shows the differences between “gNB transitions” and “UE transitions”.

In RAN1#118, some companies proposed to have a limit on the number of transitions (from UE point of view) within e.g. a slot or the TDD UL-DL pattern period. From our point of view, we do not see any need to restrict the number of transitions in any manner, i.e. the UE can switch between Tx and Rx as many times as needed (based on scheduling or configuration) and it is up to the gNB to ensure the Tx-Rx switching time is respected, and gNB can guarantee the overhead due to such Tx-Rx switching time is kept to a reasonable level. Note that current specifications do not have any restriction in terms of number of Tx-Rx transitions for operation in flexible symbols; on top of that, some slot formats in TS 38.213, Table 11.1.1-1 allow up to 4 transitions within ONE slot, e.g. entry 51 with D->U->D->U. Based on these, we do not see motivation to introduce a different and more restrictive behavior for SBFD symbols.

From UE point of view, the number of transitions between Tx (UL) and Rx (DL) within the TDD pattern period is determined based on the configured/scheduled transmissions/receptions for the UE. The number of actual transitions between Tx and Rx within the TDD pattern period can be the same, smaller or larger than the transitions between SBFD- and non-SBFD symbols indicated via the SBFD time-domain configuration.

Current specifications do not have any restriction in terms of number of number Tx-Rx transitions for operation in flexible symbols. Some slot formats in TS 38.213, Table 11.1.1-1 allow D->U->D->U within ONE slot.

RAN1 does not specify any limit regarding the number of Tx-Rx transitions the UE can perform within a time period (e.g. slot or TDD UL-DL pattern period). It is up to the gNB to ensure that the Tx-Rx switching time is respected, and that the overhead due to such Tx-Rx switching time is kept to a reasonable level.

Figure 5: Example of UE Tx-Rx switching according to gNB scheduling.

## Tx/Rx occasions spanning across symbol types within a slot

The following has been agreed in RAN1#119:

Agreement For a physical channel/signal occasion mapped to SBFD and non-SBFD symbols within a slot, an SBFD aware UE does not transmit or receive the physical channel/signal within the slot, except - PRACH transmission in a valid RO across SBFD symbols and non-SBFD symbols is subject to the agreements in AI 9.3.2. - For PUSCH repetition type B, down-select from the following options. - Option 1: A nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols. - Option 2: A nominal repetition is segmented into actual repetitions as in legacy operation. An actual repetition mapped to both SBFD and non-SBFD symbols is dropped.

From the above agreement, RAN1 still need to down-select between Option 1 and Option 2 for PUSCH repetition type B. Between the two options, Option 2 doesn’t allow any optimization while Option 1 allows contiguous transmissions of the repetitions without any dropping. In the context of PUSCH repetition type B for URLLC purpose then Option 1 (which can increase the number of repetitions by better utilizing the resources) is clearly a better choice. Therefore, we propose the following:

For PUSCH repetition type B, support Option 1, i.e., a nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols.

## SBFD with UL Tx switching

UL Tx switching was specified together with unaligned TDD patterns across component carriers (CCs) for maximizing the usage of UE’s hardware in carrier aggregation (CA) scenarios. Indeed, considering an example shown in Figure 6, where two CCs denoted as CC#1 and CC#2 are configured with DDDDDDDSUU and DDDSUDDDSU patterns, respectively, and the former is shifted by 3 slots. Thanks to this technique, a single Tx chain UE would be able to operate in CA or a two (or more) Tx chains UE can use all Tx chains in a single CC to improve the throughput.

Figure 6. Example of UL Tx switching with unaligned TDD patterns.

The above UL Tx switching with unaligned TDD pattern may face an issue when SBFD is configured in one of the CCs. Indeed, let’s consider similar example as in Figure 6, but now SBFD is configured in CC#1 with the 4th to the 8th slots in the 10-slot TDD pattern of CC#1 are configured to be SBFD slots (denoted by F1 to F5), as illustrated in Figure 7.

Figure 7. Example of UL Tx switching in SBFD scenario.

As an example, let’s consider an SBFD-aware UE with a single Tx chain operating in CA with unaligned TDD patterns shown above. Given that SBFD configuration (time and frequency) is commonly configured for all UEs in a cell, the considered UE is also configured with SBFD slots in F1 to F5. This UE has issue with Tx switching when the SBFD slots in CC#1 (F3 & F4) overlap with the UL slots/symbols in CC#2. Indeed, it’s unclear whether the UE should use its single Tx chain on CC#1 or CC#2 at the overlapping symbol.

Although in theory the scenario can be avoided by configuration, but such assumption is unrealistic in practice. On the one hand, one may argue that it’s up to NW to configure SBFD to avoid such overlapping, however it’s very restrictive for NW since the scheduler needs to consider TDD patterns in all CCs and SBFD slots configuration. On the other hand, one may argue that it’s up to NW to schedule UL transmission in only one CC in that case, however it’s very hard for NW’s scheduler to control it, especially for semi-statically configured transmissions. In addition, there is a preparation time needed for Tx switching and therefore UE needs to know well in advance which CC it should switch the Tx chain to. This issue needs further discussion in RAN1, therefore, we propose the following:

RAN1 to discuss potential issue with UL Tx switching and unaligned TDD pattern when there is at least one CC is configured with SBFD.

# Enhancements on physical channels/signals across SBFD symbols and non-SBFD symbols

## Remaining issues on Configuration 2

DL reception occasions across SBFD and non-SBFD symbols in different slots:

Option 5 has been agreed for DL reception occasions across SBFD and non-SBFD symbols as noted in the following agreements:

Working Assumption For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), - Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid. - For SPS PDSCH, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only - For PDSCH repetitions, FFS whether the number of PRBs for TBS determination is based on assigned PRBs, or based on assigned PRBs within DL usable PRBs only, or based on assigned PRBs within DL usable PRBs of the first repetition. - For multi-PDSCH scheduled by a single DCI, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only.

For multi-PDSCH and PDSCH repetitions the following has been furthermore agreed in RAN1#118bis and RAN1#119, respectively:

Agreement For multi-PDSCH scheduled by a single DCI with Configuration 2, the number of PRBs for TBS determination for a PDSCH in SBFD symbols is based on assigned PRBs within DL usable PRBs only. The number of PRBs for TBS determination for a PDSCH in non-SBFD symbols is based on assigned PRBs. Agreement For dynamically scheduled PDSCH repetitions with Configuration 2, for TBS determination. - Option 1: - If the first repetition occasion indicated by scheduling DCI is in SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs. - If the first repetition occasion indicated by scheduling DCI is in non-SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs.

For SPS PDSCH, the following options for TBS determination were discussed in RAN1#119 but not agreed:

- Proposal 2-9 Proposed Agreement: For SPS PDSCH with Configuration 2, down-select from the following options for TBS determination. - Option 1: For SPS PDSCH with or without repetitions, - If the first SPS PDSCH occasion after activation is in SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs. - If the first SPS PDSCH occasion after activation is in non-SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs. - Option 2: For SPS PDSCH with or without repetitions, - The number of PRBs for TBS determination is based on intersection between assigned PRBs and DL usable PRBs. - Option 3: - For SPS PDSCH without repetitions, the number of PRBs for TBS determination for an SPS PDSCH occasion in SBFD symbols is based on assigned PRBs within DL usable PRBs only. The number of PRBs for TBS determination for an SPS PDSCH occasion in non-SBFD symbols is based on assigned PRBs. - For SPS PDSCH with repetitions, if the first repetition for an SPS PDSCH transmission occasion is in SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs. If the first repetition for an SPS PDSCH transmission occasion is in non-SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs.

Looking at the options above, on the one hand, Option 1 and Option 2 have the characteristic that the TBS size is kept the same for all transmit occasions on both SBFD and non-SBFD symbol types. This is suitable for serving applications with periodic traffic and deterministic payload size. On the other hand, Option 3 is better aligned with the agreed behaviour for handling dynamic PDSCH with or without repetitions, i.e. TBS determination for each SPS PDSCH occasion would be performed in a similar way as a dynamic PDSCH in the corresponding slot. From this point of view, our first preference is Option 3, followed by Option 1, while our least-preferred option is Option 2 as it would introduce yet another way of doing TBS determination in the specification (based on intersection between assigned PRBs and DL usable PRBs).

From the above analysis, we propose the following:

For an SPS PDSCH configuration with or without repetitions with Configuration 2, adopt Option 3 as discussed in RAN1#119.

  - For SPS PDSCH without repetitions, the number of PRBs for TBS determination for an SPS PDSCH occasion in SBFD symbols is based on assigned PRBs within DL usable PRBs only. The number of PRBs for TBS determination for an SPS PDSCH occasion in non-SBFD symbols is based on assigned PRBs.

  - For SPS PDSCH with repetitions, if the first repetition for an SPS PDSCH transmission occasion is in SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs. If the first repetition for an SPS PDSCH transmission occasion is in non-SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs.

UL transmissions across SBFD and non-SBFD symbols in different slots:

The following agreements have been reached for UL transmission occasions across SBFD and non-SBFD symbols:

Agreement (RAN1#118) For a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), - Option 2: Single resource configuration/indication for non-SBFD symbols and RB offset(s) configuration/indication/determination to determine frequency resource for SBFD symbols - The numbers of PRBs are the same for PUSCH transmissions in SBFD symbols and PUSCH transmissions in non-SBFD symbols Agreement At least in case when PUSCH frequency hopping is not enabled, for a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, - The number of PRBs for PUSCH transmissions in SBFD and non-SBFD symbols is determined as legacy. - The PRBs for PUSCH transmissions in non-SBFD symbols are determined as legacy. - Consider the following options for determining starting PRB for PUSCH transmissions in SBFD symbols: - Option 1: One or multiple RBoffsetSBFD are configured and the starting PRB for PUSCH in SBFD symbol is determined according to one of the following equation: - Equation 1-A: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFD - Negative values of RBoffsetSBFD are not precluded. - Equation 1-B: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFDmodNBWPsize - Equation 1-C: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize - Equation 1-D: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD-RBstartUL SB+RBoffsetSBFDmodNUL SBsize - Option 2: The starting PRB for PUSCH in SBFD symbol is determined according to one of the following equation: - Equation 2-A: RBstartSBFD = RBstartUL SB+mod(RBstartnon-SBFD,NUL SBsize -NPUSCHsize) - Equation 2-B: RBstartSBFD = RBstartUL SB+mod(RBstartnon-SBFD,NUL SBsize ) - Equation 2-C: RBstartSBFD = round(RBstartnon-SBFD* NUL SBsizeNBWPsize) + RBstartUL SB - Equation 2-D: RBstartSBFD = RBstartUL SB+RBstartnon-SBFD - The variables are defined as follows: - RBstartSBFD is the starting PRB index of PUSCH in SBFD symbol with reference to the start of UL active BWP - RBstartnon-SBFD is the starting PRB index of PUSCH in non-SBFD symbol with reference to the start of UL active BWP - RBstartUL SB is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP - NBWPsize is the number of PRBs of UL BWP - NUL SBsize is the number of PRBs of UL usable PRBs - NPUSCHsize is the number of PRBs for PUSCH transmissions - No additional configuration/indication to enable/disable whether to apply RBSBFDoffset in SBFD symbols. - FFS: Condition to enable/disable - UE does not expect that the PRBs for PUSCH transmissions in SBFD symbols after applying RBSBFDoffset to be overlapped with PRBs outside UL usable PRBs. - It applies at least to RA type 1. FFS for RA type 0. - Note: Other equations are not precluded. RAN1#119: Agreement At least in case when PUSCH frequency hopping is not enabled, for a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for determining starting PRB for PUSCH transmissions in SBFD symbols, - Equation 1-C2: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize - If RBoffsetSBFD is not configured, it is zero

The above “FFS: Condition to enable/disable” was added mainly to address the case when the RBstartnon-SBFD is within the UL subband, in this case whether one should still “enable” (apply) the equation for calculating RBstartSBFD or simply set RBstartSBFD= RBstartnon-SBFD. However, looking at the agreed Equation 1-C2, one can still find an RBoffsetSBFD, such that RBstartSBFD= RBstartnon-SBFDfor the RB indices within the UL subband. Therefore, RAN1 should not add any further condition related to this FFS.

RAN1 should not introduce any further condition related to the enabling/disabling of the determination of starting PRB for PUSCH transmissions in SBFD symbols.

## PDCCH

As RAN1 discussed in SI phase, since PDCCH resources may be interrupted by UL subband/guard band in SBFD symbols/slots, RAN1 has already listed the candidate solutions to solve the PDCCH resource allocation in SBFD symbols/slots.

Above all, as the DL frequency resources in DL or flexible symbols/slots and in SBFD symbols/slots are different, SBFD UE should have different principle or rule for PDCCH detection. Since PDCCH in SBFD symbols/slots can be only in DL subband(s) in frequency domain, there should be different PDCCH resources in different slot types, as shown in Figure 8. From this point of view, to align with the definition of CORESET and search space, it is good to define different CORESET in SBFD and non-SBFD slots while they can be corresponding to the same search space, so that the PDCCH can be searched in different frequency resource for same PDCCH format.

Figure 8: PDCCH resource in non-SBFD and SBFD slots

SBFD UE should have different PDCCH resources in SBFD and non-SBFD symbols/slots. Search space for same PDCCH format(s) should be corresponding to different frequency resources, i.e. CORESET, on different slot types.

Considering different frequency resource in SBFD and non-SBFD symbols/slots, if only one CORESET is configured for a SBFD UE for both SBFD and non-SBFD slots, then some CCE/REG may overlap with the UL subband/guard band and not available for PDCCH in SBFD slot. In this case, all the PDCCH candidates that the UE monitors should be fully on the DL subband, and the UE should not need to monitor PDCCH candidates that overlap with a CCE/REG outside the DL subband which also helps to avoid unnecessary power consumption.

Some PDCCH candidates, e.g. with aggregation level 4/8/16, may be disrupted as they overlap with UL subband/guard band.

Different number of PDCCH candidates are available in SBFD and non-SBFD slots.

Only the available/non-disrupted PDCCH candidates can be considered in PDCCH detection, in both SBFD and non-SBFD slots.

## CSI Reporting

The following was agreed in RAN1#117:

Agreement For CSI report associated with periodic/semi-persistent CSI-RS, discuss and decide whether to support the following options. - Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only. - gNB configuration may not ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only. - For the CSI-ReportConfig associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. For the CSI-ReportConfig associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation. - Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols Agreement For a CSI reporting subband which overlaps with at least one PRB within DL usable PRBs and one or more PRBs outside DL usable PRBs, the CSI reporting subband includes PRB(s) within DL usable PRBs only. Agreement For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

As noted in the agreement, RAN1 has agreed to support separate CSI reports on SBFD and non-SBFD symbols, where the valid symbol type for CSI derivation for the CSI report is explicitly configured. In our understanding, this means that a valid symbol type should be provided as part of the CSI-ReportConfig IE

The valid symbol type for CSI derivation for the CSI report is explicitly configured in CSI-ReportConfig.

# Configurations for channels and signals on SBFD symbols and non-SBFD symbols

## Separate configuration of Frequency hopping offsets

The following was agreed in RAN1#118:

Agreement Support separate configurations of FH offsets for SBFD symbols and non-SBFD symbols, for a type 1 CG PUSCH with Configuration 2. - Introduce a new RRC parameter in rrc-ConfiguredUplinkGrant in ConfiguredGrantConfig to configure FH offset for SBFD symbols. Support separate configurations of FH offset lists for SBFD symbols and non-SBFD symbols, for PUSCH scheduled by DCI and type 2 CG PUSCH. - Introduce new RRC parameters in PUSCH-Config to configure a set of FH offsets for SBFD symbols. - Support separate configurations for DCI format 0_2 and other DCI formats as legacy - FFS: The number of bits used to indicate FH offset in DCI is determined as legacy UE applies the FH offset/FH offset list according to the symbol type of the PUSCH transmissions. FFS UE behaviour when FH offset/FH offset list is not provided for SBFD symbols, e.g. FH is disabled for PUSCH transmissions in SBFD symbols, or FH offset/FH offset list for non-SBFD symbols are applied for SBFD symbols etc. Agreement Support separate frequency configurations for SBFD symbols and non-SBFD symbols in the same PUCCH-Resource. - pucch-ResourceId is not separately configured for SBFD and non-SBFD symbols - Support separate configurations of startingPRB and secondHopPRB for SBFD symbols and non-SBFD symbols - Introduce new RRC parameters in PUCCH-Resource to configure starting PRB and second hop PRB for SBFD symbols - FFS whether to support separate configurations of intraSlotFrequencyHopping for Configuration 1 or for both Configuration 1 and 2 - No change on the maximum number of PUCCH resources supported by a UE - Above PUCCH resources with the same pucch-ResourceId is counted as 1 resource FFS: UE behaviour when no separate configuration is provided for SBFD symbols, e.g. PUCCH transmissions in SBFD symbols for this pucch-ResourceId is not expected, or configurations for non-SBFD symbols are applied for SBFD symbols (in which case it is not expected that the configurations would lead to unexpected transmissions) etc. RAN1#118bis: Agreement The number of configured FH offsets of FH offset list for SBFD symbols is the same as the number of FH offsets of FH offset list for non-SBFD symbols. - The number of configured FH offsets of FH offset list is determined based on the UL BWP size as legacy. The number of bits used to indicate FH offset in DCI is determined as legacy.

This section discusses the remaining issues highlighted in the FFS points of the above agreements. Concerning UE behaviour when FH offset/FH offset list or separate frequency configuration is not provided for SBFD symbols, two examples have been provided in the FFS point regarding how this scenario should be handled namely, i) FH is disabled for PUSCH transmissions in SBFD symbols and, ii) FH offset/FH offset list for non-SBFD symbols are applied for SBFD symbols. For the latter, a procedure needs to be further specified for clarifying how the FH offset/FH offset list for non-SBFD symbols can be used for SBFD symbol. If RAN1 would like to adopt this approach, then the following logical questions should be clarified:

- If reusing FH offset/FH offset list for non-SBFD symbols is straightforward, why did RAN1 agree on configuring a separate FH offset/FH offset list for SBFD symbols in the first place? This would result in two solutions being adopted in RAN1 for the same issue which, in principle, should be avoided for standardization.

- If reusing FH offset/FH offset list for non-SBFD symbols is not straightforward, why would RAN1 specify it given the specification efforts and given that gNB can ensure configuring the additional parameter when there is a need for FH in SBFD symbols?

From the above analysis, a safer choice should be the former solution, i.e., FH is disabled for PUSCH transmissions in SBFD symbols in such case.

If FH offset/FH offset list is not provided for SBFD symbols, FH is disabled for PUSCH transmissions in SBFD symbols. If no separate configuration is provided for SBFD symbols, PUCCH transmissions in SBFD symbols for the pucch-ResourceId is not expected.

Concerning whether to support separate configurations of intraSlotFrequencyHopping for Configuration 1 or for both Configuration 1 and 2, the key question is that whether intra-slot FH can be enabled in one symbol type and disabled in another symbol type for Configuration 2. In this regard, it may be beneficial to support such functionality for Configuration 2 given that resource management for multiple UEs in such a small UL sub-band bandwidth could be challenging and might be, at high traffic, frequency hopping would make it even worse (from the entire network performance point of view, but not from a single UE point of view). It’s worth noting that, gNB has full freedom to enable/disable such intra-slot FH, when needed. Therefore, the following is proposed:

RAN1 to support separate configurations of intraSlotFrequencyHopping for both Configurations 1 and 2.

## Separate configurations for SRS

The following was agreed in RAN1#118bis:

Agreement Support separate SRS configurations for SBFD and non-SBFD symbols. Select one from the following options. - Option 1: Support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage. - SRS-ResourceSet configured for SBFD symbol is only applicable to SRS transmission occasions in SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in non-SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only SBFD symbols are available for SRS transmission. - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the SBFD symbols. - SRS-ResourceSet configured for non-SBFD symbol is only applicable to SRS transmission occasions in non-SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only non-SBFD symbols are available for SRS transmission. - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the non-SBFD symbols. - FFS applicable usages - Option 2: Support separate configurations for SBFD and non-SBFD symbols in the same SRS-Resource. - At least freqDomainPosition, freqDomainShift and freqHopping are separately configured. - FFS separate configurations of other parameters - FFS applicable usages

Furthermore, the following agreement was reached in RAN1#119:

Agreement Red text is working assumption. For separate SRS configurations for SBFD and non-SBFD symbols, support Option 1. Option 1: Support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage. - SRS-ResourceSet configured for SBFD symbol is only applicable to SRS transmission occasions in SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in non-SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only SBFD symbols are available for SRS transmission. - FFS impact on availableSlotOffsetList - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the SBFD symbols. - FFS: whether there is impact on frequency hopping counter - SRS-ResourceSet configured for non-SBFD symbol is only applicable to SRS transmission occasions in non-SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only non-SBFD symbols are available for SRS transmission. - FFS impact on availableSlotOffsetList - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the non-SBFD symbols. - FFS: whether there is impact on frequency hopping counter - The number of SRS resources in a SRS-ResourceSet for SBFD symbols is the same as the number of SRS resources in a SRS-ResourceSet for non-SBFD symbols for the same usage. - For SRS-ResourceSets with usage set to 'nonCodebook', only one SRS port for each SRS resource is configured as legacy. - For SRS-ResourceSets with usage set to 'Codebook', - Except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', the numbers of SRS ports are the same for all the SRS resources in the two SRS-ResourceSets - FFS when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2' - Maintain single TRP behaviour with respect to DCI fields and RRC parameters in rrc-ConfiguredUplinkGrant - SRS resource set indicator (SRSI) field is absent from DCI. srs-ResourceSetId is absent in rrc-ConfiguredUplinkGrant. - If txConfig is set to 'codebook', second precoding information (TPMI) field is absent from DCI and precodingAndNumberOfLayers2 is absent from rrc-ConfiguredUplinkGrant. - Second SRS resource indicator (SRI) field is absent from DCI - For DG PUSCH without repetition within a slot, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in the same symbol type as that of PUSCH transmission, where the SRS resource prior to the PDCCH carrying the SRI. - If txConfig is set to 'codebook', the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to the same symbol type as that of PUSCH transmission - For DG PUSCH with repetition/multi-PUSCH scheduled by a single DCI/TBoMS/Type 2 CG PUSCH with Configuration 1, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in the same symbol type as the valid symbol type of PUSCH transmission, where the SRS resource prior to the PDCCH carrying the SRI. - If txConfig is set to 'codebook', the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to the same symbol type as the valid symbol type of PUSCH transmission - For DG PUSCH with repetition/multi-PUSCH scheduled by a single DCI/TBoMS/Type 2 CG PUSCH with Configuration 2, the SRS resource indicator is applicable to both SBFD and non-SBFD SRS-ResourceSets. For PUSCH transmissions in SBFD symbols, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in SBFD symbols, where the SRS resource is prior to the PDCCH carrying the SRI. For PUSCH transmissions in non-SBFD symbols, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in non-SBFD symbols, where the SRS resource is prior to the PDCCH carrying the SRI. - If txConfig is set to 'codebook', for PUSCH transmission in SBFD symbols, the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to SBFD symbols. For PUSCH transmission in non-SBFD symbols, the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to non-SBFD symbols. - If only a single SRS resource is configured in both SRS resource sets, the SRI field is absent from DCI. In this case “SRS resource identified by the SRI” in the above bullets corresponds to the single SRS resource in each SRS resource set. - srs-ResourceIndicator2 is absent from rrc-ConfiguredUplinkGrant. - For Type 1 CG PUSCH with Configuration 1, srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in the same symbol type as the valid symbol type of PUSCH transmission. - If txConfig is set to 'codebook', precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to the same symbol type as the valid symbol type of PUSCH transmission. - For Type 1 CG PUSCH with Configuration 2, srs-ResourceIndicator is applicable to both SBFD and non-SBFD SRS-ResourceSets. For PUSCH transmissions in SBFD symbols, the srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in SBFD symbols. For PUSCH transmissions in non-SBFD symbols, the srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in non-SBFD symbols. - If txConfig is set to 'codebook', for PUSCH transmission in SBFD symbols, precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to SBFD symbols. For PUSCH transmission in non-SBFD symbols, precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to non-SBFD symbols. - If txConfig is set to 'nonCodebook', each SRS resource set is associated with an associated CSI-RS. - Applicable to usage set to 'codebook' or 'nonCodebook' - FFS usage set to 'beamManagement' - Not applicable to usage set to 'antennaSwitching’

This section discusses remaining open points in the above agreement made in RAN1#119.

Concerning “FFS impact on availableSlotOffsetList”, the following can be noted from Section 6.2.1 of TS 38.214:

For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a slot level offset is defined by the higher layer parameter slotOffset. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a list of up to four different available slot offset values from the reference slot n + k to the slot where the aperiodic SRS resource set is transmitted where n is the slot with triggering DCI and k is slotOffset, can be configured by the higher layer parameter availableSlotOffsetList.

It can be observed from the above specs excerpt that, if the available slot offset is counted on the available slot for SRS transmission, then the above agreement just further clarifies the definition of available slot in case there are SBFD slots. Therefore, it seems there is no impact on the availableSlotOffsetList.

The agreement in RAN1#119 just further clarifies the definition of available slot in case there are SBFD slots. Therefore, it seems there is no impact on the availableSlotOffsetList.

Concerning “FFS: whether there is impact on frequency hopping counter”, the following can be noted from Section 6.4.1.4.3 of TS 38.211:

noffset2FH=ninithop+nSRSTxHopping mod Nhop-ninithopmSRS,0-moverlaphopNscRB and - kF∈0,1,…,PF-1 is given by the higher-layer parameter StartRBIndex if configured, otherwise kF=0; - khop is given by Table 6.4.1.4.3-3 with khop=nSRSb'=bhopBSRSNb' mod PF Nbhop=1 if the higher-layer parameter EnableStartRBHopping is configured, otherwise khop=0. - moverlaphop∈0,1,2,4 is given by the higher-layer parameter overlapValue in TxHoppingConfig. - nSRSTxHopping=0,1,…,NhopsSRS-1 is the hop transmission counter in the time domain, where nSRSTxHopping=1,2,…,NhopsSRS-1 corresponds to the order of the higher-layer parameter SlotOffsetForRemainingHops in slotOffsetForRemainingHopsList, wherein the UE expects to be configured with the starting slot offset and starting symbol of the NhopsSRS hops in an ascending order sequentially in time domain. - ninithop=nshiftmSRS,0-moverlaphop is the initial hop index.

It can be observed from the above excerpt that the hop transmission counter may be relevant to the FFS. However, the hop transmission counter was introduced for positioning purpose and any optimization for it should be deprioritized in the design of SBFD.

RAN1 should avoid micro optimization on frequency hopping counter if there is no clear benefit.

Concerning “FFS when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2'”, as long as the antenna ports configurations are the same for the two SRS-ResourceSets then there shouldn’t be any issue with 'fullpowerMode2'.

As long as the antenna ports configurations are the same for the two SRS-ResourceSets then there should not be any issue with 'fullpowerMode2'.

Concerning “FFS usage set to  'beamManagement'”, the ‘beamManagement’ usage already supports multiple SRS-ResourceSets. Therefore, for beam management purpose, the introduction of new SRS-ResourceSets for SBFD and non-SBFD symbols doesn’t seem to be necessary. RAN1 should avoid any further enhancement for the ‘beamManagement’ usage.

RAN1 should avoid any further enhancement for the ‘beamManagement’ usage.

# Conclusion

In this contribution we have provided our views regarding the required signaling to support SBFD operation, as well as UE transmission, reception and measurement behaviors. The following observations and proposals have been made:

Proposal 1:	The working assumption in RAN1 #116 agreement for cell specific configuration of frequency location of SBFD subband should be confirmed.

Proposal 2:	In addition to cell-specific configuration of DL and UL subbands, signaling of UE-specific configuration can be supported, e.g. to configure some UEs to operate with larger guardband due to poorer filtering capabilities or different UE-UE CLI conditions compared guardband of gNB.

Proposal 3:	Signaling of the SBFD time and frequency configuration via RRC dedicated signaling can be considered, additionally to signaling in SIB, to support dual connectivity or carrier aggregation with SBFD carriers.

Proposal 4:	RAN1 should finalize the LS on UE-specific guardband and send to RAN4.

Observation 1:	The TDD UL-DL pattern (provided via TDD-UL-DL-ConfigCommon) has typically a short periodicity of 1..5 ms, while some reference signals (e.g. SSB or CSI-RS for tracking) are transmitted with much larger periodicity (e.g. 20 .. 160 ms). The SBFD time-domain configuration shall allow SBFD operation in the majority of the time slots, while still allow some ‘sparse’ slots to operate as legacy/DL-only manner.

Proposal 5:	The periodic UL-DL pattern configuration can be complemented with additional cell-specific signaling that can change the configuration or format for one or some slots.

Observation 2:	For link direction determination, Option 2 (link direction is indicated by gNB explicitly) assigns a pre-defined link direction (e.g. per slot) to the UE which diminishes the benefits of SBFD.

Observation 3:	In contrast, Option 1 (based on collision handling rules) is more flexible and better aligned with legacy UE handling. Option 1 could benefit of additional indications that allows the UE to determine UL/DL priorities e.g. for some (but not all) UL/DL collision cases.

Proposal 6:	For the link direction determination, support Option 1 from RAN1#116 as baseline:

	Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any).

Note: Option 1 should not preclude enhancements to existing collision handling rules for SBFD operation.

Proposal 7:	If option 2 should be further discussed based on Option 1, option 2 should not fix the link direction in advance resulting diminishing the benefit of SBFD. For instance, existing collision handling rules may be supplemented with additional indications to determine UL/DL priorities for upcoming slot(s) (potentially limited to specific UL/DL collision cases) but without fixing the link direction in advance.

Proposal 8:	For collision Case 3 (semi-statically configured DL reception vs. semi-statically configured UL transmission), if link direction priority indication is provided to the UE, semi-static UL and DL transmissions can overlap in time and the link direction priority indication is used to prioritize one or another signal.

Proposal 9:	For link direction priority indication, RAN1 to consider at least the following for SBFD:

	RRC level-signaling can indicate whether to prioritize semi-static UL or semi-static DL in certain slots in case of overlapping between semi-static UL and semi-static DL signals.

	gNB indicates dynamically whether the UE should prioritize UL or DL transmissions in upcoming slot(s).

Observation 4:	On collisions between configured SSB and UL transmission, limiting the SSB block/CORESET0 to be within DL subband in Option 1 have significant drawbacks, e.g. it prevents SBFD operation in FR2 carriers of 100 MHz bandwidth, or even 200 MHz considering raster size of 17.28 MHz, or FR1 with limited bandwidth as 20/30/40MHz. Also, it limits the flexibility for placing the SSB according to operator-specific preferences.

Observation 5:	On collisions between configured SSB and UL transmission, transitions between SBFD and non-SBFD operation are not only for Option 2, but also apply to Option 1 considering that it is desirable to transmit the SSBs using all the antenna elements of the gNB antenna panel. This allows the SSB signals to reach the cell-edge and keep the SSB transmissions unchanged as compared to legacy TDD operation.

Proposal 10:	On collisions between configured SSB and UL transmission, RAN1 to revert the working assumption reached RAN1#118bis and adopt Option 2 instead (a slot consisting of SSB symbols is considered as a full DL slot)

Proposal 11:	RAN1 can also discuss some compromise solution, e.g. on which case Option 1 is selected and on which case Option 2 is selected.

Observation 6:	The overlapping of more than two channels in UL/DL directions is possible in Rel-15/16 specs, with the only difference that the collision would happen in a ‘flexible’ symbol rather than in a ‘SBFD’ symbol.

Proposal 12:	Transmission/receptions on unusable resources, e.g. when using configuration 1, should be excluded before handling collisions between channels and signals. Remaining handling of the collision between two or more DL/UL channels or signals follows existing/legacy procedure.

Proposal 13:	For a flexible symbol which is not configured with SBFD subbands, TDD-UL-DL-ConfigDedicated can override the flexible symbol into an UL symbol or a DL symbol.  For symbols configured with SBFD subbands, downselect between:

	Option 1: The UE does not expect dedicated TDD UL-DL configuration on SBFD symbols

	Option 2: The UE ignores dedicated TDD UL-DL configuration overlapping with SBFD symbols

Observation 7:	From UE point of view, the number of transitions between Tx (UL) and Rx (DL) within the TDD pattern period is determined based on the configured/scheduled transmissions/receptions for the UE. The number of actual transitions between Tx and Rx within the TDD pattern period can be the same, smaller or larger than the transitions between SBFD- and non-SBFD symbols indicated via the SBFD time-domain configuration.

Observation 8:	Current specifications do not have any restriction in terms of number of number Tx-Rx transitions for operation in flexible symbols. Some slot formats in TS 38.213, Table 11.1.1-1 allow D->U->D->U within ONE slot.

Proposal 14:	RAN1 does not specify any limit regarding the number of Tx-Rx transitions the UE can perform within a time period (e.g. slot or TDD UL-DL pattern period). It is up to the gNB to ensure that the Tx-Rx switching time is respected, and that the overhead due to such Tx-Rx switching time is kept to a reasonable level.

Proposal 15:	For PUSCH repetition type B, support Option 1, i.e., a nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols.

Proposal 16:	RAN1 to discuss potential issue with UL Tx switching and unaligned TDD pattern when there is at least one CC is configured with SBFD.

Proposal 17:	For an SPS PDSCH configuration with or without repetitions with Configuration 2, adopt Option 3 as discussed in RAN1#119.

	For SPS PDSCH without repetitions, the number of PRBs for TBS determination for an SPS PDSCH occasion in SBFD symbols is based on assigned PRBs within DL usable PRBs only. The number of PRBs for TBS determination for an SPS PDSCH occasion in non-SBFD symbols is based on assigned PRBs.

	For SPS PDSCH with repetitions, if the first repetition for an SPS PDSCH transmission occasion is in SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs. If the first repetition for an SPS PDSCH transmission occasion is in non-SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs.

Proposal 18:	RAN1 should not introduce any further condition related to the enabling/disabling of the determination of starting PRB for PUSCH transmissions in SBFD symbols.

Proposal 19:	SBFD UE should have different PDCCH resources in SBFD and non-SBFD symbols/slots. Search space for same PDCCH format(s) should be corresponding to different frequency resources, i.e. CORESET, on different slot types.

Observation 9:	Some PDCCH candidates, e.g. with aggregation level 4/8/16, may be disrupted as they overlap with UL subband/guard band.

Observation 10:	Different number of PDCCH candidates are available in SBFD and non-SBFD slots.

Proposal 20:	Only the available/non-disrupted PDCCH candidates can be considered in PDCCH detection, in both SBFD and non-SBFD slots.

Proposal 21:	The valid symbol type for CSI derivation for the CSI report is explicitly configured in CSI-ReportConfig.

Proposal 22:	If FH offset/FH offset list is not provided for SBFD symbols, FH is disabled for PUSCH transmissions in SBFD symbols. If no separate configuration is provided for SBFD symbols, PUCCH transmissions in SBFD symbols for the pucch-ResourceId is not expected.

Proposal 23:	RAN1 to support separate configurations of intraSlotFrequencyHopping for both Configurations 1 and 2.

Observation 11:	The agreement in RAN1#119 just further clarifies the definition of available slot in case there are SBFD slots. Therefore, it seems there is no impact on the availableSlotOffsetList.

Proposal 24:	RAN1 should avoid micro optimization on frequency hopping counter if there is no clear benefit.

Observation 12:	As long as the antenna ports configurations are the same for the two SRS-ResourceSets then there should not be any issue with 'fullpowerMode2'.

Proposal 25:	RAN1 should avoid any further enhancement for the ‘beamManagement’ usage.