3GPP TSG RAN WG1 Meeting #120	R1-2501025

Athens, Greece, February 17th – 21st, 2025

Source:	MediaTek Inc.

Title:	LP-WUS operation in CONNECTED modes

Agenda item:	9.6.3

Document for:	Discussion and decision

# Introduction

This contribution proposes reusing C-DRX settings for LP-WUS MOs in connected mode (Option 1-1) and aligning Option 1-2 with Option 1-1. It also suggests specific candidate values for the UE capability report on the minimum time gap and provides recommendations on RRC parameters.

# C-DRX Setting Reuse

Agreement - For LP-WUS MOs in connected mode for Option 1-1, following is considered further discussion and possible down-selection - Approach 1: - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - UE selectively monitors LP-WUS in the LP-WUS MOsidentified by other parameters such as time offset prior to the start of drx-onDurationTimer. - Approach 2: - LP-WUS MOs, including periodicity and offset, are identified based on the slot that drx-onDurationTimer would start and a new RRC parameter indicating the time offset until the slot that drx-onDurationTimer would start. - FFS: AnRRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time] - FFS: the consecutive LP-WUS MOsin time identified by the RRC parameterare associated with a same C-DRX cycle; - FFS: LP-WUS MOsin time identified by the RRC parameterprovides consistent indication for the same C-DRX cycle of the UE, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOsassociated with the same C-DRX cycle. - For LP-WUS MOsin connected mode for Option 1-2, following is considered further - Approach 1: - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - UE monitors LP-WUS in the LP-WUS MOsand starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset. - Approach 2: - Periodicity/offset of the slot that the new timer for PDCCH monitoring may be potentially triggered by LP-WUS are configured by RRC parameters. - LP-WUS MOs, including periodicity and offset, are identified based on the slot that new timerwould start and a new RRC parameter indicating the time offset until the slot that new timerwould start. - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time] - FFS: the consecutive LP-WUS MOsin time identified by the RRC parameterare associated with the same slot that the new timer would start; - FFS: LP-WUS MOsin time identified by the RRC parameterprovides consistent indication for the same slot that the new timer would start, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOsassociated with the same slot that the new timer would start.

For Option 1-1 (replace DCP), Approach 2 suggests that the C-DRX setting should be reused.

Approach 2 leverages the C-DRX cycle, ensuring that LP-WUS monitoring occasions are synchronized with DRX cycles. This reduces complexity and avoids potential overlaps between LP-WUS occasions and active DRX periods, thereby simplifying UE behavior and processing.

Proposal 1:	For Option 1-1 (replace DCP), Approach 2: C-DRX setting should be reused.

For Option 1-2, Approach 2 should align with Option 1-1.

A unified framework for both Option 1-1 and Option 1-2 is preferred. Applying Approach 2 to Option 1-2 ensures consistency with Option 1-1. This alignment reduces complexity and enhances reliability. Also, it may lower configuration overhead, leading to more efficient configuration.

Proposal 2:	For Option 1-2, Approach 2 should be considered to align with Option 1-1.

# Minimum time gap

[Fri]Combined Proposal 3.2-1/3.2-2c: For future meetings: Consider the following alternatives for UE capability report on the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring: - Alt 1: For the X = [2, 3, 4] candidate values for the UE capability report on the minimum time gap, consider {0.25, 0.5, 1, 2, 3, 6, 10, 20}ms as starting point - Note: Other values are not precluded - FFS whether different values are selected for OOK-based WUR and OFDM-based WUR - FFS whether/how to consider the case of CA - Alt 2: For the UE capability report on the minimum time gap, UE reports a pair of values for main radio ramp up and the number of SSBs/TRSs needed for synchronization (D, NSSB/NTRS). This may require RAN4 coordination - Alt 3: UE capability report on the minimum time gap includes the main radio ramp up time. The number of SSBs/TRSs needed for synchronization is defined in specs, and different number of SSBs/TRSs can be defined for different UE capability on the main radio ramp up time. This may require RAN4 coordination - Alt 4: UE capability report on the minimum time gap includes the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later. - FFS: For the case of UAI

In RAN1#118bis, companies agreed UE reports one value per SCS. Since reporting one value per SCS is the same as the legacy design of R16 DCI format 2-6, we recommend following the same mechanism as shown in Alt 1 and reusing the same values as R16 DCI format 2-6 from 38.213 Table 10.3-1.

Proposal 3:	For UE capability report, support Alt 1: For the X = 2 candidate values for the UE capability report on the minimum time gap, consider 1ms and 3ms as starting point.

# RRC parameters

The recommended RRC parameters are listed below.

| Sub-feature group | Parameter name in the spec | Description | Value range | Default value | Per | UE-specific or Cell-specific | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Option 1‑1 Configuration | OPTION1_1_CONFIG | Configuration parameters for Option 1‑1 of LP‑WUS CONNECTED mode operation, allowing independent setup separate from Option 1‑2. Includes periodicity, offset, and monitoring parameters specific to Option 1‑1. | To be specified | To be specified | Per UE | UE-specific | Agreement in R1#118b: … Support Option 1‑1: LP‑WUS monitoring according to the LP‑WUS monitoring configuration before drx‑onDurationTimer to trigger the starting of the drx‑onDurationTimer. … |
| Option 1‑2 Configuration | OPTION1_2_CONFIG | Configuration parameters for Option 1‑2 of LP‑WUS CONNECTED mode operation, allowing independent setup separate from Option 1‑1. Includes periodicity, offset, and monitoring parameters specific to Option 1‑2. | To be specified | To be specified | Per UE | UE-specific | Agreement in R1#118b: … Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. … |
| LP-WUS QCL Configuration | LPWUS_QCL_TYPES | Specifies the supported QCL sources (SSB and/or CSI‑RS) and the corresponding LP‑WUSQCL types (Type A, Type C, and Type D) for LP‑WUS monitoring in RRC CONNECTED mode. | {SSB, CSI-RS} with {Type A, Type C} and Type D | To be specified | Per UE | UE-specific | Agreement in R1#119: … SSB and/or CSI‑RS can be the QCL source of LP‑WUSQCL type(s) is {Type A or Type C – for down‑selection} and Type D, when applicable. |
| Option 1‑2 Configuration | PERIODIC_UL_REPORTING | Specifies whether periodic CSI/L1-RSRP reporting is enabled or disabled in LP‑WUS CONNECTED mode based on Option 1‑2 configurations. | Enabled, Disabled | Disabled by default | Per UE | UE-specific | Agreement in R1#119: For Option 1‑2, … The UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting… |
| Reporting Time Offset Configuration | NUM_TIME_OFFSET | Specifies the number of candidate time offset values (X) that the UE can report for each SCS to determine the minimum time gap between LP‑WUS reception and MR to start PDCCH monitoring. | X (e.g., 2, 3, 4) | 2 | Per UE | UE-specific | Agreement in R1#118b: For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap … |
| Reporting Time Offset Configuration | TIME_OFFSET_UAI | Configures whether the UE can indicate its preference for a specific configured time offset between LP‑WUS and PDCCH reception using the UAI mechanism. | Enabled, Disabled | Enabled by default | Per UE | UE-specific | Agreement in R1#118b: … UE can indicate its preference for configured time offset |
| NR‑DC Configuration | LPWUS_NR_DC_CONFIG | Specifies that LP‑WUS is supported when UE is configured with NR‑DC in RRC CONNECTED mode. Includes parameters to configure LP‑WUS triggers for PDCCH monitoring within the same cell group. | To be specified | To be specified | Per UE | UE-specific | Agreement in R1#118b: LP‑WUS is supported when UE is configured with NR‑DC in RRC CONNECTED mode … |
| NR-CA Configuration | LPWUS_NR_CA_CONFIG | Specifies that LP-WUS is supported when UE is configured with CA in RRC CONNECTED mode. Includes parameters to configure LP-WUS triggers for PDCCH monitoring in CA mode. | To be specified | To be specified | Per UE | UE-specific | Agreement in R1#118: LP‑WUS is at least supported for the case where a UE is configured with CA in RRC CONNECTED mode. FFS: DC" |

# Conclusion

In this contribution, we have the following proposals:

Proposal 1:	For Option 1-1 (replace DCP), Approach 2: C-DRX setting should be reused.

Proposal 2:	For Option 1-2, Approach 2 should be considered to align with Option 1-1.

Proposal 3:	For UE capability report, support Alt 1: For the X = 2 candidate values for the UE capability report on the minimum time gap, consider 1ms and 3ms as starting point.