3GPP TSG RAN WG1 #120                                                                                       R1-2501072

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.4.2

Source:	Lenovo

Title:	Discussion on the Ambient IoT physical layer design aspects for Line coding, FEC, CRC, Repetition

Document for:	Discussion

# Introduction

In RAN#106 meeting, for Rel-19 work item of ambient IoT to include device type 1 was agreed. The relevant scopes have been extensively down-scoped in RP-243326 [1]. This contribution provides discussion into the physical layer design aspect such as line coding, FEC, CRC and repetition for Ambient IoT devices.

# Physical layer design for Ambient IoT devices

In TSG RAN#106, device type 1 containing ~1 device power consumption was agreed to be specified in Rel19 Ambient IoT normative phase.

The Ambient IoT device type 1 having a target power consumption of ~1 µW should have a better design to achieve more coverage compared to UHF RFID. The design to achieve more coverage for Ambient IoT device type 1 should go beyond the implementation aspects such as higher transmit power, lower tag return loss and backscatter loss. Moreover, the sampling clock accuracy after calibration for device type 1 which was heavily discussed during RAN1#119 should be discussed during the scoping phase and captured as one of the design requirement for device type 1 as many of the design features starting from X-ambles i.e., preamble, midamble, postamble, D2R small frequency shift for device type 1, multiple access i.e., TDMA, FDMA, CDMA for Msg1 etc., depends on the accuracy. Since the target is to have better design feature for device type 1 compared to UHF RFID and there are many references showing that the implementation captured in TR for 104 PPM clock accuracy after calibration is achievable using clock counting or using the external carrier wave, the design of Ambient IoT type 1 should design features according to 104 clock accuracy after calibration.

Proposal 1: 3GPP Ambient IoT device type 1 normative work should target better design feature compared to UHF RFID to achieve better coverage:

- The design requirement should take into account, the maximum achievable clock accuracy which is primarily the target to design X-amble, D2R multiple access etc.,

- Maximum achievable clock accuracy after clock calibration for Ambient IoT device type 1 should be kept as 104 PPM for design target during normative phase.

## Line coding

Either the Manchester line code in TR 38.769 or no line code (one to be down-selected);

The Rel19 WI objective guidance on the line coding includes:

R2D transmission supports only the Manchester line code in TR 38.769

D2R transmission supports:

Manchester line coding is better than no line coding and helps with clock recovery from the state transition.

For Manchester encoding, the bit-to-chip mapping for 0.5 code rate is bit 0→chips{10}, bit 1→chips{01}.

Proposal 2: Confirm Manchester line coding for PRDCH and PDRCH with bit-to-chip mapping of 0.5 code rate: bit 0→chips{10}, bit 1→chips{01}

## Repetition

The performance of block level repetition or TB level repetition by repeating the output of the FEC provides more time diversity and the provides ~1.5dB SNR improvement per repetition. To reduce the complexity and power consumption at the device, it is necessary to reduce the usage of convolutional encoder at the device. Therefore, the repetition can be applied after the FEC to avoid calling FEC multiple times.

Figure 1: PDRCH Transmitter

Observation 1: For PDRCH Repetition before the FEC increases the usage of FEC, thereby increasing the power consumption and complexity.

Proposal 3:  For PDRCH repetition, support block repetition after the FEC

## Coding

The Rel19 WI objective guidance on the line coding includes:

R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis).

The generator polynomials from TS 36.212 can be taken as baseline to compare any potential other generator polynomials for the device 1. Complexity reduction should be a metric to evaluate other generator polynomials against baseline. The number of shift registers depends on the constrained length and the K<6 reduces the number of shift registers thereby reducing complexity. The selection of suitable code rates depends on the overhead, complexity and power consumption.  The number of code rates supported should be restricted to 2 since there is only marginal improvement with decreasing code rate from ½ to ¼ in the TR.

Observation 2: Decreasing code rate provides marginal performance improvement as seen from the results captured in the TR 38.769

Proposal 4: For PDRCH coding, support constrained length of (k<6) and only two code rates ½, ¼  as multiple supported code rates increase the overhead of R2D control signal, complexity/power consumption at the device.

## Device availability/ un-availability

The number of power state supported by device 1 including its functionality should be discussed in RAN1. The RAN1 should further check whether the memory retention power state is supported or not to retain the content in the volatile memory, the threshold to enter the memory retention state and the state transition procedure from ON to memory retention state and from memory retention state to ON/OFF.

Proposal 5: Discuss number of power state supported by device 1 as it important to understand whether content such as timers, inventoried state can be preserved:

- 2 states – ON, OFF

- 3 states – ON, Memory retention state, OFF

The percent of the energy availability for a given capacitor size is important as it determines whether the device 1 can be successfully inventoried or not since the random-access latency due to the slotted aloha is high. Secondly, the supported power state is important as it determines whether the device 1 can receive the initially paging message and enter the memory retention state after selecting a slot for transmission. In the memory retention state, the device 1 can count the slot by running a low frequency clock. This will prolong the availability of the device 1 without unnecessarily monitoring all slots thereby reducing the capacitor size and reducing cost. However, due to high SFO of device 1, the device 1 can drift and need to enter the ON state periodically i.e., access trigger message to perform synchronization and calibration.

The ensure that the device has enough energy availability, the initial inventory request or paging command or access round command contains percent of energy availability metric for the device 1 to participate in an inventory or access round and the device can chose the slot according to its available energy, this increases the chance of device to be inventoried.

Figure 2: Comparison of outage for different scheduling mechanism for Ambient IoT

Proposal 6: Discuss whether the device can goto memory retention state within inventory round to save power

Proposal 7: Discuss how to limit device participating in the inventory round, random access round depending on its availability of stored energy

Proposal 8: Support energy aware slot selection in an inventory round as opposed to random slot selection.

# Conclusion

Below is the summary of proposals and observations from our contribution.

Proposal 1: 3GPP Ambient IoT device type 1 normative work should target better design feature compared to UHF RFID to achieve better coverage:

- The design requirement should take into account, the maximum achievable clock accuracy which is primarily the target to design X-amble, D2R multiple access etc.,

- Maximum achievable clock accuracy after clock calibration for Ambient IoT device type 1 should be kept as 104 PPM for design target during normative phase.

Proposal 2: Confirm Manchester line coding for PRDCH and PDRCH with bit-to-chip mapping of 0.5 code rate: bit 0→chips{10}, bit 1→chips{01}

Observation 1: For PDRCH Repetition before the FEC increases the usage of FEC, thereby increasing the power consumption and complexity.

Proposal 3:  For PDRCH repetition, support block repetition after the FEC

Observation 2: Decreasing code rate provides marginal performance improvement as seen from the results captured in the TR 38.769

Proposal 4: For PDRCH coding, support constrained length of (k<6) and only two code rates ½, ¼  as multiple supported code rates increase the overhead of R2D control signal, complexity/power consumption at the device.

Proposal 5: Discuss number of power state supported by device 1 as it important to understand whether content such as timers, inventoried state can be preserved:

- 2 states – ON, OFF

- 3 states – ON, Memory retention state, OFF

Proposal 6: Discuss whether the device can goto memory retention state within inventory round to save power

Proposal 7: Discuss how to limit device participating in the inventory round, random access round depending on its availability of stored energy

Proposal 8: Support energy aware slot selection in an inventory round as opposed to random slot selection.

# References

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”,   RAN#104, Dec. 2024