3GPP TSG RAN WG1 #120		             			           	R1-2501607

Athens, Greece, February 17th –21st, 2025

Agenda Item: 		9.7.1

Source: 		Moderator (AT&T)

Title: 	 FL Summary #5 on ISAC Scenarios and Calibrations

Document for: 	  Discussion and Decision

# Introduction

This document provides a summary of the discussion of the ISAC channel model deployment scenarios and calibrations, which is within the objectives in the Rel-19 recently approved SID (subsequently revised most recently at RAN#105) on the “Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR” [1] as per the objectives shown below:

It should be noted from RAN#105, the ISAC study checkpoint concluded with the ISAC study item scope will only include channel modelling aspects.

At RAN1#118-bis and RAN1#119, some good progress was made with the following agreements endorsed in the ISAC session report [2] regarding specific evaluation parameters for the Automotive, Human, AGV and Objects creating hazards sensing targets in addition to the already agreed UAV sensing scenarios. However, to further the work and enable channel model calibrations, initial proposals for channel model calibration assumptions, parameters, and values should be discussed. On the last point, an email discussion on simulation assumptions for channel model calibration, from January 8th-17th invited companies to provide initial channel model calibration assumptions, parameters, and values. Inputs were collected and the discussions are to be carried over to RAN1#120. Additionally, the following agreements were made at the last meeting:

Agreement

For UAV sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118 as a baseline:

The detailed scenario description in this clause can be used for channel model calibration.

ISAC-UAV

Details on ISAC-UAV scenarios are listed in Table x.

Table x. Evaluation parameters for UAV sensing scenarios

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios |  | UMi, UMa, RMa [38.901] UMi-AV, UMa-AV, RMa-AV |
| Sensing transmitters and receivers properties | Rx/Tx Locations | Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenarios. NOTE1: This may include aerial UEs for UMi-AV, UMa-AV, RMa-AV communication scenarios. In this case, other Rx/Tx properties (e.g. mobility) are also taken from the corresponding communication scenario. |
| Sensing target | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Outdoor |
|  | 3D mobility | Horizontal velocity: uniform distribution between 0 and 180km/h, if horizontal velocity is not fixed to 0. Vertical velocity: 0km/h, optional {20, 40} km/h NOTE2: 3D mobility can be horizontal only or vertical only or a combination for each sensing target FFS: time-varying velocity. NOTE 3: time-varying velocity may be considered for future evaluations. |
|  | 3D distribution | Horizontal plane: Option A: N targets uniformly distributed within one cell. Option B: N targets uniformly distributed per cell. Option C: N targets uniformly distributed within an area not necessarily determined by cell boundaries. FFS: Value of N, defined area, and other distributions N = {1, 2, 3, 4, 5} NOTE4: N=0 may be considered for the evaluation of false alarm Vertical plane: Option A: Uniform between 1.5m and 300m. Option B: Fixed height value chosen from {25, 50, 100, 200, 300} m assuming vertical velocity is equal to 0. FFS Other options are not precluded. NOTE5: target(s) are outside the minimum distance to the Tx/Rx |
|  | Orientation | Random in horizontal domain |
|  | Physical characteristics (e.g., size) | Size: - Option 1: 1.6m x 1.5m x 0.7m - Option 2: 0.3m x 0.4m x 0.2m FFS: Material(s), Structure, Other size(s) |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option B: Min distances based on min. TRP/UE distances defined in TR36.777 as a starting point. NOTE5: the sensing target is assumed in the far field of sensing Tx/Rx Option C: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a target Option 2: 10 meters |
| [Unintended/Environment objects, e.g., types, characteristics, mobility, distribution, etc.] |  | FFS |

NOTE: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

Agreement

For Automotive sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline:

The detailed scenario description in this clause can be used for channel model calibration.

ISAC-Automotive

Details on ISAC-Automotive scenarios are listed in Table x.

Table x. Evaluation parameters for Automotive sensing scenarios

| Parameters |  | Values |
| --- | --- | --- |
| Applicable communication scenarios |  | Highway, Urban Grid. NOTE1 |
| Sensing transmitters and receivers properties |  | Rx/Tx locations are selected among the TRPs and UEs (e.g., VRU, vehicle, RSU-type UEs) locations in the corresponding communication scenario. NOTE2 FFS: Additional option: ISD between TRPs of Urban Grid is 250m |
| Sensing target | LOS/NLOS | LOS and NLOS (including NLOSv) |
|  | Outdoor/indoor | Outdoor |
|  | Mobility (horizontal plane only) | Based on TR37.885 mobility for urban grid or highway scenario |
|  | Distribution (horizontal) | Based on dropping in TR37.885 per urban grid or highway communication scenario |
|  | Orientation | Lane direction in horizontal plane |
|  | Physical characteristics (e.g., size) | Type 1/2 (passenger vehicle) Type 3 (truck/bus) Vehicle type distribution per TR 37.885 as a starting point FFS: Other sizes, additional distributions, and vehicle types, e.g. one new type of e-scooter/motorcycle/bike |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option 1: Min distances based on min. TRP/UE distances defined in TR37.885 as a starting point. Option 2: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx NOTE3: the sensing target is assumed in the far field of sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [10] m. value of x is FFS |
| Environment Objects, e.g., types, characteristics, mobility, distribution, etc. |  | EO Type 2 for Urban Grid - FFS: details, e.g. up to 4 walls modelled as EO type 2, per building of size [413m x 230m x 20m]. FFS: number of buildings, how many walls are modelled, additional building sizes, etc. |

NOTE1: calibration for UMi, Uma, RMa is not performed for the automotive scenario, but UMi, Uma, RMa can be considered for future evaluations of the automotive sensing target scenarios. Calibration for UMi, Uma, RMa is expected to be performed for another sensing scenario.

NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

Agreement

For Human (indoor and outdoor) sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline:

The detailed scenario description in this clause can be used for channel model calibration.

ISAC-Human

Details on ISAC-Human scenarios are listed in Table x.

Table x. Evaluation parameters for Human (indoor and outdoor) sensing scenarios

| Parameters |  | Indoor Values | Outdoor Values |
| --- | --- | --- | --- |
| Applicable communication scenarios NOTE1 |  | Indoor office, indoor factory [TR38.901] Indoor room [TR38.808] | UMi, Uma, RMa [TR38.901] |
| Sensing transmitters and receivers properties | Rx/Tx Locations NOTE 2 | Rx/Tx locations are selected among the TRPs and UE locations in the corresponding communication scenario | Rx/Tx locations are selected among the TRPs and UE locations in the corresponding communication scenario |
|  | Rx/Tx Mobility for UEs | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 3km/hr | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 10km/hr |
| Sensing target | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | Outdoor/indoor | Indoor | Outdoor |
|  | 3D mobility | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 3km/hr (horizontal plane with random direction straight-line trajectory) | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 10km/hr (horizontal plane with random direction straight-line trajectory) |
|  | 3D distribution | N targets uniformly distributed over the horizontal area of the convex hull of the TRP deployment FFS: Value of N NOTE1: N=0 may be considered for the evaluation of false alarm | Option A: N targets uniformly distributed within one cell. Option B: N targets uniformly distributed per cell. Option C: N targets uniformly distributed within an area not necessarily determined by cell boundaries. Uniform in horizontal plane NOTE1: N=0 may be considered for the evaluation of false alarm |
|  | Orientation | Random over the horizontal area | Random over the horizontal area |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m | Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx Option 2: Min distances defined in TR 38.901 and TR36.843 and TR38.859as a starting point NOTE2: the sensing target is assumed in the far field of sensing Tx/Rx | Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx Option 2: Min distances defined in TR 38.901 and TR36.843 and TR38.859 as a starting point NOTE3: the sensing target is assumed in the far field of sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [x] m. value of x is FFS | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [x] m. value of x is FFS |
| Environment Objects, e.g., types, characteristics, mobility, distribution, etc. |  | FFS, based on outcome for AI 9.7.2 | FFS, based on outcome for AI 9.7.2 |

NOTE1: For the human (indoor and outdoor) sensing targets, additional communication scenarios can be considered for future evaluations. Channel model calibration for Urban Grid with outdoor humans is expected to be performed from Objects creating hazards on the road/railway sensing scenarios.

NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

Agreement

For AGV sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline:

The detailed scenario description in this clause can be used for channel model calibration.

ISAC-AGV

Details on ISAC-AGV are listed in Table x.

Table x. Evaluation parameters for Automated Guided Vehicles

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios NOTE1 |  | InF (TR38.901 including Table 7.8-7) |
| Sensing transmitters and receivers properties NOTE2 |  | Rx/Tx location are selected among the TRPs and UEs location in the corresponding communication scenario Rx/Tx Mobility for UEs - Option 1: 0 km/h - Option 2: 3km/h - Option 3: Uniform distribution between 0km/h and 3km/h |
| Sensing target | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Indoor |
|  | 3D mobility | Horizontal velocity with random straight-line trajectory - Option 1: Uniform distribution in the range of up to 30 km/h - Option 2: Fixed velocities [3, 10] km/h |
|  | 3D distribution | Option A: Uniformly distributed in the convex hull of the horizontal BS deployment Option B: Uniformly distributed in horizontal plane |
|  | Orientation | Horizontal plane only |
|  | Physical characteristics (e.g., size) | Size (L x W x H) - Option 1: 0.5m x 1.0m x 0.5m - Option 2: 1.5 m x 3.0m x 1.5 m - FFS: Material, Additional sizes, and AGV size distribution |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx from the sensing target Option 2: Min distances based on min. TRP/UE distances defined in TR38.901 NOTE: the sensing target is assumed in the far field of sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option A: At least larger than the physical size of a target Option B: Fixed value, [x] m. value of x is FFS |
| Environment objects, e.g., types, characteristics, mobility, distribution, etc. |  | FFS |

NOTE1: For the AGV sensing targets, additional communication scenarios can be considered for future evaluations.

NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

NOTE3: RAN1 can further discuss narrowing down the number of sub-scenarios of InF

Agreement

For Objects creating hazards sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline:

The detailed scenario description in this clause can be used for channel model calibration.

ISAC-Hazards

Details on ISAC-Hazards are listed in Table x.

Table x. Evaluation parameters for objects creating hazards

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios NOTE1 |  | Highway, Urban grid, HST (High Speed Train) |
| Sensing transmitters and receivers properties NOTE2 | Rx/Tx Locations | Rx/Tx locations are selected among the TRPs and UEs (e.g., VRU, vehicle, RSU-type UEs) locations in the corresponding communication scenarios. FFS: Option 2 Additional option ISD between TRPs of Urban Grid is 250 m |
| Sensing target | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Outdoor |
|  | 3D mobility | Horizontal velocity: up to [10] km/h for humans and animals FFS: Additional velocities, trajectory |
|  | 3D distribution | Uniformly distributed in horizontal plane |
|  | Orientation | Random distribution in horizontal plane |
|  | Physical characteristics (e.g., size) | For human/pedestrians: Child: 0.2m x 0.3m x 1m Adult: 0.5m x 0.5m x 1.75m For animals: Size: 1.5m x 0.5m x 1 m FFS: other types/sizes of targets may be considered for future evaluations |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx from the sensing target Option 2: based on min TRP/UE distances defined in TR37.885 and TR38.802 and TR36.843 and TR38.859 NOTE: the sensing target is assumed in the far field of sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [10] m. value of x is FFS |
| Environment objects, e.g., types, characteristics, mobility, distribution, etc. |  | EO Type 2 for Urban Grid - FFS: details, e.g. up to 4 walls modelled as EO type 2, per building of size [413m x 230m x 20m]. FFS: number of buildings, how many walls are modelled, additional building sizes, etc. |

NOTE1: For the objects creating hazards sensing targets, additional communication scenarios can be considered for future evaluations.

NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

## Moderator Tentative Plan

For RAN#120, RAN1 should take into account the above agreements as a starting point and further progress the ISAC deployment scenarios following proposed plan and start to define the channel model calibration assumptions:

- At RAN1#119, the chair guidance for further work included the following:

1. Rapporteurs are encouraged to start providing a draft CR for both agendas to RAN1#120

2. Jerome to provide an initial proposal for calibrations discussions by the end of RAN1#119

3. RAN1 agenda will clarify that input on calibrations discussions is to be provided to agenda 9.7.1 starting at RAN1#120

- Clean up ISAC deployment scenarios as necessary for sensing targets including UAV, automotive, humans, AGV, objects creating hazards, using the agreed tables as a starting point in order to get the tables to consensus and void of FFSs/square brackets. However, some options are acceptable given a wider range of values may be necessary for full ISAC evaluations. These options may be downselected when companies begin to evaluate the ISAC channel model.

- Channel model calibration parameters and values/ranges need to be further discussed based on the January 2025 email discussion [3]. Discuss and agree further channel model calibration parameters/values.

  - Logistics for simulation assumptions for channel model calibration to be defined

  - Logistics for running CR

- Others and next steps

Please note that in this FL summary, a FL proposal may be designated as (H)(M)(L) to indicate its high, medium or low priority for online or offline discussions in this meeting. Nevertheless, we encourage all interested companies to provide feedback on all FL proposals. The FL may revise the priority of the proposals based on inputs from interested companies during the meeting, if it is deemed necessary.

Note that additional proposals are expected to be developed as online and offline discussions progress.

## Proposals for online & offline discussion/agreement (Round 1) – NOT TREATED see Round 2

Proposal 3-1:  (HIGH)

      - For ISAC channel modelling calibration, RAN1 considers both large-scale and full-scale calibration to include parameters and values for at least the following: large scale parameters, where fast fading is not included and full calibration, which includes fast fading.

- NOTE1: Additional calibrations for spatial consistency should also be considered as a 2nd priority.

- NOTE2: It is assumed that ISAC calibrations can not be directly inferred from the communications modelling and will be based on further progress in 9.7.2.

Proposal 3-2:  (HIGH)

      - RAN1 considers calibration methodology to separate calibration of the target channel and the background channel for mono-static (if necessary), as well as combined channel (combination of target and background channel).

Proposal 3-3:  (HIGH)

      - For ISAC calibrations, RAN1 does not limit nor explicitly prioritize any sensing targets, deployment scenarios, or sensing modes.

Proposal 3-4:  (HIGH)

      - Calibration metrics should at least include the following:

- For large scale calibration: coupling loss

- For full calibration: coupling loss, delay spread and angle spread

- Additional calibration metrics are not precluded.

Proposal 3-5:  (HIGH)

      - Interested companies are requested to provide large-scale and full-scale calibration results starting after RAN1#120.

Proposal 4-1:  (MED) The following updates are proposed to the current baseline agreed sensing target deployment scenario tables from RAN1#119:

      - Remove the row for “Environment Objects, e.g., types, characteristics, mobility, distribution, etc.” in all sensing target scenario tables and create separate tables for EO and their associated characteristics.

        - As a starting point, EO table as follows (green denotes previous RAN1 agreement):

Table x. EO Characteristics for ISAC scenarios

| EO | Type 1/ Type 2 | Applicable Scenario(s) | Size(s) | Material | Distribution | Mobility |
| --- | --- | --- | --- | --- | --- | --- |
| Building Type A | Type 2 | Automotive/Hazards - Urban Grid | 413m x 230m x 20m | Concrete, glass. Up to 4 walls | Along street grid. FFS number of buildings | N/A |
| Building Type B | Type 2 | Automotive/Hazards - Highway | 10-30m x 10-30m x 3-10m | Concrete, wood, glass. Up to 4 walls. | Along highway | N/A |

NOTE: Additional EOs can be modeled for further ISAC evaluations.

Proposal 4-2:  (HIGH)

    - RAN1 agrees that EOs are not considered for ISAC channel model calibrations in Rel-19.

Proposal 4-3:  (MED)

If multiple targets are modelled, for the Minimum 3D distance between sensing targets, RAN1 considers Option A: “At least larger than the physical size of a target” for all sensing target scenarios.

## Proposals for Tuesday offline discussion (Round 2)

Proposal 3-1:  (HIGH)

      - For ISAC channel modelling calibration, RAN1 considers both large-scale and full-scale calibration to include parameters and values for at least the following: large scale parameters, where fast fading is not included and full calibration, which includes fast fading.

- NOTE1: Additional calibrations for spatial consistency should also be considered.

- NOTE2: It is assumed that ISAC calibrations can not be directly inferred from the communications modelling and will be based on further progress in 9.7.2.

Option A: Large Scale calibrations are considered. Vivo, ZTE, AT&T, HW, Nokia, Sony, LGE, BUPT, Continental, Sharp, NIST, QC, Lenovo, Spreadtrum (14)

Option B: Large Scale calibrations are not considered: OPPO, Ericsson, MTK, SAM (4)

Proposal 3-2:  (HIGH)

      - RAN1 considers calibration methodology to separate calibration of the target channel and the background channel , and, if necessary combined channel (combination of target and background channel).

      - NOTE: The assumption is that background channel calibration is necessary only if we introduce something new

      - FFS: Details of calibrations for target channel.

Proposal 3-3:  (HIGH)

      - For ISAC calibrations, RAN1 does not limit nor explicitly prioritize any sensing targets, deployment scenarios, or sensing modes.

Proposal 3-4:  (HIGH)

      - Calibration metrics should at least include the following:

- For large scale calibration: coupling loss (target channel, PL Tx/Rx-Target, shadow fading, A…)

- For full calibration: coupling loss, delay spread, and angle spread

- Additional calibration metrics are not precluded.

Proposal 3-5:  (HIGH)

      - Interested companies are requested to provide large-scale and full-scale calibration results starting after RAN1#120.

Proposal 4-1:  (MED) The following updates are proposed to the current baseline agreed sensing target deployment scenario tables from RAN1#119:

      - Remove the row for “Environment Objects, e.g., types, characteristics, mobility, distribution, etc.” in all sensing target scenario tables and create separate tables for EO and their associated characteristics.

        - As a starting point, EO table as follows (green denotes previous RAN1 agreement):

Table x. EO Characteristics for ISAC scenarios

| EO | Type 1/ Type 2 | Applicable Scenario(s) | Size(s) | Material | Distribution | Mobility |
| --- | --- | --- | --- | --- | --- | --- |
| Building Type A | Type 2 | Automotive/Hazards - Urban Grid | 413m x 230m x 20m | Concrete, glass. Up to 4 walls | Along street grid. FFS number of buildings | N/A |
| Building Type B | Type 2 | Automotive/Hazards - Highway | 10-30m x 10-30m x 3-10m | Concrete, wood, glass. Up to 4 walls. | Along highway | N/A |

NOTE: Additional EOs can be modeled for further ISAC evaluations.

Proposal 4-2:  (HIGH)

    - Inclusion of EO type 1 and/or EO type 2 in CM calibrations is optional

Proposal 4-3:  (MED)

If multiple targets are modelled, for the Minimum 3D distance between sensing targets, RAN1 considers Option A: “At least larger than the physical size of a target” for all sensing target scenarios.

## RAN1#120 Summary, Open Issues, and Next Steps

### Introduction

From the moderator perspective and outcome/progress from RAN1#120, the following summary proposes a way forward post-RAN1#120 for topics associated with agenda item 9.7.1, including deployment scenarios and calibration assumptions.

As a summary, the following agreements were made at RAN1#120, which are taken from the draft chair notes and the calibration summary table (version without change bars):

Agreement

For ISAC channel modelling calibration, RAN1 considers both large-scale and full-scale calibration to include parameters and values for at least the following:

    - large scale parameters, where fast fading is not included

    - full-scale calibration parameters, which includes fast fading.

- NOTE0: one part of calibration work does not include additional components and does not include spatial consistency

    - FFS: whether spatial consistency is specified as an additional component for ISAC CM

- NOTE1: additional calibrations including spatial consistency can also be considered case by case for different scenarios.

- NOTE2: Inclusion of EO in ISAC CM calibrations can also be considered case by case for different scenarios.

Agreement

Calibration of ISAC CM includes separate calibration of the target channel and of the background channel

- FFS: additional calibration for the combined channel (combination of target and background channel).

Agreement

For the purposes of large scale calibration for UAV sensing targets, the following calibration parameters are proposed below in Table x.

Table x. Simulation assumptions for large scale calibration for UAV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | UMa-AV |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic Note: further down-selection of the sensing modes for UAV sensing is not precluded |
| Sectorization | 3 sectors per cell site: 30, 150 and 270 degrees |
| Carrier Frequency | FR1: 6 GHz FR2: 30 GHz |
| BS antenna configurations | Single dual-pol isotropic antenna |
| BS Tx power | FR1: 56dBm FR2: 41dBm |
| Bandwidth | FR1: 100MHz FR2: 400MHz |
| BS noise figure | FR1: 5dB FR2: 7dB |
| UT antenna configurations | (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| UT noise figure | FR1: 9dB FR2: 10dB |
| Sensing target distribution | 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m. |
| Component A of the RCS for each scattering point | a fixed value of A |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | 10 m |
| Wrapping Method | No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping |
| Metrics | Coupling loss (based on LOS pathloss) - FFS: how to select sensing Tx and Rx FFS: additional metrics, wideband SIR and SINR based on RSRP if interference is modelled. |

### Open Issues

From the baseline large scale calibration agreements for UAV sensing targets, there are several issues remaining including, e.g., the methodology for selecting sensing Tx and Rx which is to be used for associated coupling loss, any additional metrics, full calibration parameters, and calibration for additional sensing targets.

For the FFS for coupling loss, the ambiguity is in regard to which TRP(s) should be selected for the Sensing Tx and Rx in the TRP monostatic sensing and pairs of TRPs for bistatic sensing modes. In the moderator opinion, at least companies should provide details for TRP selection, if supplying calibration simulation results. Furthermore, it is suggested that companies align the sensing TRP selection criteria (e.g., based on coupling loss) and N number of TRPs (N number of TRPs could be equal to 4 for both bistatic and monostatic sensing).

The following is proposed: For sensing TRP selection: At least companies should provide details for TRP selection. One recommended methodology is the Tx-Rx pair with the largest large-scale RSRP. Note the Tx and Rx should not belong the same site for bistatic mode. Best [N] TRPs to be selected. Value of N is [4].

One additional FFS is on additional metrics. Companies are requested to at least provide coupling loss for large scale calibrations (based on LOS pathloss Sensing Tx/Rx-Target for the target channel) and also coupling loss for the background channel. If interference is modelled (e.g., from TRPs in the cell layout) wideband SIR and SINR based on RSRP may be included, or perhaps some other metrics.

Furthermore, there was an attempt to discuss fast fading and full calibration assumptions and metrics. This requires selection of a fast fading channel model based on TR 36.777. There are 3 possible options in 36.777 and RAN1 will need to downselect one of these options. At a minimum, companies should converge to select on model from TR36.777 for fast fading.

Additionally, companies should downselect Target/Rx/Tx mobility and distribution options from the UAV deployment scenarios agreements in previous meetings. These details can be discussed within companies and provided in RAN1#120-bis. The metrics for full calibration should also include (at a minimum) coupling loss, CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) for full calibration, and perhaps additional metrics, which are FFS.

RAN1 should also further address the other sensing target calibration assumptions.  For other UAV UE-based sensing modes, additional parameters may be necessary for UE specific calibration parameters.

# Contact Information:

See sub-folder in the inbox for AI 9.7 for list of contacts for ISAC topics.

# Topic #1: General Calibration proposals

Some of the related proposals are copied below.

| Company | Views/Proposals |
| --- | --- |
| [3] Moderator AT&T | Based on the inputs and discussion in R1-2500059 [3] Email discussion summary on ISAC CM calibration assumptions, there were a number of inputs resulting in the following summary: Topic 4-1: The following high-level channel model calibration proposals have been discussed during [Post-119-ISAC-01] to progress the ISAC channel modelling: - RAN1 should discuss and down-select between two general options for the methodology for channel model calibration: - OPTION 1: Similar to the stochastic communication channel model calibration approach in TR 38.901 for ISAC channel modelling, which includes parameters and tables for at least the following: large scale parameters, where fast fading is not included and full calibration, which includes fast fading (attached tables for some details). - OPTION 2: A step-wise approach for the ISAC channel model calibration simulations which consists of deterministic models for the Tx/Rx-target channels, background channels and combination of the target/background channels, e.g., CDL models limited to channel metrics based on received power and multipath/delay/angular spread. - NOTE1: Companies are encouraged to provide justification and proposals for either Option 1 or Option 2. - All 5 sensing target types identified in the SID should be represented in new ISAC channel model calibration clause(s). Note: This does not preclude including calibration assumptions for more than one sensing target into the same table(s). - TR 38.901 Running CR details for how and where to capture CM calibration are to be determined at RAN1#120, e.g., new ISAC-specific sub-sections, annex, reference, etc. - Additional guiding principles for defining calibration assumptions to be further discussed: - Calibration assumptions should consider only parameters necessary to align implementations of the ISAC channel model, including target channel and background channel. - CM Calibration should allow for using a single Tx and a single Rx - Should consider implementations of the RCS and polarization models for the various sensing targets - With limited TUs, no explicit prioritization for the different sensing modes/scenarios is required, but can be implied based on company inputs, or lack thereof. - Interested companies to continue to provide updates in the moderator supplied templates used in the [Post-119-ISAC-01] email discussion if the conventional approach is preferred, or separate tables if a deterministic approach to CM calibration is preferred. - Details on how to define calibration metrics are FFS, but some initial metrics can be included in the xls templates and companies are encouraged to provide further inputs to progress these aspects. - RAN1 to converge on parameters/values and related options if applicable, at RAN1#120. Topic 4-2: Interested companies provided some specific ISAC simulation assumptions/parameters, including values for the various sensing targets for large scale parameters and full calibration. - Attached spreadsheets may be used as a basis for additional tdoc submissions, consolidation, and alignment at RAN1#120 if the consensus is to move forward with the conventional stochastic approach. - The majority of companies responding to the [Post-119-ISAC-01] email discussion is supportive to calibrate spatial consistency. Details of the parameters for spatial consistency are FFS and should be further discussed at RAN1#120. |
| [4] Xiaomi, AT&T | Based on the discussion at RAN1#119 [2], a draft CR should be started at RAN1#120. R1-2500998 Draft CR for TR 38.901 to introduce channel model for ISAC Xiaomi, AT&T. |
| [5] Huawei | Proposal 1: System level simulation is essential for ISAC channel model calibration. Proposal 2: The carrier frequency of 7GHz is important for ISAC channel model calibration. Proposal 3: The BS Tx power in TR38.901 as the baseline is scaled per the bandwidth. For instance, the BS Tx power for 400MHz bandwidth after scaling is 49 + 10*log10(400MHz/20MHz), i.e., 62dBm. Proposal 5: For large scale calibration, coupling loss and the geometry are the metrics to be calibrated, where the coupling loss = antenna_pattern_gain + pathloss + [RCS component A]; the geometry is the SIR SIR =S_(serving cell)/(I_neighbor+I_(serving cell) ) where the signal S is the coupling loss from the serving cell for a sensing target paired with a Tx and a Rx; the interference I is the coupling loss from the neighboring BS and from other sensing target paired with a Tx and a Rx from the serving cell. Proposal 6: For full calibration, coupling loss, the geometry, and the delay spread and the Angle Spread (ASD, ZSD, ASA, ZSA) are the metrics to be calibrated, where the coupling loss = antenna_pattern_gain + pathloss + received power of all the paths with RCS; the geometry is the SIR SIR =S_(serving cell)/(I_neighbor+I_(serving cell) ) where the signal S is defined as the received power of all the paths transmitted from Tx pass though the sensing target then received by the Rx; the interference I counts interference from both neighboring BS and the serving cell BS. Proposal 7: Agree a set of parameter values based on the input during the email discussion summarized in R1-2500059 at RAN1#120 meeting, to leave sufficient time for calibration. |
| [6] CATT | Proposal 7: For calibration, support merging Option 1 and Option 2, and thus the following are calibrated: - Large scale parameters of target channel, background channel and combined channel, respectively; - Full calibration of target channel, background channel and combined channel, respectively. Proposal 8: For calibration, calibrating one target of one sensing target type in one simulation is the baseline, except for spatial consistency if calibrated. - Calibration of multiple targets of one sensing target type in one simulation can be done voluntarily; - Calibration of multiple targets of multiple sensing target types in one simulation is not pursued. Proposal 9: For calibration, - Support CM Calibration using a single Tx node and a single Rx node; - Support the following calibration metric for large scale parameters, - Coupling loss – serving cell (based on LOS pathloss); - Support the following two calibration metrics for full calibration, - CDF of coupling loss, - CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) according to circular angle spread definition of TR 25.996. |
| [7] China Telecom | Proposal 1: Both large-scale and full-scale calibration are needed. Proposal 2: Small scale RCS parameters for scattering points of targets are needed for full-scale calibration. Proposal 3: The sensing modes for each scenario in channel model calibration needs to be aligned first. It’s not preferred to calibrate each mode for each scenario. Proposal 4: Support to include UT related parameters such as UT height and UT Tx power if UE-based sensing mode is selected for calibration. Proposal 5: It should be reported which concatenation method of Tx-target and target-Rx link is used in channel model calibration. |
| [8] CMCC | Proposal 1: Support Option 1 as methodology for channel model calibration: • OPTION 1: Similar to the stochastic communication channel model calibration approach in TR 38.901 for ISAC channel modelling, which includes parameters and tables for at least the following: large scale parameters, where fast fading is not included and full calibration, which includes fast fading (attached tables for some details). |
| [9] CALTTA | Proposal 1: For ISAC channel model calibration, Option 1 is supported. - Similar to the stochastic communication channel model calibration approach in TR 38.901 for ISAC channel modelling, which includes parameters and tables for at least the following: large scale parameters, where fast fading is not included and full calibration, which includes fast fading. Proposal 2: The most interested sensing scenarios with the sensing targets are supported to calibrate, in order to reduce the workload. - At least the sensing targets of UAV and vehicles should be calibrated. |
| [10] vivo | Proposal 1: RAN1 uses Option 1 approach for channel model calibration. Proposal 2: RAN1 calibrates the target channel and combined channel (combination of target channel and background channel). Proposal 3: RAN1 study how to determine the serving transmitter-target-receiver link(s) for calibration. The following options can be considered as start point: - Option 1: The link with best coupling loss or RSRP is considered as the serving link. - Option 2: All the links in the simulation map are considered as the serving link. - Option 3: Partial link(s) in some specific area as the serving link(s). - For monostatic sensing mode, the specific area can be the cell where the sensing TRP or the sensing UE is. - For the TRP-UE and UE-TRP bistatic sensing mode, the specific area can be the serving cell of UE. - For the TRP-TRP and UE-UE bistatic sensing mode, the specific area can be the two cells of the two sensing TRP and the cell(s) where the two UEs are attached. Proposal 4: The calibration metrics for large scale are coupling loss of target channel and wideband SIR/SINR of combined channel. Proposal 5: The wideband SIR/SINR of combined channel can be defined as following: - The received signal power S is defined as received power summation at sensing receiver of the signal across the sensing target from serving sensing transmitter; - The received interference power I is defined as the summation of I1 and I2, where I1 is the received power summation at sensing receiver of the signal not across the sensing target from serving sensing transmitter, and I2, is the received power summation at sensing receiver from the other signal/data transmitters; - The SIR equals to S/I1+I2 and SINR equals to S/I1+I2+N0. Proposal 6: The calibration metrics for full calibration are: - Coupling loss based on large-scale and small-scale channel for target channel. - Wideband SIR/SINR. - Delay spread and Angle Spread (ASD, ZSD, ASA, ZSA) for both target channel and combined channel. - The largest PRB singular values, the smallest PRB singular values and the ratio between the largest PRB singular value and the smallest PRB singular value for both target channel and combined channel. Proposal 7: The calibration metrics for spatial consistency are: - Coupling loss on small-scaling channel of target channel; - Wideband SINR before receiver of combined channel; - Cross-correlation coefficient of delay in the target channel for the third cluster between paired links; - Cross-correlation coefficient of AOA in the target channel for the third cluster between paired links; - Cross-correlation coefficient of LOS/NLOS status in the target channel between paired links; - Cross correlation coefficient of the target channel response on the first non-DC subcarrier in an OFDM symbol on antenna port 0 received on the first UE antenna over multiple realizations between paired links. Proposal 8: RAN1 determines fixed RCS values for each sensing target for the channel model calibration. Proposal 9: RAN1 does not limit the deployment scenarios for calibration, and it is up to company inputs. Proposal 10: RAN1 does not limit the sensing mode, and it is up to company inputs. Proposal 11: For outdoor deployment scenarios, N=1~5 targets uniformly distributed per cell. For indoor deployment scenarios, N=12~60 targets uniformly distributed over the horizontal area of the convex hull of the TRP deployment. Proposal 12: Full convolution is used as a baseline for ISAC channel calibration. Proposal 13: Whether or how to do power normalization should be determined for ISAC channel calibration if ray level 1-by-1 random coupling convolution between Tx-target link and target-Rx link is used. Proposal 14: Both two scenarios are selected for spatial consistency calibration of ISAC channel model: - Single target related to multi-receiver - Multi-target related to single receiver |
| [13] OPPO | Proposal 2: The target channel and background channel should be calibrated at least separately. Proposal 3: For ISAC channel model calibration, use the calibration approach similar to the one defined in TR 38.901 for the stochastic communication channel model. Proposal 6: The calibration metrics include: - CDF’s of ray-level power, delay and angles - Delay-to-power profile and angle-to-power profile - SNR/SINR that may matter to sensing instead of communication |
| [15] ZTE | Proposal 1: Support both large scale calibration and full calibration for ISAC channel model calibration, similar to the stochastic communication channel model calibration approach in TR 38.901 Proposal 2: All 5 sensing target types should be calibrated, with the following considerations: - BS antenna configuration: (M, N, P, Mg, Ng; Mp, Np) = (16, 16, 1, 1, 1, 1, 1) - Antenna virtualization: analog beam towards sensing target - Calibration metrics: - Large scale calibration: coupling loss - Full calibration: CDF of coupling loss and CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) according to circular angle spread definition of TR 25.996 - The power Pn,m should take into account RCS and CPMtx,sp,rx impact - New parameters: - Small scale RCS - XPR of sensing target - Selection of Tx-Rx pair: select a Tx-Rx pair with the largest large-scale RSRP for each sensing target Proposal 3: Support calibration parameters in Table 1, 2, 3, 4, 5 for sensing scenarios for different sensing targets |
| [16] Sony | Proposal 1: Support ISAC Channel Model Calibration Methodology Option 1 (Similar to the stochastic communication channel model calibration approach in TR 38.901 for ISAC channel modelling). Proposal 2: ISAC channel model calibration to include at least large scale and full calibration calibrations. Other calibrations (e.g., spatial consistency, RCS modelling) can be considered. Proposal 3: Re-use the calibration metrics that have been used in the existing TR 38.901 (e.g., coupling loss, geometry with/wo noise, etc) and apply to all ISAC scenarios. FFS other metrics. Proposal 4: Consider CM calibration parameters/assumptions for UAV, automotive vehicle (highway, urban) and AGV for large scale and full calibration as listed in the separate xls file. |
| [17] Nokia | Proposal 6: Discuss and agree on the performance metrics for calibration of the ISAC channel modeling and the timeline for the calibration. Before doing the full-scale calibration with the small-scale fading, we also feel that we need to calibrate the target channel, background channel, and RCS separately in a link-level setup before integrating them with the simulations for full-scale calibration. We may start with the large-scale calibration results discussion in RAN1- #120bis and the full-scale results discussion in RAN1-#121 |
| [18] SK Telecom | Proposal 1. it is preferable to consider channel model calibration using Option 2's step-wise approach. Proposal 2. It is recommended to use the similar assumptions for large scale calibration as shown in Table 7.8-1 of TR 38.901. Proposal 3. As shown in Table 1, it is suggested that interested companies check one of the scenarios in TR 38.901 corresponding to each of ISAC target and perform channel model calibration for only one agreed representative scenario for initial stage. Table 1. CM Calibration Table between ISAC Target and TR 38.901 TR 38.901 Scenarios; ISAC Targets. 1. UAM; 2. Human; 3. AGV; 4. Automotive; 5. Hazard. a. Urban Micro – street canyon. b. Urban Macro. c. Indoor office. d. Rural Macro. e. Indoor Factory. |
| [19] NVIDIA | Observation 1: Wireless channel modelling needs to provide consistency and, above all, a correct representation of the frequency, spatial, and temporal correlation across base stations, devices, and objects in the environment. Observation 2: ISAC evaluation without a propagation model grounded on the underlying physics of the scattering phenomena is simply unnatural, prone to modelling error and possibly a huge waste of effort for the industry. Observation 3: Deterministic, physics-based modelling for wireless propagation, especially ray tracing, are essential for ISAC evaluation. Observation 4: Task Group IEEE 802.11bf has embraced ray tracing based channel model for WiFi sensing. Observation 5: Though ray tracing has been considered in the map-based hybrid channel model in TR 38.901, the map-based hybrid model defined is not calibrated and has not been used in 3GPP simulation campaigns. Observation 6: A common reference scenario defined for ray tracing not only can be used for ISAC evaluation but also other key technologies toward 6G, including reconfigurable intelligent surface (RIS), larger antenna arrays in new spectrum such as 7-24 GHz and sub-THz bands, AI/ML, to name a few. Based on the discussion in the previous sections we propose the following: Proposal 1: Define a common reference scenario for ray tracing to be used in ISAC evaluation. Proposal 2: Select one the following option to define a common reference scenario for ray tracing to be used in ISAC evaluation: • Option 1: Real-scenario map that is a virtual representation of a real area on earth. • Option 2: Synthetic-scenario map that is artificially constructed to mimic a certain environment such as urban macro, rural macro, indoor office, or indoor factory. Proposal 3: Describe the scene geometry and the characteristics of the materials involved in the ISAC deployment scenarios to be used for ray tracing in ISAC evaluation. |
| [20] Xiaomi | Proposal 4: It is up to each company to select and report interested scenarios for calibrations. RAN1 will collect results only for a scenario if sufficient number of companies run the calibration. Proposal 5: For the calibration of the ISAC channel modeling, consider at least: - Large-scale and full calibration with start directly after RAN1 #120. - Spatial consistency calibration. Proposal 6: For a potential sensing Tx, the potential sensing Rx are defined as follows. - for TRP-TRP bistatic modes, a TRP chosen from other sites can be considered as potential sensing Rx; - for UE-UE bistatic modes, any other UE can be considered as potential sensing Rx; - for TRP-UE bistatic mode, any UE can be considered as potential sensing Rx; - for UE-TRP bistatic mode, any TRP can be considered as potential sensing Rx; - for TRP monostatic and UE monostatic modes, sensing Rx is the same entity as sensing Tx. Proposal 7: For a specific sensing target, only the one pair of (sensing Tx, sensing Rx) should be considered that has the highest large-scale RSRP of the target channel. Proposal 8: Performance metrics for calibration should be determined for sensing, and the metrics in TR 38.901 can be considered as the starting point. - Large-scale calibration: Coupling loss, SINR, and SIR. - Full calibration: Coupling loss, SIR, CDF of Delay Spread and Angle Spread. - Spatial consistency calibration: Coupling loss, SINR, Cross-correlation coefficient of delay/AOA/LOS-NLOS status/channel response, CDF of average varying rate of power/delay/AOA. Proposal 9: The definitions of performance metrics for calibration should be clarified for sensing. - The target channel should be considered for calibration of coupling loss, CDF of Delay Spread and Angle Spread, Cross-correlation coefficient of delay, AOA/LOS-NLOS status/channel response, CDF of average varying rate of power/delay/AOA. - The definitions of the metrics in TR38.901 can be reused, wherein the communication channel between TRP and UE is replaced by the target channel between sensing Tx and sensing Rx. - Both the target channel and background channel should be considered for calibration of SIR and SINR. - Target channel should be considered as signal (S). - Target channel of other targets and background channel for the same pair of sensing Tx and sensing Rx shall be considered as Interference (I). - The rays considered in large-scale calibration/full calibration/spatial consistency calibration are as follows when referring to TR 38.901. - The target channel (based on LOS pathloss) and background channel (based on LOS pathloss) should be considered in S and I in large-scale calibration. - The LOS ray and NLOS rays of target channel and background channel should be considered in S and I in full calibration/spatial consistency calibration. Proposal 10: For large-scale calibration, the simulation assumption in tableⅠ can be used as a starting point. Proposal 11: For full calibration, the simulation assumption in tableⅡ can be used as a starting point. |
| [22] Ericsson | Proposal 9 Perform separate calibration of the target and the background channel. Otherwise, implementation errors in the target channel may NOT be visible in many metrics that typically are used to characterize the channel because background channel contains orders of magnitude more power than the target channel. Proposal 10 Since ISAC channel model reuses many existing channel models and TRs for different types of Tx-Rx and Tx/Rx-target links, there is no need to recalibrate some of these models, which have previously been calibrated. Proposal 11 Calibration metrics should be strictly limited to channel metrics based on received power and multipath. Proposal 12 Calibration of target channel only requires a single deterministically modelled target/tx/rx. Proposal 13 Target channel calibration is to compare companies' implementations of the RCS and polarization models for various targets. This calibration can be performed separately for different types of targets and Type-1 EOs as needed. • In order to isolate the target modeling and mitigate the random variations caused by stochastic factors, deterministic CDL models can be used for Tx-target and target-Rx links and concatenated. • The geometry of a single target and a pair of sensing Tx and Rx, including movement of one or more of these, can be deterministically specified in the calibration assumptions. • Metrics can be received power, mean delay/angle, delay/angular spread, and Doppler frequency shift determined on the concatenated links would be useful. Proposal 14 Background channel calibration is performed for new monostatic background channel. The revised bistatic background channel models may be introduced and in need of calibration. There is no need to recalibrate the channel defined in existing TRs. • Background channel calibration is independent from target type and can be separately performed for different types of Tx/Rx such as macro base stations, micro base stations, outdoor UEs, indoor UEs, etc. • Typical metrics would be the received power, mean delay/angle and delay/angular spread. Proposal 15 Calibration of the combination of background and target channel depends on the progress in 9.7.2 and is needed in case of agreements on aspects such as cluster/path reduction, blocking by targets or Type 2 EOs influencing the background channel, normaliztion. Deterministic models for the background channels and Tx/Rx-target channels such as CDL models could be utilized. Proposal 16 Do not support large-scale calibration. Proposal 17 To calibrate companies’ implementation of target channel, one target and a pair of Tx and Rx is sufficient. Proposal 18 Full calibration with multiple sensing Tx and Rx is for background channel only. Proposal 19 Some considerations of calibration parameters: • Minimum 3D distance between sensing targets is not needed if a single intended or unintended target is modelled in target channel. • Fixed speed is used to calibrate Doppler frequency modelling. • Consider a stationary UAV or bird with rotating rotors or flipping wings. |
| [23] Apple | Proposal 1: For channel model calibration, RAN1 should adopt the stochastic communication channel model calibration approach in TR 38.901 for ISAC channel modelling for • large scale parameters, where fast fading is not included • full calibration, which includes fast fading • Spatial consistency Proposal 2: On the issue of the spatial consistency model/parameters and for some parameters such as the mean and variance of the RCS, progress in Agenda Item 9.7.2 is needed. |
| [24] Samsung | Proposal 3 RAN1 clarify the purpose and range of ISAC channel model calibration work considering below possible approach: o Approach #1: Extract sensing specific features and perform calibration (e.g., RCS distributions per sensing mode) o Approach #2: Perform calibration for each channel individually (e.g., CDF of coupling loss for target channel, CDF of coupling loss for background channel, and CDF of coupling loss for merged channel (target and background channel) in BS-BS bistatic sensing mode with full calibration) o Approach #3: After merging target channel and background channel, perform calibration (e.g., CDF of coupling loss for merged channel only in BS-BS bistatic with full calibration) Observation 4. In monostatic sensing modes, transmit and receive antennas could exist at the same location. Under such circumstances, sensing would typically employ full-duplex methods unless distance become excessively large, necessitating consideration of antenna panel separations designed to mitigate self-interference Observation 5. Differences in antenna configurations affecting antenna gains in channel impulse response Proposal 4 RAN1 clarify the antenna configuration for monostatic sensing mode taking into account possible options: o Option #1: The total number of antenna elements in monostatic equal to that of bistatic. That means, if the total number of antennas in bistatic is N, then the number of transmitting antennas in monostatic is N/2, and the number of receiving antennas is also N/2 o Option #2: The total number of antenna elements in monostatic double that of bistatic. That means, if the total number of antennas in bistatic is N, then the number of transmitting antennas in monostatic is N, and the number of receiving antennas is also N Proposal 5 RAN1 selects unified options and performs calibration Observation 6. Evaluation in TR36.777 was conducted using frequency bands at either 2 GHz or 700 MHz. It seems unlikely that the channel parameters specified in TR36.777 (e.g., pathloss and fast fading channel parameters) could represent all frequency ranges within ISAC channel study scope Proposal 6 For calibration of UAV as sensing target, channel parameters of UMa, UMi and RMa in TR38.901 is used Observation 7. If targets are distributed only in limited number of cells as Option A, it may be necessary to increase the number of simulations with different drop seeds from the perspective of securing statistical performance. And Option C lacks clarity regarding cell boundary-less distribution and appears to be specialized for specific application. Proposal 7 For 3D distribution of sensing target, RAN1 select to Option B (N targets uniformly distributed per cell) in horizontal plane and select to Option A (Uniform between 1.5m and 300m) in vertical plane as baseline Proposal 8 For minimum distance between pairs of Tx/Rx and sensing targets, RAN1 select to Option B (distance defined in TR36.777) as baseline Proposal 9 RAN1 discuss the detailed simulation parameters of UAV scenario for calibration with Table 1 Observation 8. When a human outdoor is assumed as a sensing target, their typical surroundings may include vehicles, people, trees, etc., and further, surrounding buildings and ground. These may interfere with sensing performance due to reflections in the sensing environment. In order to reflect more realistic situations, such modelling of EOs is required. Proposal 10 For human indoor and outdoor as sensing target, RAN1 discuss the detailed environment objects Proposal 11 RAN1 discuss the detailed simulation parameters of human outdoor scenario for calibration with Table 2 Proposal 12 RAN1 discuss the detailed simulation parameters of human indoor scenario for calibration with Table 3 Proposal 13 RAN1 consider the road condition on UMi and RMa for Urban Grid scenarios Proposal 14 RAN1 discuss the detailed cell-layout for Highway scenarios between option 1 and 2 in TR36.885 Proposal 15 RAN1 discuss the detailed simulation parameters of automotive scenario for calibration with Table 4 Proposal 16 RAN1 discuss the detailed Indoor Factory scenario considering cell layout Proposal 17 RAN1 discuss the detailed simulation parameters of AGV scenario for calibration with Table 6 Proposal 18 Discuss whether link between sensing target and background environment is considered and necessity of definition of minimum distance between sensing target and environment object Proposal 19 RAN1 introduce all studied scenarios for ISAC in TR 38.901. Proposal 20 Discuss how to make working scope of channel modeling for 6 sensing modes and whether RAN1 need to focus on specific some of sensing mode considering with two types as below - Type #1: a common channel modelling framework can be applied for all 6 sensing mode with some adaptation for each mode (e.g., sensing TX, RX change between BS and UE) - Type #2: individual channel modelling should be studied for each sensing mode Proposal 21 RAN1 supports the channel parameters for ISAC with possible frequency dependency Proposal 22 RAN1 study and model the ISAC channel considering validation of efficacy for frequency bands |
| [25] CAICT | Proposal 1: It is suggested to the reduce the options in the assumptions for channel model calibration and thus to reduce the workload accordingly. Proposal 2: Metrics for channel model calibration are suggested to use: 1) Coupling loss – serving cell (based on LOS pathloss) 2) Geometry (based on LOS pathloss) with and without white noise 3) CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) from the serving cell (according to circular angle spread definition of TR 25.996) for full calibration where 1) and 2) for large scale calibration and 1),2) and 3) for full calibration Proposal 3: It is suggested to use assumptions in Table1/2/3 for channel model calibration for sensing scenarios of UAV, automotive vehicles, and humans, respectively. |
| [27] MediaTek | Proposal 1: Prioritize the full calibration method for ISAC channel modelling. Proposal 2: Suggesting only consider the target channel H_target calibration if background channel H_background is modelled with the same communication channel procedure. Proposal 3: Suggesting the following channel modelling calibration parameters for UAV and Human indoor use cases. {see tdoc} |
| [29] Lenovo | Observation 1: For the purpose of the ISAC channel model calibration, it would not be sufficient to monitor the metrics only associated with the combined ISAC channel, since the impact of the target channel may be lost/buried in the impact of modeling mismatches among companies for the background channel. Observation 2: The channel modeling framework as discussed in AI 9.7.2 consists of various novel steps, e.g., modeling of the sensing target, modeling of the target channel, modifications to the background channel, combination step for the background and target channel. Proposal 1: Consider separate metrics for calibration of the target channel, background channel and the combined ISAC channel, including at least the coupling loss, delay and angle spread each associated with the target channel, the background channel and the combined ISAC channel. Proposal 2: In order to better calibrate the channel modeling implementation among companies, it is preferred to take a stepwise approach with a limited number of novel features present in each calibration step. Proposal 3: If the stepwise calibration process is agreed, the calibration assumptions and metrics can be discussed and defined at different steps, where link-level assumptions can be used for calibration of the target object modeling. Proposal 4: All the 5 sensing target types and sensing modes are intended to be captured by the channel model calibration process based on the company contributions and scenario interest. Proposal 5: The following simplifications can be made at the first step, when applicable, on the simulation assumptions for channel calibration: • Assuming a single antenna element at the Tx and Rx side with isotropic radiation pattern with zero rotation/down-tilt. • No EO modeled within the ISAC channel at the first step • No need to make adjustments for subcarrier spacing in the calibration step |
| [33] AT&T | Observation 1: A hybrid approach to ISAC channel modelling calibrations could allow for a balance of realistic but not an overly complex means of ISAC CM simulations. Proposal 1: RAN1 considers a hybrid approach to CM calibration whereby simulations would consist of deterministic models for the Tx/Rx-target paths at least for the LOS paths. Additionally, more complex components will be derived stochastically, such as the background channel and modeling of environmental objects. Proposal 2: In addition to large scale calibration and full calibration that includes fast fading, RAN1 should consider calibration for spatial consistency as part of the ISAC channel modelling study. Details of the parameters for spatial consistency are FFS and should be further discussed. Proposal 3: RAN1 considers all 5 sensing target types identified in the SID should be represented in new TR 38.901 ISAC channel model calibration clause(s). With limited TUs remaining for the ISAC CM study, no explicit prioritization for the different sensing modes/scenarios is required but can be implied based on company contributions. Proposal 4: For ISAC CM calibration assumptions, the company inputs captured in R1-2500059 Email discussion summary on ISAC CM calibration assumptions [6] should be used as a starting point to harmonize the calibration assumptions. The calibration parameters for each sensing target and a given scenario should be consistently reflected in the tabular summary (tabs). |
| [34] BUPT | Proposal 1: RAN1 adopts Option 1 for channel model calibration. Proposal 2: The target channel and Mono-static background channel need to be calibrated. The combined channel can be used solely for SIR/SINR calibration. Proposal 3: The calibration of the ISAC channel model can follow three phases: large scale calibration, full calibration, and additional calibration (SC and EO). Proposal 4: Centering around the target, select the strongest Tx-target and target-Rx pairs for simulation, with the simulated beam direction pointing towards the target. Proposal 5: When considering SIR/SINR calibration, the interference can be divided into two parts: the background channel from the same transmitter and the combined channel from different transmitters. Proposal 6: Selecting fixed RCS values for use in channel model calibration. Proposal 7：When Option 3 is used for calibration, NLOS-NLOS power normalization should be applied to align the power with that of Option 0, ensuring compliance with the objective physical law of energy conservation. |
| [35] Qualcomm | Proposal 7: Support the following high level principles for channel model calibration: - Support both large scale calibration and full calibration - All 5 sensing target types identified in the SID should be represented in new ISAC channel model calibration clause(s) similar to TR 38.901 section 7.8. |

Moderator discussion: There is significant interest to make some progress on CM calibration and calibration assumptions/parameters to align the ISAC CM evaluations across different companies. This will also enable RAN1 to have an apples-to-apples comparison of evaluation results for sensing solutions for future ISAC study item/work items. Based on the general proposals submitted to this meeting, the following proposals are made:

## Proposal 3-1 (FL1)

Proposal 3-1:  (HIGH) The following guiding principles for CM calibration methodologies are proposed to progress the ISAC channel modelling:

      - For ISAC channel modelling calibration, RAN1 considers both large-scale and full-scale calibration to include parameters and values for at least the following: large scale parameters, where fast fading is not included and full calibration, which includes fast fading.

- NOTE1: Additional calibrations for spatial consistency should also be considered.

- NOTE2: It is assumed that calibrations can not be directly inferred from the communications modelling and will be based on further progress in 9.7.2.

      - RAN1 considers calibration methodology to separate calibration of the target channel and the background channel for mono-static (if necessary), as well as combined channel (combination of target and background channel).

      - For ISAC calibrations, RAN1 does not limit nor explicitly prioritize any sensing targets, deployment scenarios, or sensing modes.

      - Calibration metrics should at least include the following:

- For large scale calibration: coupling loss

- For full calibration: coupling loss, delay spread and angle spread

- Additional calibration metrics are not precluded.

      - Interested companies are requested to provide large-scale and full-scale calibration results starting after RAN1#120.

Companies can provide comments/inputs in the following table:

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE |  | Generally OK. For Note 1, we prefer the following change since RCS modelling of a sensing target can be included in large scale calibration and full calibration. For calibration methodology of 2, we suggest to prioritize target channel calibration. For background channel or combined channel, if legacy 901 background channel is reused, then there is no need to re-calibrate. Support bullet 3,4,5 - For ISAC channel modelling calibration, RAN1 considers both large-scale and full-scale calibration to include parameters and values for at least the following: large scale parameters, where fast fading is not included and full calibration, which includes fast fading. - NOTE1: Additional calibrations for spatial consistency, RCS modelling should also be considered. - At least support calibration of target channel - RAN1 further considers calibration methodology to separate calibration of the target channel and the background channel, as well as combined channel (combination of target and background channel) if new background channel is defined. |
| Ericsson |  | On bullet 1: Firstly, “Large scale parameters” as defined for the communication channel in present TR 38.901 includes parameters generated in steps 2, 3, and 4, namely LOS/NLOS state, indoor/outdoor state, path loss between a BS and a UT, and delay spread (DS), angular spreads (ASA, ASD, ZSA, ZSD), Ricean K-factor, and shadow fading (SF). Hence these parameters are unambiguous and have been used for calibrating implementations of step 2-4 in the channel model in a “large scale calibration”. In the draft CR in R1-2500998, the procedure consists of performing these steps three times, once for the Tx-Rx (background) channel, once for the Tx-target link, and once for the target-Rx link. Present agreement is to reuse models and parameters for steps 2-4 for each of the three links from previous 38.901 and other TRs as a starting point. (In some cases, for example for the monostatic background case, new agreements on models and procedures for the equivalents of step 2-4 may be needed). For the target channel, as outline in step 9 in the draft CR, two different sets of clusters and rays are to be combined and weighted by the respective RCS in Step 10. Therefore, the large scale parameters per path generated in the respective steps 2-4 for each of the Tx-target and target-Rx link are not representative of the path loss and multipath dispersion of the target channel. The path loss and multipath dispersion (delay and angular spreads) of the target channel can of course be computed, but this by necessity requires small scale parameters such as rays and clusters. Hence, there is no obvious purpose of a “large scale calibration” that doesn’t take these small scale parameters into account. It may be proposed to use a mean RCS calculated over 360 azimuth angles and 180 zenith angles for large-scale calibration. However, for a target, paths of some angles, rather than all angles, are generated in target channel. From the perspective of sensing Tx and Rx, BS and UE observe different zenith angular ranges, due to their heights relative to targets. For UAVs in 1.5m~300m, BS’s zenith angular range is [0, 180] and UE’s [0, 90], while for targets on the ground, BS’s zenith angular range is [90, 180] and UE’s [0, 180]. Therefore, if mean RCS is included in large-scale calibration, the angular range required to calculate mean RCS depends on small scale parameters. We therefore propose to skip “large scale calibration” as it is unnecessary and unneeded and only increases the work load. Secondly, full calibration can be used for background channel calibration, while target channel calibration can be performed in a deterministic way, e.g., by concatenating two CDL models for Tx-target and target-Rx links. |
| Huawei, HiSilicon |  | We agree the change from ZTE for the second bullet. In addition, we suggest the following changes: 1. For ISAC channel modelling calibration, RAN1 considers both large-scale and full-scale calibration to include parameters and values for at least the following: large scale parameters, where fast fading is not included and full calibration, which includes fast fading. - NOTE1: Additional calibrations for spatial consistency also be considered. 3. For ISAC calibrations, RAN1 does not limit nor explicitly prioritize any sensing targets, deployment scenarios, or sensing modes. |
| CATT, CICTCI | Y in general | We agree the change from ZTE for the second bullet, since: - Target channel along is important to calibrated; subsequently, the combined channel. - Background channel may be reused (e.g. for bi-static) and if so, no need to calibrate redundantly. |
| Samsung |  | Generally fine. For Note 1, we would like to clarify the purpose of each calibration. In the case of large-scale calibration, what insights can RAN1 expect? If there is no clear intention or motivation, considering only full calibration should be sufficient. For Note 2, the current suggestion from FL seems appropriate. The Rel-19 ISAC channel modeling study introduced many new features to reflect the sensing environment for both the target and background channels. In the target channel, RCS modeling and concatenation between two segments were introduced. In the background channel, a new modeling method for monostatic sensing mode was adopted. To better understand channel modeling implementation and gain meaningful insights from calibration, we believe it is more appropriate to conduct calibration by dividing each channel separately. |
| Spreadtrum | Yes | - For bullet 1, both large scale and full scale calibration are necessary. - For note 1, RCS should be considered in large scale and full calibration, but we suggest to simplify (or even drop) the B1 of RCS to reduce the complexity of simulation. - For bullet 2, considering that combination methodology of target channel and background channel is not finished in 9.7.2, we suggest to calibrate the target channel and background channel separately first. |
| Panasonic |  | We are fine with the proposal. For proposed change from ZTE, it is fine to us as well. |
| Qualcomm |  | - RCS not needed to be calibrated - Background channel not needed to be calibrated for some cases (e.g., bistatic Background channel) - Prefer to remove the sentence: “RAN1 will document results only for a scenario if a sufficient number of companies submit results for the calibration” |
| Apple |  | Okay with ZTE’s update Also support the removal of the sentence on a condition for documentation. |
| Lenovo |  | On bullet point 1, we are generally OK. Some large scale parameters of the target channel are not directly inferred (or at least there is currently no agreement on it), hence it can be Bullet point 2 is OK. Bullet point 3 is OK. For Bullet point 4, for the background channel, we also need metrics that represents the channel dynamic range. The current metrics only represent the full channel which is dominated by the few strongest clusters. For background channel modeling, the low-energy clusters are still effective when located at proximity of the target. Hence, we suggest to further consider CDF of cluster delay, power (as representative of the clusters within the channel, and not the full channel). |
| Xiaomi |  | Generally OK. For NOTE1, same view as ZTE since RCS includes two parts: [A] for large scale and [B1, B2] for small scale. [A] can be calibrated by large scale calibration and [B1, B2] can be calibrated by full calibration. For calibration methodology, we prefer to update it as following because the background channel of bi-static can refer to current TR, such as 38.901, 36.777, 37.885 and so on. We don’t need to calibrate those background channel to reduce the calibration work load. For the background channel of mono-static, new model is under discussion and will be applied for ISAC channel. So it’s necessary to calibrate. RAN1 considers calibration methodology to separate calibration of the target channel and the background channel for mono-static, as well as combined channel (combination of target and background channel). As for the calibration metrics, coupling loss could be used for large scale calibration and full calibration, delay spread and angle spread could be used for full calibration, and the fourth bullet could be updated: 4. Calibration metrics should at least include metrics based on coupling loss, delay spread and angle spread. - For large scale calibration: coupling loss - For full calibration: coupling loss, delay spread and angle spread Additional calibration metrics are not precluded. |
| BUPT | Yes | By the way, the target channel is essential for calibration. For the Bi-static background channel, the use of the existing TRs is widely agreed upon, and the calibration of this part can be downplayed. The Mono-static background channel is a new concept and requires consideration for calibration. The combined channel can be used solely for SIR/SINR calibration. |
| MediaTek |  | For bullet 1, we need to firstly clarify what’s the sensing large scale calibration since the definition has not been agreed in AI 9.7.2. For bullet 2, we agree with ZTE’s change to prioritize the target channel calibration. If the background channel are totally same with the existing communication channel, there is no need to calibrate it. |
| Nokia | Yes | We are fine with the proposals. Our opinion is that both large-scale and full simulations are necessary. We are also supporting the calibration of the target and background channels separately. |
| Moderator |  | Updated proposal in redlines based on feedback thus far, including the discussion in the offline session. Additional feedback is encouraged. |
| Ericsson |  | We still don't see the need for large-scale calibration. If it is only for the following new parameters, PL and fading in the equation have been calibrated before. The new addition is RCS component A, which companies will agree one value for a type of sensing target for calibration. The point of large-scale calibration is limited to mathematics? LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2 Regarding calibration methodology of separate calibration of the target channel and background channel, we suggest deterministic CDL channel model is used to model Tx-target and target-Rx links. The deterministic CDL can remove the interference of stochastic NLOS clusters and highlight the implementation errors of target’s RCS and XPM. The target channel is a concatenation of two CDL models with target’s RCS and XPM. This only requires one target and a pair of sensing Tx and Rx. |

## General Calibration proposals (Round 2) – Updated in offline session

Proposal 3-1:  (HIGH)

      - For ISAC channel modelling calibration, RAN1 considers both large-scale and full-scale calibration to include parameters and values for at least the following:

    - large scale parameters, where fast fading is not included

    - full-scale calibration parameters, which includes fast fading.

- NOTE1: Additional calibrations for spatial consistency should also be considered.

- NOTE2: It is assumed that ISAC calibrations can not be directly inferred from the communications modelling and will be based on further progress in 9.7.2.

Proposal 3-2:  (HIGH)

      - RAN1 considers a step-wise calibration methodology to separate calibration of the target channel and the background channel, and if not redundant, combined channel (combination of target and background channel).

      - NOTE: The assumption is that background channel calibration is necessary as new aspects will be introduced for ISAC channel modelling not addressed in communications channel model.

      - FFS: Details of calibrations for target channel.

Proposal 3-3:  (HIGH)

      - For ISAC calibrations, RAN1 does not limit nor explicitly prioritize any sensing targets, deployment scenarios, or sensing modes.

Proposal 3-4:  (HIGH)

      - Calibration metrics should at least include the following:

- For large scale calibration: coupling loss (target channel, PL Tx/Rx-Target, shadow fading, A…)

- For full calibration: coupling loss (if not redundant with large scale calibration), delay spread, and angle spread

- Additional calibration metrics are not precluded.

Proposal 3-5:  (HIGH)

      - Interested companies are requested to provide large-scale and full-scale calibration results starting after RAN1#120.

Companies can provide comments/inputs in the following table:

| Company | Yes/No | Comments |
| --- | --- | --- |
| Moderator |  | Please comment on the above proposals based on the Tuesday offline discussion. Also note that not all companies are required to perform calibrations for, e.g., all scenarios, modes, STs, etc. |
| ZTE |  | Generally ok. One clarification question for Proposal 3-2, what does the “step-wise” mean? If companies’ common understanding is to separately calibrate target channel, background channel and combine channel, then we prefer to delete the “step-wise”. |

## General Calibration proposals - Round 3 – For Thursday online session

Proposal 3-3:  (HIGH)

      - For ISAC calibrations, RAN1 does not limit nor explicitly prioritize any sensing targets, deployment scenarios, or sensing modes.

Proposal 3-4:  (HIGH)

      - Calibration metrics should at least include the following:

- For large scale calibration: coupling loss (incorporating power scaling factor, antenna gain)  (target channel, PL Tx/Rx-Target, shadow fading, A…)

- For full calibration: coupling loss (if not redundant with large scale calibration), delay spread, and angle spread, background cluster power metrics

- Additional calibration metrics are not precluded.

Proposal 3-5:  (HIGH)

      - Interested companies are requested to provide large-scale and/or full-scale calibration results starting after RAN1#120.

Companies can provide comments/inputs in the following table:

| Company | Yes/No | Comments |
| --- | --- | --- |

# Topic #2: Updates to the deployment scenario agreements

The related proposals are copied below.

| Company | Views/Proposals |
| --- | --- |
| [6] CATT | Proposal 1: For UAV sensing scenarios, for unintended/environment objects, - Do not model EO type-2; - Do not model EO type-1. Instead, consider the following two alternatives: - Alt1: Consider birds as additional sensing targets (unintended objects) in this scenario; - Alt2: Consider birds as random clutters in this scenario, i.e. no unintended object or EO is considered in this scenario; - Note: Unintended UAV, if needed, can be treated as part of multi target sensing; Proposal 2: For Automotive sensing scenarios, for the EO type-2 in Urban Grid, when modelled, - The number of buildings follows the road configuration and warp around rule in TR 37.885/36.885; for calibration, the number of building can be reduced for simplicity, e.g. to 1; - Consider shifting the road grids by (ISD3-10,32*ISD3-10) m and thus BSs are placed at the corner of the top of buildings; - 4 walls per building are modelled. The impact of a wall is ignored if specular reflection condition is not met at the surface of the wall; - No need to introduce additional building size. Proposal 3: For human sensing scenarios: - For minimum 3D distance between sensing targets, in Option 2, consider x = 10 m for outdoor, and x = 3 m for indoor; - No need to model any EO in these scenarios. Proposal 4: For AGV sensing scenario: - For physical characteristics, no need investigate materials, additional size or size distribution; - For minimum 3D distance between sensing targets, in Option B, consider x = 10 m; - No need to model any EO in these scenarios; - For down-selection of sub-scenarios of InF, select InF-SH for calibration. Proposal 5: For sensing objects creating hazards: - For 3D mobility, adopt the following options listed below; Option 1: 0 km/h Option 2: 5 km/h Option 3: Uniform distribution between 0 km/h and 10 km/h (horizontal plane with random direction straight-line trajectory). - For EO type-2 in Urban Grid, when modelled, - The number of buildings follows the road configuration and warp around rule in TR 37.885/36.885; for calibration, the number of building can be reduced for simplicity, e.g. to 1; - Consider shifting the road grids by (ISD3-10,32*ISD3-10) m and thus BSs are placed at the corner of the top of buildings; - 4 walls per building are modelled. The impact of a wall is ignored if specular reflection condition is not met at the surface of the wall; - No need to introduce additional building size. |
| [12] Tiami | Table 1. Building as an EO: Characteristics for different scenarios. EO type: Building; UMi; UMa; RMa. Size; Height: 10-30m width: 10-50m length: 10-50m; Height: 15-60m width: 20-100m length: 20-100m; Height: 3-15m width: 10-30m length: 10-30m. Material; Concrete, glass; Concrete, glass; Concrete, wood, glass. Proposal 1. For EOs that are common across different deployment scenarios such as UMi, UMa, RMa, etc., define EO characteristics in a separate table and and refer to it for different target types. |
| [13] OPPO | Observation 1: How to model “if horizontal velocity is not fixed to 0” in 3D mobility of UAV scenario is undefined, which makes the UAV scenario assumption table incomplete. Proposal 1: A Note is attached to “if horizontal velocity is not fixed to 0” in 3D mobility, providing the following probability of non-zero horizontal velocity Probnonzero horizontal speed=1-Prob11-Prob2 where - Prob1=1hUAV≤h0exp-hUAV-h0a0hUAV>h0 - Prob2=0hUAV≤h1p0hUAV>h1 - {h0, a0,h1, p0} are constants and their values are FFS. |
| [17] Nokia | Proposal 1: Adopt the changes to Table 2 for UAV sensing scenarios Proposal 2.: For automobile sensing scenarios, the changes to Table 4 should be adopted. Proposal 3.: For human sensing scenarios, the changes to Table 6 should be adopted. Proposal 4.: For Automated Guided Vehicles (AGV) target scenarios, the changes to Table 8 should be adopted. Proposal 5.: For objects creating hazards target scenarios, the changes to Table 10 should be adopted. |
| [20] Xiaomi | Proposal 1：EO is not modelled in UAV sensing scenarios. Proposal 2: The fix value of min 3D distance between sensing targets at human indoor scenario is set to 1m. Proposal 3: EO is not modelled in human indoor scenario. |
| [21] IDCC | Proposal 1: Include the following description for sensing Tx/Rx location for aerial UEs for UAV sensing scenario: - Horizontal plane: Uniformly distributed within the cell - Vertical plane: - Option A: Uniform between 1.5m and 300m. - Option B: Fixed height value chosen from {25, 50, 100, 200, 300} m assuming vertical velocity is equal to 0. Proposal 2: For UAV, Human and AGV sensing targets, support a static sensing transmitter/receiver with velocity of 0m/s (Option 1). Proposal 3: For UAV, Human and AGV targets, support a mobile sensing transmitter/receiver where the velocity is determined randomly within a specified range based on characteristics of the UE types (Option 3). Proposal 4: Time-varying velocity modelling for mobile sensing transmitter/receiver and sensing targets is supported for different sensing targets for future evaluations. Proposal 5: For mobile sensing transmitters/receivers and sensing targets, define the time-varying velocity model based on an autoregressive (AR) model where the velocity depends on its value in previous time instances and is influenced by random factors. Proposal 6: Support Option C “N targets uniformly distributed within an area not necessarily determined by cell boundaries” for target distribution in the horizontal plane from UAV scenario for the outdoor deployment scenarios corresponding to different sensing targets. Proposal 7: Support continuous distribution such as Option A “Uniform between 1.5m and 300m” from the UAV scenario for vertical plane distribution for different sensing scenarios. Proposal 8: Use a uniform distribution for target distribution in both horizontal and vertical planes across the sensing scenarios for various sensing targets. Proposal 9: For indoor scenarios such as for AGV and human sensing, support Option B “Uniformly distributed in horizontal plane” for 3D distribution. Proposal 10: Support dropping of no sensing target (N=0) and multiple (e.g., N>1) sensing targets with one or more of the sensing targets considered as unintended objects. Proposal 11: Support Option 1 “At least larger than the physical size of a target” for the minimum distance between sensing targets for the sensing scenarios corresponding to different sensing targets. Proposal 12: Incorporate the micro-Doppler signatures of the sensing targets, unintended targets for different deployment scenarios. Proposal 13: Use Table 1 for the UMa deployment scenario for UAV sensing. Proposal 14: Use Table 2 for indoor deployment scenarios for indoor human sensing. Proposal 15: Use Table 3 for InF deployment scenario for AGV sensing. |
| [22] Ericsson | Proposal 1 Discussion on calibration parameters should not put a restriction on the validity of channel model. Proposal 2 Applicable communication scenarios for a type of sensing targets are not limited to those agreed for calibration. Proposal 3 Time-constant speeds are used for calibration to save efforts, while the channel model should support time-varying velocity for future evaluation purpose. Proposal 4 Support Option 1: At least larger than the physical size of a sensing target for the parameter minimum 3D distance between sensing targets. Proposal 5 A smaller building size than 413mx230mx20m in urban grid can be considered at least for RMa scenario, e.g. 12mx10mx10m. Proposal 6 Two prerequisites should be met to model specular reflection of Type-2 EO: 1) the Type-2 EO is of the same size as or larger than the first Fresnel zone, and 2) the incident angle equals the reflection angle. Proposal 7 One table of parameters in section 9.7.1 is sufficient. There is no need to generate tables for separate sensing scenarios in section 9.7.1. Proposal 8 Support the parameters in Table 1 for section 7.9.1 of 38.901. |
| [23] Apple | Proposal 3: For Human indoors targets • The evaluation/calibration should discuss if the relative locations of the sensing transmitters, receivers and targets in the UMi/UMa scenario should be considered with the following options: o Option 1: sensing transmitter outdoors, sensing receiver outdoors only, o Option 2: sensing transmitter outdoors, sensing receiver indoors Proposal 4: For unintended/environment object modeling for the different use cases the specific type of EOs, the following options are considered: • Option 1: explicit EO (EO-Type 1/2) o Indoor office (human, furniture, floor, walls), o Indoor Home (human, furniture, floor, walls) o Outdoor (human, walls, buildings) • Option 2: increased clutter similar to Indoor Factory modelling (InF-DH, InF-DL, InF-SH, InF-SL). |
| [24] Samsung | Proposal 1 RAN1 consider the sensing target as EO type-1 for each sensing target scenario in unintended/environment object category Proposal 2 RAN1 discuss the detailed shape and size for EO type-2 in unintended/environment object category |
| [28] LGE | Proposal 1: Only a single type of sensing target that needs to be detected in the associated sensing scenario is dropped for evaluation. Proposal 2: If the multiple types of sensing target objects are allowed for dropping in a sensing scenario, except the sensing target that needs to be detected in the sensing scenario, other types of sensing target objects are modeled as type-1 EO. Proposal 3: In UAV sensing, for the horizontal plane of 3D distribution of a sensing target, both option A and C are supported. • Option A: N targets uniformly distributed within one cell. • Option C: N targets uniformly distributed within an area not necessarily determined by cell Proposal 4: In UAV sensing, in option C for the horizontal plane of 3D distribution of a sensing target, ‘the area not necessarily determined by cell boundaries’ is defined by a sphere space, which is represented by the 3D center position and the radius. Proposal 5: In automotive sensing, the min. distance between vehicles on the same lane is 2 meters. Proposal 6: In human sensing, for the horizontal plane of 3D distribution of a sensing target, both option A and C are supported. • Option A: N targets uniformly distributed within one cell. • Option C: N targets uniformly distributed within an area not necessarily determined by cell Proposal 7: In human sensing, for option C for the horizontal plane of 3D distribution of a sensing target, ‘the area not necessarily determined by cell boundaries’ is defined by a 2D circle space, which is represented by the center position and the radius. Proposal 8: In AGV sensing, for the horizontal plane of 3D distribution of a sensing target, option B is supported. • Option B: Uniformly distributed in horizontal plane Proposal 9: For the automated guided vehicle sensing scenario, the walls, ceil and floor inside the factory are modeled as type-2 EO. |
| [35] Qualcomm | Proposal 1: Support the following updates in the Evaluation parameter table for UAV sensing scenarios: - 3D distribution in the horizontal plane, support: - Option A: N targets uniformly distributed within one cell - 3D distribution in the vertical plane, support: - Option A: Uniform between 1.5m and 300m - sizes of UAV: - make Option 2 (0.3m x 0.4m x 0.2m) as baseline - minimum 3D distances between sensing target: - Support Option 1 (At least larger than the physical size of a target) Proposal 2: For the UAV Evaluation parameter table, with regards to the Unintended objects we propose the following: - For the purpose of sensing the i-th target, the N-1 remaining uniformly distributed targets in the cell are considered an unintended objects - Do not introduce Environment Objects Type 1 or Type 2 Proposal 3: Support the following updates in the Evaluation parameter table for humans sensing scenarios: - For the Rx/Tx mobility, - support Option 3 - For the 3D mobility, - support Option 3 - For the 3D distribution in indoor, - support N=[1, 5] targets - For the 3D distribution in outdoor, - Option A: N targets uniformly distributed within one cell - For the EO in outdoor - Do not include any Environment object - For the EO in indoor - Include as an optional scenario, one or more of the: - 4 outside walls, - the ceiling and - the ground floor Proposal 4: Support the following updates in the evaluation parameter table for automotive vehicle sensing scenarios: - With regards to the minimum 3D distances between targets: - Support Option 1 - With regards to the environment objects: - EO Type 2 should be an optional modelling component in urban grid (blockage & pathloss is still modelled) - 4 walls (as EO type 2) per building of size 413m X 230m X Hm, where H > BS Height Proposal 5: Support the following updates in the evaluation parameter table for automotive vehicle sensing scenarios: - Rx/Tx Mobility for UEs - Option 3: Uniform distribution between 0km/h and 3km/h - 3D mobility - Option 1: Uniform distribution in the range of up to 30 km/h - 3D distribution - Option A: Uniformly distributed in the convex hull of the horizontal BS deployment - Minimum 3D distance between sensing targets - Option A: At least larger than the physical size of a target - Environment Objects - Include as an optional scenario, one or more of the: - 4 outside walls, - the ceiling and - the ground floor Proposal 6: Support the following updates in the evaluation parameter table for automotive vehicle sensing scenarios: - 3D mobility - Horizontal velocity: up to 10 km/h for humans and animals - Minimum 3D distance between sensing targets - Option 1: At least larger than the physical size of a target - Environment Objects - Add EO Type-2 according to the urban grid layout of 37.885 with the following details: - 4 walls (as EO type 2) per building of size 413m X 230m X Hm (FR1) and 186m X 72m X Hm (FR2), where H > BS Height |

Moderator Discussion: Based on the submitted proposals of possible updates to the evaluation parameter/values for the current baseline deployment scenario tables, the following proposals are made to further converge the deployment scenario for UAV targets. Although the baseline scenario tables have been agreed in previous meetings, several FFSs/square brackets and ambiguities remain. The goal will be to have the scenario tables for each sensing target ready to be included into the draft CR.

Specifically on the updates below;

            - Each of the sensing target deployment scenarios have a placeholder row for Environment objects, e.g., types, characteristics, mobility, distribution, etc, with FFS and it is recognized that further progress needs to happen on 9.7.2 before we can fully understand what characteristics need to be specified for EOs. Additionally, EO types of the same kind may span across multiple sensing target scenarios/use cases. It should be possible to separate the EOs from the scenarios into a separate table, which would apply for any applicable scenario. Furthermore, some companies have suggested that RAN1 may not need to model EOs for certain scenarios.

            - Two options have remained on Minimum 3D distance between sensing targets; Option A: At least larger than the physical size of a target, Option B: Fixed value, [x] m. value of x is FFS. It is recognized that targets need to at least satisfy Option A to avoid overlapping targets. Option B may seemingly be arbitrary, or in the case of the Automotive scenario, not possible for certain vehicle types (Option B is defined as 10m, but vehicle type 3 truck/bus is defined as 13m in length). Option A may be the least common denominator and can apply to all sensing target scenarios.

## Proposal 4-1 (FL1)

Proposal 4-1:  (MED) The following updates are proposed to the current baseline agreed sensing target deployment scenario tables from RAN1#119:

      - Remove the row for “Environment Objects, e.g., types, characteristics, mobility, distribution, etc.” in all sensing target scenario tables and create separate tables for EO and their associated characteristics.

        - NOTE: Table of EOs and associated characteristics are to be further tabulated during RAN1#120 and may have some dependencies on progress for 9.7.2.

        - Example of EO table as follows (green denotes previous RAN1 agreement):

Table x. EO Characteristics for ISAC scenarios

| EO |  | Type 1/ Type 2 |  | Applicable Scenario(s) |  | Size(s) |  | Material |  | Distribution |  | Mobility |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Building Type A |  | Type 2 |  | Automotive/Hazards - Urban Grid |  | 413m x 230m x 20m |  | Concrete, glass. Up to 4 walls |  | Along street grid. FFS number of buildings |  | N/A |  |
| Building Type B |  | Type 2 |  | Automotive/Hazards - Highway |  | 10-30m x 10-30m x 3-10m |  | Concrete, wood, glass. Up to 4 walls. |  | Along highway |  | N/A |  |

NOTE: Additional EOs can be modeled for further ISAC evaluations.

      - If multiple targets are modelled, for the Minimum 3D distance between sensing targets, RAN1 considers Option A: At least larger than the physical size of a target for all sensing target scenarios.

Companies can provide comments/inputs in the following table:

| Company | Yes/No | Comments |
| --- | --- | --- |
| EURECOM |  | Agree with the separate tables for EO-type 2. Agree with the minimum 3D distance between sensing targets to be at least larger than the physical size of a target for all sensing target scenarios |
| ZTE |  | For minimum 3D distance between sensing targets, we prefer to use fixed value. If concerns are regarding automotive vehicle scenario, we can consider a larger fixed value for minimum 3D distance between sensing targets to cover vehicle type 3 truck/bus . |
| Ericsson |  | We propose to model only one target for a sensing scenario for the sake of simplicity. If multiple targets are modelled, we are fine with the second bullet. |
| Huawei, HiSilicon |  | Regarding the first bullet, we spent a lot of time discussing and agreeing on those tables. Prefer to not change it before seeing what the exact specific table would be for the EO and how the dependency on progress for 9.7.2 would be. |
| CATT, CICTCI |  | - For the 1st bullet, we afraid this may spread EO type-2 modeling in scenarios rather than Urban Grid, which we haven’t agreed. - For the 2nd bullet, we also prefer a fixed value to simplify simulation. This will also align with UAV table in Section 5. A general consideration can be: - Targets outdoor (incl. UAV): 10 m - Targets indoor: 3 m |
| Samsung |  | If EO consideration is optional feature, we support first bullet in calibration stage. For second bullet, although we are fine with both options, it will be better to select one option at least for calibration purpose. |
| Qualcomm |  | Support the 2nd bullet. With regards to the 1st bullet, a bit unclear why a separate table is needed, but we can accept it if it is a majority view. |
| Apple |  | Okay with 2nd bullet. |
| Lenovo | Yes |  |
| Xiaomi |  | For the first bullet, we are fine to create separate tables for EO if this is consensus. We think EO can be optionally modelled which depends on the evaluation requirement and scenarios. And we prefer to not model EO for calibration. So we suggest to change it as following: Remove the row for “Environment Objects, e.g., types, characteristics, mobility, distribution, etc.” in all sensing target scenario tables and create separate tables for EO and their associated characteristics. Necessary EOs (Type 1 or Type 2) can be optionally modelled for future ISAC evaluations and EO isn’t modelled in calibration phase. should also be included in the calibration assumptions and for any future ISAC evaluations. NOTE: Table of EOs and associated characteristics are to be further tabulated during RAN1#120 and may have some dependencies on progress for 9.7.2. For min 3D distance between sensing target, we prefer to keep both option A and option B in consideration. |
| MediaTek |  | Generally ok with the proposals |
| Nokia |  | We are fine with the second bullet on the 3D minimum distance. For the EO modeling, we would like to see the content of the table before agreeing. |
| Moderator |  | Updated proposal in redlines based on feedback thus far. Additional feedback is encouraged. Further updates included from the offline session. |
| Ericsson |  | We think Type-1 and Type-2 EO are very important parts of Rel-19 ISAC channel calibration. We suggest adding Type-1 EO in the table, including - Bird Option 1: UAV sensing scenario, similar distribution and size as UAV Option 1 - Bird Option 2: UAV sensing scenario, similar distribution and size as UAV Option 2 - Tree: all sensing scenarios, 5m x 5m x 15 m. Distribution can be uniform distribution, along the street grid or highway As agreed that Type-1 EOs have comparable physical characteristics as sensing targets, a tree swaying in the wind may generate similar Doppler as a sensing target and is a challenge for sensing algorithm to distinguish it from sensing targets of interest. |

## Proposal 4-2 (FL2) – ROUND 2

Proposal 4-1:  (HIGH)

    - Inclusion of EO type 1 and/or EO type 2 in ISAC CM calibrations is optional

Proposal 4-2:  (MED) The following updates are proposed to the current baseline agreed sensing target deployment scenario tables from RAN1#119:

      - Remove the row for “Environment Objects, e.g., types, characteristics, mobility, distribution, etc.” in all sensing target scenario tables and create separate tables for EO and their associated characteristics.

      - As a starting point, EO table as follows (green denotes previous RAN1 agreements):

Table x. EO Characteristics for ISAC scenarios

| EO | Type 1/ Type 2 | Applicable Scenario(s) | Size(s) | Material | Distribution | Mobility |
| --- | --- | --- | --- | --- | --- | --- |
| Building Type A | Type 2 | Automotive/Hazards - Urban Grid | 413m x 230m x 20m | Concrete, glass. Up to 4 walls | Along street grid. FFS number of buildings | N/A |
| Building Type B | Type 2 | Automotive/Hazards - Highway | [10-30m x 10-30m x 3-10m] | Concrete, wood, glass. Up to 4 walls. | Along highway | N/A |

NOTE: Additional EOs are not precluded and may also need to be modeled for further ISAC evaluations.

Proposal 4-3:  (MED)

If multiple targets are modelled, for the Minimum 3D distance between sensing targets, RAN1 considers Option A: “At least larger than the physical size of a target” for all sensing target scenarios.

Companies can provide comments/inputs in the following table:

| Company | Yes/No | Comments |
| --- | --- | --- |
| Moderator |  | Please comment on the above proposals based on the Tuesday offline discussion. Also note that not all companies are required to perform calibrations for, e.g., all scenarios, number of STs greater than 1, or EOs, etc. |
| ZTE |  | For proposal 4-1: Before agreeing on optional calibration EO type 1, we need clarify on necessity of EO type1 and what physical characteristics of EO type 1 is still not clear in such a late stage. For proposal 4-2: Clarification on building type B is needed. Do we randomly pick building size from 10-30m x 10-30m x 3-10m and distribute those buildings randomly along highway? |

## Proposal 4-3 (FL3) – ROUND 3 – For the Thursday online session

Proposal 4-2:  (MED) The following updates are proposed to the current baseline agreed sensing target deployment scenario tables from RAN1#119:

      - Remove the row for “Environment Objects, e.g., types, characteristics, mobility, distribution, etc.” in all sensing target scenario tables and create separate tables for EO and their associated characteristics.

      - As a starting point, EO table as follows (green denotes previous RAN1 agreements):

Table x. EO Characteristics for ISAC scenarios

| EO | Type 1/ Type 2 | Applicable Scenario(s) | Size(s) | Material | Distribution | Mobility |
| --- | --- | --- | --- | --- | --- | --- |
| Building Type A | Type 2 | Automotive/Hazards - Urban Grid | 413m x 230m x 20m | Concrete, glass. Up to 4 walls | Along street grid. FFS number of buildings | N/A |
| Building Type B | Type 2 | Automotive/Hazards - Highway | [10-30m x 10-30m x 3-10m] | Concrete, wood, glass. Up to 4 walls. | Along highway | N/A |

NOTE: Additional EOs are not precluded and may also need to be modeled for further ISAC evaluations.

Proposal 4-3:  (MED)

If multiple targets are modelled, for the Minimum 3D distance between sensing targets, RAN1 considers Option A: “At least larger than the physical size of a target” for all sensing target scenarios.

Companies can provide comments/inputs in the following table:

| Company | Yes/No | Comments |
| --- | --- | --- |

# Topic #3: Calibration assumptions for UAV sensing targets

The related proposals are copied below.

| Company | Views/Proposals |
| --- | --- |
| [8] CMCC | Proposal 2: Support the channel model calibration parameters for large scale in Table 1 and channel model calibration parameters for full calibration in Table 2. Table 1. Channel model calibration parameters for large scale Parameter; UAV; Automotive Urban; Hazards. Scenario; UMa-AV 19 macro sites, 3 sectors per site; Urban grid; Highway. Sensing mode; BS-BS bi-static. BS mono-static; BS-BS bi-static. BS mono-static; BS-BS bi-static. BS mono-static. Sectorization; 3 sectors per cell site: 30, 150 and 270 degrees; 3 sectors per cell site: 30, 150 and 270 degrees; 3 sectors per cell site: 30, 150 and 270 degrees. Carrier Frequency; FR1: 6GHz; FR1: 6GHz; FR1: 6GHz. subcarrier spacing; FR1: 30kHz; FR1: 30kHz; FR1: 30kHz. BS height; 25m; 25m; 35m. BS antenna configurations; For 6GHz (M,N,P,Mg,Ng)=(32,16,2,1,1;1,1); For 6GHz (M,N,P,Mg,Ng)=(32,16,2,1,1;1,1); For 6GHz (M,N,P,Mg,Ng)=(32,16,2,1,1;1,1). BS antenna electrical downtilting; 102 degrees; 102 degrees; 102 degrees. Antenna virtualization; DFT precoding according to TR 36.897 with application of panning and tilting angles; DFT precoding according to TR 36.897 with application of panning and tilting angles; DFT precoding according to TR 36.897 with application of panning and tilting angles. Bandwidth; FR1: 100MHz; FR1: 100MHz; FR1: 100MHz. BS noise figure; FR1: 5dB; FR1: 5dB; FR1: 5dB. Fast fading channel; Fast fading channel is not modeled; Fast fading channel is not modeled; Fast fading channel is not modeled. Sensing target distribution; Horizontal:Option B: 2 targets uniformly distributed per cell. Vertical: Option B:Fixed height value chosen from {25, 50, 100, 200, 300} m; according to 37.885 Vehicle type distribution: 100% vehicle type 2. Uniform distributed in each lane. The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}.; Uniformly distributed in horizontal plane. Sensing target mobility; Horizontal velocity: uniform distribution between 0 and 180km/h. Vertical velocity: 0km/h; All the vehicles in the same lane have the same speed. Vehicle speed is 60 km/h in all the lanes.; Horizontal velocity: 10km/h. large-scale RCS for each scattering point; A = the mean value or 1 square meter.; A=1 square meter; /. Minimum 3D distances between pairs of Tx/Rx and sensing target; 10m; Min distances based on min. TRP/UE distances defined in TR37.885 as a starting point.; /. Minimum 3D distance between sensing targets; 10m; The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}.; Option 2: Fixed value with 10m. Wrapping method; distance-based wrap-around; geographical distance-based wrapping; geographical distance-based wrapping. Metrics; 1) Coupling loss – serving cell (based on LOS pathloss) 2) Geometry (based on LOS pathloss) with and without white noise. Define the interference in sensing.; 1) Coupling loss – serving cell (based on LOS pathloss) 2) Geometry (based on LOS pathloss) with and without white noise. Define the interference in sensing.; 1) Coupling loss – serving cell (based on LOS pathloss) 2) Geometry (based on LOS pathloss) with and without white noise. Define the interference in sensing.. Selection of Tx-Rx pair for metrics statistcs; The Tx-Rx pair with the largest large-scale RSRP. Note the TX and RX should not belong the same site under bistatic mode.; The Tx-Rx pair with the largest large-scale RSRP. Note the TX and RX should not belong the same site under bistatic mode.; The Tx-Rx pair with the largest large-scale RSRP. Note the TX and RX should not belong the same site under bistatic mode.. Table 2. Channel model calibration parameters for full calibration Parameter; Values; Values; Values. Scenario; UMa-AV 19 macro sites, 3 sectors per site; Urban grid; Highway. Sensing mode; BS-BS bi-static. BS mono-static; BS-BS bi-static. BS mono-static; BS-BS bi-static. BS mono-static. Sectorization; 3 sectors per cell site: 30, 150 and 270 degrees; 3 sectors per cell site: 30, 150 and 270 degrees; 3 sectors per cell site: 30, 150 and 270 degrees. Carrier Frequency; FR1: 6GHz; FR1: 6GHz; FR1: 6GHz. subcarrier spacing; FR1: 30kHz; FR1: 30kHz; FR1: 30kHz. BS height; 25m; 25m; 35m. BS antenna configurations; For 6GHz (M,N,P,Mg,Ng)=(32,16,2,1,1,1,1); For 6GHz (M,N,P,Mg,Ng)=(32,16,2,1,1,1,1); For 6GHz (M,N,P,Mg,Ng)=(32,16,2,1,1,1,1). BS antenna electrical downtilting; 102 degrees; 102 degrees; 102 degrees. Antenna virtualization; DFT precoding according to TR 36.897 with application of panning and tilting angles; DFT precoding according to TR 36.897 with application of panning and tilting angles; DFT precoding according to TR 36.897 with application of panning and tilting angles. Bandwidth; FR1: 100MHz; FR1: 100MHz; FR1: 100MHz. BS noise figure; FR1: 5dB; FR1: 5dB; FR1: 5dB. Fast fading channel; Fast fading channel is modeled; Fast fading channel is modelled; Fast fading channel is modeled. Sensing target distribution; Horizontal: Option B: 2 targets uniformly distributed per cell. Vertical: Option B: Fixed height value chosen from {25, 50, 100, 200, 300} m; according to 37.885 Vehicle type distribution: 100% vehicle type 2. Uniform distributed in each lane. The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}.; Uniformly distributed in horizontal plane. Sensing target mobility; Horizontal velocity: uniform distribution between 0 and 180km/h. Vertical velocity: 0km/h; All the vehicles in the same lane have the same speed. Vehicle speed is 60 km/h in all the lanes.; Horizontal velocity: 10km/h. large-scale RCS for each scattering point; A = the mean value or 1 square meter.; A=1 square meter; /. Minimum 3D distances between pairs of Tx/Rx and sensing target; 10m; Min distances based on min. TRP/UE distances defined in TR37.885 as a starting point.; /. Minimum 3D distance between sensing targets; 10m; The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}.; Option 2: Fixed value with 10m. Wrapping method; distance-based wrap-around; geographical distance-based wrapping; geographical distance-based wrapping. Metrics; 1) CDF of coupling loss 2) CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) according to circular angle spread definition of TR 25.996; 1) CDF of coupling loss 2) CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) according to circular angle spread definition of TR 25.996; 1) CDF of coupling loss 2) CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) according to circular angle spread definition of TR 25.996. |
| [11] EURECOM | Proposal 1: The parameters of CM calibration for full calibration for UAV use cases are shown in Table 1. Proposal 2: The parameters of CM calibration for large scale for UAV use cases are shown in Table 2. Table 1. The parameters of CM calibration for full calibration for UAV use cases Parameters; Values. Scenario; UMa-AV 19 macro sites, 3 sectors per site. Sensing mode; TRP bistatic/monostatic. Sectorization; 3 sectors per site. Carrier Frequency; FR1: 6 GHz; FR2: 30 GHz. Subcarrier spacing; FR1: 30 kHz; FR2: 120 kHz. BS height; 25m. BS antenna configurations; For FR1: (M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;2,8) (dH,dV) = (0.5, 0.8)λ, +45°/-45° polarization For FR2: (M,N,P,Mg,Ng;Mp,Np) = (4,16,2,2,2; 1,1) (d_H,d_V ) = (0.5, 0.5)λ, (dH,g, dV,g) = (4.0, 2.0)λ, +45°/-45° polarization. BS antenna electrical downtilting; 102 degrees. Antenna virtualization; DFT precoding according to TR 36.897 with application of panning and tilting angles. BS Tx power; FR1: 49dBm FR2: 35dBm. Bandwidth; FR1: 100MHz FR2: 400MHz. BS noise figure; FR1: 5dB FR2: 7dB. UT antenna configurations; (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1). UT distribution; 10 terrestrial UEs per cell. UT attachment; Based on pathloss with a margin. Polarized antenna modelling; Model 2 in TR 38.901. UT noise figure; FR1:9dB FR2:10dB. Fast fading channel; Fast fading channel is modeled. Sensing target distribution; Uniform between 1.5m and 300m. Sensing target mobility; Horizontal : uniform distribution between 0 and 180 km/h Vertical : 0 km/h. Large-scale RCS for each scattering point; The mean value of target RCS. Minimum 3D distances between pairs of Tx/Rx and sensing target; 10 m. Minimum 3D distance between sensing targets; 10 m. Metrics; CDF of coupling loss CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA). The parameters of CM calibration for large scale for UAV use cases are shown in Table 2. Table 2. The parameters of CM calibration for large scale for UAV use cases Parameters; Values. Scenario; UMa-AV 19 macro sites, 3 sectors per site. Sensing mode; TRP bistatic/monostatic. Sectorization; 3 sectors per site. Carrier Frequency; FR1: 6 GHz; FR2: 30 GHz. Subcarrier spacing; FR1: 30 kHz; FR2: 120 kHz. BS height; 25m. BS antenna configurations; For FR1: (M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;2,8) (dH,dV) = (0.5, 0.8)λ, +45°/-45° polarization For FR2: (M,N,P,Mg,Ng;Mp,Np) = (4,16,2,2,2; 1,1) (d_H,d_V ) = (0.5, 0.5)λ, (dH,g, dV,g) = (4.0, 2.0)λ, +45°/-45° polarization. BS antenna electrical downtilting; 102 degrees. Antenna virtualization; DFT precoding according to TR 36.897 with application of panning and tilting angles. BS Tx power; FR1: 49dBm FR2: 35dBm. Bandwidth; FR1: 100MHz FR2: 400MHz. BS noise figure; FR1: 5dB FR2: 7dB. UT antenna configurations; (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1). UT distribution; 10 terrestrial UEs per cell. UT attachment; Based on pathloss with a margin. Polarized antenna modelling; Model 2 in TR 38.901. UT noise figure; FR1:9dB FR2:10dB. Fast fading channel; Fast fading channel is not modeled. Sensing target distribution; Uniform between 1.5m and 300m. Sensing target mobility; Horizontal : uniform distribution between 0 and 180 km/h Vertical : 0 km/h. Large-scale RCS for each scattering point; The mean value of target RCS. Minimum 3D distances between pairs of Tx/Rx and sensing target; 10 m. Minimum 3D distance between sensing targets; 10 m. Metrics; Coupling loss based on PL Wideband SIR and SINR based on geometry (based on pathloss). |
| [13] OPPO | Table 2 could be used as simulation assumption for UAV scenarios. Table-2 Simulation assumptions for UAV scenario Parameter; Values. Scenario; UMa-AV. Sensing mode; gNB-gNB bi-static, gNB-UE bi-static, UE-UE bi-static (incl. UAV bi-static). Sectorization; 3 sectors per cell site: 30, 150 and 270 degrees. Carrier Frequency; FR1: 6GHz; FR2: 30GHz. subcarrier spacing; Although we did not identify where in calibration to use SCS and do not think the channel model calibration should depend on signal fed in, just in case: FR1: selective from {15,30}kHz; FR2: 120KHz. BS height; 25m. BS antenna configurations; For FR1: (M,N,P,M_g,N_g;M_p,N_p )= (8,8,2,1,1;2,8) (d_H,d_V ) = (0.5, 0.8)λ, +45°/-45° polarization. BS antenna electrical downtilting; Same as in Table 7.8-1 (38.901): 102 degree. Antenna virtualization; Same as in Table 7.8-1 (38.901): DFT precoding according to TR 36.897 with application of panning and tilting angles. BS Tx power; Values from Table 7.8-2 (38.901): FR1: 49dBm FR2: 35dBm. Bandwidth; FR1/FR2: 100MHz. BS noise figure; 5dB. UT antenna configurations; For FR1/FR2: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization. Handover margin (for calibration); value from Table 7.8-1 (38.901): 0dB. UT distribution; Horizontal plane: uniformly distributed within one cell Vertical plane: - Normal UT (outdoor): 1.5m; - Normal UT (indoor): 3(nfl-1)+1.5 (m), where nfl is random integer in [1~8]; - UAV-UT: Uniform between 1.5m and 300m.. UT attachment; Based on pathloss considering LOS angle Note: The other option of "Based on RSRP from CRS port 0" may need further clarification because there is no CRS port 0 in NR.. Polarized antenna modelling; Either Model-1 or Model-2. UT array orientation; Same as in Table 7.8-2 (38.901): ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 90 degree, ΩUT, = 0 degree. UT noise figure; Same as in Table 7.8-1 (38.901): 9 dB. Fast fading channel; Fast fading channel is modelled. Sensing target distribution; One target w/ multiple drops Horizontal plane: uniformly distributed within one cell Vertical plane: Option A: Uniform between 1.5m and 300m.. Sensing target mobility; Horizontal velocity: uniform distribution between 0 and 180km/h, 0km/h. Vertical velocity: 0km/h,. large-scale RCS for each scattering point; 0.01 or 1 m2. EO distribution; not modelled. Minimum 3D distances between pairs of Tx/Rx and sensing target; 10m. Minimum 3D distance between sensing targets; 10m. Wrapping method; geographical distance based wrapping. Assumptions of power relationship between target channel and background channel; Power normalization according to RAN1 further agreement. Note: This is also one of reasons for us to suggest merging the large-scale calibration into full calibration because the large-scale factor needs to be added before power normalization over target/background channels.. Proposal 5: Use simulation assumptions in Table 2 for calibration of UAV scenarios. |

Moderator Discussion: Based on the proposals submitted to RAN1#120 and the calibration email discussion [3], along with the associated majority of proposals that included calibration parameters for UAV sensing targets, the following proposals are made to converge the ISAC channel model calibration assumptions.

NOTE1: These agreements can be used as a baseline for other sensing targets and associated calibration assumptions that are planned to be further discussed at RAN1#120 in sections 6-9 in this summary (to be completed).

NOTE2: Values/Ranges/Parameters are based on current majority views where possible, but additional values/ranges/parameters are not precluded.

## Proposal 5-1 (FL2) – For Wednesday offline session

Proposal 5-1:  (HIGH) For the purposes of full calibration for UAV sensing targets, the following calibration parameters are proposed below in Table x. Note: Parameter values in bold denote majority view for a given parameter.

Table x. Simulation assumptions for full calibration for UAV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | UMa-AV, UMi-AV, RMa-AV, UMi, UMa, RMa, |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic |
| Sectorization | 3 sectors per cell site: 30, 150 and 270 degrees. 3 sectors per cell site: 0, 120 and 240 degrees |
| Carrier Frequency | FR1: 6 GHz; FR2: 30 GHz FR3: 7GHz/15GHz |
| Subcarrier spacing | FR1: 30 kHz; FR2: 120 kHz [May not be necessary] |
| BS height | Defined in the scenario and not necessary [remove row] |
| BS antenna configurations | Single polar antenna, and all 256 elements are mapped to a single port. (M, N, P, Mg, Ng; Mp, Np) = (16, 16, 1, 1, 1, 1, 1) For FR1: (M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;2,8) (dH,dV) = (0.5, 0.8)λ, +45°/-45° polarization For FR2: (M,N,P,Mg,Ng;Mp,Np) = (4,16,2,2,2; 1,1) (d_H,d_V ) = (0.5, 0.5)λ, (dH,g, dV,g) = (4.0, 2.0)λ, +45°/-45° polarization |
| BS antenna electrical downtilting | 102 degrees |
| Antenna virtualization | Analog beam towards to the sensing target DFT precoding according to TR 36.897 with application of panning and tilting angles |
| BS Tx power | FR1: 49dBm, 53dBm, 44dBm for UMi-AV FR2: 40dBm, 35dBm |
| Bandwidth | FR1: 100MHz FR2: 400MHz |
| BS noise figure | FR1: 5dB FR2: 7dB |
| HO margin | Not needed 0 dB |
| UT antenna configurations | (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) not needed |
| UT distribution | Not necessary. Not needed for monostatic. 10 terrestrial UEs per cell with uniform distribution |
| UT attachment | Not needed for ISAC channel model calibration Based on pathloss considering LOS angle |
| Polarized antenna modelling | Model 2 in TR 38.901 |
| UT array orientation | Not Necessary ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 90 degree, ΩUT, = 0 degree |
| UT noise figure | FR1: 9dB FR2: 10dB |
| Fast fading channel | Fast fading channel is modeled. [FFS fast fading model from 36.777] |
| Sensing target distribution | N targets uniformly distributed within a cell. Vertical distribution: between 1.5m and 300m or Fixed height value chosen from {25, 50, 100, 200, 300} m. N= {1, 2, 3, 4, 5} |
| Sensing target mobility | Horizontal: uniform distribution between 0 and 180 km/h Vertical: 0 km/h |
| Large-scale RCS for each scattering point | Mean value of target RCS |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | 10 m |
| Minimum 3D distance between sensing targets | 10 m |
| Metrics | CDF of coupling loss CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) Wideband SIR and SINR based on RSRP |

Companies can provide comments/inputs in the following table:

| Company | Yes/No | Comments |
| --- | --- | --- |
| EURECOM | Y | Agree with the values in bold |
| ZTE |  | For scenario: - we prefer to consider both UMa-AV and RMa-AV For BS antenna configuration: - we propose to consider larger antenna elements, for example, for simplicity of calibration, single polar is used, and all 256 elements are mapped to a single port. (M, N, P, Mg, Ng; Mp, Np) = (16, 16, 1, 1, 1, 1, 1) Antenna virtualization: - for simplicity of calibration, we suggest analog beam towards to the sensing target UT related parameters: - for TRP monostatic and TRP bistatic, there is no need to discuss UT related parameters including UT antenna configurations, UT distribution, UT attachment, UT array orientation, UT noise figure Sensing target distribution: - Horizontal: 5 targets uniformly distributed within the center cell; Vertical: fixed height with 200m Moreover, we suggest to additionally consider the following new parameters: - Small-scale RCS: RCS is one of key characteristics of sensing target, both large-scale RCS and small-scale RCS should be considered during calibration. - XPR of sensing target: the distribution of sensing target XPR - Selection of Tx-Rx pair: we prefer to select a Tx-Rx pair with the largest large-scale RSRP for each sensing target. |
| Ericsson |  | We have the following comments. - Proposal 3-1, bullet 2 and its implications on calibration should be resolved before discussing these details. - The row of ‘Large-scale RCS for each scattering point’ can be replaced with ‘sizes of UAVs’. Large or small UAV’s corresponding RCS are derived from 9.7.2 agreements. - We suggest Doppler shift as one of metrics. Therefore, a fixed speed is preferred. Orientation is missing in the table. - For Minimum 3D distance between sensing targets, we support Option A in Proposal 4-1 for all sensing targets. |
| Huawei, HiSilicon |  | For BS antenna configuration: We also suggest large antenna elements, say (16, 2, 1,1;8,16) BS antenna electrical downtilting For UAV, we suggest it should be 90 degrees. Bandwidth We suggest 200MHz should be included for 6/7GHz carrier frequency, UT related parameters: This is for UAV mono-static or bi-static. UE dropping is not needed. Orthogonal sensing RS from multiple TRPs is just one possibility. The metrics can include geometry, the definition about signal and interference can be further discussed. |
| CATT, CICTCI |  | Fine with most configurations in bold. Some suggestions: - Antenna virtualization Prefer ‘Analog beam towards to the sensing target’, which is simpler and easier to align among companies - UT related parameters No need to consider, if only TRP sensing is calibrated. We should be very careful to create new option(s) which is hard to converge. |
| Samsung |  | For scenario, - Geographic assumptions for channel generation may be reusable from TR36.777 while channel parameters should adhere to those values defined in TR38.901. Since existing channel document does not include methods for creating UAV-specific channels, basic assumptions such as 3D distribution and mobility can be referenced from TR36.777. However, questions arise regarding channel parameters because evaluation in TR36.777 was conducted using frequency bands at either 2 GHz or 700 MHz. It seems unlikely that the channel parameters specified in TR36.777 (e.g., pathloss and fast fading channel parameters) could represent all frequency ranges within ISAC channel study scope. Therefore, if these channel parameters are to be reused through ISAC channel calibration work, their applicability across different frequency bands (i.e., upper 2 GHz) would need to be verified first. For sectorization, - it is up to companies’ implementation in their SLS. We are not sure it needs to aligns For frequency, - if scenario considers UMA-AV, FR2 should be considered carefully - For BS antenna configuration, - in monostatic sensing modes, Tx and Rx antennas could exist at the same location. Under such circumstances, sensing would typically employ full-duplex methods unless distance become excessively large, necessitating consideration of antenna panel separations designed to mitigate self-interference. Based on this consideration, the below options for antenna configuration can be considered. - Option #1: The total number of antenna elements in monostatic equal to that of bistatic. That means, if the total number of antennas in bistatic is N, then the number of transmitting antennas in monostatic is N/2, and the number of receiving antennas is also N/2. - Option #2: The total number of antenna elements in monostatic double that of bistatic. That means, if the total number of antennas in bistatic is N, then the number of transmitting antennas in monostatic is N, and the number of receiving antennas is also N. - Due to differences in antenna configurations affecting antenna gains in channel impulse response, common assumptions will be necessary for efficient calibration work. |
| vivo | Yes with some comments | We have several comments on this proposal. - Firstly, one TX power selection for each FR calibration is necessary that 49dBm is for FR1 and 40dBm is for FR2. In addition, the TX power for FR3 should be defined as well. - The bandwidth for FR3 should be added with the value of 200MHz. - The noise figure for FR3 should be added with the value of 7dB - The UT attachment method should be modified to RSRP based method to align with the calibration methodology defined in TR38.901 for full calibration. However, the pathloss based attachment is desirable for large scale calibration. - The other additional parameters are proposed. The noise power density should be defined as -174dBm/Hz. The convolution method should be also clarified according to the agreement in the last meeting. - Finally, the metrics referring to the channel should be defined. For instance, the Coupling loss CDF of target channel should be used for calibration. |
| Panasonic |  | We are fine with the proposal. |
| Qualcomm |  | - Antenna virtualization: Also, prefer ‘Analog beam towards to the sensing target’, which is simpler and easier to align among companies - We also think this is a useful clarification: “ Selection of Tx-Rx pair: we prefer to select a Tx-Rx pair with the largest large-scale RSRP for each sensing target. |
| Lenovo |  | The scenario is fine For the BS antenna config, we suggest to use a simple single element (single polarized) antenna with no virtualization, for the following reason: Antenna array gain is very useful when the target position is known and can be beamformed towards the target for tracking and velocity estimation etc. However, the target position is not known before detection. Taking a beam strongly towards the target, implies that the target position is known and further, it filters out the impact of clutter which exists in the detection phase. For the metrics related to background channel, we suggest to also include cluster energy, delay CDF. |
| Xiaomi |  | We are generally fine with the values in bold except for for the following parameters - For sectorization, we prefer the second option. - For BS Tx power, different power should be determined for UMa-AV, UMi-AV, and RMa-AV. - For FR1: 53dBm, 38dBm, 53dBm for UMa-AV, UMi-AV, and RMa-AV - For FR2: 40dBm, 30dBm for UMa-AV and UMi-AV - If calibration sensing mode only includes TRP mono-static and TRP bi-static for UAV scenarios, all UT related parameters are not necessary. - For UT array orientation, it is needed when UT antenna pattern is not isotropic - For fast fading channel, more details are needed. The reference TRs used for Tx-target, target-Rx, and Tx-Rx (background channel) should be determined and could be discussed in a separate table. - For sensing target distribution, we prefer the configuration as 2 targets uniformly distributed per cell on horizontal and uniform between 1.5m and 300m on vertical. - For metrics, wideband SIR is before receiver without noise can be used instead of wideband SIR and SINR based on RSRP. - Some additional parameters are also needed: - For RCS in full calibration, small scale RCS is needed. - We would to add one row for target size because there are two different size of UAV in the evaluation parameters table. We prefer to consider the small size UAV for calibration. - The BS antenna pattern should be determined - The UT antenna pattern should be determined The UT height, UT type (i.e., Terrestrial UE or UAV UE), UT mobility, min distance between sensing Tx and sensing Rx should also be determined. |
| BUPT | Yes | Using the mean value of the target RCS for full calibration not only avoids altering the statistical properties of the channel due to RCS variations but also reduces simulation complexity. |
| InterDigital |  | We support to include gNB-UE bistatic in UAV scenario. As we discussed in our contribution, the use case in Section 5.12 considers UE as a UAV. |
| Nokia |  | We are generally fine with the parameters in bold with the following changes The Number of targets N should be 5 or at least a number greater than 1 We prefer target horizontal mobility to be a fixed-value For the bi-static sensing scenario, RX-target paring is necessary, and we propose pathloss based paring |

## Proposal 5-2 (FL2) – For Wednesday offline session

Proposal 5-2:  (HIGH) For the purposes of large scale calibration for UAV sensing targets, the following calibration parameters are proposed below in Table x. Parameter values in bold denote majority view for a given parameter.

Table x. Simulation assumptions for large scale calibration for UAV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | UMa-AV, UMi-AV, RMa-AV, UMi, UMa, RMa, |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic |
| Sectorization | 3 sectors per cell site: 30, 150 and 270 degrees. 3 sectors per cell site: 0, 120 and 240 degrees |
| Carrier Frequency | FR1: 6 GHz; FR2: 30 GHz FR3: 7GHz/15GHz |
| Subcarrier spacing | FR1: 30 kHz; FR2: 120 kHz [May not be necessary] |
| BS height | Defined in the scenario and not necessary [remove row] |
| BS antenna configurations | Single polar antenna, and all 256 elements are mapped to a single port. (M, N, P, Mg, Ng; Mp, Np) = (16, 16, 1, 1, 1, 1, 1) For FR1: (M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;2,8) (dH,dV) = (0.5, 0.8)λ, +45°/-45° polarization For FR2: (M,N,P,Mg,Ng;Mp,Np) = (4,16,2,2,2; 1,1) (d_H,d_V ) = (0.5, 0.5)λ, (dH,g, dV,g) = (4.0, 2.0)λ, +45°/-45° polarization |
| BS antenna electrical downtilting | 102 degrees |
| Antenna virtualization | Analog beam towards to the sensing target DFT precoding according to TR 36.897 with application of panning and tilting angles |
| BS Tx power | FR1: 49dBm, 53dBm, 44dBm for UMi-AV FR2: 40dBm, 35dBm |
| Bandwidth | FR1: 100MHz FR2: 400MHz |
| BS noise figure | FR1: 5dB FR2: 7dB |
| HO margin | Not needed 0 dB |
| UT antenna configurations | (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) [not needed] |
| UT distribution | Not necessary. Not needed for monostatic. 10 terrestrial UEs per cell with uniform distribution |
| UT attachment | Not needed for ISAC channel model calibration Based on pathloss considering LOS angle |
| UT noise figure | FR1: 9dB FR2: 10dB |
| Fast fading channel | Fast fading channel is not modeled |
| Sensing target distribution | N targets uniformly distributed within a cell. Vertical distribution: between 1.5m and 300m or Fixed height value chosen from {25, 50, 100, 200, 300} m. N= {1, 2, 3, 4, 5} |
| Sensing target mobility | Horizontal: uniform distribution between 0 and 180 km/h Vertical: 0 km/h |
| Large-scale RCS for each scattering point | Mean value of target RCS |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | 10 m |
| Minimum 3D distance between sensing targets | 10 m |
| Wrapping Method | Geographical distance based wrapping |
| Metrics | Coupling loss – serving cell (based on LOS pathloss) Geometry (based on LOS pathloss) with and without white noise Wideband SIR and SINR based on RSRP |

Companies can provide comments/inputs in the following table:

| Company | Yes/No | Comments |
| --- | --- | --- |
| EURECOM | Y | Agree with the values in bold |
| ZTE |  | Suggest most comments for Proposal 5-1. For Metrics, we only support coupling loss for large scale calibration. The definition of geometry (based on LOS pathloss) with and without white noise for ISAC channel modelling should be discussed before agreeing it. More clarification on the necessity is needed since sensing is different from communication channel, sequence based sensing RS can be used for the purpose of detection and tracking, where sensing RS from multiple TRPs can be orthogonal. |
| Ericsson |  | Proposal 3-1, bullets 1, 2 and their implications on calibration should be resolved before discussing these details. In particular, we need to agree whether large-scale calibration should be performed and, in agenda item 9.7.2, how to calculate the “path loss for the target channel”. Since there seems to be no agreement on such calculation then the metrics proposed here are not applicable. Same comments to ‘Large-scale RCS for each scattering point’ and ‘Minimum 3D distances between pairs of Tx/Rx and sensing target’ as in Proposal 5-1. |
| Huawei, HiSilicon |  | BS antenna electrical downtilting For large scale calibration, analog beam towards to the sensing target is sufficient. Other comments regarding BS antenna configurations, bandwidth are similar to the previous proposal for full calibration |
| CATT, CICTCI |  | Comments are mostly the same with previous one. In addition, for metric here, we have similar suggestion as ZTE: ‘only coupling loss’ is enough. |
| vivo | Yes with some comments | We have several comments on this proposal. - Firstly, one TX power selection for each FR calibration is necessary that 49dBm is for FR1 and 40dBm is for FR2. In addition, the TX power for FR3 should be defined as well. - The bandwidth for FR3 should be added with the value of 200MHz. - The noise figure for FR3 should be added with the value of 7dB - The other additional parameters are proposed. The noise power density should be defined as -174dBm/Hz. The convolution method should be also clarified according to the agreement in last meeting. - Finally, the metrics referring to the channel should be defined. For instance, the Coupling loss CDF of target channel should be used for calibration. And the Wideband SIR and SINR based on RSRP is not valid. |
| Panasonic |  | We are fine with the proposal. |
| Qualcomm |  | We tend to prefer to have only the coupling loss as metric |
| Xiaomi |  | Same comments for the proposal 5-1 are suggested except for the fast fading channel and metrics. - The large scale channel in large scale calibration should be determined with the fast fading channel used for Tx-target, target-Rx, and Tx-Rx. - For the metrics, wideband SIR and SINR based on RSRP are redundant when geometry (based on LOS pathloss) with and without white noise are used. |
| InterDigital |  | We support to include gNB-UE bistatic in UAV scenario. As we discussed in our contribution, the use case in Section 5.12 considers UE as a UAV. |
| Nokia |  | Same comments as the previous proposal. In addition, We like to see a clear definition of Geometry |

## Proposal 5-1 (FL3) – For Thursday online session

Proposal 5-1:  (HIGH) For the purposes of full calibration for UAV sensing targets, the following calibration parameters are proposed below in Table x.

Note: The table below is for informative purposes and the parameter values in bold denote majority view for a given parameter.

Table x. Simulation assumptions for full calibration for UAV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | UMa-AV, UMi-AV, RMa-AV, UMi, UMa, RMa, |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic |
| Sectorization | 3 sectors per cell site: 30, 150 and 270 degrees. 3 sectors per cell site: 0, 120 and 240 degrees |
| Carrier Frequency | FR1: 6 GHz; FR2: 30 GHz FR3: 7GHz/15GHz |
| Subcarrier spacing | FR1: 30 kHz; FR2: 120 kHz [May not be necessary] |
| BS height | Defined in the scenario and not necessary [remove row] |
| BS antenna configurations | Single polar antenna, and all 256 elements are mapped to a single port. (M, N, P, Mg, Ng; Mp, Np) = (16, 16, 1, 1, 1, 1, 1) For FR1: (M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;2,8) (dH,dV) = (0.5, 0.8)λ, +45°/-45° polarization For FR2: (M,N,P,Mg,Ng;Mp,Np) = (4,16,2,2,2; 1,1) (d_H,d_V ) = (0.5, 0.5)λ, (dH,g, dV,g) = (4.0, 2.0)λ, +45°/-45° polarization |
| BS antenna electrical downtilting | 102 degrees |
| Antenna virtualization | Analog beam towards to the sensing target DFT precoding according to TR 36.897 with application of panning and tilting angles |
| BS Tx power | FR1: 49dBm, 53dBm, 44dBm for UMi-AV FR2: 40dBm, 35dBm |
| Bandwidth | FR1: 100MHz FR2: 400MHz |
| BS noise figure | FR1: 5dB FR2: 7dB |
| HO margin | Not needed 0 dB |
| UT antenna configurations | (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) not needed |
| UT distribution | Not necessary. Not needed for monostatic. 10 terrestrial UEs per cell with uniform distribution |
| UT attachment | Not needed for ISAC channel model calibration Based on pathloss considering LOS angle |
| Polarized antenna modelling | Model 2 in TR 38.901 |
| UT array orientation | Not Necessary ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 90 degree, ΩUT, = 0 degree |
| UT noise figure | FR1: 9dB FR2: 10dB |
| Fast fading channel | Fast fading channel is modeled. [FFS fast fading model from 36.777] |
| Sensing target distribution | N targets uniformly distributed within a cell. Vertical distribution: between 1.5m and 300m or Fixed height value chosen from {25, 50, 100, 200, 300} m. N= {1, 2, 3, 4, 5} |
| Sensing target mobility | Horizontal: uniform distribution between 0 and 180 km/h Vertical: 0 km/h |
| Large-scale RCS for each scattering point | Mean value of target RCS |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | 10 m |
| Minimum 3D distance between sensing targets | 10 m |
| Metrics | CDF of coupling loss CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) Wideband SIR and SINR based on RSRP |

Companies can provide comments/inputs in the following table:

| Company | Yes/No | Comments |
| --- | --- | --- |

## Proposal 5-2 (FL3) – For Thursday online session

Proposal 5-2:  (HIGH) For the purposes of large scale calibration for UAV sensing targets, the following calibration parameters are proposed below in Table x.

Note: The table below is for informative purposes and the Parameter values in bold denote majority view for a given parameter.

Table x. Simulation assumptions for large scale calibration for UAV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | UMa-AV, UMi-AV, RMa-AV, UMi, UMa, RMa, |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic |
| Sectorization | 3 sectors per cell site: 30, 150 and 270 degrees. 3 sectors per cell site: 0, 120 and 240 degrees |
| Carrier Frequency | FR1: 6 GHz; FR2: 30 GHz FR3: 7GHz/15GHz |
| Subcarrier spacing | FR1: 30 kHz; FR2: 120 kHz [May not be necessary] |
| BS height | Defined in the scenario and not necessary [remove row] |
| BS antenna configurations | Single polar antenna, and all 256 elements are mapped to a single port. (M, N, P, Mg, Ng; Mp, Np) = (16, 16, 1, 1, 1, 1, 1) For FR1: (M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;2,8) (dH,dV) = (0.5, 0.8)λ, +45°/-45° polarization For FR2: (M,N,P,Mg,Ng;Mp,Np) = (4,16,2,2,2; 1,1) (d_H,d_V ) = (0.5, 0.5)λ, (dH,g, dV,g) = (4.0, 2.0)λ, +45°/-45° polarization |
| BS antenna electrical downtilting | 102 degrees |
| Antenna virtualization | Analog beam towards to the sensing target DFT precoding according to TR 36.897 with application of panning and tilting angles |
| BS Tx power | FR1: 49dBm, 53dBm, 44dBm for UMi-AV FR2: 40dBm, 35dBm |
| Bandwidth | FR1: 100MHz FR2: 400MHz |
| BS noise figure | FR1: 5dB FR2: 7dB |
| HO margin | Not needed 0 dB |
| UT antenna configurations | (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) [not needed] |
| UT distribution | Not necessary. Not needed for monostatic. 10 terrestrial UEs per cell with uniform distribution |
| UT attachment | Not needed for ISAC channel model calibration Based on pathloss considering LOS angle |
| UT noise figure | FR1: 9dB FR2: 10dB |
| Fast fading channel | Fast fading channel is not modeled |
| Sensing target distribution | N targets uniformly distributed within a cell. Vertical distribution: between 1.5m and 300m or Fixed height value chosen from {25, 50, 100, 200, 300} m. N= {1, 2, 3, 4, 5} |
| Sensing target mobility | Horizontal: uniform distribution between 0 and 180 km/h Vertical: 0 km/h |
| Large-scale RCS for each scattering point | Mean value of target RCS |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | 10 m |
| Minimum 3D distance between sensing targets | 10 m |
| Wrapping Method | Geographical distance based wrapping |
| Metrics | Coupling loss – serving cell (based on LOS pathloss) Geometry (based on LOS pathloss) with and without white noise Wideband SIR and SINR based on RSRP |

Companies can provide comments/inputs in the following table:

| Company | Yes/No | Comments |
| --- | --- | --- |

## Proposal 5-2-1 (FL4) – For Thursday session

Proposal 5-2-rev1

For the purposes of full calibration for UAV sensing targets, the following calibration parameters are proposed below in Table x.

Note 1: For large scale calibrations, fast fading is not included.

Note 2: The table below is for informative purposes and the Parameter values in bold denote majority view for a given parameter.

Table x. Simulation assumptions for full calibration for UAV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | UMa-AV |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic |
| Sectorization | 3 sectors per cell site: 30, 150 and 270 degrees |
| Carrier Frequency | FR1: 6 GHz FR2: 30 GHz |
| BS antenna configurations | For FR1: (M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;2,8) (dH,dV) = (0.5, 0.8)λ, +45°/-45° polarization For FR2: (M,N,P,Mg,Ng;Mp,Np) = (4,16,2,2,2; 1,1) (d_H,d_V ) = (0.5, 0.5)λ, (dH,g, dV,g) = (4.0, 2.0)λ, +45°/-45° polarization |
| Antenna virtualization | Option A: Analog beam towards to the sensing target Option B: DFT precoding according to TR 36.897 with application of panning and tilting angles. NOTE: Companies to provide details for antenna virtualization and beam selection, e.g., criteria for analog beam selection. |
| BS Tx power | FR1: 56dBm FR2: 40dBm |
| Bandwidth | FR1: 100MHz FR2: 400MHz |
| BS noise figure | FR1: 5dB FR2: 7dB |
| UT antenna configurations | (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| UT noise figure | FR1: 9dB FR2: 10dB |
| Fast fading channel | Fast fading channel is modeled based on TR36.777 |
| Sensing target distribution | 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m. N= {1} |
| Component A of RCS for each scattering point | A fixed value |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | 10 m |
| Minimum 3D distance between sensing targets | 10 m |
| Wrapping Method | No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping |
| Metrics | Coupling loss – serving cell (based on LOS pathloss) CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) for full calibration. FFS: Additional metrics, e.g., wideband SIR and SINR based on RSRP if interference is modelled. |

Companies can provide comments/inputs in the following table:

| Company | Yes/No | Comments |
| --- | --- | --- |
| Moderator |  | Some additional update based on offline discussion in v29. |

# Topic #4: Calibration assumptions for Automotive sensing targets

The related proposals are copied below.

| Company | Views/Proposals |
| --- | --- |
| [5] Huawei | Proposal 4: The calibration of EO Type-2 modelling can be performed under the scenario of sensing vehicle in urban grid, where the 9 buildings with the size of 413mx230mx20m and 4 walls per building in the urban grid can be considered. |
| [8] CMCC | Proposal 2: Support the channel model calibration parameters for large scale in Table 1 and channel model calibration parameters for full calibration in Table 2. Table 1. Channel model calibration parameters for large scale Parameter; UAV; Automotive Urban; Hazards. Scenario; UMa-AV 19 macro sites, 3 sectors per site; Urban grid; Highway. Sensing mode; BS-BS bi-static. BS mono-static; BS-BS bi-static. BS mono-static; BS-BS bi-static. BS mono-static. Sectorization; 3 sectors per cell site: 30, 150 and 270 degrees; 3 sectors per cell site: 30, 150 and 270 degrees; 3 sectors per cell site: 30, 150 and 270 degrees. Carrier Frequency; FR1: 6GHz; FR1: 6GHz; FR1: 6GHz. subcarrier spacing; FR1: 30kHz; FR1: 30kHz; FR1: 30kHz. BS height; 25m; 25m; 35m. BS antenna configurations; For 6GHz (M,N,P,Mg,Ng)=(32,16,2,1,1;1,1); For 6GHz (M,N,P,Mg,Ng)=(32,16,2,1,1;1,1); For 6GHz (M,N,P,Mg,Ng)=(32,16,2,1,1;1,1). BS antenna electrical downtilting; 102 degrees; 102 degrees; 102 degrees. Antenna virtualization; DFT precoding according to TR 36.897 with application of panning and tilting angles; DFT precoding according to TR 36.897 with application of panning and tilting angles; DFT precoding according to TR 36.897 with application of panning and tilting angles. Bandwidth; FR1: 100MHz; FR1: 100MHz; FR1: 100MHz. BS noise figure; FR1: 5dB; FR1: 5dB; FR1: 5dB. Fast fading channel; Fast fading channel is not modeled; Fast fading channel is not modeled; Fast fading channel is not modeled. Sensing target distribution; Horizontal:Option B: 2 targets uniformly distributed per cell. Vertical: Option B:Fixed height value chosen from {25, 50, 100, 200, 300} m; according to 37.885 Vehicle type distribution: 100% vehicle type 2. Uniform distributed in each lane. The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}.; Uniformly distributed in horizontal plane. Sensing target mobility; Horizontal velocity: uniform distribution between 0 and 180km/h. Vertical velocity: 0km/h; All the vehicles in the same lane have the same speed. Vehicle speed is 60 km/h in all the lanes.; Horizontal velocity: 10km/h. large-scale RCS for each scattering point; A = the mean value or 1 square meter.; A=1 square meter; /. Minimum 3D distances between pairs of Tx/Rx and sensing target; 10m; Min distances based on min. TRP/UE distances defined in TR37.885 as a starting point.; /. Minimum 3D distance between sensing targets; 10m; The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}.; Option 2: Fixed value with 10m. Wrapping method; distance-based wrap-around; geographical distance-based wrapping; geographical distance-based wrapping. Metrics; 1) Coupling loss – serving cell (based on LOS pathloss) 2) Geometry (based on LOS pathloss) with and without white noise. Define the interference in sensing.; 1) Coupling loss – serving cell (based on LOS pathloss) 2) Geometry (based on LOS pathloss) with and without white noise. Define the interference in sensing.; 1) Coupling loss – serving cell (based on LOS pathloss) 2) Geometry (based on LOS pathloss) with and without white noise. Define the interference in sensing.. Selection of Tx-Rx pair for metrics statistcs; The Tx-Rx pair with the largest large-scale RSRP. Note the TX and RX should not belong the same site under bistatic mode.; The Tx-Rx pair with the largest large-scale RSRP. Note the TX and RX should not belong the same site under bistatic mode.; The Tx-Rx pair with the largest large-scale RSRP. Note the TX and RX should not belong the same site under bistatic mode.. Table 2. Channel model calibration parameters for full calibration Parameter; Values; Values; Values. Scenario; UMa-AV 19 macro sites, 3 sectors per site; Urban grid; Highway. Sensing mode; BS-BS bi-static. BS mono-static; BS-BS bi-static. BS mono-static; BS-BS bi-static. BS mono-static. Sectorization; 3 sectors per cell site: 30, 150 and 270 degrees; 3 sectors per cell site: 30, 150 and 270 degrees; 3 sectors per cell site: 30, 150 and 270 degrees. Carrier Frequency; FR1: 6GHz; FR1: 6GHz; FR1: 6GHz. subcarrier spacing; FR1: 30kHz; FR1: 30kHz; FR1: 30kHz. BS height; 25m; 25m; 35m. BS antenna configurations; For 6GHz (M,N,P,Mg,Ng)=(32,16,2,1,1,1,1); For 6GHz (M,N,P,Mg,Ng)=(32,16,2,1,1,1,1); For 6GHz (M,N,P,Mg,Ng)=(32,16,2,1,1,1,1). BS antenna electrical downtilting; 102 degrees; 102 degrees; 102 degrees. Antenna virtualization; DFT precoding according to TR 36.897 with application of panning and tilting angles; DFT precoding according to TR 36.897 with application of panning and tilting angles; DFT precoding according to TR 36.897 with application of panning and tilting angles. Bandwidth; FR1: 100MHz; FR1: 100MHz; FR1: 100MHz. BS noise figure; FR1: 5dB; FR1: 5dB; FR1: 5dB. Fast fading channel; Fast fading channel is modeled; Fast fading channel is modelled; Fast fading channel is modeled. Sensing target distribution; Horizontal: Option B: 2 targets uniformly distributed per cell. Vertical: Option B: Fixed height value chosen from {25, 50, 100, 200, 300} m; according to 37.885 Vehicle type distribution: 100% vehicle type 2. Uniform distributed in each lane. The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}.; Uniformly distributed in horizontal plane. Sensing target mobility; Horizontal velocity: uniform distribution between 0 and 180km/h. Vertical velocity: 0km/h; All the vehicles in the same lane have the same speed. Vehicle speed is 60 km/h in all the lanes.; Horizontal velocity: 10km/h. large-scale RCS for each scattering point; A = the mean value or 1 square meter.; A=1 square meter; /. Minimum 3D distances between pairs of Tx/Rx and sensing target; 10m; Min distances based on min. TRP/UE distances defined in TR37.885 as a starting point.; /. Minimum 3D distance between sensing targets; 10m; The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}.; Option 2: Fixed value with 10m. Wrapping method; distance-based wrap-around; geographical distance-based wrapping; geographical distance-based wrapping. Metrics; 1) CDF of coupling loss 2) CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) according to circular angle spread definition of TR 25.996; 1) CDF of coupling loss 2) CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) according to circular angle spread definition of TR 25.996; 1) CDF of coupling loss 2) CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) according to circular angle spread definition of TR 25.996. |
| [11] EURECOM | Table 5. The parameters of CM calibration for full calibration for Automotive vehicles use cases Parameters; Values. Scenario; Urban grid, Highway. Sensing mode; TRP bistatic/monostatic. Sectorization; 3 sectors per site, 30, 150 and 270 degrees. Carrier Frequency; FR1: 6 GHz FR2: 30 GHz. Subcarrier spacing; FR1: 30 kHz FR2: 120 kHz. BS height; Urban grid: 25m Highway: 35m. BS antenna configurations; (M,N,P,Mg,Ng;Mp,Np)=(16,16,1,1,1,1,1). BS antenna electrical downtilting; 102 degrees. Antenna virtualization; DFT precoding according to TR 36.897 with application of panning and tilting angles. BS Tx power; FR1: 49dBm FR2: 43dBm. Bandwidth; FR1: 100MHz FR2: 400MHz. BS noise figure; FR1: 5dB FR2: 7dB. UT antenna configurations; (M,N,P,Mg,Ng;Mp,Np) = (2,2,1,1,1,1,1). UT distribution; Uniform distribution. UT attachment; Based on pathloss. Polarized antenna modelling; Model 2 in TR 38.901. UT noise figure; FR1:9dB FR2:13dB. Fast fading channel; Fast fading channel is modeled. Sensing target distribution; Dropping option A in 37.885. Sensing target mobility; Based on Option A deployment in TR37.885 for Urban grid. Large-scale RCS for each scattering point; The mean value of target RCS. Minimum 3D distances between pairs of Tx/Rx and sensing target; Min distances based on min. TRP/UE distances defined in TR37.885 as a starting point.. Minimum 3D distance between sensing targets; 10 m. Metrics; CDF of coupling loss CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA). UT height; Vehicle UT: 1.6m Pedestrian UE: 1.5m RSU: 5m. UT Tx power; 23dBm. The parameters of CM calibration for large scale for Automotive vehicles use cases are shown in Table 6. Table 6. The parameters of CM calibration for large scale for Automotive vehicles use cases Parameters; Values. Scenario; Urban grid, Highway. Sensing mode; TRP bistatic/monostatic. Sectorization; 3 sectors per site, 30, 150 and 270 degrees. Carrier Frequency; FR1: 6 GHz FR2: 30 GHz. Subcarrier spacing; FR1: 30 kHz FR2: 120 kHz. BS height; Urban grid: 25m Highway: 35m. BS antenna configurations; (M,N,P,Mg,Ng;Mp,Np)=(16,16,1,1,1,1,1). BS antenna electrical downtilting; 102 degrees. Antenna virtualization; DFT precoding according to TR 36.897 with application of panning and tilting angles. BS Tx power; FR1: 49dBm FR2: 43dBm. Bandwidth; FR1: 100MHz FR2: 400MHz. BS noise figure; FR1: 5dB FR2: 7dB. UT antenna configurations; (M,N,P,Mg,Ng;Mp,Np) = (2,2,1,1,1,1,1). UT distribution; Uniform distribution. UT attachment; Based on pathloss. Polarized antenna modelling; Model 2 in TR 38.901. UT noise figure; FR1:9dB FR2:13dB. Fast fading channel; Fast fading channel is not modeled. Sensing target distribution; Dropping option A in 37.885. Sensing target mobility; Based on Option A deployment in TR37.885 for Urban grid. Large-scale RCS for each scattering point; The mean value of target RCS. Minimum 3D distances between pairs of Tx/Rx and sensing target; Min distances based on min. TRP/UE distances defined in TR37.885 as a starting point.. Minimum 3D distance between sensing targets; 10 m. Metrics; Coupling loss based on PL Wideband SIR and SINR based on geometry (based on pathloss). Proposal 5: The parameters of CM calibration for full calibration for Automotive vehicles use cases are shown in Table 5. Proposal 6: The parameters of CM calibration for large scale for Automotive vehicles use cases are shown in Table 6. |
| [19] NVIDIA | Proposal 4: For the urban grid defined for V2X in TR 37.885, the scene geometry can be described by including assumption on building height, and assumptions on the materials for the buildings and roads can be included. Proposal 5: Agree on the evaluation parameters for urban grid scenario as in Table 1. Environment Objects, e.g., types, characteristics, mobility, distribution, etc.; EO Type 2 for Urban Grid - Building size: 4 walls modelled as EO type 2, per building of size 413m x 230m x 20m - Number of buildings: 9 as laid out in urban grid scenario, resulting in 36 walls in total - Building material: Concrete - Ground material: Medium dry ground. |
| [26] Continental | Proposal 1: ISAC channel model calibration for automotive should focus on Urban Grid and highway. In case of prioritization, Urban Grid should be addressed first. Proposal 2: ISAC channel model calibration for automotive should consider, as much as possible, the parameters and guidelines indicated in TR 37.885 and TR 38.901, with the corresponding adaptations and considerations required by the ISAC framework (e.g., target channel, background channel, and sensing targets aspects). Proposal 3: For the ISAC channel model calibration for automotive scenarios, RAN1 should consider ISAC-capable (road/street-level) infrastructure, e.g., UE-RSU. Proposal 4: For the ISAC channel model calibration for automotive scenarios, a step-wise approach (which consists of deterministic models for the Tx/Rx-target channels, background channels and combination of the target/background channels) shall be used/adopted by RAN1. Proposal 5: For the ISAC channel model calibration for automotive scenarios, sensing targets shall cover, at least, some of the ones indicated in R1-2405964, such as vehicles, pedestrians, bikes, etc. Proposal 6: For the ISAC channel model calibration for automotive scenarios, RAN1 should consider the parameters/values indicated in Table 2.1. |

Moderator Discussion: Based on the many proposals and majority views/proposals, the following proposals are made to further converge the deployment scenario for Automotive targets:

## Proposal 6-1 (FL1)

Proposal 6-1(MED) TBD pending above agreements

Companies can provide comments/inputs in the following table:

| Company | Yes/No | Comments |
| --- | --- | --- |

# Topic #5: Calibration assumptions for Humans sensing targets (outdoors/indoors)

The related proposals are copied below.

| Company | Views/Proposals |
| --- | --- |
| [11] EURECOM | Table 3. The parameters of CM calibration for full calibration for human indoor/outdoor use cases Parameters; Indoor values; Outdoor values. Scenario; Indoor office 120m*50m*3m ISD: 20m; UMa, UMi. Sensing mode; All sensing modes; All sensing modes. Sectorization; single sector per site; 3 sectors per site. Carrier Frequency; FR1: 6 GHz; FR2: 30 GHz; FR1: 6 GHz; FR2: 30 GHz. Subcarrier spacing; FR1: 30 kHz; FR2: 120 kHz; FR1: 30 kHz; FR2: 120 kHz. BS height; 3m; UMa: 25m; UMi: 10m. BS antenna configurations; For FR1: (M,N,P,Mg,Ng;Mp,Np) = (4,4,2,1,1; 4,4) , (dH,dV) = (0.5, 0.5)λ, +45°/-45° polarization For FR2: (M,N,P,Mg,Ng;Mp,Np)= (16,8,2,1,1; 1,1) , (dH,dV) = (0.5, 0.5)λ, +45°/-45° polarization; [M,N,P,Mg,Ng]=[4,4,2,1,2], dv=dh=0.5λ, dvg=dhg=2.5λ. BS antenna electrical downtilting; 102 degrees; 102 degrees. Antenna virtualization; DFT precoding according to TR 36.897 with application of panning and tilting angles. BS Tx power; FR1: 24dBm FR2: 23dBm; FR1: UMa: 49dBm; UMi: 44dBm FR2: UMa: 37dBm; UMi: 37dBm. Bandwidth; FR1: 100MHz FR2: 400MHz; FR1: 100MHz FR2: 400MHz. BS noise figure; FR1: 5dB FR2: 7dB; FR1: 5dB FR2: 7dB. UT antenna configurations; (M,N,P,Mg,Ng;Mp,Np) = (2,2,1,1,1;1,1); (M,N,P,Mg,Ng;Mp,Np) = (2,2,1,1,1;1,1). UT distribution; 100% indoor, uniformly distributed over the area of the convex hull of the TRP deployment; uniformly distributed. UT attachment; /; /. Polarized antenna modelling; Model 2 in TR 38.901. UT noise figure; FR1:9dB FR2:10dB; FR1:9dB FR2:10dB. Fast fading channel; Fast fading channel is modeled; Fast fading channel is modeled. Sensing target distribution; 5 targets uniformly distributed over the horizontal area of the convex hull of the TRP deployment; 5 sensing targets uniformly distributed within the center cell. Sensing target mobility; 3 km/h; 3 km/h. Large-scale RCS for each scattering point; The mean value of target RCS; The mean value of target RCS. Minimum 3D distances between pairs of Tx/Rx and sensing target; TR 38.901 as a starting point; TR 38.901 as a starting point. Minimum 3D distance between sensing targets; At least larger than the physical size of a sensing target; At least larger than the physical size of a sensing target. Metrics; CDF of coupling loss CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA); CDF of coupling loss CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA). UT height; 1.5m; 1.5m. UT Tx power; 23dB; 23dB. Table 4. The parameters of CM calibration for large scale for human indoor/outdoor use cases Parameters; Indoor values; Outdoor values. Scenario; Indoor office 120m*50m*3m ISD: 20m; UMa, UMi. Sensing mode; All sensing modes; All sensing modes. Sectorization; single sector per site; 3 sectors per site. Carrier Frequency; FR1: 6 GHz; FR2: 30 GHz; FR1: 6 GHz; FR2: 30 GHz. Subcarrier spacing; FR1: 30 kHz; FR2: 120 kHz; FR1: 30 kHz; FR2: 120 kHz. BS height; 3m; UMa: 25m; UMi: 10m. BS antenna configurations; For FR1: (M,N,P,Mg,Ng;Mp,Np) = (4,4,2,1,1; 4,4) , (dH,dV) = (0.5, 0.5)λ, +45°/-45° polarization For FR2: (M,N,P,Mg,Ng;Mp,Np)= (16,8,2,1,1; 1,1) , (dH,dV) = (0.5, 0.5)λ, +45°/-45° polarization; [M,N,P,Mg,Ng]=[4,4,2,1,2], dv=dh=0.5λ, dvg=dhg=2.5λ. BS antenna electrical downtilting; 102 degrees; 102 degrees. Antenna virtualization; DFT precoding according to TR 36.897 with application of panning and tilting angles. BS Tx power; FR1: 24dBm FR2: 23dBm; FR1: UMa: 49dBm; UMi: 44dBm FR2: UMa: 37dBm; UMi: 37dBm. Bandwidth; FR1: 100MHz FR2: 400MHz; FR1: 100MHz FR2: 400MHz. BS noise figure; FR1: 5dB FR2: 7dB; FR1: 5dB FR2: 7dB. UT antenna configurations; (M,N,P,Mg,Ng;Mp,Np) = (2,2,1,1,1;1,1); (M,N,P,Mg,Ng;Mp,Np) = (2,2,1,1,1;1,1). UT distribution; 100% indoor, uniformly distributed over the area of the convex hull of the TRP deployment; uniformly distributed. UT attachment; /; /. Polarized antenna modelling; Model 2 in TR 38.901. UT noise figure; FR1:9dB FR2:10dB; FR1:9dB FR2:10dB. Fast fading channel; Fast fading channel is not modeled; Fast fading channel is not modeled. Sensing target distribution; 5 targets uniformly distributed over the horizontal area of the convex hull of the TRP deployment; 5 sensing targets uniformly distributed within the center cell. Sensing target mobility; 3 km/h; 3 km/h. Large-scale RCS for each scattering point; The mean value of target RCS; The mean value of target RCS. Minimum 3D distances between pairs of Tx/Rx and sensing target; TR 38.901 as a starting point; TR 38.901 as a starting point. Minimum 3D distance between sensing targets; At least larger than the physical size of a sensing target; At least larger than the physical size of a sensing target. Metrics; Coupling loss – serving cell (based on LOS pathloss) Geometry (based on LOS pathloss) with and without white noise.; Coupling loss – serving cell (based on LOS pathloss) Geometry (based on LOS pathloss) with and without white noise.. Proposal 3: The parameters of CM calibration for full calibration for human indoor/outdoor use cases are shown in Table 3. Proposal 4: The parameters of CM calibration for large scale for human indoor/outdoor use cases are shown in Table 4. |
| [13] OPPO | Table-1 Simulation assumptions for human indoor/outdoor scenario Parameter; Values. Scenario; Indoor office: 120m*50m*3m ISD=20m. Sensing mode; gNB bi-static, gNB-UE bi-static, UE bi-static. Sectorization; Single sector. Carrier Frequency; FR1:6GHz FR2:30GHz. subcarrier spacing; Although we did not identify where in calibration to use SCS: FR1: selective from {15,30}kHz; FR2: 120KHz. BS height; 3m. BS antenna configurations; (M,N,P,M_g,N_g;M_p,N_p ) = (4,4,2,1,1; 4,4) (d_H,d_V )= (0.5, 0.5)λ, +45°/-45° polarization. BS antenna electrical downtilting; Value from Table 7.8-1 (38.901): 110 degree. Antenna virtualization; DFT precoding according to TR 36.897 with application of panning and tilting angles. BS Tx power; Value from Table 7.8-1 (38.901): 24dBm. Bandwidth; FR1/FR2: 100MHz. BS noise figure; 5dB. UT antenna configurations; (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization. Handover margin (for calibration); value from Table 7.8-1 (38.901): 0dB. UT distribution; Horizontal plane: uniformly distributed within the scenario deployment area Vertical plane:1.5m. UT attachment; Based on pathloss considering LOS angle. Polarized antenna modelling; Either Model-1 or Model-2. UT array orientation; ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 90 degree, ΩUT, = 0 degree. UT noise figure; 9dB. Fast fading channel; Fast fading channel is modelled. Sensing target distribution; N<=5 targets uniformly distributed over the horizontal area of the convex hull of the TRP deployment. Multi-drops.. Sensing target mobility; 0km/h or 3km/h. large-scale RCS for each scattering point; A=1m2. EO distribution; Not modeled. Minimum 3D distances between pairs of Tx/Rx and sensing target; 1m. Minimum 3D distance between sensing targets. Wrapping method; No wrap-around. Metrics; 1) Coupling loss (but need a RAN1 agreement on the definition); 2) CDF of power/delay/angle spread 3) Profiles of delay-to-power, angle-to-power and delay-angle-to-power. 4) SNR/SIR, where the “S” part may refer to useful rays for sensing instead of all rays carrying the sensing signal energy. FFS whether to separate 2) and 3) for LOS+LOS rays, LOS+NLOS rays, NLOS+LOS rays and NLOS+NLOS rays.. additional parameters?. UT height; 1.5m. UT Tx power; 23dBm. Assumptions of power relationship between target channel and background channel; Power normalization according to RAN1 further agreement. Note: This is also one of reasons for us to suggest merging the large-scale calibration into full calibration because the large-scale factor needs to be added before power normalization over target/background channels.. Proposal 4: Use simulation assumptions in Table 1 for calibration of human indoor/outdoor scenarios. |

Moderator Discussion: Based on the many proposals and majority views/proposals, the following proposals are made to further converge the deployment scenario for human (indoor/outdoor) targets:

## Proposal 7-1 (FL1)

Proposal 7-1:  (MED) TBD pending above agreements

Companies can provide comments/inputs in the following table:

| Company | Yes/No | Comments |
| --- | --- | --- |

# Topic #6: Calibration assumptions for AGV sensing targets (outdoors/indoors)

The related proposals are copied below.

| Company | Views/Proposals |
| --- | --- |
| [11] EURECOM | Table 7. The parameters of CM calibration for full calibration for AGV use cases Parameters; Values. Scenario; Indoor factory. Sensing mode; TRP bistatic/monostatic. Sectorization; 3 sectors per site, 30, 150 and 270 degrees. Carrier Frequency; FR1: 6 GHz FR2: 30 GHz. Subcarrier spacing; FR1: 30 kHz FR2: 120 kHz. BS height; 8m. BS antenna configurations; (M,N,P,Mg,Ng;Mp,Np)=(16,16,1,1,1,1,1). BS antenna electrical downtilting; /. Antenna virtualization; /. BS Tx power; 24dBm. Bandwidth; FR1: 100MHz FR2: 400MHz. BS noise figure; FR1: 5dB FR2: 7dB. UT antenna configurations; /. UT distribution; Uniform distribution. UT attachment; Based on RSRP. Polarized antenna modelling; Model 2 in TR 38.901. UT noise figure; FR1:9dB FR2:13dB. Fast fading channel; Fast fading channel is modeled. Sensing target distribution; 5 targets uniformly distributed in the convex hull of the horizontal BS deployment. Sensing target mobility; Uniform distribution between 0km/h and 30km/h. Large-scale RCS for each scattering point; The mean value of target RCS. Minimum 3D distances between pairs of Tx/Rx and sensing target; 0m. Minimum 3D distance between sensing targets; At least larger than the physical size of a sensing target. Metrics; CDF of coupling loss CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA). The parameters of CM calibration for large scale for AGV use cases are shown in Table 8. Table 8. The parameters of CM calibration for large scale for AGV use cases Parameters; Values. Scenario; Indoor factory. Sensing mode; TRP bistatic/monostatic. Sectorization; 3 sectors per site, 30, 150 and 270 degrees. Carrier Frequency; FR1: 6 GHz FR2: 30 GHz. Subcarrier spacing; FR1: 30 kHz FR2: 120 kHz. BS height; 8m. BS antenna configurations; (M,N,P,Mg,Ng;Mp,Np)=(16,16,1,1,1,1,1). BS antenna electrical downtilting; /. Antenna virtualization; /. BS Tx power; 24dBm. Bandwidth; FR1: 100MHz FR2: 400MHz. BS noise figure; FR1: 5dB FR2: 7dB. UT antenna configurations; /. UT distribution; Uniform distribution. UT attachment; Based on pathloss. Polarized antenna modelling; Model 2 in TR 38.901. UT noise figure; FR1:9dB FR2:13dB. Fast fading channel; Fast fading channel is not modeled. Sensing target distribution; 5 targets uniformly distributed in the convex hull of the horizontal BS deployment. Sensing target mobility; Uniform distribution between 0km/h and 30km/h. Large-scale RCS for each scattering point; The mean value of target RCS. Minimum 3D distances between pairs of Tx/Rx and sensing target; 0m. Minimum 3D distance between sensing targets; At least larger than the physical size of a sensing target. Metrics; Coupling loss based on PL Wideband SIR and SINR. Proposal 7: The parameters of CM calibration for full calibration for AGV use cases are shown in Table 7. Proposal 8: The parameters of CM calibration for large scale for AGV use cases are shown in Table 8. |

## 8.1 Proposal 8-1 (FL1)

Proposal 8-1:  (MED) TBD Pending above agreements:

| Company | Yes/No | Comments |
| --- | --- | --- |

# Topic #7: Calibration assumptions for Hazardous objects sensing targets

The related proposals are copied below.

| Company | Views/Proposals |
| --- | --- |
| [8] CMCC | Proposal 2: Support the channel model calibration parameters for large scale in Table 1 and channel model calibration parameters for full calibration in Table 2. Table 1. Channel model calibration parameters for large scale Parameter; UAV; Automotive Urban; Hazards. Scenario; UMa-AV 19 macro sites, 3 sectors per site; Urban grid; Highway. Sensing mode; BS-BS bi-static. BS mono-static; BS-BS bi-static. BS mono-static; BS-BS bi-static. BS mono-static. Sectorization; 3 sectors per cell site: 30, 150 and 270 degrees; 3 sectors per cell site: 30, 150 and 270 degrees; 3 sectors per cell site: 30, 150 and 270 degrees. Carrier Frequency; FR1: 6GHz; FR1: 6GHz; FR1: 6GHz. subcarrier spacing; FR1: 30kHz; FR1: 30kHz; FR1: 30kHz. BS height; 25m; 25m; 35m. BS antenna configurations; For 6GHz (M,N,P,Mg,Ng)=(32,16,2,1,1;1,1); For 6GHz (M,N,P,Mg,Ng)=(32,16,2,1,1;1,1); For 6GHz (M,N,P,Mg,Ng)=(32,16,2,1,1;1,1). BS antenna electrical downtilting; 102 degrees; 102 degrees; 102 degrees. Antenna virtualization; DFT precoding according to TR 36.897 with application of panning and tilting angles; DFT precoding according to TR 36.897 with application of panning and tilting angles; DFT precoding according to TR 36.897 with application of panning and tilting angles. Bandwidth; FR1: 100MHz; FR1: 100MHz; FR1: 100MHz. BS noise figure; FR1: 5dB; FR1: 5dB; FR1: 5dB. Fast fading channel; Fast fading channel is not modeled; Fast fading channel is not modeled; Fast fading channel is not modeled. Sensing target distribution; Horizontal:Option B: 2 targets uniformly distributed per cell. Vertical: Option B:Fixed height value chosen from {25, 50, 100, 200, 300} m; according to 37.885 Vehicle type distribution: 100% vehicle type 2. Uniform distributed in each lane. The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}.; Uniformly distributed in horizontal plane. Sensing target mobility; Horizontal velocity: uniform distribution between 0 and 180km/h. Vertical velocity: 0km/h; All the vehicles in the same lane have the same speed. Vehicle speed is 60 km/h in all the lanes.; Horizontal velocity: 10km/h. large-scale RCS for each scattering point; A = the mean value or 1 square meter.; A=1 square meter; /. Minimum 3D distances between pairs of Tx/Rx and sensing target; 10m; Min distances based on min. TRP/UE distances defined in TR37.885 as a starting point.; /. Minimum 3D distance between sensing targets; 10m; The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}.; Option 2: Fixed value with 10m. Wrapping method; distance-based wrap-around; geographical distance-based wrapping; geographical distance-based wrapping. Metrics; 1) Coupling loss – serving cell (based on LOS pathloss) 2) Geometry (based on LOS pathloss) with and without white noise. Define the interference in sensing.; 1) Coupling loss – serving cell (based on LOS pathloss) 2) Geometry (based on LOS pathloss) with and without white noise. Define the interference in sensing.; 1) Coupling loss – serving cell (based on LOS pathloss) 2) Geometry (based on LOS pathloss) with and without white noise. Define the interference in sensing.. Selection of Tx-Rx pair for metrics statistcs; The Tx-Rx pair with the largest large-scale RSRP. Note the TX and RX should not belong the same site under bistatic mode.; The Tx-Rx pair with the largest large-scale RSRP. Note the TX and RX should not belong the same site under bistatic mode.; The Tx-Rx pair with the largest large-scale RSRP. Note the TX and RX should not belong the same site under bistatic mode.. Table 2. Channel model calibration parameters for full calibration Parameter; Values; Values; Values. Scenario; UMa-AV 19 macro sites, 3 sectors per site; Urban grid; Highway. Sensing mode; BS-BS bi-static. BS mono-static; BS-BS bi-static. BS mono-static; BS-BS bi-static. BS mono-static. Sectorization; 3 sectors per cell site: 30, 150 and 270 degrees; 3 sectors per cell site: 30, 150 and 270 degrees; 3 sectors per cell site: 30, 150 and 270 degrees. Carrier Frequency; FR1: 6GHz; FR1: 6GHz; FR1: 6GHz. subcarrier spacing; FR1: 30kHz; FR1: 30kHz; FR1: 30kHz. BS height; 25m; 25m; 35m. BS antenna configurations; For 6GHz (M,N,P,Mg,Ng)=(32,16,2,1,1,1,1); For 6GHz (M,N,P,Mg,Ng)=(32,16,2,1,1,1,1); For 6GHz (M,N,P,Mg,Ng)=(32,16,2,1,1,1,1). BS antenna electrical downtilting; 102 degrees; 102 degrees; 102 degrees. Antenna virtualization; DFT precoding according to TR 36.897 with application of panning and tilting angles; DFT precoding according to TR 36.897 with application of panning and tilting angles; DFT precoding according to TR 36.897 with application of panning and tilting angles. Bandwidth; FR1: 100MHz; FR1: 100MHz; FR1: 100MHz. BS noise figure; FR1: 5dB; FR1: 5dB; FR1: 5dB. Fast fading channel; Fast fading channel is modeled; Fast fading channel is modelled; Fast fading channel is modeled. Sensing target distribution; Horizontal: Option B: 2 targets uniformly distributed per cell. Vertical: Option B: Fixed height value chosen from {25, 50, 100, 200, 300} m; according to 37.885 Vehicle type distribution: 100% vehicle type 2. Uniform distributed in each lane. The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}.; Uniformly distributed in horizontal plane. Sensing target mobility; Horizontal velocity: uniform distribution between 0 and 180km/h. Vertical velocity: 0km/h; All the vehicles in the same lane have the same speed. Vehicle speed is 60 km/h in all the lanes.; Horizontal velocity: 10km/h. large-scale RCS for each scattering point; A = the mean value or 1 square meter.; A=1 square meter; /. Minimum 3D distances between pairs of Tx/Rx and sensing target; 10m; Min distances based on min. TRP/UE distances defined in TR37.885 as a starting point.; /. Minimum 3D distance between sensing targets; 10m; The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}.; Option 2: Fixed value with 10m. Wrapping method; distance-based wrap-around; geographical distance-based wrapping; geographical distance-based wrapping. Metrics; 1) CDF of coupling loss 2) CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) according to circular angle spread definition of TR 25.996; 1) CDF of coupling loss 2) CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) according to circular angle spread definition of TR 25.996; 1) CDF of coupling loss 2) CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) according to circular angle spread definition of TR 25.996. |
| [11] EURECOM | Table 9. The parameters of CM calibration for full calibration for objects creating hazards on roads/railways use cases Parameters; Values. Scenario; Highway. Sensing mode; All 6 sensing modes. Sectorization; 3 sectors per site, 30, 150 and 270 degrees. Carrier Frequency; FR1: 6 GHz FR2: 30 GHz. Subcarrier spacing; FR1: 30 kHz FR2: 120 kHz. BS height; 35m. BS antenna configurations; (M,N,P,Mg,Ng;Mp,Np)=(16,16,1,1,1,1,1). BS antenna electrical downtilting; 102 degree. Antenna virtualization; DFT precoding according to TR 36.897 with application of panning and tilting angles. BS Tx power; FR1: 49dBm FR2: 43dBm. Bandwidth; FR1: 100MHz FR2: 400MHz. BS noise figure; FR1: 5dB FR2: 7dB. UT antenna configurations; (M,N,P,Mg,Ng;Mp,Np)=(2,2,1,1,1,1,1). UT distribution; /. UT attachment; /. Polarized antenna modelling; Model 2 in TR 38.901. UT noise figure; FR1:9dB FR2:13dB. Fast fading channel; Fast fading channel is modeled. Sensing target distribution; Uniformly distributed in horizontal plane. Sensing target mobility; 10km/h. Large-scale RCS for each scattering point; /. Minimum 3D distances between pairs of Tx/Rx and sensing target; 35m. Minimum 3D distance between sensing targets; 10m. Metrics; CDF of coupling loss CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA). UT height; Vehicle UT: 1.6m Pedestrian UE: 1.5m RSU: 5m. UT Tx power; 23dBm. The parameters of CM calibration for large scale for objects creating hazards on roads/railways use cases are shown in Table 10. Table 10. The parameters of CM calibration for large scale for objects creating hazards on roads/railways use cases Parameters; Values. Scenario; Highway. Sensing mode; All 6 sensing modes. Sectorization; 3 sectors per site, 30, 150 and 270 degrees. Carrier Frequency; FR1: 6 GHz FR2: 30 GHz. Subcarrier spacing; FR1: 30 kHz FR2: 120 kHz. BS height; 35m. BS antenna configurations; (M,N,P,Mg,Ng;Mp,Np)=(16,16,1,1,1,1,1). BS antenna electrical downtilting; 102 degree. Antenna virtualization; DFT precoding according to TR 36.897 with application of panning and tilting angles. BS Tx power; FR1: 49dBm FR2: 43dBm. Bandwidth; FR1: 100MHz FR2: 400MHz. BS noise figure; FR1: 5dB FR2: 7dB. UT antenna configurations; (M,N,P,Mg,Ng;Mp,Np)=(2,2,1,1,1,1,1). UT distribution; /. UT attachment; /. Polarized antenna modelling; Model 2 in TR 38.901. UT noise figure; FR1:9dB FR2:13dB. Fast fading channel; Fast fading channel is not modeled. Sensing target distribution; Uniformly distributed in horizontal plane. Sensing target mobility; 10km/h. Large-scale RCS for each scattering point; /. Minimum 3D distances between pairs of Tx/Rx and sensing target; 35m. Minimum 3D distance between sensing targets; 10m. Metrics; CDF of coupling loss. Proposal 9: The parameters of CM calibration for full calibration for objects creating hazards on roads/railways use cases are shown in Table 9. Proposal 10: The parameters of CM calibration for large scale for objects creating hazards on roads/railways use cases are shown in Table 10. |

## Proposal 9-1 (FL1)

Proposal 9-1:  (MED) TBD pending above discussions

| Company | Yes/No | Comments |
| --- | --- | --- |

# References

- RP-242348, 	“Revised SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR”, Xiaomi, AT&T

- R1-2410846	Session notes for 9.7 (Study on channel modelling for Integrated Sensing And Communication for NR), Ad-Hoc Chair (Huawei). RAN1#119.

- R1-2500059	Email discussion summary on ISAC CM calibration assumptions	Moderator (AT&T)

- R1-2500998	Draft CR for TR 38.901 to introduce channel model for ISAC	Xiaomi, AT&T

- R1-2500071	Deployment scenarios for ISAC channel model	Huawei, HiSilicon

- R1-2500234	Discussion on ISAC deployment scenarios	CATT, CICTCI

- R1-2500266	Discussion on ISAC deployment scenarios	China Telecom

- R1-2500297	Discussion on ISAC deployment scenarios	CMCC, China Southern Power Grid

- R1-2500312	Discussion on ISAC scenario			CALTTA

- R1-2500360	Views on Rel-19 ISAC deployment scenarios	vivo

- R1-2500413	Discussion on ISAC deployment scenarios and requirements	EURECOM

- R1-2500417	Discussion on ISAC deployment scenarios	Tiami Networks

- R1-2500462	Discussion on ISAC deployment scenarios and calibration	OPPO

- R1-2500482	Discussion on ISAC Deployment Scenarios	Tejas Network Limited

- R1-2500576	Discussion on ISAC deployment scenarios	ZTE Corporation, Sanechips

- R1-2500659	Discussion on ISAC deployment scenarios	Sony

- R1-2500679	Discussion on ISAC Deployment Scenarios	Nokia, Nokia Shanghai Bell

- R1-2500684	Discussion on ISAC deployment scenarios	SK Telecom

- R1-2500691	Deployment scenarios for integrated sensing and communication with NR	NVIDIA

- R1-2500742	Deployment scenarios and evaluation assumptions for ISAC channel model	Xiaomi

- R1-2500747	Discussion on ISAC deployment scenarios	InterDigital, Inc.

- R1-2500755	Discussion on ISAC Deployment Scenarios	Ericsson

- R1-2500796	Discussion on ISAC deployment scenarios	Apple

- R1-2500860	Discussion on  ISAC deployment scenarios	Samsung

- R1-2500891	Considerations on ISCA deployment scenarios	CAICT

- R1-2501011	Discussion on ISAC deployment scenarios for Automotive	Continental Automotive

- R1-2501026	Discussion on ISAC deployment scenario	MediaTek Inc.

- R1-2501045	Discussion on ISAC deployment scenarios	LG Electronics

- R1-2501059	Discussion on ISAC deployment scenarios	Lenovo

- R1-2501076	FL Summary #1 on ISAC Scenarios and Calibrations	Moderator (AT&T)

- R1-2501077	FL Summary #2 on ISAC Scenarios and Calibrations	Moderator (AT&T)

- R1-2501078	FL Summary #3 on ISAC Scenarios and Calibrations	Moderator (AT&T)

- R1-2501081	ISAC channel model calibration and scenario parameters	AT&T, FirstNet

- R1-2501135	Discussion on ISAC channel calibration	BUPT, CMCC

- R1-2501166	Discussion on ISAC deployment scenarios	Qualcomm Incorporated

# Annex: RAN1 Endorsed agreements

RAN1#116 Agreements

Agreement

For progressing ISAC study, the following sensing targets and existing communication scenarios will be considered as a starting point:

Note1: the table below does not imply that the sensing target will be placed at positions defined for UEs and BSs in the scenarios in the right column.

Note2: the table below does not imply that UEs are necessarily placed at positions defined for UEs in the scenarios in the right column.

Note3: the existing communication scenarios are listed with the intent to use the evaluation parameters defined for those scenarios, as a starting point.

| Sensing Targets | scenarios |
| --- | --- |
| UAVs | RMa-AV, UMa-AV, UMi-AV (TR 36.777) |
| Humans indoors | InF, Indoor Office, [Indoor Room (TR 38.808)], [UMi, UMa] |
| Humans outdoors | UMi, UMa, [RMa] |
| Automotive vehicles (at least outdoors) | Highway, Urban grid, UMa, UMi, RMa |
| Automated guided vehicles (e.g. in indoor factories) | InF |
| Objects creating hazards on roads/railways (examples defined in TR 22.837) | Highway, Urban grid, HST |

Agreement

For ISAC channel modelling, RAN1 uses the sensing related terminology as defined in TS22.137 or TR22.837 as a starting point for discussion purposes with the following definitions:

- Sensing transmitter: the TRP or a UE that sends out the sensing signal which the sensing service will use in its operation. A sensing transmitter can be located in the same or different TRP or a UE as the sensing receiver.

- Sensing receiver: the TRP or a UE that receives the sensing signal which the sensing service will use in its operation. A sensing receiver can be located in the same or different TRP or a UE as the sensing transmitter.

- Sensing target: target that need to be sensed by deriving characteristics of the objects within the environment from the sensing signal.

- Background environment: background (clutter and/or environmental objects) that are not the sensing target(s).

- Mono-static sensing: sensing where the sensing transmitter and sensing receiver are co-located in the same TRP or UE.

- Bi-static sensing: sensing where the sensing transmitter and sensing receiver are in different TRPs or UEs.

- Multi-static sensing: sensing where there are multiple sensing transmitters and/or multiple sensing receivers, for a sensing target.

- Sensing signal: Transmissions on the 3GPP radio interface that can be used for sensing purposes.

RAN1#116-bis Agreements

Agreement

RAN1 agrees the following ISAC terminology with minor modifications as follows:

For ISAC channel modelling, RAN1 uses the sensing related terminology as defined in TS22.137 or TR22.837 as a starting point for discussion purposes with the following definitions:

- Sensing transmitter: the TRP or a UE that sends out the sensing signal which the sensing service will use in its operation. A sensing transmitter can be located in the same or different TRP or a UE as the sensing receiver.

- Sensing receiver: the TRP or a UE that receives the sensing signal which the sensing service will use in its operation. A sensing receiver can be located in the same or different TRP or a UE as the sensing transmitter.

- Sensing target: target that need to be sensed by deriving characteristics of the objects within the environment from the sensing signal.

- Background environment: background (clutter and/or environmental objects) that are not the sensing target(s).

- Mono-static sensing: sensing where the sensing transmitter and sensing receiver are co-located in the same TRP or UE.

- Bi-static sensing: sensing where the sensing transmitter and sensing receiver are in different TRPs or UEs.

- Multi-static sensing: sensing where there are multiple sensing transmitters and/or multiple sensing receivers, for a sensing target.

- Sensing signal: Transmissions on the 3GPP radio interface that can be used for sensing purposes.

Agreement

Any TRP and/or UE location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations. FFS: other possible sensing transmitters and receivers locations.

Agreement

The following table can be used by companies to propose values for each sensing target

- Additional parameters/rows can be added if needed

Table x. Evaluation parameter template for sensing scenarios

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios |  |  |
| Sensing transmitters and receivers properties |  |  |
| Supported sensing modes |  |  |
| Sensing target | Outdoor/indoor |  |
|  | 3D mobility |  |
|  | 3D distribution |  |
|  | Orientation |  |
|  | Physical characteristics (e.g., size) |  |
| [Unintended/Environment objects] | Types |  |
|  | 3D mobility |  |
|  | 3D distribution |  |
|  | Orientation |  |
|  | Physical characteristics (e.g., size) |  |
| [Sensing area] |  |  |
| Minimum 3D distances between pairs of Tx/Rx/sensing target/[unintended objects] |  |  |

RAN1#117 Agreements

Agreement

For each of the sensing target deployment scenarios using the template agreed in RAN#116-bis, the following principles apply:

- For defining sensing Tx and sensing Rx properties (e.g., cell layout, BS antenna height, and minimum distance), scenario parameter values for the applicable communication scenarios in 38.901 are considered as a starting point. Updates to these evaluation parameter values for ISAC scenarios will consider the following:

  - aerial UEs parameter values as defined in TR 36.777

  - indoor room scenario parameter values defined in TR 38.808

  - automotive scenario parameter values as defined in TR 37.885, 38.859 for Urban grid/Highway

  - Minimum distances between Tx/Rx and target are not defined in the existing communications scenarios and shall be included in the sensing target deployment scenarios.

  - Note: Only deviation from the existing evaluation parameters in the applicable communication scenarios need to be explicitly defined in the ISAC scenario tables.

- For defining sensing target properties, as a baseline

- Evaluation parameter values can be taken from additional TRs, e.g., TR 36.777, 37.885, 38.859, etc.

- Size of sensing targets based on TR 22.837, 37.885 (e.g. for automotive), 38.901 (e.g. for AGV size), etc

Agreement

For ISAC deployment scenarios, carrier frequency, bandwidth, and SCS are not included in the evaluation parameters templates for sensing scenarios, but may be included in the evaluation/calibration phase.

Agreement

For UAV sensing target scenarios, the following table is used as a starting point for deployment scenario parameters/values.

Note: Additional parameters, value/value ranges are not precluded.

Table x. Evaluation parameters for UAV sensing scenarios

| Parameters |  |  | Value |
| --- | --- | --- | --- |
| Applicable communication scenarios |  |  | UMi, UMa, RMa [38.901] UMi-AV, UMa-AV, RMa-AV |
| Sensing transmitters and receivers properties | Rx/Tx Locations |  | Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenario Note1: this may include aerial UEs for UMi-AV, UMa-AV, RMa-AV communication scenarios. [In this case, other Rx/Tx properties (e.g. mobility) are also taken from the corresponding communication scenario.] |
| Supported sensing modes |  |  | [All 6 sensing modes] |
| Sensing target |  | Outdoor/indoor | Outdoor |
|  |  | 3D mobility | Horizontal velocity: Up to 160 km/h [FFS specific velocity(ies) or random distribution] [FFS vertical plane velocity] |
|  |  | 3D distribution | [Uniform between a minimum and maximum height] [Uniform in horizontal domain at a given height] |
|  |  | Orientation | Random in horizontal domain |
|  |  | Physical characteristics (e.g., size) | UAV object type(s) [FFS] |
| [Sensing area] |  |  |  |
| Minimum 3D distances between pairs of Tx/Rx and sensing target/[unintended objects] |  |  | FFS |
| Minimum 3D distance between sensing targets |  |  | FFS |
| [Unintended/Environment objects, e.g., types, characteristics, mobility, distribution, etc.] |  |  | FFS |

Agreement

RAN1 agrees to the following revised evaluation parameters values for the UAV sensing target scenarios:

| Parameters | Value |
| --- | --- |
| Sensing transmitters and receivers properties | Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenario Note 1: Other Rx/Tx properties (e.g. mobility) can also be taken from the corresponding communication scenario. Note 2: This may include aerial UEs as Rx/Tx that can be selected among locations in the UMi-AV, UMa-AV, RMa-AV communication scenarios. |

RAN1#118 Agreements

Conclusion

RAN1 will consider the recommendations for the physical characteristics (e.g., sizes, shapes, materials, velocities, etc.) of sensing targets and objects provided in 5GAA LS (R1-2405964), along with the relevant characteristics defined in 3GPP TRs, within the scope of the Rel-19 study item.

- No LS response from RAN1 to 5GAA is necessary.

- R1-2405964 is proposed to be NOTED.

Agreement

General principles for all sensing target deployment scenarios should consider the following:

- “Sensing mode” is removed in the scenario tables, but may be included in the evaluation/calibration phase. Per the SI, all sensing modes are possible for the deployment scenarios.

- “Sensing area” may be addressed as part of the sensing target distribution and/or Tx/Rx characteristics and/or cell layout.

Agreement

For UAV sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#117 as a baseline.

Note: Additional parameters, value/value ranges are not precluded.

The detailed scenario description in this clause can be used for channel model calibration.

ISAC-UAV

Details on ISAC-UAV scenarios are listed in Table x.

Table x. Evaluation parameters for UAV sensing scenarios

| Parameters |  |  | Value |
| --- | --- | --- | --- |
| Applicable communication scenarios |  |  | UMi, UMa, RMa [38.901] UMi-AV, UMa-AV, RMa-AV |
| Sensing transmitters and receivers properties | Rx/Tx Locations |  | Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenarios. Note1: This may include aerial UEs for UMi-AV, UMa-AV, RMa-AV communication scenarios. In this case, other Rx/Tx properties (e.g. mobility) are also taken from the corresponding communication scenario. |
| Sensing target |  | Outdoor/indoor | Outdoor |
|  |  | 3D mobility | Horizontal velocity: uniform distribution between 0 and 180km/h, if horizontal velocity is not fixed to 0. Vertical velocity: 0km/h, optional {20, 40} km/h NOTE2: 3D mobility can be horizontal only or vertical only or a combination for each sensing target FFS: time-varying velocity. |
|  |  | 3D distribution | Horizontal plane: Option A: N targets uniformly distributed within one cell. Option B: N targets uniformly distributed per cell. Option C: N targets uniformly distributed within an area not necessarily determined by cell boundaries. FFS: Value of N, defined area, and other distributions Vertical plane: Option A: Uniform between 1.5m and 300m. Option B: Fixed height value chosen from {25, 50, 100, 200, 300} m assuming vertical velocity is equal to 0. FFS Other options are not precluded. Note2: target(s) are outside the minimum distance to the Tx/Rx |
|  |  | Orientation | Random in horizontal domain |
|  |  | Physical characteristics (e.g., size) | Size: - Option 1: 1.6m x 1.5m x 0.7m - Option 2: 0.3m x 0.4m x 0.2m FFS: Material(s), Structure, Other size(s) |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  |  | Option B: Min distances based on min. TRP/UE distances defined in TR36.777 as a starting point. Option C: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  |  | Option 1: At least larger than the physical size of a target Option 2: 10 meters |
| [Unintended/Environment objects, e.g., types, characteristics, mobility, distribution, etc.] |  |  | FFS |

Note: further down-selection between the options in the table is not precluded.

RAN1#118-bis Agreements

Agreement

For Automotive sensing target scenarios, the following table is used as a starting point for deployment scenario parameters/values.

The detailed scenario description in this clause can be used for channel model calibration.

Note: Additional parameters, value/value ranges are not precluded.

Table x. Evaluation parameters for Automotive sensing scenarios

| Parameters |  | Values |
| --- | --- | --- |
| Applicable communication scenarios |  | Highway, Urban Grid. NOTE1 |
| Sensing transmitters and receivers properties |  | Rx/Tx locations are selected among the TRPs and UEs (e.g., VRU, vehicle, RSU-type UEs) locations in the corresponding communication scenario. NOTE2 FFS: Option 2: ISD between TRPs of Urban Grid is 250 meters |
| Sensing target | LOS/NLOS | LOS and NLOS (including NLOSv) |
|  | Outdoor/indoor | Outdoor |
|  | Mobility (horizontal plane only) | Based on TR37.885 mobility for urban grid or highway scenario |
|  | Distribution (horizontal) | Based on dropping in TR37.885 per urban grid or highway communication scenario |
|  | Orientation | Lane direction in horizontal plane |
|  | Physical characteristics (e.g., size) | Type 1/2 (passenger vehicle) Type 3 (truck/bus) Vehicle type distribution per TR 37.885 as a starting point FFS: Other sizes, additional distributions, and vehicle types, e.g. one new type of e-scooter/motorcycle/bike |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option 1: Min distances based on min. TRP/UE distances defined in TR37.885 as a starting point. Option 2: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [x] m. value of x is FFS |
| Environment Objects, e.g., types, characteristics, mobility, distribution, etc. |  | EO Type 2 for Urban Grid - FFS: details, e.g. 4 walls (as EO type 2) per building of size [413mx230mx20m] |

NOTE1: calibration for UMi, Uma, RMa is not performed for the automotive scenario, but UMi, Uma, RMa can be considered for future evaluations of the automotive sensing target scenarios. Calibration for UMi, Uma, RMa is expected to be performed for another sensing scenario.

NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

Agreement

For Human sensing target scenarios, (indoor and outdoor), the following table is used as a starting point for deployment scenario parameters/values.

The detailed scenario description in this clause can be used for channel model calibration.

Note: Additional parameters, value/value ranges are not precluded.

Table x. Evaluation parameters for Human (indoor and outdoor) sensing scenarios

| Parameters |  | Indoor Values | Outdoor Values |
| --- | --- | --- | --- |
| Applicable communication scenarios NOTE1 |  | Indoor office, indoor factory [TR38.901] Indoor room [TR38.808] | UMi, Uma, RMa [TR38.901] |
| Sensing transmitters and receivers properties | Rx/Tx Locations NOTE 2 | Rx/Tx locations are selected among the TRPs and UE locations in the corresponding communication scenario | Rx/Tx locations are selected among the TRPs and UE locations in the corresponding communication scenario |
|  | Rx/Tx Mobility for UEs | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 3km/hr | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 10km/hr |
| Sensing target | Outdoor/indoor | Indoor | Outdoor |
|  | 3D mobility | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 3km/hr (horizontal plane with random direction straight-line trajectory) | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 10km/hr (horizontal plane with random direction straight-line trajectory) |
|  | 3D distribution | N targets uniformly distributed over the horizontal area of the convex hull of the TRP deployment FFS: Value of N | Uniform in horizontal plane |
|  | Orientation | Random over the horizontal area | Random over the horizontal area |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m | Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx Option 2: Min distances defined in TR 38.901 as a starting point | Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx Option 2: Min distances defined in TR 38.901 as a starting point |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [x] m. value of x is FFS | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [x] m. value of x is FFS |
| Environment Objects, e.g., types, characteristics, mobility, distribution, etc. |  | FFS, based on outcome for AI 9.7.2 | FFS, based on outcome for AI 9.7.2 |

NOTE1: For the human (indoor and outdoor) sensing targets, additional communication scenarios can be considered for future evaluations. Channel model calibration for Urban Grid with outdoor humans is expected to be performed from Objects creating hazards on the road/railway sensing scenarios.

NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

Agreement

For Automated Guided Vehicles (AGV) target scenarios, the following table is used as a starting point for deployment scenario parameters/values.

The detailed scenario description in this clause can be used for channel model calibration.

Note: Additional parameters, value/value ranges are not precluded.

Table x. Evaluation parameters for Automated Guided Vehicles

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios NOTE1 |  | InF (TR38.901 including Table 7.8-7) |
| Sensing transmitters and receivers properties NOTE2 |  | Rx/Tx location are selected among the TRPs and UEs location in the corresponding communication scenario Rx/Tx Mobility for UEs - Option 1: 0 km/h - Option 2: 3km/h - Option 3: Uniform distribution between 0km/h and 3km/h |
| Sensing target | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Indoor |
|  | 3D mobility | Horizontal velocity with random straight-line trajectory - Option 1: Uniform distribution in the range of up to 30 km/h - Option 2: Fixed velocities [3, 10] km/h |
|  | 3D distribution | Option A: Uniformly distributed in the convex hull of the horizontal BS deployment Option B: Uniformly distributed in horizontal plane |
|  | Orientation | Horizontal plane only |
|  | Physical characteristics (e.g., size) | Size (L x W x H) - Option 1: 0.5m x 1.0m x 0.5m - Option 2: 1.5 m x 3.0m x 1.5 m - FFS: Material, Additional sizes, and AGV size distribution |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx from the sensing target Option 2: Min distances based on min. TRP/UE distances defined in TR38.901 |
| Minimum 3D distance between sensing targets |  | Option A: At least larger than the physical size of a target Option B: Fixed value, [x] m. value of x is FFS |
| Environment objects, e.g., types, characteristics, mobility, distribution, etc. |  | FFS |

NOTE1: For the AGV sensing targets, additional communication scenarios can be considered for future evaluations.

NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

NOTE3: RAN1 can further discuss narrowing down the number of sub-scenarios of InF

Agreement

For objects creating hazards use cases, RAN1 to consider the following table as a starting point for deployment scenario parameters/values.

The detailed scenario description in this clause can be used for channel model calibration.

Note: Additional parameters, value/value ranges are not precluded.

Table x. Evaluation parameters for objects creating hazards

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios NOTE1 |  | Highway, Urban grid, HST (High Speed Train) |
| Sensing transmitters and receivers properties NOTE2 | Rx/Tx Locations | Rx/Tx locations are selected among the TRPs and UEs (e.g., VRU, vehicle, RSU-type UEs) locations in the corresponding communication scenarios. FFS: Option 2: ISD between TRPs of Urban Grid is 250 meters |
| Sensing target | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Outdoor |
|  | 3D mobility | Horizontal velocity: up to [10] km/h for humans and animals FFS: Additional velocities, trajectory |
|  | 3D distribution | Uniformly distributed in horizontal plane |
|  | Orientation | Random distribution in horizontal plane |
|  | Physical characteristics (e.g., size) | For human/pedestrians: Child: 0.2m x 0.3m x 1m Adult: 0.5m x 0.5m x 1.75m For animals: Size: 1.5m x 0.5m x 1 m FFS: other types of targets |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx from the sensing target Option 2: based on TR37.885 and TR38.802 |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a target Option 2: Fixed value, [x] m. value of x is FFS |
| Environment objects, e.g., types, characteristics, mobility, distribution, etc. |  | EO Type 2 for Urban Grid - FFS: details, e.g. 4 walls (as EO type 2) per building of size [413mx230mx20m] |

NOTE1: For the objects creating hazards sensing targets, additional communication scenarios can be considered for future evaluations.

NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

RAN1#119 Agreements

Agreement

For UAV sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118 as a baseline:

The detailed scenario description in this clause can be used for channel model calibration.

ISAC-UAV

Details on ISAC-UAV scenarios are listed in Table x.

Table x. Evaluation parameters for UAV sensing scenarios

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios |  | UMi, UMa, RMa [38.901] UMi-AV, UMa-AV, RMa-AV |
| Sensing transmitters and receivers properties | Rx/Tx Locations | Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenarios. NOTE1: This may include aerial UEs for UMi-AV, UMa-AV, RMa-AV communication scenarios. In this case, other Rx/Tx properties (e.g. mobility) are also taken from the corresponding communication scenario. |
| Sensing target | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Outdoor |
|  | 3D mobility | Horizontal velocity: uniform distribution between 0 and 180km/h, if horizontal velocity is not fixed to 0. Vertical velocity: 0km/h, optional {20, 40} km/h NOTE2: 3D mobility can be horizontal only or vertical only or a combination for each sensing target FFS: time-varying velocity. NOTE 3: time-varying velocity may be considered for future evaluations. |
|  | 3D distribution | Horizontal plane: Option A: N targets uniformly distributed within one cell. Option B: N targets uniformly distributed per cell. Option C: N targets uniformly distributed within an area not necessarily determined by cell boundaries. FFS: Value of N, defined area, and other distributions N = {1, 2, 3, 4, 5} NOTE4: N=0 may be considered for the evaluation of false alarm Vertical plane: Option A: Uniform between 1.5m and 300m. Option B: Fixed height value chosen from {25, 50, 100, 200, 300} m assuming vertical velocity is equal to 0. FFS Other options are not precluded. NOTE5: target(s) are outside the minimum distance to the Tx/Rx |
|  | Orientation | Random in horizontal domain |
|  | Physical characteristics (e.g., size) | Size: - Option 1: 1.6m x 1.5m x 0.7m - Option 2: 0.3m x 0.4m x 0.2m FFS: Material(s), Structure, Other size(s) |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option B: Min distances based on min. TRP/UE distances defined in TR36.777 as a starting point. NOTE5: the sensing target is assumed in the far field of sensing Tx/Rx Option C: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a target Option 2: 10 meters |
| [Unintended/Environment objects, e.g., types, characteristics, mobility, distribution, etc.] |  | FFS |

NOTE: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

Agreement

For Automotive sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline:

The detailed scenario description in this clause can be used for channel model calibration.

ISAC-Automotive

Details on ISAC-Automotive scenarios are listed in Table x.

Table x. Evaluation parameters for Automotive sensing scenarios

| Parameters |  | Values |
| --- | --- | --- |
| Applicable communication scenarios |  | Highway, Urban Grid. NOTE1 |
| Sensing transmitters and receivers properties |  | Rx/Tx locations are selected among the TRPs and UEs (e.g., VRU, vehicle, RSU-type UEs) locations in the corresponding communication scenario. NOTE2 FFS: Additional option: ISD between TRPs of Urban Grid is 250m |
| Sensing target | LOS/NLOS | LOS and NLOS (including NLOSv) |
|  | Outdoor/indoor | Outdoor |
|  | Mobility (horizontal plane only) | Based on TR37.885 mobility for urban grid or highway scenario |
|  | Distribution (horizontal) | Based on dropping in TR37.885 per urban grid or highway communication scenario |
|  | Orientation | Lane direction in horizontal plane |
|  | Physical characteristics (e.g., size) | Type 1/2 (passenger vehicle) Type 3 (truck/bus) Vehicle type distribution per TR 37.885 as a starting point FFS: Other sizes, additional distributions, and vehicle types, e.g. one new type of e-scooter/motorcycle/bike |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option 1: Min distances based on min. TRP/UE distances defined in TR37.885 as a starting point. Option 2: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx NOTE3: the sensing target is assumed in the far field of sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [10] m. value of x is FFS |
| Environment Objects, e.g., types, characteristics, mobility, distribution, etc. |  | EO Type 2 for Urban Grid - FFS: details, e.g. up to 4 walls modelled as EO type 2, per building of size [413m x 230m x 20m]. FFS: number of buildings, how many walls are modelled, additional building sizes, etc. |

NOTE1: calibration for UMi, Uma, RMa is not performed for the automotive scenario, but UMi, Uma, RMa can be considered for future evaluations of the automotive sensing target scenarios. Calibration for UMi, Uma, RMa is expected to be performed for another sensing scenario.

NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

Agreement

For Human (indoor and outdoor) sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline:

The detailed scenario description in this clause can be used for channel model calibration.

ISAC-Human

Details on ISAC-Human scenarios are listed in Table x.

Table x. Evaluation parameters for Human (indoor and outdoor) sensing scenarios

| Parameters |  | Indoor Values | Outdoor Values |
| --- | --- | --- | --- |
| Applicable communication scenarios NOTE1 |  | Indoor office, indoor factory [TR38.901] Indoor room [TR38.808] | UMi, Uma, RMa [TR38.901] |
| Sensing transmitters and receivers properties | Rx/Tx Locations NOTE 2 | Rx/Tx locations are selected among the TRPs and UE locations in the corresponding communication scenario | Rx/Tx locations are selected among the TRPs and UE locations in the corresponding communication scenario |
|  | Rx/Tx Mobility for UEs | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 3km/hr | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 10km/hr |
| Sensing target | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | Outdoor/indoor | Indoor | Outdoor |
|  | 3D mobility | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 3km/hr (horizontal plane with random direction straight-line trajectory) | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 10km/hr (horizontal plane with random direction straight-line trajectory) |
|  | 3D distribution | N targets uniformly distributed over the horizontal area of the convex hull of the TRP deployment FFS: Value of N NOTE1: N=0 may be considered for the evaluation of false alarm | Option A: N targets uniformly distributed within one cell. Option B: N targets uniformly distributed per cell. Option C: N targets uniformly distributed within an area not necessarily determined by cell boundaries. Uniform in horizontal plane NOTE1: N=0 may be considered for the evaluation of false alarm |
|  | Orientation | Random over the horizontal area | Random over the horizontal area |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m | Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx Option 2: Min distances defined in TR 38.901 and TR36.843 and TR38.859as a starting point NOTE2: the sensing target is assumed in the far field of sensing Tx/Rx | Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx Option 2: Min distances defined in TR 38.901 and TR36.843 and TR38.859 as a starting point NOTE3: the sensing target is assumed in the far field of sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [x] m. value of x is FFS | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [x] m. value of x is FFS |
| Environment Objects, e.g., types, characteristics, mobility, distribution, etc. |  | FFS, based on outcome for AI 9.7.2 | FFS, based on outcome for AI 9.7.2 |

NOTE1: For the human (indoor and outdoor) sensing targets, additional communication scenarios can be considered for future evaluations. Channel model calibration for Urban Grid with outdoor humans is expected to be performed from Objects creating hazards on the road/railway sensing scenarios.

NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

Agreement

For AGV sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline:

The detailed scenario description in this clause can be used for channel model calibration.

ISAC-AGV

Details on ISAC-AGV are listed in Table x.

Table x. Evaluation parameters for Automated Guided Vehicles

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios NOTE1 |  | InF (TR38.901 including Table 7.8-7) |
| Sensing transmitters and receivers properties NOTE2 |  | Rx/Tx location are selected among the TRPs and UEs location in the corresponding communication scenario Rx/Tx Mobility for UEs - Option 1: 0 km/h - Option 2: 3km/h - Option 3: Uniform distribution between 0km/h and 3km/h |
| Sensing target | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Indoor |
|  | 3D mobility | Horizontal velocity with random straight-line trajectory - Option 1: Uniform distribution in the range of up to 30 km/h - Option 2: Fixed velocities [3, 10] km/h |
|  | 3D distribution | Option A: Uniformly distributed in the convex hull of the horizontal BS deployment Option B: Uniformly distributed in horizontal plane |
|  | Orientation | Horizontal plane only |
|  | Physical characteristics (e.g., size) | Size (L x W x H) - Option 1: 0.5m x 1.0m x 0.5m - Option 2: 1.5 m x 3.0m x 1.5 m - FFS: Material, Additional sizes, and AGV size distribution |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx from the sensing target Option 2: Min distances based on min. TRP/UE distances defined in TR38.901 NOTE: the sensing target is assumed in the far field of sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option A: At least larger than the physical size of a target Option B: Fixed value, [x] m. value of x is FFS |
| Environment objects, e.g., types, characteristics, mobility, distribution, etc. |  | FFS |

NOTE1: For the AGV sensing targets, additional communication scenarios can be considered for future evaluations.

NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

NOTE3: RAN1 can further discuss narrowing down the number of sub-scenarios of InF

Agreement

For Objects creating hazards sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline:

The detailed scenario description in this clause can be used for channel model calibration.

ISAC-Hazards

Details on ISAC-Hazards are listed in Table x.

Table x. Evaluation parameters for objects creating hazards

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios NOTE1 |  | Highway, Urban grid, HST (High Speed Train) |
| Sensing transmitters and receivers properties NOTE2 | Rx/Tx Locations | Rx/Tx locations are selected among the TRPs and UEs (e.g., VRU, vehicle, RSU-type UEs) locations in the corresponding communication scenarios. FFS: Option 2 Additional option ISD between TRPs of Urban Grid is 250 m |
| Sensing target | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Outdoor |
|  | 3D mobility | Horizontal velocity: up to [10] km/h for humans and animals FFS: Additional velocities, trajectory |
|  | 3D distribution | Uniformly distributed in horizontal plane |
|  | Orientation | Random distribution in horizontal plane |
|  | Physical characteristics (e.g., size) | For human/pedestrians: Child: 0.2m x 0.3m x 1m Adult: 0.5m x 0.5m x 1.75m For animals: Size: 1.5m x 0.5m x 1 m FFS: other types/sizes of targets may be considered for future evaluations |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx from the sensing target Option 2: based on min TRP/UE distances defined in TR37.885 and TR38.802 and TR36.843 and TR38.859 NOTE: the sensing target is assumed in the far field of sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [10] m. value of x is FFS |
| Environment objects, e.g., types, characteristics, mobility, distribution, etc. |  | EO Type 2 for Urban Grid - FFS: details, e.g. up to 4 walls modelled as EO type 2, per building of size [413m x 230m x 20m]. FFS: number of buildings, how many walls are modelled, additional building sizes, etc. |

NOTE1: For the objects creating hazards sensing targets, additional communication scenarios can be considered for future evaluations.

NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.