3GPP TSG-RAN WG1 Meeting #120bis	R1- 2501775

Wuhan, China, April 7th – April 11th, 2025

Agenda Item:	9.1.1

Source:	Ericsson

Title:	AI/ML for beam management

Document for:	Discussion

# Introduction

In RAN#102 meeting, a new WID is approved for Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface [2]. In this contribution, we focus on the normative work for the use case of beam management enhancements.

# Configuration of UE-sided model

## Set A/B Configuration

Agreement For UE-sided model, for configuring the resource for data collection purpose, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report. - One CSI-ResourceConfigId is configured for Set A. - One CSI-ResourceConfigId is configured for Set B. - Note: UE performs measurement on all resources - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B - FFS: whether/how to support 'aperiodic' CSI RS Note: This is not related to whether/how to support delivery/transmission of the collected data for training for UE-sided model. Agreement For UE-sided model, in CSI-ReportConfig for inference - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B - FFS: At least BM-Case 1, the applicability for 'aperiodic' CSI RS

During RAN1#120, it was agreed that one or two associated IDs can be configured in CSI-ReportConfig, when set B is a subset of set A, then only a single associated ID is needed. Since only one associated ID is configured for set A in the CSI report configuration, the resourceConfig for set A needs to be limited to a single resourceSet also for aPeriodic resource type. Otherwise, the UE assumptions of the associated ID and the resourceSets are unclear.

- For UE-sided model, in CSI-ReportConfig, the resourceConfig for set A can only include a single resource set.

Moreover, one FFS from RAN1#120 is the aPeriodic inference for BM-Case 1. For AP inference report, the UE may be configured with multiple resource sets within the CSI resource configuration. Which resource set that is then transmitted is indicated via the AssociatedReportConfigInfo part of the aPeriodicTriggerState, together with the QCL assumptions. For simplicity, RAN1 should assume that set A is configured according to the CSI-ReportConfig, and the AssociatedReportConfigInfo outlines the assumptions on the set B measurements.

- For UE-sided model, for aPeriodic CSI inference report configuration, AssociatedReportConfigInfo comprises the UE assumptions regarding set B.

Set B is a subset of set A

When set B is a subset of set A, the set B is indicated according to the NZP-CSI-RS-ResourceId/SSB-Index. In this scenario, there could be multiple resourceSets in the set B resource configuration, each set having a different combination of the NZP-CSI-RS-ResourceId/SSB-Index. That is, there is no specification impact for this case and could be supported.

Set B is not a subset of set A

For the scenario when set B is not a subset of set A, since the CSI-ReportConfig only includes a single set B associated ID, the UE would not know for which resourceSet the associated ID is applicable for. Possible solutions could comprise of limit the resourceConfig of set B to a single resourceSet, or configure the associated ID in the associatedCSIreportConfig. For the latter solution, it could be complex for UE to support multiple setBs that are triggered in DCI, then the UE would need to quickly load a specific model.

- For UE-sided model, for aPeriodic CSI inference report configuration, when set B is NOT a subset of set A, the resourceConfig for set B can only include a single resourceSet.

## Support for aPeriodic UE-sided data collection

RAN2 Agreements on NW side data collection - ….…. - Measurements on aperiodic CSI resources are not reported for NW sided data collection. - ….….

The UE-sided data collection needs to be done efficiently, for example enabling UEs to share the set A/B resources via NW configuring periodic or semi-persistent resources. Regarding the “FFS: whether/how to support 'aperiodic' CSI RS”, the motivation is not clear why not P/SP reporting is not sufficient, and supporting AP would imply standardization challenges in configuring both set A and set B resources in the aPeriodic reporting configuration (e.g. part of the AssociatedReportConfigInfo). Due to the lack of time left of the work item, it is proposed to follow the RAN2 agreement on NW-sided data collection also for UE-sided data collection.

For UE-sided model, for configuring the data collection resources, do not support aPeriodic CSI-RS

# Inference result report for UE-sided model

Once a UE is configured with an AI/ML model/functionality, then the UE is expected to report the requested radio measurement predictions to the NW via UCI. There are situations where the UE could fail to provide the requested reports, or even would want to request the NW to deactivate the AL/ML model/functionality due to performance issues. RAN2 for example made the following agreement in RAN2#128.

Agreements 1. When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network. FFS detailed design 2. When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

The agreement indicates that the UE may report non-applicability of the AI/ML functionality (equivalent to the UE not being able to generate reliable predictions anymore) using RRC signalling and then expecting the NW to deactivate the functionality in response. Using RRC signalling to deactivate the AI/ML functionality has some drawbacks that should be addressed. First, RRC signalling may introduce delays (for requesting resources to send the request to the NW and signalling to the UE to deactivate the feature) which in turn may degrade the performance, since during these delays the prediction (also called inference) reports received from the UE are not accurate or could not be trusted. Secondly, the UE might temporarily not be able to provide the requested prediction report. In such cases, the UE behaviour should be defined so that the NW can correctly interpret the received inference results, without necessarily deactivating completely the AI/ML functionality. Since the support for UE reporting a probability of being a top 1 beam which could have mitigated this issue is not expected to be standardized during Rel-19, RAN1 should at least introduce support for an indication in the UCI indicating if the inference report (i.e. AI/ML prediction report) is invalid or inaccurate.

For the UE-sided inference result report, if not supporting UE report of probabilities, support UE indicating in UCI if the inference result report is inaccurate/invalid (e.g. setting the CRI or SSB-RI to a certain “dummy” value)

For time domain-based beam prediction predicting beams for future time instances, the accuracy of the prediction may generally vary for these future time instances. For instance, the further away the future time instance is, the less accurate the prediction may be (i.e., it may become harder to ensure accurate prediction as we try to predict further into the future). This was also shown in the results conducted in the Rel-18 study item, captured in 38.843. Since the support for UE reporting a probability is not expected to be standardized during Rel-19, which could have enabled the UE to indicate the uncertainty in each time instance, the NW should at least be able to configure a different number of UE reported K beams in each time instance.

For the UE-sided BM-Case 2 inference report configuration, support NW configuring the number K of UE-reported beams for each time instance.

# Performance monitoring of UE-sided model

## UE-assisted performance metrics

During RAN1#120, the following two options for UE performance metrics were discussed,

At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s). Where K is the number of predicted beam(s) in the corresponding inference report, - Where Top M beam(s) is the beam(s) with largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, and M is up to 4 - Where Top M beams are the M beams that are within x dB of the best beam with the largest measured value(s) of L1-RSRP(s) of the resource set(s) - X is NW configured on CSI report configuration for monitoring - X= 0,1 dB, FFS other values

To understand the impact of the two performance metric options, the data from an Urban Macro scenario is studied. In the scenario, the NW is equipped with 64 beams and the deployment is 80% indoor, and 20 % outdoor UEs. Next, the impact of option A is evaluated by comparing the strongest beam with the 2nd and the 4th strongest beam. This is illustrated in Figure 1, where the figure illustrates for example that in 20% of the cases for K=4, the UE might classify a prediction as correct, even if the RSRP difference is larger than 4 dB to the strongest (note the large tail indicating that also 10 dB could be labelled as correct). This illustrates that the option A is misleading in assessing the UE model performance metric.

In an Urban macro deployment with 64 beams, having a prediction performance metric based on the Top-4 beams in a monitoring resource set, will in 20% of the cases also label a beam with more than 4dB difference to the strongest as a correct prediction.

Figure 1: CDF of RSRP difference between the strongest and the 4th strongest beam

Option B does not have the issues outlined above for option A, since the UE will consider the difference to the strongest beam when calculating the metric. This option is evaluated for the same scenario, and the CDF in the figure below indicates the distribution of the number of M beams based on x. The figure shows that x=1dB, there is less than 4 beams, while for x=3dB, M is less than 8 beams. Due to the shortcomings of option A, our preference is option B, where the value of x can be 1 dB or 3 dB.

Figure 2: CDF of the number of M beams for different values of x in one urban macro deployment scenario, M beams that are within x dB of the best beam.

For the UE-sided model performance metric, support, at least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s). Where K is the number of predicted beam(s) in the corresponding inference report,

  - Where Top-M beams are the M beams that are within x dB of the best beam with the largest measured value(s) of L1-RSRP(s) of the resource set(s)

  - X is NW configured on CSI report configuration for monitoring

  - X= 0 dB,1 dB, 3 dB

## UE-assisted performance monitoring resource configuration

Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

FFS on the timing related issues

There might be an issue on which of the monitoring resource sets in time that the UE should use to determine the performance metric during the N monitoring occasions. One example comprises the option to use the closest in “time”. However, the time might not be the best dimension, in particular for rapid channel changes. The UE could determine that for BM case 1, an earlier time instance is better at representing the channel conditions for a certain inference occasion. Also, in case the monitoring resources are transmitted with different subsets of set A, the UE might benefit to take the resourceSet that are closest in the spatial domain.

The monitoring resource set should be selected that best represent the UE inference channel conditions and its predictions. RAN1 should further discuss an exact definition.

FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report

The monitoring report is dependent on the inference resource configuration, in case the inference resources are aPeriodic, there is no motivation on support periodic monitoring resources and reporting in such scenario. There is further an issue on whether how the UE should report an aggregated metric (over N monitoring occasions) when a monitoring report and resource configuration is done in an aPeriodic report with aPeriodic resources, in such configuration, the UE would only estimate a one-shot metric and the benefit in supporting such scenario is not motivated. However, the aPeriodic report could be supported for the case when the resources are of periodic or semi-persistent type, then the aPeriodic report would comprise the N samples collected over the periodic resources. This scenario is illustrated in the figure below.

Figure 3: aPeriodic report using periodic or semi-persistent resources

For UE-sided performance monitoring, for the reportConfigType of the UE-sided model performance metric,

  - Support periodic and semi-persistent reportConfigType if the linked inference report is also periodic or semi-persistent

  - Support aPeriodic monitoring report only for the scenario where the UE has first been configured with a periodic or semi-persistent monitoring/inference resource config type

The monitoring is linked to an inference configuration (or vice versa) according to the RAN1 agreements, how a monitoring and inference configuration is triggered is an open issue. One solution is to activate them in separate messages according to existing standard, however, this can lead to unnecessary high signaling overhead since the two configurations are linked. Another solution is to use a single DCI/MAC-CE message to both activate the inference and the monitoring resource for enabling the UE to calculate the performance of its AI/ML model. This leads to reduced signaling overhead on the control channel, it also mitigates timing related issues since they are triggered in the same message.

For UE-sided performance monitoring, support joint activation of the monitoring and linked inference report configuration

FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

RAN1 agreed that the monitoring report should be linked to an inference report config as illustrated in the figure below. To enable support for a UE to understand which of the beams in set A that are transmitted, the monitoring resourceSet can reuse the same resources from set A, with the assumption that they are transmitted with the same NW parameters as set A. This enables the UE to identify the connection between RSs in the monitoring resourceSet and the set A beams.

Figure 4: Monitoring resource sets points to the same resources as set A, UE can assume such resources are transmitted with the same NW transmission parameters

To ensure efficient transmission of the underlying resource for monitoring, the resources should be allocated in a minimum number of slots by using adjacent OFDM symbols for the monitoring (sub)set. The NW may need to use a different resource mapping for a resource ID than what was used during training when configuring the monitoring/inference. This should be possible because the associated ID only provides the UE with assumptions about the NW transmission parameters, not the resource mapping. Note that the UE has already trained a model and does not require data collection configuration which may had created conflicting resource mapping for the same resource ID. Therefore, there is no need to introduce an explicit mapping from the training resource to the monitoring resource, as the resource mapping does not need to be consistent.

The resource mapping of an NZP-CSI-RS-resources may not be the same in the monitoring and training data collection step. The NW does not need to configure UE with training data collection resources if UE has a trained model, and can optimize the time-domain allocation of the resource for monitoring (with possible subsets of set A beams).

For UE-sided model, to identify the connection between RSs in the resource set(s) for monitoring and Set A beams:

  - UE can assume the same NW transmission parameters for the resourceIDs (all or a subset) that are present both in the monitoring and set A resourceSet,

  - Note: The resource mapping for monitoring resourceIDs may not be the same as during the UE training data collection

## UE-assisted performance metrics reporting events

To achieve sufficient statistics to determine at the NW whether to activate/deactivate a functionality or not, there is a need to have a lot of samples. One issue occurs for the low-accuracy (erroneous) models, the NW might spend unnecessary number of resources for the purpose on performance metric calculation on models that are performing significantly bad, e.g. an accuracy below e.g. 50%. There could be a benefit to define an event, where the UE can indicate early if a functionality is not working properly.

For UE-assisted performance metric calculation, at least support an event where the UE can early indicate if a functionality is not working properly, for example prediction accuracy is below 50% after collecting N/2 samples

# Beam Indication Enhancements

(for #120) Issue #7.2: whether any enhancement is needed for P2? (FL0/FL1) proposal 7.2-1a: For UE-side model and/or NW-side model, further study whether/how Top-K beam measurements (P2 sweep) can be introduced in the existing CSI framework, with the following aspects as a starting point - How NW indicates which beams in set A that are part of the Top-K measurements - How NW can configure/indicate a dynamic number K RS resources for measurements - How NW can dynamically indicate the QCL relationship for the pre-configured RS resources of the Top-K beams.

In existing P2 procedure, for example when using hierarchical beamforming in left figure below, the NW has a limited number of narrow beam measurement combinations during P2 due to limitations in the number of aPeriodicTriggerStates. The NW can only configure a limited set of aPeriodicTriggerStates to enable UE to measure the narrow beams within each wide beam, where the main difference of the aPeriodicTriggerStates is the qcl-info for each state.

Figure 5: The large number of possible Top-K configurations with AI/ML is complex for NW to configure using legacy CSI framework

With the introduction of AI/ML beam predictions, one possibility is that the Top-K narrow beams are part of separate wide beams as highlighted in the right part of the figure. It is not clear how the NW could dynamically indicate the subset of set A beams that are part of the top-K beam measurements within the CSI framework. This information would be needed for the UE to understand which TCI state to apply for its Top-K measurements.

One solution is that the NW transmits Top-K beams according to the UE predictions, which would be a solution that avoids frequent updates of the beams in top-K using MAC CE or DCI. One downside with such solution is that it does not include NW-specific information, e.g. which beams that are not feasible to transmit due to interference or “new” beams outside of set A. This may be mitigated by NW configuring another set of beams in the same aPeriodicTriggerState.

Another solution could comprise of dynamically update the TCI states that are part of the Top-K set part of MAC CE, this however may lead to excessive amount of MAC CE messages, and should be carefully introduced given the limitations on LCID. Not solving this issue may lead to inadequate performance of the AI-BM use case, hence our proposal is the following:

For enabling a P2 sweep based on predicted Top-K beams, RAN1 considers the following two solutions:

  - Option 1: NW indicates that the corresponding beam IDs/TCI states for the P2 ResourceSet follows the UE predicted Top-K beams, FFS on details including,

    - linkage of the inference report and the P2 (prediction result measurements) CSI-Report configuration, including BM-Case 2 when UE predicts N time instances

    - transmission order of the beams in the subsequent P2 (prediction result measurements)

  - Option 2: For both UE/NW sided model, NW configures dynamically as part of the MAC CE the beam IDs or TCI states that are part of the Top-K beam sweep.

    - FFS on details including the signaling overhead

  - FFS other options, including

    - Option 3: NW configures a number of aPeriodicTriggerStates, and the UE provides Top-K predictions restricted to the valid TCI states matching one of the aPeriodicTrigger states,

    - Option 4: The NW uses legacy signaling, and in a best-effort manner configure a set of aPeriodicTriggerStates

    - Option5: NW configures all Set A measurement resources, and UE is expected to measure only predicted top-K beams after each inference result.

    - Note 1: Option 3 and 4 would imply reduced performance since P2 cannot support all Top-K combinations

  - Note 2: NW indicating beam IDs could be used during monitoring

# UE measurement report for NW sided model

Agreement For NW-sided model, for inference, in a beam report initiated by network, based on one measurement resource set, support the report of more than 4 beam related information in L1 signaling - Note: Purpose, such as above “For NW-sided model, for inference”, will not be specified in RAN 1 specifications - FFS on the report content for beam related information - FFS on max number of reported beam related information in one report

During the Rel-18 evaluations, set B typically comprised a ratio that is 1/4 or 1/8 of Set A for spatial beam prediction, and setB equal to setA for BM-case2 in some evaluations. Hence, the possible number of beams of set B could be equal to the number of beams in set A (for BM-case2), therefore, one straightforward solution is to consider the number of beams of 32, or 64 or 256 beams as the number of beams for UE to report.

From the Table 6.3.1.1.2-8 in 38.212, up to 4 beams can be supported to report on UCI over PUSCH/PUCCH. From the Table 6.3.1.1.2-6 in 38.212, the number of bits needed for CRI and SSBRI depends on the total needed reported beams. For example, if 128 CSI-RS resources in total are configured by NW in RRC, which means 7 bits needed for reporting the CRI. So, for each beam, it needs 7 bits for CRI with 7 or 4 bits for absolute or differential RSRP. Based on the limit of Polar code (e.g., 1706 bits), the max potential number of beams reporting on UCI for the considered example above could be supported up to 1706/11=155. One straightforward way to support this with less spec impact is simply to extend the value range of the parameter nrofReportedRS in 38.331.

Furthermore, in case all beams are reported, the CRI field is not needed, hence one could also support the scenario of 256 beams (1706/4=426) evaluated in the study item. Note that the number of beams to be reported is upper limited by the resourceSet size, which may be extended to cover 256 resources depending on the work item progress. In any case, since 256 beams could fit into the report message as discussed above, hence the number of reported beams can comprise all beams in the resourceSet.

For NW-sided model, regarding max number of reported beam related information in one report, support reporting all measured beams in a resourceSet.

# UE measurement reporting overhead reduction for NW sided model

### Mechanisms to reduce inference data collection overhead for NW sided model

Agreement For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set - FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams - FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents) - FFS on beam information - Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specifications

In the Rel-18 evaluations, it was shown that good prediction accuracy could be obtained even with reporting of RSRP only for a subset of set B beams, thereby significantly reducing the reporting overhead. For example, it was found [1][3] that a 70% reporting overhead could be achieved while maintaining close to optimal prediction accuracy by having the UE report RSRP only for the strongest beam and any other beams within 15 dB of the strongest beam. In some cases, it may also be beneficial for the network to impose a bound on the maximum number of beams that the UE can report. For example, it was found in [1][3] that for one evaluation scenario, almost 70% reporting overhead reduction could be obtained with minimal prediction accuracy loss if 5 strongest beams were reported.

For NW-sided model inference, support NW configuration for UEs to pre-process set B beams to reduce reporting overhead, via:

  - Support configuring reporting of only beams within X dB of the strongest beam,

  - Support configuring reporting of at most N strongest set B beams.

When reporting information about measurements of multiple past time instances (in one reporting instance for BM-Case2), it is important to explore the possibility of reducing the reporting overhead. For example, the UE can report the past measurements as a temporal variance or a polynomial approximation.

For NW-sided model inference, support methods for UEs to compress the set B temporal domain measurement results to reduce the reporting overhead.

### Mechanisms to reduce training data collection overhead for NW sided model

Regarding data collection for NW-side AI/ML model of BM-Case1 and BM-Case2, the following approaches have been identified for overhead reduction: - the omission/selection of collected data - the compression of collected data - Note1: For the different purposes of data collection, the overhead reduction mechanisms and corresponding specification impacts may be different. - Note2: Support of any mechanism(s) (if necessary) for each LCM purpose and the potential spec impact (if any) are separate discussions - Note 3: UE complexity and power consumption should be considered

Reducing radio resource overhead for data collection can be achieved by means of minimizing the radio measurements and assistance information that needs to be collected for model training/inference/monitoring. For example, by only collecting the relevant samples, and configuring such data collection in certain favorable conditions. One example for collecting relevant data samples is to avoid reporting duplicated/similar samples from the UE, a static UE might experience similar channel conditions over a certain time period and should in such scenario avoid reporting duplicated samples when collecting data for training an AI/ML model. Also, there can be other issues related to the data collection, for example there is a sudden change in UE channel conditions during a large beam sweep. In this case such sample could be disregarded (removed) for overhead reduction.

For NW-sided data collection, RAN1 studies possible “omission/selection of collected data” by the following aspects as a starting point,

  - Possibility for UE to avoid signalling “duplicated” samples,

  - Possibility for UE to avoid signalling data based on certain events, one event can comprise that the UE experienced large channel variation during set A measurements.

  - Note: RAN2 can use such study when designing data collection procedures

# AI/ML Processing for UE-sided model

## CSI Processing Units

### #Issue #4 CPU & timeline For UE-side model, at least for AI/ML based beam management, for processing of a CSI report at least for inference, study the following options: - Option 1: The CPU occupied by the CSI report at least for inference and the CPU occupied by the non-AI/ML based CSI report for share the same pool. - Option 2: Introduce a dedicated CPU pool as AI/ML-based CSI processing unit (ACPU), FFS - Alt 1: Processing of a CSI report at least for inference only occupies a number of ACPU(s) for a number of symbols - Note: the legacy CPU pool will not impact by the activation of AI/ML functionality - Alt 2: Processing of a CSI report at least for inference occupies X number of ACPU(s) for M symbols and Y number of CPU(s) for N symbols - FFS on whether M=N, X=Y. - FFS whether AI/ML based beam management and other AI/ML based features, e.g., AI/ML based CSI prediction, share the same (A)CPU pool or not.

In the legacy framework, there is a restriction on the number of simultaneous CSI calculations that can be performed by the UE (higher-layer configured NCPU). For example, when the UE is configured with aPeriodic reporting with reporting quantity set to CRI-RSRSP, one CPU is occupied from the last symbol in PDCCH that triggered the report, to the last symbol carrying the corresponding report. In contrast, for periodic/semi-persistent reporting, the CPU is occupied from the latest CSI-RS resource used to calculate the report, until the reporting is performed. For the beam prediction feature, the UE needs to perform inference after measuring the last set B CSI resource. There is raised concern whether the standard needs to introduce a separate CSI processing unit or if one can reuse the existing limitations. It is unclear what number of calculations that needs to be done for a UE that might have a highly efficient model with a limited set of parameters. The input/output size is in the range of 4-64 beams, which is not that large in ML terms. Hence, it is not clear if the UE will use dedicated hardware for such inference or use the hardware for the CSI calculations.

It is up to UE implementation on whether to use dedicated AI hardware or reusing the hardware resources for legacy non-AI based CSI reporting to generate an AI based CSI report.

Due to the uncertainty in the UE hardware assumptions, our preference is to first conclude on the UE hardware assumption prior to introducing a new AI-BM-CPU definition.

RAN1 discuss and align on the assumptions of UE implementation for AI based CSI reporting first, before discussing the detailed solutions for the CSI processing criteria and timeline issues.

If it is the common understanding in RAN1 that at least for some AI-based CSI reporting features or under some conditions/configurations for an AI-based reporting feature, a UE can use dedicated AI hardware for generating a CSI report, then, there is a need to define a new type of processing unit, which we refer to as AI-CPU in the following discussion, to align the CPU counting and CSI processing timeline between the UE and the gNB for AI-based CSI reporting. A UE may indicate to the NW using UE capability signalling on which CPU type (e.g., legacy CPU or AI-CPU) will be used for generating CSI reports associated with an AI/ML based CSI reporting feature.

If dedicated AI hardware will be used at least for some AI-based CSI reporting, then, support separate processing unit pools (i.e., legacy CPU pool and AI-CPU pool) between CSI reporting generated using legacy CPU and CSI reporting generated using AI-CPU.

If a UE can use different types of process unit pools for different AI based CSI reporting features, then, support UE indicating the type of process unit pool (i.e., legacy CPU pool and AI-CPU pool) for CSI processing for an AI-based CSI reporting feature.

## Active CSI resources

The UE should not have more active CSI-RS ports or resources in active BWPs than its reported capability. The definition of the active NZP CSI-RS Resource duration is the following:

- Aperiodic CSI-RS: Active from the end of the PDCCH containing the request to the end of the scheduled PUSCH containing the report.

- Semi-persistent CSI-RS: Active from the end of the activation command to the end of the deactivation command.

- Periodic CSI-RS: Active from when it is configured by higher layer signaling to when the configuration is released.

The definition of active CSI-RS resources for the purpose of AI/ML inference operation needs to be discussed, the UE should not be configured with an infinite number of CSI-RS resources for the purpose of inference, however, these CSI-RS resources are not transmitted for inference, so it is counter-intuitive to denote them as active when configured. One solution can comprise that the CSI-RS resources part of set A is not considered as active even when they are triggered/configured. This would enable the NW to configure many non-transmitted CSI-RS resources for the purpose of set A.

Even if the predicted resources are inactive, the UE should have a limit on the number of CSI-RS resources it can predict. However, the number of resources is not the main issue, as the model's complexity remains similar whether it predicts 32 or 64 resources. Instead, the associated ID used to inform the UE about the network transmission properties on set A/B may impact the models used by the UE. Therefore, it may be more complex for a UE to manage two associated IDs with 16 CSI-RS resources each, compared to a single associated ID with 32 CSI-RS resources. It would be more reasonable to include a limit on the number of associated IDs that could be configured, since each associated ID may correspond to a separate model.

Regarding active CSI-RS resources for AI/ML

  - Predicted CSI-Resources that are part of set A for inference is not considered as active,

  - Measured CSI-Resources that are part of set A for training/monitoring is considered as active,

  - The number of simultaneously active associated IDs needs to be restricted and indicated via UE capability.

## CSI timeline

For aPeriodic CSI reporting, two timing values have been introduced for ensuring that UEs have sufficient time in measuring and calculating the CSI feedback. The Z3 defines the time for DCI to the UE reporting on PUSCH, while Z3’ additionally indicates the time required from last measured symbol to the PUSCH report. For AI/ML inference, the UE need to calculate the corresponding predictions within the existing definitions. Our understanding is that models can be of low complexity, and it is unclear if there is motivation on introducing new values for AI/ML.

For UE-sided model CSI processing timeline, existing values of Z3 and Z3’ for aPeriodic CSI reporting may be sufficient. RAN1 to discuss and align on the assumptions of UE implementation before concluding if new values are needed.

# Conclusions

In the previous sections we made the following observations:

Observation 1	In an Urban macro deployment with 64 beams, having a prediction performance metric based on the Top-4 beams in a monitoring resource set, will in 20% of the cases also label a beam with more than 4dB difference to the strongest as a correct prediction.

Observation 2	The monitoring resource set should be selected that best represent the UE inference channel conditions and its predictions. RAN1 should further discuss an exact definition.

Observation 3	The resource mapping of an NZP-CSI-RS-resources may not be the same in the monitoring and training data collection step. The NW does not need to configure UE with training data collection resources if UE has a trained model, and can optimize the time-domain allocation of the resource for monitoring (with possible subsets of set A beams).

Observation 4	It is up to UE implementation on whether to use dedicated AI hardware or reusing the hardware resources for legacy non-AI based CSI reporting to generate an AI based CSI report.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For UE-sided model, in CSI-ReportConfig, the resourceConfig for set A can only include a single resource set.

Proposal 2	For UE-sided model, for aPeriodic CSI inference report configuration, AssociatedReportConfigInfo comprises the UE assumptions regarding set B.

Proposal 3	For UE-sided model, for aPeriodic CSI inference report configuration, when set B is NOT a subset of set A, the resourceConfig for set B can only include a single resourceSet.

Proposal 4	For UE-sided model, for configuring the data collection resources, do not support aPeriodic CSI-RS

Proposal 5	For the UE-sided inference result report, if not supporting UE report of probabilities, support UE indicating in UCI if the inference result report is inaccurate/invalid (e.g. setting the CRI or SSB-RI to a certain “dummy” value)

Proposal 6	For the UE-sided BM-Case 2 inference report configuration, support NW configuring the number K of UE-reported beams for each time instance.

Proposal 7	For the UE-sided model performance metric, support, at least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s). Where K is the number of predicted beam(s) in the corresponding inference report,

	Where Top-M beams are the M beams that are within x dB of the best beam with the largest measured value(s) of L1-RSRP(s) of the resource set(s)

	X is NW configured on CSI report configuration for monitoring

	X= 0 dB,1 dB, 3 dB

Proposal 8	For UE-sided performance monitoring, for the reportConfigType of the UE-sided model performance metric,

	Support periodic and semi-persistent reportConfigType if the linked inference report is also periodic or semi-persistent

	Support aPeriodic monitoring report only for the scenario where the UE has first been configured with a periodic or semi-persistent monitoring/inference resource config type

Proposal 9	For UE-sided performance monitoring, support joint activation of the monitoring and linked inference report configuration

Proposal 10	For UE-sided model, to identify the connection between RSs in the resource set(s) for monitoring and Set A beams:

	UE can assume the same NW transmission parameters for the resourceIDs (all or a subset) that are present both in the monitoring and set A resourceSet,

	Note: The resource mapping for monitoring resourceIDs may not be the same as during the UE training data collection

Proposal 11	For UE-assisted performance metric calculation, at least support an event where the UE can early indicate if a functionality is not working properly, for example prediction accuracy is below 50% after collecting N/2 samples

Proposal 12	For enabling a P2 sweep based on predicted Top-K beams, RAN1 considers the following two solutions:

	Option 1: NW indicates that the corresponding beam IDs/TCI states for the P2 ResourceSet follows the UE predicted Top-K beams, FFS on details including,

i.	linkage of the inference report and the P2 (prediction result measurements) CSI-Report configuration, including BM-Case 2 when UE predicts N time instances

ii.	transmission order of the beams in the subsequent P2 (prediction result measurements)

	Option 2: For both UE/NW sided model, NW configures dynamically as part of the MAC CE the beam IDs or TCI states that are part of the Top-K beam sweep.

i.	FFS on details including the signaling overhead

	FFS other options, including

i.	Option 3: NW configures a number of aPeriodicTriggerStates, and the UE provides Top-K predictions restricted to the valid TCI states matching one of the aPeriodicTrigger states,

ii.	Option 4: The NW uses legacy signaling, and in a best-effort manner configure a set of aPeriodicTriggerStates

iv.	Note 1: Option 3 and 4 would imply reduced performance since P2 cannot support all Top-K combinations

	Note 2: NW indicating beam IDs could be used during monitoring

Proposal 13	For NW-sided model, regarding max number of reported beam related information in one report, support reporting all measured beams in a resourceSet.

Proposal 14	For NW-sided model inference, support NW configuration for UEs to pre-process set B beams to reduce reporting overhead, via:

	Support configuring reporting of only beams within X dB of the strongest beam,

	Support configuring reporting of at most N strongest set B beams.

Proposal 15	For NW-sided model inference, support methods for UEs to compress the set B temporal domain measurement results to reduce the reporting overhead.

Proposal 16	For NW-sided data collection, RAN1 studies possible “omission/selection of collected data” by the following aspects as a starting point,

	Possibility for UE to avoid signalling “duplicated” samples,

	Possibility for UE to avoid signalling data based on certain events, one event can comprise that the UE experienced large channel variation during set A measurements.

	Note: RAN2 can use such study when designing data collection procedures

Proposal 17	RAN1 discuss and align on the assumptions of UE implementation for AI based CSI reporting first, before discussing the detailed solutions for the CSI processing criteria and timeline issues.

Proposal 18	If dedicated AI hardware will be used at least for some AI-based CSI reporting, then, support separate processing unit pools (i.e., legacy CPU pool and AI-CPU pool) between CSI reporting generated using legacy CPU and CSI reporting generated using AI-CPU.

Proposal 19	If a UE can use different types of process unit pools for different AI based CSI reporting features, then, support UE indicating the type of process unit pool (i.e., legacy CPU pool and AI-CPU pool) for CSI processing for an AI-based CSI reporting feature.

Proposal 20	Regarding active CSI-RS resources for AI/ML

	Predicted CSI-Resources that are part of set A for inference is not considered as active,

	Measured CSI-Resources that are part of set A for training/monitoring is considered as active,

	The number of simultaneously active associated IDs needs to be restricted and indicated via UE capability.

Proposal 21	For UE-sided model CSI processing timeline, existing values of Z3 and Z3’ for aPeriodic CSI reporting may be sufficient. RAN1 to discuss and align on the assumptions of UE implementation before concluding if new values are needed.

# References

3GPP TR 38.843 V18.0.0 (2024-01), Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18).

RP-234039, "New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface," Qualcomm (Moderator), 3GPP TSG RAN Meeting #102, Edinburgh, Scotland, December 11-15, 2023.

R1-2304749, Evaluations of AIML for beam management, Ericsson, 3GPP TSG-RAN WG1 Meeting 113, May 22-26, 2023.