3GPP TSG RAN WG1 #120-bis 					                     R1-2502015

Wuhan, China, Apr 7th – 11th, 2025

Source:                Tejas Networks Pvt. Ltd.

Title:                     Discussion on gNB-gNB CLI handling

Agenda Item:       9.3.3

Document for:     Discussion and decision

# Introduction

The RAN#104 meeting discussed and revised the Work Item Description on the Evolution of NR Duplex Operation in RP-241614[1].

Agreement

Followings are assumed based on TR 38.858

  - SBFD at the gNB side

  - Half duplex operation at the UE side

  - FR1 and FR2-1

  - SBFD operation Option 4, i.e., both time and frequency locations of sub-bands for SBFD operation are known to SBFD aware UEs

  - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side

  - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies

  - One UL sub-band for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier

- Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators

For enhancements for CLI handling, the following are recommended from RAN1's perspective:

- Information exchange of semi-static cell-specific SBFD time and frequency location configuration [RAN3]

- Information exchange of measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS [RAN3]

- Information exchange of strongest DL beam information [RAN3]

  - Information exchange of CLI-mitigation request [RAN3]

  - Note: No normative behaviour is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signalling can be discussed in RAN3.

- UL resource muting for PUSCH, including [RAN1, RAN2, RAN4]

  - Indication/Determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain

    - Note: No new DCI field / MAC CE is introduced

  - PUSCH resource mapping, i.e., rate-matching around the muted REs

  - UCI resource determination in symbols with muted REs

    - Note: No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212.

  - Note: UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH.

  - Measurement reporting

    - At least aperiodic reporting

      - Note: Periodic and semi-persistent reporting can be considered

    - New report quantities: e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices

      - Note: At least wideband reporting is supported

    - UCI bits generation

    - Priority rules for multiple CSI reporting

    - Note: The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point.

  - CLI measurement accuracy requirement [RAN4]

- Note: The above information exchanges are subject to RAN3’s assessment of feasibility and specification impact.

In this contribution, we provide our views on the indication/determination of UL resource muting for PUSCH, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in the time domain and UCI resource determination, and power boosting of the UL muting symbol.

# Introduction to SBFD CLI handling

In the case of SBFD UL reception, the gNB receives the signal from its serving UEs (desired signal), and it also receives CLI interference from the neighbouring gNBs of the same network and/or different network gNBs, due to which there is a significant drop in UL throughput performance. To measure the interference for its mitigation, UE needs to mute some of the symbols such that the UL receiver at gNB can measure the interference due to neighbouring gNBs accurately and use it for equalizer weight calculation to suppress interference. The UL Resource muting for PUSCH is done assuming comb-2 for DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in the time domain per slot. As precoding/beamforming applied for PDCCH symbols and PDSCH symbols can be different, this would lead to uplink symbols in a slot experiencing different gNB-gNB CLI. To measure the gNB-gNB CLI, in uplink, we can mute one symbol out of the first three symbols of the slot to capture the interference due to PDCCH channel leakage of the interfering gNB and one OFDM symbol out of the remaining symbols of the slot to measure the gNB-gNB interference due to PDSCH channel leakage. The information pertaining to the symbol(s) to be muted needs to be present at both UE and gNB.

### UL resource muting for transform precoding enabled

When transform precoding is enabled and an UL muting symbol overlaps a symbol containing PT-RS, the following 3 alternatives are studied for PT-RS insertion in a block of MscPUSCH2 samples prior to transform precoding in moderator summary of RAN1-120 [7]:

- Alt-1

  - NgroupPT-RS and Nsampgroup are determined as legacy from 38.214 Table 6.2.3.2-1

  - PT-RS sample positions determined as legacy, but with MscPUSCH replaced by MscPUSCH2  in 38.211 Table 6.4.1.2.2.2-1

- Alt-2

  - NgroupPT-RS and Nsampgroup are determined as legacy, but with NRB replaced by NRB2 in 38.214 Table 6.2.3.2-1

  - PT-RS sample positions determined as legacy, but with MscPUSCH replaced by MscPUSCH2  in 38.211 Table 6.4.1.2.2.2-1

- Alt-3

  - NgroupPT-RS and Nsampgroup are determined as legacy from 38.214 Table 6.2.3.2-1

  - PT-RS sample positions are determined as legacy, but only for the first NgroupPT-RS2 groups

In alternative-1, substituting MscPUSCH with MscPUSCH2 in 3GPP TS 38.211 Table 6.4.1.2.2.2-1 confines all the PT-RS sample positions to the first half of the symbol (i.e., within MscPUSCH2 positions). This adjustment effectively doubles the PT-RS density compared to scenarios where UL resource muting is disabled but symbols contain PT-RS. The increased density arises because the same number of PT-RS samples are concentrated within half the sample positions when muting is enabled.

Observation 1:  In case of alternative-1 the PT-RS density will increase for the symbol containing PT-RS with UL resource muting as compared to the symbol containing PT-RS without UL resource muting when transform precoding is enabled.

In the case of alternative-2, the density of PTRS may not remain consistent to the symbol containing PT-RS when comparing scenarios with and without UL resource muting.

Example-1, if NRB is greater than or equal to NRB4, replacing NRB with NRB/2 in Table 6.2.3.2-1 of 38.214 could lead to two different outcomes. If NRB/2 is less than NRB4 but greater than or equal to NRB3, the number of PT-RS groups and samples per group would both be 4, resulting in the same PTRS density for muted and non-muted symbols. However, if NRB/2 is less than NRB3 but greater than or equal to NRB2, the number of PT-RS groups would be 4 but the number of samples per group would be 2, leading to a lower PTRS density in muted symbols compared to non-muted symbols.

Example-2, if NRB is greater than or equal to NRB2 but less than NRB3 , replacing NRB with NRB/2 in Table 6.2.3.2-1 of 38.214 could also lead to two different outcomes. If NRB/2 is less than NRB2 but greater than or equal to NRB1, the number of PT-RS groups would be 2 but samples per group would both be 4, resulting in the higher PTRS density for muted as compared to non-muted symbols. However, if NRB/2 is less than NRB1 but greater than or equal to NRB0, the number of PT-RS groups would be 2 and the number of samples per group would also be 2, leading to a same PTRS density in muted symbols and non-muted symbols.

Table 6.2.3.2-1: PT-RS group pattern as a function of scheduled bandwidth

| Scheduled bandwidth | Number of PT-RS groups | Number of samples per PT-RS group |
| --- | --- | --- |
| NRB0 NRB < NRB1 | 2 | 2 |
| NRB1 NRB < NRB2 | 2 | 4 |
| NRB2 NRB < NRB3 | 4 | 2 |
| NRB3 NRB < NRB4 | 4 | 4 |
| NRB4 NRB | 8 | 4 |

Observation 2: When transform precoding is enabled and an UL muting symbol overlaps a symbol containing PT-RS, if we replace NRB with NRB/2 in Table 6.2.3.2-1 of 38.214, the PTRS density does not remain consistent with respect to the symbol containing PTRS without UL resource muting.

In the context of alternative-3, the positions of PT-RS samples are determined using the legacy method. However, only half of the PT-RS groups, specifically the first NgroupPT-RS2 groups, are utilized for PT-RS transmission during the UL muting symbol. This ensures that the PT-RS density on the UL muting symbol remains consistent with that on non-muting symbols. As, the PT-RS sample positions are calculated based on MscPUSCH as in Table 6.4.1.2.2.2-1 of TS 38.211 for UL muting symbol also, resulting in the PT-RS sample positions for the UL muting symbol being identical to the PT-RS sample positions of the first  NgroupPT-RS2 PT-RS groups of non-muting symbols.

Proposal 1: When transform precoding is enabled and an UL muting symbol overlaps a symbol containing PT-RS, we support the PT-RS sample positions are determined as legacy, but only the first NgroupPT-RS2 groups are used for PT-RS transmission.

Agreement

For UL muting applied for UL TBoMS, the index of the starting coded bit in a slot, except the first slot, within the 𝑁𝑠 slots allocated for the transmission of TB processing over multiple slots is determined according to Section 6.2.5, TS38.212 as follows

k0=k0'+H+τmodNcb

and H is down-selected from one of the following options

  - Option 1: 𝐻 is the total number of coded bits available for transmission of the transport block in the previous slot within the 𝑁𝑠 slots assuming no UCI multiplexing and assuming no muted REs.

  - Option 2: 𝐻 is the total number of coded bits available for transmission of the transport block in the previous slot within the 𝑁𝑠 slots assuming no UCI multiplexing and considering the muted REs

Note: No impact on TBS determination in Option 1 or Option 2.

UL muting applied for UL TBoMS

We believe that Option 1 has a lesser specification impact compared to Option 2. Additionally, the complexity at the User Equipment (UE) remains the same as in legacy systems for Option 1. However, Option 2 introduces additional complexity by requiring the subtraction of the number of bits used to transmit on muted Resource Elements (REs). Therefore, we support Option 1 for the calculation of H.

Proposal 2:  For UL muting applied for UL TBoMS, the index of the starting coded bit in a slot, except the first slot, within the 𝑁𝑠 slots allocated for the transmission of TB processing over multiple slots is determined according to Section 6.2.5, TS38.212 as follows

k0=k0'+H+τmodNcb

and H is the total number of coded bits available for transmission of the transport block in the previous slot within the 𝑁𝑠 slots assuming no UCI multiplexing and assuming no muted REs.

### UL resource muting for PUSCH that is a retransmission

As we know, PUSCH support up to 3 retransmissions for a HARQ process, it may happen that 2nd and 3rd retransmission happen after large delay with respect to PUSCH new data transmission of a HARQ process. This large delay in 2nd and 3rd retransmission can cause change in the CLI from neighbouring gNBs with respect to PUSCH new data transmission of that HARQ process, which require the UL resource muting for CLI measurement.

Proposal 3: Support UL resource muting for the retransmission if UL resource muting is done for new data transmission for that PUSCH.

### UL resource muting for PUSCH carrying certain UCI type with high priority

The RS, HARQ, RRC signalling and CSI are the high priority UCIs. The UCI type HARQ and RS have very low payload size. We think, the performance of these UCI types does not degrade significantly due to UL resource muting. For UCI type CSI and RRC signalling, we can consider UL resource muting to be not applied.

Proposal 4: Support PUSCH carrying certain UCI type with high priority, UL resource muting is not applied.

### UL resource muting for multi-PUSCH scheduled by a single DCI

As, the time domain resources allocation for the each PUSCH transmission are configured separately for all the PUSCH scheduled by a single DCI. The time location of UL resource muting can be turned ON/OFF dynamically for each PUSCH using TDRA field in DCI.

Proposal 5: Support the UL resource muting for each of the PUSCH transmission of multi-PUSCH scheduled by a single DCI can be turned ON/OFF using their TDRA field in DCI.

### UL resource muting for a PUSCH with the following conditions

- Condition 1: When the number of PUSCH symbols is less than threshold, UL resource muting is not applied

- Condition 2: If the scheduled PUSCH uses a MCS lower than certain reference MCS index, UL resource muting is not applied

- Condition 3: When the number of scheduled RBs is less than threshold, UL resource muting is not applied

- Condition 4: When the number of DMRS symbols is larger than threshold, UL resource muting is not applied

The condition 1, 2, 3 and 4, can be handled using dynamic turn ON/OFF UL resource muting using TDRA field in DCI. Hence, no discussion required on the condition 1,2 3 and 4.

Proposal 6: The UL resource muting can be turned ON/OFF dynamically using TDRA field in DCI. Hence no conditional check required for following conditions,

- Condition 1: When the number of PUSCH symbols is less than threshold, UL resource muting is not applied

- Condition 2: If the scheduled PUSCH uses a MCS lower than certain reference MCS index, UL resource muting is not applied

- Condition 3: When the number of scheduled RBs is less than threshold, UL resource muting is not applied

- Condition 4: When the number of DMRS symbols is larger than threshold, UL resource muting is not applied

# Conclusion

In this contribution, we discuss about CLI handling with the following proposals and observations.

Observation 1:  In case of alternative-1 the PT-RS density will increase for the symbol containing PT-RS with UL resource muting as compared to the symbol containing PT-RS without UL resource muting when transform precoding is enabled.

Observation 2: When transform precoding is enabled and an UL muting symbol overlaps a symbol containing PT-RS, if we replace NRB with NRB/2 in Table 6.2.3.2-1 of 38.214, the PTRS density does not remain consistent with respect to the symbol containing PTRS without UL resource muting.

Proposal 1: When transform precoding is enabled and an UL muting symbol overlaps a symbol containing PT-RS, we support the PT-RS sample positions are determined as legacy, but only the first NgroupPT-RS2 groups are used for PT-RS transmission.

Proposal 2:  For UL muting applied for UL TBoMS, the index of the starting coded bit in a slot, except the first slot, within the 𝑁𝑠 slots allocated for the transmission of TB processing over multiple slots is determined according to Section 6.2.5, TS38.212 as follows

k0=k0'+H+τmodNcb

and H is the total number of coded bits available for transmission of the transport block in the previous slot within the 𝑁𝑠 slots assuming no UCI multiplexing and assuming no muted REs.

Proposal 3: Support UL resource muting for the retransmission if UL resource muting is done for new data transmission for that PUSCH.

Proposal 4: Support PUSCH carrying certain UCI type with high priority, UL resource muting is not applied.

Proposal 5: Support the UL resource muting for each of the PUSCH transmission of multi-PUSCH scheduled by a single DCI can be turned ON/OFF using their TDRA field in DCI

Proposal 6: The UL resource muting can be turned ON/OFF dynamically using TDRA field in DCI. Hence no conditional check required for following conditions,

- Condition 1: When the number of PUSCH symbols is less than threshold, UL resource muting is not applied

- Condition 2: If the scheduled PUSCH uses a MCS lower than certain reference MCS index, UL resource muting is not applied

- Condition 3: When the number of scheduled RBs is less than threshold, UL resource muting is not applied

- Condition 4: When the number of DMRS symbols is larger than threshold, UL resource muting is not applied

# Reference

- RP-241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD).

- RP-240172, Evolution of NR duplex operation: Sub-band full duplex (SBFD).

- 3GPP TR 38.858: “Study on Evolution of NR Duplex Evolution”.

- R1-2403894, Discussion on gNB-gNB CLI handling, Tejas Networks Ltd.

- RP-240789, Evolution of NR duplex operation: Sub-band full duplex (SBFD).

- Chair notes RAN1 119 eom0.

- R1-2501457, Summary #3 of CLI handling