3GPP TSG RAN WG1 #120bis	                                   					       												                           R1-2502741

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	9.15.6

Source:	Moderator (AT&T)

Title:	Summary of UE features for NR mobility enhancements Phase 4

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion [120bis-R19-UE_features] during RAN1 #120bis. According to the Chair’s Notes:

[120bis-R19-UE_features] Email discussion on Rel-19 UE features – Ralf (AT&T), Naoya (DOCOMO), Hiroki (DOCOMO) - To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

The following was discussed during RAN1 #120bis within the scope of [120bis-R19-UE_features].

# Summary of Contributions Submitted to RAN1 #120bis

The following is the moderator’s summary of contributions submitted to RAN1 #120bis in this agenda item.

| Company | Summary |
| --- | --- |
| ZTE Corporation/Sanechips [1] | Intra- and inter-frequency measurement In Rel-18 LTM, SSB based intra/inter-frequency L1-RSRP measurement and reporting have been supported. In Rel-19 LTM, it was agreed to introduce CSI-RS based measurement and RAN1 also discussed about whether it is necessary to define L1 intra-frequency and inter-frequency measurement for CSI-RS, and then the following conclusion in RAN1#118bis meeting was reached. Agreement From RAN1 perspective, there is no restriction with regards to the frequency location of CSI-RS used for L1-measurement. Based on the above agreement, it means that whether to distinguish CSI-RS based L1 intra-frequency and inter-frequency measurement will be up to RAN4. Thus, RAN1 should wait for RAN4’s progress on this issue. Semi-persistent CSI-RS In RAN1#120 meeting, it was agreed that SP CSI-RS is supported for candidate cell L1-RSRP measurement and whether to support it will be up to UE capability, corresponding conclusion is as below. Thus, we propose to add a feature on SP CSI-RS. Agreement - Confirm the following working assumption made in RAN1#118bis Working Assumption(RAN1#118bis) In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 Send an LS to RAN2 to inform this agreement. Final LS in R1-2501500.. CSI-RS based L1-RSRP measurement reporting According to the following agreements in previous meetings, it has been clearly stated that Rel-18 LTM CSI reporting configuration will be extended to Rel-19 CSI-RS based L1-RSRP measurement reporting. Thus, the existing FGs related L1-RSRP measurement reporting in Rel-18 LTM should be updated to accommodate CSI-RS measurement result. Agreement (RAN1#118) - CSI-RS based L1-RSRP report is supported for gNB scheduled measurement reporting - FFS: CSI-RS based L1-SINR report is supported for gNB scheduled measurement reporting - Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting Agreement (RAN1#118bis) The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows: - UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI. - Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18) - The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used - No L1 specified filtering for time and spatial domain is introduced - No enhancement on how to report L cells x M beams - Periodic reporting on PUCCH is supported - FFS: semi-persistent reporting on PUCCH/PUSCH, and aperiodic reporting on PUSCH Agreement (RAN1#118bis) For CSI-RS based L1-measurement report by gNB scheduled measurement reporting, semi-persistent reporting on PUCCH/PUSCH and aperiodic reporting on PUSCH are supported. CSI-RS as QCL source RS in LTM TCI state In RAN1#119 meeting, CSI-RS has been agreed as QCL source RS in LTM TCI state, as below. In such case, the existing TCI state related FGs in Rel-18 LTM should be updated to accommodate CSI-RS as QCL source RS in LTM TCI state, such as FG 45-3/3a/4/4a. Agreement (RAN1#119) CSI-RS for BM as the referenceSignal with QCL-TypeD is supported for an LTM TCI state, where TRS is configured as referenceSignal with QCL-TypeA. CSI acquisition In RAN1#119 meeting, it has reached a working assumption on introducing a UE capability for CSI-RS measurement before receiving LTM Cell Switch Command MAC CE and confirming it in RAN1#120 meeting, as yellow highlighted part. Working Assumption (RAN1#119) As baseline, CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE. - The report is sent directly to target cell - Introduce UE capability for CSI-RS measurement can start before reception of LTM CSC MAC CE Other than UE capability, strive for no additional spec impact compared to the baseline (only one triggering mechanism will be specified) Agreement (RAN1#120) For target cell CSI acquisition, - A UE is provided with RRC configurations for periodic CSI-RS resource(s) and CSI report(s) for one or more candidate cell - For a candidate cell, - down-select from the following alternatives: - Alt 1: A single CSI report configuration is configured - Alt 2: Multiple CSI report configurations can be configured - down-select from the following alternatives: - Alt X: A single CSI-RS resource for CMR is associated with a CSI report configuration - Alt Y: Multiple CSI-RS resources for CMR can be associated with a CSI report configuration - FFS: Semi-persistent CSI-RS resource - After the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability - FFS: whether or how to select a subset of CSI-RS resources to measure - FFS: when the UE may start measuring the configured CSI-RS resources - UE determines the CSI report configuration based on the CSC - After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration - The latest available measured CSI on target cell resource(s) is conveyed at least by a single report, and the report is sent to the target cell - Option 1: to use UCI - Option 2: to use MAC CE Note: with this agreement, the working assumption made in RAN1#119 is automatically confirmed.. Based on the above, we have the following initial thoughts for UE features on CSI acquisition: - CSI-RS measurement and CSI reporting for CSI acquisition after LTM Cell Switch Command MAC CE is a mandatory feature. - CSI-RS measurement for CSI acquisition before LTM Cell Switch Command MAC CE is an optional feature. - Regarding the maximum number of candidate cells which UE supports CSI acquisition for, it can be any one of: 1, 2, 3, 4, 5, 6, 7. In such case, the maximum value is not set to 8 because the maximum number of candidate cells is 8 and serving cell can be configured as one of candidate cells. - Whether to introduce additional UE feature of selecting a subset of CSI-RS resources for CSI measurement from a set of CSI-RS resources can be determined after further conclusion is reached. |
| Vivo [2] | Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (Sidelink WI only)”.; Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. Xx_NR_Mobility_PH4; Xx-1-1; gNB scheduled Intra-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure measured on periodic CSI-RS; 1. Support of intra-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16} Component 4 candidate values: Aperiodic: {0,1,2,3,4} Periodic: {1,2,3,4} Semi-persistent: {0,1,2,3,4}; Optional with capability signaling. Xx_NR_Mobility_PH4; Xx-1-2; gNB scheduled Intra-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure measured on semi-persistent CSI-RS; 1. Support of intra-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs; Xx-1-1; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16} Component 4 candidate values: Aperiodic: {0,1,2,3,4} Semi-persistent: {1,2,3,4}; Optional with capability signaling. Xx_NR_Mobility_PH4; Xx-1-3; gNB scheduled Inter-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure measured on periodic CSI-RS; 1. Support of inter-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for inter-frequency L1-RSRP measurement 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for inter-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs; Xx-1-1; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16} Component 4 candidate values: Aperiodic: {0,1,2,3,4} Periodic: {1,2,3,4} Semi-persistent: {0,1,2,3,4}; Optional with capability signaling. Xx_NR_Mobility_PH4; Xx-1-4; gNB scheduled Inter-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure measured on semi-persistent CSI-RS; 1. Support of inter-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for inter-frequency L1-RSRP measurement 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for inter-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs; Xx-1-1, Xx-1-3; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16} Component 4 candidate values: Aperiodic: {0,1,2,3,4} Semi-persistent: {1,2,3,4}; Optional with capability signaling. Xx_NR_Mobility_PH4; Xx-1-5; Event triggered L1-RSRP reporting measured on SSB for event LTM2; 1. Support of even triggered L1-RSRP reporting measured on SSB for event LTM2; Optional with capability signaling. Xx_NR_Mobility_PH4; Xx-1-6; Event triggered L1-RSRP reporting measured on SSB for event LTM3; 1. Support of even triggered L1-RSRP reporting measured on candidate cell(s) SSB for event LTM3 2. Maximum number of RRC configured candidate cells for even triggered L1-RSRP measurement 3. Support of up to L candidate cells and M beams in one report where a SSBRI-RSRP pair is used for each beam report L1-RSRP measurement; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16}; Optional with capability signaling. Xx_NR_Mobility_PH4; Xx-1-7; Event triggered L1-RSRP reporting measured on SSB for event LTM4; 1. Support of even triggered L1-RSRP reporting measured on candidate cell(s) SSB for event LTM4 2. Maximum number of RRC configured candidate cells for even triggered L1-RSRP measurement 3. Support of up to L candidate cells and M beams in one report where a SSBRI-RSRP pair is used for each beam report L1-RSRP measurement; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16}; Optional with capability signaling. Xx_NR_Mobility_PH4; Xx-1-8; Event triggered L1-RSRP reporting measured on SSB for event LTM5; 1.Support of even triggered L1-RSRP reporting measured on candidate cell(s) SSB for event LTM5 2. Maximum number of RRC configured candidate cells for even triggered L1-RSRP measurement 3. Support of up to L candidate cells and M beams in one report where a SSBRI-RSRP pair is used for each beam report L1-RSRP measurement; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16}; Optional with capability signaling. Xx_NR_Mobility_PH4; Xx-1-9; Event triggered L1-RSRP reporting measured on periodic CSI-RS for event LTM2; 1.Support of even triggered L1-RSRP reporting measured on periodic CSI-RS for event LTM2; Xx-1-5; Optional with capability signaling. Xx_NR_Mobility_PH4; Xx-1-10; Event triggered L1-RSRP reporting measured on periodic CSI-RS for event LTM3; 1.Support of even triggered L1-RSRP reporting measured on candidate cell(s) periodic CSI-RS for event LTM3 2. Maximum number of RRC configured candidate cells for even triggered L1-RSRP measurement 3. Support of up to L candidate cells and M beams in one report where a SSBRI-RSRP pair is used for each beam report L1-RSRP measurement; Xx-1-6; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16}; Optional with capability signaling. Xx_NR_Mobility_PH4; Xx-1-11; Event triggered L1-RSRP reporting measured on periodic CSI-RS for event LTM4; 1.Support of even triggered L1-RSRP reporting measured on candidate cell(s) periodic CSI-RS for event LTM4 2. Maximum number of RRC configured candidate cells for even triggered L1-RSRP measurement 3. Support of up to L candidate cells and M beams in one report where a SSBRI-RSRP pair is used for each beam report L1-RSRP measurement; Xx-1-7; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16}; Optional with capability signaling. Xx_NR_Mobility_PH4; Xx-1-12; Event triggered L1-RSRP reporting measured on periodic CSI-RS for event LTM5; 1.Support of even triggered L1-RSRP reporting measured on candidate cell(s) periodic CSI-RS for event LTM5 2. Maximum number of RRC configured candidate cells for even triggered L1-RSRP measurement 3. Support of up to L candidate cells and M beams in one report where a SSBRI-RSRP pair is used for each beam report L1-RSRP measurement; Xx-1-8; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16}; Optional with capability signaling. Xx_NR_Mobility_PH4; Xx-1-13; CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS; 1.Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on candidate(target) cell periodic CSI-RS; Optional with capability signaling. Xx_NR_Mobility_PH4; Xx-1-14; CSI-RS measurement before reception of LTM CSC MAC CE based on periodic CSI-RS; 1.Support of CSI-RS measurement before reception of CSC MAC CE and CSI reporting after reception of LTM CSC MAC CE based on candidate(target) cell periodic CSI-RS; Xx-1-14; Optional with capability signaling. |
| CATT [3] | According to the revised WID[1], the WI object on Measurements related enhancements for purpose of supporting LTM was agreed as follows: - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.. Compared with Rel-18 LTM, the following three key enhancements on L1 measurements have been specified in Rel-19 LTM: - support event triggered L1 measurement reporting - support CSI-RS measurements for LTM procedures - support CSI acquisition on candidate cell(s) based on CSI-RS Since UE can work independently with each feature, these enhancements should be defined as separate basic UE features. Based on RAN1 agreements made in the previous meetings, we have the following proposals [2-6]: Proposal 1: On L1 measurement for Rel-19 LTM, introduce three basic UE features: - Event triggered L1 measurement reporting - CSI-RS measurements for LTM procedures - CSI acquisition on candidate cell(s) Proposal 2: For FG event triggered L1 measurement reporting, the following components are at least included: - Support of event triggered L1 measurement reporting based on one event instance - Support of Event LTM2/LTM3/LTM4/LTM5 based measurement and report - Support of SSB and CSI-RS measurement for event triggered L1 measurement reporting Proposal 3: For FG CSI-RS measurements for LTM procedures, the following components are at least included: - Support periodic CSI-RS for candidate cell L1-RSRP measurement for gNB scheduled reporting - Support peridic, semi-persistent and aperiodic reporting Proposal 4: For FG CSI acquisition on candidate cell(s), the following components are at least included: - Support CSI reporting to the target cell after reception of LTM CSC MAC-CE - Support periodic CSI-RS measurement after reception of LTM CSC MAC-CE In RAN1#120 meeting [2], the support of semi-persistent CSI-RS for candidate cell L1-RSRP measurement for gNB scheduling reporting has been agreed to be subjected to UE capability. Therefore, besides the basic UE feature, a separate UE feature should be introduced on gNB scheduled L1-RSRP reporting. Agreement - Confirm the following working assumption made in RAN1#118bis Working Assumption(RAN1#118bis) In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 Send an LS to RAN2 to inform this agreement. Final LS in R1-2501500.. Proposal 5: With the prerequisite of FG CSI-RS measurements for LTM procedures, a separate UE feature, semi-persistent CSI-RS measurements for LTM procedures, is introduced on gNB scheduled L1-RSRP reporting. The component at least includes: - Support Semi-persistent CSI-RS for candidate cell L1-RSRP measurement for gNB scheduled reporting In RAN1#120 meeting [2], the following agreement on target cell CSI acquisition was achieved. Before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability. Therefore, a separate UE feature should be introduced on CSI acquisition. Agreement For target cell CSI acquisition, - A UE is provided with RRC configurations for periodic CSI-RS resource(s) and CSI report(s) for one or more candidate cell - For a candidate cell, - down-select from the following alternatives: - Alt 1: A single CSI report configuration is configured - Alt 2: Multiple CSI report configurations can be configured - down-select from the following alternatives: - Alt X: A single CSI-RS resource for CMR is associated with a CSI report configuration - Alt Y: Multiple CSI-RS resources for CMR can be associated with a CSI report configuration - FFS: Semi-persistent CSI-RS resource - After the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability - FFS: whether or how to select a subset of CSI-RS resources to measure - FFS: when the UE may start measuring the configured CSI-RS resources - UE determines the CSI report configuration based on the CSC - After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration - The latest available measured CSI on target cell resource(s) is conveyed at least by a single report, and the report is sent to the target cell - Option 1: to use UCI - Option 2: to use MAC CE Note: with this agreement, the working assumption made in RAN1#119 is automatically confirmed.. Proposal 6: With the prerequisite of FG CSI acquisition on candidate cell(s), a separate UE feature, CSI-RS measurement before LTM CSC MAC-CE, is introduced on CSI acquisition. The component at least includes: Support CSI-RS measurement starts before reception of LTM CSC MAC CE For conditional intra-CU LTM, UE performs the beam measurement according to the gNB configuration, which could be based on L1 measurements or L3 measurements. UE checks the measurement results with the execution condition, if the execution condition in the event(e.g. condA3/condA5) is satisfied, UE will perform handover. Since UE can work independently during this procedure whatever event triggered L1 measurement reporting/gNB scheduling based L1 measurement exists or not. Conditional intra-CU LTM can also be supported as a basic feature separately. Proposal 7: Support conditional intra-CU LTM as a basic UE feature. Proposal 8: For FG conditional intra-CU LTM, the following components are at least included: - Support of conditional LTM based on one event instance. - Support of Event LTM3/LTM5 based conditional LTM execution condition. - Support SSB and CSI-RS measurement for intra-CU conditional LTM. - Support RACH based and RACH-less conditional intra-CU LTM |
| Nokia [4] | In Rel-19, mobility enhancements in RAN1 could be categorized in the following main items: - (gNB scheduled) CSI-RS based L1-RSRP measuremnent and reorting - Event triggered L1-RSRP measurement reporting - CSI-RS based LTM TCI states (for early TCI state activation and TCI state indication in the cell switch command) - Early CSI Acquisiiton of LTM candidate cells Based on the RAN1 agreements reached so far for the above LTM features, a skeleton of UE FGs structure is proposed in Table 1. Index; Feature group; Components; Note. LTM-1; CSI-RS based L1 measurement and reports for LTM; - Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) - Support of inter-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) - Note inter-frequency measurements may be considered as a separate FG - Support of periodic reporting on PUCCH, semi-persistent reporting on PUCCH/PUSCH, and aperiodic report on PUSCH - Support of beam reporting of L cells with M beams from each of the L cells, - M = {1,2,3,4}, and L = {1,2,3,4}, - MxL = {1,2,3,4,6,8,9,12,16} - Support of intra-frequency and inter-frequency L1- RSRP measurement and reporting based on semi-persistent CSI-RS(s) of candidate cell(s) - Note this may be considered as a separate FG - Support of always inclusion of M L1-RSRP measurements of CSI-RS(s) from current SpCell in the measurement report - Note this may be considered as a separate FG; Agreement - Support L1-RSRP measurement based on CSI-RS - FFS: Support L1-SINR measurement based on CSI-RS Agreement - CSI-RS based L1-RSRP report is supported for gNB scheduled measurement reporting - FFS: CSI-RS based L1-SINR report is supported for gNB scheduled measurement reporting - Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting Agreement For gNB scheduled reporting and event triggered reporting - At least periodic CSI-RS is supported for L1-RSRP measurement for candidate cell - FFS: aperiodic and semi-persistent CSI-RS - At least CSI-RS for beam management is supported for L1-RSRP measurement for candidate cell - FFS: CSI-RS for mobility Agreement The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows: - UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI. - Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18) - The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used - No L1 specified filtering for time and spatial domain is introduced - No enhancement on how to report L cells x M beams - Periodic reporting on PUCCH is supported - FFS: semi-persistent reporting on PUCCH/PUSCH, and aperiodic reporting on PUSCH Agreement From RAN1 perspective, there is no restriction with regards to the frequency location of CSI-RS used for L1-measurement. Agreement For CSI-RS based L1-measurement report by gNB scheduled measurement reporting, semi-persistent reporting on PUCCH/PUSCH and aperiodic reporting on PUSCH are supported. Conclusion There is no consensus in RAN1 on the support L1-SINR measurement based on CSI-RS for candidate cells Agreement - Confirm the following working assumption made in RAN1#118bis Working Assumption(RAN1#118bis) In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 Send an LS to RAN2 to inform this agreement. Final LS in R1-2501500 Conclusion There is no consensus on the support of aperiodic CSI-RS resource for gNB scheduled reporting or event triggered reporting. LTM-2; CSI-RS based LTM TCI state activation and indication; - Support of CSI-RS as QCL source RS with QCL-TypeD in the LTM TCI state configuration when TRS is configured as the QCL source RS with QCL-TypeA; Agreement CSI-RS for BM as the referenceSignal with QCL-TypeD is supported for an LTM TCI state, where TRS is configured as referenceSignal with QCL-TypeA.. LTM-3; Event triggered L1 measurement reporting; - Support of intra-frequency L1- RSRP measurement and event triggered reporting based on SSB(s) of candidate cell(s) - Support of inter-frequency L1- RSRP measurement and event triggered reporting based on SSB(s) of candidate cell(s) - Note inter-frequency measurements may be considered as a separate FG - Support of intra-frequency L1- RSRP measurement and event triggered reporting based on periodic CSI-RS(s) of candidate cell(s) - Support of inter-frequency L1- RSRP measurement and event triggered reporting based on periodic CSI-RS(s) of candidate cell(s) - Note inter-frequency measurements may be considered as a separate FG - Support of determining serving cell RS for event evaluation is based on the type of the RS(s) for candidate cell(s); Agreement - SSB based L1-RSRP measurements is supported for event triggered reporting - CSI-RS based L1-RSRP measurements is supported for event triggered reporting - FFS: CSI-RS based L1-SINR measurements is supported for event triggered reporting Agreement The serving cell RS for event evaluation is at least derived from QCL RS or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell - QCL RS above is the RS w.r.t. QCL-TypeD when the indicated joint/DL TCI state is configured with two QCL RSs - FFS: Details on determination of QCL RS or SSB QCLed with QCL RS Note: This does not imply the support of mTRP scenarios Agreement For the identification of the serving cell RS for event evaluation, If the RS(s) for candidate cell(s) are CSI-RS configured in a CSI-RS resource set configured with repetition, QCL RS of the indicated TCI-state is used for the serving cell; otherwise, SSB QCLed with QCL RS of the indicated TCI-state is used for the serving cell. - UE does not expect the following configuration: - CSI-RS resource in the indicated TCI state of serving cell is NOT configured in a CSI-RS resource set configured with repetition, and - CSI-RS is configured as measurement resource for the candidate cell(s). Conclusion There is no RAN1 consensus to support SP CSI-RS for event triggered reporting Conclusion There is no consensus on the support of aperiodic CSI-RS resource for gNB scheduled reporting or event triggered reporting. LTM-4; CSI Acquisition of candidate cell(s); - Support of CSI acquisition based on periodic CSI-RS(s) of target cell after the reception of the cell switch command - Support of reporting CSI to the target cell after the cell switch - Support of CSI acquisition of candidate cell(s) before the reception of cell switch command - Note: one component of this FG should be maximum number of cells supported for CSI measurements - Note: this may be considered as a separate FG; Agreement The following alternatives are further studied: - Alt-1: CSI-RS measurement and CSI reporting operations are performed before reception of LTM Cell Switch Command (CSC) MAC CE. - The report is sent to the serving cell and transferred to the candidate/target cell(s) - Alt-2: CSI-RS measurement can start before reception of LTM CSC MAC CE and CSI reporting operation is performed after reception of LTM CSC MAC CE. - The report is sent directly to target cell - Alt-3: CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE. - The report is sent directly to target cell Agreement For target cell CSI acquisition, - A UE is provided with RRC configurations for periodic CSI-RS resource(s) and CSI report(s) for one or more candidate cell - For a candidate cell, - down-select from the following alternatives: - Alt 1: A single CSI report configuration is configured - Alt 2: Multiple CSI report configurations can be configured - down-select from the following alternatives: - Alt X: A single CSI-RS resource for CMR is associated with a CSI report configuration - Alt Y: Multiple CSI-RS resources for CMR can be associated with a CSI report configuration - FFS: Semi-persistent CSI-RS resource - After the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability - FFS: whether or how to select a subset of CSI-RS resources to measure - FFS: when the UE may start measuring the configured CSI-RS resources - UE determines the CSI report configuration based on the CSC - After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration - The latest available measured CSI on target cell resource(s) is conveyed at least by a single report, and the report is sent to the target cell - Option 1: to use UCI - Option 2: to use MAC CE Note: with this agreement, the working assumption made in RAN1#119 is automatically confirmed.. |
| CMCC [5] | In mobility, at least three feature groups should be supported, including gNB scheduled CSI-RS based LTM beam reporting, Event triggered reporting for LTM, CSI acquisition for LTM. In addition, based on the agreements achieved, support of semi-persistent CSI-RS and CSI-RS measurement based on the configured CSI-RS resource(s) after the RRC configuration and before the reception of CSC as optional UE capabilities. Based on the above analyses, we suggest taking the following UE FGs for mobility. Proposal 1: Take the following UE FGs for the feature of mobility: Features; Index; Feature group; Components. 63. NR_Mob_Ph4-Core; 63-1-1; Intra-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure based on periodic CSI-RS(s); 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement 4. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 5. Maximum number of LTM CSI report configs. 63. NR_Mob_Ph4-Core; 63-1-1a; Inter-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure based on periodic CSI-RS(s); 1. Support of inter-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra- and inter-frequency L1-RSRP measurement 4. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra- and inter-frequency L1-RSRP measurement. 63. NR_Mob_Ph4-Core; 63-1-1b; Intra-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure based on semi-persistent CSI-RS(s); 1. Support of intra-frequency L1- RSRP measurement and reporting based on semi-persistent CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement 4. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 5. Maximum number of LTM CSI report configs. 63. NR_Mob_Ph4-Core; 63-1-1c; Inter-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure based on semi-persistent CSI-RS(s); 1. Support of inter-frequency L1- RSRP measurement and reporting based on semi-persistent CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra- and inter-frequency L1-RSRP measurement 4. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra- and inter-frequency L1-RSRP measurement. 63. NR_Mob_Ph4-Core; 63-1-2; Event triggered reporting for L1-L2 Triggered Mobility (LTM); 1. Support of event triggered reporting for LTM based on periodic CSI-RS(s) and SSB(s) 2. Support of LTM2, LTM3, LTM4, LTM5 based measurement and report 3. Support of MAC CE based event triggered beam report 4. Maximum number of the RRC configured candidate cells and beams 5. Support of serving cell beam measurement for event evaluation by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively. 63. NR_Mob_Ph4-Core; 63-1-3; CSI acquisition of candidate cell(s) for L1-L2 Triggered Mobility (LTM); 1. Support of CSI acquisition of candidate cell(s) for LTM 2. Maximum number of the RRC configured candidate cells and CSI-RS resources 3. Maximum number of CSI report configs. 63. NR_Mob_Ph4-Core; 63-1-3a; Measure CSI based on the configured CSI-RS resource(s) after the RRC configuration and before the reception of CSC; 1. Support of measure CSI based on the configured CSI-RS resource(s) after the RRC configuration and before the reception of CSC. |
| Huawei/HiSilicon [6] | According to the following agreements reached so far on NW triggered L1 measurement for beam management, an individual FG for NW triggered L1-RSRP measurement and reporting based on periodic NZP CSI-RS resource(s) of candidate cell(s) should be specified. Considering whether to support inter-frequency measurement is still under discussion in RAN4, at least this FG should be applied to intra-frequency measurement from RAN1 perspective. Before measuring the CSI-RS(s) in a candidate cell, UE usually measures the SSB in the corresponding cells. Thus, we think FG45-1 should be prerequisite of this FG. Considering the similar report mechanism between R18 LTM and R19 LTM, the components and the corresponding value ranges of FG45-1 can be inherited in this new FG. In addition, a separate FG to allow configuring the L1-RSRP(s) of serving cell is always included can be added, similar as FG45-2. Agreement - CSI-RS based L1-RSRP report is supported for gNB scheduled measurement reporting - FFS: CSI-RS based L1-SINR report is supported for gNB scheduled measurement reporting - Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting Agreement The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows: - UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI. - Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18) - The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used - No L1 specified filtering for time and spatial domain is introduced - No enhancement on how to report L cells x M beams - Periodic reporting on PUCCH is supported - FFS: semi-persistent reporting on PUCCH/PUSCH, and aperiodic reporting on PUSCH Agreement For CSI-RS based L1-measurement report by gNB scheduled measurement reporting, semi-persistent reporting on PUCCH/PUSCH and aperiodic reporting on PUSCH are supported. Proposal 1: Add a new FG (FG X-1) for NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s). FG45-1 should be prerequisite of FGX-1. The components and the corresponding value ranges of FG45-1 can be inherited. Proposal 2: Add a new FG (FG X-2) for inclusion of current SpCell in the L1 measurement report based on CSI-RS (s). In addition to periodic CSI-RS, semi-persistent CSI-RS is supported subject to UE capability according to the following agreement. Thus, a separate FG should be added to let UE report the capability. Agreement - Confirm the following working assumption made in RAN1#118bis Working Assumption(RAN1#118bis) In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 Send an LS to RAN2 to inform about this agreement.. Proposal 3: Add a new FG (FG X-3) to support the NW triggered intra-frequency L1- RSRP measurement and reporting based on semi-persistent CSI-RS(s). FGX-1 should be the prerequisite. According to the following agreement and the working assumption, at least CSI-RS measurement and CSI reporting for the target cell based on periodic CSI-RS(s) can be added as a separate FG (FG X-4). The maximum number of configurable candidate cell(s) for CSI acquisition should be reported. As the reporting content has not confirmed, the prerequisite of this new FG can be further discussed. For a UE with the capability to measure CSI of candidate cell(s) before LTM cell switch, a separate FG (FG X-5) should be introduced. As UE is required to measure CSI before cell switch, the maximum number of candidate cell(s) on which UE can perform CSI measurement before LTM Cell Switch command should be reported. The capability of CSI measurement for serving cell and candidate cell should be counted together. Agreement For target cell CSI acquisition, - A UE is provided with RRC configurations for periodic CSI-RS resource(s) and CSI report(s) for one or more candidate cell - For a candidate cell, - down-select from the following alternatives: - Alt 1: A single CSI report configuration is configured - Alt 2: Multiple CSI report configurations can be configured - down-select from the following alternatives: - Alt X: A single CSI-RS resource for CMR is associated with a CSI report configuration - Alt Y: Multiple CSI-RS resources for CMR can be associated with a CSI report configuration - FFS: Semi-persistent CSI-RS resource - After the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability - FFS: whether or how to select a subset of CSI-RS resources to measure - FFS: when the UE may start measuring the configured CSI-RS resources - UE determines the CSI report configuration based on the CSC - After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration - The latest available measured CSI on target cell resource(s) is conveyed at least by a single report, and the report is sent to the target cell - Option 1: to use UCI - Option 2: to use MAC CE Note: with this agreement, the working assumption made in RAN1#119 is automatically confirmed. Working Assumption As baseline, CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE. - The report is sent directly to target cell - Introduce UE capability for CSI-RS measurement can start before reception of LTM CSC MAC CE - Other than UE capability, strive for no additional spec impact compared to the baseline (only one triggering mechanism will be specified). Proposal 4: The following FGs could be added for Rel-19 LTM, - FG X-4: Support of intra-frequency CSI acquisition for LTM. - FG X-5: Support of intra-frequency CSI acquisition before LTM CSC MAC CE for LTM, FGX-4 as prerequisite. Proposal 5: For X-4, support the following components: - Support of intra-frequency CSI measurement based on periodic CSI-RS(s) of candidate cell(s) after LTM CSC MAC CE - Support of CSI reporting to target cell - The maximum number of candidate cell(s) UE can be configured to measure CSI Proposal 6: For X-5, support the following components: - Support of intra-frequency CSI measurement based on periodic CSI-RS(s) of candidate cell(s) before LTM CSC MAC CE - Maximum number of candidate cell(s) on which UE can perform intra-frequency CSI acquisition before LTM CSC MAC CE For LTM event triggered reporting, we prefer UE features are be discussed in RAN2 where most of agreements are achieved. Proposal 7: UE features of LTM event triggered reporting can be discussed in RAN2. For the granularity of UE features, there was a long debate between per band and per BC in R18 LTM. We propose to follow R18 LTM where all L1 measurement and reporting related UE capabilities are reported per band combination to allow sharing the capability of CA. Proposal 8: The reporting granularity of all L1 measurement and measurement related FG in R19 LTM should be per band combination. Based on the discussion above, we propose RAN1 to adopt the Table 1 as UE features for R19 LTM. Proposal 9: Adopt Table in Appendix as UE features for R19 LTM NR_Mob_enh2; X-1; NW triggered Intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; - Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) - Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement - Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement - Maximum number of LTM CSI report configs; 45-1; Yes; No; UE does not support intra-frequency L1-RSRP measurement for candidate cell (s) based on CSI-RS (s) in Rel-19 LTM operations; Per BC; No; No; n/a; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16} Component 4 candidate values: Aperiodic: {0,1,2,3,4} Periodic: {1,2,3,4} Semi-persistent: {0,1,2,3,4}; Optional with capability signalling. 45. NR_Mob_enh2; X-2; Inclusion of current SpCell in the L1 measurement report based on CSI-RS (s); 1. Support of always including the current SpCell in the L1 measurement report based on CSI-RS (s); X-1 or X-3; Yes; No; UE does not always include measurement report for SpCell in the L1 measurement report based on CSI-RS (s); Per BC; No; No; n/a; Optional with capability signalling. NR_Mob_enh2; X-3; Intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; - Support of intra-frequency L1- RSRP measurement and reporting based on semi-persistent CSI-RS(s) of candidate cell(s); X-1; Yes; No; UE does not support intra-frequency L1-RSRP measurement for candidate cell (s) based on semi-persistent CSI-RS (s) in Rel-19 LTM operations; Per BC; No; No; n/a; Optional with capability signalling. NR_Mob_enh2; X-4; Intra-frequency CSI acquisition for L1-L2 Triggered Mobility (LTM) procedure; - Support of intra-frequency CSI measurement based on periodic CSI-RS(s) of candidate cell(s) after LTM CSC MAC CE - Support of CSI reporting to target cell - Maximum number of RRC configured candidate cells for intra-frequency CSI acquisition; Yes; No; UE does not support intra-frequency CSI acquisition for candidate cell (s) in Rel-19 LTM operations; Per BC; No; No; n/a; Component 3 candidate values: [TBD]; Optional with capability signalling. NR_Mob_enh2; X-5; Intra-frequency CSI acquisition before LTM CSC MAC CE for L1-L2 Triggered Mobility (LTM) procedure; - Support of intra-frequency CSI measurement based on periodic CSI-RS(s) of candidate cell(s) before LTM CSC MAC CE - Maximum number of candidate cell(s) on which UE can perform intra-frequency CSI measurement before LTM CSC MAC CE; X-4; Yes; No; UE does not support intra-frequency CSI acquisition before LTM CSC MAC CE for candidate cell (s) in Rel-19 LTM operations; Per BC; No; No; n/a; Component 2 candidate values: [TBD]; Optional with capability signalling. |
| OPPO [7] | UE features for CSI-RS based L1 measurement In Rel-19, L1-RSRP measurement based on CSI-RS is supported. Periodic reporting on PUCCH, semi-persistent reporting on PUCCH/PUSCH and aperiodic reporting in PUSCH are supported. Both periodic and semi-persistent CSI-RS resource are supported: Agreement - Support L1-RSRP measurement based on CSI-RS - FFS: Support L1-SINR measurement based on CSI-RS Agreement The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows: - UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI. - Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18) - The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used - No L1 specified filtering for time and spatial domain is introduced - No enhancement on how to report L cells x M beams - Periodic reporting on PUCCH is supported Agreement For CSI-RS based L1-measurement report by gNB scheduled measurement reporting, semi-persistent reporting on PUCCH/PUSCH and aperiodic reporting on PUSCH are supported Agreement - Confirm the following working assumption made in RAN1#118bis Working Assumption(RAN1#118bis) In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 Send an LS to RAN2 to inform this agreement. Final LS in R1-2501500.. First of all, L1-RSRP based on CSI-RS of candidate cell shall be a UE capability. Supporting periodic reporting, semi-persistent reporting and aperiodic reporting shall be separate UE capability. For each type of reporting, the UE capability shall include the maximum number of candidate cells, maximum number of CSI-RS resources of each candidate cell and maximum number of CSI-RS resources across all the candidate cell. Then for each type of L1-RSRP reporting based on CSI-RS of candidate cell, the maximal number of CSI-RS resource per candidate cell and the maximal number of CSI-RS resources across all the candidate cells shall be UE capability. Maximum number of reported L1-RSRP measurement per candidate cell and maximum number of reported L1-RSRP of candidate cells are UE capability. Proposal 1: Support the following UE features for Rel-19 CSI-RS based L1-RSRP measurement for LTM: - Supporting L1-RSRP measurement on CSI-RS resource of candidate cell is UE capability; - Supporting periodic reporting, semi-persistent reporting and aperiodic reporting are separate UE capability; - For L1-RSRP measurement, the UE capability includes the maximal number of CSI-RS resources per candidate cell, maximal number of candidate cells and maximal number of CSI-RS resources across all candidate cells; - For L1-RSRP measurement reporting, the UE capability shall include the maximal number of reported CRI of one candidate cell, the maximal number of reported candidate cell. UE features for CSI acquisition on candidate cell For the function of CSI acquisition on candidate cell, it is supported that the UE can start measuring the CSI-RS before or after the LTM cell switch MAC CE. And it was agreed that stating measurement before the LTM cell switch MAC CE is subject to UE capability. Agreement For target cell CSI acquisition, - A UE is provided with RRC configurations for periodic CSI-RS resource(s) and CSI report(s) for one or more candidate cell - For a candidate cell, - down-select from the following alternatives: - Alt 1: A single CSI report configuration is configured - Alt 2: Multiple CSI report configurations can be configured - down-select from the following alternatives: - Alt X: A single CSI-RS resource for CMR is associated with a CSI report configuration - Alt Y: Multiple CSI-RS resources for CMR can be associated with a CSI report configuration - FFS: Semi-persistent CSI-RS resource - After the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability - FFS: whether or how to select a subset of CSI-RS resources to measure - FFS: when the UE may start measuring the configured CSI-RS resources - UE determines the CSI report configuration based on the CSC - After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration - The latest available measured CSI on target cell resource(s) is conveyed at least by a single report, and the report is sent to the target cell - Option 1: to use UCI - Option 2: to use MAC CE Note: with this agreement, the working assumption made in RAN1#119 is automatically confirmed.. Supporting CSI acquisition of candidate cell in LTM shall be one UE capability. As agreed, measuring CSI of candidate cell before LTM CSC shall be one UE capability. The UE capability shall include: maximal number of candidate cells that can be configured for CSI acquisition of candidate cell. Proposal 2: Support the following UE features for Rel-19 target cell CSI acquisition: - Measure CSI of candidate cell before LTM Cell switch command is a separate UE capability; - The maximal number of candidate cells for CSI acquisition; UE features for LTM event trigger L1 measurement reporting For LTM event trigger L1 measurement reporting, a few LTM events are supported and RAN1 has made the following agreement on L1-RSRP measurement for LTM event trigger and the methods of how to determine the beam RS for the serving cell. Agreement - SSB based L1-RSRP measurements is supported for event triggered reporting - CSI-RS based L1-RSRP measurements is supported for event triggered reporting - FFS: CSI-RS based L1-SINR measurements is supported for event triggered reporting Agreement The serving cell RS for event evaluation is at least derived from QCL RS or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell - QCL RS above is the RS w.r.t. QCL-TypeD when the indicated joint/DL TCI state is configured with two QCL RSs - FFS: Details on determination of QCL RS or SSB QCLed with QCL RS Note: This does not imply the support of mTRP scenarios. First of all, supporting each LTM event shall be a separate UE capability. Proposal 3: For Rel-19 LTM event trigger L1 measurement reporting, support the following UE capability: - Support Event LTM 2 is UE capability; - Support Event LTM 3 is UE capability; - Support Event LTM 4 is UE capability; - Support Event LTM 5 is UE capability; As agreed, either SSB or CSI-RS can be used for L1-RSRP measurement for event triggered reporting. Supporting either of them shall be UE capability. Proposal 4: Supporting SSB based L1-RSRP measurement for LTM event triggered reporting is UE capability; Proposal 5: Supporting CSI-RS based L1-RSRP measurement for LTM event triggered reporting is UE capability; For LTM event trigger reporting, the UE is requested to measure SSBs or CSI-RS resources of one or more candidate cells. It is subject to UE capability of how many RSs the UE is able to measure. Proposal 6: For Rel-19 LTM event trigger L1 measurement reporting, support the following UE capability: - Maximal number of SSBs per candidate cell for L1-RSRP measurement; - Maximal number of CSI-RS per candidate cell for L1-RSRP measurement; - Maximal number of SSBs of all candidate cells for L1-RSRP measurement; - Maximal number of CSI-RS of all candidate cells for L1-RSRP measurement; - Maximal number of candidate cells for LTM event trigger reporting; For Event LTM 3 and 5, the UE can report the L1-RSRP measurement of some SSBs or CSI-RS resources of candidate cells. The maximal number of L1-RSRP measurements of one candidate cell and the maximal number of candidate cell included in the beam reporting shall be subject to the UE capability. Proposal 7: For Rel-19 LTM event trigger L1 measurement reporting, support the following UE capability: - Maximal number of L1-RSRP reports of one candidate cell in one beam report; - Maximal number of candidate cells reported in one beam report; - Maximal number of L1-RSRP reports of all candidate cell in one beam report; |
| Samsung [8] | In the following, we provide a list of UE capabilities that need to be supported for Rel-19 LTM. In particular, our views on the UE capabilities are based on at least the following agreements made in the agenda item 9.9.1 of Rel-19 LTM. Agreement - Support L1-RSRP measurement based on CSI-RS - FFS: Support L1-SINR measurement based on CSI-RS Agreement - Explicit configuration of CSI-RS resource(s) for candidate cell(s) for L1-measurement is supported Agreement - CSI-RS based L1-RSRP report is supported for gNB scheduled measurement reporting - FFS: CSI-RS based L1-SINR report is supported for gNB scheduled measurement reporting - Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting Agreement - SSB based L1-RSRP measurements is supported for event triggered reporting - CSI-RS based L1-RSRP measurements is supported for event triggered reporting - FFS: CSI-RS based L1-SINR measurements is supported for event triggered reporting Agreement For gNB scheduled reporting and event triggered reporting - At least periodic CSI-RS is supported for L1-RSRP measurement for candidate cell - FFS: aperiodic and semi-persistent CSI-RS - At least CSI-RS for beam management is supported for L1-RSRP measurement for candidate cell - FFS: CSI-RS for mobility Agreement - For the identification of the serving cell RS for event evaluation, - At least the following options are further studied in RAN1, where different options could apply to different LTM event - Option. 1: Derived from QCL (type-D) RS(s) of the indicated joint/DL TCI state for the serving cell - Option. 2: Derived from QCL RS(s) or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell - QCL RS or SSB is configured by the network - Option. 3: Measurement RS(s) is/are explicitly configured - Option. 4: Derived from QCL RSs of activated TCI states with the best quality, or SSB which is QCLed with the QCL RSs of activated TCI states with the best quality. - Option 6: Derived from QCL RSs of activated TCI states, or SSB which is QCLed with the QCL RSs of activated TCI states - The RSs of the candidate cell(s) for event evaluation are explicitly configure - Note: Companies are encouraged to take into account the RAN2 agreement (i.e current beam rather than best beam) for their further study. Agreement The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows: - UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI. - Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18) - The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used - No L1 specified filtering for time and spatial domain is introduced - No enhancement on how to report L cells x M beams - Periodic reporting on PUCCH is supported - FFS: semi-persistent reporting on PUCCH/PUSCH, and aperiodic reporting on PUSCH Agreement For CSI-RS based L1-measurement report by gNB scheduled measurement reporting, semi-persistent reporting on PUCCH/PUSCH and aperiodic reporting on PUSCH are supported Agreement The serving cell RS for event evaluation is at least derived from QCL RS or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell - QCL RS above is the RS w.r.t. QCL-TypeD when the indicated joint/DL TCI state is configured with two QCL RSs - FFS: Details on determination of QCL RS or SSB QCLed with QCL RS Note: This does not imply the support of mTRP scenarios Agreement For the identification of the serving cell RS for event evaluation, If the RS(s) for candidate cell(s) are CSI-RS configured in a CSI-RS resource set configured with repetition, QCL RS of the indicated TCI-state is used for the serving cell; otherwise, SSB QCLed with QCL RS of the indicated TCI-state is used for the serving cell. - UE does not expect the following configuration: - CSI-RS resource in the indicated TCI state of serving cell is NOT configured in a CSI-RS resource set configured with repetition, and - CSI-RS is configured as measurement resource for the candidate cell(s). Agreement CSI-RS for BM as the referenceSignal with QCL-TypeD is supported for an LTM TCI state, where TRS is configured as referenceSignal with QCL-TypeA Agreement - Confirm the following working assumption made in RAN1#118bis Working Assumption(RAN1#118bis) In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective - Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 - Send an LS to RAN2 to inform this agreement. Final LS in R1-2501500. Agreement An LTM report configuration for L1-RSRP is associated with a single resource config that includes: - Alt.A: a single resource set containing CSI-RS resources corresponding to multiple candidate cells. i.e., the same design as that of SSB in Rel-18 LTM. - FFS: how to associate between the measurement CSI-RS resources and candidate cells - explicit or implicit signaling of candidate cells Agreement For target cell CSI acquisition, - A UE is provided with RRC configurations for periodic CSI-RS resource(s) and CSI report(s) for one or more candidate cell - For a candidate cell, - down-select from the following alternatives: - Alt 1: A single CSI report configuration is configured - Alt 2: Multiple CSI report configurations can be configured - down-select from the following alternatives: - Alt X: A single CSI-RS resource for CMR is associated with a CSI report configuration - Alt Y: Multiple CSI-RS resources for CMR can be associated with a CSI report configuration - FFS: Semi-persistent CSI-RS resource - After the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability - FFS: whether or how to select a subset of CSI-RS resources to measure - FFS: when the UE may start measuring the configured CSI-RS resources - UE determines the CSI report configuration based on the CSC - After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration - The latest available measured CSI on target cell resource(s) is conveyed at least by a single report, and the report is sent to the target cell - Option 1: to use UCI - Option 2: to use MAC CE Note: with this agreement, the working assumption made in RAN1#119 is automatically confirmed.. Proposal 1. Support the following list of UE capabilities for Rel-19 LTM. Index; Feature group; Components; Prerequisite FGs; Consequence if the feature is not supported by the UE; Type; Need of FR1/FR2 differentiation; Note; Mandatory /Optional. X-1-1; CSI-RS based gNB scheduled measurement/reporting for LTM; 1. Support periodic CSI-RS based L1-RSRP measurement and reporting 2. Support of a single resource set comprising CSI-RS resource(s) for candidate cell(s) 3. Support of periodic, semi-persistent and aperiodic reporting; CSI-RS based gNB scheduled measurement/reporting for Rel-19 LTM is not supported; Per band; N/A; Component 1: the periodic CSI-RS is configured in an NZP CSI-RS resource set configured with repetition; Optional with capability signalling. X-1-2; Semi-persistent CSI-RS based gNB scheduled measurement/reporting for LTM; 1. Support of semi-persistent CSI-RS based L1-RSRP measurement and reporting; X-1-1; Semi-persistent CSI-RS based gNB scheduled measurement/reporting for Rel-19 LTM is not supported; Per band; N/A; Component 1: the periodic CSI-RS is configured in an NZP CSI-RS resource set configured with repetition; Optional with capability signalling. X-1-3; Event-triggered measurement/reporting for LTM; 1. Support of SSB and periodic CSI-RS based L1-RSRP measurement 2. Support of serving cell RS by using QCL RS in the indicated TCI state or the corresponding QCL SSB; Event-triggered measurement for Rel-19 LTM is not supported; Per band; N/A; Component 1: the periodic CSI-RS is configured in an NZP CSI-RS resource set configured with repetition; Optional with capability signalling. X-1-4; CSI acquisition for Rel-19 LTM; 1. Support of periodic CSI-RS resource(s) 2. Support of CSI-RS measurement after CSC 3. Support of CSI reporting after CSC; CSI acquisition for Rel-19 LTM is not supported; Per band; N/A; Optional with capability signaling. X-1-5; CSI-RS measurement before CSC for CSI acquisition; 1. Support of maximum number of candidate cell(s); X-1-4; CSI-RS measurement before CSC for CSI acquisition for Rel-19 LTM is not supported; Per band; N/A; Component 1: candidate values of {1, …, N}; Optional with capability signaling. |
| Xiaomi [9] | 59. NR_Mobility_Ph4; 59-x-1; CSI-RS for gNB scheduled LTM CSI reporting; 1.Support of L1-RSRP measurement based on periodic CSI-RS resource 2.Maximum number of CSI-RS resource in CSI-RS resource set 3.Support of semi-persistent reporting on PUCCH/PUSCH and aperiodic reporting on PUSCH; FFS; yes; n/a; CSI-RS for gNB scheduled LTM CSI reporting is not supported; FFS; FFS; FFS; FFS; [Component 2 candidate values: {1, 2, … , x}]; Optional with capability signalling. 59. NR_Mobility_Ph4; 59-x-2; Semi-persistent CSI-RS for gNB scheduled LTM CSI reporting; 1.Support of L1-RSRP measurement based on semi-persistent CSI-RS resource 2.Maximum number of CSI-RS resource in CSI-RS resource set 3.Support of semi-persistent reporting on PUCCH/PUSCH and aperiodic reporting on PUSCH; FFS; yes; n/a; Semi-persistent CSI-RS for gNB scheduled LTM CSI reporting is not supported; FFS; FFS; FFS; FFS; [Component 2 candidate values: {1, 2, … , x}]; Optional with capability signalling. 59. NR_Mobility_Ph4; 59-x-3; Event-triggered LTM CSI reporting; 1.Support of L1-RSRP measurement based on SSB 2.Support of L1-RSRP measurement based on periodic CSI-RS resource 3.Support of serving cell RS derived from QCL Type D RS in the indicated TCI state and the corresponding QCL SSB for different RS type for candidate cell(s), respectively 4.Maximum number of CSI-RS resource in CSI-RS resource set 5.Maximum number of SSB in LTM-CSI-SSB-ResourceSet 6.Maximum number of reported RSs 7.Support of reporting serving cell RS enabled by RRC; FFS; yes; n/a; Event-triggered LTM CSI reporting is not supported; FFS; FFS; FFS; FFS; [Component 4 candidate values: {1, 2, … , x}] [Component 5 candidate values: {1, 2, … , x}] [Component 6 candidate values: {1, 2, … , x}]; Optional with capability signalling. 59. NR_Mobility_Ph4; 59-x-4; CSI acquisition for target cell; 1.Support of CSI measurement on CSI-RS resource of target cell after LTM CSC MAC CE. 2.Support of reporting CSI report of target cell to target cell; FFS; yes; n/a; CSI acquisition for target cell is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. 59. NR_Mobility_Ph4; 59-x-5; CSI measurement for candidate cell before LTM CSC MAC CE; 1.Maximum number of candidate cells for CSI measurement 2.Maximum number of CSI-RS resources of candidate cell(s) for CSI measurement; FFS; yes; n/a; CSI measurement for candidate cell before LTM CSC MAC CE is not supported; FFS; FFS; FFS; FFS; [Component 1 candidate values: {1, 2, … , x}] [Component 2 candidate values: {1, 2, … , x}]; Optional with capability signalling. |
| Ericsson [10] | For beam-management based on CSI-RS we should introduce feature-groups analogous to the corresponding Release 18 feature groups for SSB [2]. For Rel-19 Mobility, introduce FGs for intra- resp. inter-frequency L1 measurements measurement and reports based on periodic CSI-RS for LTM. Example adaptation of the Rel. 18 feature groups for SSBs are shown below: - FG 63-1: Intra-frequency L1 measurements and reports based on periodic CSI-RS for L1/L2 Triggered Mobility (LTM) procedure - Components: - Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) - Support of up to L candidate cells and M beams per cell in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement - Prerequisite feature groups: The corresponding feature group for SSB, FG 45-1 - FG 63-1a: Inter-frequency L1 measurements and reports based on periodic CSI-RS for L1/L2 Triggered Mobility (LTM) procedure - Components - Support of inter- frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) - Support of up to L candidate cells and M beams per cell in one report where a CRI-RSRP pair is used for each beam report for intra- and inter-frequency L1-RSRP measurement - Prerequisite feature groups: The corresponding feature group for SSB, FG 45-1a and 63-1. According to RAN1 agreement, support for semi-persistent CSI-RS is a separate capability. We propose to introduce an FG accordingly: For Rel-19 Mobility, introduce an FG for L1 measurements and reports based semi-persistent CSI-RS. As an example, the feature-group can have the following description and components: - FG 62-2: L1 measurements and reports based on semi-persistent CSI-RS for L1/L2 Triggered Mobility (LTM) procedure - Components - Support for L1-RSRP measurement and reporting based on semi-persistent CSI-RS(s) of candidate cell(s) - Prerequisite feature groups: FG 63-1 or FG 63-1a CSI measurement and reporting after LTM Cell Switch Command MAC CE should be another feature-group: For Rel-19 Mobility introduce an FG for CSI measurement and report for Target Cell after LTM Cell Switch Command MAC CE An example is shown below: - FG 63-3: CSI measurement and reporting for LTM Target Cell after reception of LTM Cell Switch Command MAC CE - Components - Support measurement of CSI-RS(s) and reporting CSI to the target cell. - Maximum number of CSI-RS resources for CMR per cell According to RAN1 agreement, CSI measurements before LTM Cell Switch Command MAC CE is a separate capability: For Rel-19 Mobility introduce an FG for CSI measurement for LTM Candidate Cell before LTM Cell Switch Command MAC CE Example: - FG 63-4: CSI measurement on LTM Candidate Cell after RRC configuration and before reception of LTM Cell Switch Command MAC CE - Components - Support measurement of CSI-RS(s) on LTM Candidate Cell(s). - Prerequisite feature groups: FG63-3 There may be additional components depending on what RAN1 decides regarding whether/how to select a subset of CSI-RS resources to measure and when the UE may start measurement operations. For event-driven reporting for LTM, it is not clear to us if there is a need for a RAN1 feature. As RAN2 has specified most of the critical functionality for event-driven reporting, it would be up to RAN2 to define the corresponding features: RAN2 defines the UE features for event-driven reporting for LTM. |
| Apple [11] | 63. NR_Mob_Ph4; 63-1; Intra-frequency L1 measurement and reports based on CSI-RS for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of intra-frequency L1- RSRP measurement and reporting based on CSI-RS resource(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resouce 4. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 5. Maximum number of LTM CSI report configs using CSI-RS as measurement resouce; 2-21 or 2-22 or 2-23 or 2-23a; Yes; No; UE does not support intra-frequency L1 measurement and reports based on CSI-RS for Rel-19 LTM operation; Per BC; No; No; n/a; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M x L: {1,2,3,4, 6, 8, 9, 12, 16} Component 5 candidate values: Aperiodic: {0,1,2,3,4} Periodic: {1,2,3,4} Semi-persistent: {0,1,2,3,4}; Optional with capability signalling. 63. NR_Mob_Ph4; 63-2; Inter-frequency L1 measurement and reports based on CSI-RS for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of inter- frequency L1- RSRP measurement and reporting based on CSI-RS resource(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra- and inter-frequency L1-RSRP measurement on CSI-RS resouce 4. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra- and inter-frequency L1-RSRP measurement; 2-21 or 2-22 or 2-23 or 2-23a, 63-1; Yes; No; UE does not support inter-frequency L1 measurement and reports based on CSI-RS for Rel-19 LTM operation; Per BC; No; No; n/a; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: L: {1,2,3,4} M: {1,2,3,4} M ´ L: {1,2,3,4, 6, 8, 9, 12, 16}; Optional with capability signalling. |
| NTT DOCOMO, INC. [12] | For NR mobility, the following UE capability related agreements have been reached: Agreement - Support L1-RSRP measurement based on CSI-RS - FFS: Support L1-SINR measurement based on CSI-RS Agreement - CSI-RS based L1-RSRP report is supported for gNB scheduled measurement reporting - FFS: CSI-RS based L1-SINR report is supported for gNB scheduled measurement reporting - Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting Agreement - SSB based L1-RSRP measurements is supported for event triggered reporting - CSI-RS based L1-RSRP measurements is supported for event triggered reporting - FFS: CSI-RS based L1-SINR measurements is supported for event triggered reporting Agreement For gNB scheduled reporting and event triggered reporting - At least periodic CSI-RS is supported for L1-RSRP measurement for candidate cell - FFS: aperiodic and semi-persistent CSI-RS - At least CSI-RS for beam management is supported for L1-RSRP measurement for candidate cell - FFS: CSI-RS for mobility Agreement - Confirm the following working assumption made in RAN1#118bis Working Assumption(RAN1#118bis) In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 Send an LS to RAN2 to inform this agreement. Final LS in R1-2501500. Agreement The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows: - UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI. - Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18) - The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used - No L1 specified filtering for time and spatial domain is introduced - No enhancement on how to report L cells x M beams - Periodic reporting on PUCCH is supported - FFS: semi-persistent reporting on PUCCH/PUSCH, and aperiodic reporting on PUSCH Agreement For CSI-RS based L1-measurement report by gNB scheduled measurement reporting, semi-persistent reporting on PUCCH/PUSCH and aperiodic reporting on PUSCH are supported Agreement For the identification of the serving cell RS for event evaluation, If the RS(s) for candidate cell(s) are CSI-RS configured in a CSI-RS resource set configured with repetition, QCL RS of the indicated TCI-state is used for the serving cell; otherwise, SSB QCLed with QCL RS of the indicated TCI-state is used for the serving cell. - UE does not expect the following configuration: - CSI-RS resource in the indicated TCI state of serving cell is NOT configured in a CSI-RS resource set configured with repetition, and - CSI-RS is configured as measurement resource for the candidate cell(s). Agreement CSI-RS for BM as the referenceSignal with QCL-TypeD is supported for an LTM TCI state, where TRS is configured as referenceSignal with QCL-TypeA Agreement For target cell CSI acquisition, - A UE is provided with RRC configurations for periodic CSI-RS resource(s) and CSI report(s) for one or more candidate cell - For a candidate cell, - down-select from the following alternatives: - Alt 1: A single CSI report configuration is configured - Alt 2: Multiple CSI report configurations can be configured - down-select from the following alternatives: - Alt X: A single CSI-RS resource for CMR is associated with a CSI report configuration - Alt Y: Multiple CSI-RS resources for CMR can be associated with a CSI report configuration - FFS: Semi-persistent CSI-RS resource - After the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability - FFS: whether or how to select a subset of CSI-RS resources to measure - FFS: when the UE may start measuring the configured CSI-RS resources - UE determines the CSI report configuration based on the CSC - After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration - The latest available measured CSI on target cell resource(s) is conveyed at least by a single report, and the report is sent to the target cell - Option 1: to use UCI - Option 2: to use MAC CE Note: with this agreement, the working assumption made in RAN1#119 is automatically confirmed.. Based on the above, we have the following initial thoughts on UE features for NR mobility: - For the support of gNB-scheduled L1-RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s), Event-triggered L1-RSRP measurement and reporting based on RS(s) of candidate cell(s), and CSI measurement and report based on periodic CSI-RS(s) of candidate cell(s), intra-frequency and inter-frequency should be separate FG same as UE feature of Rel-18 LTM. Thus, the following FG should be made for each function. - gNB-scheduled intra-frequency L1-RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) - gNB-scheduled inter-frequency L1-RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) - Event-triggered intra-frequency L1-RSRP measurement and reporting based on RS(s) of candidate cell(s) - Event-triggered inter-frequency L1-RSRP measurement and reporting based on RS(s) of candidate cell(s) - Intra-frequency CSI measurement and report based on periodic CSI-RS(s) of candidate cell(s) (after reception of LTM CSC MAC CE) - Inter-frequency CSI measurement and report based on periodic CSI-RS(s) of candidate cell(s) (after reception of LTM CSC MAC CE) - For the following function, separate FG should be made. - SP-CSI-RS of candidate cell(s) for gNB-scheduled L1-RSRP measurement and reporting - Early CSI measurement based on periodic CSI-RS(s) of candidate cell(s) before reception of LTM CSC MAC CE - For component of the following each FG, - For the FG of gNB-scheduled intra-L1-RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s), - “Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement” should be included. - “Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement” should be included. - For the FG of gNB-scheduled inter-L1-RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s), - “Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra- and inter-frequency L1-RSRP measurement” should be included. - “Maximum number of RRC configured candidate cells for intra- and inter-frequency L1-RSRP measurement” should be included. - For the FG of intra-frequency Event-triggered L1-RSRP measurement and reporting based on RS(s) of candidate cell(s), - “Supported measurement resource RS (e.g., SSB, periodic CSI-RS, both)” should be included. - “Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement” should be included. - For the FG of inter-frequency Event-triggered L1-RSRP measurement and reporting based on RS(s) of candidate cell(s), - “Supported measurement resource RS (e.g., SSB, periodic CSI-RS, both)” should be included. - “Maximum number of RRC configured candidate cells for intra- and inter-frequency L1-RSRP measurement” should be included. |

# Discussion Items during RAN1 #120bis

After review of contributions submitted to RAN1 #120bis in this agenda item, the following topics were identified by the moderator for discussion during RAN1 #120bis.

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |

## Intra- and inter-frequency measurement

After review of contributions submitted to RAN1 #120bis in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 63. NR_Mob_Ph4 | 63-1 | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resouce 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource | FFS | Yes |  | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-2 | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resouce 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource | FFS | Yes |  | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |
| 63. NR_Mob_Ph4 | 63-3 | NW triggered inter-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of inter-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for inter-frequency L1-RSRP measurement on CSI-RS resouce 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for inter-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs | FFS | Yes |  | NW triggered inter-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |
| 63. NR_Mob_Ph4 | 63-4 | NW triggered inter-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of inter-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for inter-frequency L1-RSRP measurement on CSI-RS resouce 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for inter-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs | FFS | Yes |  | NW triggered inter-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Regarding whether to define intra-/inter-frequency L1-RSRP measurement for CSI-RS, it has been discussed in the previous RAN1 online session previous, but the final conclusion is left to RAN4 for decision. At present, there is no conclusion on this issue from RAN4. Thus, we tend to postpone the discussion of intra-/inter-frequency L1-RSRP measurement related FGs. |
| Nokia | We are fine with the above proposal. However, the followinh FGs are missing for the existing RAN1 agreements: - Inclusion of current SpCell in the L1 measurement report of CSI-RSs: Support of always inclusion of M L1-RSRP measurements of CSI-RS(s) from current SpCell in the measurement report Agreement The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows: - UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI. - Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18) - The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used - No L1 specified filtering for time and spatial domain is introduced - No enhancement on how to report L cells x M beams - Periodic reporting on PUCCH is supported - FFS: semi-persistent reporting on PUCCH/PUSCH, and aperiodic reporting on PUSCH - CSI-RS based LTM TCI state activation and indication: Support of CSI-RS as QCL source RS with QCL-TypeD in the LTM TCI state configuration when TRS is configured as the QCL source RS with QCL-TypeA Agreement CSI-RS for BM as the referenceSignal with QCL-TypeD is supported for an LTM TCI state, where TRS is configured as referenceSignal with QCL-TypeA. |
| Ericsson | Support – we believe it’s OK to define intra- and inter-frequency measurements here. We agree with Nokia that support for always including the serving cell measurements should be a component, just as for Rel-18 Then the use of CSI-RS for BM as QCL source RS would be a separate FG – it’s not directly related to the measurements. |
| Huawei, HiSilicon | At least intra frequency CSI-RS measurement should be supported. Wheterh inter frequency measurement should be supported is up to RAN4. The same definintion on the intra frequency measurement for beam management can be used here. Agree with Nokia and Ericsson’s comment to define FG to always include serving cell result in the report. |
| NTT DOCOMO | We are fine with the above proposal. For support for always including the serving cell measurements, we agree with Nokia, Ericsson, and Huawei. |
| MediaTek | OK with the proposal. On the always including the serving cell measurements, it was defined as separate FG in R18. Similarly, we can later define a separate FG for this. On the CSI-RS for BM as QCL RS, we can later introduce have separate FG(s) (either one FG or two FGs for activation and indication). |
| Moderator (AT&T) | Discuss online. Use the versions from the Moderator as starting point for discussion. |

## Event triggered L1-RSRP reporting

After review of contributions submitted to RAN1 #120bis in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 63. NR_Mob_Ph4 | 63-5 | Event triggered L1-RSRP reporting measured on SSB for event LTM2 | 1. Support of even triggered L1-RSRP reporting for LTM based on [periodic CSI-RS(s) and] SSB(s) 2. Support of [LTM2, LTM3, LTM4, LTM5] based measurement and report 3. Support of MAC CE based event triggered beam report 4. Maximum number of the RRC configured candidate cells and beams 5. Support of serving cell beam measurement for event evaluation by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively | FFS | Yes |  | Event triggered L1-RSRP reporting measured on [periodic CSI-RS(s) and] SSB(s) for event [LTM2, LTM3, LTM4, LTM5] is not supported | FFS | FFS | FFS | FFS | FFS: separate row/FG for periodic CSI-RS FFS: separate rows/FGs for LTM2, LTM3, LTM4, LTM5 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | For FG name, “...... for event LTM2” should be updated as “..... for event evaluation”, it should be applicable to all LTM events rather than only for LTM2. Component1: periodic CSI-RS has been agreed for event-triggered reporting in RAN1#118 meeting. Besides, a typo should be fixed, i.e., “...of even triggered...” should be changed as “... of event triggered....” Agreement For gNB scheduled reporting and event triggered reporting - At least periodic CSI-RS is supported for L1-RSRP measurement for candidate cell - FFS: aperiodic and semi-persistent CSI-RS - At least CSI-RS for beam management is supported for L1-RSRP measurement for candidate cell FFS: CSI-RS for mobility. Finally, we tend to have a unified design for all LTM events. Thus, it is unnecessary to define different FGs for LTM2,3,4,5. |
| Ericsson | Event-driven reporting for LTM is primarily a RAN2 feature, we prefer to wait with these FGs until RAN2 has defined the basic baselines. |
| Huawei, HiSilicon | We think the FG for event trigger should be left to RAN2 |
| NTT DOCOMO | We share same view as Ericsson that it would be better to wait for RAN2 discussion. |
| MediaTek | Agree with companies that we can wait for RAN2 on this FG. |
| Moderator (AT&T) | Wait for more progress in RAN2 |

## CSI Acquisition

After review of contributions submitted to RAN1 #120bis in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 63. NR_Mob_Ph4 | 63-6 | Intra-frequency CSI-RS measurement and CSI reporting for target cell after reception of LTM CSC MAC CE L1-L2 Triggered Mobility (LTM) procedure | 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of the RRC configured candidate cells and CSI-RS resources 3. Maximum number of CSI report configs | FFS | Yes |  | Intra-frequency CSI-RS measurement and CSI reporting for target cell after reception of LTM CSC MAC CE L1-L2 Triggered Mobility (LTM) procedure is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-7 | Intra-frequency CSI-RS measurement for candidate cell after the RRC configuration of configured CSI-RS resource(s) and before reception of LTM CSC MAC CE for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of CSI-RS measurement before reception of CSC MAC CE and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of CSI-RS resources of candidate cell(s) for CSI measurement | FFS | Yes |  | Intra-frequency CSI-RS measurement for candidate cell after the RRC configuration of configured CSI-RS resource(s) and before reception of LTM CSC MAC CE for L1-L2 Triggered Mobility (LTM) procedure is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Similar comments as section 3.1. |
| Nokia | There was no discussion on intra or inter-frequency support for CSI acquisition, that’s why intra-frquency should be removed from both FGs. Also, the disucssion on the number of candidate cells and CSI-RSs resource/report config is still open. We should remove component 2 and 3 from both FGs for now. |
| Ericsson | Support to include the FG. We agree with Nokia that we should wait with components 2 and 3. It is not clear why these should be reported independently. |
| Huawei, HiSilicon | At least intra frequency CSI-RS measurement should be supported. Wheterh inter frequency measurement should be supported is up to RAN4. The same definintion on the intra frequency measurement for beam management can be used here. |
| NTT DOCOMO | Support introduction these FG in principle. “Intra-frequency” should be removed from both FG name for now because we did not deiscuss that thing. For component 2/3 should be removed for now because it would be unclear based on the current agreement. |
| Moderator (AT&T) | Agree the rows without components 2 and 3. Open issues can be left FFS |

# Conclusion

Agreements reached during RAN1 #120bis as part of this agenda item are summarized in [ ].

# References

- R1-2501786, Discussion on UE features for NR mobility enhancements Phase 4, ZTE Corporation/Sanechips

- R1-2501834, Discussion on UE features for NR mobility enhancements Phase 4, vivo

- R1-2501984, Discussion on UE features for NR mobility enhancements Phase 4, CATT

- R1-2502138, UE Features for NR mobility enhancements Phase 4, Nokia

- R1-2502183, Discussion on UE features for NR mobility enhancements Phase 4, CMCC

- R1-2502247, UE features for NR mobility enhancements phase 4, Huawei/HiSilicon

- R1-2502299, Discussion on UE features for NR mobility enhancements, OPPO

- R1-2502396, UE features for NR mobility enhancements Phase 4, Samsung

- R1-2502464, Discussion on UE features for NR mobility enhancements Phase 4, Xiaomi

- R1-2502563, UE features for NR mobility enhancements phase 4, Ericsson

- R1-2502644, Rapporteur input on UE features for Rel-19 NR Mobility enhancements Ph4, Apple

- R1-2502790, Discussion on UE features for NR mobility enhancemens Phase4, NTT DOCOMO, INC.