3GPP TSG RAN WG1 #120bis			R1-250XXXX

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	9.13

Source:	Ad-Hoc Chair (Huawei)

Title:	Session notes for 9.13 LTE-based 5G Broadcast Phase 2

Document for:	Discussion, Decision

## LTE-based 5G Broadcast Phase 2

Please refer to RP-250794 for detailed scope of the WI for LTE-based 5G Broadcast Phase 2.

Rapporteur to provide initial input on higher layer signalling under agenda item 9.13. For input on higher layer signalling from any other source, please include it as part of your tdoc to relevant sub-agenda items.

[120bis-R19-Broadcast] Email discussion on Rel-19 LTE-bad 5G Broadcast Phase 2 – Jordi (EBU)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2502548	Higher layer parameters for Rel-19 LTE-based 5G Broadcast after RAN1#120	EBU

### Time-frequency interleaver design for MBMS dedicated cells

R1-2501912	Discussion on time-frequency interleaver design for LTE-based 5G Broadcast	ZTE Corporation, Sanechips

R1-2502209	On time-frequency interleavers for LTE-based 5G Broadcast	Huawei, HiSilicon

R1-2502390	Views on LTE-based 5G broadcast	Samsung

R1-2502546	Further details on open issues regarding TFI for LTE-based 5G Broadcast	EBU

R1-2502582	Design of Time-frequency interleaver for 5G Broadcast	Shanghai Jiao Tong University

R1-2502861	Time and Frequency Interleaving for 5G Broadcast	Qualcomm Incorporated

R1-2503001	FL summary#1 on Rel-19 LTE-based 5G Broadcast Phase 2	Moderator (EBU)

Agreement

Support at least the following values of time interleaver depth:

- Values M={4,8,16,32} and N={1,2,4,8,16} are supported.

  - FFS: UE behaviour in case values of M and N exceed UE capabilities in terms of soft buffer size and maximum TBS supportable in a TTI

- FFS: larger values, including M=64, N={32,64}.

Proposal

UE to discard the interleaved MTCH when the number of the transmitted TBs of the interleaved MTCH exceed UE capability.

- Note: no RAN1 specification impact

Proposal

RAN1 to minimize resource wastage due to the potential mismatch between MSI periodicity and the (MxN) pattern:

- Adopt an extended set of configurable periodicities for MCH scheduling period for R19 PMCHs that includes the intermediate values among the periodicities defined for the legacy configurations.

- FFS: other solutions

Proposal

In order to reduce the resource wastage due to the potential mismatch between MSI periodicity and the (MxN) pattern:

- Adopt an extended set of configurable periodicities for MCH scheduling period for R19 PMCHs that includes the intermediate values among the periodicities defined for the legacy configurations.

R1-2503002	FL summary#2 on Rel-19 LTE-based 5G Broadcast Phase 2	Moderator (EBU)

Agreement

In case of time interleaving, in order to reduce the resource wastage due to the potential mismatch between MSI periodicity and the (MxN) pattern:

- Adopt an extended set of configurable periodicities for MCH scheduling period for R19 PMCHs that includes the intermediate values among the periodicities defined for the legacy configurations.

  - May include the case of scheduling different M/N within the same MSI periodicity

  - FFS the values of the new periodicities.

- RAN1 to specify UE behavior, if needed, in case of residual mismatch even after new periodicities are adopted, taking into account the amount of subframe available at the end of the periodicity

  - Option 1: Drop the time-interleaved TBs which do not have all the N RVs transmitted within the MSI periodicity

  - Option 2: Drop the time-interleaved TBs which could not be successfully decoded.

    - It is up to network to decide whether to schedule or not schedule MTCH transmission in the residual mismatch part of the MSI periodicity. UE knows this from MSI (legacy behaviour)

  - Other Options not precluded

Agreement

The number of columns, C, of the frequency interleaver is C= #CBs Nsymb ×m, where #CBs denotes the number of CBs in the TB (including scaling, if any), Nsymb denotes the number of OFDM symbols of the PMCH,

- Working assumption:

  - m = Nsymbgcd(Nsymb,#CBs), and gcd(.) denotes the operation to compute the greatest common divisor.

Agreement

The following steps are applied to the rows of the frequency interleaver:

- Perform inter-row permutation with a reorder of the row indexes Ri', which is calculated as:

Ri'=R'i-1+d########################,          R'i-1≤R-d-1 #i×dR#,                 else.

With, 1≤i≤R-1, R'0=0, and d=R/R/RC-C+1.

- Perform a cyclic shift to the elements in the j-th row with an offset S(j) defined as:

Sj=modj+12×-1j,C, j=0,1,2,…,R-1

Proposal

For PMCH with time interleaving, the soft buffer size for the transport block is denoted as NIR.

NIR=NsoftKC∙min⁡(M,Nlayer∙Mlimit)

The soft buffer size for the code block is denoted as Ncb=min⁡(NIRC,Kw).

Where Nsoft , KC , Mlimit and Kw are same as defined in TS 36.212 clause 5.1. M is the number of TBs in the basic transmission pattern of PMCH with time interleaving, Nlayer is the number of the supported layers for spatial multiplexing in DL-SCH according to the UE category.

R1-2503003	FL summary#3 on Rel-19 LTE-based 5G Broadcast Phase 2	Moderator (EBU)

Agreement

LBRM parameters (e.g. NIR) are consistent between the broadcast network and the (multiple) UEs that are receiving the broadcast transmission.

- These multiple UEs may belong to different UE categories, thus having different values of Nsoft, NIR.

- FFS: how the consistent NIR and NCB is calculated

Proposal1

For the TB transmitted in N subframes, the starting pointer for the circular buffer is set as  k0n=2RsubblockTC+rvnEmin, where Emin=NLQmG'/C (as per TS 36.212), and rvn∈{0…N-1} is the redundancy version of the n-th subframe and mapped sequentially in the N subframes (i.e., rvn to subframen pattern is [0,1,2,…N-1]).

Proposal2

For the TB transmitted in N subframes, the starting pointer for the circular buffer is set as  k0n=2RsubblockTC+nEmin, where Emin=NLQmG'/C (as per TS 36.212), and n∈{0…N-1} is mapped sequentially in the N subframes.

Agreement

The agreement in the previous meeting is updated as follows:

Regarding configuration for time interleaving transmission, the following options can be considered:

- Option 1: Per PMCH configuration, e.g., via pmch-Config

- Option 2: Per MBMS session, e.g., via MBMS-SessionInfo or via MAC CE

- Alt1: If per MBMS session configuration is done, a T_proc of 3 ms between the end of the MSI and the first MTCH in the MCH scheduling period is needed for UE de-rate matching.

- Alt2: If per MBMS session configuration is done, other options (than Alt1 above) are not precluded, e.g. that allow the UE to know the M, N values of the first MTCH scheduled after the MSI

Agreement

Obtain the initial TBS (𝑇𝐵𝑆′) as per current specifications, and calculate the final TBS as 𝑇𝐵𝑆=𝑟𝑜𝑢𝑛𝑑{𝑇𝐵𝑆′×𝑁}

- The operation “round{x}” is mapping x to the closest TBS from all the existing TBS values in section 7.1.7.2 TS 36.213.