3GPP TSG-RAN WG1 Meeting #121	R1-2503281

St Julian’s, Malta, May 19 – 23, 2025

Agenda Item:	9.11.2

Source:	Huawei, HiSilicon

Title:	Discussion on HD-FDD RedCap UEs and eRedCap UEs for FR1-NTN

Document for:	Discussion and Decision

# Introduction

In the previous meeting RAN#120bis, the following consensus was achieved [1] with the focus on specifying enhancements targeting the HD-FDD collision cases 3 and 4 for mitigating the issues caused by the mismatch between actual TA used by the UE and TA assumed for the UE at the gNB:

Agreement Update the working assumption in RAN1 #120 with the following updates: For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is supported: - Handling of collision with PDSCH (at least for system information) scheduled by Type-0/0A-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2. - Handling of collision with PDSCH scheduled by Type-1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2. - FFS: handling of PDCCH ordered PRACH transmission - For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that DL is prioritized. - Network is allowed to indicate UL overriding DL for all cases - This is signaled by one UE specific RRC parameter - Note1: if DL is prioritized, the DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized. Note2: UE shall comply to the following existing procedure in 38.331: - UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

In this contribution, we provide our views and propose solutions as per the conclusions and agreements achieved thus far.

# Discussion

## TA mismatch in NTN scenario

In NTN, the TA mismatch between gNB and UE on the determination of DL and UL overlapping and back-to-back non-overlapping symbols is due to the large open loop TA adjustment, where UE compensates TA by itself according to the UE position and serving satellite ephemeris, i.e. NUE, adjTA as defined in clause 4.3.1 of TS38.211.

When UE is configured to report TTA  (based on the UE capability of uplink-TA-Reporting-r17) to gNB, including the component of NUE, adjTA, the misalignment between gNB and UE is caused by the report granularity of the Timing Advance Report MAC CE and the infrequent reporting instances. The mismatch between the TA latest known to gNB and the actual TA used by UE can be as large as 16ms when UE is configured with conditional TA reporting by offsetThresholdTA of 15ms plus the 1ms report granularity of the Timing Advance Report MAC CE as defined in clause 6.1.3.56 of TS38.321. Whereas the TA mismatch is still up to 1.5ms when the UE is configured with the least offsetThresholdTA value of 0.5ms which entails the most frequent triggering of TA reporting at the UE.

From gNB’s perspective, if a collision happens at T0 assuming the latest reported TA from UE, the actual collision that happens to UE can be any time from T0-TAmax,mc to T0+TAmax,mc where TAmax,mc is the maximum TA mismatch between gNB and UE according to the TA report configuration. It should be noted that for a UE incapable of TA reporting, the maximum TA mismatch TAmax,mc can be as large as the difference between the maximum and minimum TA in the cell coverage area. For example, the TA difference is round 0.72ms in a LEO600 edge cell covered by 4 SSB beams, assuming 50km beam diameter, at 30-degree elevation angle. It can also be as large as 6.83ms in a GEO edge cell.

Observation 1: Based on the current TA reporting mechanism, the TA mismatch between UE and gNB can be as large as +/- 16ms with the least frequent report triggering, and up to +/- 1.5ms with the most frequent report triggering.

Observation 2: For a UE incapable of TA reporting or not configured with TA reporting, the TA mismatch can be as large as the difference between the maximum and minimum TA possible in the cell coverage area.

## Collision handling for Case 4

In our view, RAN1 should strive to devise a solution for the priority rule for Case 4 that is similar in principle to the solution agreed for Case 3. That is, to adopt a default rule that can be applied to all use cases unless the network indicates otherwise, with the exception of some use cases that can be left to UE implementation, such as collision of dynamically scheduled UL with PDSCH scheduled by a PDCCH CSS.

Therefore, we propose to confirm the WA from RAN1#120bis wherein for use cases other than collision with PDSCH scheduled by PDCCH in CSS, DL is prioritized by default unless the network has indicated otherwise via RRC or the UL cancellation timeline cannot be satisfied. It is worth noting that this approach leads to a unified technical solution. Such unification harmonizes the technical solution across the newly specified procedures for collision Cases 3 and 4. This in turn improves the clarity of the standard specifications. Considering UE might detect DCI format 1-0 and 0-0 with CRC scrambled with C-RNTI, and MCS-C-RNTI, or a CS-RNTI in Type 0/0A CSS, there is no need to restrict the exceptional case only for system information.

Nonetheless, it was left for further investigation whether or not handling of the potential collision of dynamically scheduled DL with a PDCCH ordered PRACH transmission could be an exceptional use case to the default rule. In our view, there is no need to make another exceptional use case to prioritize the transmission of PDCCH-ordered PRACH since gNB can resend PDCCH order if it does not detect the PRACH transmission. It is also worth noting that when a UE loses the UL synchronization, this does not mean that the UE has lost its DL synchronization nor that it will not be able to receive any critical DL signals/channels.

Observation 3: When a UE loses UL synchronization, this does not mean that the UE has lost its DL synchronization nor that it will not be able to receive any critical DL signals/channels.

Proposal 1: For collision case 4 (dynamic DL and dynamic UL), the updated WA from RAN1#120bis can be confirmed by removing “at least for system information” and FFS points of the exceptional use case for PDCCH-ordered PRACH.

### Comments on draft Editor CR to TS38.213

After RAN1#120bis, an email discussion took place to discuss the draft Editor CR on TS38.213 capturing the agreements and conclusions for this WI. In the following clauses our Editor has captured the agreed working assumption for the most part, but indicated for the PDSCH occurrences highlighted in yellow below that it was unclear from the wording of the WA whether the collision handling rules, as default or when network override is configured via RRC, exclude the collision with a PDSCH scheduled by Type 0/0A/1/2-PDCCH CSS.

Therefore, we propose that RAN1 clarifies that for Case 4, collision with a PDSCH scheduled by Type 0/0A/1/2-PDCCH CSS is excluded from the both, the default UE behaviour, i.e., DL reception is prioritized, and the UE behaviour when network override is configured via RRC.

The following TP may also be endorsed to capture RAN1’s understanding into the next version of the Editor CR to TS38.213:

TP # 1

## *** Unchanged parts are omitted *** If a HD-UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH in a set of symbols, or if a HD-UE operating on an NTN serving cell is not provided ntnRedCapCollisionCase4UlPriority and detects a DCI format indicating to the HD-UE to transmit SRS, or PUCCH, or PUSCH in a set of symbols, and the HD-UE detects a DCI format indicating to the HD-UE to receive CSI-RS or PDSCH, except for a DCI format provided by a PDCCH received according to a Type0/0A/1/2-PDCCH CSS set on an NTN serving cell, in a subset of symbols from the set of symbols, then - the HD-UE does not expect to cancel the transmission of the PUCCH or PUSCH in the set of symbols if the first symbol in the set occurs within Tproc,2 relative to a last symbol of a PDCCH reception where the HD-UE detects the DCI format; otherwise, the HD-UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], determined from clauses 9 and 9.2.5 or clause 6.1 of [6, TS 38.214]. - the HD-UE does not expect to cancel the transmission of SRS in symbols from the subset of symbols that occur within Tproc,2 relative to a last symbol of a PDCCH reception where the HD-UE detects the DCI format. The HD-UE cancels the SRS transmission in remaining symbols from the subset of symbols. Tproc,2 is the PUSCH preparation time for HD-UE processing capability 1 [6, TS 38.214] assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the SRS, PUCCH, PUSCH. If a HD-UE operating on an NTN serving cell is provided ntnRedCapCollisionCase4UlPriority and detects a DCI format indicating to the HD-UE transmit SRS, or PUCCH, or PUSCH in a set of symbols, and the HD-UE detects a DCI format indicating to the HD-UE to receive CSI-RS or PDSCH, except for a DCI format provided by a PDCCH received according to a Type0/0A/1/2-PDCCH CSS, in any symbol from the set of symbols, the HD-UE does not receive the CSI-RS or PDSCH. ## *** Unchanged parts are omitted ***

Proposal 2: RAN1 to clarify in a conclusion that for Case 4, collision with a PDSCH scheduled by Type 0/0A/1/2-PDCCH CSS is excluded from both, the default UE behaviour, i.e., DL reception is prioritized, and when network override is configured via RRC (TP#1 in R1-2503281 can be endorsed for reference).

## Enhanced TA reporting for reducing TA mismatch

It was agreed in RAN1#118 [2] that TA reporting enhancements for Rel-19 NTN HD-FDD (e)Redcap UE can be further studied to mitigate the collisions of Case 3 and Case 4. Several candidate enhancements were listed as follows:

Agreement To mitigate the collisions of case3 and case4, the following TA reporting enhancements for Rel-19 NTN HD-FDD (e)Redcap UE can be further studied: - Finer TA report granularity - Smaller TA offset threshold - Trigger a TA report when the collision is detected at the UE - TA reporting with a new triggering mechanism from gNB - TA drifting rate reporting Note: The benefit, complexity, power consumption and signaling overhead of each scheme is to be further investigated. Note: other solutions can also be studied

First, it is important to note that TA reporting enhancement is beneficial for HD-FDD RedCap/eRedCap UEs regardless of whether gNB attempts to avoid a collision, e.g. by conservative scheduling, or cancellations would occur consequently for Cases 3 and 4.

Second, as analysed earlier in Section 2.1, with the current TA report mechanism, the TA mismatch, and thus the ambiguity period on UE timing at gNB side, can be large due to 1) the TA report granularity, and more importantly 2) the configured TA report triggering offset threshold. Therefore, at least for UEs capable of TA reporting, enhancing the TA report without significantly increasing the reporting overhead to reduce the TA mismatch should be the design objective when selecting from the candidate enhancements listed above.

Since the dynamic overhead of TA reporting depends on both the report size and the frequency of triggering, it is important for an enhanced TA report to use a MAC CE of same or smaller size as that of the legacy TAR (2 Octets: 14bits + 2 Reserved bits), especially if gNB intends to configure a small triggering threshold offsetThresholdTA to cope with TA variations at the UE.

Observation 4: It is important for an enhanced TA report to use a MAC CE of same or smaller size as that of the legacy TAR (2 Octets: 14bits + 2 Reserved bits), especially if gNB intends to configure a small triggering threshold offsetThresholdTA to cope with TA variations at the UE.

In terms of finer TA report granularity and smaller offset threshold;

In our view, TA reporting granularity finer than a slot duration (1ms) is beneficial to mitigating the HD-FDD collisions for (e)RedCap operation in FR1-NTN. This due to the fact that not all the TDRA of colliding DL and UL channels/signals are slot based, e.g., PDCCH/CSI-RS, and PUSCH Type B/PUCCH/SRS, respectively. Also, according to existing Rel-17 spec for HD-FDD, cancellation of SRS is partial for only the symbols in collision with a DL reception and not for the whole slot. In addition, a UL transmission and a non-overlapping DL reception could be determined as colliding if the gap is less than the minimum transition gap (~13µs) irrespective of the slot duration. Finally, the actual TA value is applied at the UE to the UL transmissions in units of Tc and not in slot units.

Observation 5: TA reporting granularity finer than a slot duration (1ms) is beneficial to mitigating the HD-FDD collisions for (e)RedCap operation in FR1-NTN due to the following reasons:

- Not all the TDRA of colliding DL signals/channels are slot based, e.g. PDCCH and CSI-RS

- Not all the TDRA of colliding UL signals/channels are slot based, e.g. PUSCH rep Type B, PUCCH and SRS

- According to existing Rel-17 spec for HD-FDD, cancellation of SRS is partial for only the symbols in collision with a DL reception and not for the whole slot

- Actual TA is applied at the UE to the UL transmissions in units of Tc and not in slot units

- A UL transmission and a non-overlapping DL reception could be determined as colliding if the gap is less than the minimum transition gap (~13µs) irrespective of the slot duration

In order to investigate the performance benefits, the signalling overhead and the feasibility of each TA enhancement as per the above agreement, SL simulations have been conducted as discussed in the next subsections 2.31 and 2.3.2.

Based on the SLS results and the observations therein we bring forward the following proposal:

Proposal 3: RAN1 supports enhanced TA reporting with at least a configurable report granularity finer than 1ms, i.e., X µs, and triggering offset threshold values less than 0.5 ms

- Note: Assuming a maximum TA of 520 ms, for a 16-bit MAC CE, X ≥ 7.9351 µs (or 15601*Tc)

### SLS Results w/o Collision Avoidance: Ratio of cancelled CG PUSCH resources under Cases 1/3

We have conducted SL simulations based on the assumptions and parameters agreed for Rel-19 NTN DL coverage enhancement WI objective for Set 1-2. These SL simulations calculate the ratio of PUSCH resources cancelled as a result of HD collisions out of the CG resources allocated to each UE. Thus, in the simulations herein, gNB does NOT attempt to avoid the HD collisions and does NOT apply any resource reservation before or after the common and UE-specific DL resources based on the latest reported TA.

FTP3 DL traffic, 16 active Earth-fixed beams with one SSB per beam, and SSB/SIB1/SIB19 periodicity of 20ms were assumed. CG resources are allocated to each UE as 8 full-slot repetitions per 20ms CG period, wherein the CG slot offset is configured to each UE based on the associated SSB timing. In addition, the following assumptions and parameters were considered as applicable to the HD-FDD operation of (e)RedCap UEs and the legacy/enhanced TA reporting schemes in use:

- Common and UE-specific DL receptions have been prioritized to UL transmission according to Case 1 and Case 3 default priority rule.

- A triggered (e)TAR is sent on the earliest available resources after a minimum delay of at least the one-way propagation delay between the UE and the Satellite node plus the PUSCH preparation delay in FR1.

- (e)TAR triggering offset threshold values considered are {0.1, 0.05} ms in addition to the 0.5ms minimum value available for the legacy TAR

- (e)TAR granularities considered are {1OS, and 13 us being the DL-UL Gap required for HD operation} in addition to the 1 ms granularity of the legacy TAR

- Triggering rate for each (e)TAR scheme has been computed as the total number of triggered reports over the whole simulation time.

- In addition, the maximum value of the TA delta has been recorded over the whole simulation time for each of the triggering offset threshold values to allow for calculating the number of bits needed in a new 1-Octet MAC CE assuming a granularity of the DL-UL-Gap duration.

Table 1. Gains in reducing the Ratio of cancelled CG PUSCH resources under collision Case 1 and Case 3 for legacy and enhanced TA reporting schemes employing different values of report granularity and triggering offset

From the results captured in Table 1, the following observations can be made:

- In comparison to the No-TAR scheme (gNB using the max possible TA value estimated over the cell), the legacy TAR scheme with 1ms report granularity achieves a gain of 24% reduction in the ratio of CG PUSCH cancellation from the CG resources allocated to the UE

- The performance of the legacy TAR w.r.t the No-TAR scheme conforms to the group’s intuition that is no further gain is expected from the legacy 1ms TAR even if new values < 0.5 ms are introduced for the triggering offset threshold.

- Taking the legacy 1ms TAR with the 0.5ms threshold as the baseline scheme, the performance of enhanced TAR schemes with a report granularity finer than 1ms, e.g. 1OS and 13us, aligns with observations made in some contributions [3], that introducing finer TAR granularity without introducing new values < 0.5 ms for the triggering offset threshold does not provide any relative gain.

  - eTAR with 1OS granularity provides a gain of ~38% on top of Baseline scheme when sending a 16-bit report every 2.35 sec

  - eTAR with 1OS granularity provides a gain of ~10% on top of Baseline scheme when sending a 16-bit report every 4.6 sec

  - eTAR with a DL-UL gap (13us) granularity provides a gain of ~39% on top of Baseline scheme when sending report every 2.35 sec

  - eTAR with a DL-UL gap (13us) granularity provides a gain of ~12% on top of Baseline scheme when sending a 16-bit report every 4.6 sec

  - If eTAR employs TA delta reporting with 1-Octet MAC CE, we can offset the increased triggering frequency due to the 50% saving in signaling overhead

Observation 6: Based on the SLS results assessing the gains in reducing the ratio of cancelled CG PUSCH resources under Cases 1/3 for the legacy and enhanced TA reporting schemes employing different values of report granularity and triggering offset threshold, the following conclusions can be made:

- Compared to a No-TAR scheme, legacy TAR with 1ms report granularity achieves a gain of 24% reduction in the ratio of CG PUSCH cancellation

- Results conform to the group’s intuition that is no further gain is expected from the legacy 1ms TAR even if new values < 0.5 ms are introduced for the triggering offset threshold

- Taking the legacy 1ms TAR with the 0.5ms threshold as the baseline scheme,

  - Introducing finer TAR granularity without introducing new values < 0.5 ms for the triggering offset threshold does not provide any further relative gain

  - Enhanced TAR schemes with granularity finer than 1 ms, can achieve relative gains up to 39% or 12% with new triggering thresholds of 0.05 ms or 0.1ms, respectively

- If eTAR employs TA delta reporting with 1-Octet MAC CE, the 50% saving in signalling overhead helps to offset the increased triggering frequency

### SLS Results with gNB Resource Reservations: Percentage of Available UL Resources

We have also conducted SL simulations based on the assumptions and parameters agreed for Rel-19 NTN DL coverage enhancement WI objective for Set 1-2, yet in these SL simulations we calculate the remaining resources available for UL scheduling for each UE when gNB applies the TA ambiguity period before and after the common and UE-specific DL transmissions to avoid collisions based on the latest reported TA.

Given the discussion in Rel-19 NTN UE Features, this simulation is quite relevant for (e)RedCap HD-FDD UEs that do not indicate the new Rel-19 capability of handling Case 3/4 collisions in which case the gNB would have to resort to resource reservations to prevent the occurrence of case 3/4 error cases for such incapable UEs

Observation 7: Despite the priority rules defined in Rel-19 for HD-FDD collision handling in Cases 3 and 4, performance evaluations considering gNB resource reservations for HD-FDD collision avoidance are still relevant to the (e)RedCap UEs that do not indicate the new Rel-19 capability of handling Case 3/4 collisions.

FTP3 DL traffic, 16 active Earth-fixed beams with one SSB per beam, and SSB/SIB1/SIB19 periodicity of 20ms were assumed. In addition, the following assumptions and parameters were considered as applicable to the HD-FDD operation of (e)RedCap UEs and the legacy/enhanced TA reporting schemes in use:

- TA ambiguity period in symbols after the required DL-UL gap from the end of a DL transmission, or before the required UL-DL gap from the start of the DL transmission, accounts for the configured triggering offset threshold plus the (e)TAR granularity as shown in Figure 1.

- Common and UE-specific DL reception has been prioritized to UL transmission to assess the impact on resource availability, i.e., according to Case 1 and Case 3 default priority.

- Same assumptions as in previous Section 2.3.1 for (e)TAR latency, triggering thresholds, and report granularities were considered.

Figure 1. Example of UL cancellation at gNB based on the latest (e)TAR, report granularity and triggering offset; and UL cancellation at the UE based on the actual TA.

From the results captured in Table 2, the following observations can be made:

- The legacy TAR scheme with the 1ms granularity, even with the smallest value of the triggering offset threshold of 0.5ms, substantially renders the NTN (e)RedCap HD UEs resource deprived (~ 8%) for scheduling UL transmissions. This emphasizes the need for RAN1 to consider enhancing the TA reporting for enabling the HD operation of (e)RedCap UEs, especially UEs incapable of handling Case 3 and Case 4.

- Across all schemes, using a smaller triggering offset threshold renders more available resources for UL scheduling at the expense of increasing the triggering rate by the same factor. However, in contrast to the legacy TAR scheme, the enhanced TA reporting schemes with finer granularity (1OS or less) render from 27% to 40% more resources for UL when the threshold ranges from 0.5ms to 0.05ms, respectively.

- Across all schemes, for any given triggering threshold, the eTAR scheme with granularity of 13us renders the largest percentage of UL available resources that is closest to the ideal scheme assuming an absolute (non-quantized) TA report. Intuitively, the granularity of 13us (required DL-UL/UL-DL Gap in FR1) allows for better HD collision avoidance than the granularity of 1OS.

Table 2. SLS results for legacy and enhanced TA reporting schemes employing different values of report granularity and triggering offset threshold

Observation 8: Based on the SL simulation results, the following observations are made:

-	Even with the smallest value of the triggering offset threshold (0.5ms), the legacy TAR scheme with 1ms granularity renders substantially limited resources (~ 8%) for scheduling UL. Thus, TA reporting enhancements are essential for enabling the HD operation of (e)RedCap UEs, especially UEs incapable of handling Cases 3/4

-	Smaller triggering thresholds render more resources available for UL at the expense of increased triggering rate by the same factor. With finer granularity (1OS or less), around 27% to 40% more resources are released for UL when the threshold ranges from 0.5ms to 0.05ms.

-	Enhanced TAR scheme with 13us granularity renders the largest percentage of UL available resources closest to the ideal scheme assuming an absolute (non-quantized) TA report.

- 	The granularity of 13us allows for better HD collision avoidance than the granularity of 1OS, the relative gain is from 8.5% to 3.5% when the triggering threshold ranges from 0.5ms to 0.05ms, respectively.

### Other FFS Points on TA Reporting Enhancements

TA reporting with a new triggering mechanism from gNB: It should be noted that a new triggering mechanism from gNB could be easily integrated with our proposed enhancements above for finer TA granularity with the following potential impact to the legacy TAR procedure depicted in 38.321. That is, the legacy TAR procedure may be extended to include additional triggering events for the UE MAC entity to recover from missed eTAR/TADR at gNB:

A Timing Advance report (TAR) shall be triggered if any of the following events occur:

-	upon indication from upper layers to trigger a Timing Advance report;

-	upon configuration of offsetThresholdTA by upper layers, if the UE has not previously reported Timing Advance value to current Serving Cell;

-	if the variation between the current estimate of the Timing Advance value and the last reported Timing Advance value is equal to or larger than offsetThresholdTA, if configured;

-	upon determining that a Timing Advance Delta Report has not been received at gNB, or upon receiving a request via a DL MAC CE or via a scheduling DCI (e.g. UL grant) to send the latest TA value in a legacy TAR

Proposal 4: the legacy TAR procedure can be extended to include additional triggering events upon receiving a request via a DL MAC CE or via a scheduling DCI (e.g. UL grant) to send the latest TA value in a legacy TAR or for the UE MAC entity to recover from missed (e)TAR at gNB.

## UE collision determination & scenarios with more than two Transmissions/Receptions

The issue of HD collisions involving ‘more than two overlapping transmissions’ was discussed in last meeting in the FLS [3] yet no conclusion was made due to the lack of time. The following proposal was made by FL to close this issue in Rel-19:

[Proposal 2-1]: The issue of more than two overlapping transmissions in NTN can be left to the UE implementation.

Considering the following facts;

- A typical HD-FDD collision involves one UL transmission and one DL reception, and they do not have to be overlapping (DL-UL or UL-DL switching gap less than required).

- Only the UE knows the actual UL timing based on its actual TA as well as the UL timing assumed by gNB based on the latest reported TA.

First, the proposal needs to clarify that the issue results from involving more than one UL transmission or one DL reception.

Second, as we explained in our previous contribution [4], from the UE’s perspective, a UL transmission could be involved in collisions with more than 2 DL receptions; one assumed by gNB and another as determined by the UE, each could be subject to two different conflicting priority rule as shown in Figure 3, e.g., the first following Case 1 and the other following Case 3 with UL is prioritized. So, the proposal needs to clarify that the two involved UL transmissions or DL receptions are both determined by the UE based on the actual TA.

As such, we can support the FL's proposal with the following clarifications:

[Proposal 2-1] Modified: The issue of a HD-FDD collision in NTN involving more than two overlapping UL transmissions or DL receptions in NTN as determined by the UE based on the actual TA can be left to the UE implementation.

Proposal 5: To conclude on the issue of collisions with more than two Transmissions/Receptions, clarify the following:

- the issue results from collisions involving more than one UL transmission or one DL reception

- the more than one UL transmissions or DL receptions involved are as determined by the UE based on the actual TA

Figure 3. A UL transmission could be involved in collisions with more than 2 DL receptions; one assumed by gNB and another as determined by the UE, each could be subject to two different conflicting priority rule, e.g., the first following Case 1 and the other following Case 3 with UL prioritized

## 2.5 RRC Parameters

After RAN1#120bis, an email discussion took place to discuss the RRC parameters for some Rel-19 WIs. However, the RRC parameters for NR NTN are expected to be discussed again in this meeting. Therefore, following Table 4 lists the relevant details according to the recommendations for RAN1 RRC parameter preparation provided in [5], and assuming Proposal 1 is agreeable:

- We note that while the group has been referring to the newly agreed procedures for HD collisions cases as ‘Case 3’ and ‘Case 4’, in our understanding such terminologies are not defined anywhere in RAN1 spec and therefore should not be incorporated into the respective RRC parameter names.

- Also, it should be noted that the contents of ‘Description’ column are incorrect for both parameters and need to be corrected to align with quoted RAN1 agreements in the last column, e.g., as shown in Table 4 below.

Table 4: (e)RedCap related RRC parameters collected in RAN1 [6]

| Sub-feature group | RAN2 Parent IE | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Support of HD-FDD RedCap and HD-FDD eRedCap UEs operating in FR1-NTN bands | BWP-UplinkDedicated | ntn-PrioritizeUl-semiStaticHDcollisions | New | ntn-PrioritizeUl-semiStaticHDcollisions | In RRC_CONNECTED mode, for DL-UL collisions in case 3 (semi-static DL reception colliding with semi-static UL transmission) other than collisions with Type 0/0A/1/2 PDCCH in CSS, DL reception is prioritized by default. If this parameter is configured, it indicates network overriding of the default priority rule, i.e., UL is prioritized in such collisions | {0,1} (True, False) | 0 (False) | Per UE | No |
| Support of HD-FDD RedCap and HD-FDD eRedCap UEs operating in FR1-NTN bands | BWP-UplinkDedicated | ntn-PrioritizeUl-DynamicHDcollisions | New | ntn-PrioritizeUl-DynamicHDcollisions | In RRC_CONNECTED mode, for DL-UL collisions in case 4 (dynamically scheduled DL reception colliding with dynamically scheduled UL transmission) other than collisions with PDSCH scheduled by Type 0/0A/1 PDCCH in CSS, DL reception is prioritized by default. This parameter indicates network overriding of the default priority rule, i.e., UL is prioritized in such collisions. | {0,1} (True, False) | 0 (False) | Per UE | No |

Proposal 6: RAN1 to discuss the proposed corrections to the RRC parameters in Table 4 and include the outcome in the LS to RAN2.

# Conclusions

In this contribution, enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB for HD collisions cases (cases 3 and 4) to support HD-FDD RedCap UEs and eRedCap UEs operation in FR1-NTN are discussed. The following observations and proposals are presented:

Observation 1: Based on the current TA reporting mechanism, the TA mismatch between UE and gNB can be as large as +/- 16ms with the least frequent report triggering, and up to +/- 1.5ms with the most frequent report triggering.

Observation 2: For a UE incapable of TA reporting or not configured with TA reporting, the TA mismatch can be as large as the difference between the maximum and minimum TA possible in the cell coverage area.

Observation 3: When a UE loses UL synchronization, this does not mean that the UE has lost its DL synchronization nor that it will not be able to receive any critical DL signals/channels.

Observation 4: It is important for an enhanced TA report to use a MAC CE of same or smaller size as that of the legacy TAR (2 Octets: 14bits + 2 Reserved bits), especially if gNB intends to configure a small triggering threshold offsetThresholdTA to cope with TA variations at the UE.

Observation 5: TA reporting granularity finer than a slot duration (1ms) is beneficial to mitigating the HD-FDD collisions for (e)RedCap operation in FR1-NTN due to the following reasons:

- Not all the TDRA of colliding DL signals/channels are slot based, e.g. PDCCH and CSI-RS

- Not all the TDRA of colliding UL signals/channels are slot based, e.g. PUSCH rep Type B, PUCCH and SRS

- According to existing Rel-17 spec for HD-FDD, cancellation of SRS is partial for only the symbols in collision with a DL reception and not for the whole slot

- Actual TA is applied at the UE to the UL transmissions in units of Tc and not in slot units

- A UL transmission and a non-overlapping DL reception could be determined as colliding if the gap is less than the minimum transition gap (~13µs) irrespective of the slot duration

Observation 6: Based on the SLS results assessing the gains in reducing the ratio of cancelled CG PUSCH resources under Cases 1/3 for the legacy and enhanced TA reporting schemes employing different values of report granularity and triggering offset threshold, the following conclusions can be made:

- Compared to a No-TAR scheme, legacy TAR with 1ms report granularity achieves a gain of 24% reduction in the ratio of CG PUSCH cancellation

- Results conform to the group’s intuition that is no further gain is expected from the legacy 1ms TAR even if new values < 0.5 ms are introduced for the triggering offset threshold

- Taking the legacy 1ms TAR with the 0.5ms threshold as the baseline scheme,

  - Introducing finer TAR granularity without introducing new values < 0.5 ms for the triggering offset threshold does not provide any further relative gain

  - Enhanced TAR schemes with granularity finer than 1 ms, can achieve relative gains up to 39% or 12% with new triggering thresholds of 0.05 ms or 0.1ms, respectively

- If eTAR employs TA delta reporting with 1-Octet MAC CE, the 50% saving in signalling overhead helps to offset the increased triggering frequency

Observation 7: Despite the priority rules defined in Rel-19 for HD-FDD collision handling in Cases 3 and 4, performance evaluations considering gNB resource reservations for HD-FDD collision avoidance are still relevant to the (e)RedCap UEs that do not indicate the new Rel-19 capability of handling Case 3/4 collisions.

Observation 8: Based on the SL simulation results, the following observations are made:

-	Even with the smallest value of the triggering offset threshold (0.5ms), the legacy TAR scheme with 1ms granularity renders substantially limited resources (~ 8%) for scheduling UL. Thus, TA reporting enhancements are essential for enabling the HD operation of (e)RedCap UEs, especially UEs incapable of handling Cases 3/4

-	Smaller triggering thresholds render more resources available for UL at the expense of increased triggering rate by the same factor. With finer granularity (1OS or less), around 27% to 40% more resources are released for UL when the threshold ranges from 0.5ms to 0.05ms.

-	Enhanced TAR scheme with 13us granularity renders the largest percentage of UL available resources closest to the ideal scheme assuming an absolute (non-quantized) TA report.

- 	The granularity of 13us allows for better HD collision avoidance than the granularity of 1OS, the relative gain is from 8.5% to 3.5% when the triggering threshold ranges from 0.5ms to 0.05ms, respectively.

Proposal 1: For collision case 4 (dynamic DL and dynamic UL), the updated WA from RAN1#120bis can be confirmed by removing “at least for system information” and FFS points of the exceptional use case for PDCCH-ordered PRACH.

Proposal 2: RAN1 to clarify in a conclusion that for Case 4, collision with a PDSCH scheduled by Type 0/0A/1/2-PDCCH CSS is excluded from both, the default UE behaviour, i.e., DL reception is prioritized, and when network override is configured via RRC (TP#1 in R1-2503281 can be endorsed for reference).

Proposal 3: RAN1 supports enhanced TA reporting with at least a configurable report granularity finer than 1ms, i.e., X µs, and triggering offset threshold values less than 0.5 ms

- Note: Assuming a maximum TA of 520 ms, for a 16-bit MAC CE, X ≥ 7.9351 µs (or 15601*Tc)

Proposal 4: the legacy TAR procedure can be extended to include additional triggering events upon receiving a request via a DL MAC CE or via a scheduling DCI (e.g. UL grant) to send the latest TA value in a legacy TAR or for the UE MAC entity to recover from missed (e)TAR at gNB.

Proposal 5: To conclude on the issue of collisions with more than two Transmissions/Receptions, clarify the following:

- the issue results from collisions involving more than one UL transmission or one DL reception

- the more than one UL transmissions or DL receptions involved are as determined by the UE based on the actual TA

Proposal 6: RAN1 to discuss the proposed corrections to the RRC parameters in Table 4 and include the outcome in the LS to RAN2.

# References

Chairman notes, RAN1#120bis.

Chairman notes, RAN1#118.

R1-2503050, Summary #2 for Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	Moderator, CATT

R1-2502220, Discussion on HD-FDD RedCap UEs and eRedCap UEs for FR1-NTN	Huawei, HiSilicon

R1-2305769, Recommendations for RAN1 RRC Parameter Preparation, Ericsson.

R1-2503156, Collection of Rel-19 higher layers parameters list Post RAN1#120bis, Ericsson.

RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3.