3GPP TSG RAN WG1 #121 Meeting		             		R1-2503418

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	9.5.3

Source:	NEC

Title:	Discussion on adaptation of common signal/channel transmissions

Document for:	Discussion

# Introduction

The following agreements have been reached in RAN1#120bis [1]:

Agreement

For adaptation of PRACH in time-domain, at least for 4-step RACH, at least for DCI 1_0 with P-RNTI,

- Support configuration of the additional PRACH resources within [the same] RACH-ConfigCommon in SIB1 used to configure the legacy PRACH resources

  - the legacy PRACH resources used for ‘additional RO validation before the SSB-RO mapping’ are configured in the RACH-ConfigCommon

Agreement

For DCI-based adaptation for additional PRACH resources, the following 1-bit field is used for adaptation indication in DCI format 1_0 with P-RNTI

- Use one bit from the Bits 5-8 within the Short Message field (from upper layers)

- Send LS to RAN2 to confirm the use of this bit.

Above applies for cell that transmits the DCI for connected UEs and IDLE/INACTIVE mode UEs

Agreement

For DCI-based adaptation for additional PRACH resources, for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI

- the validity duration is configured via higher layer signalling.

Agreement

For DCI-based adaptation for additional PRACH resources, PRACH mask that identifies the subset of the additional PRACH resources is applicable at unit of

- PRACH association period

- This PRACH mask applies to every [configured] K SSB RO association pattern period(s)

Agreement

For adaptation of SSB in time-domain, for DCI 2_9-based SSB burst periodicity adaptation for an SCell,

- The DCI is scrambled a new RNTI,

  - Same search space and DCI size as that of cell DTX/DRX DCI if gNB configures both

Agreement

For DCI 2_9-based SSB burst periodicity adaptation for an SCell

- the starting location of the information block for SSB burst periodicity indication for a SCell within the DCI format 2_9 is configured using a new RRC parameter

- the length of the information block is given by ceil(log2(1+X)), where UE is configured with X additional SSB burst periodicities for the SCell

Agreement

For adaptation of SSB in time-domain, UE can be configured with X (<=Xmax) additional SSB burst periodicities for an SCell.

- Xmax=2

Agreement

Separate configuration of the following parameters for the additional PRACH resources at least for 4-step RACH is supported

- CB-PreamblesPerSSB

Agreement

LS on DCI-based PRACH adaptation endorsed with the ACTION part modified compared to draft LS in R1-2503085 as follows:

- ACTION: RAN1 respectfully asks RAN2 to confirm whether the use of above bit is feasible.

Final LS in R1-2503086.

Agreement

For adaptation of PRACH in time-domain, for a connected mode UE,

- One of the reserved bits of PDCCH order (DCI 1_0 with C-RNTI) is used for the new DCI field that indicates the availability of additional PRACH resources.

Working Assumption

When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first [actually] transmitted SSB within the first [possible] SSB burst according to the indicated SSB burst periodicity that is no earlier than the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell [in Table 11.5-1 of TS 38.213].

- FFS: how to determine the first [possible] SSB burst

Agreement

For DCI-based adaptation for additional PRACH resources, the PRACH mask to identify the subset of the additional PRACH resources is given by:

- Option 1-2:  Semi-static signalling of a PRACH mask index and a value of K (number of association pattern periods)

  - For K: one from up to four candidate values {2,4,8, [1 or 16]}

| Mask index | Indication of association periods (AP) for subset of additional PRACH resources within every K association pattern periods (APP) |
| --- | --- |
| 0 | 1st half of the APs in K APPs |
| 1 | 1st quarter of the APs in K APPs |
| 2 | 1st eighth of the APs in K APPs |
| 3 | 1st sixteenth of the APs in K APPs |

Agreement

- For DCI-based adaptation for additional PRACH resources, the reference point for the availability of additional PRACH resources indicated by DCI 1_0 with P-RNTI is the start of first frame of the current SI modification period where UE receives the DCI

- The validity duration configured by higher layer signalling for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI is

  - (Option 4) Multiple of SI modification period ({[1],2,[3],4,8,[],[],..})

In this contribution, we discuss our views on the issues to support adaptation of SSB in time domain for connected mode UE’s SCell and PRACH adaptation for UEs in idle/inactive and connected mode.

# Discussion

## Mechanism of SSB Adaptation in Time Domain

It was agreed in RAN1#118 that both the adaptation for SSB that is NCD-SSB on sync raster (B2’) and the adaptation for SSB that is NCD-SSB not on sync raster (B3’) are supported [3]. And for the mechanism for SSB adaptation further agreements were reached in RAN1#120 [2] and RAN1#120bis [1].

Agreement

For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell

- UE is configured with SSB burst periodicity using legacy signalling for the SCell

- UE is configured with X additional SSB burst periodicity for the SCell

  - FFS: Value of X

- SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity

  - FFS: Whether there is specification impact

  - Note: This does not impact the discussion on OD-SSB

- For switching the periodicity, down-select between

  - (MAC-CE)

    - MAC-CE details for SSB burst periodicity adaptation is up to RAN2.

  - (DCI)

    - Alt 1-3: DCI based signalling is used

Agreement

For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell

- Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI]

- FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1)

Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

Agreement

For DCI 2_9-based SSB burst periodicity adaptation for an SCell

- the starting location of the information block for SSB burst periodicity indication for a SCell within the DCI format 2_9 is configured using a new RRC parameter

- the length of the information block is given by ceil(log2(1+X)), where UE is configured with X additional SSB burst periodicities for the SCell

Agreement

For adaptation of SSB in time-domain, UE can be configured with X (<=Xmax) additional SSB burst periodicities for an SCell.

- Xmax=2

Here, we discuss the remaining open points about SSB adaptation mechanism.

There was discussion related to PRACH occasion validity, handling PDCCH reception and PDSCH rate matching based on SSB adaptation in RAN1#120bis.

Legacy mechanisms for determining PRACH occasion validity, handling PDCCH reception (e.g., rate matching around SSB), and PDSCH rate matching rely on the semi-statically configured SSB transmission periodicity and pattern. However, the introduction of SSB burst periodicity adaptation in Rel-19 allows for dynamic switching between a legacy-configured periodicity and one or more additional configured periodicities, triggered via DCI (e.g., DCI format 2_9).

Requiring UEs to dynamically adjust PRACH validity checks, PDCCH monitoring behavior, and PDSCH rate matching based on the potentially rapidly changing SSB periodicity indicated by DCI introduces significant UE implementation complexity. Furthermore, if a UE fails to decode the DCI indicating an SSB periodicity switch (e.g., due to target error rate of PDCCH and/or poor channel conditions), its understanding of the current SSB pattern will diverge from the gNB's assumption. This ambiguity can lead to missed PRACH opportunities, incorrect PDCCH monitoring, or failed PDSCH reception due to rate matching mismatches.

To avoid this complexity and ambiguity, we propose that PRACH validity determination, PDCCH reception procedures, and PDSCH rate matching related to SSB positions should operate based on a single, semi-statically determined SSB periodicity, regardless of the instantaneous periodicity indicated by DCI. Specifically, the UE should use the lowest periodicity value among all configured SSB burst periodicity values (the legacy configured one and any additional ones) for these operations. To further simplify implementation and minimize changes compared to legacy behavior, it is recommended that the SSB burst periodicity configured via legacy signalling (e.g., in ServingCellConfigCommon) is set to the shortest periodicity intended for the cell's operation.

Proposal 1: For determining PRACH occasion validity, valid PDCCH monitoring occasions, and PDSCH rate matching around SSBs, the UE shall use the lowest  SSB burst periodicity value among all configured SSB burst periodicities for the SCell, irrespective of the dynamically indicated SSB burst periodicity via DCI.

Proposal 2: It is recommended that the SSB burst periodicity configured via legacy signalling (e.g., in ServingCellConfigCommon) corresponds to the lowest SSB burst periodicity intended for the SCell operation.

RAN1 has a working assumption that when a UE receives a DCI indicating a switch in SSB burst periodicity for an SCell, the UE assumes the SSB transmission according to the new periodicity starts from the beginning of the first slot containing the first actually transmitted SSB within the "first [possible] SSB burst" that occurs no earlier than slot m+d (where m is DCI reception slot and d is processing delay). While this defines the earliest possible start boundary and confirms the UE uses the indicated periodicity, the exact timing alignment of this "first possible SSB burst" relative to the previous timing grid remains an FFS.

This ambiguity arises because a given periodicity can be realized with different time offsets or phases. Consider switching from a 20ms periodicity to a 40ms periodicity. If the 20ms bursts occurred in specific SFNs (e.g., corresponding to pattern {0, 2, 4, 6,...} relative to a cycle start), switching to 40ms presents multiple alignment possibilities. For instance, should the 40ms bursts align with the previous pattern {0, 4, 8,...} (Option 1) or pattern {2, 6, 10,...} (Option 2)? The current working assumption does not specify which alignment the UE should expect, potentially leading to misalignment between UE expectation and gNB transmission after the switch.

To resolve this ambiguity and provide necessary network control, we propose that the specific time offset or alignment for each configured SSB burst periodicity (legacy and additional) should be configurable by the gNB via RRC signaling. This allows the gNB to deterministically set the timing grid for each periodicity value. Such control is beneficial, for example, to align SSB transmissions with specific neighbour cell patterns for measurement purposes, or potentially align them with Cell DTX/DRX active/inactive periods configured for the SCell. Configuring the offset for each potential periodicity allows the gNB to manage these alignments predictably when switching occurs.

Proposal 3: Support RRC configuration of a time offset parameter associated with each configured SSB burst periodicity for an SCell. The UE uses the configured offset corresponding to the DCI-indicated periodicity to determine the exact timing occasions for SSB burst transmission starting from the first valid occasion no earlier than slot m+d.

In connected mode, when CD-SSB and NCD-SSB (with SSB adaptation) are present, RRM measurement on the cell can be based on CD-SSB or NCD-SSB or both similar to on-demand SSB discussion. For a cell supporting NCD SSB with CD-SSB periodically transmitted on the cell, a CSI report configuration can be associated with both of NCD-SSB and CD-SSB. Alternatively, a CSI report configuration is associated with one of CD-SSB and NCD-SSB. The content of CSI measurement reporting can be configurable by the network.

Proposal 4: Support CSI report configuration associated with both of CD-SSB and NCD-SSB (with SSB adaptation) or one of CD-SSB and NCD-SSB based on configuration.

## PRACH Adaptation

### Additional PRACH Configuration

Agreement

For adaptation of PRACH in time-domain, at least for 4-step RACH, at least for DCI 1_0 with P-RNTI,

- Support configuration of the additional PRACH resources within [the same] RACH-ConfigCommon in SIB1 used to configure the legacy PRACH resources

  - the legacy PRACH resources used for ‘additional RO validation before the SSB-RO mapping’ are configured in the RACH-ConfigCommon

RAN1 previously agreed that the configuration for additional PRACH resources (introduced for PRACH adaptation in time-domain) resides within the RACH-ConfigCommon IE, alongside the configuration for legacy PRACH resources. While this integration simplifies the overall structure, RACH-ConfigCommon contains various parameters beyond the basic PRACH occasion definition. One such parameter is featureCombinationPreamblesList, which allows the network to partition and reserve specific PRACH preambles for UEs supporting certain features (e.g., RedCap, SDT, Slicing). A point requiring clarification is whether this parameter, when configured within RACH-ConfigCommon, should automatically apply identically to both the legacy PRACH resources and the newly defined additional PRACH resources, or if separate applicability is needed.

While some parameters within RACH-ConfigCommon, such as rsrp-ThresholdSSB, might reasonably apply commonly to both legacy and additional resources, applying featureCombinationPreamblesList uniformly might be overly restrictive. This list defines the number of preambles reserved for specific features based on the anticipated UE population supporting those features. It's plausible that the distribution of specific features among the legacy UE population (primarily using legacy ROs) could differ significantly from the distribution among NES-capable UEs targeted by the additional PRACH resources. For instance, the proportion of RedCap UEs accessing additional ROs might be different from those accessing legacy ROs. Therefore, the network should have the flexibility to configure different preamble partitioning schemes (i.e., different values within featureCombinationPreamblesList) tailored specifically to the expected usage of the additional PRACH resources, separate from the partitioning applied to legacy resources.

Proposal 5: Support separate configuration of featureCombinationPreamblesList for the additional PRACH resources configured within RACH-ConfigCommon.

### Validity for PRACH adaptation

When PRACH resources are indicated as available, there should be a mechanism based on which UE can determine when the additional PRACH resources are no longer considered available. A timer-based mechanism can be used for this purpose rather than using separate deactivation DCI signaling because network would not be able to reliably determine if all the camped UEs have received the deactivation indication or not.

Following agreement was reached in RAN1#120 for validity timer [2].

Agreement

For DCI-based adaptation for additional PRACH resources, DCI 1_0 with P-RNTI indicates the availability information for additional PRACH resource from a reference point and for a validity time duration

- FFS: Validity time duration for availability is configured by higher layer signaling or predefined

- FFS: Location of the reference point defined in the specification

- FFS: Value/granularity of the validity time duration.

- FFS: Whether DCI can be used to explicitly deactivate the additional PRACH resources

Further RAN1 concluded on following agreement in RAN1#120bis for validity timer [1].

Agreement

For DCI-based adaptation for additional PRACH resources, for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI

- the validity duration is configured via higher layer signalling.

It was also agreed in RAN1#120 that DCI-based adaptation will not indicate the subset of the additional PRACH resources for availability [2].

Agreement

For DCI-based adaptation for additional PRACH resources, support optional semi-static signalling of a single PRACH mask to identify the subset of the additional PRACH resources

- The mask is applicable at unit of

  - Alt 1: PRACH association period

  - Alt 2: PRACH association pattern period

  - Alt 3: SFN level

- The PRACH association period is determined based on valid additional ROs only.

- The mask is applied after valid RO determination and SSB-RO mapping.

- Note: The existing behaviour in TS 38.213 "An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH block indexes repeats at most every 160 msec." is not impacted due to application of the mask.

- This is applicable at least for adaptation for DCI 1_0 with P-RNTI

- The DCI does not indicate PRACH mask selection

- FFS: how the mask is identified

  - Option 1: The PRACH mask is from a PRACH mask table

    - Pre-defined table with N=[4 or 8 or 16] rows

    - The semi-static signalling indicates a PRACH mask index

  - Option 2: The PRACH mask is based on configuration parameters e.g. bitmap at SFN-level, periodic time domain window, …

To optimize PRACH resource utilization, it is advantageous for the network to dynamically control the availability of additional PRACH resources using PRACH indication. This allows the network to adapt to anticipated load variations. For example, during periods of high expected PRACH load, the network can indicate that additional PRACH resources are available for an extended duration, ensuring sufficient access. Conversely, when the expected PRACH load is low, the network can signal a shorter availability duration, minimizing resource allocation overhead.

Observation 1: High network energy savings can be achieved if DCI-based adaptation can control the activated PRACH resources depending on the cell load.

To implement DCI-based adaptation for controlling available PRACH resources, we propose that the network include the validity time duration within the PRACH adaptation DCI indication. Specifically, the cell could configure a set of multiple PRACH validity duration values within the PRACH configuration. The DCI indication would then select one of these pre-configured values to be applied, allowing for dynamic adjustment of resource availability.

Proposal 6: Support gNB to configure one or more values of PRACH validity duration for additional PRACH resources. One of the PRACH validity duration is indicated by the DCI indication for PRACH adaptation which should be used by UE to determine the available PRACH resources.

### PDCCH order indication for PRACH adaptation

RAN1 also reached an agreement where PDCCH order can indicate the availability of additional PRACH resources for connected mode UEs [1][2].

Agreement

For adaption of PRACH in time-domain, for a connected mode UE, support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH.

- FFS: UE behaviour (e.g. applicable resources for PRACH mask index) when it is indicated of additional PRACH resource(s)

- FFS: Details on how to reuse existing bit for the 1-bit indication

Agreement

For adaptation of PRACH in time-domain, for a connected mode UE,

- One of the reserved bits of PDCCH order (DCI 1_0 with C-RNTI) is used for the new DCI field that indicates the availability of additional PRACH resources.

There are two open points left for further discussion. The first point is the UE behavior for selecting and performing PRACH after receiving the PDCCH order which indicates availability of additional PRACH resources. For PDCCH order indicating contention-free RACH, we think that network is likely to assign contention-free resources either for legacy PRACH resources or additional PRACH resources but not both. Hence, if network indicates availability of additional PRACH resources it would mainly be intended to request the UE to initiate RACH on additional PRACH resources (where contention-free RACH may be available). Thereby,

    - If additional PRACH resources are not indicated as available, then UE selects the legacy PRACH resources for PRACH transmission

    - If additional PRACH resources are indicated as available, then UE selects the additional PRACH resources for PRACH transmission

Proposal 7: If UE receives a PDCCH order and the PDCCH order can indicate availability of additional PRACH resources, the UE selects the PRACH resources for PRACH transmission according to

    - If additional PRACH resources are not indicated as available, then UE selects the legacy PRACH resources for PRACH transmission

    - If additional PRACH resources are indicated as available, then UE selects the additional PRACH resources for PRACH transmission

The PDCCH order includes specific details regarding the PRACH resources for PRACH transmission, such as the Random-Access Preamble index and PRACH Mask index. As previously established, the UE should select either legacy or additional PRACH resources based on the availability indication. Therefore, the Random-Access Preamble index and PRACH Mask index provided in the PDCCH order must be interpreted as applicable to the PRACH resource ultimately selected by the UE for PRACH transmission.

Proposal 8: UE applies the values of Random Access Preamble index and PRACH Mask index to the PRACH resource type (additional vs legacy) selected by UE for PRACH transmission.

# Conclusion

From the discussion, we have the following proposals:

Proposal 1: For determining PRACH occasion validity, valid PDCCH monitoring occasions, and PDSCH rate matching around SSBs, the UE shall use the lowest  SSB burst periodicity value among all configured SSB burst periodicities for the SCell, irrespective of the dynamically indicated SSB burst periodicity via DCI.

Proposal 2: It is recommended that the SSB burst periodicity configured via legacy signalling (e.g., in ServingCellConfigCommon) corresponds to the lowest SSB burst periodicity intended for the SCell operation.

Proposal 3: Support RRC configuration of a time offset parameter associated with each configured SSB burst periodicity for an SCell. The UE uses the configured offset corresponding to the DCI-indicated periodicity to determine the exact timing occasions for SSB burst transmission starting from the first valid occasion no earlier than slot m+d.

Proposal 4: Support CSI report configuration associated with both of CD-SSB and NCD-SSB (with SSB adaptation) or one of CD-SSB and NCD-SSB based on configuration.

Proposal 5: Support separate configuration of featureCombinationPreamblesList for the additional PRACH resources configured within RACH-ConfigCommon.

Proposal 6: Support gNB to configure one or more values of PRACH validity duration for additional PRACH resources. One of the PRACH validity duration is indicated by the DCI indication for PRACH adaptation which should be used by UE to determine the available PRACH resources.

Proposal 7: If UE receives a PDCCH order and the PDCCH order can indicate availability of additional PRACH resources, the UE selects the PRACH resources for PRACH transmission according to

    - If additional PRACH resources are not indicated as available, then UE selects the legacy PRACH resources for PRACH transmission

    - If additional PRACH resources are indicated as available, then UE selects the additional PRACH resources for PRACH transmission

Proposal 8: UE applies the values of Random Access Preamble index and PRACH Mask index to the PRACH resource type (additional vs legacy) selected by UE for PRACH transmission.

# References

[1] Chairman's notes RAN1#120bis

[2] Chairman's notes RAN1#120

[3] Chairman's notes RAN1#118