3GPP TSG RAN WG1 #121	     R1-2503563

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.3.1

Source:	Samsung

Title:	SBFD operation and procedures

Document for:	Discussion

1. Introduction

RAN approved the updated WID [1] including the following objectives.

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - […] - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol - Followings are assumed based on TR 38.858 - SBFD at the gNB side - Half duplex operation at the UE side - FR1 and FR2-1 - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators - […]

In this contribution, we discuss remaining issues on semi-static indication of subbands and UE’s transmission, reception and measurement behaviours in SBFD symbol and non-SBFD symbols.

# 2. Remaining Issues on SBFD Operations

  - 2.1. Remaining Issus on Semi-static indication of SBFD subbands

    - 2.1.1. Frequency domain indication of SBFD subbands

UE specific configuration on frequency location of SBFD subbands

The remaining issue is whether to support UE-specific frequency domain location of subbands. As shown in the agreement below, RAN1 agreed to support cell-specific configuration on time domain location, but the cell-specific frequency domain indication is still working assumption.

Agreement For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier. - FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands

From RAN4’s study and Section 9 of TR38.858, the SBFD operation requires additional components like RF/IF subband filtering or analog filter to avoid RX blocking. These components should be tuned offline for a specific UL subband configuration. Therefore, the frequency configuration of subbands should be cell-specific or gNB-specific, not UE-specific.

One argument of benefit of the UE-specific frequency configuration of subband is to reduce inter-UE CLI. It is because the UE-specific frequency configuration of subbands can provide larger guardband compared to the guardband of cell-specific frequency configuration of subbands. However, the guardband between two UEs (TX UE and RX UE) can be implicitly created by gNB’s scheduling. Since RAN1 agreed to support L1-based CLI measurement and reporting, if the inter-UE CLI between two UE is severe, gNB will not schedule the two UEs in the same symbols. Therefore, without the UE-specific frequency configuration of subbands, gNB can avoid strong inter-UE CLI.

Proposal 1. Confirm the cell-specific configuration on frequency location of SBFD subbands.

- UE-specific frequency domain location of SBFD subbands is not necessary

    - 2.1.2. Time domain indication of SBFD subbands

Explicit Guard Symbols for the transient period

RAN1 has sent a LS to RAN1 to inform their agreements on the transient period between SBFD symbols and non-SBFD symbols. Our view is shown in R1-2503543 [2].

  - 2.2. Remaining Issues on UE Behaviour and Procedures

    - 2.2.1. Remaining issues on CSI and Others

CSI-RS resource across two DL subbands

For CSI-RS resource across two DL subband, the following was agreed in RAN1#118bis.

Agreement For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.

The associated timeline for CSI-RS across two DL subbands was agreed as above. Apart from CSI processing timeline, the non-contiguous CSI-RS in frequency may also have impact to the counting of active resources and active ports. Non-contiguous CSI-RS have two chunks of consecutive PRBs for channel estimation which is doubled in comparison with legacy. Hence, it is good for RAN1 to further discuss whether to change the counting rule for non-contiguous CSI-RS in frequency (e.g., double the number of resource counts and port counts for the CSI-RS across two DL subbands).

Proposal 2. In case of CSI-RS across the two DL subbands, consider to double the counting of active resources and active ports for the CSI-RS.

Another issue for CSI-RS across the two DL subbands is the associated minimum bandwidth. In the legacy, the bandwidth of CSI-RS within DL BWP is equal or greater than min⁡(24,NBWPsize). In case of a CSI-RS across the two DL subbands, similar restriction should be defined with extension. In case of a CSI-RS across the two DL subbands, the frequency domain resources of the CSI-RS consists of 2 parts in different DL subbands. the number of PRBs for each of the 2 parts is ≥ min⁡(24,NDL-SBsize), where NDL-SBsize is the number of PRBs of DL subband which includes the associated part of the CSI-RS.

Proposal 3. In case of a CSI-RS across the two DL subbands, the frequency domain resources of the CSI-RS consists of 2 parts in different DL subbands.

- At least for CSI acquisition, the number of PRBs for each of the 2 parts is ≥ min⁡(24,NDL-SBsize), where NDL-SBsize is the number of PRBs of DL subband which includes the associated part of the CSI-RS

Clarification on TS38.214 specification for CSI Processing timeline

The endorsed CR for TS38.214 included the following texts.

TS 38.214 #### 5.4 UE CSI computation time <omitted texts> For a CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations, provided by csi-ReportSubConfigToAddModList, only (Z2,Z2') of table 5.4-2 is applicable. If a UE is configured with SBFD symbols, the UE may expect Z and Z' to be scaled by factor of 2 if CSI-RS overlaps with two DL subbands in SBFD symbol(s). <omitted texts>

However, the description of Z timeline scaling is incorrect since the current wording scales the outcome of multiple CSI reports. In fact, only the Z timeline (i.e., (Z(m),Z'(m))) for the CSI report associated with CSI-RS overlaps with two DL subbands should be scaled. In addition, it should be clarified that the CSI-RS resource is for channel measurement and within active DL BWP. Hence, we suggest to adopt the following TP.

Proposal 4. Adopt the following text proposal for TS38.214

TS 38.214 #### 5.4 UE CSI computation time <omitted texts> For a CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations, provided by csi-ReportSubConfigToAddModList, only (Z2,Z2') of table 5.4-2 is applicable. If a UE is configured with SBFD symbols, the UE may expect Z and Z' to be scaled by factor of 2 and if a CSI report corresponding to a CSI-ReportConfig is associated with CSI-RS resource(s) for channel measurement overlaps with two DL subbands in active DL BWP in SBFD symbol(s), the (Z(m),Z'(m)) corresponding to the CSI report is scaled by a factor of 2. <omitted texts>

Determination on valid slot for CSI reference resource

Agreement For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

In RAN#118bis, the valid symbol type for CSI derivation was agreed. A follow-up issue is how to determine the CSI reference resource for the associated CSI report. In TS38.214, the definition of a CSI reference resource for a given CSI report is as follows.

TS38.214 #### 5.2.2.5 CSI reference resource definition The CSI reference resource for a serving cell is defined as follows: - In the frequency domain, the CSI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CSI relates. - In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6 TS 38.213], and where μKoffsetis the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1 and for FR2-NTN, - where +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL and and are the subcarrier spacing configurations for DL and UL, respectively, and Nslot, offsetCA and are determined by higher-layer configured ca-SlotOffset for the cells transmitting the uplink and downlink, as defined in clause 4.5 of [4, TS 38.211] - where for periodic and semi-persistent CSI reporting - if a single CSI-RS/SSB resource is configured for channel measurement nCSI_ref is the smallest value greater than or equal to 4⋅2µDL, such that it corresponds to a valid downlink slot, or - if multiple CSI-RS/SSB resources are configured for channel measurement nCSI_ref is the smallest value greater than or equal to , such that it corresponds to a valid downlink slot. - where for aperiodic CSI reporting, if the UE is indicated by the DCI to report CSI in the same slot as the CSI request, nCSI_ref is such that the reference resource is in the same valid downlink slot as the corresponding CSI request, otherwise nCSI_ref is the smallest value greater than or equal to , such that slot n- nCSI_ref corresponds to a valid downlink slot, where Z' corresponds to the delay requirement as defined in Clause 5.4. - when periodic or semi-persistent CSI-RS/CSI-IM or SSB is used for channel/interference measurements, the UE is not expected to measure channel/interference on the CSI-RS/CSI-IM/SSB whose last OFDM symbol is received up to Z' symbols before transmission time of the first OFDM symbol of the aperiodic CSI reporting. A slot in a serving cell shall be considered to be a valid downlink slot if: - it comprises at least one higher layer configured downlink or flexible symbol, and - it does not fall within a configured measurement gap for that UE If there is no valid downlink slot for the CSI reference resource corresponding to a CSI Report Setting in a serving cell, CSI reporting is omitted for the serving cell in uplink slot n'.

Note that the valid downlink slot is used to identify the slot where the associated CSI derivation is based on. However, the existing valid slot has no differentiation between SBFD symbol and non-SBFD symbol which is incorrect for the CSI derivation for a particular the valid symbol type (i.e., SBFD symbol or non-SBFD symbol). Hence, we have the following proposal to re-define the valid downlink slot associated with CSI reference resource in this case.

Proposal 5. If the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is configured, the valid downlink slot associated with CSI reference resource for the CSI report is defined as follows:

- A slot comprises at least one higher layer configured downlink or flexible symbol, where the symbol is with the same symbol type as the configured valid symbol type; and

- The slot does not fall within a configured measurement gap for that UE

Proposal 6. Adopt the following text proposal for TS38.214

TS38.214 #### 5.2.2.5 CSI reference resource definition The CSI reference resource for a serving cell is defined as follows: - In the frequency domain, the CSI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CSI relates. - In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6 TS 38.213], and where μKoffsetis the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1 and for FR2-NTN, - where +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL and and are the subcarrier spacing configurations for DL and UL, respectively, and Nslot, offsetCA and are determined by higher-layer configured ca-SlotOffset for the cells transmitting the uplink and downlink, as defined in clause 4.5 of [4, TS 38.211] - where for periodic and semi-persistent CSI reporting - if a single CSI-RS/SSB resource is configured for channel measurement nCSI_ref is the smallest value greater than or equal to 4⋅2µDL, such that it corresponds to a valid downlink slot, or - if multiple CSI-RS/SSB resources are configured for channel measurement nCSI_ref is the smallest value greater than or equal to , such that it corresponds to a valid downlink slot. - where for aperiodic CSI reporting, if the UE is indicated by the DCI to report CSI in the same slot as the CSI request, nCSI_ref is such that the reference resource is in the same valid downlink slot as the corresponding CSI request, otherwise nCSI_ref is the smallest value greater than or equal to , such that slot n- nCSI_ref corresponds to a valid downlink slot, where Z' corresponds to the delay requirement as defined in Clause 5.4. - when periodic or semi-persistent CSI-RS/CSI-IM or SSB is used for channel/interference measurements, the UE is not expected to measure channel/interference on the CSI-RS/CSI-IM/SSB whose last OFDM symbol is received up to Z' symbols before transmission time of the first OFDM symbol of the aperiodic CSI reporting. A slot in a serving cell shall be considered to be a valid downlink slot if: - it comprises at least one higher layer configured downlink or flexible symbol, and - it comprises at least one of either SBFD symbol(s) or non-SBFD symbol(s) as indicated by symbolType if applicable, and - it does not fall within a configured measurement gap for that UE If there is no valid downlink slot for the CSI reference resource corresponding to a CSI Report Setting in a serving cell, CSI reporting is omitted for the serving cell in uplink slot n'.

Currently, if timeRestrictionForChannelMeasurements or timeRestrictionForInterferenceMeasurements in CSI-ReportConfig is set to "Configured", the latest measurement occasion of the resource for channel measurement and/or interference measurement is used for CSI computation. However, if the valid symbol type for CSI derivation is configured, the latest occasion of the associated periodic/semi-persistent should be within the symbols with the same symbol type as the configured valid symbol type for CSI derivation.

Proposal 7. If the timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured", and if the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is configured,

- The most recent occasion of the periodic/semi-persistent CSI-RS resource for channel measurement for CSI derivation is within the symbols with the same symbol type as the configured valid symbol type

Proposal 8. If the timeRestrictionForInterferenceMeasurements in CSI-ReportConfig is set to "Configured", and if the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is configured,

- The most recent occasion of periodic/semi-persistent CSI-IM and/or NZP CSI-RS for interference measurement for CSI derivation is within the symbols with the same symbol type as the configured valid symbol type

Proposal 9. Adopt the following text proposal for TS38.214

TS 38.214 #### 5.2.2.1 Channel quality indicator (CQI) <omitted texts> If the higher layer parameter timeRestrictionForChannelMeasurements is set to "notConfigured", the UE shall derive the channel measurements for computing CSI value reported in uplink slot n based on only the NZP CSI-RS, no later than the CSI reference resource, (defined in [4, TS 38.211]) associated with the CSI resource setting. If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured", the UE shall derive the channel measurements for computing CSI reported in uplink slot n based on only the most recent, no later than the CSI reference resource, in cell DTX active period of a serving cell if cell DTX is activated or in either SBFD symbol(s) or non-SBFD symbol(s) as indicated by symbolType for semi-persistent or periodic CSI-RS, occasion of NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting on the serving cell. If the higher layer parameter timeRestrictionForInterferenceMeasurements is set to "notConfigured", the UE shall derive the interference measurements for computing CSI value reported in uplink slot n based on only the CSI-IM and/or NZP CSI-RS for interference measurement no later than the CSI reference resource associated with the CSI resource setting. If the higher layer parameter timeRestrictionForInterferenceMeasurements in CSI-ReportConfig is set to "Configured", the UE shall derive the interference measurements for computing the CSI value reported in uplink slot n based on the most recent, no later than the CSI reference resource, in cell DTX active period of a serving cell if cell DTX is activated or in either SBFD symbol(s) or non-SBFD symbol(s) as indicated by symbolType for semi-persistent or periodic CSI-RS, occasion of CSI-IM and/or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) associated with the CSI resource setting on the serving cell. <omitted texts> #### 5.2.1.4.3 L1-RSRP Reporting <omitted texts> If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "notConfigured", the UE shall derive the channel measurements for computing L1-RSRP value reported in uplink slot n based on only the SS/PBCH or NZP CSI-RS, no later than the CSI reference resource, (defined in [4, TS 38.211]) associated with the CSI resource setting. If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured", the UE shall derive the channel measurements for computing L1-RSRP reported in uplink slot n based on only the most recent, no later than the CSI reference resource, in either SBFD symbol(s) or non-SBFD symbol(s) as indicated by symbolType for semi-persistent or periodic CSI-RS, occasion of SS/PBCH or NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting. <omitted texts>

In the existing TS38.214, the CSI report for a CSI-ReportConfig is transmitted only if at least one measurement occasion of RS is received. For CSI-ReportConfig configured with symbolType, only measurement occasion in valid symbol type can be used. Hence, the transmission condition of the CSI report should be one measurement occasion of RS within valid symbols indicated by symbolType. Hence, we propose the corresponding TP as follows.

Proposal 10. Adopt the following text proposal for TS38.214

TS 38.214 #### 5.2.2.5 CSI reference resource definition <omitted texts> For a CSI report with reportQuantity not set to ‘ssb-Index-RSRP’, ‘ssb-Index-SINR’, ‘ssb-Index-RSRP-Index’ or ‘ssb- Index-SINR-Index’, after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement no later than CSI reference resource and drops the report otherwise. For a CSI report configuration containing a list of sub-configurations provided by csi- ReportSubConfigToAddModList, after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report including one or more sub-reports only after receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement, per sub-configuration, no later than CSI reference resource and drops the report otherwise, where the sub-configuration is the activated/triggered one for AP/SP-CSI reporting, or the configured one for P-CSI reporting. For a CSI-ReportConfig configured with two Resource Groups and 𝑁 Resource Pairs for channel measurement in the corresponding CSI-RS Resource Set, as described in clause 5.2.1.4.1, after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each of the CSI-RS resources in the corresponding CSI-RS Resource Set for channel measurement and one CSI-IM occasion for each of the CSI-IM resources in the corresponding Resource Set for interference measurement no later than the CSI reference resource and within the same DRX Active Time when DRX is configured, and drops the report otherwise. For a CSI-ReportConfig configured with codebookType set to 'typeII-CJT-r18' or 'typeII-CJT-PortSelection-r18', after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each of the CSI-RS resources in the corresponding CSI-RS Resource Set for channel measurement and one CSI-RS and/or CSI-IM resource transmission occasion for the CSI-RS and/or CSI-IM resource in the corresponding Resource Set for interference measurement no later than the CSI reference resource and within the same DRX Active Time, when DRX is configured, and drops the report otherwise. For a CSI-ReportConfig configured with codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18', after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only if receiving at least one aperiodic or 𝐾𝑝 periodic or semipersistent consecutive CSI-RS transmission occasions for each CSI-RS resource in the corresponding CSI-RS Resource Set for channel measurement and one CSI-RS and/or CSI-IM resource transmission occasion for the CSI-RS and/or CSI-IM resource in the corresponding Resource Set for interference measurement no later than the CSI reference resource and within the same DRX Active Time, when DRX is configured, and drops the report otherwise. The value of 𝐾𝑝∈{1,2,4} is indicated by UE capability, as defined in clause 5.2.1.6. For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'tdcp', after the CSI report (re)configuration, serving cell activation, BWP change, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each CSI-RS resource in the 𝐾𝑇𝑅𝑆 CSI-RS Resource Sets of the CSI-RS Resource Setting for channel measurement no later than the CSI reference resource within the same DRX active time, when DRX is configured, and drop the report otherwise. For a CSI-ReportConfig configured with symbolType, after the CSI report (re)configuration, serving cell activation, BWP change, the UE reports a CSI report only after receiving at least one semi-persistent or periodic CSI-RS transmission occasion for each of the CSI-RS resources in the corresponding CSI-RS Resource Set for channel measurement and/or one CSI-RS and/or CSI-IM resource transmission occasion for each of the CSI-RS and/or CSI-IM resource in the corresponding Resource Set for interference measurement, no later than the CSI reference resource and within either SBFD symbol(s) or non-SBFD symbol(s) as indicated by symbolType, and drops the report otherwise. <omitted texts>

Impact on associated CSI-RS for non-codebook based PUSCH transmission

As agreed in RAN1#119, separate associated CSI-RS can be used for different symbol types (SBFD symbols and non-SBFD symbols respectively). For non-codebook based transmission, the UE can calculate the precoder used for the transmission of SRS based on measurement of an associated NZP CSI-RS resource.

TS38.214-i50 6.1.1.2 Non-Codebook based UL transmission For non-codebook based transmission, the UE can calculate the precoder used for the transmission of SRS based on measurement of an associated NZP CSI-RS resource.

In case of associated CSI-RS is periodic or semi-persistent, the transmission occasion of the associated CSI-RS may be fall within both SBFD symbols or non-SBFD symbols. To avoid miscalculation of SRS precoder, UE should calculate the precoder used for the transmission of SRS based on measurement of CSI-RS in the same symbol type as for the corresponding SRS-ResourceSet. For example, if the SRS-ResourceSet is for SBFD symbols, the measurement of the corresponding associated CSI-RS in SBFD symbols are used for precoder calculation. If the SRS-ResourceSet is for non-SBFD symbols, the measurement of the corresponding associated CSI-RS in non-SBFD symbols are used for precoder calculation.

Proposal 11. For non-codebook based PUSCH transmission, in case of a periodic or semi-persistent associated CSI-RS for an SRS-ResourceSet

- UE calculate the precoder used for the transmission of SRS based on measurement of CSI-RS in the same symbol type as the symbol type for the SRS-ResourceSet.

Aperiodic associated CSI-RS can be used for aperiodic SRS. The time domain position of aperiodic CSI-RS reception is coupled with triggering DCI for PUSCH. Based on the description in the latest TS38.214, the aperiodic CSI-RS is located in the same slot as the corresponding SRS request field (which is the same slot as the triggering DCI).

TS38.214-i50 6.1.1.2 Non-Codebook based UL transmission If the UE configured with aperiodic SRS associated with aperiodic NZP CSI-RS resource, the presence of the associated CSI-RS is indicated by the SRS request field if the value of the SRS request field is not '00' as in Table 7.3.1.1.2-24 of [5, TS 38.212] and if the scheduling DCI is not used for cross carrier or cross bandwidth part scheduling. … The CSI-RS is located in the same slot as the SRS request field. If the UE configured with aperiodic SRS associated with aperiodic NZP CSI-RS resource, any of the TCI states configured in the scheduled CC shall not be configured with qcl-Type set to 'typeD'.

In case of the operation of configuration 1 and configuration 2, the symbol type of triggering DCI and the symbol type of the corresponding SRS and PUSCH could be different. This is problematic since if the triggering DCI is in the symbol type (SBFD symbols or non-SBFD symbols) which is not the same as the corresponding SRS and PUSCH, the precoder of the SRS and PUSCH cannot be calculated. In particular, for configuration 2, if the legacy rule is followed, two associated CSI-RS are needed to be transmitted at the same slot with the triggering DCI. This means that at least one of the associated CSI-RS is of the different symbol from the symbol type of the corresponding SRS and PUSCH.

To solve this issue, one solution is to change the slot for aperiodic associated CSI-RS. For example, an aperiodic associated CSI-RS for an SRS-ResourceSet is located at the first/earliest available slot which is no earlier than the slot with the scheduling DCI. Here, a slot is defined as an available slot if the symbols of the CSI-RS in the slot are of the same symbol type as the symbol type for the SRS-ResourceSet. In this way, aperiodic CSI-RS can be located in the same symbol type as the symbol type for the corresponding SRS-ResourceSet.

Proposal 12. For non-codebook based PUSCH transmission, an aperiodic associated CSI-RS for an SRS-ResourceSet is located at the first available slot which is no earlier than the slot with the scheduling DCI.

- A slot is defined as an available slot if the symbols of the CSI-RS in the slot are of the same symbol type as the symbol type for the SRS-ResourceSet.

Wideband PRG of PDSCH

In the last RAN1#118 meeting, RAN1 concluded that wideband precoder is only supported for contiguous PRBs as follow:

Conclusion If PRG is determined as wideband, UE does not expect to be scheduled with non-contiguous PRBs in SBFD symbols.

In case of prb-BundlingType=staticBundling, n2, n4 or wideband can be configured. If ‘wideband’ is configured, as per the conclusion, only one DL subband can be utilized for PDSCH reception. In other words, considering D-U-D SBFD configuration with 20MHz UL subband and 40MHz DL subband, the peak DL throughput is severely degraded when ‘wideband’ is configured (less than half peak DL throughput). To avoid such a degradation, ‘n2’ may be used when the scheduled PRB is non-contiguous as a fallback PRG granularity. Note that the PRG size of a PDSCH scheduled by DCI format 1_0 is always 2.

In case of prb-BundlingType=dynamicBundling, 1-bit PRB bundling size indicator in a DCI format may indicate PRG size. For example, if ‘0’ is indicated, the PRG size is determined from ‘bundleSizeSet2’. Otherwise, the PRG size is determined from ‘bundleSizeSet1’ If ‘bundleSizeSet1’ includes ‘n2-wideband’ or ‘n4-wideband’, the PRG size is further determined by checking the number of scheduled PRBs exceeds half of DL BWP size. That is, wideband PRG is determined only if the number of scheduled PRBs exceeds half of DL BWP. Consider the case where DL BWP contains NBWP=273 PRBs and each of two DL subbands contains NDLSB=106 PRBs (51 PRBs for UL subband and 5 RBs for each of two guardbands). Then, even if all of PRBs in one DL subband are scheduled in a PDSCH, the PRG granularity cannot be wideband since the # of scheduled contiguous PRBs < NBWP/2. To resolve this issue, RAN1 may consider the DL subband size, instead of DL BWP size, when determining PRG size.

Proposal 13. In case of prb-BundlingType=staticBundling and bundleSize=wideband, UE may assume the PRG size is equal to 2 when the scheduled PRBs overlap with two DL subbands in SBFD symbol.

Proposal 14. In case of prb-BundlingType=dynamicBundling and bundleSizeSet1= n2-wideband or n4-wideband, if PRB bundling size indicator field indicates ‘1’, the DL subband size can be used for PRG size determination, instead of DL BWP size.

Whether to count rate matching capability

It was agreed that the assigned PRBs within DL usable PRBs are valid for single slot PDSCH scheduled at least by DCI in USS. Also, as a working assumption, the same rule was agreed for SPS PDSCH without repetitions, PDSCH with repetitions, and multi-PDSCH scheduled by a single DCI under Configuration 2.

Agreement For frequency domain resource allocation Type 1 for PDSCH in a single slot scheduled at least by DCI format in USS when PRG is determined as one of the values among {2, 4}, - Option 1-1: Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. Assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping. - Existing RB indexing and VRB-to-PRB mapping are reused - The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only - FFS: DMRS sequence mapping - FFS: when PRG is determined as wideband - FFS: partial PRG Working Assumption For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), - Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid. - For SPS PDSCH, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only - For PDSCH repetitions, FFS whether the number of PRBs for TBS determination is based on assigned PRBs, or based on assigned PRBs within DL usable PRBs only, or based on assigned PRBs within DL usable PRBs of the first repetition. - For multi-PDSCH scheduled by a single DCI, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only. Agreement - For multi-PDSCH scheduled by a single DCI with Configuration 2, the number of PRBs for TBS determination for a PDSCH in SBFD symbols is based on assigned PRBs within DL usable PRBs only. The number of PRBs for TBS determination for a PDSCH in non-SBFD symbols is based on assigned PRBs. Agreement For dynamically scheduled PDSCH repetitions with Configuration 2, for TBS determination. - Option 1: - If the first repetition occasion indicated by scheduling DCI is in SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs. - If the first repetition occasion indicated by scheduling DCI is in non-SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs.

From perspective of the PDSCH resource determination and mapping, the procedure to determine valid PRBs within DL usable PRBs can be considered as a special case of rate-matching. That is, the UL subband may be considered as rate-matching pattern. From the TS38.214, a UE may be configured with up to 4 RateMatchPattern(s) per BWP and up to 4 RateMatchPattern(s) per serving cell. For example, once a UE supports semi-static rate matching, this UE has to support up to 4 rate matching patters per BWP and per cell.

Without increasing UE complexity, the determination of valid PRBs within DL usable PRBs can be counted towards the maximum number of RateMatchPattern(s) per serving cell or the maximum number of RateMatchPattern(s) per BWP. Or, RAN1 may introduce additional UE capability for the determination of valid PRBs within DL usable PRBs and keep the existing up to 4 RateMatchPattern(s) per serving cell or per BWP.

Proposal 15. RAN1 to discuss whether to count the determination of value PRBs within DL usable PRBs towards the maximum number of RateMatchPattern(s) per serving cell or BWP or introduce a separate UE capability.

Type-1 HARQ-ACK codebook

RAN1 has agreed a PDSCH does not received when the PDSCH is mapped to both SBFD symbols and non-SBFD symbols. For Type-1 HARQ-ACK codebook, if a PDSCH candidate overlaps with UL symbols, the PDSCH candidate is removed to reduce unnecessary bits in Type-1 HARQ-ACK codebook. Similarly, if a PDSCH candidate overlaps with both non-SBFD symbols and SBFD symbols, the PDSCH candidates can be excluded.

Proposal 16. For type-1 HARQ-ACK codebook, if a PDSCH candidate overlaps with both SBFD symbols and non-SBFD symbols, the PDSCH candidate is excluded.

    - 2.2.2 Remaining issues on separate UL power control parameters

The following agreement on the support of separate UL power control parameters in SBFD/non-SBFD slots was made in RAN1#119.

Agreement For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, - Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols - New P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively

Support for separate UL power control of PUSCH transmissions in the SBFD UL subband and the legacy UL slot also requires to address the corresponding PCMAX and/or PHR reporting from the SBFD-aware UE.

2.2.2.1 PCMAX

Some companies have motivated the use of separate PCMAX in for the configured maximum UL transmit power in SBFD and non-SBFD slot. The reason is control for inter-UE CLI including the case of adjacent channel CLI and to impose an RRC controllable maximum output power for any UL transmission from the UE in UL subband. Note that RRC can use the signaled p-max to reduce the maximum configured output power from the UE as by Rel-15 NR, but then the maximum output power for the Rel-19 SBFD-aware UE in the normal UL slot must also be reduced. This would heavily penalize the UL throughput of an SBFD-aware UE in the normal UL slot where better receptions can be expected.

PCMAX is determined by the UE as by section 6 in 38.101.

Figure 1: Configured transmitted power (PCMAX) as determined by the UE (section 6.2.4 in 38.101-1)

PCMAX is determined by the UE within PCMAX,L and PCMAX,H bounds. The UE computes PCMAX depending on its supported UE Power Class in the band. RRC signalling can then impose an upper max limit through signaled RRC parameter p-max. The signaled p-max value is accounted for as part of the delta_P_PowerClass parameter. For example, if the network signals p-max = 3dB to the UE, a PC3 (23dBm) UE would then determine 20 dBm as maximum configured output power PCMAX. However, for a PC2 UE (26 dBm), delta_P_PowerClass with the p-max is determined as a function of the UL duty cycle.

There is only one/same single RRC signaled p-max per UE for any type of UL slot on the carrier, i.e, for both SBFD/non-SBFD slots. If the network chooses to set p-max = 6dB to reduce the PCMAX of the PC3 UE in the SBFD UL subband from 23 dBm to 17 dBm, this also means reducing the PCMAX in the normal UL slot.

38.213 uses the UE determined PCMAX value from 38.101 as input.

Figure 2: UL transmit power for PUSCH or PUCCH and PCMAX (section 7 in 38.213)

This PCMAX value is “generic”. For example, PCMAX in 38.213 doesn’t distinguish between effectively resulting different PCMAX values for a PUSCH, PUCCH or SRS transmission by the UE from the 38.213 perspective. For the same UE Power Class, and same signaled p-max, a UE computed PCMAX for PUSCH and a PCMAX for SRS can well result in different values. SRS can allow for different “power allowances” in 38.101 Section 6.2.4. For the same UE Power Class, and same signaled p-max, the UE would typically compute a different PCMAX “per slot” or “per transmission instance”. For different UL signals/channels, e.g., PUSCH or PUCCH in a slot, different resulting PCMAX values often result. One reason is A-MPR resulting in different values for PUCCH often at BWP edge where larger A-MPR for outer RBs are used when compared to PUSCH which is often more centered therefore more located on inner RBs. Also, the A-MPRs depend on the PUSCH modulation order. This effectively forces the UE to apply another PCMAX value based on the PUSCH transmit occasion and depending on the UL grant.

The UE computes different PCMAX values for different slots and for different UL signal/channel types, and a PCMAX value for example can be different for the same channel type, such as between PUSCH in UL slot n and another UL slot n+1 due to modulation order, or the frequency-domain allocation going into A-MPR.

However, RRC doesn’t allow currently to impose a separate max or upper limit for the configured maximum output power in SBFD/non-SBFD slots.

In our view, it is needed and critical to support separate maximum configured output power for SBFD/non-SBFD slots in Rel-19, respectively. We see 2 options,

Option 1: one/same RRC signaled p-max value but use of a new/additional SBFD slot specific delta offset

Option 2: two separate RRC signaled p-max values, one/legacy for non-SBFD, and new/additional for the SBFD slots.

Both Option 1 and 2 impact section 6.2.4 in 38.101 (for FR1) and corresponding updates to sections 7.1, 7.2in 38.213. Both options have a small associated RRC signaling impact. Further discussion/decision on Option 1 or 2 should occur in RAN4 after RAN1 indicates by LS to RAN4 that support for separate p-max is needed.

Proposal 17 Send a LS to RAN4 that separate maximum configured output power for SBFD/non-SBFD slots should be supported, e.g., using Options 1 or 2 as a starting point.

- Option 1: one/same RRC signaled p-max value but use of a new/additional SBFD slot specific delta offset

- Option 2: two separate RRC signaled p-max values, e.g., legacy p-max for non-SBFD, and new/additional for the SBFD slots.

2.2.2.1 PHR

Power headroom reporting is specified in Section 7.7 of 38.213 and Section 5.4.6 of 38.321.

The UE signals the PHR through a MAC-CE. NR supports 3 types of PHR: PHR Type 1 for PUSCH transmission, PHR Type 2 for simultaneous PUSCH and PUCCH transmission in an LTE Cell Group in EN-DC, and PHR Type 3 for SRS transmission.

In case of carrier aggregation, e.g., when no transmission from a UE occurs on an activated SCell, the UE uses a reference power to signal a virtual PHR report. For FR2, the PHR can also contain P-MPR information from the UE for the MPR based mitigation to address MPE. In the cases of CA or DC, a single MAC-CE can contain multiple PHRs. A PHR is per carrier and does not explicitly consider beam-based operation. One reason is that the gNB may control the DL/UL beams used for transmissions/receptions and may determine the gNB-UE beam arrangement corresponding to a certain PHR.

The gNB can configure the PHR from a UE to occur periodically, i.e., timer based. It is possible to configure a prohibit timer to control a minimum time between two PHRs in order to reduce the UL signaling load. The PHR can also be triggered by a UE due to events such as a change in pathloss or due to MPE. The UE can also include a PHR in a PUSCH instead of performing padding.

For PUSCH, the PHR Type 1 provides the power headroom assuming PUSCH-only transmission on the serving cell, e.g., PHR Type 1 is valid for a certain CC assuming that the UE was scheduled a PUSCH transmission during a certain duration. The UE includes the PHR and the corresponding value of configured UE maximum output power PCMAX in the PHR MAC CE. The value of PCMAX is configured by the gNB and therefore is in principle known to the gNB. A reason for the UE to include the value of PCMAX in the PHR is that ambiguity can exist in some cases. For example, a separate value for PCMAX can be configured for the NUL carrier and the SUL carrier. Even if both carriers belong to the same cell, e.g., are both associated with the same DL carrier, the gNB needs to know for which UL carrier the UE provides the PHR. Power headroom is a measure of the difference between PCMAX and the UE transmit power that the UE would have used assuming no upper limit on the transmit power. Power headroom can therefore also be a negative value, indicating that the UE transmit power on the carrier was limited by PCMAX at the time of the PHR computation by the UE. For example, a negative PHR value may result when the gNB has scheduled a higher data rate than the UE could support for the available transmission power. The gNB knows the MCS and resource allocation used for a PUSCH transmission by the UE in the time duration corresponding to the PHR. The gNB can determine appropriate combinations of MCS and resource allocation assuming that the DL path loss observed by the UE remains constant. PHR Type-1 can also be reported when there is no actual PUSCH transmission using an assumed or virtual reference transmission format for a PUSCH.

For Rel-19 SBFD, support of separate UL power control for DG or CG based PUSCH transmissions in the SBFD UL subband and the legacy UL slot then also means that the UE should report PHR separately for the SBFD and non-SBFD slots. Different open-loop parameterization is typically expected in SBFD/non-SBFD slots, and the two separate closed-loop PC components separately track and set the UL Tx power in the SBFD slots and the non-SBFD slot.

PHR reporting as by Rel-18 NR does not allow to distinguish between PUSCH transmissions occurring in the SBFD UL subband and the legacy UL slot. It is not possible for the network to obtain a reported PHR value separately for the PUSCH transmissions occurring in the SBFD UL subband and PUSCH transmission in the legacy UL slot. For SBFD configuration 1, PHR reporting then loses its purpose. The gNB should know the PHR value of the PUSCH transmissions in the SBFD UL subband which often require a different MCS setting and often use smaller UL scheduling BW than PUSCH transmissions from the UE in the legacy UL slot.

We think that the introduction of support for separate PHR Type 1 reporting in SBFD and non-SBFD slots can be supported with only small impacts. NR already allows that the MAC-CE contains multiple PHRs from the UE, e.g., in the cases of CA or DC. This existing functionality can be extended to allow separate PHR per SBFD/non-SBFD slot type. The case of PHR Type 3 reporting, SRS, should be discussed/decided as part of the supported SRS configurations in SBFD/non-SBFD slots.

Proposal 18 For the single TRP scenario, for separate UL power control of PUSCH transmissions in SBFD symbols and non-SBFD symbols, support separate PHR Type 1 reporting.

    - 2.2.3 Remaining Issues on collision Handling

The following proposal was discussed in RAN1#120 meeting regarding the collision handling of overlapping channels.

Proposed Agreement: For collision handling in SBFD symbols for SBFD aware UEs, down select one from the following options - Option 1: - Step 1: Resolving SBFD specific collision between transmissions/receptions and unusable resources (e.g. invalid symbol type for Configuration 1, Tx/Rx occasion mapped to SBFD and non-SBFD symbols etc.) , if any. - Step 2: Resolving the collision as in legacy, if any. - Option 2: - Step 1: Resolving SBFD specific collision between PUCCH with repetitions/PUSCH and unusable resources, if any. - Step 2: Resolving the collision as in legacy, if any. - Step 3: Resolving SBFD specific collision between transmissions/receptions and unusable resources, if any. - Option 3: - Step 1: Resolving the collision as in legacy, if any. - Step 2: Resolving SBFD specific collision between transmissions/receptions and unusable resources, if any.

And, the following analysis has been provided from moderator.

Case 1) Option 1: PUCCH is cancelled in step 1. DG PUSCH can be transmitted Option 2: PUCCH is NOT cancelled in step 1. UCI is multiplexed in DG PUSCH and transmitted Option 3: UCI is multiplexed in DG PUSCH and transmitted Case 2) Option 1: PUCCH is cancelled in step 1. DG PUSCH can be transmitted Option 2: PUCCH is cancelled in step 1. DG PUSCH can be transmitted Option 3: DG PUSCH is canceled in step 1

Among the three options, Option 2 provide the best performance as clarified above and thus is preferred. One issue for Option 1 and PUCCH with repetitions for Option 2 is the timeline issue during HARQ-ACK overriding procedure. It should clarify at what time the collision is resolved, for simplicity, the PUCCH with repetitions in Option 2 can be removed to avoid the timeline issue. We have the following proposal.

Proposal 19 Support the following orders for collision handling (Option 2) with the following update

- Step 1: Resolving SBFD specific collision between PUCCH with repetitions/PUSCH transmissions and unusable resources, if any.

- Step 2: Resolving the collision as in legacy, if any.

- Step 3: Resolving SBFD specific collision between transmissions/receptions and unusable resources, if any.

    - 2.2.4 Multi-Carrier SBFD Operation

In the last RAN1#120 meeting, RAN1 agreed to support SBFD operation on one TDD carrier in multi-carrier scenario.

Agreement Support SBFD operation on one TDD carrier in multi-carrier scenario. - Note: If the TDD carrier for SBFD operation is an Scell, support UE dedicated signaling for cell-specific configuration of time and frequency location of SBFD subbands for the Scell. - FFS whether/how to handle half-duplex CA case

From RAN1’s perspective, no additional specification impacts are expected except for half-duplex CA case. For half-duplex CA case, we briefly summary UE behaviors.

First, UE needs to determine a reference cell for a symbol. The reference cell is determined as the active cell with the smallest cell index among the configured multiple serving cells (in case of the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA). This UE behavior can be used for multi-carrier scenario regardless of SBFD operation. It means, the reference cell can be a TDD carrier with SBFD operation.

And then, UE behaviors have been defined for collision cases. The collision cases and the corresponding UE behaviors are shown in Table 1. The collision cases do not include SBFD symbols. SBFD UL subband can configured on Downlink symbols and flexible symbols and UE can be scheduled with DL reception or UL transmission on SBFD symbol. Thus, with respect to collision handling across cells, SBFD symbols can be considered as flexible symbol. This approach has been used when higher layer-configured DL reception on reference cell overlaps with Uplink symbol on another cell. In this case, Uplink symbol is considered as flexible symbols for the collision handling purpose.

Table 1. Collision cases and UE behaviors for half-duplex CA case

| Reference cell Another cell |  | Downlink | Uplink symbol | Flexible symbol |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Higher layer configured DL | Higher layer configured UL | DCI scheduled DL | DCI scheduled UL |
| Downlink |  |  | Same frequency band: Error case Different frequency band: Assume flexible symbol, Not required to receive higher-layer configured DL Not expect to transmit higher-layer configured UL |  | Downlink flexible |  |  |
| Uplink |  | Same frequency band: Error case Different frequency band: Assume flexible symbol Not required to receive higher-layer configured DL Not expect to transmit higher-layer configured UL |  | Uplink flexible |  |  |  |
| Flexible symbol | Higher-layer configured DL |  | Do not receive DL on another cell |  | Do not receive DL on another cell |  |  |
|  | Higher-layer configured UL | Do not transmit UL on another cell |  | Do not transmit UL on another cell |  |  |  |
|  | DCI scheduled DL |  | Error case |  | Error case |  | Error case |
|  | DCI scheduled UL | Same frequency band: Error case Different frequency band: Transmit UL |  | Same frequency band: Error case Different frequency band: not required to receive higher-layer configured DL |  | Error case |  |

Proposal 20For half-duplex CA case, SBFD symbol is considered as flexible symbol.

For intra-band CA case, if one of the multiple serving cell is configured with SBFD operation, the channel state information in any one of the multiple serving cells will experience dramatic change across SBFD symbols and non-SBFD symbols. This is because the NW-side setting of SBFD symbols could be very different from that of non-SBFD symbols (the same reason to introduce valid symbol type for CSI derivation in CSI-ReportConfig). Hence, it is important to support the RS measurement for CSI derivation (i.e., the CSI-ResourceConfig associated with a CSI-ReportConfig) for a particular valid symbol type in a serving cell which is different from the serving cell for SBFD operation.

Proposal 21. For intra-band CA case, the valid symbol type for CSI derivation can be configured in CSI-ReportConfig, where the CSI-ResourceConfig associated with the CSI-ReportConfig is in a serving cell which is the same or different from the serving cell for the TDD carrier for SBFD operation.

    - 2.2.5 Radio link monitoring

In RAN1#120, a reply LS on CSI-RS measurement with SBFD operation was sent by RAN1 to RAN4 [3].

In the reply LS, RAN1 indicated that Configuration 1 and Configuration 2 from the relevant RAN1#117/118bis/120 agreements are not applicable to CSI-RS resource configuration and CSI-RS based RLM/BFD/CBD measurement.

In the discussions on the reply LS, the following options were identified as starting point for the RLF/BFD/CBD measurement using CSI-RS resources.

(…) RAN1 needs to first determine how a SBFD-aware UE performs RLM/BFD/CBD measurement using CSI-RS resources that can be across SBFD and non-SBFD symbols. The following options can be considered as a starting point for discussion. - Option 1: CSI-RS resources across SBFD and non-SBFD symbols can be used for the same RLM/BFD/CBD measurement - Single radio link quality evaluation/indication using CSI resources that can be in both slot types. - out-of-sync: radio link quality is less than predefined threshold “for all resources [including resources in both SBFD and non-SBFD symbols] in the set of resources for radio link monitoring. - in-sync: radio link quality is better than predefined threshold “for any resource [resource can be in either SBFD symbols or non-SBFD symbols] in the set of resources for radio link monitoring” Fig. 1: RLM/BFD/CBD measurement using CSI-RS resources across SBFD and non-SBFD symbols - Option 2: CSI-RS resources across SBFD and non-SBFD symbols cannot be used for the same RLM/BFD/CBD measurement. - A first radio link quality evaluation/indication using CSI-RS resources in SBFD symbols. - For declaring “out-of-sync/in-sync” on SBFD symbols. - A second radio link quality evaluation/indication using CSI-RS resources in non-SBFD symbols. - For declaring “out-of-sync/in-sync” on non-SBFD symbols. Fig. 2: RLM/BFD/CBD measurement using CSI-RS resources on either SBFD or non-SBFD symbols

Radio link monitoring is specified in Section 5 of 38.213 and radio link failure related actions in section 5.3.10 of 38.331.

Radio link monitoring and/or radio link failure applies to the PCell when in RRC_CONNECTED state. Therefore, the Rel-19 specification impact (if any) when SBFD is introduced needs to considered. SSB or CSI-RS can be used to configure radio link monitoring and link recovery procedures for the UE. Depending on the SBFD antenna configuration, it becomes possible that the DL signal reception of the radio link monitoring RS in the SBFD DL subbands is worse than that of the SSBs.

For RLF, the UE must evaluate the in-sync and out-of-sync criteria. NR assumes that “comparable” reception quality applies across the radio link monitoring RS. This assumption may or may not apply when SBFD is supported. However, the NR in-sync and out-of-sync criteria for RLF are evaluated by the UE using the same BLER threshold levels for all configured radio link monitoring RS.

The UE performs RLM using RS(s) corresponding to resource indexes provided by RadioLinkMonitoringRS for the active DL BWP or, if RadioLinkMonitoringRS is not provided for the active DL BWP, using RS(s) provided for the active TCI state for PDCCH receptions in CORESETs on the active DL BWP. The UE is either configured with a CSI-RS resource index or with a SS/PBCH block index. For a CSI-RS resource, parameter powerControlOffsetSS is not applicable and a UE expects to be configured only 'noCDM' from cdm-Type, only 'one' and 'three' from density, and only '1 port' from nrofPorts.

The UE can be configured with up to NLR-RLM RadioLinkMonitoringRS for link recovery procedures and for radio link monitoring. From the NLR-RLM RadioLinkMonitoringRS, up to NRLM RadioLinkMonitoringRS can be used for radio link monitoring depending on LMAX, and up to two RadioLinkMonitoringRS can be used for link recovery procedures. For example, in the NR band n78, LMAX = 8, NLR-RLM = 6 and NRLM = 4.

In non-DRX mode of operation, the L1 in the UE assesses once per indication period the radio link quality, evaluated over the previous time period against thresholds Qout and Qin configured by rlmInSyncOutOfSyncThreshold. The UE determines the indication period as the maximum between the shortest periodicity for radio link monitoring resources and 10 msec. In DRX mode operation, the UE determines the indication period as the maximum between the shortest periodicity for radio link monitoring resources and the DRX period.

On each RLM-RS resource, the UE estimates the DL radio link quality and compares it to the thresholds Qout and Qin for the purpose of monitoring the DL radio link quality of the cell.

The threshold Qout is defined as the level at which the DL radio link cannot be reliably received and corresponds to the out-of-sync block error rate BLERout as defined in 38.133. For SSB-based and for CSI-RS based radio link monitoring, Qout_SSB and Qout_CSI-RS, respectively, are derived based on hypothetical PDCCH transmission parameters.

The threshold Qin is defined as the level at which the DL radio link quality can be received with higher reliability than at Qout and corresponds to the in-sync block error rate BLERin as defined in 38.133. For SSB-based and CSI-RS based radio link monitoring, Qin_SSB and Qin_CSI-RS are defined.

The UE then evaluates whether the DL radio link quality on the configured RLM-RS resource, i.e., SSB based and/or CSI-RS based becomes worse than the threshold Qout_SSB or becomes better than the threshold Qin_SSB with respect to evaluation periods defined in 38.133. The evaluation periods may be adjusted based on considerations such as measurement gaps or SMTC occasions.

The out-of-sync block error rate BLERout and in-sync block error rate BLERin are determined by network configuration for the UE via parameter rlmInSyncOutOfSyncThreshold. When the UE is not configured rlmInSyncOutOfSyncThreshold from the network, the UE determines out-of-sync and in-sync BLER using Configuration #0 in 38.133 as default.

As by 38.213, the UE L1 would indicate, in frames where the radio link quality is assessed, out-of-sync to higher layers when the radio link quality is worse than the threshold Qout for all resources in the set of radio link monitoring RS. When the radio link quality is better than the threshold Qin for any resource in the set of resources for radio link monitoring, the UE L1 indicates, in frames where the radio link quality is assessed, in-sync to higher layers.

When CSI-RS is configured for the UE as RLM RS, the CSI-RS in the DL subband is therefore penalized by worse reception when compared to the SSB in non-SBFD symbols. The RLF evaluation however assumes one/same out-of-sync BLER level and one/same in-sync BLER level for both SSB and CSI-RS based radio link monitoring RS.

When a set of SSB based and CSI-RS based RLM-RS resources is configured for the UE in SBFD slots, an out-of-sync on the CSI-RS in the SBFD slots is then typically declared earlier than when evaluating on the SSBs in a DL slot where the UE experiences better Rx SINR conditions. Similarly, in-sync will be declared by the UE based on the typically better reception quality on the SSBs. Alternatively, RLM-RS resources using only SSBs can be configured for the UE radio link monitoring.

In both cases, the ability to effectively use the CSI-RS as radio link monitoring RS is then lost with SBFD. Note that similar considerations also apply to the BM / link recovery procedures in 38.213.

From the network scheduling perspective, radio link monitoring is critical to ensure that the UE maintains basic connectivity, i.e., can be reached by data/control. When the UE declares RLF, this results in an attempted RRC connection re-establishment procedure for a number of times during which no data transmission/reception from/to the UE is possible at all, i.e., loss of connectivity. When only SSBs are available for radio link monitoring with SBFD, RLF must then be based on the DL common coverage beams. Note that the ability to monitor reception conditions in the SBFD slots for purpose of scheduling by the network is still possible using the CSI reports.

Observation 1. Support for radio link monitoring by the UE on SBFD and non-SBFD slots is critical Rel-19 functionality to prevent higher layer interruptions from premature RLF declaration by the UE

Observation 2. RLF detection by the UE using only the SSBs as NLR-RLM RadioLinkMonitoringRS is not sufficient to support RLM for all SBFD antenna configuration options

When considering the 2 identified options, Option 2 allows for separate RLF detection for the SBFD and non-SBFD slots. The CSI-RS based resources configured as RadioLinkMonitoringRS, or separate CSI-RS based or SSB based resources configured as RadioLinkMonitoringRS across SBFD and non-SBFD symbols cannot be used for the same RLM/BFD/CBD measurement.

We see however 2 possible sub-options which vary much in the associated/expected specification effort.

Option 2a is use one/same counting for Qin/Qout across SBFD/non-SBFD resources but then introduce an offset for CSI-RS based RadioLinkMonitoringRS in SBFD symbols. The offset can either be introduced using a threshold delta or a new/additional BLER configuration (in addition to BLER Configuration #0) for the set of RadioLinkMonitoringRS to set the rlmInSyncOutOfSyncThreshold. Resulting specification impact can then be expected mostly in RAN4.

Option 2b would rely on separate RLF detection behavior by the UE, i.e., separate counting of Qin/Qout, in the SBFD/non-SBFD resources. It would then also be needed to specify the corresponding RLF behavior from RAN1 and RAN2 perspective. For example, should RLF be declared as soon as Qout for all RadioLinkMonitoringRS on SBFD symbols to trigger RLF procedures according to 38.331, or be subject to Qin/Qout on the SSB symbols configured as RadioLinkMonitoringRS? We expect more specification effort and more impact across RAN1/2/4 from Option 2b.

In summary, our preference is therefore Option 2a with remaining details, i.e., offset based, or new/additional BLER configuration left for discussion/decision in RAN4.

Proposal 22. For radio link monitoring by the UE with SBFD, support Option 2a:

- SSB-based or CSI-RS-based resources configured as RadioLinkMonitoringRS across SBFD and non-SBFD symbols can be used for the same RLM/BFD/CBD measurement

- Single radio link quality evaluation/indication using SSB or CSI-RS resources with offset value or new/additional BLER configuration for SBFD symbols

# 3. Conclusion

This contribution discussed the SBFD operation design and following observations and proposals are given:

Observation 1. Support for radio link monitoring by the UE on SBFD and non-SBFD slots is critical Rel-19 functionality to prevent higher layer interruptions from premature RLF declaration by the UE

Observation 2. RLF detection by the UE using only the SSBs as NLR-RLM RadioLinkMonitoringRS is not sufficient to support RLM for all SBFD antenna configuration options

Proposal 1. Confirm the cell-specific configuration on frequency location of SBFD subbands.

- UE-specific frequency domain location of SBFD subbands is not necessary

Proposal 2. In case of CSI-RS across the two DL subbands, consider to double the counting of active resources and active ports for the CSI-RS.

Proposal 3. In case of a CSI-RS across the two DL subbands, the frequency domain resources of the CSI-RS consists of 2 parts in different DL subbands.

- At least for CSI acquisition, the number of PRBs for each of the 2 parts is ≥ min⁡(24,NDL-SBsize), where NDL-SBsize is the number of PRBs of DL subband which includes the associated part of the CSI-RS

Proposal 4. Adopt the following text proposal for TS38.214

TS 38.214 #### 5.4 UE CSI computation time <omitted texts> For a CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations, provided by csi-ReportSubConfigToAddModList, only (Z2,Z2') of table 5.4-2 is applicable. If a UE is configured with SBFD symbols, the UE may expect Z and Z' to be scaled by factor of 2 and if a CSI report corresponding to a CSI-ReportConfig is associated with CSI-RS resource(s) for channel measurement overlaps with two DL subbands in active DL BWP in SBFD symbol(s), the (Z(m),Z'(m)) corresponding to the CSI report is scaled by a factor of 2. <omitted texts>

Proposal 5. If the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is configured, the valid downlink slot associated with CSI reference resource for the CSI report is defined as follows:

- A slot comprises at least one higher layer configured downlink or flexible symbol, where the symbol is with the same symbol type as the configured valid symbol type; and

- The slot does not fall within a configured measurement gap for that UE

Proposal 6. Adopt the following text proposal for TS38.214

TS38.214 #### 5.2.2.5 CSI reference resource definition The CSI reference resource for a serving cell is defined as follows: - In the frequency domain, the CSI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CSI relates. - In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6 TS 38.213], and where μKoffsetis the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1 and for FR2-NTN, - where +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL and and are the subcarrier spacing configurations for DL and UL, respectively, and Nslot, offsetCA and are determined by higher-layer configured ca-SlotOffset for the cells transmitting the uplink and downlink, as defined in clause 4.5 of [4, TS 38.211] - where for periodic and semi-persistent CSI reporting - if a single CSI-RS/SSB resource is configured for channel measurement nCSI_ref is the smallest value greater than or equal to 4⋅2µDL, such that it corresponds to a valid downlink slot, or - if multiple CSI-RS/SSB resources are configured for channel measurement nCSI_ref is the smallest value greater than or equal to , such that it corresponds to a valid downlink slot. - where for aperiodic CSI reporting, if the UE is indicated by the DCI to report CSI in the same slot as the CSI request, nCSI_ref is such that the reference resource is in the same valid downlink slot as the corresponding CSI request, otherwise nCSI_ref is the smallest value greater than or equal to , such that slot n- nCSI_ref corresponds to a valid downlink slot, where Z' corresponds to the delay requirement as defined in Clause 5.4. - when periodic or semi-persistent CSI-RS/CSI-IM or SSB is used for channel/interference measurements, the UE is not expected to measure channel/interference on the CSI-RS/CSI-IM/SSB whose last OFDM symbol is received up to Z' symbols before transmission time of the first OFDM symbol of the aperiodic CSI reporting. A slot in a serving cell shall be considered to be a valid downlink slot if: - it comprises at least one higher layer configured downlink or flexible symbol, and - it comprises at least one of either SBFD symbol(s) or non-SBFD symbol(s) as indicated by symbolType if applicable, and - it does not fall within a configured measurement gap for that UE If there is no valid downlink slot for the CSI reference resource corresponding to a CSI Report Setting in a serving cell, CSI reporting is omitted for the serving cell in uplink slot n'.

Proposal 7. If the timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured", and if the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is configured,

- The most recent occasion of the periodic/semi-persistent CSI-RS resource for channel measurement for CSI derivation is within the symbols with the same symbol type as the configured valid symbol type

Proposal 8. If the timeRestrictionForInterferenceMeasurements in CSI-ReportConfig is set to "Configured", and if the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is configured,

- The most recent occasion of periodic/semi-persistent CSI-IM and/or NZP CSI-RS for interference measurement for CSI derivation is within the symbols with the same symbol type as the configured valid symbol type

Proposal 9. Adopt the following text proposal for TS38.214

TS 38.214 #### 5.2.2.1 Channel quality indicator (CQI) <omitted texts> If the higher layer parameter timeRestrictionForChannelMeasurements is set to "notConfigured", the UE shall derive the channel measurements for computing CSI value reported in uplink slot n based on only the NZP CSI-RS, no later than the CSI reference resource, (defined in [4, TS 38.211]) associated with the CSI resource setting. If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured", the UE shall derive the channel measurements for computing CSI reported in uplink slot n based on only the most recent, no later than the CSI reference resource, in cell DTX active period of a serving cell if cell DTX is activated or in either SBFD symbol(s) or non-SBFD symbol(s) as indicated by symbolType for semi-persistent or periodic CSI-RS, occasion of NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting on the serving cell. If the higher layer parameter timeRestrictionForInterferenceMeasurements is set to "notConfigured", the UE shall derive the interference measurements for computing CSI value reported in uplink slot n based on only the CSI-IM and/or NZP CSI-RS for interference measurement no later than the CSI reference resource associated with the CSI resource setting. If the higher layer parameter timeRestrictionForInterferenceMeasurements in CSI-ReportConfig is set to "Configured", the UE shall derive the interference measurements for computing the CSI value reported in uplink slot n based on the most recent, no later than the CSI reference resource, in cell DTX active period of a serving cell if cell DTX is activated or in either SBFD symbol(s) or non-SBFD symbol(s) as indicated by symbolType for semi-persistent or periodic CSI-RS, occasion of CSI-IM and/or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) associated with the CSI resource setting on the serving cell. <omitted texts> #### 5.2.1.4.3 L1-RSRP Reporting <omitted texts> If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "notConfigured", the UE shall derive the channel measurements for computing L1-RSRP value reported in uplink slot n based on only the SS/PBCH or NZP CSI-RS, no later than the CSI reference resource, (defined in [4, TS 38.211]) associated with the CSI resource setting. If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured", the UE shall derive the channel measurements for computing L1-RSRP reported in uplink slot n based on only the most recent, no later than the CSI reference resource, in either SBFD symbol(s) or non-SBFD symbol(s) as indicated by symbolType for semi-persistent or periodic CSI-RS, occasion of SS/PBCH or NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting. <omitted texts>

Proposal 10. Adopt the following text proposal for TS38.214

TS 38.214 #### 5.2.2.5 CSI reference resource definition <omitted texts> For a CSI report with reportQuantity not set to ‘ssb-Index-RSRP’, ‘ssb-Index-SINR’, ‘ssb-Index-RSRP-Index’ or ‘ssb- Index-SINR-Index’, after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement no later than CSI reference resource and drops the report otherwise. For a CSI report configuration containing a list of sub-configurations provided by csi- ReportSubConfigToAddModList, after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report including one or more sub-reports only after receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement, per sub-configuration, no later than CSI reference resource and drops the report otherwise, where the sub-configuration is the activated/triggered one for AP/SP-CSI reporting, or the configured one for P-CSI reporting. For a CSI-ReportConfig configured with two Resource Groups and 𝑁 Resource Pairs for channel measurement in the corresponding CSI-RS Resource Set, as described in clause 5.2.1.4.1, after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each of the CSI-RS resources in the corresponding CSI-RS Resource Set for channel measurement and one CSI-IM occasion for each of the CSI-IM resources in the corresponding Resource Set for interference measurement no later than the CSI reference resource and within the same DRX Active Time when DRX is configured, and drops the report otherwise. For a CSI-ReportConfig configured with codebookType set to 'typeII-CJT-r18' or 'typeII-CJT-PortSelection-r18', after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each of the CSI-RS resources in the corresponding CSI-RS Resource Set for channel measurement and one CSI-RS and/or CSI-IM resource transmission occasion for the CSI-RS and/or CSI-IM resource in the corresponding Resource Set for interference measurement no later than the CSI reference resource and within the same DRX Active Time, when DRX is configured, and drops the report otherwise. For a CSI-ReportConfig configured with codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18', after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only if receiving at least one aperiodic or 𝐾𝑝 periodic or semipersistent consecutive CSI-RS transmission occasions for each CSI-RS resource in the corresponding CSI-RS Resource Set for channel measurement and one CSI-RS and/or CSI-IM resource transmission occasion for the CSI-RS and/or CSI-IM resource in the corresponding Resource Set for interference measurement no later than the CSI reference resource and within the same DRX Active Time, when DRX is configured, and drops the report otherwise. The value of 𝐾𝑝∈{1,2,4} is indicated by UE capability, as defined in clause 5.2.1.6. For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'tdcp', after the CSI report (re)configuration, serving cell activation, BWP change, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each CSI-RS resource in the 𝐾𝑇𝑅𝑆 CSI-RS Resource Sets of the CSI-RS Resource Setting for channel measurement no later than the CSI reference resource within the same DRX active time, when DRX is configured, and drop the report otherwise. For a CSI-ReportConfig configured with symbolType, after the CSI report (re)configuration, serving cell activation, BWP change, the UE reports a CSI report only after receiving at least one semi-persistent or periodic CSI-RS transmission occasion for each of the CSI-RS resources in the corresponding CSI-RS Resource Set for channel measurement and/or one CSI-RS and/or CSI-IM resource transmission occasion for each of the CSI-RS and/or CSI-IM resource in the corresponding Resource Set for interference measurement, no later than the CSI reference resource and within either SBFD symbol(s) or non-SBFD symbol(s) as indicated by symbolType, and drops the report otherwise. <omitted texts>

Proposal 11. For non-codebook based PUSCH transmission, in case of a periodic or semi-persistent associated CSI-RS for an SRS-ResourceSet

- UE calculate the precoder used for the transmission of SRS based on measurement of CSI-RS in the same symbol type as the symbol type for the SRS-ResourceSet.

Proposal 12. For non-codebook based PUSCH transmission, an aperiodic associated CSI-RS for an SRS-ResourceSet is located at the first available slot which is no earlier than the slot with the scheduling DCI.

- A slot is defined as an available slot if the symbols of the CSI-RS in the slot are of the same symbol type as the symbol type for the SRS-ResourceSet.

Proposal 13. In case of prb-BundlingType=staticBundling and bundleSize=wideband, UE may assume the PRG size is equal to 2 when the scheduled PRBs overlap with two DL subbands in SBFD symbol.

Proposal 14. In case of prb-BundlingType=dynamicBundling and bundleSizeSet1= n2-wideband or n4-wideband, if PRB bundling size indicator field indicates ‘1’, the DL subband size can be used for PRG size determination, instead of DL BWP size.

Proposal 15. RAN1 to discuss whether to count the determination of value PRBs within DL usable PRBs towards the maximum number of RateMatchPattern(s) per serving cell or BWP or introduce a separate UE capability.

Proposal 16. For type-1 HARQ-ACK codebook, if a PDSCH candidate overlaps with both SBFD symbols and non-SBFD symbols, the PDSCH candidate is excluded.

Proposal 17. Send a LS to RAN4 that separate maximum configured output power for SBFD/non-SBFD slots should be supported, e.g., using Options 1 or 2 as a starting point.

- Option 1: one/same RRC signaled p-max value but use of a new/additional SBFD slot specific delta offset

- Option 2: two separate RRC signaled p-max values, e.g., legacy p-max for non-SBFD, and new/additional for the SBFD slots.

Proposal 18. For the single TRP scenario, for separate UL power control of PUSCH transmissions in SBFD symbols and non-SBFD symbols, support separate PHR Type 1 reporting.

Proposal 19. Support the following orders for collision handling (Option 2) with the following update

- Step 1: Resolving SBFD specific collision between PUCCH with repetitions/PUSCH transmissions and unusable resources, if any.

- Step 2: Resolving the collision as in legacy, if any.

- Step 3: Resolving SBFD specific collision between transmissions/receptions and unusable resources, if any.

Proposal 20. For half-duplex CA case, SBFD symbol is considered as flexible symbol.

Proposal 21. For intra-band CA case, the valid symbol type for CSI derivation can be configured in CSI-ReportConfig, where the CSI-ResourceConfig associated with the CSI-ReportConfig is in a serving cell which is the same or different from the serving cell for the TDD carrier for SBFD operation.

Proposal 22. For radio link monitoring by the UE with SBFD, support Option 2a:

- SSB-based or CSI-RS-based resources configured as RadioLinkMonitoringRS across SBFD and non-SBFD symbols can be used for the same RLM/BFD/CBD measurement

- Single radio link quality evaluation/indication using SSB or CSI-RS resources with offset value or new/additional BLER configuration for SBFD symbols

# Reference

[1] RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD),” Huawei, 3GPP TSG RAN Meeting #104

[2] R1-2503543, “Discussion on RAN4 LS on the transient period for SBFD operation” Samsung

[3] R1-2502351 “Discussion on LS reply to RAN4 on L1 CLI measurement” Samsung