3GPP TSG RAN WG1 #121		R1-2503588

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	9.12.2

Source:	Samsung

Title:	Discussion on low band carrier aggregation via switching

Document for:	Discussion and decision

# Introduction

A WI on enhancements of low band carrier aggregation via switching was endorsed in [1]. The objective is to introduce physical layer procedures and requirements to enable low band carrier aggregation via switching as following.

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

The following agreements were made in RAN1#120b meeting [2].

Agreement - For Rel-19 low NR band carrier aggregation via switching, support only semi-static configuration of switching pattern to a UE based on UE-specific RRC configuration between the following two cases, - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - FDD carrier 1 is PCell and SDL carrier 2 is SCell. - SCS 15KHz on both carriers. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, it is not expected that the same slot (1ms) contains both symbol(s) configured as FDD carrier and symbol(s) configured as SDL carrier. Agreement For Rel-19 low NR band carrier aggregation via switching, the semi-static switching pattern and corresponding switching gap are based on the downlink timing (DL reception time at UE side) of Pcell. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the time unit (resolution) of the semi-static switching pattern configuration is slot (1ms for 15kHz SCS). Agreement - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design - NW configures a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2. - The pattern starts from the beginning of SFN 0 of PCell. - Restriction of maximum X switch(es) within Y slot(s). - FFS: detailed value and relation of X and Y. - FFS: other restriction. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, down-select one of the following solutions: - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier. - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier. - Alt 3: Switching gap is always assumed at the SDL carrier. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed). - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL. - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA. - FFS: whether it has specification impact.

This contribution discusses how to support low band carrier aggregation via switching.

# Discussion

It was agreed in RAN1#120b meeting to support semi-static switching pattern with bitmap for LB-CA with the granularity of one slot (1 ms). One remaining issue is the details of restriction of maximum X switch(es) within Y slot(s). It is straightforward to define the value of X per the supported periodicity 40ms. To reduce the wasted resource due to the switching gap and to ease UE implementation, the values of X should not be very large, for example, up to 8.

Proposal 1: Regarding the restriction of maximum X switch(es) within Y slot(s), support Y = 40 and X = 8.

To ensure the QoS of UL data transmission and latency of HARQ-ACK feedback, the continued period of SCell operation should not be long. For 15kHz SCS, the candidate values of K1 are {1,2,3,4,5,6,7,8}, the number of continued slots for SCell should not be larger than 8, otherwise, the UE cannot provide HARQ-ACK for PDSCHs scheduled on the latest slots on PCell before the switching as shown in Figure 1.

Figure 1

Observation 1: For low band carrier aggregation via switching, if the number of continuous slots on SCell is larger than 7, HARQ-ACK of a PDSCH reception in the last slot on PCell cannot be provided due to the K1 value restriction.

Proposal 2: For low band carrier aggregation via switching, the maximum of continuous slots on SCell is 7.

The switching gap was discussed in RAN#120b meeting and the remaining open issue is the switching from FDD carrier to SDL carrier. Alt 1 (Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier) can help avoid the impact on PDCCH monitoring in the first few symbols on SCell. However, it would impact the UL transmission. Whether a UL channel/signal can be transmitted depends on the ending symbol of the UL channel/signal as well as the TA. For example, for slot-based scheduling, if the TA is less than the switching gap, the UL transmission would overlap with the gap and cannot be transmitted by the UE as shown in Figure 2. On the other hand, if the TA is larger than the switching gap or the UL channel/signal ends earlier, the UL transmission may overlap with the gap and can be transmitted.

Figure 2

In addition, Alt 1 would impact the UCI multiplexing procedure, an example is given in Figure 3, there can be two candidate UE behaviours.

Option 1) The UE first drops the PUCCH including the UCI due to colliding with the switching gap and only transmits the PUSCH without the UCI.

Option 2) The UE first multiplexes the UCI in the PUSCH and transmits the PUSCH with the UCI.

There can be more complicated cases which would be impacted by Alt1.

Figure 3

Observation 2: Alt 1 (Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier) would impact on UL transmissions on PCell as well as UCI multiplexing procedure.

Alt 3 (Switching gap is always assumed at the SDL carrier.) can avoid the impact on UL transmission and is much simpler than Alt 1 and thus is preferred.

Proposal 3: For switching from FDD carrier to SDL carrier, support Alt 3, i.e., Switching gap is always assumed at the SDL carrier.)

For a UE configured with carrier switching, the semi-static transmissions/receptions can be cancelled due to carrier switching while dynamic transmissions/receptions should be ensured by the network by proper scheduling and the UE does not expect to be dynamically scheduled with a transmission/reception on a serving cell and the transmission/reception overlaps with non-operation period of the serving cell or the switching gap.

Proposal 4: For low band carrier aggregation via switching,

- The UE does not receive or transmit a semi-static signal/channel on a serving cell if the semi-static signal/channel overlaps with a non-operation period of the serving cell or the switching gap.

- The UE does not expect to be scheduled with a transmission/reception on a serving cell by a DCI format and the transmission/reception overlaps with a non-operation period of the serving cell or the switching gap.

Timeline for LB-CA carrier switching

The carrier switching period would result in additional processing time for a UL transmission associated with a PDCCH or PDSCH similar as BWP switching. For example, a PDCCH schedules a PUSCH, a PDCCH indicates SRS transmission, a RAR schedules a PUSCH transmission, and a PUCCH with HARQ-ACK for a PDSCH or PDCCH reception. An example is given in figure 4, the legacy timeline N1 may not be enough considering carrier switching would introduce additional processing time.

Figure 4

Proposal 5: For low band carrier aggregation via switching, additional processing time should be introduced for the following cases if carrier switching happens between a DL reception and a UL transmission,

- a PDCCH reception schedules a PUSCH transmission

- a PDCCH reception indicates SRS transmission

- a RAR schedules a PUSCH transmission

- a PUCCH transmission with HARQ-ACK for a PDSCH or PDCCH reception

HARQ-ACK codebook enhancements

For Type-1 HARQ-ACK codebook, if a candidate PDSCH reception on a serving cell overlaps with a non-operation period of the serving cell, the UE does not receive a PDSCH on the candidate PDSCH reception, therefore, the UE does not need to generate HARQ-ACK bits for the candidate PDSCH reception. Similarly, if a SPS PDSCH on a serving cell overlaps with a non-operation period of the serving cell, the UE does not need to generate HARQ-ACK bits for the SPS PDSCH.

Proposal 6: For low band carrier aggregation via switching,

- If a UE is provided with Type-1 HARQ-ACK codebook, and a candidate PDSCH reception on a serving cell overlaps with a non-operation period of the serving cell, the candidate PDSCH reception is excluded from the set of occasions for candidate PDSCH receptions.

- If a SPS PDSCH reception on a serving cell overlaps with a non-operation period of the serving cell, the UE does not receive the SPS PDSCH and does not transmit HARQ-ACK for the SPS PDSCH reception.

# Conclusion

The contribution discusses low band carrier aggregation via switching and the proposals and observations are summarized below.

Proposal 1: Regarding the restriction of maximum X switch(es) within Y slot(s), support Y = 40 and X = 8.

Observation 1: For low band carrier aggregation via switching, if the number of continuous slots on SCell is larger than 7, HARQ-ACK of a PDSCH reception in the last slot on PCell cannot be provided due to the K1 value restriction.

Proposal 2: For low band carrier aggregation via switching, the maximum of continuous slots on SCell is 7.

Observation 2: Alt 1 (Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier) would impact on UL transmissions on PCell as well as UCI multiplexing procedure.

Proposal 3: For switching from FDD carrier to SDL carrier, support Alt 3, i.e., Switching gap is always assumed at the SDL carrier.)

Proposal 4: For low band carrier aggregation via switching,

- The UE does not receive or transmit a semi-static signal/channel on a serving cell if the semi-static signal/channel overlaps with a non-operation period of the serving cell or the switching gap.

- The UE does not expect to be scheduled with a transmission/reception on a serving cell by a DCI format and the transmission/reception overlaps with a non-operation period of the serving cell or the switching gap.

Proposal 5: For low band carrier aggregation via switching, additional processing time should be introduced for the following cases if carrier switching happens between a DL reception and a UL transmission,

- a PDCCH reception schedules a PUSCH transmission

- a PDCCH reception indicates SRS transmission

- a RAR schedules a PUSCH transmission

- a PUCCH transmission with HARQ-ACK for a PDSCH or PDCCH reception

Proposal 6: For low band carrier aggregation via switching,

- If a UE is provided with Type-1 HARQ-ACK codebook, and a candidate PDSCH reception on a serving cell overlaps with a non-operation period of the serving cell, the candidate PDSCH reception is excluded from the set of occasions for candidate PDSCH receptions.

- If a SPS PDSCH reception on a serving cell overlaps with a non-operation period of the serving cell, the UE does not receive the SPS PDSCH and does not transmit HARQ-ACK for the SPS PDSCH reception.

# References

- RP-250247, Revised WID on Low NR band carrier aggregation via switching

- Chair’s notes of RAN1#120b meeting