3GPP TSG RAN WG1 Meeting #121																R1- 2503759

St. Julian’s, Malta, May 19th –23rd, 2025

Agenda item:		9.3.1

Title:						Discussion on SBFD Tx/Rx/measurement procedures

Source:				Fraunhofer HHI, Fraunhofer IIS

Document for:		Discussion

# Introduction

The WID on Evolution of NR Duplex Operation was agreed [1] in RAN#102. This contribution focuses on some of the open issues related to the DL receptions and UL transmissions across SBFD and non-SBFD symbols within a slot and across slots.

# Physical signals/Channels across SBFD and non-SBFD symbols in different slots

In meeting RAN1#119 [3], the following agreement was made on the UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots.

Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, - Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability. - SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP. - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP. - For SRS, only Configuration 1 is applicable.

Since configuration 1 is the default capability of the UE, it can also be the default configuration of the UE. The gNB does not need to provide any configuration if the UE is not capable of configuration 2. Based on this, the following proposal is made.

Proposal 1: Configuration 1 is considered as the default configuration if no configuration is provided.

## Configuration 1

### SRS

The following agreement was made in meeting RAN1#119 [3] wrt. SRS.

Agreement Red text is working assumption. For separate SRS configurations for SBFD and non-SBFD symbols, support Option 1. Option 1: Support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage. - SRS-ResourceSet configured for SBFD symbol is only applicable to SRS transmission occasions in SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in non-SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only SBFD symbols are available for SRS transmission. - FFS impact on availableSlotOffsetList - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the SBFD symbols. - FFS: whether there is impact on frequency hopping counter - SRS-ResourceSet configured for non-SBFD symbol is only applicable to SRS transmission occasions in non-SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only non-SBFD symbols are available for SRS transmission. - FFS impact on availableSlotOffsetList - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the non-SBFD symbols. - FFS: whether there is impact on frequency hopping counter - The number of SRS resources in a SRS-ResourceSet for SBFD symbols is the same as the number of SRS resources in a SRS-ResourceSet for non-SBFD symbols for the same usage. - For SRS-ResourceSets with usage set to 'nonCodebook', only one SRS port for each SRS resource is configured as legacy. - For SRS-ResourceSets with usage set to 'Codebook', - Except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', the numbers of SRS ports are the same for all the SRS resources in the two SRS-ResourceSets - FFS when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2' - Maintain single TRP behaviour with respect to DCI fields and RRC parameters in rrc-ConfiguredUplinkGrant - SRS resource set indicator (SRSI) field is absent from DCI. srs-ResourceSetId is absent in rrc-ConfiguredUplinkGrant. - If txConfig is set to 'codebook', second precoding information (TPMI) field is absent from DCI and precodingAndNumberOfLayers2 is absent from rrc-ConfiguredUplinkGrant. - Second SRS resource indicator (SRI) field is absent from DCI - For DG PUSCH without repetition within a slot, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in the same symbol type as that of PUSCH transmission, where the SRS resource prior to the PDCCH carrying the SRI. - If txConfig is set to 'codebook', the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to the same symbol type as that of PUSCH transmission - For DG PUSCH with repetition/multi-PUSCH scheduled by a single DCI/TBoMS/Type 2 CG PUSCH with Configuration 1, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in the same symbol type as the valid symbol type of PUSCH transmission, where the SRS resource prior to the PDCCH carrying the SRI. - If txConfig is set to 'codebook', the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to the same symbol type as the valid symbol type of PUSCH transmission - For DG PUSCH with repetition/multi-PUSCH scheduled by a single DCI/TBoMS/Type 2 CG PUSCH with Configuration 2, the SRS resource indicator is applicable to both SBFD and non-SBFD SRS-ResourceSets. For PUSCH transmissions in SBFD symbols, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in SBFD symbols, where the SRS resource is prior to the PDCCH carrying the SRI. For PUSCH transmissions in non-SBFD symbols, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in non-SBFD symbols, where the SRS resource is prior to the PDCCH carrying the SRI. - If txConfig is set to 'codebook', for PUSCH transmission in SBFD symbols, the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to SBFD symbols. For PUSCH transmission in non-SBFD symbols, the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to non-SBFD symbols. - If only a single SRS resource is configured in both SRS resource sets, the SRI field is absent from DCI. In this case “SRS resource identified by the SRI” in the above bullets corresponds to the single SRS resource in each SRS resource set. - srs-ResourceIndicator2 is absent from rrc-ConfiguredUplinkGrant. - For Type 1 CG PUSCH with Configuration 1, srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in the same symbol type as the valid symbol type of PUSCH transmission. - If txConfig is set to 'codebook', precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to the same symbol type as the valid symbol type of PUSCH transmission. - For Type 1 CG PUSCH with Configuration 2, srs-ResourceIndicator is applicable to both SBFD and non-SBFD SRS-ResourceSets. For PUSCH transmissions in SBFD symbols, the srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in SBFD symbols. For PUSCH transmissions in non-SBFD symbols, the srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in non-SBFD symbols. - If txConfig is set to 'codebook', for PUSCH transmission in SBFD symbols, precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to SBFD symbols. For PUSCH transmission in non-SBFD symbols, precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to non-SBFD symbols. - If txConfig is set to 'nonCodebook', each SRS resource set is associated with an associated CSI-RS. - Applicable to usage set to 'codebook' or 'nonCodebook' - FFS usage set to 'beamManagement' - Not applicable to usage set to 'antennaSwitching’

An SRS resource or resource set may be associated with an ‘associatedCSI-RS’ or have a spatial relation info defined. If the SRS is transmitted on an SBFD symbol, it is expected that the associated CSI-RS will be on an SBFD symbol. This might be explicitly configured by the gNB, e.g., when the CSI-RS resource is restricted to only SBFD symbols, or implicitly understood by the UE, e.g., when the CSI-RS resource is on both SBFD and non-SBFD symbols and the UE chooses the SBFD symbols only. The same behaviour will be applicable for spatial relation info. Hence, the following is proposed.

Proposal 2: For SRS in SBFD symbols, the associated CSI-RS and the RS associated with spatial relation info should be in an SBFD symbol. For SRS in non-SBFD symbols, the associated CSI-RS and the RS associated with spatial relation info should be in a non-SBFD symbol.

For usage set 'antennaSwitching’, separate SRS resource sets for SBFD symbols and for non-SBFD symbols are not supported. However, SRS resources are of type Configuration 1, so the SRS resources within a resource set is valid only for one symbol type. If the SRS resource set is not explicitly associated with either SBFD or non-SBFD symbols, the SRS resource within the resource set has to be associated with either SBFD or non-SBFD symbols. Hence, the valid symbol type for an SRS resource needs to be implicitly or explicitly informed to the UE. E.g., for AP-SRS, the valid symbol type is determined based on the symbol type of the first scheduled SRS and for SP-SRS or P-SRS, the valid symbol type is indicated in RRC. Based on the above discussion, the following proposal is made.

Proposal 3: For SRS usage set to 'antennaSwitching', an SRS resource set is not explicitly associated with either SBFD or non-SBFD symbols. The valid symbol type for each SRS resource is explicitly or implicitly determined by the UE.

## Configuration 2

### MCS determination for SPS and CG

As shown in Figure 1, the available DL resources for the UE to receive the SPS is fewer in SBFD slots as compared to non-SBFD slots. Also, the presence of cross link interference (CLI) in the SBFD symbols make the interference conditions worse in an SBFD symbol as compared to a non-SBFD symbol. Hence, the DL MCS for the SPS in SBFD symbols can have a different value as compared to a non-SBFD symbol. As shown in Figure 1, the SPS in slot 0 (SBFD slot) uses MCS 2 whereas the SPS in slot 1 (non-SBFD slot) uses MCS 1.

Figure 1: Different MCS for SPS PDSCH in SBFD and non-SBFD symbols

The MCS for the SBFD symbols can be explicitly indicated by the gNB and/or derived by the UE based on some factors. Explicit indication can contain the MCS value using the DCI used for scheduling the SPS resources. Also, an offset MCS value with the MCS for non-SBFD symbols as the baseline can be indicated and the UE can derive the MCS for the SBFD symbols from it. Same will be applicable for CG as well.

Based on the above discussion, the following proposal is made.

Proposal 4: For SPS and CG, indication or determination of different MCS values are supported for SBFD and non-SBFD symbols in different slots.

### Other UE Tx/Rx behavior

For PDSCH in configuration 2, only the resources within the DL usable resources are valid in SBFD symbols. For example, let us consider Figure 2. Here, the SPS occasion on the SBFD slot/symbols is such that very few usable DL resources are available, e.g., 1 PRB. Even if the UE receives the SPS in such a case, the performance will be quite degraded. Thus, it is a better choice to either ignore or postpone the SPS occasion as shown in Figure 2.

Figure 2: SPS occasion ignored/postponed due to very few usable DL resources

In our view, dropping the SPS on such occasion is better as compared to postponing it since it has lesser specification impact.

Proposal 5: For SPS PDSCH in configuration 2, the UE drops the reception of SPS in SBFD symbols if the DL usable resources are lesser than a threshold.

# PUSCH frequency hopping

The following agreement was made in RAN1#119 [3] with respect to FH offsets for SBFD symbols and non-SBFD symbols.

Agreement For PUSCH intra-slot frequency hopping in SBFD symbols, the starting RB in each hop is given by: RBstart=RBstart, i=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, i=1 For PUSCH inter-slot frequency hopping in SBFD symbols and when pusch-DMRS-Bundling is not enabled, or for inter-slot frequency hopping for a PUSCH in SBFD symbols scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot nsμ is given by: RBstart(nsμ)=RBstart, nsμmod2=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, nsμmod2=1 Where - RBUL SB start is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP. - NUL SBsize is the number of UL usable PRBs. - RBstart is the starting PRB index of the first PUSCH hop with reference to the start of UL active BWP. For PUSCH transmissions with Configuration 2, RBstart is the starting PRB index with reference to the start of UL active BWP after applying RB offset between non-SBFD symbols and SBFD symbols. - RB offset is the frequency hopping offset for PUSCH in SBFD symbols - Note: Definition of nsμ is unchanged from existing specifications Decide in RAN1#119, one of the following options: - Alt1: Remove -RBUL SB start in the above equations - Alt2: Remove square brackets from [-RBUL SB start] in the above equations

The above agreement provides the formulae for intra-slot and inter-slot PUSCH FH. For inter-slot FH with DMRS bundling, the following formula is used in legacy NR:

The two main parameters of DMRS bundling are pusch-TimeDomainWindowLength, which gives the number of consecutive slots from which the DMRS are processed together for channel estimation and, pusch-FrequencyHoppingInterval, which gives the number of consecutive slots for which the FH remains the same and the DMRS is also bundled. Each nominal time domain window (TDW) can be split into actual TDWs if there is a phase discontinuity. For PUSCH across SBFD and non-SBFD symbols, there can be a phase discontinuity when transition from SBFD symbols to non-SBFD symbols happen or vice versa. In that case, the nominal TDW can be split into actual TDWs. That is, DMRS bundling across both SBFD and non-SBFD symbols is avoided. The DMRS is bundled only within 1 symbol type, either SBFD or non-SBFD.

Proposal 6: For PUSCH DMRS bundling, DMRS bundling across both SBFD and non-SBFD symbols is avoided.

# SBFD operation in multi-carrier scenario

The following agreement was made in RAN1#120 [1].

Agreement Support SBFD operation on one TDD carrier in multi-carrier scenario. - Note: If the TDD carrier for SBFD operation is an Scell, support UE dedicated signaling for cell-specific configuration of time and frequency location of SBFD subbands for the Scell. - FFS whether/how to handle half-duplex CA case

The figure below shows an example of TDD carrier aggregation (CA) in a multi-carrier scenario, having a primary carrier and a secondary carrier, with SBFD operated on the secondary carrier. This can be used for additional transmissions in UL, without having to wait for UL resources on the primary carrier. Furthermore, this reduces the delay in UL, which may be crucial for feedback transmissions, and which would help to improve the overall reliability in UL. Moreover, this would allow to have continuous UL transmission, also in TDD systems, which would enhance the UL coverage.

From a gNB perspective, the gNB would have a higher flexibility in resource assignment, since it could allocate UL resources in a time continuous manner. In connected mode, utilizing SBFD UL resources would allow for a larger number of RACH occasions, e.g., to be used for contention-free RACH, which may be utilized on the secondary carrier for specific purposes, such as handover procedures, interference mitigation, or load balancing. Nevertheless, this is just one scenario, and SBFD slots could also be configured on the primary carrier, having comparable benefits.

Figure 3 TDD carrier in multi-carrier scenario with continuous UL on a secondary carrier.

According to TS 38.213, in case of CA,

the UE determines a reference cell for a symbol as an active cell with the smallest cell index among

- the configured multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and

- the cells of each band respectively if the UE is capable of simultaneous transmission and reception by simultaneousRxTxInterBandCA for the configured multiple serving cells.

Further, the symbol in the reference cell is defined as following.

Where the symbol is configured as

- downlink, or uplink, as indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated

- uplink, if the symbol is flexible and the UE is configured to transmit SRS, PUCCH, PUSCH, or PRACH on the symbol

- downlink, if the symbol is flexible and the UE is configured to receive PDCCH, PDSCH or CSI-RS on the symbol.

Now, when one of the TDD carriers have SBFD configured on it, the reference cell can be an SBFD configured cell. In such a case, when a symbol is configured as DL by tdd-UL-DL-ConfigurationCommon it can be further configured to be an SBFD symbol. This implies that now the symbol has both UL and DL resources instead of only DL resources. Hence, whether the UE will be scheduled in UL or DL in that particular symbol can be decided later, e.g., using lower layer signalling like DCI. Similarly, for symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, the symbol can be further configured to be an SBFD symbol. Based on the above explanation, whether a symbol in a reference cell will be DL or UL does not remain straightforward anymore when the carrier has SBFD configured on it.

Further, a number of collision handling rules or UE behaviour are defined in the existing CA operation specifically to handle the half-duplex CA case in NR with respect to the reference cell. These might have to be enhanced to accommodate a reference cell with SBFD configuration or even when some other cell which is a part of the aggregated carriers has an SBFD configuration.

Based on the above discussion, we propose the following:

Proposal 7: For SBFD operation on one TDD carrier in a multi-carrier scenario, the existing directional collision handling for half-duplex TDD CA is reused by treating SBFD symbols as flexible symbols.

# Conclusions

The following proposals have been made in this document:

Proposal 1: Configuration 1 is considered as the default configuration if no configuration is provided.

Proposal 2: For SRS in SBFD symbols, the associated CSI-RS and the RS associated with spatial relation info should be in an SBFD symbol. For SRS in non-SBFD symbols, the associated CSI-RS and the RS associated with spatial relation info should be in a non-SBFD symbol.

Proposal 3: For SRS usage set to 'antennaSwitching', an SRS resource set is not explicitly associated with either SBFD or non-SBFD symbols. The valid symbol type for each SRS resource is explicitly or implicitly determined by the UE.

Proposal 4: For SPS and CG, indication or determination of different MCS values are supported for SBFD and non-SBFD symbols in different slots.

Proposal 5: For SPS PDSCH in configuration 2, the UE drops the reception of SPS in SBFD symbols if the DL usable resources are lesser than a threshold.

Proposal 6: For PUSCH DMRS bundling, DMRS bundling across both SBFD and non-SBFD symbols is avoided.

Proposal 7: For SBFD operation on one TDD carrier in a multi-carrier scenario, the existing directional collision handling for half-duplex TDD CA is reused by treating SBFD symbols as flexible symbols.

# References

- RP-234035, New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), RAN #102, Dec. 11-15, 2023.

- Chair notes, 3GPP RAN1#120, Athens, Greece, Feb 17th -21st, 2025.

- Chair notes, 3GPP RAN1#119, Orlando, US, Nov 18th -22nd, 2024.