3GPP TSG RAN WG1 #121                                                                                                 R1-2504166

St Julian’s Malta, May 19th – 23th, 2025

Source:               TCL

Title:                    Discussion on NR-NTN Downlink Coverage Enhancement

Agenda item:      9.11.1

Document for:    Discussion and Decision

- Introduction

In the RAN #106 meeting, a revised WID was approved for Rel-19 NR NTN [1], one of the objectives of the WID is related to downlink coverage enhancement with NR NTN operating, the details are shown below:

- Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km

In RAN1#120bis meeting [2], the following agreements have been achieved.

Agreement

When PDCCH CSS type-0 repetition is performed, for SIB1 link level enhancement, support PDSCH repetition of SIB1 transmitted within the same slot as the type0-CSS PDCCH repetition.

  - UE supporting SIB1 PDSCH coverage enhancement assumes that the PDCCH and associated PDSCH to be repeated in both slots where the corresponding PDCCHs are transmitted.

  - Each PDSCH SIB1 repetition is within the same slot of each PDCCH candidate for scheduling DCI

  - The two associated PDSCHs have the same RV

FFS: Whether it is supported that type-0 PDCCH repetition is not performed while the PDSCH-SIB1 repetition is performed, and if so whether/how to handle the slot determination.

Agreement

For enabling/disabling SIB1 PDSCH repetition, RAN1 to consider the following options:

  - Option 1: Using reserved bit(s) in PBCH payload.

  - Option 2: Using scheduling PDCCH.

  - Option 3: The enabling/disabling of SIB1 PDSCH repetition is implicitly indicating by the enabling/disabling of Type-0 CSS PDCCH repetition.

Agreement

For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1:

- Enabling/disabling using reserved bit(s) in PBCH payload

  - No UE behavior is defined for UE in connected mode specifically for the case where the network changes its signaling between enabling and disabling PDCCH repetition for Type0 PDCCH CSS.

Agreement

For Msg4 PDSCH repetition scheme, the Msg4 PDSCH is repeated in N consecutive slots:

- The same resource allocation is assumed for all repetitions

- The supported repetition factors are: 2 and 4

  - The network configures a single value between 2 and 4 at a given time

- The RV cycling is used for each repetition

  - If N=4:

| Starting RV | RV applicable to the nth repetition/transmission |  |  |  |
| --- | --- | --- | --- | --- |
|  | n mod 4 = 0 | n mod 4 = 1 | n mod 4 = 2 | n mod 4 = 3 |
| 0 | 0 | 2 | 3 | 1 |
| 2 | 2 | 3 | 1 | 0 |
| 3 | 3 | 1 | 0 | 2 |
| 1 | 1 | 0 | 2 | 3 |

Agreement

For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, support repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index ( n0 as defined in section 13 of TS 38.213) at least for M=2.

- Repeated PDCCH candidates share the same aggregation level (AL), coded bits and same candidate index

  - Note: if the network repeats the Type 0 PDCCH across two consecutive slots, a legacy UE might decode the PDCCH and associated PDSCH in one slot and skip PDCCH monitoring in the other slot.

Note: further discuss the potential solution for M=1 and M=1/2.

Working assumption

For PDCCH CSS other than Type-0 CSS and other than Type-3 CSS for common search spaces other than SearchSpaceZero, intra-slot PDCCH repetition is supported.

RAN1 to down select between option 1 and option 2:

Option 1: Use same CORESET and two different SS (SS Set1 and SS Set2)

- Linking two PDCCH candidates (adopt the same mechanism for SS linking specified in Release 17)

- FFS: Blind decoding limit

Option 2: Use same CORESET associated with one SS which is repeated by introducing symbol domain offset X

- FFS: Blind decoding limit

- FFS: details configuration and signalling

Agreement

For enabling/disabling Msg4 PDSCH repetition, RAN1 to down-select among the following options:

- Option 1: UE specific PDSCH with Msg4 repetition activation indicated via PDCCH- DCI Format 1_0

  - FFS: indication details.

  - FFS: whether/how network is informed by the UE that certain conditions are met to trigger Msg4 PDSCH repetition (e.g. RSRP detected at UE is less than x dB)

- Option 2: The enabling/disabling of Msg4 PDSCH repetition is implicitly indicated by the enabling/disabling of SIB1 PDSCH repetition.

- Option 3: The enabling/disabling is indicated by SIB1 configuration

  - FFS: whether/how network is informed by the UE that certain conditions are met to trigger Msg4 PDSCH repetition (e.g. RSRP detected at UE is less than x dB)

FFS: Whether UE reports its capability

In this contribution, we share our views on NR-NTN downlink coverage enhancements based on system level and link level.

- Discussion

System-level enhancement

It is claimed in Rel-19 NTN WI [1] that system-level enhancements are discussed to increase the system-level coverage of FR1-NTN and/or FR2-NTN allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprints, this section will be focused on system level enhancements to improve DL coverage.

    - SSB periodicity extending to 160ms

As discussed in the RAN1#116 meeting, the additional satellite parameter sets, i.e., LEO600km Set 1-1/1-2/1-3 have been agreed, it is observed that only 1.5% or 10.02% beam footprints are served, this limits the coverage area that the satellite can serve. In current spec, a default 20ms SSB periodicity is assumed to be a UE for cell discovery, as agreed for satellite parameters set1-1/1-2/1-3, a satellite beam footprint can be up to 1058, it’s very hard to cover such a large beam footprint within 20 ms, within an SSB burst, 4 footprints can be served by SSB, such, for set 1-1/1-3, the total number of  4*106 =424  footprint beams can be served, it’s more serious for set 1-2, only 4*16 =64 footprint beam can be served. Thus, the periodicity of SSB to a UE for cell discovery needs to be enlarged, and the extended SSB periodicity of 160ms has been agreed in the previous meeting. As defined in TR 38.821[2], satellite beams or satellites are not considered to be visible from the UE perspective in NTN. Both options a) the same PCI (Physical Cell Identifier) for several satellite beams and b) one PCI per satellite beam, can be considered in NTN, a satellite beam can consist of one SSB or more SSB beams. One cell (PCI) can have a maximum of L SSB beams, where L can be 4, 8, or 64 depending on the band, which means, mapping one SSB beam/cell to a/multiple satellite beams or multiple SSB/cells to a/multiple satellite beams are based on implementation way.

In Rel-18, for power saving at the gNB side, cell level-based DTX/DRX has been introduced, and cell DTX/DRX was introduced in the time domain to allow the network to configure non-active periods, e.g., cell DTX/DRX pattern. The pattern configuration for cell DTX/DRX is common for the UEs in the cell, and the cell DTX and cell DRX patterns can be configured and activated separately. When cell DTX is configured, the UE does not need to monitor PDCCH or SPS occasions during the cell DTX idle period. When cell DRX is configured and activated for the concerned cell, the UE does not transmit on CG resources or SR during the cell DRX idle period. Similarly,  for Rel-19 NTN, TDM-ed beam activation within the extended periodicity for different satellite beam will be needed, it also denotes as “beam hopping” by companies. However, the definition of bean hopping has not yet been finalized. Specifically, it remains unclear whether beam hopping applies to both DL and UL beam pairs, or if it is limited to DL beams while UL beams remain continuously active. From our perspective, to simplify the configuration, beam hopping should be applied to both DL and UL beam pairs. In other words, when a beam is activated, the UE can utilize this beam for both Tx/Rx simultaneously.

Proposal 1: When extended periodicity of SSB is assumed, the definition of “beam hopping” needs to be clarified, e.g., beam hopping can be applied to both DL and UL beam pairs, when a beam is activated, both DL and UL can be regarded as activated simultaneously.

    - Potential solution to improve DL coverage based on system level

In the current specification[4], for operation without shared spectrum channel access and for the SS/PBCH block and CORESET multiplexing pattern 1, a UE monitors PDCCH in the Type0-PDCCH CSS set over two slots. For SS/PBCH block with index i, the UE determines an index of slot n0 as n0=O∙2μ+i∙MmodNslotframe,μ  that is in a frame with system frame number (SFN) SFNC satisfying SFNcmod2=0 if O∙2μ+i∙MNslotframe,μmod2=0, or in a frame with SFN satisfying SFNcmod2=1 if O∙2μ+i∙MNslotframe,μmod2=1 where μ∈0,1,2,3,5,6 based on the SCS for PDCCH receptions in the CORESET [4, TS 38.211]. It means UE need to monitor candidate PDCCH(s) within type0-CSS per 2 radio frame. However, when extended periodicity of default SSB is assumed, the periodic of candidate location of the PDCCH within type0-CSS is also enlarged. To handle this, in our views, the most straightforward way is to define the valid time window associated with the SSB, then UE only need to monitor PDCCH within the time window.

Proposal 2: For the default SSB periodicity extension and beam hopping, a valid time window associated with the SSB for the PDCCH monitoring in the type0-PDCCH CSS can be defined.

In the current specification, the random access procedure imposes a strict timing relationship between an UL signal/data and its corresponding downlink signal/data. Specifically, in response to a PRACH transmission, the UE attempts to detect a DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI during the RAR window. The RAR window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, that is at least one symbol, after the last symbol of the last PRACH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set as defined in clause 10.1. For NTN, the RAR window starts after an additional TTA+kmac msec.

However, during the random access procedure with beam hopping, the RAR window may fall within the deactivation period of a satellite beam. As a result, the UE is unable to receive the RAR from the gNB. A similar issue arises for Msg4 PDSCH reception within the “ra-contentionResolutionTimer”. Thus, the impacts of beam hopping on the random access procedure need to be studied.

Proposal 3: The impacts of “beam hopping” on the random access procedure need to be studied, particularly with respect to the RAR window and RA contention resolution time window.

Link level enhancement

System level-based enhancements to DL coverage improvement are analyzed in section 2.1, this section, however, will concentrate on link-level enhancements designed to enhance DL coverage, proposing potential coverage improvements specifically for the Msg4 PDSCH channels.

In terms of enhancement for PDSCH conveying Msg4, PDSCH with repetition is the most straightforward way, such repetition could be applied in the time domain. To enable PDSCH to convey Msg4 with repetition, at least the repetition number and/or the repetition pattern need to be determined. In the current specification, an additional column associated with the repetition number into the TDRA table for PUSCH has been standardized. Similarly, for enabling Msg4 PDSCH repetition, the mechanism of PUSCH repetition indicator can be re-used. In actual, under the current specifications, for DCI formats containing TC-RNTI, the TDRA table will either be a default table or a table within PDSCH-ConfigCommon, thus, a column associated with Msg4 repetition number can be added into the corresponding table(s). In addition, for the UE capability of Msg4 with repetition reporting, Msg3 PUSCH can be used.

Proposal 4: For enabling Msg4 PDSCH repetition, the repetition number can be joint coding with TDRA.

Proposal 5: The UE capability of Msg4 with repetition needs to be reported, and Msg3 PUSCH can be utilized to carry this information.

- Conclusion

In this contribution, the following proposals have been made:

Proposal 1: When extended periodicity of SSB is assumed, the definition of “beam hopping” needs to be clarified, e.g., beam hopping can be applied to both DL and UL beam pairs, when a beam is activated, both DL and UL can be regarded as activated simultaneously.

Proposal 2: For the default SSB periodicity extension and beam hopping, a valid time window associated with the SSB for the PDCCH monitoring in the type0-PDCCH CSS can be defined.

Proposal 3: The impacts of “beam hopping” on the random access procedure need to be studied, particularly with respect to the RAR window and RA contention resolution time window.

Proposal 4: For enabling Msg4 PDSCH repetition, the repetition number can be joint coding with TDRA.

Proposal 5: The UE capability of Msg4 with repetition needs to be reported, and Msg3 PUSCH can be utilized to carry this information.

4. References

[1] RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3.

[2] 3GPP RAN1 #120bis meeting, Chairman notes.

[3] TR 38.821, Solutions for NR to support non-terrestrial networks (NTN).

[4] TS 38.213, Physical layer procedures for control.