()3GPP TSG RAN WG1 #121			R1-2504841

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.15.5

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary#2 of discussion on Rel-19 NR UE features for LP-WUS/WUR

Document for:	Discussion and Decision

- Introduction

This document summarizes contributions submitted to AI 9.15.5. regarding Rel-19 NR UE features for LP-WUS/WUR.

[121-R19-UE_features] Email discussion on Rel-19 UE features – Ralf (AT&T), Naoya (DOCOMO), Ralf (AT&T) - To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

- Companies’ inputs

The following inputs have been provided in companies’ contributions in this meeting:

| Huawei, HiSilicon | Discussion IDLE/INACTIVE mode (i.e., FG 62-1 and FG 62-1a) 62-1; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK [2. The support of LP-SS based RRM measurement and/or SSB-based measurement, corresponding to component 1] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported; Per Band; n/a; n/a; n/a; Component 1 candidate values: {OOK-based, OFDM-based, both} Component 5 candidate values: FFS [For UE supporting both X-1 and X-2, UE is not allowed to report different candidate values for component 1]; Optional with capability signalling. 62-1a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence [2. FFS: The support of LP-SS based RRM measurement and/or [X. FFS: The support of SSB-based RRM measurement] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence is not supported; Per band; n/a; n/a; n/a; Component 1 candidate values: {OOK-based, OFDM-based, both} Component 5 candidate values: FFS [For UE supporting both X-1 and X-2, UE is not allowed to report different candidate values for component 1]; Optional with capability signalling. The above was agreed as the starting point for further discussion. For the existing components marked as yellow (i.e., unstable), we have the following comments: - For component 2, for a LP-WUR which can monitor LP-WUS via OOK modulation, it is natural to support LP-SS based RRM measurement. For a LP-WUR which can monitor LP-WUS via overlaid sequence, RAN4 has already agreed to define the same requirements for LP-SS based RRM measurement for OFDM based receiver as OOK based receiver [2]. Thus, component 2 should be confirmed for both FG 62-1 and FG 62-1a. - For component 3 and 4, the RRM relaxation/offloading is mainly discussed in RAN2 and RAN4. It is better to leave it to RAN2 or RAN4 to define. Thus, component 3 and 4 can be removed for both FG 62-1 and FG 62-1a. However, based on the simulation results during the study item phase, the power saving gain provided by LP-WUS itself may be marginal, if RRM measurement is not offloaded/relaxed. Therefore, a note should be added for FG 62-1 and FG 62-1a that ‘a UE reporting support of this FG shall support offloading and relaxation’. - For component 5, reporting wakeup delay is a key feature for LP-WUS in IDLE/INACTIVE, which significantly impacts the UE timeline. Thus, component 5 should be confirmed for both FG 62-1 and FG 62-1a, and the candidate values can be further added after they are agreed. - For component X of FG 62-1a, because LP-WUR cannot achieve the same RAN4 accuracy requirement as Rel-15 SSB-based RRM measurement defined for MR and such fact is not clearly reflected in RAN4 progress yet, the component X of FG 62-1a should be pending. Beside the above existing components, the following components are also necessary: - The M value (i.e., number of OOK symbols per OFDM symbol) is a key parameter for LP-WUS, which defines the granularity to process received signal. Per our understanding, supporting all M values is not a big burden to UE. Thus, a new component ‘support of M=1, 2 and 4, where M is the number of OOK symbols per OFDM symbol’ should be added for both FG 62-1 and FG 62-1a. - Beside M value, the SCS is also another key parameter to defines the granularity to process received signal. Also, the SCS impacts the BW of LP-WUS/LP-SS. Therefore, a new component on SCS and BW should be added for both FG 62-1 and FG 62-1a, i.e., ‘support of both SCS 15kHz and 30kHz of CP-OFDM symbol embedded with LP-WUS for FR1 and both SCS 60kHz and 120kHz for FR2. The SCS of CP-OFDM symbol embedded with LP-SS is the same as that of LP-WUS. Both the bandwidths of LP-WUS and LP-SS are 11 PRBs’. - Exit condition is an important UE procedure, which ensures the UE performance for paging information reception. Thus, a new component on should be added for both FG 62-1 and FG 62-1a, i.e., ‘support of exit operation based on configured channel condition threshold’. - It was agreed in RAN1#120 that the maximum number of subgroups per PO is 31. Also, while one to multiple mapping between LO and POs was agreed, there was a restriction saying that the maximum number of codepoints would not increase (i.e., up to 32 codepoints per LO) [4]. Therefore, a new component on should be added for both FG 62-1 and FG 62-1a, i.e., ‘Support of up to 31 number of subgroups per PO and up to 32 codepoint values per LO.’ Based on the discussion above, we have the following proposal - On the UE feature components of LP-WUS in IDLE/INACTIVE mode, - Component 2 and 5 are confirmed for both FG 62-1 and 62-1a - Component 3 and 4 are removed for both FG 62-1 and 62-1a - A note is added: a UE reporting support of this FG shall support offloading and relaxation - New components are added for both FG 62-1 and 62-1a, including - Support of M=1, 2 and 4, where M is the number of OOK symbols per OFDM symbol. - Support of both SCS 15kHz and 30kHz of CP-OFDM symbol embedded with LP-WUS for FR1 and both SCS 60kHz and 120kHz for FR2. The SCS of CP-OFDM symbol embedded with LP-SS is the same as that of LP-WUS. Both the bandwidths of LP-WUS and LP-SS are 11 PRBs. - Support of exit operation based on configured channel condition threshold. - Support of up to 31 number of subgroups per PO and up to 32 codepoint values per LO. CONNECTED mode (i.e., FG 62-2, FG 62-2a and FG 62-3) 62-2; LP-WUS operation in CONNECTED mode based on OOK signal; 1. LP-WUS operation in CONNECTED mode based on OOK signal 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode FFS: M values FFS: SCS FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in CONNECTED mode based on OOK signal is not supported; [Per Band]; n/a; n/a; n/a; Component 2 candidate values: FFS Component 3 candidate values: FFS; Optional with capability signalling. 62-2a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence; 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode FFS: M values FFS: SCS FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported; [Per Band]; n/a; n/a; n/a; Component 2 candidate values: FFS Component 3 candidate values: FFS; Optional with capability signalling. 62-3; LP-WUS frequency resource outside active DL BWP in addition to the one within active DL BWP for LP-WUS operation in CONNECTED mode; 1. Support of LP-WUS frequency resource outside active DL BWP in addition to the one within active DL BWP for LP-WUS operation in CONNECTED mode; FG 62-2 or 62-2a; YES; n/a; LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported; [Per band]; n/a; n/a; n/a; Optional with capability signalling. The above was agreed as the starting point for further discussion. For the existing components marked as yellow (i.e., unstable), we have the following comments: - For component 2, currently two options of basic UE procedure are agreed, i.e., Option 1-1 (where LP-WUS triggers drx-OnDurationTimer) and Option 1-2 (where LP-WUS triggers a new timer). In our view, there is no different hardware requirement for supporting the two options. And we suggest to have a unified UE feature for the two options on UE procedure in CONNECTED mode. Thus, component 2 should be modified for both FG 62-2 and FG 62-2a as ‘support UE procedure based on both Option 1-1 and Option 1-2’. - For component 3, Minimum time gap acts as the same role as the wakeup delay, which is a key feature for LP-WUS in CONNECTED mode and significantly impacts the UE timeline. Thus, component 3 should be confirmed for both FG 62-2 and FG 62-2a, and the candidate values can be further added after they are agreed. Beside the above existing components, the following components are also necessary: - For the similar analysis as in Section 2.1, a new component ‘support of M=1, 2 and 4, where M is the number of OOK symbols per OFDM symbol’ should be added for both FG 62-2 and FG 62-2a. - For the similar analysis as in Section 2.1, a new component on SCS and BW should be added for both FG 62-2 and FG 62-2a while LP-SS is not necessarily mentioned, i.e., ‘support of both SCS 15kHz and 30kHz of CP-OFDM symbol embedded with LP-WUS for FR1 and both SCS 60kHz and 120kHz for FR2. The bandwidths of LP-WUS are 11 PRBs’. - It was agreed in RAN1#120 that the maximum number of codepoints to be checked by UE per MO depends on UE capability [4]. Therefore, a new component should be added for both FG 62-2 and FG 62-2a, i.e., ‘the maximum number of codepoints to be checked by UE per MO’. - It was agreed in RAN1#120 that a basic UE capability is to receive LP-WUS within active DL BWP [4]. Therefore, a new component should be added for both FG 62-2 and FG 62-2a, i.e., ‘support of receiving LP-WUS within Active BWP’. Besides, now columns should be added - In RAN1 #118bis, it was agreed that UE can indicate its preference for configured time offset between LP-WUS and PDCCH reception using UAI mechanism, where UAI is optional [3]. Therefore, a new row should be added, i.e., ‘support of UAI indication of additional [larger] offset where the offset is between the triggering LP-WUS and triggered PDCCH for LP-WUS in CONNECTED mode’. The Prerequisite feature groups of this newly added row can be 62-2 and/or 62-2a. Based on the discussion above, we have the following proposal - On the UE features of LP-WUS in CONNECTED mode, - Component 2 is updated as ‘support UE procedure based on both Option 1-1 and Option 1-2’. - Component 3 is confirmed - New components are added for both FG 62-2 and 62-2a, including - Support of M=1, 2 and 4, where M is the number of OOK symbols per OFDM symbol. - Support of both SCS 15kHz and 30kHz of CP-OFDM symbol embedded with LP-WUS for FR1 and both SCS 60kHz and 120kHz for FR2. The bandwidths of LP-WUS are 11 PRBs. - The maximum number of codepoints to be checked by UE per MO - Support of receiving LP-WUS within Active BWP. - New rows are added, for which the prerequisite feature groups is 62-2 and/or 62-2a - Support of UAI indication of additional [larger] offset where the offset is between the triggering LP-WUS and triggered PDCCH for LP-WUS in CONNECTED mode Type (granularity) of UE feature Agreement Send an LS to RAN2 and RAN4 to convey the following In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. Request RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs. Final LS in R1-2407559. Agreement: Adopt “Per band” for FG 62-1 and 62-1a:. Depending on different UE implementation, LP-WUR may be supported for different bands (see the agreed LS above). Some discussion in RAN1/RAN2/RANP also considers the case that LP-WUS reception is only supported by UE in some of the bands. Therefore, the type (i.e., the granularity) of UE feature for LP-WUS should be per band for single carrier operation. Additionally, in RAN1#120bis, “per band” was adopted for FG 62-1 and FG 62-1a. Thus, for CONNECTED mode, for single carrier operations, the type should be also confirmed as “per Band”. - The type (i.e., the granularity) of UE feature for LP-WUS is per band for single carrier operation in CONNECTED mode. Summary According to the above discussion, we have the following proposal. - The table 1 in Appendix is adopted. |
| --- | --- |
| ZTE Corporation, Sanechips | 2 Discussions Issue1: 62-1 components For 62-1, the following needs further discussion [2. The support of LP-SS based RRM measurement and/or SSB-based measurement, corresponding to component 1] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation. From our understanding, component2 and component 5 can be confirmed. However, for component 3, we do not need to introduce this kind of UE capability in RAN1. Regarding the wake-up delay, the exact values can be further confirmed. As for M values, M={1 2 4} should be confirmed. Proposal 1: for 62-1 - Component 2 and componen4 can be confirmed. - it is assumed in RAN1 component2 would be introduced in RAN2/RAN4 - M values include {1 2 4} Issue 2: 62-1a components For 62-1a, the following needs further discussion [2. FFS: The support of LP-SS based RRM measurement and/or [X. FFS: The support of SSB-based RRM measurement] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation. For 62-1a, since introducing LP-SS based measurement for OFDM WUR would introduce additional complexity, it is needed to discuss whether it is supported as additional optional feature. If it is optional feature, we may need further determine how to introduce it, otherwise, it could be combined together with component X. Other components are similar as 62-1. Proposal 2: - Component X and componen4 can be confirmed. - it is assumed in RAN1 component3 would be introduced in RAN2/RAN4 - M values include {1 2 4} - Discuss component 2 is optional or not Issue 3: 62-2/62-2a components For 62-2/62-2a, the following needs further discussion 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode FFS: M values FFS: SCS FFS: Other components and/or further FG separation. One thing is we may need to clarify what’s the difference between component 1 and component2. In our understanding, in component1, the LP-WUS operation also can includes the procedures. Another thing is for connected mode, an additional discussion comes from the following agreement Working Assumption Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs: - Codepoint based - The maximum number of codepoints checked per MO by a UE is up to 8 - Depending on UE capability, a UE may support less than 8. A component may be needed to describe the maximum number of codepoints checked per MO. As for the exact values, e.g., 4 or 8 could be considered. Proposal 3: For 62-2/62-2a - consider a component to describe the the maximum number of codepoints checked per MO. - clarify the difference between component1 and component2 |
| vivo | - NR_LPWUS - Proposal with change mark (based on RAN1#120 endorsed version in R1-2501397 and RAN1#120bis further agreements in R1-2502966) Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (Sidelink WI only)”.; Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. 62. NR_LPWUS; 62-1; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK 2. The support of LP-SS based RRM measurement with all M values (M=1,2,4) for LP-SS 3. The support of further RRM relaxation of UE MR 4. The support of serving RRM measurement offloading from UE MR to LR 5. The supported wake-up delay 6. The supported M values for LP-WUS 7. The support of LP-WUS/LP-SS frequency resource within or outside the initial DL BWP where UE receives paging but within the same carrier FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported; Per Band; n/a; n/a; n/a; Component 5 candidate values: - For SSB periodicity 20ms, candidate values:[70]ms, [500]ms, [900]ms - FFS for other SSB periodicities. Component 6: - For 15KHz SCS: support all M values (M=1,2,4) - For 30KHz SCS: Support one of {M=1,2 or M=1,2,4} - For 60KHz SCS: FFS - For 120KHz SCS: FFS; Optional with capability signalling. 62. NR_LPWUS; 62-1a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence(s); 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence(s), all M values (M=1,2,4) supported 2. FFS: The support of LP-SS based RRM measurement X. The support of SSB-based RRM measurement 3. The support of further RRM relaxation of UE MR 4. The support of serving RRM measurement offloading from UE MR to LR 5. The supported wake-up delay 6.The support of LP-WUS/LP-SS frequency resource within or outside the initial DL BWP where UE receives paging but within the same carrier 7. The support of maximum 16, 8, 4 candidate overlaid sequences for M=1,2,4, respectively. FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence(s) is not supported; Per Band; n/a; n/a; n/a; Component 5 candidate values: - For SSB periodicity 20ms, candidate values:[70]ms, [500]ms, [900]ms - FFS for other SSB periodicities.; Optional with capability signalling. 62. NR_LPWUS; 62-2; LP-WUS operation in CONNECTED mode based on OOK signal; - LP-WUS operation in CONNECTED mode to trigger PDCCH monitoring based on OOK signal，all M values (M=1,2,4) supported - Support of LP-WUS frequency resource within active DL BWP - The supported procedure(s) - Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode - The supported maximum number of codepoints monitored per LP-WUS MO FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in CONNECTED mode based on OOK signal is not supported; Per band; n/a; n/a; n/a; Component 3 candidate values: (Option 1-1, Option 1-2, both) Component 4 candidate values: {V1=[3,5,6], V2=[10,13,20], V3=[33,42]}ms where V1<V2<V3 Component 5: candidate value: any integer value in the range of 2~8; Optional with capability signalling. 62. NR_LPWUS; 62-2a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s); - LP-WUS operation in CONNECTED mode to trigger PDCCH monitoring based on OFDM overlaid sequence(s), all M values (M=1,2,4) supported - Support of LP-WUS frequency resource within active DL BWP - The supported procedure(s) - Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode - The supported maximum number of codepoints monitored per LP-WUS MO - The support of maximum 16, 8, 4 candidate overlaid sequences for M=1,2,4, respectively. FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported; Per band; n/a; n/a; n/a; Component 3 candidate values: (Option 1-1, Option 1-2, both) Component 4 candidate values: {V1=[3,5,6], V2=[10,13,20], V3=[33,42]}ms where V1<V2<V3 Component 5: candidate value: any integer value in the range of 2~8; Optional with capability signalling. 62. NR_LPWUS; 62-3; LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode; 1. Support of LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode; FG 62-2 or 62-2a; YES; n/a; LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| Nokia | Discussion and proposals IDLE/INACTIVE mode FGs 62-1 and 62-1a The outcome of RAN1#120bis resulted with two FGs for idle/inactive mode [1]: We propose the following modifications: Proposals on FG62-1: - Component 2: Remove square brackets - Component 3: Modify the description to reflect that the UE is configurable with the relaxed requirements: - [3. The support for configuration of relaxed RRM requirements of further RRM relaxation of UE MR] - Component 4: Remove the square brackets and modify the description to reflect that the UE is configurable with the RRM measurement offloading: [4. The support for configuration of serving cell RRM measurement offloading] - Component 5: Remove square brackets - Component 6 (new): Add a component for supported M values and corresponding L-values and remove the corresponding FFS: 6. The supported M and L values for OOK - M=1, L = {6,8} - M=2, L = {12,16} - M=4, L = {16,32} - FFS on other components: Remove the FFS line Proposals on FG62-1a: - FG name and Component 1: Remove the word ‘overlaid’, as the sequence is a sequence, there is no different sequence or different FG for support of OFDM sequence that is not overlaid. - Component 2: Remove square brackets and FFS - Component X: Remove square brackets and FFS, renumber as component 2a - Component 3: Remove square brackets and clarify the description and confirm the component, and avoid specific references to a receiver type: [3. The support for configuration of relaxed RRM requirements of further RRM relaxation of UE MR] - Component 4: Remove the square brackets and modify the description to reflect that the UE is configurable with the RRM measurement offloading: [4. The support for configuration of serving cell RRM measurement offloading] - Component 5: Remove square brackets. Add component values in [] Component 5 candidate values: FFS [{Capability 1, Capability 2, Capability 3}] - Component 6 (new): Add a component for supported M values and corresponding L-values and remove the corresponding FFS: 6. The supported M and L values for OOK - M=1, L = {6,8} - M=2, L = {12,16} - M=4, L = {16,32} - FFS on other components: Remove the FFS line. Resulting with the following updates to FG62-1 and 62-1a: Index; Feature group; Components; Prerequisite feature groups; Type; Note. 62-1; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK [2. The support of LP-SS based RRM measurement and/or SSB-based measurement, corresponding to component 1] [3. The support for configuration of relaxed RRM requirements of further RRM relaxation of UE MR] [4. The support for configuration of serving cell RRM measurement offloading] [5. The supported wake-up delay] 6. The supported M and L values for OOK - M=1, L = {6,8} - M=2, L = {12,16} - M=4, L = {16,32} FFS: M values FFS: Other components and/or further FG separation; Per Band; Component 5 candidate values: FFS [{Capability 1, Capability 2, Capability 3}]. 62-1a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence [2. FFS: The support of LP-SS based RRM measurement and/or [X. FFS: 2a. The support of SSB-based RRM measurement] [3. The support for configuration of relaxed RRM requirements of further RRM relaxation of UE MR] [4. The support for configuration of serving cell RRM measurement offloading] [5. The supported wake-up delay] 6. The supported M and L values for OOK - M=1, L = {6,8} - M=2, L = {12,16} - M=4, L = {16,32} FFS: M values FFS: Other components and/or further FG separation; Per band; Component 5 candidate values: FFS [{Capability 1, Capability 2, Capability 3}]. CONNECTED mode FGs 62-2, 62-2a and 62-3 The outcome of RAN1#120bis resulted with three FGs for CONNECTED mode [1]: We propose the following modifications: Proposals on FG62-2 and 62-2a: - FG 62-2a name: Remove the word ‘overlaid’, as the sequence is a sequence, there is no different sequence or different FG for support of OFDM sequence that is not overlaid. - FG 62-2a component 1: Remove the word ‘overlaid’, as the sequence is a sequence, there is no different sequence or different FG for support of OFDM sequence that is not overlaid. - FG 62-2a consequence if not supported: Remove the word ‘overlaid’, as the sequence is a sequence, there is no different sequence or different FG for support of OFDM sequence that is not overlaid. - Component 2: Confirm the component, add candidate values {1-1, 1-1 and 1-2} and clarify to what the procedures refer to. 1-1: LP-WUS monitoring before the drx-onDurationTimer 1-2: P-WUS monitoring outside C-DRX active time Component 2 candidate values: FFS {1-1, 1-1 and 1-2} - Component 3: Confirm the component. Update the candidate values to reflect the current situation in the WI, final values to be defined within the work item agenda. Component 3 candidate values: FFS {[3,5,6], [10,13,20], [33,42]} ms. - FFS, M values: Make this a component 5, with M values 1, 2 and 4 supported with the corresponding L-values agreed in RAN1#120bis. 5. The supported M and L values for OOK - M=1, L = {6,8} - M=2, L = {12,16} - M=4, L = {16,32} - FFS, SCS: Remove the FFS - FFS, Other components: Remove the FFS - FG type: Make the FGs ‘Per Band’ Proposals on FG62-3: - FG type: Make the FG ‘Per Band’ Index; Feature group; Components; Consequence if the feature is not supported by the UE; Prerequisite feature groups; Type; Note. 62-2; LP-WUS operation in CONNECTED mode based on OOK signal; 1. LP-WUS operation in CONNECTED mode based on OOK signal 2. The supported procedure(s) 1-1: LP-WUS monitoring before the drx-onDurationTimer 1-2: P-WUS monitoring outside C-DRX active time 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP 5. The supported M and L values for OOK - M=1, L = {6,8} - M=2, L = {12,16} - M=4, L = {16,32} FFS: M values FFS: SCS FFS: Other components and/or further FG separation; LP-WUS operation in CONNECTED mode based on OOK signal is not supported; [Per band]; Component 2 candidate values: FFS {1-1, 1-1 and 1-2} Component 3 candidate values: FFS. 62-2a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence; 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. The supported procedure(s) 1-1: LP-WUS monitoring before the drx-onDurationTimer 1-2: P-WUS monitoring outside C-DRX active time 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP 5. The supported M and L values for OOK - M=1, L = {6,8} - M=2, L = {12,16} - M=4, L = {16,32} FFS: M values FFS: SCS FFS: Other components and/or further FG separation; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported; [Per band]; Component 2 candidate values: FFS {1-1, 1-1 and 1-2} Component 3 candidate values: FFS {[3,5,6], [10,13,20], [33,42]} ms.. 62-3; LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode; 1. Support of LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode; LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported; FG 62-2 or 62-2a; [Per band]. |
| Samsung | LP-WUS operation in RRC IDLE/INACTIVE mode FG 62-1 and 62-1a We suggest the merging of the possible “M” values for OOK-4 operation into the first component as a mandatory feature for all UEs supporting LP-WUS. This is because, it is difficult and inefficient for the gNB to group the UEs based on their supported M values especially for OOK-based LP-WUS signalling. For the supported values of M corresponding to each SCS, M=4 with 30kHz is still a working assumption and has not been confirmed yet, so we keep it in the brackets. For components 2 in FG 62-1, we believe the existing proposals by FL are sufficient, thus we suggest removing highlights and brackets. However, for FG 62-1a, which channel/signal should be mandatory to perform RRM measurement should be further discussed considering the related issues (e.g., whether LP-SS transmission in RRC IDLE/INACTIVE mode can be configured by the gNB, the case when the overlaid OFDM sequence is not configured by the gNB for LP-SS with M=1). Thus, we keep the brackets for component For components 3, and 4 in FG 62-1/62-1a, these components are related to RAN2/RAN4 requirement for MR RRM measurement relaxation and RRM measurement offloading to LR. In RAN1, how to measure the quantities based on LP-SS have been mainly discussed rather than designing the requirement or conditions for RRM measurement, we suggest to leave them for RAN2/RAN4 UE feature discussion. Since the potential candidate values for wake-up delay have not been agreed yet, we suggest removing the brackets around component 5 in FG 62-1/62-1a, but keeping the potential candidate values still as an FFS. Proposal 1: Update FGs 62-1 and 62-1a as follows: - RRM measurement relaxation/offloading related components can be left to RAN2/RAN4 UE feature discussion 62. NR_LPWUS; 62-1; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK signal with M=1,2, [and 4] [2. The support of LP-SS based RRM measurement] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [53. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported; Per Band; n/a; n/a; n/a; Component 53 candidate values: FFS; Optional with capability signalling. 62. NR_LPWUS; 62-1a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence with M=1,2, [and 4] [2. FFS: The support of LP-SS based RRM measurement and/or [X3. FFS: The support of SSB-based RRM measurement] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [54. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence is not supported; Per band; n/a; n/a; n/a; Component 54 candidate values: FFS; Optional with capability signalling. - LP-WUS operation in RRC CONNECTED mode FG 62-2 and 62-2a In this section, we focus attention on the RRC CONNECTED mode. Similar to FG 62-1 and FG 62-1a, we suggest that these feature groups should be supported per band and thus suggest the removal of the brackets. We also suggest the merging of the possible “M” values for OOK-4 operation into the first component as a mandatory feature for all UEs supporting LP-WUS. This is because, it is difficult and inefficient for the gNB to group the UEs based on their supported M values especially for OOK-based LP-WUS signalling. For the supported values of M corresponding to each SCS, M=4 with 30kHz is still a working assumption and has not been confirmed yet, so we keep it in the brackets. Since the potential candidate values for minimum time gap between LP-WUS reception and UE to start PDCCH monitoring have not been agreed yet, we suggest removing the highlight for component 3 in FG 62-2 and FG 62-2a, but keeping the potential candidate values still as FFS. We don’t see the need for adding new components based on SCS since these can follow those of the MR and thus suggest the removal of the corresponding FFS. Proposal 2: Update FGs 62-2 and 62-2a as follows: 62. NR_LPWUS; 62-2; LP-WUS operation in CONNECTED mode based on OOK signal; 1. LP-WUS operation in CONNECTED mode based on OOK signal with M=1,2, [and 4] 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP FFS: M values FFS: SCS FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in CONNECTED mode based on OOK signal is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: FFS Component 3 candidate values: FFS; Optional with capability signalling. 62. NR_LPWUS; 62-2a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence; 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) with M=1,2, [and 4] 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP FFS: M values FFS: SCS FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: FFS Component 3 candidate values: FFS; Optional with capability signalling. FG 62-3 In the last RAN1 meeting #120-bis, a new UE capability was added for LP-WUS frequency resource selection outside active DL BWP, i.e. FG 62-3. We suggest to support this FG per band. Proposal 3: Update FG 62-3 for RRC CONNECTED UEs as follows: 62. NR_LPWUS; 62-3; LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode; 1. Support of LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode; FG 62-2 or 62-2a; YES; n/a; LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported; [Per band]; n/a; n/a; n/a; Optional with capability signalling. |
| CATT | UE Features of LP-WUS/LP-WUR in RAN1 The LP-WUS/WUR work is to design a LP-WUS signal to trigger UE wakeup with UE in IDLE/Inactive and CONNECTED mode to achieve the UE power saving. The UE features of LP-WUS including the UE supporting the LP-WUS/WUR and LP-SS, the IDLE/Inactive UE support of LP-WUS triggering wakeup and paging subgrouping, the UE report of the wakeup delay in IDLE/Inactive state. It was agreed in RAN1#120 that the UE feature group of supporting LP-WUS in the IDLE/Inactive state would have two subgroups with LP-WUS OOK and OFDM receivers. The support of RRM measurements for the OOK/OFDM receiver would measure the LP-SS for the entry/exit condition of LP-WUS monitoring and RRM measurement offloading since the coverage of the LP-WUS signals might not be the same as that of NR signals/channels. For OFDM receivers, the RRM measurements based on LP-SS or SSB should be the UE implementation since the entry/exist condition would be determined by the UE with their LP-WUS implementation. However, the RRM relaxation should be part of the RRM measurement as one package since the RRM measurements would not provide the UE power saving. The support of wakeup delay of OOK receiver should also be included in the component. Proposal 1: The UE support of RRM measurements with relaxation for entry/exit condition of LP-WUS monitoring and RRM measurement offloading should be the component. The support of wakeup delay indication should be included in the component since the transition time from the power saving state to the NR operation state is different for LP-WUS receiver. Proposal 2: The RRM measurements of LP-WUS OFDM receivers should not include the component of measurements based on LP-SS or SSB. Since the OOK-4 waveform has different M values, all M values of LP-WUS and LP-SS waveforms should be supported. It should not have a UE capability indication for UE supporting subset of M values of OOK-4 waveforms. If UE only supports subset of M values of OOK-4 waveform with UE capability indications, the network could not operate the LP-WUS for all UEs with different capabilities. Proposal 3: The LP-WUS OOK receiver should support all specified M-values without UE capability Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (Sidelink WI only)”.; Consequence if the feature is not supported by the UE; Type; Features; Index; Feature group; Components; Prerequisite feature groups. NR_LPWUS; 62-1; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK 2. The support of LP-SS based RRM measurement 3. The support of further RRM relaxation of UE MR 4 The support of serving RRM measurement offloading 5. The supported wake-up delay; No; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported; Per Band; n/a; n/a; n/a; Candidate values: 70, 500, 900 ms; Optional with capability signalling. NR_LPWUS; 62-1a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM sequence; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence. 2. The support of SSB-based RRM measurement 3. The support of serving RRM measurement offloading 4. The supported wake-up delay; No; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM is not supported; Per Band; n/a; n/a; n/a; Candidate values: 70, 500, 900 ms; Optional with capability signalling. The CONNECTED mode UE support of LP-WUS triggering the drxOnDurationTimer and the CONNECTED mode UE support of LP-WUS triggering the new Timer. The UE support of LP-WUS triggering UE wakeup in CONNECTED state should support the C-DRX adaptation of drxOnDurationTimer in option 1-1 and new Timer in option 1-2. Since UE needs to have at least one SSB to perform time/frequency tracking and channel estimation update for coherent PDCCH demodulation when drxOnDurationTimer or New Timer is triggered. Thus, the minimum gap between the reception and PDCCH monitoring for UE in CONNECTED state should have candidate value 20 ms. Proposal 4: The candidate value of minimum gap between the reception and PDCCH monitoring for UE in CONNECTED state should have candidate value 20 ms. NR_LPWUS; 62-2; LP-WUS operation in CONNECTED mode based on OOK signal; 1. LP-WUS operation in CONNECTED mode based on OOK-1 and OOK-4 for all M-values 2. The support of drxOnDuration timer and new Timer 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode; No; YES; n/a; LP-WUS operation in CONNECTED mode based on OOK signal is not supported; Per Band; n/a; n/a; n/a; Component 3: 20 ms; Optional with capability signalling. NR_LPWUS; 62-2a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence; 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. The support of drxOnDuration timer and new Timer 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode; NO; YES; n/a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported; Per Band; n/a; n/a; n/a; Component 3: 20 ms; Optional with capability signalling. It was agreed in RAN1#120 that the LP-WUS configuration could be outside Active DL BWP for the CONNECTED mode UE support of LP-WUS triggering the drxOnDurationTimer and the new Timer. The UE capability FG 62-3 was agreed to capture the capability. The pre-requisite of the FG 62-3 should be the FG 62-2 or 62-3 that UE supported the LP-WUS in CONNECTED mode based on either OOK signal or OFDM overlaid sequence. The FG 62-3 should be per band with the update feature list as follows, NR_LPWUS; 62-3; LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode; 1. Support of LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode; FG 62-2 or 62-2a; YES; n/a; LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported; Per band; n/a; n/a; n/a; Optional with capability signalling. Proposal 5: The pre-requisite of the FG 62-3 should be the FG 62-2 or 62-3 that UE supported the LP-WUS in CONNECTED mode based on either OOK signal or OFDM overlaid sequence Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (Sidelink WI only)”.; Consequence if the feature is not supported by the UE; Type; Features; Index; Feature group; Components; Prerequisite feature groups. NR_LPWUS; 62-1; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK 2. The support of LP-SS based RRM measurement 3. The support of further RRM relaxation of UE MR 4 The support of serving RRM measurement offloading 5. The supported wake-up delay; No; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported; Per Band; n/a; n/a; n/a; Candidate values: 70, 500, 900 ms; Optional with capability signalling. NR_LPWUS; 62-1a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM sequence; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence. 2. The support of SSB-based RRM measurement 3. The support of serving RRM measurement offloading 4. The supported wake-up delay; No; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM is not supported; Per Band; n/a; n/a; n/a; Candidate values: 70, 500, 900 ms; Optional with capability signalling. NR_LPWUS; 62-2; LP-WUS operation in CONNECTED mode based on OOK signal; 1. LP-WUS operation in CONNECTED mode based on OOK-1 and OOK-4 for all M-values 2. The support of drxOnDuration timer and new Timer 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode; No; YES; n/a; LP-WUS operation in CONNECTED mode based on OOK signal is not supported; Per Band; n/a; n/a; n/a; Component 3: 20 ms; Optional with capability signalling. NR_LPWUS; 62-2a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence; 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. The support of drxOnDuration timer and new Timer 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode; NO; YES; n/a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported; Per Band; n/a; n/a; n/a; Component 3: 20 ms; Optional with capability signalling. NR_LPWUS; 62-3; LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode; 1. Support of LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode; FG 62-2 or 62-2a; YES; n/a; LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| CMCC | 2 Discussion - LP-WUS operation in IDLE/INACTIVE mode During RAN1 discussion, we have the following agreement related with the UE feature for LP-WUS operation in IDLE/INACTIVE mode [1]. Agreement: Adopt “Per band” for FG 62-1 and 62-1a: Agreement: Update FG 62-1 and 62-1a as follows: 62-1; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK [2. The support of LP-SS based RRM measurement and/or SSB-based measurement, corresponding to component 1] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported; Per Band; n/a; n/a; n/a; Component 1 candidate values: {OOK-based, OFDM-based, both} Component 5 candidate values: FFS [For UE supporting both X-1 and X-2, UE is not allowed to report different candidate values for component 1]; Optional with capability signalling. 62-1a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence [2. FFS: The support of LP-SS based RRM measurement and/or [X. FFS: The support of SSB-based RRM measurement] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence is not supported; Per band; n/a; n/a; n/a; Component 1 candidate values: {OOK-based, OFDM-based, both} Component 5 candidate values: FFS [For UE supporting both X-1 and X-2, UE is not allowed to report different candidate values for component 1]; Optional with capability signalling.. Regarding the FFS part (i.e. wording in yellow highlight), our considerations are as follows: - Regarding the FFS part on Components column, we support to keep the wording in bracket according to the previous RAN1 agreement. Moreover, regarding the FFS on M values, our understanding is that different M values will not cause significant difference to the UE implementation. Thus, we suggest to remove the FFS. - Regarding the FFS part on Prerequisite feature groups, our view is that there is no direct relation between 62-1 and 62-1a since UE may only support one type of LP-WUS. - Regarding the FFS part on Note column, since RAN1 has sent a LS to RAN4 for the specific values, it is recommended to keep the FFS until RAN4 send the LS reply back. - One more thing worth noting is whether to introduce a separate FG for overlaid sequence not carrying information. Our view is that since the UE supports overlaid sequence has to anyway perform correlation in time domain according to the boundary of slot/symbol, such UE capability can be included within FG 62-1a. Proposal 1: Take the following UE feature list for the feature of LP-WUS operation in IDLE/INACTIVE mode: 62-1; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK 2. The support of LP-SS based RRM measurement 3. The support of further RRM relaxation of UE MR 4. The support of serving RRM measurement offloading 5. The supported wake-up delay; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported; Per Band; n/a; n/a; n/a; Component 5 candidate values: FFS; Optional with capability signalling. 62-1a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence 2. FFS: The support of LP-SS based RRM measurement 3. The support of SSB-based RRM measurement 4. The support of further RRM relaxation of UE MR 5. The support of serving RRM measurement offloading 6. The supported wake-up delay; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence is not supported; Per band; n/a; n/a; n/a; Component 5 candidate values: FFS; Optional with capability signalling. - LP-WUS operation in CONNECTED mode During RAN1 discussion, we have the following agreement related with the UE feature for LP-WUS operation in CONNECTED mode [1-2]. Agreement: Introduce the following FG(s), with FFSs highlighted yellow: X-2; LP-WUS operation in CONNECTED mode based on OOK signal; 1. LP-WUS operation in CONNECTED mode based on OOK signal 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode FFS: M values FFS: SCS FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in CONNECTED mode based on OOK signal is not supported; [Per Band]; n/a; n/a; n/a; Component 2 candidate values: FFS Component 3 candidate values: FFS; Optional with capability signalling. X-2a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence; 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode FFS: M values FFS: SCS FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported; [Per Band]; n/a; n/a; n/a; Component 2 candidate values: FFS Component 3 candidate values: FFS; Optional with capability signalling. Agreement: - Define new FG 62-3 for the support of LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode - Add a component in FG 62-2/2a for the support of LP-WUS frequency resource within active DL BWP 62-3; LP-WUS frequency resource outside active DL BWP in addition to the one within active DL BWP for LP-WUS operation in CONNECTED mode; 1. Support of LP-WUS frequency resource outside active DL BWP in addition to the one within active DL BWP for LP-WUS operation in CONNECTED mode; FG 62-2 or 62-2a; YES; n/a; LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported; [Per band]; n/a; n/a; n/a; Optional with capability signalling.. Regarding the FFS part (i.e. wording in yellow highlight), our considerations are as follows: - Regarding the FFS part on Components column, we support to keep the wording in bullets according to the previous RAN1 agreement. Moreover, regarding the FFS on M values and SCS, our understanding is that different M values will not cause significant difference to the UE implementation. Thus, we suggest to remove the FFS. - Regarding the FFS part on Prerequisite feature groups, our view is that there is no direct relation between 62-2 and 62-2a since UE may only support one type of LP-WUS. - Regarding the FFS part on Type column, we support the current wording. - Regarding the FFS part on Note column, since RAN1 has already agreed that UE does not monitor LP-WUS during C-DRX active time and Option 1-1 and Option 1-2 are simultaneously configured for the same UE, only {Option 1-1, Option 1-2} can be supported for supported procedures. Furthermore, since RAN1 is still discussing the specific values of minimum time gap, it is recommended to keep the FFS until RAN1 or other RAN WG finalize such issue. Proposal 2: Take the following UE feature list for the feature of LP-WUS operation in CONNECTED mode: 62-2; LP-WUS operation in CONNECTED mode based on OOK signal; 1. LP-WUS operation in CONNECTED mode based on OOK signal 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in CONNECTED mode based on OOK signal is not supported; Per Band; n/a; n/a; n/a; Component 2 candidate values: {Option 1-1, Option 1-2} Component 3 candidate values: FFS; Optional with capability signalling. 62-2a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence; 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported; Per Band; n/a; n/a; n/a; Component 2 candidate values: {Option 1-1, Option 1-2} Component 3 candidate values: FFS; Optional with capability signalling. 62-3; LP-WUS frequency resource outside active DL BWP in addition to the one within active DL BWP for LP-WUS operation in CONNECTED mode; 1. Support of LP-WUS frequency resource outside active DL BWP in addition to the one within active DL BWP for LP-WUS operation in CONNECTED mode; FG 62-2 or 62-2a; YES; n/a; LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported; Per Band; n/a; n/a; n/a; Optional with capability signalling. |
| Xiaomi | Discussion In RAN1 #120 meeting, we had initial discussion and made agreement on UE features for NR LP-WUS/WUR. Generally, the UE feature is split by different LR types and by separate functions of RRC idle/inactive and RRC connected. For RRC connected mode, LP-WUS is supported for DC, and the following agreement was achieved in RAN1#118 and RAN1 #120bis meeting, Agreement LP-WUS is supported when UE is configured with NR-DC in RRC CONNECTED mode - Above is supported for the case PDCCH monitoring is triggered by LP-WUS in the same cell group - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable. Agreement For the case when UE is configured with NR-DC with CA without dual DRX groups or without CA in RRC CONNECTED mode, LP-WUS can be configured only on PCell/PSCell per cell-group. In DC scenario, if UE needs to receive LP-WUS on both MCG and SCG, UE should be able to monitor 2 LP-WUS simultaneously on different carriers. And currently, RAN2 is also discussing monitor 2 LP-WUS simultaneously on dual C-DRX group. The supported band combination of simultaneously LP-WUS monitoring on different carriers should be indicated in capability signalling for RRC connected mode. Proposal 1: The supported band combination of simultaneously LP-WUS monitoring on different carriers should be indicated in capability signalling for RRC connected mode. In our companion contribution[1], it is proposed that UE can report its capability to indicate whether it support different M values between LP-SS and LP-WUS. And also, the maximum number of OFDM overlaid sequences can be checked by LR per OOK ON chip should also be a UE capability. So we suggest to add those capabilities to FG 62-1/1a, and FG 62-2a separately. And for the supported SCS, 15kHz/30kHz are the candidate values involved in previous simulations and discussion, and should be included in UE capability reporting. Based on the agreed UE feature table made in last meeting, the following Table 1 is proposed for UE feature discussion related to LP-WUS/WUR. To make it clear for review, the changed/new added parts compared to the table agreed in last meeting is highlighted with blue. Table 1: Proposed UE feature groups for Rel-19 LP-WUS/WUR in NR Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (Sidelink WI only)”.; Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. NR_LPWUS-Core; 62-1; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK [2. The support of LP-SS based RRM measurement and/or SSB-based measurement, corresponding to component 1] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] 6. Whether support different M values between LP-SS and LP-WUS FFS: M values FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported; Per Band; n/a; n/a; n/a; Component 1 candidate values: {OOK-based, OFDM-based, both} Component 5 candidate values: FFS Component 6 candidate values: {Yes, No} [For UE supporting both X-1 and X-2, UE is not allowed to report different candidate values for component 1]; Optional with capability signalling. 62-1a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence [2. FFS: The support of LP-SS based RRM measurement and/or [X. FFS: The support of SSB-based RRM measurement] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] 6. Whether support different M values between LP-SS and LP-WUS 7. The supported SCS for LR FFS: M values FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence is not supported; Per band; n/a; n/a; n/a; Component 1 candidate values: {OOK-based, OFDM-based, both} Component 5 candidate values: FFS Component 6 candidate values: {Yes, No} Component 7 candidate values: {15kHz, 30kHz, both} [For UE supporting both X-1 and X-2, UE is not allowed to report different candidate values for component 1]; Optional with capability signalling. 62-2; LP-WUS operation in CONNECTED mode based on OOK signal; 1. LP-WUS operation in CONNECTED mode based on OOK signal 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode FFS: M values FFS: SCS FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in CONNECTED mode based on OOK signal is not supported; [Per Band] or per BC; n/a; n/a; n/a; Component 2 candidate values: FFS Component 3 candidate values: FFS; Optional with capability signalling. 62-2a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence; 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. The supported maximum number of OFDM overlaid sequences can be checked by LR per OOK ON chip. 5. The supported SCS for LR FFS: M values FFS: SCS FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported; [Per Band] or per BC; n/a; n/a; n/a; Component 2 candidate values: FFS Component 3 candidate values: FFS Component 4 candidate values: {2,4,6,8} Component 5 candidate values:{15kHz, 30kHz, both}; Optional with capability signalling. 62-3; LP-WUS frequency resource outside active DL BWP in addition to the one within active DL BWP for LP-WUS operation in CONNECTED mode; 1. Support of LP-WUS frequency resource outside active DL BWP in addition to the one within active DL BWP for LP-WUS operation in CONNECTED mode; FG 62-2 or 62-2a; YES; n/a; LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported; [Per band]; n/a; n/a; n/a; Optional with capability signalling. |
| Ericsson | 2 Discussion 2.1 Idle Mode Following are the agreed Idle mode UE features after RAN1#120bis [1]. 62. NR_LPWUS; 62-1; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK [2. The support of LP-SS based RRM measurement and/or SSB-based measurement, corresponding to component 1] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported; Per Band; n/a; n/a; n/a; Component 5 candidate values: FFS; Optional with capability signalling. 62. NR_LPWUS; 62-1a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence [2. FFS: The support of LP-SS based RRM measurement and/or [X. FFS: The support of SSB-based RRM measurement] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence is not supported; Per band; n/a; n/a; n/a; Component 5 candidate values: FFS; Optional with capability signalling. General Comments - Regarding the following part of RAN1#120 agreement “…FFSs on whether/which to define as a basic FG for LP-WUS/WUR (and potential prerequisite relationship in between),…”, we do not see the need to select one FG between 62-1 and 62-1a as basic FG for LP-WUS/WUR. From prerequisite perspective, support for 62-1 should not require support for 62-1a or vice versa. FG 62-1 - Components - “[2. The support of LP-SS based RRM measurement and/or SSB-based measurement, corresponding to component 1]”- suggest revising this component as “2. Support of RRM measurement in Idle/Inactive mode based on LP-SS.”. - “[3. The support of further RRM relaxation of UE MR]” and “[4. The support of serving RRM measurement offloading” - prefer to delete. These can be added as separate FG by RAN2/RAN4. - “[5. The supported wake-up delay]”- suggest revising this component as “3. Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring.”. - This to align with agreed RAN1 definition for the term “wake-up delay” in RAN1#118. - “FFS: M values” – suggest adding new components as “4. Supported number of OOK symbols per OFDM symbol (M) for LP-WUS, candidate value set for M {{(1,2}, {1,2,4}}.” and “4a. Supported number of OOK symbols per OFDM symbol (M) for LP-SS, candidate value set for M {{(1,2}, {1,2,4}}.” - i.e., support for M=1,2 should be included as part of FG 62-1 capability indication while support for M=4 can be indicated using additional capability bit. - Evaluations in [3] show that M=4 does not provide any coverage/capacity benefit for Idle mode over M=1,2. Given this, we think it would be unnecessary UE requirement to mandate always supporting M-4 for Idle mode. - Suggest adding new component “5. Detection of LP-WUS information from binary OOK sequences” - This clarifies the difference on how UE obtains LP-WUS information compared to FG 62-1a FG 62-1a - OK to confirm FG name as “LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence” - FG type can be confirmed as “Per Band” - Components - “[2. FFS: The support of LP-SS based RRM measurement and/or” and “[X. FFS: The support of SSB-based RRM measurement]” – suggest revising these as following two components – “2. Support of SSB-based Idle/Inactive mode RRM measurement at least when LP-WUS PRBs are within the PRBs occupied by SSB” and “3. Support of Idle/Inactive RRM measurement when LP-WUS PRBs are not within the PRBs occupied by SSB: candidate value set {SSB-based, LP-SS-based; “FFS: SSB-based and LP-SS-based”}” - If LP-WUS is overlapping with SSB PRBs then FG 62-1a UE should support SSB-based RRM measurements. - For the non-overlapping case, given discussion in RAN1#120, we are ok to include additional capability bit(s) on how RRM measurement is supported. i.e., SSB-based or LP-SS-based. If UE is capable of both LP-SS-based and SSB-based RRM measurements, details of when/how the UE uses LP-SS or SSB are not clear and more progress is needed to clarify these. So, we suggest keeping such candidate value FFS at this point. - “[3. The support of further RRM relaxation of UE MR]” and “[4. The support of serving RRM measurement offloading” – same comment as given for FG 62-1 - “[5. The supported wake-up delay]”- same comment as given for FG 62-1 - “FFS: M values” – same comment as given for FG 62-1 - Suggest adding new component “6. Detection of LP-WUS information from OFDM overlaid sequences” - This clarifies the difference on how UE obtains LP-WUS information compared to FG 62-1 Additional FGs - In RAN1#120, UE capability for a UE that can only support the case of overlaid OFDM sequence not carrying information was discussed. Similar to FG 62-1, such UE gets LP-WUS information from binary OOK sequences. Supporting such UEs should not restrict gNB to configure only one overlaid sequence. Considering these aspects, our preference is to define a separate FG if capability indication for such UEs is to be supported. Initial proposal for such an FG is given below in FG 62-1b “LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence not carrying information”. Support of FG 62-1 should be prerequisite feature for such UE to avoid gNB restrictions on overlaid OFDM sequences. Our proposed updates to the FG descriptions are given below 62-1; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK 2. Support of RRM measurement in Idle/Inactive mode based on LP-SS 3. Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. Candidate value set FFS 4. Supported number of OOK symbols per OFDM symbol (M) for LP-WUS. Candidate value set for M: {{(1,2}, {1,2,4}}. 4a. Supported number of OOK symbols per OFDM symbol (M) for LP-SS. Candidate value set for M: {{(1,2}, {1,2,4}}. 5. Detection of LP-WUS information from binary OOK sequences FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported; Per Band; n/a; n/a; n/a; Component 3 Candidate value set FFS Component 4 Candidate value set for M {{(1,2}, {1,2,4}}. Component 4a Candidate value set for M {{(1,2}, {1,2,4}}.; Optional with capability signalling. 62-1a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM 2. Support of SSB-based Idle/Inactive mode RRM measurement at least when LP-WUS PRBs are within the PRBs occupied by SSB 2a. Support of Idle/Inactive RRM measurement when LP-WUS PRBs are not within the PRBs occupied by SSB. Candidate value set: {SSB-based, LP-SS-based; FFS:”SSB-based and LP-SS-based”} 3. Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. Candidate value set FFS 4. Supported number of OOK symbols per OFDM symbol (M) for LP-WUS and LP-SS. Candidate value set for M: {{(1,2}, {1,2,4}}. 4a. Supported number of OOK symbols per OFDM symbol (M) for LP-SS. Candidate value set for M: {{(1,2}, {1,2,4}}. 5. Detection of LP-WUS information from OFDM overlaid sequences FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM is not supported; Per Band; n/a; n/a; n/a; Component 2a Candidate value set: {SSB-based, LP-SS-based; FFS:”SSB-based and LP-SS-based”} Component 3 Candidate value set FFS Component 4 Candidate value set for M {{(1,2}, {1,2,4}}. Component 4a Candidate value set for M {{(1,2}, {1,2,4}}.; Optional with capability signalling. 62-1b; LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence not carrying information; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring in a cell with one configured OFDM overlaid sequence (OFDM overlaid sequence not carrying information) 2. Support of SSB-based RRM measurement at least when LP-WUS PRBs are within the PRBs occupied by SSB. 2a. Support of RRM measurement when LP-WUS PRBs are not within the PRBs occupied by SSB: candidate value set {“SSB-based”; “LP-SS-based”; “FFS: SSB-based and LP-SS-based”} 3. Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. Candidate value set FFS 4. Supported number of OOK symbols per OFDM symbol (M) for LP-WUS and LP-SS. Candidate value set for M: {{(1,2}, {1,2,4}}. 4a. Supported number of OOK symbols per OFDM symbol (M) for LP-SS. Candidate value set for M: {{(1,2}, {1,2,4}}. 5. Detection of LP-WUS information from binary OOK sequences Note: In a cell where more than one OFDM overlaid sequences are configured, the UE operation for detection for LP-WUS information is same as FG 62-1. FFS: Other components and/or further FG separation; 62-1; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequences not carrying information is not supported; [Per Band]; n/a; n/a; n/a; Component 2a Candidate value set: {SSB-based, LP-SS-based; FFS:”SSB-based and LP-SS-based”} Component 3 Candidate value set FFS Component 4 Candidate value set for M {{(1,2}, {1,2,4}}. Component 4a Candidate value set for M {{(1,2}, {1,2,4}}.; Optional with capability signalling. 2.2 Connected Mode Following are the agreed Connected mode UE features after RAN1#120bis [1]. 62. NR_LPWUS; 62-2; LP-WUS operation in CONNECTED mode based on OOK signal; 1. LP-WUS operation in CONNECTED mode based on OOK signal 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP FFS: M values FFS: SCS FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in CONNECTED mode based on OOK signal is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: FFS Component 3 candidate values: FFS; Optional with capability signalling. 62. NR_LPWUS; 62-2a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence; 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP FFS: M values FFS: SCS FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: FFS Component 3 candidate values: FFS; Optional with capability signalling. 62. NR_LPWUS; 62-3; LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode; 1. Support of LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode; FG 62-2 or 62-2a; YES; n/a; LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported; [Per band]; n/a; n/a; n/a; Optional with capability signalling. FG 62-2 - Components - “2. The supported procedure(s)” – propose the candidate value set for supported procedures as {Option 1-1 only, either Option 1-1 or Option 1-2 configured for a serving cell} - Our preference is all FG 62-2 UEs support LP-WUS procedure to trigger PDCCH monitoring in RRC connected mode based on Option 1-1 and a separate capability bit is used to indicate additional support for Option 1-2 - “3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode” – ok to confirm this component - “FFS: M values” – propose to update as “4. Supported number of OOK symbols per OFDM symbol (M) for LP-WUS, candidate value set for M {{(1,2}, {1,2,4}}” - Evaluations in [3] indicate no/marginal capacity benefit for M=4 for connected mode. Given this, we think it would be unnecessary UE requirement to mandate always supporting M-4 for connected mode. - Suggest adding new component “5. Detection of LP-WUS information from binary OOK sequences” - This clarifies the difference on how UE obtains LP-WUS information compared to FG 62-2a FG 62-2a - Components - OK to confirm the description of component 1 – “LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s)” - “2. The supported procedure(s)” – same comment as for FG 62-2 - “3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode” – same comment as for FG 62-2 - “FFS: M values” – same comment as for FG 62-2 - Suggest adding new component “5. Detection of LP-WUS information from OFDM overlaid sequences” - This clarifies the difference on how UE obtains LP-WUS information compared to FG 62-2 Our proposed updates to the FG descriptions are given below 62-2; LP-WUS operation in CONNECTED mode based on OOK signal; 1. LP-WUS operation in CONNECTED mode based on OOK signal 2. The supported procedure(s). Candidate values: {Option 1-1 only, either Option 1-1 or Option 1-2 configured for a serving cell} 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Supported number of OOK symbols per OFDM symbol (M) for LP-WUS. Candidate value set for M: {{(1,2}, {1,2,4}} 5. Detection of LP-WUS information from binary OOK sequences FFS: SCS FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in CONNECTED mode based on OOK signal is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {Option 1-1 only, either Option 1-1 or Option 1-2 configured for a serving cell} Component 4 candidate values: {{(1,2}, {1,2,4}} Component 3 candidate values: FFS; Optional with capability signalling. 62-2a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence; 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. The supported procedure(s). Candidate values: {Option 1-1 only, either Option 1-1 or Option 1-2 configured for a serving cell} 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Supported number of OOK symbols per OFDM symbol (M) for LP-WUS. Candidate value set for M: {{(1,2}, {1,2,4}} 5. Detection of LP-WUS information from OFDM overlaid sequences FFS: SCS FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: {Option 1-1 only, either Option 1-1 or Option 1-2 configured for a serving cell} Component 4 candidate values: {{(1,2}, {1,2,4}} Component 3 candidate values: FFS; Optional with capability signalling. |
| OPPO | UE features for LP-WUS/WUR in IDLE/INACTIVE mode We consider the WUS/WUR features in IDLE/INACTIVE mode as baseline. As the starting point, 2 UE feature groups are introduced: LP-WUS operation in IDLE/INACTIVE mode based on OOK signal, LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence. Similar components are introduced for CONNECTED mode. For the support of further RRM relaxation of UE MR, this can be confirmed as the both feature groups may introduce RRM relaxation. It is not identified difference among 2 feature groups. The serving RRM measurement offloading should also be included for both. The wake-up delay had been discussed in session for IDLE/INACTIVE mode. It is expected a delay value should be specified for UE to reporting. Although the Connected Mode may have different values, there may not need separated values between OOK and OFDM receivers. The wake-up time is essentially the ramp-up time for main receiver. We should consider report single value. Further, there should be a dedicate FG, to avoid conflict reported wake-up times for a terminal. In the updated list, the component 5 is given to wait for pending discussion of 9.6.2. However, a separated FG is still expected. Proposal 1: For both FGs of LP-WUS operation in IDLE/INACTIVE mode based on OOK signal and OFDM overlaid sequence, 2 components are introduced: The support of further RRM relaxation of UE MR; The support of serving RRM measurement offloading. Introduce an FG for supported wake-up delay for the IDLE/INACTIVE mode, further decide the reporting of candidate delay values based on outcome of 9.6.2 discussion. UE features for LP-WUS/WUR in CONNECTED mode The wake-up delay, minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode, has similar issue as for IDLE/INACTIVE mode. Even the candidate values could be different to IDLE mode, they should be same for different LR receivers in CONNECTED mode. We also suggest it to be a dedicate FG in the CONNECTED mode, and FFS the reporting candidate values. Similar as for idle mode, a separated FG is expected in the next update. For the UE with different number of codepoints, the capability should include support of multiple codepoints. For CONNECTED mode, the different number of supported decoding codepoints may require different complexities and results in different power consumption. UE should also be able to reported it. FFS candidate values in 9.6.1. Proposal 2: Introduce an FG for supported wake-up delay for the CONNECTED mode, further decide the candidate delay values based on outcome of 9.6.3 discussion. Introduce a component for multiple decoding codepoints for the CONNECTED mode into 2 FGs, further decide of candidate values. For the connected UE, the per band capability for FGs can be confirmed. Even the per FR could be possible, we can not preclude the LR for connected mode can only support fractional of frequency bands. Proposal 3: For FG 62-2, 62-2a and 62-3, confirm them as ‘per band’. |
| LGE | Discussion Based on the agreements under AI 9.6 and AI 9.15.5, we propose followings for Rel-19 LP-WUS/WUR UE features. The support of LP-SS/SSB based RRM measurement in IDLE/INACTIVE LP-SS/SSB-based RRM measurement is crucial for power saving and efficient LP-WUS reception when operating with OOK waveforms in IDLE/INACTIVE mode. For OFDM waveform-based LP-WUS operation, the UE can leverage existing SSB-based RRM measurements. We believe that SSB-based RRM measurement capabilities are independent of band combination scenarios and thus can be shared across a single UE FG. Therefore, we propose maintaining LP-SS based RRM measurement as a component in 62-1 and 62-1a, while defining SSB-based RRM measurement as a separate per-UE FG. Furthermore, if the FG for SSB-based RRM measurement is reported by the UE, it could be FFS that it is allow for UE to indicate that LP-SS based RRM measurement is not supported for 62-1a. Proposal #1: Remove the component X (FFS: The support of SSB-based RRM measurement) in 62-1a and define the separate FG for SSB-based RRM measurement. The type of the new FG should be per UE. Further RRM relaxation of UE MR and serving RRM measurement offloading of UE MR in IDLE/INACTIVE Both RRM relaxation and RRM measurement offloading are critical components that directly impact power saving performance when utilizing LP-WUS. The dependency between these features and efficient LP-WUS operation seems clear. Those features significantly reduce the measurement burden on the MR, which translates directly to lower power consumption during IDLE/INACTIVE states. Given their power saving gain, these components should remain as components within the 62-1 and 62-1a feature groups rather than being separated FG. This approach ensures that devices supporting LP-WUS also support the essential RRM features that maximize its power saving benefits. Proposal #2: Keep component 3 (The support of further RRM relaxation of UE MR) and 4 (The support of serving RRM measurement offloading) in 62-1 and 62-1a. The supported procedure(s) in CONNECTED mode per WUR type The supported procedures in CONNECTED mode are dependent on how the MR is indicated to start PDCCH monitoring (i.e., Option 1-1 and Option 1-2). Therefore, we propose configuring this as a common FG shared by both OOK and OFDM based LP-WUR in CONNECTED modes rather than defining it as a component within individual FGs. Proposal #3: Define single separated FG for the supported procedure(s) in CONNECTED mode for both LP-WUR types. The type of this FG is per UE. Maximum number of codepoint per MO and frequency resource allocation in CONNECTED mode Based on RAN1 agreements, the maximum number of codepoints per MO can be added as the separate FG. For example, 2, 4, or 8 can be considered. 62-2 and 62-2a should be prerequisite feature groups of new separate FG like 62-3. Proposal #4: Define the separate FG to indicate the maximum number of codepoint per MO for CONNECTED mode. The type of this FG is per band. Type of 62-2, 62-2a, and 62-3 The type of 62-2, 62-2a, and 62-3 is per band the same as 62-1 and 62-1a. Proposal #5: The type of 62-2, 62-2a, and 62-3 is per band. |
| Apple | UE Features for R19 LP-WUS For idle/inactive mode, we propose the following update as shown in red font: 62. NR_LPWUS; 62-1; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK signal [2. The support of LP-SS based RRM measurement and/or SSB-based measurement, corresponding to component 1] [3. The support of further RRM relaxation of UE MR] [4. The support of serving cell RRM measurement offloading from MR to LR] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported; Per Band; n/a; n/a; n/a; Component 5 candidate values: FFS; Optional with capability signalling. 62. NR_LPWUS; 62-1a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence 2. The support of SSB-based RRM measurement (FFS: The support of LP-SS based RRM measurement) [2. FFS: The support of LP-SS based RRM measurement and/or [X. FFS: The support of SSB-based RRM measurement] [3. The support of further RRM relaxation of UE MR] [4. The support of serving cell RRM measurement offloading from MR to LR] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence is not supported; Per band; n/a; n/a; n/a; Component 5 candidate values: FFS; Optional with capability signalling. For connected mode, we propose the following update as shown in red font: 62. NR_LPWUS; 62-2; LP-WUS operation in CONNECTED mode based on OOK signal; 1. LP-WUS operation in CONNECTED mode based on OOK signal 2. The support of UE procedures Option 1-1 (LP-WUS triggering the start of drx-onDurationTimer) and/or Option 1-2 (LP-WUS monitoring outside at least legacy C-DRX active time that triggers PDCCH monitoring) 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP 5. Maximum number of codepoints checked per MO by a UE 6. Support of UAI to indicate the preference for configured time offset between LP-WUS and PDCCH reception FFS: M values FFS: SCS FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in CONNECTED mode based on OOK signal is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: FFS {Option 1-1, Option 1-2, Option 1-1 and 1-2} Component 3 candidate values: FFS Component 5 candidate values: {2, 4, 6, 8} Component 6 candidate values: {Yes, No}; Optional with capability signalling. 62. NR_LPWUS; 62-2a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence; 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. The support of UE procedures Option 1-1 (LP-WUS triggering the start of drx-onDurationTimer) and/or Option 1-2 (LP-WUS monitoring outside at least legacy C-DRX active time that triggers PDCCH monitoring) 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP 5. Maximum number of codepoints checked per MO by a UE 6. Support of UAI to indicate the preference for configured time offset between LP-WUS and PDCCH reception FFS: M values FFS: SCS FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: FFS {Option 1-1, Option 1-2, Option 1-1 and 1-2} Component 3 candidate values: FFS Component 5 candidate values: {2, 4, 6, 8} Component 6 candidate values: {Yes, No}; Optional with capability signalling. 62. NR_LPWUS; 62-3; LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode; 1. Support of LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode; FG 62-2 or 62-2a; YES; n/a; LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported; [Per band]; n/a; n/a; n/a; Optional with capability signalling. 62. NR_LPWUS; 62-4; LP-WUS operation on two cell groups in NR-DC in CONNECTED mode based on OOK signal; 1. Support of monitoring LP-WUS on two cell groups in NR-DC with CA without dual DRX groups or without CA based on OOK signal; YES; n/a; LP-WUS operation on two cell groups in NR-DC in CONNECTED mode based on OOK signal is not supported; Per band combination; n/a; n/a; n/a; Optional with capability signalling. 62. NR_LPWUS; 62-5; LP-WUS operation on two cell groups in NR-DC in CONNECTED mode based on OFDM overlaid sequence; 1. Support of monitoring LP-WUS on two cell groups in NR-DC with CA without dual DRX groups or without CA based on OFDM overlaid sequence; YES; n/a; LP-WUS operation on two cell groups in NR-DC in CONNECTED mode based on OFDM overlaid sequence is not supported; Per band combination; n/a; n/a; n/a; Optional with capability signalling. |
| Qualcomm | - LP-WUS procedure for idle/inactive state In this section, we discuss the UE capabilities for LP-WUS in idle and inactive state. This is based on RAN1 #120bis session notes in R1-2502966 and summary of discussions in R1-2502972. - FG 62-1: LP-WUS operation in IDLE/INACTIVE mode based on OOK signal In FG 62-1, there are three tentative components 2, 3 and 4 related to RRM measurement in LP-WUR - [2. The support of LP-SS based RRM measurement] - [3. The support of further RRM relaxation of UE MR] - [4. The support of serving RRM measurement offloading] In RAN1, the majority view is that to achieve material power saving gain, UE needs to support both LP-WUS monitoring and RRM measurement by LP-WUR. Discussions in RAN2 and RAN4 show similar views. Given this, we think components 2, 3 and 4 can be included in FG 62-1. For M values, the LP-WUR should be able to support all M values agreed in the signal design in agenda 9.6.1. For this, we propose to remove the related FFS. Proposal 2.1: - For FG 62-1 for LP-WUS operation in IDLE/INACTIVE mode based on OOK signal - Support components 2, 3 and 4 for RRM measurement in LP-WUR - Remove the FFS for M values. Note: a UE can indicate either or both FG 62-1 and FG 62-1a for the same frequency band - FG 62-1a: LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence In FG 62-1a, there are the following tentative components related to RRM measurement by LP-WUR. Similar to FG 62-1, we think the LP-WUR RRM measurement can be supported together with LP-WUS monitoring. For the OFDM LP-WUR, SSB based RRM measurement is the baseline because overlaid OFDM sequence may not be configured for LP-SS or even LP-SS may not be configured by network. SSB-based RRM measurement should be included in FG 62-1a. LP-SS based RRM measurement is up to UE implementation and the corresponding component can be removed. - [2. FFS: The support of LP-SS based RRM measurement] - [X. FFS: The support of SSB-based RRM measurement] - [3. The support of further RRM relaxation of UE MR] - [4. The support of serving RRM measurement offloading] Similar to FG 62-1, the FFS for M values can be removed. Proposal 2.2: - For 62-1a for LP-WUS operation in IDEL/INACTIVE mode based on OFDM overlaid sequence - Support components X, 3 and 4 for RRM measurement in LP-WUR - Remove component 2 for LP-SS based RRM measurement - Remove the FFS for M values. Note: a UE can indicate either or both FG 62-1 and FG 62-1a for the same frequency band - FG 62-1b: LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence not carrying information In the RAN1 #118bis agreement below, for the case that the overlaid OFDM sequence does not carry information, one overlaid sequence is configured in each cell. This allows the LP-WUR to coherently detect the LP-WUS information for an extended coverage even though the LP-WUS information is only modulated on the OOK envelope. RAN1 #118bis Agreement In case of overlaid OFDM sequence not carrying information: - Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol. - Note 1: multiple overlaid OFDM sequences are specified. Note 2: gNB can configure different overlaid OFDM sequence(s) for different cells.. In RAN1 #120, a tentative FG 62-1b was discussed to support LP-WUS operation in IDLE/INACTIVE mode based on overlaid OFDM sequence not carrying information. We support to define this feature so that the UE benefit from the extended coverage from overlaid OFDM sequences. Similar to FG 62-1a, we support to include the SSB based LP-WUR RRM measurement in FG 62-1a and remove the component for M values. Proposal 2.3: - Support to specify FG 62-1b for LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence not carrying information - Per band - Pre-requisite is FG 62-1 - Support component 2 for SSB-based RRM measurement in LP-WUR without additional condition on frequency bandwidth for SSB - Remove component 4 for M values. - FG 62-1c: LP-WUS frequency resource out of the initial DL BWP It was agreed in RAN1 #120bis that LP-WUS/LP-SS frequency resource can be out of initial DL BWP. However, LP-WUS frequency resource out of initial DL BWP imposes implementation challenges to the LP-WUR. For example, the UE cannot use shared RF module for MR and LP-WUR without RF frequency tuning. To allow for simple UE implementation, we propose to define new UE capability for the support of the LP-WUS/LP-SS frequency resource out of initial DL BWP. If LP-WUS resource is out of the initial DL BWP, LP-WUR may need to turn the RF between the MR and LP-WUR operation or before and after SSB reception. The candidate values should be determined by RAN4. Proposal 2.4: - Specify FG 62-1c for LP-WUS frequency resource out of initial DL BWP - Per band - Pre-requisite is one of {62-1, 62-1a} - RF tuning time supported by the UE: TBD by RAN4 - LP-WUS procedure for connected state - FG 62-2: LP-WUS operation in CONNECTED mode based on OOK signal A basic UE feature for LP-WUS in connected state should be per-band. Since Option 1-1 and Option 1-2 are now quite different, there must be UE capability differentiation between them at least for interoperability purposes. For LP-WUS detection based on OOK, it was agreed that the maximum number of codepoints per MO for a UE can be up to 8. We propose to make the supported value as part of FG62-2. The minimum time gap between LP-WUS and start of PDCCH monitoring can also be part of FG62-2. Proposal 3.1: - Update FG 62-2: LP-WUS operation in CONNECTED mode based on OOK signal as follows: - Per-band - Candidate values for component 2: {option 1-1, option 1-2, both} - Candidate values for component 3: {V1, V2, V3} for each SCS of LP-WUS - Component 5: Max number of codepoints checked per MO by a UE {2, [4], [8]} Note: a UE can indicate either or both FG 62-2 and FG 62-2a for the same frequency band - FG 62-2a: LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence FG62-2a can be a similar FG to FG62-2. For FG62-2a, the component 5 in FG62-2, “Max number of codepoints checked per MO by a UE”, is not applicable, since the UE monitors raw bit information instead of codepoint as per RAN1 agreement. Nevertheless, we believe for FG62-2a, similar component is necessary. LP-WUS operation based on OFDM overlaid sequence increases complexity and power consumption of LP-WUR if the UE has to detect a larger number of sequences per OOK ON chip. Depending on how sequences are allocated per OOK ON chip, the number of digital correlators used per OOK ON chip increases. Although RAN1 agreed to support max {16, 8, 4} candidate sequences per OOK ON chip for M = {1, 2, 4}, respectively, it is not realistic to assume all the UEs support and are tested with these. There must be a way for a UE to report its capability of applicable number of overlaid sequences per OOK ON chip. We propose to add Component 5: Max number of overlaid sequences applicable for a UE per OOK ON chip, as part of FG62-2a. Proposal 3.2: - Update FG 62-2a: LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence - Per-band - Candidate values for component 2: {option 1-1, option 1-2, both} - Candidate values for component 3: {V1, V2, V3} for each SCS of LP-WUS - Component 5: Max number of overlaid sequences applicable for a UE per OOK ON chip - One value from {2, 4, 8, 16} for M = 1 - One value from {2, 4, 8} for M = 2 - One value from {2, 4} for M = 4 Note: a UE can indicate either or both FG 62-2 and FG 62-2a for the same frequency band Meanwhile, we think a UE should be allowed to support LP-WUS based on OFDM overlaid sequence that does not carry information, in which case the LP-WUS information is obtained by OOK while the overlaid sequence offers coverage improvement. For the gNB to know that the UE supports the use of overlaid sequence for OOK LP-WUS, we think a separate FG should be defined, FG62-6, to be discussed later. - FG 62-3 and FG 62-4: LP-WUS in connected state for CA As we proposed in our contribution on AI 9.6.3, for CA without secondary DRX group, LP-WUS would be monitored on a specific serving cell (i.e., the primary cell) and it wakes up all the active serving cells. The minimum time gap to wake up the active serving cells could be different from the value reported in the basic UE feature. With that, a specific UE feature is necessary for CA. Since the minimum time gap to wake up also relates to the WUR types, the specific UE feature should be per WUR type (OOK and OFDM). The reporting granularity should be per-BC. The basic UE feature(s) for LP-WUS in connected state (i.e., FG 62-2 and/or FG 62-2a above) should be reported at least for one band in the BC as prerequisite for FG 62-3 and FG 62-4 for the BC. Proposal 3.3: - Specify FG 62-3: UE feature group for LP-WUS in connected mode for CA using OOK WUR: - Per-BC - LP-WUS on the PCell triggers PDCCH monitoring on all the active serving cells - UE can further report following: - Minimum time gap to wake-up, {V1, V2, V3} for all the active serving cells - FG 62-3 prerequisites: - FG 6-5 - FG 62-2 at least for one band in the BC where FG 62-3 is reported Proposal 3.4: - Specify FG 62-4: UE feature group for LP-WUS in connected mode for CA using overlaid sequence WUR - Per-BC - LP-WUS on the PCell triggers PDCCH monitoring on all the active serving cells - UE can further report following: - Minimum time gap to wake-up, {V1, V2, V3} for all the active serving cells - FG 62-4 prerequisites - FG 6-5 - FG 62-2a at least for one band in the BC where FG 62-4 is reported - FG 62-5: LP-WUS operation in CONNECTED mode based on OOK signal with an OFDM overlaid sequence As explained in Section 3.2, we think a separate FG for OOK based LP-WUS with overlaid sequence that does not carry information is necessary. The rationale is the following RAN1 agreement: Agreement In case of overlaid OFDM sequence not carrying information: - Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol. - Note 1: multiple overlaid OFDM sequences are specified. - Note 2: gNB can configure different overlaid OFDM sequence(s) for different cells.. As stated in 3.2, the LP-WUS information is obtained by OOK while the overlaid sequence offers coverage improvement. The UE should be able to support this feature without prerequisiting FG62-2a, since this way does not require the UE to support multiple overlaid sequences to get the information from LP-WUS. On the other hand, FG62-2 is necessary as a prerequisite for this UE to get the information from LP-WUS. Having said that, we propose to add a new FG62-5 for this purpose. Proposal 3.5: - Specify FG 62-5: LP-WUS operation in CONNECTED mode based on OOK signal with an OFDM overlaid sequence - Support of OOK LP-WUS with a single OFDM overlaid sequence on all the OOK ON chips - OFDM overlaid sequence does not carry information - FG 62-6 prerequisites: - FG 62-2 |
| MediaTek | IDLE/INACTIVE mode 62-1; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK [2. The support of LP-SS based RRM measurement and/or SSB-based measurement, corresponding to component 1] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported; Per Band; n/a; n/a; n/a; Component 1 candidate values: {OOK-based, OFDM-based, both} Component 5 candidate values: FFS [For UE supporting both X-1 and X-2, UE is not allowed to report different candidate values for component 1]; Optional with capability signalling. Proposals on FG 62-1: - Component 2: Support of LP-SS based RRM measurement and/or SSB-based measurement is confirmed, as it is natural for a UE supporting OOK-based LP-WUS to also support these measurements. - Component 3 and 4: Support of further RRM relaxation of UE MR and support of serving RRM measurement offloading are important for maximizing UE power saving. A note is added that a UE reporting support of this FG shall support offloading and relaxation. - Component 5: Support of reporting of wake-up delay is a key feature. We propose the candidate values [70ms, 500ms, 900ms], with the final decision to be based on R4 LS reply. - M values: The M value (number of OOK symbols per OFDM symbol) is a key parameter. We support M = 1, 2, 4, and support all options. 62-1a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence [2. FFS: The support of LP-SS based RRM measurement and/or [X. FFS: The support of SSB-based RRM measurement] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence is not supported; Per band; n/a; n/a; n/a; Component 1 candidate values: {OOK-based, OFDM-based, both} Component 5 candidate values: FFS [For UE supporting both X-1 and X-2, UE is not allowed to report different candidate values for component 1]; Optional with capability signalling. Proposals on FG 62-1a: - Component 1: Add “Note: The OFDM sequence can carry information.” Align with RP-241824, Revised WID. RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported. - Component 2: Support of LP-SS based RRM measurement is considered optional. The baseline assumption is that IDLE UE only operates in frequency locations around SSB for LR and MR, so LP-SS based RRM measurement may not be necessary for all UEs. - Component X: Support of SSB-based RRM measurement is supported. - Component 3 and 4: Support of further RRM relaxation of UE MR and support of serving RRM measurement offloading are supported. - Component 5: Reporting of wake-up delay is supported, with candidate values [70ms, 500ms, 900ms], with the final decision to be based on R4 LS reply. - M values: Support for combination with OOK, with M = 1, 2, 4. 62-1b; LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence not carrying information; 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring in a cell with one configured OFDM overlaid sequence (OFDM overlaid sequence not carrying information) 2. Support of SSB-based RRM measurement at least when LP-WUS PRBs are within the PRBs occupied by SSB. 2a. Support of RRM measurement when LP-WUS PRBs are not within the PRBs occupied by SSB: candidate value set {“SSB-based”; “LP-SS-based”; “FFS: SSB-based and LP-SS-based”} 3. Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. [4. Supported number of OOK symbols per OFDM symbol (M) for LP-WUS and LP-SS. Candidate value set for M: {{(1,2}, {1,2,4}}.] 5. Detection of LP-WUS information from binary OOK sequences FFS: Other components and/or further FG separation; [62-1]; YES; n/a; LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequences not carrying information is not supported; [Per Band]; [n/a]; [n/a]; [n/a]; Component 2a Candidate value set: {SSB-based, LP-SS-based; FFS:”SSB-based and LP-SS-based”} [Component 3 Candidate value set FFS] [Component 4 Candidate value set for M {{(1,2}, {1,2,4}}.) Note: In a cell where more than one OFDM overlaid sequences are configured, the UE operation for detection for LP-WUS information is same as FG 62-1.; Optional with capability signalling. Proposals on FG 62-1b: Propose to revise component of FG 62-1a. We do NOT support LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence not carrying information. - Component 1: Not supported, as the OFDM overlaid sequence does not carry information. This is not the aggreed WID working scope. - Component 3: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring is for further study. - Component 5: Detection of LP-WUS information from binary OOK sequences only is not in the WID scope. RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported. CONNECTED mode 62-2; LP-WUS operation in CONNECTED mode based on OOK signal; 1. LP-WUS operation in CONNECTED mode based on OOK signal 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode FFS: M values FFS: SCS FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in CONNECTED mode based on OOK signal is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: FFS Component 3 candidate values: FFS; Optional with capability signalling. Proposal on FG 62-2: - M values: We support all M options, i.e., M = 1, 2, 4. 62-2a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence; 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode FFS: M values FFS: SCS FFS: Other components and/or further FG separation; YES; n/a; LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported; [Per band]; n/a; n/a; n/a; Component 2 candidate values: FFS Component 3 candidate values: FFS; Optional with capability signalling. Proposal on FG 62-2a: - M values: Support for combination with OOK, with M = 1, 2, 4. 62-x; LP-WUS operation in CONNECTED mode based on OOK signal with an OFDM overlaid sequence; 1. LP-WUS operation in CONNECTED mode based on OOK signal with a single OFDM overlaid sequence on all the OOK ON chips; [62-2]; YES; n/a; [Per band]; [n/a]; [n/a]; [n/a]; OFDM overlaid sequence does not carry information; Optional with capability signalling. Proposal on FG 62-x: - Component 1: Not supported. The overlaid OFDM sequence does not carry information, which conflicts with the harmonized design scope in the WID. RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported. |
| DCM | - Discussion - FG 62-1/1a - High priority issues (w/ ASN.1 impact) Regarding the component 5 ([The supported wake-up delay]), following was agreed in previous RAN1 meetings: Agreement For UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - FFS: different sets of 3 candidate value for different SSB periodicities - Send an LS to RAN4 after the down-selection to confirm the number of SSBs. Final LS in R1-2501624. Agreement On how to handle the UE capability report on the wake-up delay for SSB periodicities other than 20ms, consider the following alternatives for possible down-selection in RAN1#121. - Alt 1: Do not report for SSB periodicities other than 20ms - Note: LP-WUS is not supported for SSB periodicities larger than 20ms - Alt 2: For UE capability report on the wake-up delay, the UE reports one of the following 3 capabilities (the values in one of the columns): SSB periodicity (ms); Wake-up delay (ms) UE capability 1; Wake-up delay (ms) UE capability 2; Wake-up delay (ms) UE capability 3. [5]; [x]; [x]; [x]. [10]; [x]; [x]; [x]. 20; [70]; [500]; [900]. [40]; [x]; [x]; [x]. [80]; [x]; [x]; [x]. [160]; [x]; [x]; [x]. - Alt 3: It is up to RAN4. Therefore, this component can be confirmed with candidate values as follows. Note that further refinement for different SSB periodicities can be done after the FFS is resolved in AI9.6.2. Proposal 1: - Component 5 of FG 62-1/1a is confirmed as: The supported wake-up delay - Candidate values for component 5: {[70ms], [500ms], [900ms]} Regarding the FFS on M values, following agreements were made in the last RAN1 meeting: Agreement For idle mode, regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell: - support maximum 16 candidates overlaid sequences for M=1 - support maximum 8 candidates overlaid sequences for M=2 - For candidate overlaid sequences across all OOK ON chips of LP-WUS, the number of roots (in specification) is up to [FFS: X], FFS whether the number of roots can be different for different M value. - FFS: The number of overlaid sequences applicable for a UE is no more than 2 per OOK ON chip. Agreement For RRC connected mode, maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, - support maximum 16 candidates overlaid sequences for M=1 - support maximum 8 candidates overlaid sequences for M=2 - support maximum 4 candidates overlaid sequences for M=4 (agreed in RAN1#119) - For candidate overlaid sequences across all OOK ON chips of LP-WUS, the number of roots (in specification) is up to [FFS: X], FFS whether the number of roots can be different for different M value. - FFS: The number of overlaid sequences applicable for a UE is no more than 2 per OOK ON chip. - FFS: Whether or not the candidate set of overlaid sequences change across OOK ON chips Working Assumption At least for FR1, support maximum 2 roots to be used for overlaid OFDM sequences for M=1/2/4 (max 2 for each M value). - Different roots can be picked for different M values. Agreement Regarding whether to support additional sync signal to LP-SS, support the following for LP-WUS with M=1, M=2: - Option 1A: No additional sync signal, LP-SS periodicity =320ms - Additionally support LP-SS periodicity of 160ms. According to the above working assumption, the number of roots for candidate overlaid sequences across all OOK ON chips of LP-WUS is the same for M=1/2/4, and hence, we don’t see critical need to report supported M as a component in FG 62-1/1a from this perspective. On the other hand, no consensus was made for LP-WUS with M=4 whether to support additional sync signal to LP-SS and/or shorter LP-SS periodicity. Therefore, we think the FFS on M values can be further discussed after such unclear points are resolved in 9.6.1. - Low priority issues (w/o ASN.1 impact) Regarding component 2 in FG 62-1, for OOK-based WUR, LP-SS is used for RRM measurement, and can be confirmed as: Proposal 2: - Component 2 of FG62-1 is confirmed as: The support of LP-SS based RRM measurement On the other hand, for FG 62-1a (OFDM-based WUR), LP-SS is not expected to be used for RRM measurement at least from RAN4 requirement perspective, according to the following agreement, and hence, component 2 is not necessary. Agreement Proposal 4.3-1: Update the agreements in RAN1 #118bis as below Agreement Support overlaid OFDM sequence(s) for LP-SS: - LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. - Applicable to both OOK-1 and OOK-4 - Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable. - Applicable at least for OOK-1 only and FFS for OOK-4 - From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences. Instead, component X is necessary for FG 62-1a, since SSB is expected to be used for RRM measurement for OFDM-based WUR, at least from RAN4 requirement perspective. Proposal 3: - Component 2 of FG62-1a is deleted - Component X of FG62-1a is confirmed as: The support of SSB based RRM measurement Regarding components 3 and 4, those features are being discussed in RAN4/2, and RAN1 is not included as corresponding WGs in the WID. Therefore, we can delete those components in RAN1 feature and defer the decision to RAN4/2 - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2). Proposal 4: - Components 3 and 4 of FG62-1/1a are deleted - It is up to RAN4/2 whether/how to capture the following UE capabilities - Further RRM relaxation of UE MR for both serving and neighbor cell measurements - UE serving cell RRM measurement offloaded from MR to LP-WUR Regarding the prerequisite FGs, FG 62-1/1a can be used on top of the legacy paging procedure, which is mandatory for NR UEs, and hence, any FGs are not necessary for the prerequisite FGs Proposal 5: - No prerequisite FGs are included in FG62-1/1a - FG 62-2/2a - High priority issues (w/ ASN.1 impact) Regarding the component 2, RAN1 agreed to support LP-WUS procedure Option 1-1 and Option 1-2, and Option 1-2 can be regarded as generalized feature of Option 1-1 (i.e., Option 1-1 can be realized by proper configuration of Option 1-2). Therefore, it is preferred to support {Option 1-1, Both Option 1-1 and Option 1-2} so that at least Option 1-1 is supported by all UEs using LP-WUS, and NW operation can be simplified. Proposal 6: - Component 2 of FG62-2/2a is confirmed as: The supported procedure(s) - Candidate values for component 2: {Option 1-1, Both Option 1-1 and Option 1-2} Regarding the component 3, following agreements were agreed in previous RAN1 meetings, while it may be premature to confirm the candidate values, and hence, this issue can be deferred until further progress is made in AI9.6.3. Agreement - For the X = [2, 3] candidate values for the UE capability report on the minimum time gap for each SCS, consider {0.25, 0.5, 1, 2, 3, 6, 10, 20, 30, 40}ms - Note: Other values are not precluded - Only for reported value of [20ms,] 30ms and 40ms (if supported), the reported values assume SSB periodicity of 20ms and ensures SSB(s) before PDCCH reception and during time gap - FFS: translation of minimum time gap for different SSB periodicities including different candidates for different SSB periodicities - FFS whether the same set of candidates are used for different SCSs - FFS whether different values are selected for OOK-based WUR and OFDM-based WUR - FFS whether/how to consider the case of CA - FFS: candidate values that can be indicated by UAI Agreement For the UE capability report on the minimum time gap is between the end of the last symbol of LP-WUS and the time where MR starts PDCCH monitoring regardless of SCS, support X=3 candidate values corresponding to {V1=[3,5,6], V2=[10,13,20], V3=[33,42]}ms where V1<V2<V3. - FFS: Whether to support different set of values for different receiver types. Therefore, this component can be confirmed with candidate values as follows. Note that V1/V2/V3 can be confirmed after those values are resolved in AI9.6.3. Proposal 7: - Component 5 of FG 62-2/2a is confirmed as: Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode - Candidate values for component 5: {[V1], [V2], [V3]} Regarding the FFS on M values, similar to idle/inactive mode, the number of roots for candidate overlaid sequences across all OOK ON chips of LP-WUS is the same for M=1/2/4, and hence, we don’t see critical need to report supported M as a component in FG 62-2/2a from this perspective. On the other hand, no consensus was made for LP-WUS with M=4 whether to support additional sync signal to LP-SS and/or shorter LP-SS periodicity for idle/inactive mode. While LP-SS is not assumed to be used for connected mode, whether/how to support M=4 needs further discussion in 9.6.1, and hence, we think the FFS on M values can be further discussed after such unclear points are resolved in 9.6.1. Regarding the FFS on SCS, following was agreed in RAN1#120, and hence, the FFS can be deleted. Agreement For RRC connected, for LP-WUS SCS: - Alt 1: LP-WUS SCS is same as the active DL BWP. Proposal 8: - “FFS: SCS” is deleted from FG62-2/2a Regarding the reporting type, similar to idle/inactive modes, it would be straightforward to define the reporting type of FG 62-2/2a as “per band”. It was argued by some companies in the last RAN1 meeting that CA case should be different capability. We are not sure whether it is necessary since FG 62-2/2a is the UE capability whether UE can receive LP-WUS by LR and trigger PDCCH monitoring. For LP-WUS reception itself, it is not related to CA and “per band”, where LP-WUS is monitored, would be enough. For triggering PDCCH monitoring, it is commonly controlled per cell group, including both intra-band CA and inter-band CA, and any differentiation within cell group would not be necessary. Proposal 9: - Reporting type of FG 62-2/2a is “per band” In addition to the existing FFS, following working assumption was made in RAN1#120, and UE capabilities corresponding to the highlighted parts should be introduced in FG 62-2/2a, while candidate values can be further discussed. From our perspective, at least 2 should be supported (one for the wake-up indication targeting the UE, and the other for the wake-up indication targeting all UEs). Working Assumption Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs: - Codepoint based - The maximum number of codepoints checked per MO by a UE is up to 8 - Depending on UE capability, a UE may support less than 8. Proposal 10: - Introduce new component in FG 62-2/2a: The maximum number of codepoints checked per MO by a UE - FFS Candidate values - Low priority issues (w/o ASN.1 impact) Regarding component 1 of FG62-2a, “OFDM overlaid sequence” was agreed for FG 62-1a and the same applies here. Proposal 11: - Component 1 of FG62-2a is confirmed as: LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) Regarding the prerequisite FGs, FG 62-2/2a can be used on top of the legacy C-DRX procedure, which is mandatory for NR UEs, and hence, any FGs are not necessary for the prerequisite FGs. It was argued by some companies in the last RAN1 meeting that FG 62-1/1a should be the prerequisite FG of FG 62-2/2a. We don’t think this restriction is necessary, since only FG 62-2/2a can be supported for some bands, such as FR2, where idle mode procedure may not be necessary. Proposal 12: - No prerequisite FGs are included in FG62-2/2a - FG 62-3 - High priority issues (w/ ASN.1 impact) Regarding the reporting type, similar to idle/inactive modes, FG 62-3 has prerequisite of FG 62-2/2a and it would be straightforward to have the same reporting type. Proposal 13: - Reporting type of FG 62-3 is same as those of FG 62-2/2a |

- Discussion

3.1	LP-WUS operation for IDLE/INACTIVE mode

Proposal 1-1 (closed):

For FG 62-1 Component 2, confirm “2. The support of LP-SS based RRM measurement”

| Company | Comment |
| --- | --- |
| Qualcomm | Support the proposal. |
| DOCOMO | Support |
| xiaomi | support |
| vivo | Fine to confirm. It would be good to clarify that all M values are to be supported for LP-SS in FG62-1, i.e. The support of LP-SS based RRM measurement with all M values (M=1,2,4) for LP-SS |
| Vodafone | Support |
| Ericsson1 | OK |
| ZTE, Sanechips | OK |
| Samsung | Support. |
| Huawei, HiSilicon | OK |
| Mod | With the following agreement, this discussion has been closed. Agreement: For FG 62-1 Component 2, confirm “2. The support of LP-SS based RRM measurement” |

Question 1-2 (closed):

Companies are encouraged to share views on Component 2 and Component X in FG 62-1a

- Alt-1: Confirm only Component 2 (i.e., the support of LP-SS based RRM measurement) in FG 62-1a

- Alt-2: Confirm only Component X (i.e., the support of SSB based RRM measurement) in FG 62-1a

- Alt-3: Confirm both Component 2 and X in FG 62-1a

| Company | Comment |
| --- | --- |
| Qualcomm | Support Alt-2. Our understanding is SSB based RRM measurement is considered as baseline for FG 62-1a in RAN4. |
| DOCOMO | According to the agreements made in RAN4 so far, both LP-SS based and SSB based RRM measurements are considered for the RRM requirements of OFDM-based WUR. --- Issue 1-1-3-2: On RRM requirements to be specified for SSB-based LP-WUR based only on LP-SS with and without overlaid OFDM sequence Agreement: • Define the requirements for OFDM-based LP-WUR serving cell measurement based only on LP-SS - Reuse the same requirement for LP-SS OOK based Issue 1-2-2: Periodicity for LP-WUR RRM requirements Agreement: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity. --- Therefore, it is possible to include both components 2 and X in FG62-1a. From DOCOMO perspective, we support Alt-3, while open to hear companies view to support either one in FG 62-1a and the other as another capability. |
| Xiaomi | Support Alt-3. And for each of the Component 2 and X, there should be candidate values{Yes, No} |
| vivo | Alt-2. It is our understanding that support of SSB based RRM measurement should be mandatory for OFDM-based LP-WUR according to the WID, while the support of LP-SS based measurement is optional. |
| Vodafone | Support Alt-3 |
| Ericsson1 | Among the alternatives listed above, we support Alt2 |
| Nordic | Alt-2 |
| ZTE, Sanechips | Alt3 is OK for us. |
| Samsung | Our preference is Alt-2. It’s still under the discussion whether to configure the LP-SS configuration and how to handle the case when the overlaid OFDM sequences is not configured for LP-SS with M=1 case. It means that LP-SS may not be always-on signal for LP-WUS operation while SSB is always transmitted. |
| Huawei, HiSilicon | In RAN4, both SSB-based and LP-SS based RRM measurement are supported. We support Alt-3 with a clarification for a case where M=1 of LP-SS and overlaid sequence is not configured because the design and spec impacts of this case are unclear yet and may end up with unclear UE behaviour and compliance of RAN4 requirements. Revised Alt-3: Confirm both Component 2 and X (FFS: the case where overlaid sequence is not configured on LP-SS with M=1) in FG 62-1a |
| Moderator | With the following agreement, this discussion has been closed. Agreement: On Component X in FG 62-1a: - Confirm Component X (i.e., the support of SSB based RRM measurement) |

Proposal 1-3 (closed):

Remove component 3 and component 4 in FG 62-1/1a (i.e., support of further RRM relaxation of UE MR and support of serving RRM measurement offloading)

| Company | Comment |
| --- | --- |
| Moderator | Regarding component 3 and component 4 in FG 62-1/1a, the following two aspects were discussed in the last RAN1 meeting: - Whether these components are defined in RAN1 FG or RAN2 FG - Whether RAN1 FG 62-1/1a comprise the support of these two features (i.e., further UE MR RRM relaxation and RRM measurement offloading are supported by all UEs capable of LP-WUS operation in IDLE/INACTIVE mode It seems that RAN2 had an email discussion in “[Post129bis][218][LPWUS] UE capabilities (Huawei)”, from which the following seems to be suggested: Proposal 1 (10/10): A UE indicating support of LP-WUS reception in IDLE/INACTIVE shall support UE-ID based subgrouping. Proposal 2 (6/9): RRM measurement relaxation and RRM measurement fully offloading are defined as RAN2 capability without UE capability signalling. Send LS to RAN1 to inform the agreement. Proposal 3: Way forward on the supported use cases regarding UE supporting LP-WUS reception, and RRM measurement relaxation and fully offloading: - Agree to support “Case 1” (8/10 support Case 1): UE supporting LP-WUS reception shall also support RRM measurement relaxation and RRM measurement fully offloading - Discuss on supporting “Case 2” (2/10 support Case 2): UE supports LP-WUS reception, but does not support RRM measurement relaxation and RRM measurement fully offloading - Preclude “Case 3” (1/10 support Case 3): UE supports RRM measurement relaxation and RRM measurement fully offloading, but does not support LP-WUS reception Proposal 4: Wait for further discussions in RAN1 and RAN2 for CONNECTED state LP-WUS capability.. In my understanding, Proposal 2 above touches upon the completely same things as component 3 and 4 in FG 62-1/1a. In case they are defined as RAN2 capability without capability signalling as in Proposal 2, I believe RAN1 FG shouldn’t capture them. Assuming the above proposals are likely to be agreed in RAN2 soon, my suggestion is to take Proposal 1-3, as anyway RAN2 will not likely to task RAN1 to make decision for this. |
| Qualcomm | From RAN1’s understanding of UE power saving by LP-WUR, these two components can be kept in FG 62-1/1a. |
| DOCOMO | Support |
| Xiaomi | Ok with the proposal. |
| vivo | We cannot agree to remove them for now. However, it can be kept in bracket, i.e. FFS to wait until further progress in RAN2. |
| Ericsson1 | Support |
| Nordic | Features should be part of basic feature, they are essential. We agree with VIVO view. |
| ZTE, Sanechips | Serving cell offloading should be kept which is the baseline for power saving. How MR measurement is relaxed is depending on RAN2. |
| Samsung | Ok to leave them as RAN2 capability |
| Huawei, HiSilicon | RAN2 just achieved an agreement on Tuesday for this issue, copied below. Therefore, suggest to remove these two components and capture the RAN2 agreement as a note in FG62-1/1a. |
| Moderator | With the following agreement, this discussion has been closed. Agreement: Remove component 3 and component 4 in FG 62-1/1a (i.e., support of further RRM relaxation of UE MR and support of serving RRM measurement offloading) - Add the following in Note column in both FG 62-1/1a Note: According to RAN2 agreement, UE supporting LP-WUS reception shall also support RRM measurement relaxation and RRM measurement fully offloading. How to capture this is up to RAN2 |

Proposal 1-4 (closed):

- Confirm component 5 in FG 62-1/1a (i.e., the supported wake-up delay)

- Capture candidate values as [70ms, 500ms, 900ms] in Note column by updating “Component 5 candidate values: FFS” to be “Component 5 candidate values: [70ms, 500ms, 900ms]”

| Company | Comment |
| --- | --- |
| Qualcomm | Support the proposal. |
| DOCOMO | Support. The candidate values can be confirmed/revised after the issue is resolved in AI9.6.2 |
| Xiaomi | Ok to confirm the component. But the detailed values needs further determination in AI 9.6.2. |
| vivo | Fine to confirm component 5. Regarding the candidate values, fine to include [70ms, 500ms, 900ms], but it is necessary to clarify that this is for 20ms SSB periodicity. Need to capture FFS for other SSB periodicities. |
| Ericsson1 | suggest revising this component as “3. Minimum gap time between LP-WUS reception and UE to start PDCCH monitoring.”. OK to keep candidate values as FFS. |
| Nordic | We hope there is no other values than those for 20ms SSB periodicity. 😊 |
| ZTE, Sanechips | OK with the proposal |
| Samsung | Support |
| Huawei, HiSilicon | OK. Suggest to add “FFS for SSB periodicities other than 20ms” |
| Moderator | With the following agreement, this discussion has been closed. Agreement: - Confirm component 5 in FG 62-1/1a (i.e., minimum time gap between LP-WUS reception and UE to start PDCCH monitoring) - Capture candidate values: {capability 1, capability 2, capability 3} |

Question 1-5:

Companies are encouraged to share views on whether/how to define “LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence not carrying information”

- Confirm that this feature is covered by FG 62-1a

- Introduce a new FG for this feature

  - FFS Prerequisite FG: [FG 62-1 or FG 62-1a]

| Company | Comment |
| --- | --- |
| Qualcomm | The new feature that overlaid OFDM sequence does not carry information means the WUS information is indicated only by positions of the overlaid sequences. FG 62-1a is for the case that WUS information is fully carried by the overlaid sequence indices among all the candidate overlaid sequences. The UE behavior for this new feature is also different than that for FG 62-1a. For FG 62-1a, the UE needs to detect sequence index of an unknown sequence from multiple candidate sequences or absence of any sequenc in the OOK chip. For the new feature, the UE detects the presence/absence of a known sequence in the OOK chip which is essentially OOK detection. Hence, this new feature under discussion in Question 1-5 is still for OOK modulated signal and the corresponding receiver is still an OOK receiver with coherent processing. The corresponding parent FG for this new feature should be FG 62-1 instead of FG 62-1a. We prefer to have this feature defined as a new FG. This allows the network to decide the following - If all UEs support FG 62-1, but does not support this new feature, network does not have to configure the known overlaid OFDM sequences for LP-WUS to UEs - If all UEs support FG 62-1, and at least some support this new feature, network configures the known overlaid OFDM sequences for LP-WUS to UEs As discussed above, this new feature corresponds to a simpler LP-WUS detection than that for FG 62-1a when multiple candidate overlaid OFDM sequences are configured. It is basically a capability for UE coherent processing of OOK modulated LP-WUS and the its parent FG 62-1 only requires non-coherent detection of the OOK modulated LP-WUS which is even simpler. As a coherent detector, the LR that supports the new feature can measure the SSB which is also based on known OFDM sequences. |
| DOCOMO | Our understanding is that the UE supporting FG 62-1a can receive LP-WUS using OFDM overlaid sequence irrespective of that carrying information or not. For the UE supporting FG 62-1, we understand the motivation to introduce a new FG for using OFDM overlaid sequence not carrying information to improve detection performance. Assuming gNB has full flexibility to choose whether to carry information on OFDM overlaid sequence, there are following two Tx modes: - Mode1: OOK w/ overlaid OFDM sequence w/ info - Mode2: OOK w/ overlaid OFDM sequence w/o info Then for UE supporting - FG 62-1 only: - Mode1/2: Detection of LP-WUS information from binary OOK sequences via envelop detector - FG 62-1a: - Mode1: Detection of LP-WUS information from overlaid OFDM sequences via sequence detector Early termination for UE power saving - Mode2: Detection of LP-WUS information from binary OOK sequences with the support of sequence detector for known overlaid OFDM sequence Better detection performance - This new FG in addition to FG 62-1: - Mode1: Detection of LP-WUS information from binary OOK sequences via envelop detector - Mode2: Detection of LP-WUS information from binary OOK sequences with the support of sequence detector for known overlaid OFDM sequence Better detection performance According the above assumption, this FG does not restrict any gNB behavior but achieve better detection performance, and hence, we are fine to introduce this FG as advanced capability for OOK-based WUR. Note that similar proposal is made in AI9.6.1 as follows, and we are also OK to wait for the outcome there. #### [M][FL 1] Proposal 3.3-5: Regarding single overlaid OFDM sequence, support gNB configurability of single overlaid OFDM sequence without separate UE capability. |
| Xiaomi | If we want to support this case, it should be in a new FG. Since component 1 of 62-1a (LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence) is not applicable. |
| vivo | Not agreeable. Whether and how to handle the single overlaid sequence case (i.e. the case where overlaid sequence cannot carry information) should be address in 9.6.1. |
| Vodafone | Seems to be adding more effort on gNB implementation as it would need to differentiate the LP-WUS it transmits based on different UE capabilities. It is also unclear what is the gNB behaviour for UEs that support both types. Would prefer a single procedure that applies for all idle/inactive UEs. |
| Ericsson1 | If new FG is added then prerequisite should be 62-1. We are not OK with new FG having prerequisite as 62-1a . |
| Nordic | It is not clear what would network do with UEs which indicate this incapability. |
| ZTE, Sanechips | If no information is carried, the OFDM WUR should operate as OOK WUR(62-1) to receive ON-OFF pattern. No need to define new FG. |
| Samsung | Related proposal has been discussed in 9.6.1. Suggest to wait until the related agreement is made. |
| Huawei, HiSilicon | Confirm that it has been supported by FG 62-1a and add a component or note to clarify it. It is not justified to have a new FG and is not preferred to introduce unnecessary fragmentation of UE types. The feature with single overlaid sequence on OOK symbols has been agreed in AI 9.6.1 according to the following agreement and the single sequence is configurable according to the Note 2. RAN1#118bis Agreement In case of overlaid OFDM sequence not carrying information: - Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol. - Note 1: multiple overlaid OFDM sequences are specified. - Note 2: gNB can configure different overlaid OFDM sequence(s) for different cells. |

Question 1-6:

Companies are encouraged to share views on the following questions regarding “FFS: M values” in FG 62-1/1a

- Q1-6-1: Whether a UE supporting FG 62-1 or 62-1a needs to support all M values {1,2,4} or the UE can support only a part of M values (i.e., {1,2} only)

- Q1-6-2: Whether L values associated with a particular M value should also be captured in components or not

| Company | Comment |
| --- | --- |
| Moderator | Q1-6-1: A number of companies seem to be supportive for confirming all M values {1,2,4} for both FG 62-1/1a. Meanwhile some other companies prefer to have a freedom for UE not to support particular M value (e.g., 4). I also identified that functional support of some M values may not be confirmed in WI session. Q1-6-2: A number of companies seem to suggest having a clarification of relationship between M value and L value. Since views are divergent and there seems to be something that need clarifications, I don’t plan to rush this aspect this meeting. |
| Qualcomm | Q1-6-1: Our understanding is a UE supporting FG 62-1 and 62-1a support all M values that are agreed in the LP-WUS design. I.e., if M={1,2,4} are considered for LP-WUS design, the UE supports M={1,2,4} for LP-WUS, and if M={1,2} are considered for LP-WUS design, the UE supports M={1,2}. Hence, we do not think a component for M values is not needed. Q1-6-2: Unlike connected mode, L value is not agreed to be based on UE capability for idle/inactive mode. Hence, a component for L is not needed. |
| DOCOMO | Q1-6-1: For M=4, it is still being discussed in AI9.6.1 whether/how to support additional sync signal to LP-SS, and we can wait for the outcome Q1-6-2: In general, we don’t think any detail needs to be captured in UE feature list as long as such detail is captured in RAN1 spec. For this issue, the potential restriction between M and L is being discussed in AI9.6.1, and we can wait for the outcome |
| vivo | Q1-6-1: - For 62-1, since the sync is maintained by periodic LP-SS, considering the basic oscillator implementation, it is suggested that - For LP-WUS - For 15KHz SCS: support all M values (M=1,2,4) - For 30KHz SCS: Support at least M=1,2, and support of M=4 is optional - For 60KHz SCS: FFS - For 120KHz SCS: FFS - For LP-SS, support all M values as we commented under proposal 1-1 - For 62-1a, since the sync is maintained by SSB, it is proposed to confirm support for all M values. Q1-6-2: No need to capture the L values in the UE feature, given the understanding that for each supported M value, both L values are to be supported by UE. |
| Ericsson1 | Q1-6-1: support for M=1,2 should be included as part of FG 62-1 capability indication while support for M=4 can be indicated using additional capability bit. Our evaluations show that M=4 does not provide any coverage/capacity benefit for Idle mode over M=1,2. Given this, we think it would be unnecessary UE requirement to mandate always supporting M-4 for Idle mode. |
| Nordic | 6-1 M=4 for LP-WUS should be optional for the UEs, it has been showed by several companies that M=2 and M=4 perform almost the same, this means one of them is redundant, and adds only complexity. 6-2 I suppose UE has to support all. |
| ZTE, Sanchips | All M values should be supported. All the L values for each M is supported. |
| Samsung | Q1-6-1: We think UE should support all M values specified for each SCS (M values for each SCS is under the discussion in 9.6.1). Because LP-WUS is mainly targeting to the group common signal, it seems hard for the gNB to consider the different capability of UEs on supporting M value. Q1-6-2: Do not need to capture it as long as the additional UE capability for supported L value is introduced. It is already agreed that all L value for each M should be supported by a UE. |

3.2	LP-WUS operation for IDLE/INACTIVE mode

Proposal 2-1:

Confirm Component 1 in FG 62-2a as follows:

1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s)

| Company | Comment |
| --- | --- |
| Qualcomm | Yes |
| DOCOMO | OK |
| Xiaomi | Yes |
| vivo | Wait until the further progress in 9.6.1 regarding whether and how to support the case with single overlaid sequence, i.e. when overlaid sequence cannot carry information. |
| Vodafone | OK |
| Ericsson1 | Component 1 can be confirmed as “1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s)” |
| ZTE, Sanechips | OK |
| Samsung | From my understanding, at least (s) should be kept. |
| Huawei, HiSilicon | OK. |

Proposal 2-2 (closed):

Confirm Component 2 in FG 62-2/2a “2. The supported procedure(s)”:

| Company | Comment |
| --- | --- |
| Qualcomm | Yes |
| DOCOMO | OK |
| Xiaomi | Yes |
| vivo | OK |
| Vodafone | OK |
| Ericsson1 | OK |
| ZTE, Sanechips | OK |
| Samsung | OK |
| Moderator | With the following agreement, this discussion has been closed. Agreement: Confirm Component 2 in FG 62-2/2a “2. The supported procedure(s)”: - Note: Procedure(s) refer to Option 1-1, 1-2. |

Question 2-3:

Companies are encouraged to share views on candidate values for FG 62-2/2a component 2:

- Alt-1: {Option 1-1, Option 1-2, both Option 1-1 and 1-2}

- Alt-2: {Option 1-1, both option 1-1 and 1-2}

- Alt-3: {Option 1-1, Option 1-2}

- Alt-4: No candidate values (i.e., all UEs supporting FG 62-2/2a support both Option 1-1 and 1-2)

| Company | Comment |
| --- | --- |
| Qualcomm | Alt-4 is not feasible. Since Option 1-1 and Option 1-2 are exclusive configurations/operations, Alt-4 will cause interoperability issue. Alt-3 is quite weird and does not make sense. This prohibits a UE to support both. Alt-2 implies that, assuming the FG is per-band, a UE can indicate support of Option 1-2, only if/after implementation and interoperability of Option 1-1 are ensured for the given frequency band. This would be trouble to enable Option 1-2. With the above, we propose Alt-1. |
| DOCOMO | In our understanding, Option 1-2 can be regarded as generalized feature of Option 1-1 (i.e., Option 1-1 can be realized by proper configuration of Option 1-2). Therefore, it is preferred to support {Option 1-1, Both Option 1-1 and Option 1-2} so that at least Option 1-1 is supported by all UEs using LP-WUS, and NW operation can be simplified |
| Xiaomi | Support Alt-1 |
| vivo | Alt-4 is most preferable to reduce the implementation fragmentation. Alt-1 is also fine. Alt-2 is not necessary, no need to assume option 1-1 as the default implementation. |
| Vodafone | Support either Alt-2 or Alt-4 to have at least Option 1-1 as baseline |
| Ericsson1 | Support Alt-2 |
| ZTE, Sanechips | Alt4 is preferred for a unified design. |
| Samsung | Prefer Alt 2. |
| Huawei, HiSilicon | Prefer no additional UE capability signaling for this and all UE capable of FG 62-2/2a support both Option 1-1 and Option 1-2. Alt-2 is unnecessary to prioritize Option 1-1 because of much less UE power saving gain provided by Option 1-1. |

Proposal 2-4:

For FG 62-2/2a Component 3 and its candidate values,

- Confirm Component 3 text “3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode”

- Update candidate values as “Component 3 candidate values: {[V1], [V2], [V3]}”

  - Note: Actual value for V1/V2/V3 to be decided subject to WI progress

| Company | Comment |
| --- | --- |
| Qualcomm | Yes |
| DOCOMO | Support |
| Xiaomi | Yes |
| vivo | OK |
| Ericsson1 | Support |
| ZTE, Sanechips | OK |
| Samsung | OK |

Proposal 2-5:

Add a new component “Maximum number of codepoints checked per MO” in FG 62-2 and 62-2a

- FFS: Candidate values

| Company | Comment |
| --- | --- |
| Qualcomm | We are OK with either Proposal 2-5 or Proposal 2-6 (no need both). |
| DOCOMO | Support this proposal based on the working assumption made in the last RAN1 meeting |
| Xiaomi | It can be dependent on the discussion in AI 9.6.1. It seems Moderate of AI 9.6.1 suggest to remove this capability. |
| vivo | OK |
| Ericsson1 | OK |
| ZTE, Sanechips | OK |
| Samsung | Suggest to wait to confirm the working assumptions |

Proposal 2-6:

Add a new component “Maximum number of overlaid sequences applicable per OOK ON chip” in FG 62-2a

- FFS: Candidate values

| Company | Comment |
| --- | --- |
| Qualcomm | We are OK with either Proposal 2-5 or Proposal 2-6 (no need both). |
| DOCOMO | We don’t have any explicit agreement whether UE can support less number of candidates overlaid sequences per OOK ON chip than the maximum number. We are open to discuss this issue but want to see justification from proponent for each candidate values. |
| Xiaomi | It can be dependent on the discussion in AI 9.6.1. It seems Moderate of AI 9.6.1 suggest to remove this capability. |
| vivo | Fine to capture the number of candidates overlaid sequences as agreed before, but the values shall not be FFS (should be 16/8/4 for M=1/2/4 respectively) Agreement For RRC connected mode, maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, - support maximum 16 candidates overlaid sequences for M=1 - support maximum 8 candidates overlaid sequences for M=2 - support maximum 4 candidates overlaid sequences for M=4 (agreed in RAN1#119) - For candidate overlaid sequences across all OOK ON chips of LP-WUS, the number of roots (in specification) is up to [FFS: X], FFS whether the number of roots can be different for different M value. - FFS: The number of overlaid sequences applicable for a UE is no more than 2 per OOK ON chip. - FFS: Whether or not the candidate set of overlaid sequences change across OOK ON chips |
| Ericsson1 | Agree with comment by vivo |
| ZTE, Sanechips | OK |
| Samsung | Same view with vivo |
| Huawei, HiSilicon | Similar comment as vivo, a subbullet should be added as “No additional UE capability signaling for this component” because only the number of codepoints could be a UE capability based on previous discussion and the following agreement removing the two FFS in red. RAN1#121 Agreement Proposal 3.3-4: Update the agreements as below: Agreement For idle mode, regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell: - support maximum 16 candidates overlaid sequences for M=1 - support maximum 8 candidates overlaid sequences for M=2 - For candidate overlaid sequences across all OOK ON chips of LP-WUS, the number of roots (in specification) is up to [FFS: X], FFS whether the number of roots can be different for different M value. - FFS: The number of overlaid sequences applicable for a UE is no more than 2 per OOK ON chip.. Agreement For RRC connected mode, maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, - support maximum 16 candidates overlaid sequences for M=1 - support maximum 8 candidates overlaid sequences for M=2 - support maximum 4 candidates overlaid sequences for M=4 (agreed in RAN1#119) - For candidate overlaid sequences across all OOK ON chips of LP-WUS, the number of roots (in specification) is up to [FFS: X], FFS whether the number of roots can be different for different M value. - FFS: The number of overlaid sequences applicable for a UE is no more than 2 per OOK ON chip. - FFS: Whether or not the candidate set of overlaid sequences change across OOK ON chips. Note: The WUS information carried by overlaid OFDM sequence is based on codepoint i.e. raw information bit |

Question 2-7:

Companies are encouraged to share views on the following questions regarding “FFS: M values” in FG 62-2/2a

- Q2-7-1: Whether a UE supporting FG 62-2 or 62-2a needs to support all M values {1,2,4} or the UE can support only a part of M values (i.e., {1,2} only)

- Q2-7-2: Whether L values associated with a particular M value should also be captured in components or not

| Company | Comment |
| --- | --- |
| Moderator | If companies can clarify whether different approaches could be considered between 62-1/1a and 62-2/2a (i.e., between IDLE/INACTIVE mode feature and CONNECTED feature), it could be helpful. |
| Qualcomm | Different approaches for defining FG(s) for idle/inactive and connected modes should be fine – anyway the procedures are quite different. Regarding whether to allow a UE to report supported {M, L} as part of the FG, there are still many uncertainties in the design discussed under 9.6. We prefer to see a bit more progress. |
| DOCOMO | Same comments as (second) Question 1-6. |
| vivo | For CONNECTED mode, since the sync can be maintained by periodic MR activities, UE can support all M values, i.e. Q2-7-1: all M values can be supported Q2-7-2: Not needed as LP-SS is not relevant to LP-WUS operation in CONNECTED mode. |
| Ericsson1 | For support of M values, our preference is - Candidate value set for M: {{(1,2}, {1,2,4}}. However, we are open to have different approach for connected if needed. |
| ZTE, Sanechips | All M values should be supported. No clear necessity to capture L value for each M value. |
| Samsung | Q2-7-1: Same view in IDLE/INACTIVE mode Q2-7-2: LP-SS is not necessary for LP-WUS operation in CONNECTED mode. |
| Huawei, HiSilicon | For connected mode, there is no LP-SS and value L is not needed here. |

Proposal 2-8:

Remove “FFS: SCS” from FG 62-2 and 62-2a

| Company | Comment |
| --- | --- |
| Qualcomm | Yes |
| DOCOMO | Support |
| xiaomi | One question to clarify, to remove the FFS, then UE should support all the candidate SCSs. And should we specify what are the all candidate SCSs？ |
| vivo | Fine |
| Vodafone | Support, assuming per band operation, LP-WUS should assume the same SCS of the band in question |
| Ericsson1 | OK |
| ZTE, Sanechips | The following can be considered: LP-WUS and LP-SS share the same band as SSB, and the SCS is the same as initial SSB if initial DL BWP has the same SCS as SSB |
| Samsung | Support |
| Huawei, HiSilicon | In order to avoid any future discussion to introduce a UE capable of active DL BWP with SCS#1 but incapable of LP-WUS with SCS#1, suggest to make a conclusion together: Conclusion: No additional UE capability signaling with respect of SCS of LP-WUS for FG 62-2 and 62-2a. |

Proposal 2-9:

Confirm “Per band” in FG 62-2/2a/3

| Company | Comment |
| --- | --- |
| Qualcomm | We prefer “Per band”, subject to the assumption that a UE can indicate support of LP-WUS for CA/DC using separate FG(s). Today, many non-RedCap UEs support CA/DC with various band combinations. There are many CA/DC band combinations that are regionally specific or operator specific. If FG62-2/2a/3 are defined “Per band” and if there is no additional FG(s) for CA/DC, the UE shall be able to operate LP-WUS with all the CA/DC band combinations that the UE supports. We do not think “Per band” without any additional FG(s) for CA/DC does not make sense, especially from testability perspective. If no separate FG(s) for CA/DC at all, then “Per band” would be problematic, and we need to reconsider “Per FS (per band per band combination)”. |
| DOCOMO | Support |
| Xiaomi | Share similar view as Qualcomm |
| vivo | Fine |
| Vodafone | OK to confirm |
| ZTE, Sanechips | Ok |
| Samsung | Support |

Question 2-10:

Companies are encouraged to share views on the following questions regarding whether/how to define UAI for preferred time offset between LP-WUS and PDCCH reception agreed in the following:

Agreement For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported candidate value - UE can indicate its preference for configured time offset between LP-WUS and PDCCH reception using UAI mechanism. FFS details. UAI is optional. - FFS: A single time offset is configured for a UE at a given time

- Q2-10-1: Which WG to define this feature; RAN1 or RAN2?

- Q2-10-2: In case it is to be defined in RAN1, which approach to adopt; add a component in FG 62-2/2a to report whether to support UAI, or introduce a new FG?

| Company | Comment |
| --- | --- |
| Qualcomm | RAN2 sent out an LS asking details of the UAI, and RAN1 would response it in this meeting. Perhaps it would be simpler if RAN1 includes a guidance to RAN2, especially whether the UAI should be a new FG or a part of the existing FGs, and ask RAN2 to define it. We think a new FG is necessary for the UAI, similar to RAN2 Rel-16 FGs 19-1/2/3/4/5. |
| DOCOMO | Q2-10-1: the UAI will be specified in RAN2 spec and it is better if RAN2 can define the corresponding UE feature Q2-10-2: Rather than support a component in FG 62-2/2a with candidate values of {supported, not supported}, it would be better to introduce another FG having prerequisite of {FG 62-2 or FG 62-2a} |
| Xiaomi | Q2-10-1: RAN1 Q2-10-2: a new FG is more appropriate, since it is not a mandatory so not suitable as a component of existing FG. |
| vivo | Q2-10-1, either RAN1 or RAN2 is fine Q2-10-2: add a component in FG 62-2/2a to report whether to support UAI, no need of unnecessary fragmentation. |
| Ericsson1 | Q2-10-1: OK to leave this to RAN2 |
| ZTE, Sanechips | Prefer to leave it to RAN2. |
| Samsung | Q2-10-1: It seems better to define in RAN2 spec. Q2-10-2: Because UAI is optional. Separate FG seems better. |
| Huawei, HiSilicon | UAI is optional in the RAN1 agreement. Therefore, a new FG is needed. |

Question 2-11:

Companies are encouraged to share views on the following questions related to LP-WUS operation in CONNECTED mode in CA/DC scenario:

- Q2-11-1: Whether support of LP-WUS operation in CONNECTED mode in DC scenario should be separate FG from FG 62-2/2a or not

- Q2-11-2: Whether support of LP-WUS operation in CONNECTED mode in CA scenario should be separate FG from FG 62-2/2a or not

| Company | Comment |
| --- | --- |
| Qualcomm | Yes for both questions. Today, many non-RedCap UEs support CA/DC with various band combinations. There are many CA/DC band combinations that are regionally specific or operator specific. If FG62-2/2a/3 are defined “Per band” and if there is no additional FG(s) for CA/DC, the UE shall be able to operate LP-WUS with all the CA/DC band combinations that the UE supports. We do not think “Per band” without any additional FG(s) for CA/DC does not make sense, especially from testability perspective. If no separate FG(s) for CA/DC at all, then “Per band” for FGs 62-2/2a would be problematic, and we need to reconsider “Per FS (per band per band combination)”. |
| DOCOMO | Q2-11-1: For DC scenario where LP-WUS is configured per cell group, we understand it would increase UE complexity to monitor LP-WUS for both MCG and SCG, and open to discuss this scenario Q2-11-2:We are not sure whether it is necessary since FG 62-2/2a is the UE capability whether UE can receive LP-WUS by LR and trigger PDCCH monitoring. For LP-WUS reception itself, it is not related to CA. For triggering PDCCH monitoring, it is commonly controlled per cell group, including both intra-band CA and inter-band CA, and any differentiation within cell group would not be necessary. |
| Xiaomi | Q2-11-1: Yes. For DC scenario, we have agreed that LP-WUS can be transmitted on MCG/SCG separately, so a separate FG can be used to indicate the band combinations supported by UE for receiving LP-WUS on both bands. Q2-11-1: Currently no. If there are further agreements to support two LP-WUS signals transmitted on each C-DRX group in CA scenario, then a separate FG can be considered. |
| vivo | Q2-11-1: Support of LP-WUS operation in CONNECTED mode in DC scenario should be separate FG from FG 62-2/2a, since it would require the UE to monitor LP-WUS on both cell groups Q2-11-2: Support of LP-WUS operation in CA mode does not need to be separated from FG 62-2/2a (i.e. the same handling for Rel-16 DCP with CA), since it was agreed that LP-WUS is only monitored on Pcell in cases of CA. |
| Ericsson1 | Q2-11-1: open to separate FG Q2-11-2: Prefer not to have separate FG |
| ZTE, Sanechips | Yes for DC since deploying LP-WUS in two cell group require additional complexity and hardware. No for CA case, since it does not require the UE to increase the complexity to support this. |
| Samsung | Q2-11-1: Support Q2-11-2: We don’t think it is necessary, because LP-WUS is monitored on PCell. |
| Huawei, HiSilicon | In the following agreement, cross-CG triggering is not supported. Therefore, the UE behavior of LP-WUS is the same as CA/single carrier regardless whether DC is configured. Separate FG for DC is unnecessary. Hope proponent could clarify why separate FG for DC is needed in the following example: a UE reports FG 62-2/2a for Band A and the UE is configured with single carrier on band A in PCG and configured with CA/single carrier in the SCG. In our understanding, the UE must support this DC configuration in the example. RAN1#118bis Agreement LP-WUS is supported when UE is configured with NR-DC in RRC CONNECTED mode - Above is supported for the case PDCCH monitoring is triggered by LP-WUS in the same cell group - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable |

- Conclusions

Proposals not agreed this meeting:

Question 1-5: Companies are encouraged to share views on whether/how to define “LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence not carrying information” - Alt-1: Confirm that this feature is covered by FG 62-1a - Alt-2: Introduce a new FG for this feature - FFS Prerequisite FG: [FG 62-1 or FG 62-1a] Question 2-11: Companies are encouraged to share views on the following questions related to LP-WUS operation in CONNECTED mode in CA/DC scenario: - Q2-11-1: Whether support of LP-WUS operation in CONNECTED mode in DC scenario should be separate FG from FG 62-2/2a or not - Q2-11-2: Whether support of LP-WUS operation in CONNECTED mode in CA scenario should be separate FG from FG 62-2/2a or not

References

- R1-2503293	UE features for Rel-19 LP-WUS	Huawei, HiSilicon

- R1-2503323	Discussion on WUR features	ZTE Corporation, Sanechips

- R1-2503389	UE feature for Rel-19 LP-WUS/WUR	vivo

- R1-2503403	LP-WUS/WUR UE features	Nokia

- R1-2503595	Discussion on UE features for LP-WUS/WUR	Samsung

- R1-2503785	LP-WUS/WUR UE feature	CATT

- R1-2503854	Discussion on UE features for low-power wake-up signal and receiver for NR	CMCC

- R1-2503904	UE features for NR LP-WUS/WUR	Xiaomi

- R1-2504022	UE features for Rel-19 LP-WUS/WUR WI	Ericsson

- R1-2504190	Discussion for UE features of LP-WUS/WUR	OPPO

- R1-2504257	UE features for LP-WUS/WUR for NR	LG Electronics

- R1-2504356	On Rel-19 LP-WUS/WUR UE features	Apple

- R1-2504423	UE features for LP-WUS/WUR for NR	Qualcomm Incorporated

- R1-2504485	UE features for LP-WUS/WUR for NR	MediaTek Inc.

- R1-2504527	Discussion on UE features for LP-WUS/WUR for NR	NTT DOCOMO, INC.

Appendix: LP-WUS/WUR UE features after RAN1#120bis

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62. NR_LPWUS | 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK [2. The support of LP-SS based RRM measurement and/or SSB-based measurement, corresponding to component 1] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported | Per Band | n/a | n/a | n/a | Component 5 candidate values: FFS | Optional with capability signalling |
| 62. NR_LPWUS | 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence [2. FFS: The support of LP-SS based RRM measurement and/or [X. FFS: The support of SSB-based RRM measurement] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence is not supported | Per band | n/a | n/a | n/a | Component 5 candidate values: FFS | Optional with capability signalling |
| 62. NR_LPWUS | 62-2 | LP-WUS operation in CONNECTED mode based on OOK signal | 1. LP-WUS operation in CONNECTED mode based on OOK signal 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP FFS: M values FFS: SCS FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OOK signal is not supported | [Per band] | n/a | n/a | n/a | Component 2 candidate values: FFS Component 3 candidate values: FFS | Optional with capability signalling |
| 62. NR_LPWUS | 62-2a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence | 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP FFS: M values FFS: SCS FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported | [Per band] | n/a | n/a | n/a | Component 2 candidate values: FFS Component 3 candidate values: FFS | Optional with capability signalling |
| 62. NR_LPWUS | 62-3 | LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode | 1. Support of LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode | FG 62-2 or 62-2a | YES | n/a | LP-WUS frequency resource outside active DL BWP for LP-WUS operation in CONNECTED mode is not supported | [Per band] | n/a | n/a | n/a |  | Optional with capability signalling |