3GPP TSG RAN WG1 #122                                                           R1- 2505157

Bengaluru, India, August 25th – 29th, 2025

Source:	Kyocera

Title:	AI/ML in 6GR interface

Agenda item:	11.6

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss AI/ML for 6GR and radio access network in the physical layer, focusing on the objectives agreed on in RAN1#108 meeting [1] and are outlined as follows:

AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

According to the above objectives, the initiative should focus on integrating AI/ML into the design of 6G RAN, building on the existing 5G NR AI/ML framework [2]. New (or existing) use cases should be identified considering their commercial demand (performance) and practicality (computational complexity). Moreover, the AI/ML framework should aim to develop extensible enablers tailored to the new (or existing) use cases, covering LCM procedures and data collection/management. Importantly, the 6G system must also be capable of operating without AI/ML to ensure flexibility and resilience in network design.

In the remainder of this paper, we concentrate on two key areas: the potential advancements enabled by AI/ML in 6G RAN and the inherent limitations of these technologies. A balanced examination of both aspects can help companies identify potential use cases that align with commercial viability and market demand.

# AI/ML driven enhancements in next generation RAN

In this section, we highlight potential areas where AI/ML can deliver meaningful enhancements to communication systems, areas that we believe may also hold strong commercial interest.

## Specification support for Neural Receivers

In this section, we highlight aspects where the Neural Receiver (NRx) can provide an enhancement in communication systems. It is important to note that receiver design is typically left to implementation. However, depending on the outcomes of future studies, standardization efforts may need to focus on enabling technologies that will aid the operation of the NRx.

### Enhanced NW coverage

One of the primary objectives discussed by companies during 6G Day 1 was the reduction of operational costs, both at the NW and UE levels. In FR2 frequency bands, the higher frequency leads to increased signal attenuation, which in turn reduces cell coverage. From the network perspective, this necessitates the deployment of additional cell sites to achieve coverage comparable to that of lower-frequency bands such as FR1. On the UE side, the same attenuation challenge results in diminished UL coverage. To compensate, UEs must increase their transmit power, which can negatively impact battery life and potentially push the power amplifier (PA) into its non-linear operating region.

In response to this demand,  research has highlighted the potential of AI/ML technologies to enhance network coverage in UL the direction. Notably, various academic results suggest that NRx could improve UL coverage by enabling the UE to transmit within the nonlinear region of its PA. These receivers, deployed at the gNB, can effectively decode the resulting distorted signals using advanced neural processing techniques. However, while this approach mitigates the effects of nonlinear PA operation, increasing the UE’s transmit power may lead to channel leakage into adjacent carriers, potentially impacting neighbouring cells. As a result, it may become necessary for these adjacent cells to also adopt NRx technology to maintain performance parity.

Furthermore, no system-level evaluations are present to quantify the benefits of integrating NRx at  the network  side. As a result, a study assessing the broader system-level impacts of NRx deployment is necessary.

### Reliable channel equalization

One of the primary advantages of integrating AI/ML techniques into communication systems lies in their potential to significantly improve channel equalization performance in both uplink (UL) and downlink (DL) directions. As outlined in Section 1, the development of AI/ML-based solutions must carefully balance technological innovation with system complexity. NRx have demonstrated the ability to enhance the robustness of symbol detection while simultaneously reducing the overhead associated with transmitting reference signals (RS). Figure 1 demonstrates the capability of NRx to perform resilient channel equalization under adverse propagation conditions, e.g., specifically in high-mobility scenarios, utilizing a sparse DMRS configuration. It is important to note that the LMMSE based effective channel estimation along with MMSE receiver failed to perform channel equalization in high mobility scenarios and hence excluded from Figure 1.

Figure 1 PUSCH performance.

Based on this discussion, NRx capability have the potential to pave the way for several promising use cases, including:

- DMRS Overhead Reduction: By deploying NRx at either the UE for DL or the gNB for UL, it becomes feasible to reduce the number of DMRS pilot symbols required to maintain a given BLER performance. Furthermore, NRx enables the use of non-orthogonal pilot symbols or pilots superimposed with data, thereby further minimizing pilot-related overhead.

- Custom Modulation Constellations: NRx can facilitate the design of custom constellation shapes tailored to specific RF impairments, such as phase noise (PN), improving overall system resilience.

- Higher-Order Modulations: Given NRx’s demonstrated robustness in symbol detection, it is worthwhile to explore the feasibility of employing higher-order modulation schemes, to enhance spectral efficiency.

- Data Collection and Management: Depending on the outcomes of ongoing NRx evaluations, RAN1 should consider studying LCM procedures for data collection and management to support both training and inference phases of NRx deployment.

Channel equalization through AI/ML models can be implemented at various points within the communication system. For UL transmissions, the model can be deployed on the network side, while for DL, it may reside on the UE. Alternatively, a two-sided architecture—leveraging both network and UE capabilities—can enable more advanced signal processing functionalities, such as support for arbitrary modulation schemes and RS patterns.

However, due to the significant computational demands associated with NRx, deploying them on the network side for UL processing may be the more practical approach in the near term. That said, the exploration of UE-side and two-sided implementations remains an open area of study and is not precluded.

Based on the previous discussions, we have the following proposals:

- RAN1 should initiate a study focused on identifying and evaluating the AI/ML enablers necessary for the effective deployment of Neural Receivers. This study should also explore the associated use cases that could benefit from such advancements, including but not limited to:

  - Non-Orthogonal and Superimposed Pilots: Investigating the feasibility and performance gains of using non-orthogonal pilot structures and pilots superimposed with data to reduce reference signal overhead.

  - Custom Modulation Constellations: Exploring the design of modulation constellations tailored to specific RF impairments, enabled by the adaptability of AI/ML-based receivers.

  - High-Order Modulation Schemes: Assessing the potential of NRx to support higher-order modulation formats (e.g., 4096-QAM) with improved robustness and spectral efficiency.

  - Data Collection and Management: Studying the requirements and mechanisms for efficient data collection, labeling, and management to support the training and inference processes of AI/ML models in NRx.

- Companies should provide system-level simulation results to quantify the performance gains achievable using Neural Receivers. These evaluations should also assess the feasibility and practical considerations of implementing Neural Receivers on the UE side, considering computational complexity, power consumption, and real-time processing constraints.

# Technical and operational challenges of AI/ML for 6G RAN

## NW side additional conditions (UE side models)

During the Rel-19 discussions, a key topic of interest was the handling of NW side conditions when utilizing AI/ML models on the UE side. This issue arises when the AI/ML model is trained under specific network conditions, but inference is performed under different conditions—resulting in a mismatch between the training and test data distributions. To address this distribution shift, an "associated ID" was introduced to help contextualize the network conditions during inference. However, network vendors expressed concerns that this ID could potentially expose proprietary information. As a result, its usage is restricted to within the same cell or across a limited set of cells managed by the same vendor. Consequently, while the associated ID offers a partial solution, it does not fully resolve the challenge of adapting to varying network conditions. As an alternative, more advanced AI/ML techniques—such as meta-learning—can be employed. These approaches enable the model to adapt its learned parameters based on the new data distribution, thereby improving generalization across diverse network environments.

- RAN1 should further investigate the feasibility of implementing more robust solutions to address the network-side additional conditions problem. The study may include the technology enablers of advanced AI/ML methodologies, such as meta-learning, which enable models to dynamically adapt to varying data distributions encountered during inference.

## Real time inference (UE/NW side models)

One of the primary challenges in deploying AI/ML-based solutions is the requirement for real-time inference. Inference latency is influenced by several factors, including the underlying hardware platform, software stack, and the level of code optimization. These latency constraints impose limitations on the design and complexity of neural network architectures. Given that network configurations can change within milliseconds, it is essential to utilize models capable of delivering inference within comparable timeframes. This requirement significantly restricts the architectural flexibility of neural networks and may consequently impact their overall performance and adaptability.

- During the study item phase, in addition to presenting performance gains for any considered use case, it is proposed that inference latency results also be reported. This will ensure a comprehensive evaluation of AI/ML-based solutions, particularly in scenarios where real-time responsiveness is critical.

## Other issues

### Data management and privacy preservation (UE/NW side models)

AI/ML models are vulnerable to adversarial attacks that can manipulate input data, making robustness essential, especially in sensitive or critical applications. Additionally, collecting and processing user data introduces privacy risks, highlighting the need for secure and privacy-preserving AI/ML data collection techniques.

Moreover, obtaining large datasets for AI/ML model training is difficult due to the high costs and logistical complexities of data collection. Therefore, it is essential to consider application-specific data collection procedures to avoid gathering unnecessary or irrelevant data. For example, in the case of radio link failure (RLF) prediction, data collection efforts can be focused on areas with historically high RLF rates—such as cell edges or handover zones—rather than uniformly sampling across the entire network. This targeted approach ensures that the collected data is both relevant and efficient for training models tailored to the specific use case.

### Computational resource constraints (UE side models)

Deep learning models can strain the limited computational and power resources of wireless devices, especially in edge computing scenarios. Hence, for UE side AI/ML models, it's essential to design lightweight models that maintain high performance while operating efficiently, e.g., models with limited energy consumption and energy usage.

- RAN1 should study enhancements to data collection and data management tailored to the specific new use cases.

# Conclusion

In the contribution presented, we had the following proposals:

- RAN1 should initiate a study focused on identifying and evaluating the AI/ML enablers necessary for the effective deployment of Neural Receivers. This study should also explore the associated use cases that could benefit from such advancements, including but not limited to:

  - Non-Orthogonal and Superimposed Pilots: Investigating the feasibility and performance gains of using non-orthogonal pilot structures and pilots superimposed with data to reduce reference signal overhead.

  - Custom Modulation Constellations: Exploring the design of modulation constellations tailored to specific RF impairments, enabled by the adaptability of AI/ML-based receivers.

  - High-Order Modulation Schemes: Assessing the potential of NRx to support higher-order modulation formats (e.g., 4096-QAM) with improved robustness and spectral efficiency.

  - Data Collection and Management: Studying the requirements and mechanisms for efficient data collection, labeling, and management to support the training and inference processes of AI/ML models in NRx.

- Companies should provide system-level simulation results to quantify the performance gains achievable using Neural Receivers. These evaluations should also assess the feasibility and practical considerations of implementing Neural Receivers on the UE side, considering computational complexity, power consumption, and real-time processing constraints.

- RAN1 should further investigate the feasibility of implementing more robust solutions to address the network-side additional conditions problem. The study may include the technology enablers of advanced AI/ML methodologies, such as meta-learning, which enable models to dynamically adapt to varying data distributions encountered during inference.

- During the study item phase, in addition to presenting performance gains for any considered use case, it is proposed that inference latency results also be reported. This will ensure a comprehensive evaluation of AI/ML-based solutions, particularly in scenarios where real-time responsiveness is critical.

- RAN1 should study enhancements to data collection and data management tailored to the specific new use cases.

# References

| [1] | NTT DOCOMO (Moderator), “RP-251881, New SID: Study on 6G Radio,” 3GPP TSG RAN Meeting #108, Prague, Czech Republic, 2025. |
| --- | --- |
| [2] | TR 38.843 v2.0.1, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (Release 18),” Third Generation Partnership Porject (3GPP), 2023. |

# Appendix

In this section, the simulation assumptions used to generate Figure 1 are stated and can be found as follows:

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 4 GHz |
| Channel model | CDL-E |
| Number of transmit antennas at the UE | 1 |
| Number of receive antennas at the gNB | 2 |
| Delay spread | 100 ns |
| DMRS configuration | Configuration type 1 with 1 additional position |
| Modulation | 16 QAM |
| Code rate | 658/1024 |
| Resource grid size | 26 RBs |