3GPP TSG RAN WG1 #122-bis	R1-2506741

Prague, Czechia, October 13th – 17th, 2025

Agenda Item:	11.5

Source:	Futurewei

Title:	Discussion on 6G Energy Efficiency Techniques

Document for:	Discussion and decision

# Introduction

In RAN # 108 meeting [1] the Study on 6G Radio SID was agreed.

“The study aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements study [TR38.914].

The study should strive at dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, focusing on practical user experience. The study should identify principles to ensure extensibility and deliver superior performance.”

Among the 9 detailed objectives of this study the first objective provides a general framework:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving both for network and device.

  - Enhanced spectral efficiency.

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage.

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz.

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

  - Target scalable and forward compatible design for diverse device types.

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations.

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs.

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

In RAN#109, the following agreements were made (RP-252883):

Proposal 1: For 3GPP internal evaluation, define energy efficiency as a quantitative KPI in 3GPP for both network and device.

Proposal 2: For 3GPP internal evaluation, at least empty load and partial load cases should be evaluated in 6GR study.

-	Evaluate via system level simulations (SLS)

-	Other evaluation methods are not precluded when relevant

Proposal 3: For 3GPP internal evaluation, UE/network power model is to be discussed in RAN1.

The energy efficiency item from the above objective is to be discussed in RAN1 #122bis agenda item 11.5:

“Including evaluation assumptions, proposals for Evaluations, NW power saving, UE power saving, and joint mechanisms taking both NW and UE into account for power saving, targeting to categorize proposals by RAN1#123. From RAN1#124, proposals will be distributed to respective related agenda.”

In this contribution, we discuss energy saving solutions according to time-domain, frequency-domain, spatial-domain, and power-domain categorization.

# Background

In 5G NR several energy saving solutions were adopted. These solutions aim to decrease energy consumption at UE, gNB or both. While several studies show their efficiency, since they were introduced in later releases of 5G, backward compatibility with prior designs limited their potential gains. Furthermore, some of the solutions for UE power saving and Network energy saving were developed independently without thorough consideration of the mutual impact and the energy efficiency of the overall system; see Figure 1.

Figure 1: Timeline of UE Power Saving and Network Energy Saving Work in 3GPP NR

Based on this observation, prior discussions in RAN workshop [2], and the 5G Study document [1], we could identify the following design principles for improved energy efficiency in 6G:

Native Energy Efficiency: Energy efficiency should be a fundamental requirement for all 6G design aspects from Day 1, rather than an add-on feature. This includes considering energy efficiency during the design of the 6G carrier, antenna architectures, and Control/Data procedures.

Joint Network and UE Power Saving: Proposals emphasize a unified and harmonized design for energy saving that benefits both the network and the UE simultaneously, including Idle, Inactive, and Connected RRC states. This aims to minimize unnecessary power consuming transmissions/receptions and synchronize sleep times at both the UE and gNB.

Simplified System Design: 6G should adopt a simpler design with minimized complexity and optionality to ease implementation, reduce cost, and inherently lower energy consumption.

Performance vs. Energy Trade-off: The design should allow for a balanced trade-off between energy saving gains and performance, ensuring that QoS/QoE is not compromised. If it is feasible for a feature to improve both energy efficiency and throughput/latency performance, such a feature should be prioritized.

Mandatory Feature Support: Energy saving features should be mandatory for all 6G devices and networks from the first release to ensure widespread adoption and impact.

KPI Definition: New KPIs should be defined to meaningfully measure energy consumption, especially on the network side.

Proposal 1: For 6G energy efficiency consider the following design principles:

- Native Energy Efficiency

- Joint Network and UE Power Saving

- Simplification of the Design

- Balanced Trade-off between Energy Saving and Performance

- Energy Efficiency as Mandatory Features

# Power Consumption Models and Reference Configuration

In RAN1#122, agreements were reached to reuse or update the NR’s reference configuration and power consumption models for the 6G UE and BS as follows.

Agreement Study reference configurations and power consumption model for 6G UE, considering but not restricted to the following: - TR 38.840 (UEPS), TR38.875 (RedCap), TR38.865 (eRedCap), and TR38.869 (LP-WUS/WUR) for reference configurations - TR 38.840 (UEPS), TR38.875 (RedCap), and TR38.869 (LP-WUS/WUR) for power consumption models Agreement Study how/whether to reuse or update the power model in TR 38.864 for evaluating BS power consumption for 6G BS. Agreement Study how to reuse and update reference configurations in TR 38.864 for 6G BS.

In this section, we discuss our views on the 6G UE and BS power consumption models.

## 6G UE Models and Reference Configuration

For UE power saving/energy efficiency evaluation in 6G, the Rel-16 UE power consumption model [TR38.840] and its Rel-17 adaptation for RedCap NR devices [TR38.875] can be reused as a starting point. The additional sleep power state for main radio (MR) of a UE and the low-power wake-up radio (LP-WUR) power model introduced in Rel-18 for LP-WUS [TR38.869] should also be considered. Note that the study in TR38.865 focuses on complexity and coverage impacts for eRedCap NR devices, but not power consumption.

The current status for MR’s power states, their characteristics, relative power consumption values, and sleep to non-sleep states transition energy is captured in Table 3 and Table 4 of Annex 8.1 for non-RedCap and RedCap devices assuming the following changes with respect to the reference configuration for FR1 in TR38.840: 20MHz bandwidth, 2RX chains, 1TXchain, same-slot scheduling, N=8 cells for intra-/inter-frequency measurements. Further, the current status for LP-WUR’s power states and transition energy is captured in Table 1 ([8], Table 6.3-2).

Table 1: UE Power Consumption Model for LP-WUR.

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| Off | WUR power consumption when turned off, i.e., the LP-WUR does not perform monitoring | POFF ∈ (0.001, 0.02, 0.1) |
| On | WUR power consumption when turned on, i.e., the LP-WUR performs monitoring | PON ∈ {0.01, 0.05, 0.1, 0.2, 0.5, 1, 2, 4, 10, 20, 30} |
| Note: transition energy is determined as [TLR, ramp-up *(PON-POFF)/2] where TLR, ramp-up=10 ms is the WUR ramp-up time in ms. |  |  |

In our opinion, the UE power consumption models as captured in TR38.840 and TR38.869 may be revisited or simplified. As an example, some more conservative values may be removed to reflect improvements in UE/device manufacturing technology, e.g. instead of {400,800} ms for LP-WUR transition time from ultra-deep sleep state only the value of {400}ms is needed.

Observation 1: The NR UE power consumption model in TR38.840 may be revisited or simplified to reflect improvements in 6G UEs.

Additionally, after the finalization of the OOK-based and OFDM-based LP-WUS design in the Rel-19 LP-WUS WI, there should be a clearer idea on the relative power consumption values for the LP-WUR’s power states and the Rel-18 LP-WUR’s power consumption model should be updated accordingly. For example, the relative power value for LP-WUR power consumption when turned on may be limited to PON∈0.1, 1 corresponding to envelope detection based and OFDM based WUR, respectively. Additionally, the relative power value for LP-WUR power consumption when turned off may be limited to POFF∈0.02, 0.1. The removal of the higher relative power values are due to the fact that the Rel-19 LP-WUS design is based on sequence detection that may not require implementation of any FFT module and with relaxed requirements on clock accuracy.

Observation 2: For LP-WUR evaluations, the relative power value for LP-WUR power consumption when turned on/off may be limited to only two values corresponding to envelope detection based and OFDM based WUR, respectively.

The UE power consumption scaling discussed in TR38.840 is limited to 5G NR capabilities. For example, existing scaling methodology in TR38.840 supports a maximum BWP bandwidth of 100 MHz which is applicable only for FR1, a maximum number of 4 CCs/2 CCs in DL/UL, and a maximum of 4Rx/2Tx chains.  Therefore these models should be extended to capture the expected changes in 6G such as support of higher carrier bandwidth [e.g., 200MHz for around 7GHz carrier frequencies], larger number of CCs (DL/UL), larger number of antenna elements per Tx/Rx chain, number of Tx/Rx chains, and impact of single-cell with multi-carrier design and/or sparser sync raster on measurements/cell search. Further, the impact of the support of additional frequency ranges [e.g., ~7:24GHz] in 6G should be captured in the 6G UE power consumption model.

Proposal 2: The Rel-16 UE power consumption model and framework with Rel-17 adaptation for RedCap and Rel-18 adaptation for LP-WUR can be adopted as a starting point in 6G while accounting for changes in, e.g., carrier frequency ranges, carrier bandwidths supported, carrier/sync raster  design, number of CCs supported, and antenna architectures when reference configurations and scaling are considered.

## 6G BS Models and Reference Configuration

In RAN1 # 122 companies agreed that RAN1 should study how to reuse and update reference configurations in TR 38.864 for the 6G BS and study how/whether to reuse or update the power model in TR 38.864 for evaluating power consumption of the 6G BSs. The initial estimates of the 6GR BS power consumption model may be similar with those considered in TR 38.864, Tables 5.1-2, 5.1-3, 5.1-4, 5.1-5 and the corresponding analytical formulas for UL and DL.

The reference configuration for single CC case is captured in Table 5.1-1 of the TR38.864, see Annex 8.2. The Table 5 should be extended to cover the larger BW channels (up to 400 MHz), the new 6G frequency available in the 7GHz-24GHz range, and the number of supported RUs.  We note that Table 5.1-1 considers a single CC case, this model may be scaled up for cells that support multiple carries or for the carrier aggregation scenarios.

Proposal 3: Extend the 5G reference configuration for BS power consumption model for larger bandwidth channels, new 6GR available frequency bands and new antenna architecture.

# Time-Domain Energy Efficiency Techniques

To achieve a higher energy efficiency for wireless communication, the nodes involved (UE and gNB) should minimize transmissions and/or RF link monitoring as much as possible, while still satisfying a set of basic quality requirements. This translates into minimizing the active time and/or transmit or receive power of the transceivers. The time domain techniques enable component-level shutdown during inactive periods, with no transmission and/or reception by the UE and/or BS. In this section, we discuss techniques related to synchronization and system information transmission/monitoring, duty cycled operation at BS/UE, and low power signaling procedures.

## Synchronization and System Information

Periodic transmissions may be generally needed for essential functionalities such as synchronization/reference signal and system information (e.g., SSB) delivery for initial access, RRM measurement, and/or beam/CSI acquisition. However, periodic transmissions lead to persistent energy consumption and can reduce energy efficiency, and hence, it is desirable to minimize periodic transmission activities while still fulfilling the essential functionalities.

Network Energy Saving (NES) solutions considered in 5G NR attempted to address the inefficiencies associated with those periodic transmissions. For instance, reduced periodic transmission of common signaling and control optimization techniques or replacing the periodic transmissions with on-demand transmissions was defined for SSB and SIB1 in Release 19.

In particular, SSB transmission time adaptation allowed adaptation of SSB burst periodicity based on the traffic load, but it was restricted to NCD-SSB. SSB-less SCell was extended from the intra-band contiguous FR1 co-located scenario (supported in Release-15) to inter-band FR1 co-located scenario. The scenarios and working conditions for inter-band SSB-less SCell, and how to measure and get essential information for a SCell are all specified.

On-demand SSB was introduced for SCell operation for UE in connected mode with CA, for both intra-/inter-band CA where gNB using MAC-CE indicates the start of on-demand SSB (which may be temporary). The on-demand SSB solution applies to two distinct cases, Case #1(no always-on periodic SSB in SCell) and Case #2 (there is always-on SSB transmission, with large periods).

On-demand SIB1 was defined for IDLE and INACTIVE UEs, where (Case #2) UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell.

Observation 3: On-demand SSB for SCell operation and on-demand SIB1 from an NES cell using UL WUS configuration acquired from an assisting cell (Cell A) were introduced in 5G NR for network energy saving.

The above NES features (i.e., on-demand SSB and on-demand SIB1, and SSB transmission time adaptation) were introduced in 5G at a later stage (i.e., Rel-19) and therefore constraints were added to avoid any impact on legacy users while also minimizing the specifications impact of supporting such features due to lack of time. One example of this is the constraint to avoid the impact of time-adaptation of SSB periodicity on initial access latency for legacy UEs led to avoidance of supporting time-adaptation for CD-SSB and subsequently limiting the NES benefit from this feature.Another example is the assumption that the NES cell was always under the coverage of an assisting cell (Cell A) was considered for the on-demand SIB1 feature, which may not always be satisfied in real deployments. Another assumption for the on-demand SIB1 feature was the availability of an always-on SSB in the NES cell. The later assumption limited the potential to combine the on-demand SIB1 feature with other features such as the on-demand SSB, which itself was limited to SCell operation for UE in connected mode with CA.

The work in 5G NR on NES can then be extended in 6G by the general replacement of the frequent periodic SSB/SIB1 transmissions (e.g., 20 ms periodicity) with infrequent periodic transmissions (e.g., 160 ms periodicity) for any SSB type (i.e., CD-SSB and NCD-SSB) to improve network energy saving by allowing BSs to enter sleep state. The infrequent periodic transmissions may be augmented by on-demand synchronization/reference signals and system information (e.g., SSB and SIB1) transmissions in any cell (i.e., not only the SCell or not under the coverage of Cell A) where the transmissions may be triggered at the network by traffic arrivals, UL WUS, or cell activation procedures. The support of on-demand synchronization/ reference signals and system information in any standalone cell and for UEs in any RRC state can minimize the impact of the infrequent periodic transmission of those signals/information on access latency.

Observation 4: Replacing frequent SSB/SIB1 transmissions with infrequent periodic transmissions for any SSB type (CD-SSB and NCD-SSB) can improve network energy saving.

Observation 5: Support of on-demand synchronization (SSB) and system information (SIB1) in any cell type (standalone cell or SCell) and for UEs in any RRC state can minimize the impact of the infrequent periodic SSB/SIB1 transmission on UE access latency.

For the infrequent periodic and/or on-demand transmissions, the transmissions may be repeated, or the synchronization/reference signals may have high densities in time and/or frequency domains (or low code rate, e.g., for system information) to ensure satisfactory performance which can also minimize the initial access latency. Overall, since the infrequent periodic transmissions plus on-demand transmissions may be less often than the (original) frequent periodic transmissions, energy saving can still be achieved.

Proposal 4: Study the support of infrequent periodic transmission of synchronization and system information (e.g., SSB and SIB1) for network energy saving. Further, study combining with on-demand SSB/SIB1 (in any cell type and for UEs in any RRC state) for minimized impact on UE access latency.

The above proposal can be seen as a “state of the art” for late-stage NR network energy savings, and as such is a very good basis for study for 6GR particularly without the constraints of legacy UEs that do not support the necessary features. However, we also have the paradigm in late stage NR of introducing dedicated “low power” radios geared towards efficient sequence based operations, such as with LP-WUR. Under this paradigm, we can reconsider the alternative of an LTE-like approach where frequent transmissions with high per-transmission overhead are replaced with a combination of frequent periodic lightweight transmissions (e.g., synchronization signals only such as PSS/SSS) and infrequent periodic transmissions of MIB (e.g., PBCH), i.e., separation of synchronization and PBCH transmissions. The lightweight transmissions may be supported (up to implementation) by a low power transmitter ensuring that the BS can still benefit from energy saving by allowing the higher power radio to remain in a sleep state for longer periods.

A middle ground is also possible, where the frequent transmissions with high per-transmission overhead may be replaced with a combination of frequent periodic lightweight/sequence-based transmissions (e.g., synchronization signals such as PSS/SSS and minimum system information (mMIB)) and infrequent periodic/on-demand transmissions (e.g., remaining master system information (rMIB)), i.e., separation of synchronization and PBCH transmissions with further split of the PBCH transmissions (or redefinition/split of the master system information carried in PBCH). For example, the mMIB could be considered the most critical or important information that might be in a MIB, an UL WUS configurarion as described in 4.3, or in its simplest just a single bit indicating that some part of the rMIB information has not changed for some period of time. As described in [4], the design of the 6GR SSB functionality may be optimized for one bandwidth, but still functional for a smaller one, like in 5G NR with 5MHz and 3MHz bandwidths. Due to potential sensitivity towards puncturing, this may mean that the lightweight/sequence-based transmissions (e.g., PSS/SSS and mMIB) are constrained to a smaller set of PRBs or resources than the remaining information which can tolerate some level of puncturing (e.g., rMIB). As described above, the periodic lightweight transmissions (e.g., synchronization and minimum/minimal system information) may also be supported by a low power transmitter ensuring that the BS can still benefit from energy saving by allowing the higher power radio to remain in a sleep state for longer periods.

Observation 6: Support of frequent periodic sequence-based synchronization plus minimum/minimal system information and infreqeuent periodic or on-demand remaining master system information can improve network energy saving.

Proposal 5: Study the support of frequent periodic sequence-based synchronization plus minimum/minimal system information and infreqeuent periodic or on-demand remaining master system information for network energy saving with minimal impact on UE initial access latency.

The two options for synchronization signals and system information transmission discussed above, and their characteristics can be summarized in the table below.

|  | Network Energy Efficiency | UE Access Latency |
| --- | --- | --- |
| Option 1: Infrequent always-on SSB + On-demand SSB/SIB1 | Provides more energy saving than the frequent always-on SSB transmission in 5G NR | Can result in an increase in access latency compared to 5G NR. |
| Option 2: Frequent sequence-based synchronization & mMIB + On-demand SSB/SIB1 | Provides more energy saving than the frequent always-on SSB transmission in 5G NR, but less than Option 1. | Can maintain similar access latency to 5G NR. |

## Cell DTX/DRX and UE DRX

In addition to the periodic transmissions of synchronization/system information at network, as discussed in Section 4.1, periodic/continuous monitoring, at either the network or the UE, may also be generally needed for essential functionalities such as synchronization/SSB acquisition at the UE, paging reception at the UE, receiving scheduling PDCCH at the UE, PRACH monitoring at the network, scheduling request monitoring at the network, etc. The periodic/continuous monitoring leads to continuous energy consumption which can reduce energy efficiency, especially if the monitoring requires the receiver to perform complicated operations such as blind detection. Additionally, periodic measurements, e.g., in support of mobility, can have negative impact on energy efficiency. Hence, it is desirable to minimize/reduce/offload periodic/continuous monitoring/measurement activities while still fulfilling the essential functionalities.

Duty-cycled operations in the form of Discontinuous Reception (DRX) and extended Discontinuous Reception (eDRX) represent simple and crucial time domain techniques that have always been considered for power consumption reduction in RRC_IDLE and RRC_INACTIVE states through the reduction of the number of Paging Occasions (POs) monitored by the UE. Similar UE power saving technique was also considered for RRC_CONNECTED state in the form of connected mode DRX (C-DRX).

Observation 7: Duty-cycled operations (IDRX, eDRX, and cDRX) can continue to offer simple and reliable solutions for UE power saving in 6GR.

Additionally, Cell Discontinuous Transmission and Discontinuous Reception (Cell DTX/DRX), which involves a technique where the gNB can enter a sleep state during periods of inactivity, represent another simple and crucial time domain technique that was considered for network energy saving (NES) in NR. However, the energy saving was impacted by the limited ability of a BS to enter sleep modes due to the need to be periodically transmit specific signals, e.g., Synchronization Signal Block (SSB), System Information Block (SIB)-1, and paging DCIs/messages, regardless of the UEs presence in the cell, particularly for standalone cells.

Observation 8: Duty-cycled operations (cell DTX/DRX) can continue to offer simple and reliable solutions for network energy saving in 6GR.

The coordination/alignment between the UE duty-cycled operations (i.e., IDRX, eDRX, and/or cDRX) and the cell duty-cycled operations (i.e., cell DTX/DRX) can improve the network energy saving without sacrificing the UEs performance (e.g., paging and/or access latency) and their power saving. The importance of joint/aligned Cell DTX/DRX and UE DRX was highlighted in RAN1#122 in the following FL proposal [10]:

- FL Proposal 4.2-1: Study joint Cell DTX/DRX and UE DTX/DRX at least including, - Enhancements of common signals and channels transmissions, e.g., PRACH, RAR, PUSCH, PDCCH, WUS, … - Impact on UE, e.g., latency and synchronization, - Applicable RRC states

To further highlight the importance of a joint or aligned design of cell DTX/DRX and UE duty cycled operations, we note that in 5G NR, every other frame can be a paging frame (PF) which can limit the ability of a BS to enter a sleep state and limit the benefit of the Cell DTX/DRX feature. To address such a limitation, multiple of the consecutive legacy (i.e., 5G NR) PFs can be clustered/combined in time domain, e.g., a single frame.

Observation 9: For an aligned cell DTX/DRX and UE DRX, clustering/grouping of legacy PFs in time domain may be needed to allow the BS to enter sleep state and benefit from Cell DTX/DRX.

Proposal 6: Study a joint/aligned design of cell DTX/DRX and UE duty cycled operations (i.e., IDRX, eDRX, and/or cDRX) for improved network energy saving without sacrificing UEs performance (e.g., paging/access latency) and power saving.

In 5G NR, cell DTX/DRX operation is only supported for single TRP scenario and it does not impact Random Access procedure, SSB transmission, paging, and system information broadcasting. Thus, the transmission of SSB and random access keep the same periodicity irrespective of DTX/DRX active or non-active periods, which lead to additional energy usage even during the non-active periods. A more efficient approach is to lower the periodicity of SSB transmission and system information broadcast during the non-active cell DTX period and lower the periodicity of random access procedure during non-active cell DRX periods.

Proposal 7: Consider increased energy efficiency through compatibility between different energy savings solutions. For instance, DRX/DTX type solutions must operate simultaneously with time adaptive control solutions (adaptive SSB, PRACH).

## Low Power/Complexity Signaling

To further mitigate the power consumption impact due to the need for periodic/continuous monitoring in support of functionalities such as paging/PDCCH reception at the UE, as discussed in Section 4.2, wake-up signaling was also supported in conjunction with duty cycled operation.

For example, power consumption reduction was achieved through Paging Early Indication (PEI) in Rel-17 NR for UE in RRC_IDLE and RRC_INACTIVE states, which was still subject to the duty-cycled operations. Similar power saving technique was also considered for RRC_CONNECTED state in the form of connected mode DRX (C-DRX) and Rel-16 DCI with CRC scrambled by PS-RNTI (DCP). In this case when C-DRX is configured, the UE does not have to continuously monitor PDCCH. The UE may further be indicated, when configured accordingly, whether it is required to monitor or not the PDCCH during the next occurrence of the on-duration by a DCP monitored on the active BWP (TS 38.300, Clause 11.1).

Both Rel-17 PEI and Rel-16 DCP were received by UEs via DCIs in the PDCCH. The DRX, eDRX, and C-DRX could provide higher power saving gain by increasing the duty cycle duration at the expense of higher latency to be expected by the UE. On top of DRX/eDRX or C-DRX, Rel-17 PEI and Rel-16 DCP could provide additional power saving gain without an impact on latency, but the gain is limited by the power consumption required to decode a DCI in the PDCCH.

In Rel-19, a Low Power Wake-Up Signal (LP-WUS) was introduced, which can be received by a Low Power Wake-Up Radio/Receiver (LP-WUR) at considerably lower power consumption than Rel-17 PEI and Rel-16 DCP designs. The evaluations of LP-WUS in its Rel-18 study showed that a significant power saving gain was achievable. The Rel-19 LP-WUS could therefore enable new trade-off regions of latency versus power using dedicated LP-WURs with simple architectures [8]. The Rel-19 LP-WUS design addressed two classes of LP-WUR architectures: (i) an envelope detection (ED)-based LP-WUR employing an On-Off Keying (OOK) waveform, and (ii) an OFDM-based LP-WUR utilizing OFDM sequences overlaid on the OOK waveform.

Considering the simplicity of duty-cycled operations, as discussed in Section 4.2, and the significant power-saving benefits of LP-WUS, existing DRX/eDRX, C-DRX, and LP-WUS techniques can be adopted as baseline mechanisms for 6G. It is noted that LP-WUS may lower the latency imposed by duty-cycled operation by considering the configuration of dynamic POs [8]. Furthermore, due to functional overlap with Rel-17 PEI and Rel-16 DCP, an enhanced LP-WUS mechanism with scalable design could serve as a unified solution common for all device types by replacing the Rel-17 PEI and Rel-16 DCP. This enhancement may include support for standalone OFDM-based LP-WUS (i.e., not reliant on the OOK waveform) and integration of dormancy indication functionality as defined in Rel-16 DCP.

Observation 10: LP-WUS can offer significant power saving benefits and lower latency associated with duty-cycled operations.

Observation 11: There is a functional overlap of Rel-19 LP-WUS with Rel-17 PEI and Rel-16 DCP. An enhanced LP-WUS mechanism with scalable design can serve as a unified energy efficiency solution, i.e., taking over the Rel-17 PEI and Rel-16 DCP functionalities, common for all device types.

Proposal 8: Study the energy efficiency provided by the adoption from day one of duty-cycled based operations (iDRX, eDRX, cDRX) and OOK/OFDM based LP-WUS with at least PEI/DCP functionality replacement as baseline UE power saving mechanisms in 6G.

The Rel-19 LP-WUS and Low Power Synchronization Signal (LP-SS) designs employing the OOK waveform are expected to achieve greater power saving gain compared to the OFDM-based LP-WUS design, owing to the simpler ED-based LP-WUR architecture [8]. However, the Rel-19 OOK-based LP-WUS/LP-SS designs exhibit considerably lower resource utilization efficiency. Therefore, enhancements aimed at improving overall resource utilization, e.g., through integration with other signals or channels with efficient resource utilization, may be necessary.

Observation 12: Rel-19 OOK-based LP-SS/LP-WUS designs exhibit low resource utilization efficiency.

Proposal 9: Study enhancement of OOK based LP-SS/LP-WUS resource utilization, e.g., through integration with other signals or channels of efficient resource utilization, in 6G.

As discussed in Section 4.2, multiple of consecutive legacy (i.e., 5G NR) PFs can be clustered/combined in time domain, e.g., a single frame, for a joint or aligned design of cell DTX/DRX and UE duty cycled operations (i.e., IDRX, eDRX, and/or cDRX). The paging probability per legacy PF, and corresponding UE power consumption, can be maintained by combining with the LP-WUS feature where the LP-WUS may further indicate/provide grouping information instead (or in addition to) subgrouping information. Further, 6GR UEs in RRC IDLE/INACTIVE mode may be camped in different BWPs in the same cell. In such a case, the impact of legacy PFs grouping on paging message capacity can be limited, e.g., multiple paging messages may be multiplexed in frequency domain within a single Bandwidth Part (BWP) or across multiple BWPs. The LP-WUS and/or the paging DCI may then indicate BWP switching for the reception of the paging message, which can be supported by the improvement in BWP switching latency as discussed in Section 5.1.

Observation 13: LP-WUS support of grouping and/or subgrouping information indication and combining with PFs clustering/grouping can control the paging probability and corresponding UE power consumption.

Observation 14: When UEs in RRC IDLE/INACTIVE mode are camped on different BWPs, paging messages multiplexing in frequency domain across multiple BWPs can maintain the paging message capacity with PFs clustering/grouping.

Proposal 10: Study PFs clustering/grouping combined with grouping information indication by LP-WUS as an approach for aligned Cell DTX/DRX and UE DRX.

To improve network energy efficiency, one option (Option2) of frequent periodic lightweight synchronization plus mMIB and on-demand request of rMIB, MIB, and/or SIB1 is proposed for study in Section 4.1. The request for on-demand signaling can be based on an UL WUS where the configuration for the transmission of the UL WUS may be provided as part of the mMIB. Though the UL WUS information may not need to be so dynamically adjusted, particularly in IDLE/INACTIVE mode mobility it is beneficial to be able to obtain the UL WUS configuration efficiently. To allow the synchronization plus mMIB to be transmitted/received by a simple, e.g., low power, transmitter/receiver, the design of mMIB signaling should be lightweight, e.g., sequence-based with up to 16 codewords carrying up to 4 information bits, e.g., indicating one of predefined set of configurations. Such a design can then enable the on-demand request of MIB/SIB1 in standalone cells as well as the use of a low-power transmitter at the BS in, e.g., no/low load scenarios or outside Cell DTX.

Observation 15: Combining synchronization signals (PSS/SSS) with sequence-based design for indicating UL WUS configuration can enable on-demand request of MIB/SIB1 in standalone cells and use of low-power transmitter at BS in no/low load scenarios or outside Cell DTX.

The configuration of UL WUS indicated in mMIB can allow the BS to receive the UL WUS using a simple, e.g., low power, receiver/radio over a limited number of occasions, e.g., one or two, and using a limited number of sequences. Similar to Rel-19 LP-SS/LP-WUS, the UL WUS may be based on an OFDM sequence and/or a binary OOK modulated sequence. The use of limited number of OFDM sequences and/or binary OOK modulated sequences can enable the BS to use a low power receiver/radio in, e.g., no/low load scenarios or outside Cell DRX. The use of binary OOK modulated sequences may be limited to the no/low load scenarios or outside a Cell DRX to accommodate the resource utilization that may be required for the target coverage.

Observation 16: A simple UL WUS design based on limited number of OFDM and/or binary OOK modulated sequences can allow the BS to use a low power radio in no/low load scenarios or outside Cell DRX.

Once the BS receives the UL WUS, it may replace the transmission of the periodic lightweight synchronization plus mMIB with synchronization plus rMIB/MIB. A UE may differentiate between the synchronization plus mMIB or the synchronization plus rMIB/MIB structure using, e.g., presence/absence of the sequence-based mMIB signal, i.e., presence/absence of the UL WUS configuration indication. The UE may then use the system information in rMIB to receive SIB1 and/or other system information.

Proposal 11: Study network energy efficiency by adoption of on-demand SIB1 request in standalone cells using sequence-based indication of UL-WUS configuration and UL WUS design based on limited number of OFDM and/or binary OOK modulated sequences.

## RRM Measurements

Relaxed RRM measurements for UEs based on low mobility and not-at-cell-edge criteria were introduced in NR Rel-16 with considerable power saving gain shown in TR38.840 whereas relaxed RRM measurements for (e)RedCap UEs based on stationarity and not-at-cell-edge criteria were introduced in NR Rel-17 and Rel-18. Additionally, further RRM measurement relaxation of intra-/inter-frequency neighboring cells as well as serving cell measurement relaxation and/or offloading to LP-WUR were highlighted as critical elements to fully benefit from the Rel-19 LP-WUS and minimize the main radio (MR)’s power consumption by keeping it in a sleep state as much as possible. Therefore, relaxed RRM measurements and offloading of at least serving cell measurements to a LP-WUR should, as critical elements for the significant UE power saving benefits of LP-WUS, also be adopted as baseline mechanisms in 6G.

Observation 17: RRM measurement relaxation for neighboring cells and relaxation/offloading to LP-WUR for serving cell based on stationarity, low mobility, and/or not-at-cell-edge criteria are critical elements for the significant UE power saving benefits in 6G.

Further, by enhancing the synchronization signal, e.g., LP-SS, and/or LP-WUS designs in 6G to improve coverage and/or capacity, serving cell measurements can be fully offloaded to LP-WUR and neighboring cells measurements may also be supported by LP-WUR.

Observation 18: Considering the enhancement of synchronization signals, e.g., LP-SS, and LP-WUS designs for better coverage and/or capacity, RRM measurements of serving and neighboring cells can be offloaded to LP-WUR.

Proposal 12: Study the adoption from day one of the relaxation of RRM measurements and the offloading of serving and neighboring cells RRM measurements to LP-WUR in 6G.

# Frequency-Domain Energy Efficiency Techniques

The frequency domain techniques explore the adaptation of frequency resources in general, including the shutdown of secondary cells/carriers (SCell) in multi-carrier operation and adjustments in bandwidth segments based on BWP adaptation as well as limiting the maximum bandwidth (RedCap). Such techniques may include a sparser Sync Raster and simpler carrier designs.

## BWPs and Bandwidth Adaptation

The bandwidth part (BWP) concept was introduced in NR Rel-15 and is defined as a subset of contiguous common resource blocks within a given carrier for a specific numerology. The primary purpose of BWP is the BWP adaptation (BA), which is justified by the fact that to transmit or receive the UE does not need a bandwidth as large as the channel bandwidth, and the used bandwidth can be adjusted. The power savings scales down with the bandwidth  due to both base band processing and RF power consumption. In 5G NR, the UE channel bandwidth must be placed entirely within the Base Station (BS) channel bandwidth. The BS can transmit to and/or receive from UE BWPs that are smaller than or equal to the BS transmission bandwidth configuration.

Thus, UE power saving was one of the motivations for introducing the BWP concept in NR by supporting dynamic UE bandwidth adaptation and subsequently helping reduce its power consumption. For example, the UE can switch from large bandwidth BWP for high throughput/large data transfer to low bandwidth BWP for power saving. Additionally, BWP was introduced to support flexible bandwidth (and numerology) operation by decoupling the channel bandwidth of a carrier from the UE channel bandwidth, e.g., based on capability and service requirements. This feature would then allow gNB a more efficient allocation of resources as a function of traffic type and load.

The 5G NR BWP design is defined to be very flexible where a UE can be  configured with up to four BWPs for DL and four BWPs for UL with different numerologies. However, only one BWP can be active at any given time for both DL receptions and UL transmissions. There are three mechanisms for BWP switching in 5G NR: RRC-based, DCI-based, and timer-based BWP switching. The delay for RRC-based BWP switch is the sum of processing delay for RRC procedure and the delay for UE to perform BWP switch. The processing delay for RRC procedure is in the range of 5-80 ms depending on the connection control procedure [TS 38.331], whereas the delay for the UE to perform BWP switch is around 6 milliseconds [TS 38.133, Clause 8.6.3], [11].

In DCI-based BWP switching, the network can schedule the UE to switch the active DL/UL BWP from one configured BWP to another using BWP indicator in DCI format 1_1/0_1. The BWP switch delay requirements in 5G NR are listed in Table 2 for both DCI- and timer-based BWP switch [TS 38.133, Clause 8.6.2 Table 8.6.2-1]. The switch delay denoted by TBWPswitchDelay for DCI-based BWP switch was defined as the slot offset between the DL slot in which the UE received switch request and the first slot in which the UE shall be able to receive PDSCH or transmit PUSCH on the new BWP. There were two levels of BWP switch delay requirements, i.e., for type 1 and type 2 UE, as given in Table 2.

Table 2: DCI- and timer-based BWP switch delay requirements.

| SCS (kHz) | NR Slot length (ms) | BWP switch delay requirement TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- |
|  |  | Type 1 | Type 2 |
| 15 | 1 | 1 | 3 |
| 30 | 0.5 | 2 | 5 |
| 60 | 0.25 | 3 | 9 |
| 120 | 0.125 | 6 | 18 |

In timer-based BWP switch, the network may configure a UE with a BWP inactivity timer and a default DL BWP on a serving cell. The default DL BWP is one of the DL BWPs configured to the UE and becomes the active DL BWP upon expiry of the inactivity timer. If no default DL BWP is configured, the default DL BWP is the initial DL BWP. The granularity of the BWP inactivity timer is 1 ms (i.e., 1 subframe) for FR1 and 0.5 ms for FR2, and the values have the range of 2 – 2560 ms. Timer-based BWP switch shares the same BWP switch delay requirements as DCI-based BWP switch, as shown in Table 2.

Upon an RRC (re-)configuration (which could include a BWP switch), the UE typically flushes soft buffers for all DL HARQ processes (except those for MBS broadcast) and considers the next received transmission for a TB as the very first transmission. This implies that HARQ context is generally reset or re-initialized in such scenarios. When a BWP becomes inactive, any configured uplink grants of Type 1 are suspended, and Type 2 grants, as well as configured downlink assignments, are cleared. This effectively deactivates the HARQ processes associated with these grants/assignments in the inactive BWP.

The calculation of HARQ Process IDs for configured grants is tied to the CURRENT_symbol and periodicity, which are BWP-specific. A BWP switch, especially one that changes numerology, could alter the timing context and thus influence the assignment or continuity of HARQ Process IDs for configured grants.

Observation 19: A BWP switch, especially one that changes numerology, could alter the timing context and thus influence the assignment or continuity of HARQ Process IDs for configured grants.

Proposal 13: Study UE operating bandwidth solutions for 6G to improve UE EE considering minimization of data interruption and maximization of reliability when switching between narrowband and wideband BWPs

The bwp-InactivityTimer is used to switch the active BWP to a default BWP configured by the network. This is done to save UE power, especially during periods of low activity, and it is configured per serving cell and operates idependently of DRX inactivity timer. When the UE successfully decodes a PDCCH (Physical Downlink Control Channel), indicating active data traffic, the timer is restarted. A mistmatch between the inactivity timer value and traffic pattern, cell load and channel conditions may lead to an increased number of BWP switching. Frequent BWP switching, for instance in the bursty traffic or low load conditions, may lead to an increased  number of BWP switches, which leads to increased latency and decreased throughput.

This was illustrated in [12] using system level evaluation of timer-based BWP switching capturing the impact of BWP inactivity timer and BWP switch delay on UEs performance (i.e., latency and throughput) and power saving. It was shown that UEs in adverse channel conditions, e.g., at cell edge, or with high traffic load may not experience power saving due to insufficient/lack of switching between dedicated and default BWPs, i.e., bandwidth adaptation, based on inactivity timer switching. Further, BWP switching based on inactivity timer for low load and bursty traffic may provide power saving but may lead to increased scheduling/traffic latency and subsequently lower UE perceived throughput.

Further, three case studies (i.e., Mobile Gaming, Voice over New Radio, and Video Traffic) were considered in [13], and the traffic, resource allocation, or power consumption characterization for the cases were presented, for the evaluation of power saving gain from RRC-based and/or DCI-based BWP switching. The resource allocation of the mobile gaming case is characterized by an allocation of <50 PRBs for up to 87% of the time with an allocation of 4 PRBs accounting for up to 65% of the time. The DCI-based BWP switching for this case showed an up to 50% gain in power saving compared to no BWP switching. For the voice over New Radio (VoNR) case with a quasi-fixed traffic pattern that might also include periods of silence, RRC-based and DCI-based BWP switching showed 40% gain in power saving.

Observation 20: Bandwidth adaptation using BWP switching for low load and bursty traffic can provide power saving gains but may lead to increased scheduling/traffic latency and subsequently lower UE perceived throughput, particularly for inactivity timer based switching.

A drawback of a highly flexible design (such as the BWP design) is that it requires an increased validity and testing complexity, to cover all the possible combinations. In addition, the inherent overhead of this process can add complexity to the system.

Observation 21: High flexibility of BWP design adds to inherent complexity of the 5G NR, and increases validity and testing complexity of the feature.

The above observations indicate that RAN1 should revisit the BWP concept and simplify the 5G NR BWP design while still maintaining its main purpose of saving UE energy.

For example, improving BWP switching latency can reduce or mitigate the impact of BWP switching on UE’s performance (e.g., scheduling/traffic latency and throughput) while maintaining the power saving gains. Simplifying or reducing the BWP parameters, e.g., limiting BWP update mainly to bandwidth and frequency location, can help improve the BWP switching latency.

Allowing cross BWP scheduling during BWP switching, i.e., UE receiving scheduling DCI in a BWP and the scheduled PDSCH in another BWP after switching to that BWP, may also help mitigate the BWP switching impacts on UE’s throughput performance by reducing control messages overhead.

Proposal 14: Study improving BWP switching latency by simplifying BWP design parameters, e.g., limiting BWP update to mainly bandwidth and frequency location, to mitigate BWP switching impact on latency/throughput while maintaining power saving gains.

## Multi-Carrier Enhancements and Carrier Aggregation

One of the detailed 6GR study goals [RP-252912] is to support:

- Improved spectrum utilization and operations taking into account diverse spectrum allocations.

which corresponds to the PHY layer structure study of:

- 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4]

The concept of Component Carrier (CC) is introduced in 5G in the context of Carrier Aggregation (CA). A component carrier or shortly a carrier represents a block of contiguous frequency, which can be aggregated with other carries to provide a UE with additional bandwidth. In 5G there is a one-to-one correspondence between a carrier and a cell. When multiple carriers are aggregated via CA, one of the carriers is the principal component carrier (PCC) and corresponds to a Primary Cell (PCell), while all other carries are secondary component carriers (SCC), where each corresponds to a secondary cell (SCell). A PCell serves as the main link for control and data communication between UE and network, while the SCells provide additional bandwidth. While CA defined in 5G offers high flexibility, it also poses some challenges.

Supporting different numerologies and slot types across carriers increases complexity of scheduling and HARQ ACK, uplink synchronization between cells with different bands and channel conditions together with uplink conditions for increased PAPR are additional challenges.

Energy efficiency at both UE and NW during CA was, from the beginning, one of the main concerns in RAN1. Collocation of PCell and SCell allows SSB less SCell operation, where the necessary synchronization and system information is provided by the PCell. Thus, the energy consumption at an SCell is reduced. For the scenarios when SSB of the PCell cannot be reused in the SCell, an on-demand SSB operation for SCell was defined in Rel 19. When supported, the feature allows transmission of on-demand SSB only during the activation of SCell, which reduces the network energy consumption.

In addition, when using cross-carrier scheduling feature, the PCell can schedule all data transmissions for SCell, which allows SCell to stay longer durations in a lower power mode, and UE to monitor only PScell control signals, thus saving additional energy.

Another technique used to increase energy efficiency in CA is the usage of dormant BWP concept. When BWP in an SCell is switched to dormant state, the UE stops monitoring PDCCH and performing link quality on that SCell, which provides UE energy savings.

In our opinion, all these energy efficiency features related to CA should be considered and revisited in 6GR. In addition, some other energy features, which were not considered for the CA scenarios should be considered too. For instance, in Rel-19 there was not support for on-demand SSB when SSB is a CD-SSB either in PCell or in SCell. Another case not addressed in Rel-19 on-demand SSB was Scenario #3 (“After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed”).

Proposal 15: Extend the solutions of energy saving for CA defined in 5G NR together with non-CA energy saving solutions for 6GR carrier aggregation.

For instance, using on-demand SSB and time adaptation together or using on-demand SSB and on-demand SIB1 together may lead to better energy efficiency, even in 5G they were designed for different scenarios.

As noted above, in 5G there is one-to-one correspondence between a cell and its component carrier. In other words, a cell does not use multiple carriers. For this reason, the CA concept defined PCells and SCells.  One of the issues with actual definition of CA is that with the loss of connectivity to PCell, the UE loses the connectivity to SCells as well, which may lead to service interruptions and additional energy consumption.

A possible way to solve the issues related to CA is to allow a cell to support multiple carriers. When a cell supports multiple carriers, there is no need to support multiple CD-SSB or SIBx periodic trasnmissions across multiple carries, thus some of the carriers can operate in SSB/SIBx  less mode or if necessary in on-demand SSB/SIBx mode with further energy savings.

Proposal 16: Study multi-carrier enhancements in 6GR in the context of single cell multiple carriers and one carrier one cell.

# Spatial/Power-Domain Energy Efficiency Techniques

Spatial-domain and power-domain energy efficiency techniques can be critical to 6GR designs. At the high level, the following should be considered, though some of them may be more suitable to be discussed in corresponding topics:

- Antenna architecture based techniques study which antenna architectures can be intrinsically more energy efficient, and what solutions are required for such energy-efficient antenna architectures to also achieve sufficiently high throughput performance and user experience. For one example, it is generally understood that hybrid antenna architectures can be more energy efficient than fully-digital antenna architectures, and hence hybrid antenna architectures can be studied and solutions to support hybrid antenna architectures can be proposed, such as CSI acquisition for hybrid antennas.

- Energy-efficient beam management (BM) techniques aim to reduce the high energy consumption associated with the complicated and prolonged beam acquisition / beam maintenance / beam failure recovery procedures.

- The spatial domain techniques involve dynamically activating/deactivating spatial elements, i.e., antenna ports/elements and/or TXRUs. MIMO CSI acquisition overhead reduction, DMRS overhead reduction, etc., may also be studied, especially if the port number is large.

- The power domain techniques consider more dynamic adaptation of TRP DL transmit power, including more dynamic on/off of TRPs.

These techniques may have overlaps, so the above categorization is not meant to be restrictive. In any case, duplicated discussions/solutions should be avoided.

## Antenna Architectures

MIMO antennas have been adopted by almost all 5G TRPs, and it is expected that the TRP antenna sizes will further increase in 6G. For example, antennas with a few hundreds or even a couple of thousands of antenna elements have been suggested. To support such large MIMO antennas with energy efficiency requires proper design/selection of antenna architectures.

- Antenna architecture with adaptable spatial elements: For large antennas, such as those for Massive MIMO and Extreme MIMO with up to a few thousands of antenna elements / antenna ports, for energy-efficient operation, it is important to allow the semi-static and/or dynamic adaptation of the number of active spatial elements based on service requirements (e.g., QoS) and energy-efficiency requirements.

For example, operating a base station with up to 1024 antenna elements / TXRUs / ports or even more all the time on all carriers leads to high energy consumption, overhead, and complexity that may be prohibitive in a practical network. It may be useful to have the capability of turning on/off dynamically some of the spatial elements, e.g., antenna elements, subarrays, (sub-)panels, and/or transceiver chains to save energy. Without such antenna architectures, it is difficult to support spatial-domain energy-efficient techniques.

- Hybrid Antenna Architectures for large antennas: Use hybrid antenna architectures for Massive MIMO and Extreme MIMO to reduce the number of RF chains and ADC/DAC converters, which are generally power hungry, thus lowering energy consumption.

For instance, in FR3 (i.e., upper midband of 7 GHz to 24 GHz), though fully-digital antenna architecture can have obvious advantages, high energy consumption due to a large number of TRx chains coupled with hundreds of megahertz of bandwidth and the associated baseband processing should be taken into consideration. For example, the total ADC/DAC power consumption scales roughly linearly with the product of the number of TRx chains and the bandwidth, so if from Massive MIMO on FR1 to Extreme MIMO on upper midband, the number of TRx chains grows by 4x to 8x, and the bandwidth grows by 2x to 8x, the total ADC/DAC power consumption grows by up to 8x to 64x. If, however, the number of TRx chains for upper midband remains the same as that for FR1, i.e. adopting a hybrid antenna architecture, the total ADC/DAC power consumption will be more acceptable in a practical network operating on upper midband.

- Mixed antenna architectures for mmWave FR2 BM: For mmWave FR2, generally the network adopts (mostly) analog antenna architectures. An issue with this is complicated beam (re)acquisition, which requires several rounds of measurements and reporting and hence consumes significant energy to just acquire or maintain the beam. On the other hand, if the network is equipped with an additional fully-digital receiver, one-shot beam (re)acquisition can be feasible, thus reducing the energy consumption for BM, especially for the cases where beam quality varies frequently (such as due to blockage or motion). To further reduce the energy consumption by the additional fully-digital receiver, low-resolution ADC (or even 1-bit ADC) can be utilized. Note that the additional fully-digital receiver is for receiving only and for BM only (to minimize the associated cost, complexity, and energy consumption), and data transmission/reception still rely on analog antennas; for this reason, it may be called a mixed antenna architecture for FR2. The mixed antenna architecture may potentially simplify many procedures in FR2 and significantly reduce the energy consumption.

Proposal 17: Consider the following antenna architectures for energy saving:

- Antenna architecture with adaptable spatial elements

- Hybrid Antenna Architectures for large antennas

- Mixed antenna architectures for mmWave FR2 BM

## MIMO transmission, CSI acquisition, and FR2 BM Enhancements

When supported by proper antenna architectures (when applicable), solutions to improve MIMO energy efficiency can be studied.

- Overhead reduction for MIMO CSI acquisition: CSI-RS overhead reduction has already been discussed and partially supported in 5G NR, and should continue in 6G, especially since the number of CSI-RS antenna ports keeps increasing.

In an example, the CSI-RS transmissions with 256 ports and 40 ms periodicity may be replaced with CSI-RS transmissions with selected 16 ports and 40 ms periodicity plus CSI-RS transmissions with all 256 ports and 160 ms periodicity. Alternatively, the CSI-RS with all 256 ports may be on-demand transmissions (potentially with repetitions and/or higher density) when high-speed data is to be sent. The reduced per-transmission overhead can help reduce energy consumption.

In another example, CSI-RS transmissions on a carrier of 400 MHz bandwidth and with 40 ms periodicity may be replaced with CSI-RS transmissions on 40 MHz bandwidth hopping within the carrier and 40 ms periodicity.

- MIMO transmission and CSI acquisition with adaptable spatial elements: If the MIMO antenna architecture supports semi-static and/or dynamic adaption of the number of active spatial elements, the MIMO transmission and CSI acquisition should allow the on/off of the spatial elements, manifested as the adaptation of MIMO port number, MIMO CSI quality, etc.

For example, a UE may acquire one set of CSI quantities (e.g., CQI/RI/PMI) associated with 8-port CSI-RS and another set of CSI quantities (e.g., CQI/RI/PMI) associated with 128-port CSI-RS. The difference in CSI-RS port numbers is due to the on/off of spatial elements of the network antennas. Then the network and UE can adapt between high-speed transmission with high energy consumption and low-speed transmission with low energy consumption.

- MIMO transmission and CSI acquisition with hybrid antenna architectures: If the MIMO antennas adopt a hybrid antenna architecture for Extreme MIMO at upper midband, how to support high-speed MU MIMO needs to be studied.

For MU MIMO transmission, accurate CSI acquisition is required, which is usually based on SRS in UL and/or Type II codebook scheme in DL. In particular, elementwise CSI per UE may be needed. However, for hybrid antennas, it is unclear whether it is feasible to, or how to acquire elementwise CSI, since several antenna elements are combined into one port and the antenna elements are generally ‘invisible’ for the transmitter/receiver. Nevertheless, hybrid antenna architecture may be important for energy-efficient operations of Extreme MIMO at upper midband, and efficient / effective MIMO transmission and CSI acquisition with hybrid antenna architectures should be studied.

- FR2 BM Enhancements: Energy-efficient FR2 BM procedure should be considered, especially if the above-mentioned mixed antenna architectures (with additional fully-digital 1-bit ADC receiver).

Current FR2 BM heavily relies on rounds of beam sweeping for beam (re)acquisition, beam refinement, etc. This leads to high latency and high energy consumption. If, however, the mixed antenna architectures with additional fully-digital 1-bit ADC receiver are introduced in 6G for FR2, one-shot beam (re)acquisition may be feasible, that is, rounds of beam sweeping may be no longer necessary. Design of signals/signaling/procedures should be studied in 6G.

Proposal 18: Consider the following potential enhancements related to MIMO transmission, CSI acquisition, and FR2 BM:

- Overhead reduction for MIMO CSI acquisition

- MIMO transmission and CSI acquisition with adaptable spatial elements

- MIMO transmission and CSI acquisition with hybrid antenna architectures

- Support of beam (re)acquisition with mixed antenna architectures for mmWave FR2

## Power Adaptation

Power adaptation, such as adaptation of the transmission power level, turning on/off of a TRP (including entering different sleep states, such as micro sleep, and so on), etc., can be an effective way to save energy. This has been studied in 4G and 5G, e.g., Rel-12 Small Cell On/off, Rel-17 ~ Rel-19 Network Energy Saving, etc. We expect this to continue in 6G.

One issue associated with power adaptation is the link/CSI acquisition latency. For example, when a TRP in Off state turns on to serve UEs, it may take a relative long time to establish the connection (synchronization, tracking, etc.) and then acquire CSI. If these procedures remain long, it is unlikely that the network will turn off TRP to save energy, since the overhead at turning on is high. Therefore, fast or early link/CSI acquisition may facilitate procedures with power adaptation. For example, 5G NR Rel-20 MIMO is specifying early CSI acquisition for SCell activation and SCell out-of-dormancy transition, which could be a starting point for 6G early link/CSI acquisition with power adaptation.

Proposal 19: Study early link/CSI acquisition related to power-domain adaptation for 6G energy efficiency improvement.

# Conclusions

In this contribution, we discussed energy efficiency solutions for 6GR and introduced the following set of observations and proposals.

Observations:

Observation 1: The NR UE power consumption model in TR38.840 may be revisited or simplified to reflect improvements in 6G UEs.

Observation 2: For LP-WUR evaluations, the relative power value for LP-WUR power consumption when turned on/off may be limited to only two values corresponding to envelope detection based and OFDM based WUR, respectively.

Observation 3: On-demand SSB for SCell operation and on-demand SIB1 from an NES cell using UL WUS configuration acquired from an assisting cell (Cell A) were introduced in 5G NR for network energy saving.

Observation 4: Replacing frequent SSB/SIB1 transmissions with infrequent periodic transmissions for any SSB type (CD-SSB and NCD-SSB) can improve network energy saving.

Observation 5: Support of on-demand synchronization (SSB) and system information (SIB1) in any cell type (standalone cell or SCell) and for UEs in any RRC state can minimize the impact of the infrequent periodic SSB/SIB1 transmission on UE access latency.

Observation 6: Support of frequent periodic sequence-based synchronization plus minimum/minimal system information and infreqeuent periodic or on-demand remaining master system information can improve network energy saving.

Observation 7: Duty-cycled operations (IDRX, eDRX, and cDRX) can continue to offer simple and reliable solutions for UE power saving in 6GR.

Observation 8: Duty-cycled operations (cell DTX/DRX) can continue to offer simple and reliable solutions for network energy saving in 6GR.

Observation 9: For an aligned cell DTX/DRX and UE DRX, clustering/grouping of legacy PFs in time domain may be needed to allow the BS to enter sleep state and benefit from Cell DTX/DRX.

Observation 10: LP-WUS can offer significant power saving benefits and lower latency associated with duty-cycled operations.

Observation 11: There is a functional overlap of Rel-19 LP-WUS with Rel-17 PEI and Rel-16 DCP. An enhanced LP-WUS mechanism with scalable design can serve as a unified energy efficiency solution, i.e., taking over the Rel-17 PEI and Rel-16 DCP functionalities, common for all device types.

Observation 12: Rel-19 OOK-based LP-SS/LP-WUS designs exhibit low resource utilization efficiency.

Observation 13: LP-WUS support of grouping and/or subgrouping information indication and combining with PFs clustering/grouping can control the paging probability and corresponding UE power consumption.

Observation 14: When UEs in RRC IDLE/INACTIVE mode are camped on different BWPs, paging messages multiplexing in frequency domain across multiple BWPs can maintain the paging message capacity with PFs clustering/grouping.

Observation 15: Combining synchronization signals (PSS/SSS) with sequence-based design for indicating UL WUS configuration can enable on-demand request of MIB/SIB1 in standalone cells and use of low-power transmitter at BS in no/low load scenarios or outside Cell DTX.

Observation 16: A simple UL WUS design based on limited number of OFDM and/or binary OOK modulated sequences can allow the BS to use a low power radio in no/low load scenarios or outside Cell DRX.

Observation 17: RRM measurement relaxation for neighboring cells and relaxation/offloading to LP-WUR for serving cell based on stationarity, low mobility, and/or not-at-cell-edge criteria are critical elements for the significant UE power saving benefits in 6G.

Observation 18: Considering the enhancement of synchronization signals, e.g., LP-SS, and LP-WUS designs for better coverage and/or capacity, RRM measurements of serving and neighboring cells can be offloaded to LP-WUR.

Observation 19: A BWP switch, especially one that changes numerology, could alter the timing context and thus influence the assignment or continuity of HARQ Process IDs for configured grants.

Observation 20: Bandwidth adaptation using BWP switching for low load and bursty traffic can provide power saving gains but may lead to increased scheduling/traffic latency and subsequently lower UE perceived throughput, particularly for inactivity timer based switching.

Observation 21: High flexibility of BWP design adds to inherent complexity of the 5G NR, and increases validity and testing complexity of the feature.

Proposals:

Proposal 1: For 6G energy efficiency consider the following design principles:

- Native Energy Efficiency

- Joint Network and UE Power Saving

- Simplification of the Design

- Balanced Trade-off between Energy Saving and Performance

- Energy Efficiency as Mandatory Features

Proposal 2: The Rel-16 UE power consumption model and framework with Rel-17 adaptation for RedCap and Rel-18 adaptation for LP-WUR can be adopted as a starting point in 6G while accounting for changes in, e.g., carrier frequency ranges, carrier bandwidths supported, carrier/sync raster  design, number of CCs supported, and antenna architectures when reference configurations and scaling are considered.

Proposal 3: Extend the 5G reference configuration for BS power consumption model for larger bandwidth channels, new 6GR available frequency bands and new antenna architecture.

Proposal 4: Study the support of infrequent periodic transmission of synchronization and system information (e.g., SSB and SIB1) for network energy saving. Further, study combining with on-demand SSB/SIB1 (in any cell type and for UEs in any RRC state) for minimized impact on UE access latency.

Proposal 5: Study the support of frequent periodic sequence-based synchronization plus minimum/minimal system information and infreqeuent periodic or on-demand remaining master system information for network energy saving with minimal impact on UE initial access latency.

Proposal 6: Study a joint/aligned design of cell DTX/DRX and UE duty cycled operations (i.e., IDRX, eDRX, and/or cDRX) for improved network energy saving without sacrificing UEs performance (e.g., paging/access latency) and power saving.

Proposal 7: Consider increased energy efficiency through compatibility between different energy savings solutions. For instance, DRX/DTX type solutions must operate simultaneously with time adaptive control solutions (adaptive SSB, PRACH).

Proposal 8: Study the energy efficiency provided by the adoption from day one of duty-cycled based operations (iDRX, eDRX, cDRX) and OOK/OFDM based LP-WUS with at least PEI/DCP functionality replacement as baseline UE power saving mechanisms in 6G.

Proposal 9: Study enhancement of OOK based LP-SS/LP-WUS resource utilization, e.g., through integration with other signals or channels of efficient resource utilization, in 6G.

Proposal 10: Study PFs clustering/grouping combined with grouping information indication by LP-WUS as an approach for aligned Cell DTX/DRX and UE DRX.

Proposal 11: Study network energy efficiency by adoption of on-demand SIB1 request in standalone cells using sequence-based indication of UL-WUS configuration and UL WUS design based on limited number of OFDM and/or binary OOK modulated sequences.

Proposal 12: Study the adoption from day one of the relaxation of RRM measurements and the offloading of serving and neighboring cells RRM measurements to LP-WUR in 6G.

Proposal 13: Study UE operating bandwidth solutions for 6G to improve UE EE considering minimization of data interruption and maximization of reliability when switching between narrowband and wideband BWPs

Proposal 14: Study improving BWP switching latency by simplifying BWP design parameters, e.g., limiting BWP update to mainly bandwidth and frequency location, to mitigate BWP switching impact on latency/throughput while maintaining power saving gains.

Proposal 15: Extend the solutions of energy saving for CA defined in 5G NR together with non-CA energy saving solutions for 6GR carrier aggregation.

Proposal 16: Study multi-carrier enhancements in 6GR in the context of single cell multiple carriers and one carrier one cell.

Proposal 17: Consider the following antenna architectures for energy saving:

- Antenna architecture with adaptable spatial elements

- Hybrid Antenna Architectures for large antennas

- Mixed antenna architectures for mmWave FR2 BM

Proposal 18: Consider the following potential enhancements related to MIMO transmission, CSI acquisition, and FR2 BM:

- Overhead reduction for MIMO CSI acquisition

- MIMO transmission and CSI acquisition with adaptable spatial elements

- MIMO transmission and CSI acquisition with hybrid antenna architectures

- Support of beam (re)acquisition with mixed antenna architectures for mmWave FR2

Proposal 19: Study early link/CSI acquisition related to power-domain adaptation for 6G energy efficiency improvement.

# References

RP-251881, New SID: Study on 6G Radio, NTT DOCOMO, RAN #108

6GWS-250002, Revised agenda for 3GPP workshop on 6G

R1-2505100, Draft Agenda, 3GPP TSG RAN WG1 #122

R1-2506738, “High level views on 6GR air interface”, Futurewei, RAN1#122bis, October 13th-17th, 2025

TS 38.300, NR; NR and NG-RAN Overall description; Stage-2

TS 38.304, NR; User equipment (UE) procedures in Idle mode and in RRC Inactive state

TR-38.864, Study on network energy savings for NR

TR-38.869, Study on-low-power Wake-up Signal and Receiver for NR

TR-38.875, Study on support of reduced capabilities NR devices

R1-2506602, “Final Summary of 6GR Energy Efficiency Study”, Moderators (Ericsson, MediaTek), RAN1#122, Aug 25th-29th.

Xingqin Lin, Dongsheng Yu, Henning Wiemann, "A Primer on Bandwidth Parts in 5G New Radio," in CoRR, vol. abs/2004.00761, 2020.

F. Abinader et al., "Impact of Bandwidth Part (BWP) Switching on 5G NR System Performance," 2019 IEEE 2nd 5G World Forum (5GWF), Dresden, Germany, 2019, pp. 161-166, doi: 10.1109/5GWF.2019.8911626.

MediaTek, “Bandwidth part adaptation; 5G NR user experience & power consumption enhancements,” white paper, 2018.

# Annex

## UE Power Consumption Models

The current status for MR’s power states, their characteristics, relative power consumption values, and sleep to non-sleep states transition energy is captured in Table 3 and Table 4 for non-RedCap and RedCap devices assuming the following changes with respect to the reference configuration for FR1 in TR38.840: 20MHz bandwidth, 2RX chains, 1TXchain, same-slot scheduling, N=8 cells for intra-/inter-frequency measurements.

Table 3: UE Power Consumption Model for Main Radio.

| Power State | Characteristics | Relative Power (eMBB) [20 MHz BW] | Relative Power (RedCap) [20 MHz BW] |
| --- | --- | --- | --- |
| Ultra-deep sleep | Power consumption of the main radio when LP-WUR is used for LP-WUS monitoring | 0.015 | 0.015 |
| Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | 1 (Optional: 0.5) | 0.8 |
| Light Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. | 20 | 18 |
| Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state | 45 | 31 |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 50 | 50 |
| SSB or CSI-RS processing | SSB can be used for fine time-frequency sync. And RSRP measurement of the serving/camping cell. TRS is the considered CSI-RS for sync. | 50 | 50 |
| PDCCH + PDSCH | PDCCH + PDSCH. | 120 | 120 |
| UL | Long PUCCH or PUSCH | 250 (0 dBm) 700 (23 dBm) | 250 (0 dBm) 700 (23 dBm) |
| Intra-frequency RRM measurement* | Power consumption of the main radio during intra-frequency neighbor cell measurement | 60†/80‡ | 60†/80‡ |
| Inter-frequency RRM measurement* | Power consumption of the main radio during inter-frequency neighbor cell measurement | 60†/150‡ | 60†/150‡ |
| * Synchronous SSB transmissions from cells and number of cells for intra-frequency measurement of N=8 are assumed. † Relative power accounting for measurements only ‡ Relative power accounting for both measurement and neighbor cell search Note1: 2 RX chains are assumed for the relative power values in this table, a scaling factor of ‘0.7’ can be used for non-sleep power states in case of 1 RX chain. Note2: a cell search rate of (1/4) can be considered to determine the effective (combined) relative power for intra/inter-frequency measurement and search. |  |  |  |

Table 4: UE Power Consumption for Main Radio during the State Transition.

| Sleep type | Additional transition energy: (Relative power x ms) | Total transition time |
| --- | --- | --- |
| Ultra-deep sleep | Alt 1: 15000** Alt 2: 40000** | Alt 1: 400 ms† Alt 2: 800 ms† |
| Deep sleep | 450 | 20 ms |
| Light sleep | 100 | 6 ms |
| Micro sleep | 0 | 0 ms* |
| * Immediate transition is assumed for power saving study purpose from or to a non-sleep state ** Additional transition energy for ‘Ultra-deep sleep’ power state accounts for both ramp-up and ramp-down, it does not account for energy required for (re-)synchronization. † Total transition time for ‘Ultra-deep sleep’ power state accounts for ramp-up time only, assumptions for (re-) synchronization time are separate. |  |  |

## BS Power Consumption Model

The reference configuration for single CC case as captured in Table 5.1-1 of the TR38.864 is reproduced in Table 5.

Table 5: Reference configuration for BS power consumption model

|  | Set 1 FR1 | Set 2 FR1 | Set 3 FR2 |
| --- | --- | --- | --- |
| Duplex | TDD | FDD | TDD |
| System BW | 100 MHz | 20 MHz | 100 MHz |
| SCS | 30 kHz | 15 kHz | 120 kHz |
| Number of TRP | 1 | 1 | 1 |
| Total number of DL TX RUs | 64 | 32 | 2 |
| Total DL power level | 55 dBm | 49 dBm | 33 dBm* |
| Total number of UL Rx RUs | 64 | 32 | 2 |
| *Note: EIRP limit is 63 dBm for the reference configuration. The EIRP value is scaled with the number of TxRUs. |  |  |  |