3GPP TSG RAN WG1 #122bis	                                                                               R1-2506775

Prague, Czech, Oct 13th – 17th, 2025

Agenda item:	10.1.1.2

Title:	Discussion on other aspects of CSI compression

Source:	ZTE Corporation, Sanechips

Document for:	Discussion and Decision

Introduction

AI/ML based CSI feedback has undergone extensive examination in Rel-18 and Rel-19, encompassing an evaluation of its potential benefits and an analysis of potential spec enhancements. Leveraging the insights from Rel-18 and Rel-19, Rel-20 aims to provide normative support for the CSI feedback enhancement (two-sided model) use case [1]. Specifically, the normative work of Rel-20 encompasses specification enhancements for facilitating LCM operations specific to AI/ML CSI feedback without temporal aspects, including model pairing, inference, training data collection, and performance monitoring. In this contribution, we provide our analysis and proposals on other aspects of AI/ML CSI feedback enhancement.

General views on AI/ML CSI feedback enhancement

According to the work item description for AI/ML for NR air interface enhancements, three inter-vendor training collaboration options are to be specified with a check point set in RAN#110, i.e., fully defined/specified reference model in Direction C, standardized encoder model structure plus parameter exchange ('Direction A option 3a-1' without target CSI sharing), and standardized dataset format/content plus dataset exchange ('Direction A option 4-1'). Notably, the selection of inter-vendor training collaboration options may directly impact the discussions on the other aspects. For example, in the model pairing procedures, a dataset ID or model ID may be dynamically indicated to the UE along with each exchanged dataset or set of model parameters for Direction A sub-option 4-1 or 3a-1. But for Direction C, a model ID associated with the fully defined/specified reference model may be fixed in the specification. Therefore, to ensure the integrity of the standards and the implementation flexibility, spec enhancements of other aspects should be compatible to all to be specified inter vendor training collaboration options taking the scalability design into account.

Spec enhancements of other aspects should be compatible to all to be specified inter vendor training collaboration options (i.e., Direction A options 3a-1 and 4-1, Direction C) including the associated scalability design.

The other aspects for AI/ML CSI feedback enhancement that have been identified by companies includes model pairing procedures, training data collection, and performance monitoring. Those aspects will be specified when necessary. However, it’s noted that some other aspects have already been discussed and specified in AI/ML beam management and CSI prediction use cases in Rel-19 such as training data collection framework. Given that all three AI/ML enabled use cases share the same CSI framework, it's not expected that different methods or signalings are used for the same purpose from the perspective of standard simplicity, unless there is a very strong and urgent motivation. Generally, we suggest to directly reuse the spec enhancements specified for AI/ML beam management and CSI prediction to reduce spec efforts and simplify the spec design, when applicable.

Spec enhancements specified for AI/ML beam management and CSI prediction use cases should be reused for AI/ML CSI feedback to reduce spec efforts and simplify the spec design, when applicable.

Potential specification impacts

Model pairing procedures

- Pairing ID definition

In Rel-19, the procedure to align the pairing information between CSI generation part and CSI reconstruction part were studied and the following observation was made.

Observation In CSI compression using two-sided model use case, at least the following options have been proposed by companies to define the pairing information used to enable the UE to select a CSI generation model(s) that is compatible with the CSI reconstruction model(s) used by the gNB: - Option 1: The pairing information is in the forms of the CSI reconstruction model ID that NW will use. - Option 2: The pairing information is in the forms of the CSI generation model ID that the UE will use. - Option 3: The pairing information is in the forms of the paired CSI generation model and CSI reconstruction model ID. - Option 4: The pairing information is in the forms of by the dataset ID during type 3 sequential training. - Option 5: The pairing information is in the forms of a training session ID to a prior training session (e.g., API) between NW and UE. - Option 6: The pairing information is up to UE/NW offline co-engineering alignment, transparent to 3GPP specification. - Note: the disclosure of the vendor information during the model pairing procedure and model identification procedure should be considered. - Note: If each UE side model is compatible with all NW side model, the information is not needed for the UE. Note: Above does not imply there is a need for a central entity for defining/storing/maintaining the IDs.

For AI/ML CSI feedback using two-sided model use case, to enable the UE to select an encoder compatible with the decoder used by the gNB, pairing ID shall be defined which may be in the form of specified ID, dataset ID, or model ID, depending on the inter-vendor training collaboration options. Specifically, for inter-vendor training collaboration Direction C, a reference model including its model structure and parameters is fully standardized/specified/defined, with each associated with a specified ID fixed in the specification. The specified ID can then be used for pairing purpose, regardless of the potential offline engineering at the NW or UE side. For inter-vendor training collaboration Direction A 3a-1, a scalable model structure is specified/defined and a set of scalable model parameters corresponding to the model structure are exchanged from the NW side to the UE side during model training phase, with each associated with a model ID. In this case, the model ID is dynamically assigned by the NW side. Similarly, for inter-vendor training collaboration Direction A 4-1, a dataset consisting of {target CSI, CSI feedback} are exchanged from the NW side to the UE side during model training phase, with each associated with a dataset ID assigned by the NW side.

To enable the UE to select an encoder compatible with the decoder used by the gNB, pairing ID shall be defined which may be in the form of specified ID, dataset ID, or model ID, depending on the inter-vendor training collaboration options.

- Applicability reporting

The applicability reporting procedures for AI/ML beam management have been well studied by RAN2 in Rel-19 for the alignment of applicable model/functionality ready for inference between gNB and UE. Specifically, as illustrated in the following figure, after receiving the UE capability information, the gNB will provide some candidate inference configurations (e.g., inference related parameters, full CSI report configuration) to the UE for applicability inquiry. The UE decides the applicable functionalities based on provided inference configurations and model availability in device, and then reports applicability for all the candidate inference configurations. Finally, applicable inference configurations can be activated/triggered by the gNB after the applicability reported. On a high level, all the applicability reporting procedures defined for AI/ML beam management shall be directly reused for AI/ML CSI feedback to reduce the spec efforts.

For the applicability report for AI/ML CSI feedback use case, reuse the applicability reporting procedures defined for AI/ML beam management as a starting point.

In RAN1#122 meeting, the applicability report procedure of AI/ML CSI compression were discussed and the following agreement were reached.  There are four remaining issues for further study.

Agreement: For model pairing procedure for inference, consider the applicability report procedure of Rel-19 AI/ML beam management for UE side model as a starting point. - Pairing ID(s) (i.e., ID for pairing related discussion) is indicated (e.g., in CSI-reportConfig) in Step 3. - FFS whether/how to link the Pairing ID with the inter-vendor collaboration - FFS whether/how Paring ID(s) are reported in Step 4 - FFS whether to support both Option A (CSI-ReportConfig for inference configuration) and Option B (sets of inference related parameters for applicability report only). - FFS CSI compression specific inference configuration or inference related parameters.

- Linkage between pairing ID and inter-vendor collaboration. It has been agreed in the inter-vendor training collaboration agenda that one pairing ID is assigned to one dataset. Based on our understanding, during model training phase, each delivered dataset (for Direction A option 4-1) or each set of delivered model parameters (for Direction A option 3a-1) is associated with a pairing ID. For Direction C, one or multiple models including model structure and parameters are specified, each of which is associated with a hard-coded pairing ID. Then, for applicability reporting procedure, the pairing ID is indicated to the UE to check whether the corresponding dataset or models are supported by the UE.

- Reporting of pairing ID in Step 4. It has been agreed in the inference related aspects agenda that the pairing ID is configured in the CSI reporting configuration for inference. Then, for Option A, multiple CSI reporting configurations are provided to the UE in Step 3, and the UE will feedback the applicable functionality by reporting the CSI reporting configuration ID in Step 4. There seems no necessity to additionally report pairing ID. Similarly, for Option B, the benefit of reporting pairing ID on top of the inference related parameter set ID is also not clear. Anyway, this issue can be suspended after the CSI report configuration or the inference related parameter are finalized.

- Potential down selection between Option A and Option B. It’s noted that both Option A and Option B are proposed for the applicability reporting of AI/ML beam management and got supported finally. For Option A, one or more of CSI-ReportConfig for inference configuration are provided in Step 3. However, since there is no priori information on UE-side applicability status, many of the inference configurations provided in Step 3 may become useless and result in a waste of air interface resources. As a supplement, Option B is supported where a set of inference related parameters is provided for applicability report only (not for inference) in Step 3, and applicable inference configuration will be provided to the UE in a later step. Considering the different applicable scenarios of Option A and Option B, we prefer to support both options for the applicability report procedure of AI/ML CSI compression.

- CSI compression specific inference configuration. For inter-vendor training collaboration Direction A 4-1, a dataset transferred to the UE side may consist of data samples from multiple configuration combinations of {number of Tx ports, CSI feedback payload size, subband}. Assume that the UE-part model only supports one of the multiple configuration combinations. Then, besides the pairing ID, additional indication on the number of Tx ports, CSI feedback payload size, or subband may need to be provided to the UE as part of the candidate inference configurations. Similarly, for inter-vendor training collaboration Direction A 3a-1, assuming a set of scalable model parameters transferred to the UE side can support multiple configurations, additional indication on the number of Tx ports, CSI feedback payload size, or subband may also be necessary for applicability inquiry.

For the applicability report for AI/ML CSI feedback, support both Option A (CSI-ReportConfig for inference configuration) and Option B (sets of inference related parameters for applicability report only).

The  CSI feedback specific inference configuration or inference related parameters provided to the UE shall at least include configurations of subband, Tx port and payload size.

Data collection

NW-side training data collection

For NW side data collection for training, CSI resources for channel measurement will be transmitted to the UE and then the UE reported the target CSI to the NW side for training purpose. Since the training data collection is not latency sensitive and involves a large amount of data samples exchanging, the MDT framework defined in BM and CSI prediction use cases could be reused.

- Target CSI format

For NW side data collection for training, the target CSI type shall be aligned with that defined for model inference, which may be precoding matrix in spatial-frequency domain. Regarding the target CSI format, different alternatives have been identified by companies in RAN1#122 meeting as below.

Agreement: For NW side data collection, study the solution for quantizing Target CSI, from the aspects of overhead, quantization loss/performance, and complexity. E.g.,: - Option 0: Reusing legacy e-Type II codebook - Option 1: Scalar quantization to the Target CSI. FFS number of bits for real/imaginary, whether coefficients for quantization are obtained from transformation of Target CSI. - Option 2: Enhanced methods to reduce the SVD/EVD decomposition complexity, reduce frequency/spatial/beam domain basis searching, with potentially relaxed overhead, e.g., Type II like codebook with scalar quantized W2 similar to Option 1 scalar quantization. - Option 3: Enhanced method for improved overhead saving, e.g., reporting the dominant eigenvalues/eigenvectors of per layer precoding matrix. - Option 4: eType II like codebook with new parameter values (e.g. larger L, pv, beta, amplitude, phase) - Note: the quantizaton format should consider the maximum payload size per reporting (e.g., 9KBytes) of the current higher layer signaling. - Note: At least take precoding matrix as Target CSI type. FFS channel matrix. - FFS: EVM, metric, benchmark (e.g., FP32, eT2 with PC6/8), by reusing R18 study EVM as baseline.

Among the above 5 options for quantizing target CSI, the new CSI codebook in Option 2 and 3 has rarely been evaluated in Rel-18/19 study phase and its efficiency has not been justified. Apparently, it would be quite time consuming for evaluation alignment and justification. The other three solutions have been thoroughly evaluated in Rel-18/19 study phase, which shows that quantized high resolution ground-truth CSI can achieve significant overhead reduction with minor performance loss if the parameters are appropriately selected. Specifically, for R16 eType II CB with legacy parameters in Option 0, majority companies observed -2.9%~-5.5% performance loss compared to Float32, which is deemed significant especially considering the limited gains of AI/ML CSI compression over legacy.  For scalar quantization in Option 1, according to TR 38.843, 8 bits scalar quantization achieves 75% overhead reduction and -0.14%~-0.9% performance loss. The performance loss is marginal. However, given that the payload size of target CSI with float32 format is up to  ~150K bits, the payload size of target CSI with 8 bits scalar quantization may be up to ~37.5K bits, which is still unaffordable by the current higher layer signaling in MDT framework with 9K Bytes (i.e., 72K bits) limitations. For Option 4, R16 eType II CB with new parameter of 1K-1.4K bits CSI payload size achieves 95%~97.5% overhead reduction with performance gain of 0.7%~4.3% over PC#8. Since the allowed payload size is relatively large in the MDT framework, we suggest to use higher resolution codebook based quantization (e.g., R16 eType II CB with new parameter) for target CSI reporting for better tradeoff between model performance and reporting overhead.

For NW side data collection for training, support R16 eType II CB with new parameter as the target CSI type for better tradeoff between reporting overhead and inference performance.

- Reporting of pairing ID

The controversial part is whether to report the pairing ID as part of the content for NW side data collection. On one hand, the configuration of pairing ID in NW side data collection is beneficial for both NW side and UE side. From the NW perspective, a pairing ID is pre-assigned to the UE based on some determined rules (e.g., antenna port distribution or other NW-side settings). Then, the reporting of pairing ID along with the target CSI help relief NW-side efforts on data categorization for subsequent model training. From the UE perspective, the to-be-reported training data labeled by pairing ID can also be used for the offline engineering at the UE side. On the other hand, the configuration of pairing ID may mandate the NW to categorize the data only based on the pre-assigned pairing ID. Anyway, according to Rel-18/19 evaluations, the CSI compression model is mostly impacted by NW-side settings like antenna port distribution and thus data categorization only based on NW-side settings is reasonable in most cases. Additionally, the NW can just ignore the reported pairing ID and perform data categorization based on characteristics of received data samples.

For NW side data collection of CSI compression, the content of the collected data can include pairing ID which is beneficial for both NW side and UE side.

- Additional information

For NW side data collection for training, the quality of data reported by different UEs can vary significantly. For instance, data collected by UEs at the cell edge may suffer from low signal strength and severe interference from neighboring cells, resulting in poor data quality that is detrimental to effective model training. There are two potential solutions for addressing this issue. First, UEs could report auxiliary information along with the collected data - such as SINR, CQI, or positioning information - enabling the network to assess the data quality and determine whether and how to utilize it for training. Second, the network could pre-configure a data quality threshold to the UE. The UE would then evaluate whether the collected data meets the threshold and only report data that satisfies the quality criteria. Therefore, we propose further study on the potential specification impacts related to data quality management for NW-side data collection.

To enable high-quality data collection from the UE to the NW, further study the following options:

- The UE reports data quality-related information to the NW, e.g., SINR, CQI, positioning information

- The NW configures a data quality threshold to the UE and the UE only reports qualified data

UE-side training data collection

For UE-side training data collection, some potential spec impacts were identified by companies in Rel-19 and the following agreement was reached in RAN1#122 meeting.

Agreement: To enable UE side data collection, CSI-ReportConfig can be used for configuring the resources for data collection purpose without CSI report - P-CSI-RS and SP CSI-RS are supported

As agreed, the CSI report configuration is used for configuring the CSI-RS resources for UE-side training data collection purpose. The controversial part is whether pairing ID can be configured in the data collection configuration. As far as we are concerned, the provision of pairing ID is necessary to ensure the offline engineering at the NW and UE side are based on the same dataset and thus data distribution mismatch can be avoided. For example, for inter-vendor training collaboration Direction A 3a-1, the CSI report configuration for data collection is provided to the UE for the offline engineering (e.g., fine-tuning, retraining) of the delivered models. To ensure alignment regarding NW-side additional conditions between NW-side training data and UE-side training data, the pairing ID can also be configured in the data collection configuration, serving as the function of associated ID. Additionally, some additional information such as Tx antenna port or subband may also need to be configured in the CSI report configuration for data collection, which can be further clarified.

For UE-side training data collection, pairing ID can be configured to address data distribution mismatch issues regarding NW-side/UE-side additional conditions. FFS other necessary information.

Performance monitoring

In Rel-19, the following agreement related to performance monitoring based on intermediate KPIs were achieved. Generally, the NW-side performance monitoring with target CSI reporting is more reliable and shall be the prerequisite of UE-side performance monitoring, but may result in relatively large reporting overhead.

Agreement Further study following monitoring options in Rel-19, including the necessity and feasibility, - NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1) - SRS-based monitoring - UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the output of the CSI reconstruction model at the UE (Case 2-1) - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side. - Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2) - Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model - Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook Regarding monitoring metrics: - Monitoring accuracy also includes generalization considerations, if applicable. - Complexity also includes LCM complexity, if applicable. - Monitoring overhead, latency, complexity, and accuracy analysis may have to consider using at N>1 CSI feedback occasions. - Testability of UE reported metrics Discussion may include the following aspects: - Consideration of Options 1-5 and their options for alleviating / resolving the issues related to inter-vendor training collaboration - Temporal domain aspects of CSI compression - How the above monitoring approaches or combination of them may help identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation Note: for UE-side monitoring, the final reported monitoring output, if specified, may be different, e.g., be further derived based on the output of the above approaches. Note: implementation-based monitoring solutions can be considered in assessing the necessity of the above monitoring approaches.

- NW-side performance monitoring

For NW-side performance monitoring, the UE shall report ground-truth CSI information to the network for calculating the monitoring metrics, as the network cannot directly obtain the ground-truth label. To enhance the accuracy of NW-side performance monitoring, a higher-resolution ground-truth CSI report is required from the UE. Similar to the approach for data collection, an enhanced Type-II codebook with acceptable reporting overhead can be a promising solution for NW-side performance monitoring. As demonstrated in TR 38.843, this approach provides high monitoring accuracy. Additionally, some companies have proposed using sounding reference signals for NW-side performance monitoring. However, in FDD systems, the channel reciprocity is limited, making it unreliable for the network to infer downlink channel conditions based on uplink SRS measurement. Therefore, we recommend specifying NW-side performance monitoring based on target CSI reporting, leveraging enhanced CSI codebooks with higher-resolution parameter configurations.

For AI/ML CSI feedback, support NW-side performance monitoring based on the target CSI reporting with CSI codebook enhancement via higher resolution parameter combination.

- UE-side performance monitoring

For UE-side performance monitoring, four alternatives were proposed by companies in Rel-18 as below:

- Alt 1: UE-side monitoring based on the actual CSI reconstruction model at NW side

- Alt 2: UE-side monitoring based on the output CSI transmitted from NW side to UE side

- Alt 3: UE-side monitoring based on precoded RS transmitted from NW based on the output of the CSI reconstruction model

- Alt 4: UE-side monitoring based on the UE-side proxy model

We further analyze and summarize the pros and cons of these approaches in the following Table.

Table. The analysis on UE-side performance monitoring approaches

| UE-side monitoring alternatives | Analysis |
| --- | --- |
| Alt 1: UE-side monitoring based on the actual CSI reconstruction model at NW side | - Accurate intermediate KPI can be calculated - Huge burden on the overhead of CSI reconstruction model delivery over the air - NW-side model proprietary information is disclosed to UE - UE device may not be able to perform monitoring due to the limited computational/storage capability |
| Alt 2: UE-side monitoring based on the output CSI transmitted from NW side to UE side | - Less transmission overhead compared with Alt 1 - Additional latency is needed for output CSI transmission. - Quantization loss would be introduced to output CSI due to the quantization type, e.g., codebook-based quantization. - Additional storage burden for UE may be needed since UE has to buffer the previous CSI to match the output-CSI for calculating performance metrics. |
| Alt 3: UE-side monitoring based on precoded RS transmitted from NW based on the output of the CSI reconstruction model | - Additional latency is needed for precoded RS transmission. - Time span of monitoring process is increased, where two-step procedure is needed to finish performance monitoring. - Monitoring results may not adapt to the channel variation/aging/channel estimation errors - Additional storage burden for UE may be needed since UE has to buffer the previous CSI to match the output-CSI for calculating performance metrics. |
| Alt 4: UE-side monitoring based on the UE-side proxy model | - No need to transmit CSI reconstruction model or output CSI - The weak learning capability of proxy model incurs worse generalization performance according to our observations - If the proxy model is up to UE implementation and transparent to the Network side, the monitoring performance of the UE-side proxy model is not convincing for the NW side since no additional LCM of UE-side proxy model is operated - If the proxy model is not transparent to the Network side, it would impose huge burden on the network side to additionally identify/monitor/manage the UE-side proxy model. |

For Alt1, the actual CSI reconstruction model is the proprietary information of the NW side and cannot be disclosed to the UE side. For Alt 2, the output-CSI needs to be sent back from the NW side to the UE side in forms of quantization value, e.g., scalar quantization or codebook-based quantization. It leads to additional latency and quantization loss. And the UE has to buffer the previous CSI to match the output-CSI, resulting in additional storage burden at the UE side. For Alt 3, time span of the monitoring process is increased due to the two-step procedures and thus it may suffer from channel aging issues, which needs to be further studied. For Alt 4, the proxy model shows significant generalization performance loss in terms of monitoring accuracy according to TR 38.843, and it may impose huge burden on the network side to additionally identify/monitor/manage the UE-side proxy model. Based on the analysis on the four alternatives, each one has some challenging problems to be addressed in actual deployment. Therefore, we propose to prioritize NW-side performance monitoring and take UE-side performance monitoring as second priority.

For AI/ML CSI feedback, prioritize NW-side performance monitoring and take UE-side performance monitoring as second priority.

Conclusions

In this contribution, we discuss the specification enhancements for other aspects of AI/ML based CSI feedback. We have the following proposals.

Proposals：

- Spec enhancements of other aspects should be compatible to all to be specified inter vendor training collaboration options (i.e., Direction A options 3a-1 and 4-1, Direction C) including the associated scalability design.

Spec enhancements specified for AI/ML beam management and CSI prediction use cases should be reused for AI/ML CSI feedback to reduce spec efforts and simplify the spec design, when applicable.

To enable the UE to select an encoder compatible with the decoder used by the gNB, pairing ID shall be defined which may be in the form of specified ID, dataset ID, or model ID, depending on the inter-vendor training collaboration options.

For the applicability report for AI/ML CSI feedback use case, reuse the applicability reporting procedures defined for AI/ML beam management as a starting point.

For the applicability report for AI/ML CSI feedback, support both Option A (CSI-ReportConfig for inference configuration) and Option B (sets of inference related parameters for applicability report only).

The  CSI feedback specific inference configuration or inference related parameters provided to the UE shall at least include configurations of subband, Tx port and payload size.

For NW side data collection for training, support R16 eType II CB with new parameter as the target CSI type for better tradeoff between reporting overhead and inference performance.

For NW side data collection of CSI compression, the content of the collected data can include pairing ID which is beneficial for both NW side and UE side.

To enable high-quality data collection from the UE to the NW, further study the following options:

- The UE reports data quality-related information to the NW, e.g., SINR, CQI, positioning information

- The NW configures a data quality threshold to the UE and the UE only reports qualified data

For UE-side training data collection, pairing ID can be configured to address data distribution mismatch issues regarding NW-side/UE-side additional conditions. FFS other necessary information.

For AI/ML CSI feedback, support NW-side performance monitoring based on the target CSI reporting with CSI codebook enhancement via higher resolution parameter combination.

For AI/ML CSI feedback, prioritize NW-side performance monitoring and take UE-side performance monitoring as second priority.

References

RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, Qualcomm (Moderator), RAN#108.