3GPP TSG RAN WG1 #122bis		R1-2506889

Prague, Czech, Oct 13th – 17th, 2025

Source:	vivo

Title:	Discussion on 	Inter-vendor training collaboration for two-sided AI/ML

models

Agenda Item:	10.1.2

Document for:	Discussion and Decision

Introduction

In RAN #108 meeting, AI CSI compression have been approved in the WID [1], including the following objectives:

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback

In this contribution, we focus on the second objectives and present our views on specification of standardized dataset format/content plus dataset exchange.

Specification of standardized dataset format/content

Data sample format and type

Agreement: For Option 4-1 under Direction A in AI/ML based CSI compression, support at least target CSI and CSI feedback in the exchanged dataset(s). - FFS: Target CSI type and format - FFS: CSI feedback type and format - FFS: Association between Target CSI and CSI feedback, including mapping for different number of Tx port, number of sub bands, and CSI payload size.

In the last meeting, the above agreement had been achieved. In this section, we focus on the FFS issue, including target CSI format, CSI feedback format, and association between target CSI and CSI feedback.

The type of Target CSI

Agreement: For NW side data collection, study the solution for quantizing Target CSI, from the aspects of overhead, quantization loss/performance, and complexity. E.g.,: - Option 0: Reusing legacy e-Type II codebook - Option 1: Scalar quantization to the Target CSI. FFS number of bits for real/imaginary, whether coefficients for quantization are obtained from transformation of Target CSI. - Option 2: Enhanced methods to reduce the SVD/EVD decomposition complexity, reduce frequency/spatial/beam domain basis searching, with potentially relaxed overhead, e.g., Type II like codebook with scalar quantized W2 similar to Option 1 scalar quantization. - Option 3: Enhanced method for improved overhead saving, e.g., reporting the dominant eigenvalues/eigenvectors of per layer precoding matrix. - Option 4: eType II like codebook with new parameter values (e.g. larger L, pv, beta, amplitude, phase) - Note: the quantizaton format should consider the maximum payload size per reporting (e.g., 9KBytes) of the current higher layer signaling. - Note: At least take precoding matrix as Target CSI type. FFS channel matrix. - FFS: EVM, metric, benchmark (e.g., FP32, eT2 with PC6/8), by reusing R18 study EVM as baseline. Proposal 1-3: For Option 4-1 under Direction A in AI/ML based CSI compression, for target CSI format, further study the following two approaches: • Option 1: Target CSI format reuses the same format as NW-side data collection for training. • Option 2: Target CSI format for inter-vendor training collaboration is discussed separately.

High resolution ground-truth CSI for training For the evaluation of high-resolution quantization of the ground-truth CSI for the training of CSI compression, compared to the upper-bound of Float32, quantized high resolution ground-truth CSI can achieve significant overhead reduction with minor performance loss if the parameters are appropriately selected. - For high resolution scalar quantization, - Float16 achieves 50% overhead reduction and -0.6% or less performance loss from 2 sources - 8 bits scalar quantization achieves 75% overhead reduction and -0.14%~-0.9% performance loss from 2 sources - For high resolution R16 eType II-like quantization, - R16 eType II CB with legacy parameters can achieve significant overhead reduction while with performance loss compared to Float32, wherein: - PC#6 achieves around 99% overhead reduction with -1.4% ~-1.7% performance loss from 2 sources, and -3%~-9.5% performance loss from 4 sources. - PC#8 achieves around 98% overhead reduction with 0% ~-1.7% performance loss from 3 sources, and -2.9%~-5.5% performance loss from 5 sources. - For R16 eType II CB with new parameters: - R16 eType II CB with new parameter of 1000-1400bits CSI payload size achieves 95%~97.5% overhead reduction (3~4.1 times overhead compared to PC8) with performance gain of 0.7%~4.3% over PC#8 from 4 sources. - R16 eType II CB with new parameter of 1500-2100bits CSI payload size achieves 94%~96.2% overhead reduction (4.8~6.1 times overhead compared to PC8) with performance gain of 1.3%~5.4% over PC#8 from 3 sources. - Note: it is observed by 1 source that using R16 eType II-like quantization with legacy PC may achieve close performance to Float32 by dataset dithering. - Note: the new parameters include at least one from the follows: - L= 8, 10, 12; - pv = 0.8, 0.9, 0.95; - reference amplitude = 6 bits, 8 bits; differential amplitude = 4bits; phase = 5 bits, 6 bits;

In the last meeting, FL proposed discussing the Target CSI format for inter-vendor training collaboration separately. We share FL's view that while the UE cannot collect high-resolution target CSI directly, some high-resolution target CSI can still be obtained through network-side SRS reception or interpolation algorithms.

Additionally, it's important to note that the existing UCI format (including part 1 and part 2) for CSI reporting cannot be directly reused for Target CSI format with R16 eType II CB (i.e., option 0 and option 4). This is because subband CQI and rank information are not required, and datasets organized by layer better meet the needs of layer-common model training. For example, Target CSI format with R16 eType II CB can include layer ID, non-zero amplitude coefficients of the layer and Part 2 contains the PMI of the Enhanced Type II.

TS 38.214 5.2.3 CSI reporting using PUSCH - For Enhanced Type II CSI feedback (see Clause 5.2.2.2.5), Further Enhanced Type II Port Selection CSI feedback (see Clause 5.2.2.2.7), Enhanced Type II for predicted PMI with N4=1 (see Clause 5.2.2.2.10) and Further Enhanced Type II Port Selection for predicted PMI (see Clause 5.2.2.2.11), Part 1 contains RI (if reported), CQI, and the total number of reported non-zero amplitude coefficients across layers. The fields of Part 1 – RI (if reported), CQI, and the total number of reported non-zero amplitude coefficients across layers – are separately encoded. Part 2 contains the PMI of the Enhanced Type II, Further Enhanced Type II Port Selection CSI, Enhanced Type II for predicted PMI with N4=1 or Further Enhanced Type II Port Selection for predicted PMI. Part 1 and 2 are separately encoded.

Moreover, five options (Option 0 to Option 4) have been agreed upon as potential Target CSI formats for NW-side data collection. From our perspective, Options 0, 1, and 4 are clearer, as they have been evaluated in Rel-18. Therefore, we prefer to down-select the Target CSI format for inter-vendor training collaboration from Options 0, 1, and 4. Since Option 0 and Option 4 have been evaluated in study item, here we further evaluate Option 1 and provided the evaluation results as following.

The SGCS performance of different quantization method.

| Quantization level of inf/mo data Quantization level of training data | Inference/monitoring: input and label are same quantization data as training data | Inference/monitoring: input and label are float data |  | Inference/monitoring: input is float data, label is same quantization data as training data |  |
| --- | --- | --- | --- | --- | --- |
|  | SGCS | SGCS | Gain to model trained with float data | SGCS | Gain to model trained with float data |
| No quantization (Baseline) | 0.75066(Baseline) |  |  |  |  |
| Int8 | 0.74857 | 0.74880 | -0.25% | 0.74857 | -0.28% |
| Int6 | 0.74659 | 0.75025 | -0.05% | 0.74659 | -0.54% |
| Int5 | 0.74204 | 0.75664 | 0.80% | 0.74204 | -1.15% |
| Int4 | 0.69786 | 0.75264 | 0.26% | 0.69786 | -7.03% |
| Int3 | 0.53622 | 0.69630 | -7.24% | 0.53622 | -28.57% |

Based on the evaluation results, we can observe that scalar quantization with suitable quantization level could achieve good performance while significantly reducing overhead, especially for “Inference: input and label are float data”. For example, Int8 can reduce 75% overhead, with only a marginal 0.25% performance loss. Furthermore, Int 4 can reduce 87.5% overhead, with no performance loss.

- There is no or only minor performance loss in the option 1quantization Target CSI if quantization precision of at least int4

- Target CSI format for inter-vendor training collaboration is discussed separately, and can be down select from the following options

  - Option 0: Reusing legacy e-Type II codebook

  - Option 1: Scalar quantization to the Target CSI. FFS number of bits for real/imaginary

  - Option 4: eType II like codebook with new parameter values (e.g., larger L, pv, beta, amplitude, phase)

The type of CSI feedback

For CSI feedback, the primary issue is whether the CSI feedback is pre-quantized or post-quantized?

- Option A: Pre-quantized CSI is a floating-point vector with a dimension of  zdim, the CSI feedback ∈ Rzdim

- Option B: Post-quantized CSI is a binary sequence with payload bits:b1, b2,⋯bpayloads

From an overhead and standardization perspective, Option B (post-quantized CSI) has less overhead and is simpler. For instance, the overhead of pre-quantized CSI (e.g., zdim=30, payload=60) is ten times greater than that of post-quantized CSI. Moreover, based on following evaluation as Table 1, the performance difference between Option A and Option B is minimal. Specifically, we train UE-sided encoders for pre-quantized CSI or post-quantized CSI separately, and then evaluate the end-to-end performance gap between the two models.  Our observations indicate that, the difference in SGCS before or after quantization is less than 1%. Therefore, we recommend focusing on post-quantized CSI.

SGCS(s) for two options of CSI feedback in dataset

|  | SGCS for the dataset of (Target CSI, pre-quantized CSI (i.e.,floating-point values)) | SGCS for the dataset of (Target CSI, post-quantized CSI (i.e.,binary sequence)) |
| --- | --- | --- |
| Dataset A(Data with first port phase align) | 0.739 | 0.735 (-0.5%) |
| Dataset B(Data with last port phase align) | 0.739 | 0.735 (-0.5%) |

Furthermore, if different payloads CSI feedbacks can be generated from a single target CSI using a scalable model- such as the model supporting multiple branches to produce multiple CSI feedbacks from a single target CSI, the multiple payload indications can be indicated in the dataset. Considering the quantization parameters {e.g., L and Q} need to be provided along with exchanged dataset, payload size in term of binary sequence length or real-valued dimension  dz can be mutually calculated based on each other as follows

Method 1: {Payload size, L and Q}-> dz

Method 2: {dz, L and Q }->Payload size

- CSI feedback is Post-quantized CSI, which is a binary sequence with payload bits:{b_1,b_2,⋯b_payloads}, and payload information shall be indicated along with the CSI feedback

  - FFS: the payload information is provided by payload size or the parameter combination {dz, L and Q}

The association between target CSI and CSI feedback

Additional core issue for a data sample is the mapping relationship between target CSI and CSI feedback, and the potential mapping relationship of a data units includes:

Mapping 1(one-to-one mapping): {target CSI, CSI feedback#1}, {target CSI, CSI feedback#2}

Mapping 2(one-to-many mapping): {target CSI, CSI feedback#1, ⋯, CSI feedback#i, ⋯,CSI feedback#N}

Compared to mapping 1 and mapping 2, mapping 2 can reduce overhead by omitting some repetition target CSIs. One-to-many mapping is more straightforward type when different payloads CSI feedbacks can be generated by a target CSI and a scalable model (i.e., the model with multiple branches can generate multiple CSI feedbacks from a single target CSI). And the target CSIs with same configuration should share the same CSI feedback format, Therefore, we propose mapping 2 can be considered as a potential mapping relationship between target CSI and CSI feedback, and same configuration target CSIs can be grouped together as a subgroup of a dataset.

- For a data sample, the following mapping relationship between target CSI and CSI feedback can be considered

  - One-to-many mapping: one target CSI can be compressed to multiple CSI feedbacks. e.g., {target CSI, CSI feedback#1, ⋯, CSI feedback#i, ⋯,CSI feedback#N}

  - Multiple data samples with same target CSI configuration can be grouped together as a subgroup within a dataset

Additional information along with the exchanged dataset

Agreement - For option 3a/3b/4/5a and their sub-options, at least the following potential specification impacts have been identified. Further study the necessity, feasibility, their specification impact. - Exchange - Parameter / model exchange methods, format/contents, and related spec impacts (3a/3b/5a) - Dataset exchange methods, format/type/contents of data/dataset, and related spec impacts (4) - Additional information, if necessary, that may be shared from the NW-side to help UE-side offline engineering and provide performance guidance (3a/5a/4) - Performance target (3a/5a/4) - Dataset or information related to collecting dataset (3a/5a) - Any other additional information - Model pairing (3a/3b/4/5a) - UE capability (3a/3b/4/5a) - Model related aspects, such as scalability (e.g., payload sizes, antenna ports, bandwidth), rank and layer handling (3a/3b/4/5a) - Quantization of feedback (3a/3b/4/5a) - Model structure details (3a/3b) Note: Option 3a/4/5a and option 3b serve two different deployment time scales, UE capabilities, device-side optimizations, and training methods, and therefore may be complementary to each other, with potential specification of both. - Specification of option 1, if needed from RAN1, can reuse specification of option 3a/3b, with the additional specification of parameters. Agreement For inter-vendor-collaboration Options 3a-1 and 4-1 in Direction A, performance target is confirmed as additional information along with the exchanged dataset or the model parameters. - FFS: type of performance metric - FFS: input data for evaluating the performance Agreement For inter-vendor-collaboration Options 3a-1 and 4-1 in Direction A, confirm SGCS and NMSE as the type of performance metric that may be used for the performance target shared as additional information along with the exchanged dataset or the model parameters. - FFS: when to use SGCS, NMSE, and which one to use or both, and relationship with the inter-vender collaboration sub-options. - FFS: details of the format of the performance target - Option 1: Average performance target, e.g. average SGCS and/or average NMSE - Option 2: distribution of the performance target, e.g., SGCS / NMSE for 5, 10, 20, 30 percentiles, etc. - FFS: whether multiple performance targets should be exchanged for different configurations, such as antenna ports configuration, subband configuration and payload configuration, etc., along with each exchanged dataset or model parameters

Pairing ID

Agreement: For Option 4-1 under Direction A in AI/ML based CSI compression, support exchange of single pairing ID along with dataset exchange. - One pairing ID is assigned to one dataset. - The dataset can have different number of Tx port, number of subbands, and CSI payload size configurations (including different quantization codebooks if needed). - From RAN1 perspective, the uniqueness of the pairing ID needs further studied. - FFS: the impact on pairing ID(s) when additional data samples are added to an exchanged dataset, if supported.

There are two different understandings for the first bullets:

Understanding 1: One-to-one mapping between pairing ID and dataset.

Understanding 2: Multiple -to-one mapping between pairing IDs and dataset.

Generally,  Understanding 1 is the prevailing view, as it enables clear identification of a dataset or model during pairing discussions. The understanding 2 is considered an enhancement related to the latest FFS Specifically, whether multiple pairing IDs can be associated with a single dataset. Although the mapping relationships between the datasets may differ, the new dataset remains backward compatible with the original pairing ID. This raises a new question of how to indicate backward compatibility for the new dataset.

The impact on pairing ID(s) and datasets

|  | exchange info along with dataset for initial inter vendor collaboration | exchange info along with update dataset for update inter vendor collaboration |
| --- | --- | --- |
| Understanding 1: one-to-one mapping between pairing ID and dataset. | Pairing ID # I0 | Pairing ID # I1 |
| Understanding 2: multiple-to-one mapping between pairing IDs and dataset. | {Pairing ID # I0, version#0} | solution 1: {Pairing ID # I0, version#1} |
|  | {Pairing ID # I0} | solution 2: {Pairing ID # I1, Pairing ID # I0 for backward compatible} |

In offline discussion, it was  suggested to use a version ID or a list of compatible pairing IDs to indicate compatibility. Between the two solutions, the solution 2 (pairing ID+ a list of compatible pairing ID) aligns better with the definition of a pairing ID, as it can differential the different datasets or models, and remains compatible with Understanding 1. In contrast, Solution 1 seems to apply only to software updates and not to new dataset categorization. Therefore, solution 2 is preferred.

Lastly, for the second bullet of the above agreement, the pairing ID should be unique within an operator vendor, as it can be used to identify a dataset or a model for pairing discussion.

- One-to-one mapping between pairing ID and dataset should be supported as baseline.

- The pairing ID should be unique within an operator vendor.

Performance target

The following agreement had been achieved in RAN1 #122 on the performance target in the exchanged dataset.

Agreement: For Option 4-1 under Direction A in AI/ML based CSI compression, consider the following methods for performance target in the exchanged dataset, - SGCS - FFS: Average SGCS - FFS: SGCS values at X-percentiles - NMSE: - FFS: When the exchanged CSI feedback is the floating-point values at the input of quantization - FFS: When the exchanged CSI feedback is the binary bit sequence at the output of quantization, the binary sequence will be mapped back to the floating-point values via quantization codebook FFS: Multiple performance targets for different layer when the target CSI type is precoding matrix, different configurations such as antenna ports, subband configuration and payload size configuration

Firstly, we used the dataset from RAN4 aligned mixed dataset [3] to show the feasibility of using NMSE or SGCS as the performance metric for training, since the RAN4 aligned mixed dataset is downloadable everywhere in the world.

From the figure it can be observed that there is monotonic correspondence between NMSE and SGCS. Thus, NMSE can be well mapped to SGCS to be set as a performance target. The NW can well map the performance target to either NMSE and SGCS without ambiguity.

Figure 1 SGCS performance for different NMSE target.

Agreement For the evaluation studies of Direction A, - For encoder parameter exchange from NW-side to UE-side of 3a-1, - Alt. 1: UE-side first trains a nominal decoder - Step 1: UE-side develops a nominal decoder against the exchanged encoder. - Step 2: UE-side develops actual encoder against the nominal decoder - Alt. 2: UE-side directly develops and trains the actual encoder. - Note: Target CSI is available at UE side - For dataset exchange from NW-side to UE-side {target CSI, CSI feedback} of 4-1, - Alt. 1: UE-side first trains a nominal decoder - Step 1: UE-side develops a nominal decoder using the exchanged dataset. - Step 2: UE-side develops an actual encoder against the nominal decoder. - Alt. 2: UE-side directly develops and trains the actual encoder.

From the observation, it can be clearly seen that both NMSE and SGCS can be used as performance target. During online discussion, it was suggested to narrow down the choice between NMSE and SGCS. However, from our perspective, these two metrics correspond to different encoder development methods—both of which were evaluated and agreed in the Rel-19 study. The final industry adoption remains uncertain, the standard needs to be open to different implementation approaches.

Additionally, it is important to note that NW-side SGCS is only feasible if the UE-side is able to mimic the NW -side decoder. However, if NW-side decoder utilizes additional more information to recover reconstructed target CSI (e.g., SRS measurement), the UE-side cannot mimic the network-side decoder. This mismatch may lead to suboptimal encoder training due to the decoder discrepancy. In this case, without accurate decoder, SGCS becomes infeasible, and NMSE should be used as the performance target, with only the encoder being mimicked on the UE side.

Figure 2 mismatch decoder training on UE-side Alt 1 training and Alt 2 encoder only training

Furthermore, providing two metrics does no harm and can offer more information for model training and evaluating. For example, NMSE reflects the encoder only performance, while SGCS can provide end-to-end performance, evaluating both performance targets together is also beneficial for Alt 1 training and evaluating.

Moreover, average SGCS and average NMSE are preferred over the distribution of the performance target (i.e., x% SGCS, or x% NMSE) due to their widespread use in model training and performance evaluation, and it is unclear to us how to assess whether performance meets the target distribution, and how to adjust the training process to achieve this distribution, given that the training typically focuses on optimizing overall or average performance.

Lastly, regarding the last FFS, multiple performance targets can be exchanged for different configurations, as the performance metric gaps between different configurations are significant. Specifically, as shown in the evaluation results in Tables 2 to 4, the performance metric gaps between different ports or payloads are significant, whereas the gaps between different subbands are minimal.

SGCS(s) for different ports and/or different payloads

| 8port/16port/32port mixed training | 8port | 16 port | 32 port |
| --- | --- | --- | --- |
| 59bits SGCS | 0.823 | 0.731 | 0.694 (-1.7%) |
| 104bits SGCS/Loss to upper bound | 0.874 | 0.791 (-2.8%) | 0.750 (-2%) |
| 252bits SGCS/Loss to upper bound | 0.952 (-0.2%) | 0.897 (+0.1%) | 0.862 (+0.2%) |

SGCS(s) for different subbands and/or different payloads

| 10/11/13/15subband mixed training | 10 sub-b | 11 sub-b | 13 sub-b | 15 sub-b |
| --- | --- | --- | --- | --- |
| 59bits SGCS/Loss to upper bound | 0.683 (-1.3%) | 0.704 (-1.2%) | 0.695 (-1.5%) | 0.688 (-1.1%) |
| 104bits SGCS/Loss to upper bound | 0.750 (-2.0%) | 0.772 (-1.2%) | 0.762 (-1.0%) | 0.758 (-0.7%) |
| 252bits SGCS/Loss to upper bound | 0.863 (-0.8%) | 0.872 (-0.3%) | 0.861 (0.2%) | 0.851 (+0.4%) |

SGCS(s) for different payloads

|  | PC1, 59bits | PC3, 104bits | PC4, 153bits | PC5, 202bits | PC6, 252bits | PC8, 293bits |
| --- | --- | --- | --- | --- | --- | --- |
| SGCS/Loss to upper bound | 0.705 (-0.1%) | 0.764 (-0.26%) | / | / | / | 0.764 (-0.26%) |
| SGCS/Loss to upper bound | 0.697 (-1.27%) | 0.764 (-0.26%) | 0.806 (-0.1%) | 0.838 (+0.3%) | 0.861 (+0.1%) | 0.875 (-0.2%) |

- Both NMSE and SGCS can be used as performance target shared as additional information along with the exchanged dataset.

- For performance target, there is one to one mapping relation between NMSE and SGCS. The NW can well map the performance target to either NMSE and SGCS without ambiguity.

- The performance metric gaps between different ports or payloads are significant, whereas the gaps between different subbands are minimal.

- Both SGCS and NMSE are supported as performance targets

  - Average SGCS and average NMSE can be considered

- Multiple performance targets can be exchanged for different configuration (e.g., different ports and different payloads)

Metric definition of performance target

Agreement For inter-vendor-collaboration Options 3a-1 and 4-1 in Direction A, confirm SGCS and NMSE as the type of performance metric that may be used for the performance target shared as additional information along with the exchanged dataset or the model parameters. Agreement For the definition of SGCS, Note: How to handle layer mapping mismatch, if any, is up to UE implementation. Agreement For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, - one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCS is reported - wideband frequency granularity - only for one prediction instance configured by NW - per layer, and the total number of layers is the same as the reported RI in the associated inference report - Introduce new RRC parameter for reportQuantity, e.g., ‘SGCS-r19’ - Each SGCS value is quantized with 4-bit - 15 codepoints are used to uniformly quantize in range [0.3 1] in linear scale. - One codepoint is used to indicate the value range of (0 0.3] - Only one monitoring resource set is configured in the CSI-ReportConfig - The one configured time instance (i.e. f-th doppler domain unit in one inference report) for SGCS calculation is configured in the CSI-ReportConfig for monitoring, for N4>1

For performance metric definition, average SGCS and average NMSE are two types of performance metric, and the two metrics definition are provided here, respectively.

For the definition of SGCS, a given layer l∈{1,…,v},  subband n3∈{1,…,N3}, and data instance n4∈1,…,N4，SGCS is defined as

SGCSn4,ln3=wn4,ln3Hwn4,ln3wn4,ln3wn4,ln32

where wn4,ln3 is the reconstructed precoder by NW-side decoder of l-th layer and n3-th subband, n4-th data instance, and wn4,ln3   is the corresponding precoder represented by PMI used for ground-truth target CSI for the l-th layer, n3-th subband, n4-th data instance.

- And then, average SGCS is calculated by

  - wideband frequency granularity

  - multiple data instances in dataset

  - per layer

For the definition of NMSE, it is NMSE{CSI feedback, CSI feedback} for encoder-only performance target

- For a given layer l∈{1,…,v},   and data instance n4∈1,…,N4，NMSE is defined as

NMSEn4,l=Ezn4,l-zn4,lEzn4,l2

where zn4,l=z0l,z1l,⋯,zdzl,ν-1l is the calculated CSI feedback based on UE-side CSI compression before quantization of l-th layer, n4-th data instance, and zn4,l=z0l,z1l,⋯,zdzl,ν-1l is the corresponding CSI feedback provided by NW if exchanged CSI feedback is the floating-point values, or  dequantization CSI feedback recovery performed by UE if exchanged CSI feedback is the binary bit sequence for l-th layer, n4-th data instance.

- And then, average NMSE is calculated by

  - multiple data instance in dataset

  - per layer

- For the definition of SGCS:

  - For a given layer l∈{1,…,v},  subband n3∈{1,…,N3}, and data instance n4∈1,…,N4，SGCS is defined as

SGCSn4,ln3=wn4,ln3Hwn4,ln3wn4,ln3wn4,ln32

where wn4,ln3 is the reconstructed precoder by NW-side decoder of l-th layer and n3-th subband, n4-th data instance, and wn4,ln3  is the corresponding precoder represented by PMI used for ground-truth target CSI for the l-th layer, n3-th subband, n4-th data instance.

  - And then average SGCS is calculated by

        - wideband frequency granularity

        - multiple data instances in a dataset

        - per layer

- For the definition of NMSE, it is NMSE{CSI feedback, CSI feedback} for encoder-only performance target.

  - For a given layer l∈{1,…,v},   and data instance n4∈1,…,N4，NMSE is defined as

NMSEn4,l=Ezn4,l-zn4,lEzn4,l2

where zn4,l=z0l,z1l,⋯,zdzl,ν-1l is the calculated CSI feedback based on UE-side CSI compression before quantization of l-th layer, n4-th data instance, and zn4,l=z0l,z1l,⋯,zdzl,ν-1l is the corresponding CSI feedback provided by NW if exchanged CSI feedback is the floating-point values, or dequantization CSI feedback provided by NW if exchanged CSI feedback is the binary bit sequence  for l-th layer, n4-th data instance.

  - Average NMSE is calculated by

        - multiple data instances in a dataset

        - per layer

Dataset Content

For a dataset content, the following can be included:

- Pairing ID

- Performance targets for each layer and each configuration

- Layer indication

- Configuration of target CSI and CSI feedback

- Data sample number(N) / Data sample type (one-to-one, or one-to-many)

- N Data sample is consisting of {target CSI, CSI feedback}

- For a dataset content, the following can be included:

  - Pairing ID

  - Performance targets for each layer and each configuration

  - Layer indication

  - Configuration of target CSI and CSI feedback

  - Data sample number(N) / Data sample type (one-to-one, or one-to-many)

  - N Data sample is consisting of {target CSI, CSI feedback}

Conclusion

In this contribution, we have the following observations and proposals:

- There is no or only minor performance loss in the option 1quantization Target CSI if quantization precision of at least int4.

- Target CSI format for inter-vendor training collaboration is discussed separately, and can be down select from the following options.

  - Option 0: Reusing legacy e-Type II codebook

  - Option 1: Scalar quantization to the Target CSI. FFS number of bits for real/imaginary

  - Option 4: eType II like codebook with new parameter values (e.g., larger L, pv, beta, amplitude, phase)

- CSI feedback is Post-quantized CSI, which is a binary sequence with payload bits:{b_1,b_2,⋯b_payloads}, and payload information shall be indicated along with the CSI feedback.

  - FFS: the payload information is provided by payload size or the parameter combination {dz, L and Q}.

- For a data sample, the following mapping relationship between target CSI and CSI feedback can be considered

  - One-to-many mapping: one target CSI can be compressed to multiple CSI feedbacks. e.g., {target CSI, CSI feedback#1, ⋯, CSI feedback#i, ⋯,CSI feedback#N}.

  - Multiple data samples with same target CSI configuration can be grouped together as a subgroup within a dataset.

- One-to-one mapping between pairing ID and dataset should be supported as baseline.

- The pairing ID should be unique within an operator vendor.

- Both NMSE and SGCS can be used as performance target shared as additional information along with the exchanged dataset.

- For performance target, there is one to one mapping relation between NMSE and SGCS. The NW can well map the performance target to either NMSE and SGCS without ambiguity.

- The performance metric gaps between different ports or payloads are significant, whereas the gaps between different subbands are minimal.

- Both SGCS and NMSE are supported as performance targets.

  - Average SGCS and average NMSE can be considered.

- Multiple performance targets can be exchanged for different configuration (e.g., different ports and different payloads).

- For the definition of SGCS:

  - For a given layer l∈{1,…,v},  subband n3∈{1,…,N3}, and data instance n4∈1,…,N4，SGCS is defined as

SGCSn4,ln3=wn4,ln3Hwn4,ln3wn4,ln3wn4,ln32

where wn4,ln3 is the reconstructed precoder by NW-side decoder of l-th layer and n3-th subband, n4-th data instance, and wn4,ln3  is the corresponding precoder represented by PMI used for ground-truth target CSI for the l-th layer, n3-th subband, n4-th data instance.

  - And then average SGCS is calculated by

        - wideband frequency granularity

        - multiple data instances in a dataset

        - per layer

- For the definition of NMSE, it is NMSE{CSI feedback, CSI feedback} for encoder-only performance target.

  - For a given layer l∈{1,…,v},   and data instance n4∈1,…,N4，NMSE is defined as

NMSEn4,l=Ezn4,l-zn4,lEzn4,l2

where zn4,l=z0l,z1l,⋯,zdzl,ν-1l is the calculated CSI feedback based on UE-side CSI compression before quantization of l-th layer, n4-th data instance, and zn4,l=z0l,z1l,⋯,zdzl,ν-1l is the corresponding CSI feedback provided by NW if exchanged CSI feedback is the floating-point values, or dequantization CSI feedback provided by NW if exchanged CSI feedback is the binary bit sequence  for l-th layer, n4-th data instance.

  - Average NMSE is calculated by

        - multiple data instances in a dataset

        - per layer

- For a dataset content, the following can be included:

  - Pairing ID

  - Performance targets for each layer and each configuration

  - Layer indication

  - Configuration of target CSI and CSI feedback

  - Data sample number(N) / Data sample type (one-to-one, or one-to-many)

  - N Data sample is consisting of {target CSI, CSI feedback}

- References

- 3GPP RP-251870, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements”, #108, June 9-13, 2025.

- TR 38843, Study on Artificial Intelligence (AI)/Machine Learning (ML)

- https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/Data_sharing/NR_AIML_air/CSI_compression/Datasets/R4_114.