3GPP TSG RAN WG1 #122bis                                                     R1-2506972 Prague, Czech, Oct 13th – 17th, 2025

Agenda Item:	9.1

Source:	Xiaomi

Title:	Discussion on UE features for AI/ML for NR Air Interface

Document for:	Discussion

- Introduction

In this contribution, we provide our views on the remaining issues of UE features for AI/ML based on the progress made in RAN1#122 meeting [1].

- AI/ML for beam management

According to the discussions in last RAN1 meetings, the following FGs/rows need to be supported for AI/ML based beam management.

Proposal 2-1: Support the following FGs/rows for AI/ML based beam management.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-2 | UE-side beam prediction for BM Case1 for inference | 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 for inference with UE-side model 3. Maximum number of inference report(s) configured for BM-Case1 per BWP 3a. Maximum number of inference report(s) configured for BM-Case1 across all CCs [4. Maximum number of inference report(s) activated for BM-Case1 per BWP] [4a. Maximum number of inference report(s) activated for BM-Case1 across all CCs] [5. Maximum number of inference report(s) triggered for BM-Case1 per BWP] [5a. Maximum number of inference report(s) triggered for BM-Case1 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A 7. Supported combinations of the number of resources for Set B and the number of resources for Set A 8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS 9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report 11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both} 12. Supported maximum number of predicted beams in each reporting instance FFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection 13. Supported number of occupied CPU 14. Supported number of occupied APU 15. Supported value of d for the relaxation of Z3 timeline 16. Supported value of d’ for the relaxation of Z’3 timeline 17. Index of the occupied APU pool | FFS | yes | n/a | UE-sided beam prediction for BM Case 1 for inference is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| 58. NR_AIML_air | 58-1-4 | UE-side beam prediction for BM Case2 for inference | 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 for inference with UE-side model 3. Maximum number of inference report(s) configured for BM-Case2 per BWP 3a. Maximum number of inference report(s) configured for BM-Case2 across all CCs [4. Maximum number of inference report(s) activated for BM-Case2 per BWP] [4a. Maximum number of inference report(s) activated for BM-Case2 across all CCs] [5. Maximum number of inference report(s) triggered for BM-Case2 per BWP] [5a. Maximum number of inference report(s) triggered for BM-Case2 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A 7. Supported combinations of the number of resources for Set B and the number of resources for Set A 8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS 9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report 11. Supported maximum number of predicted beams in each predicted time instance 12. Supported maximum number of predicted time instances 13. Supported maximum total number of reported predicted beams for predicted time instances in one report 19. Supported combinations of time gaps between predicted time instances and value of set B periodicity. 20. Supported BM-Case 2 sub usecase(s): e.g., setB-equals-to-setA, setB-subset-of-setA, setB-different-from-setA, or merged version(s) 21. supported number of occupied CPU 22. supported number of occupied APU 23. supported value of d for the relaxation of Z3 timeline 24. supported value of d’ for the relaxation of Z’3 timeline 17. Index of the occupied APU pool | FFS | yes | n/a | UE-side beam prediction for BM-Case2 for inference is not supported | FFS | FFS | FFS | FFS | FFS: candidate values for components | Optional with capability signalling |

- Conclusion

Based on the discussion, we propose the FGs need to be supported.

AI/ML for beam management

Proposal 2-1: Support the following FGs/rows for AI/ML based beam management.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-2 | UE-side beam prediction for BM Case1 for inference | 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 for inference with UE-side model 3. Maximum number of inference report(s) configured for BM-Case1 per BWP 3a. Maximum number of inference report(s) configured for BM-Case1 across all CCs [4. Maximum number of inference report(s) activated for BM-Case1 per BWP] [4a. Maximum number of inference report(s) activated for BM-Case1 across all CCs] [5. Maximum number of inference report(s) triggered for BM-Case1 per BWP] [5a. Maximum number of inference report(s) triggered for BM-Case1 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A 7. Supported combinations of the number of resources for Set B and the number of resources for Set A 8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS 9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report 11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both} 12. Supported maximum number of predicted beams in each reporting instance FFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection 13. Supported number of occupied CPU 14. Supported number of occupied APU 15. Supported value of d for the relaxation of Z3 timeline 16. Supported value of d’ for the relaxation of Z’3 timeline 17. Index of the occupied APU pool | FFS | yes | n/a | UE-sided beam prediction for BM Case 1 for inference is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| 58. NR_AIML_air | 58-1-4 | UE-side beam prediction for BM Case2 for inference | 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 for inference with UE-side model 3. Maximum number of inference report(s) configured for BM-Case2 per BWP 3a. Maximum number of inference report(s) configured for BM-Case2 across all CCs [4. Maximum number of inference report(s) activated for BM-Case2 per BWP] [4a. Maximum number of inference report(s) activated for BM-Case2 across all CCs] [5. Maximum number of inference report(s) triggered for BM-Case2 per BWP] [5a. Maximum number of inference report(s) triggered for BM-Case2 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A 7. Supported combinations of the number of resources for Set B and the number of resources for Set A 8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS 9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report 11. Supported maximum number of predicted beams in each predicted time instance 12. Supported maximum number of predicted time instances 13. Supported maximum total number of reported predicted beams for predicted time instances in one report 19. Supported combinations of time gaps between predicted time instances and value of set B periodicity. 20. Supported BM-Case 2 sub usecase(s): e.g., setB-equals-to-setA, setB-subset-of-setA, setB-different-from-setA, or merged version(s) 21. supported number of occupied CPU 22. supported number of occupied APU 23. supported value of d for the relaxation of Z3 timeline 24. supported value of d’ for the relaxation of Z’3 timeline 17. Index of the occupied APU pool | FFS | yes | n/a | UE-side beam prediction for BM-Case2 for inference is not supported | FFS | FFS | FFS | FFS | FFS: candidate values for components | Optional with capability signalling |

References

[1] R1-2506605,	Session Notes of AI 9.1,	3GPP RAN1# 122 meeting.