3GPP TSG RAN WG1 #122bis					           	                                                         R1-2507236

Prague, Czech, October 13th –17th, 2025

Agenda item:	9.1.1

Source:	Samsung

Title:	Remaining issues on UE features for AI/ML for NR air Interface

Document for:	Discussion and Decision

# Introduction

In this contribution, our views on capabilities for AI/ML for NR Air Interface are discussed.

# Discussion

## 2.1 AI/ML based beam management

For AI/ML based beam management, the following agreements are made [1].

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown 58. NR_AIML_air; 58-1-1; Increased number of reported RSs for beam management; 1. Support of reporting format for L1-RSRP measurements not including CRI/SSBRI other than one for the largest measured L1-RSRP in a reporting instance, if the number of reported L1-RSRPs is equal to the size of the measurement resource set. 2. Support of reporting format for L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB, if the number of reported L1-RSRPs is smaller than the size of the measurement resource set 3. Maximum number of M reported RSs, M>4; FFS; yes; n/a; Increased number of reported beams for beam management is not supported; FFS Per UE; FFS N/A; FFS N/A; FFS N/A; Component 3 candidate values: {6,8}; Optional with capability signalling. Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown 58. NR_AIML_air; 58-1-2; UE-side beam prediction for BM Case1 [for inference]; 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 [for inference] with UE-side model 3. Maximum number of inference report(s) configured for BM-Case1 per BWP 3a. Maximum number of inference report(s) configured for BM-Case1 across all CCs [4. Maximum number of inference report(s) activated for BM-Case1 per BWP] [4a. Maximum number of inference report(s) activated for BM-Case1 across all CCs] [5. Maximum number of inference report(s) triggered for BM-Case1 per BWP] [5a. Maximum number of inference report(s) triggered for BM-Case1 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] 8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS 9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report 11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both} 12. Supported maximum number of predicted beams in each reporting instanceFFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection 13. Supported number of occupied CPU 14. Supported number of occupied APU 15. Supported value of d for the relaxation of Z3 timeline 16. Supported value of d’ for the relaxation of Z’3 timeline 17. Index of the occupied APU pool; FFS; yes; n/a; UE-sided beam prediction for BM Case 1 [for inference] is not supported; FFS Per UE; FFS No; FFS No; FFS No; Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS} Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report} FFS: candidate values for components; Optional with capability signalling. Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown 58. NR_AIML_air; 58-1-3; UE-side beam prediction for BM Case1 with predicted RSRP [for inference]; 1. Support of beam prediction, reporting of predicted beam index and predicted RSRP, for BM-Case1 [for inference] 2. Supported maximum number of predicted beams with RSRP in each reporting instance; 58-1-2; yes; n/a; UE-side beam prediction for BM Case 1 with predicted RSRP [for inference] is not supported; FFS Per UE; FFS No; FFS No; FFS No; Component 2 candidate values: {1, 2, 3, 4}; Optional with capability signalling. Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown 58. NR_AIML_air; 58-1-4; UE-side beam prediction for BM Case2 [for inference]; 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 [for inference] with UE-side model 3. Maximum number of inference report(s) configured for BM-Case2 per BWP 3a. Maximum number of inference report(s) configured for BM-Case2 across all CCs [4. Maximum number of inference report(s) activated for BM-Case2 per BWP] [4a. Maximum number of inference report(s) activated for BM-Case2 across all CCs] [5. Maximum number of inference report(s) triggered for BM-Case2 per BWP] [5a. Maximum number of inference report(s) triggered for BM-Case2 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] 8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS 9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report 11. Supported maximum number of predicted beams in each predicted time instance 12. Supported maximum number of predicted time instances 13. Supported maximum total number of reported predicted beams for predicted time instances in one report 14. Supported combinations of supported value(s) of valid time duration for each predicted time instance and number of predicted beams for each value of valid time duration 21. supported number of occupied CPU 22. supported number of occupied APU 23. supported value of d for the relaxation of Z3 timeline 24. supported value of d’ for the relaxation of Z’3 timeline 2517. Index of the occupied APU pool; FFS; yes; n/a; UE-side beam prediction for BM-Case2 [for inference] is not supported; FFS Per UE; FFS No; FFS No; FFS No; Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS} Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report} FFS: candidate values for components; Optional with capability signalling. Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown 58. NR_AIML_air; 58-1-5; UE-side beam prediction for BM-Case2 with predicted RSRP [for inference]; 1. Support of beam prediction, reporting of predicted beams and predicted RSRP, for BM-Case2 (spatial and time domain beam prediction) [for inference]; 58-1-4; yes; n/a; UE-side beam prediction for BM-Case2 [for inference] is not supported; FFS Per UE; FFS No; FFS No; FFS No; Optional with capability signalling. Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed) 58. NR_AIML_air; 58-1-6; Performance monitoring for UE-sided model; 1. Support of performance monitoring with RS-PAI of AI/ML model for beam prediction. 2. Maximum total number of the configured CSI-RS resources for monitoring RS resource set 3. Maximum number of monitoring report(s) configured per BWP 4. Maximum number of monitoring report(s) configured across all CCs 5. Maximum number of monitoring occasions for RS-PAI calculation; FFS; yes; n/a; Performance monitoring for UE-sided model is not supported; FFS; FFS; FFS; FFS; FFS: component candidate values; Optional with capability signalling. Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown 58. NR_AIML_air; 58-1-7; Data collection for UE-side beam prediction [for BM case 1]; 1. Support of data collection for UE-side beam prediction [for BM case 1] 2. Support of SS/PBCH block and 1-port CSI-RS based RSRP measurements for measurement RS resource sets (Set B and Set A) for data collection 3. Supported sub-use cases 6: Supported maximum number of resources for Set B 7: Supported maximum number of resources for Set A 8. Support of SSB as RS type for Set B 9. Support of CSI-RS as RS type for Set B 10. Support of SSB as RS type for Set A 11. Support of CSI-RS as RS type for Set A; FFS; yes; n/a; Data collection for UE-side beam prediction is not supported [for BM case 1]; Per UE; N/A; N/A; N/A; Component 3 candidate values: {‘Set B equal to Set A’, ‘Set B subset of Set A’, ’Set B not a subset of Set A’} Component 6 candidate values: {4, 8, 16, 32, 64} Component 7 candidate values: {8, 16, 32, 64} Note: it is up to RAN2 whether this FG is merged into data collection FG defined by RAN2; Optional with capability signalling.

For FG 58-1-2, the capability reporting for inference report should be updated from “per band” to “per UE” since inference report is only feasible in bands in FR2. Same update should be applied for FG 58-1-3, FG 58-1-4 and FG 58-1-5.

Proposal 1. Update FG 58-1-2, FG 58-1-3, FG 58-1-4 and FG 58-1-5 from “per UE” to “per band”.

For FG 58-1-6, similar principle should be applied, which means that the capability signalling for FG 58-1-6 is per band.

Proposal 2. Update FG 58-1-6 to “per band”.

For AI/ML CSI-ReportConfigs, processing criteria is largely mirror legacy non-AI CSI-ReportConfigs behavior, but with a focus on CPU and APU budget. If the number of AI/ML CSI-ReportConfigs needing APUs on a single OFDM symbol exceeds available APUs, the UE won't update the lowest priority CSI-ReportConfigs. AI/ML CSI-ReportConfigs inference involves running the corresponding model on the NPU. This includes loading model weights and input into DRAM, performing the inference and obtaining output. Unlike legacy systems, where "CPU" capacity might limit CSI calculations, AI/ML model inference on the NPU introduces a new UE capability limit: the maximum number of simultaneous model inferences, regardless of APU occupancy.

To acknowledge this restriction at UE, one approach is to introduce a UE capability on the maximum number of active AI/ML CSI-ReportConfigs at any time. The active state can be defined using the APU occupancy window. That is, an AI/ML CSI-ReportConfigs is active during the time it occupies APU. If UE reports a value of Xactive for this capability, gNB shall not activate more than Xactive AI/ML CSI-ReportConfigs at any time.

Proposal 3: Define a UE capability on the maximum number of active AI/ML CSI-ReportConfig at any time.

- Add a component to FG 58-0-1 to define the maximum number of active AI/ML CSI-ReportConfigs across use cases

## 2.2 AI/ML based CSI prediction

Agreement

For CSI-PAI report, support

- OCPU =X, X is reported by UE capability

- For AP monitoring report, CPU occupancy starts from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the CSI-PAI report

- For SP monitoring report, CPU occupancy starts from the first symbol of the latest periodic/semi-persistent CSI-RS occasion of the inference report not later than CSI reference resource of the inference report, until the last symbol of the configured PUSCH carrying the CSI-PAI report

RAN1#122 has agreed the UE to report the value for the occupied CPU by A CSI report for monitoring.

This above can be reflected as component 5 of 58-3-5 as follows

Proposal 4: Introduce component 5 for 58-3-5 for the occupied number of CPUs for performance monitoring for CSI prediction model

| 58. NR_AIML_Air | 58-3-5 | Performance monitoring for CSI prediction model | 1. Supported of two performance metric SGCS 2. Support of one wideband frequency granularity SGCS per layer 3. Support of one configured time instance for N4>1 4. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 5. supported number of occupied CPU | 58-3-1 | Yes | n/a | Performance monitoring for CSI prediction model is not supported | Per Band and Per BC | N/A | N/A | N/A | Component 4 candidate values: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} Component 5 candidate values: {1,2} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Unlink CPU, the number of occupied APUs for CSI prediction does not linearly scale with the number of measured CSI-RS resources (K) or the number of prediction slot intervals (N4). Thus, it is better for the UE to report a single candidate value for the number of occupied APU irrespective of the values for K and N4. Thus we propose the following

Proposal 5: For component 14 of 58-3-1 and component 8 of 58-3-1, support the candidate values {1,2}  for the occupied number of CPUs for performance monitoring for CSI prediction model

| 58. NR_AIML_Air | 58-3-1 | CSI prediction for UE-sided inference when N4=1 | 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) [9. Value for CPU occupation, when P/SP-CSI-RS is configured for CMR] [10. Value for CPU occupation, when A-CSI-RS is configured for CMR] 11. Scaling factor for active resource counting Kp 12. supported value of t for the relaxation of Z and Z’ timeline 13. supported number of occupied CPU 14. supported number of occupied APU 15. Index of the occupied APU pool | 2-35 58-0-1 | yes | n/a | CSI prediction for N4=1 for inference is not supported |  |  |  |  | Component 14: candidate values {1,2,} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_Air | 58-3-2 | CSI prediction for UE-sided inference when N4>1 | 1. Support of CSI prediction for UE-sided inference when N4>1 2. Support for reporting predicted PMI with N4>1 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR 5. Support for the size of DD-basis, N4>1 76. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting 7. Supported number of occupied CPU 8. Supported number of occupied APU 9. Index of the occupied APU pool | 58-3-1 | yes | n/a | CSI prediction for N4>1 for inference is not supported | [Per band and Per BC] | FFS N/A | FFS N/A | N/A | Component 3 candidate values: a. {1,2,4,8} b. {4,8,12,16,24,32} c. {2,3,4 … 64} d. {4, …, 256} Component 6 candidate values: a. {1,2,4,8} b. {4,8,12,16,24,32} c. {4,8,12} d.{4, …, 256} Component 8: candidate values {1,2,} | Optional with capability signalling |

# Conclusion

The proposals made in this contribution are summarized below.

Proposal 1. Update FG 58-1-2, FG 58-1-3, FG 58-1-4 and FG 58-1-5 from “per UE” to “per band”.

Proposal 2. Update FG 58-1-6 to “per band”.

Proposal 3: Define a UE capability on the maximum number of active AI/ML CSI-ReportConfig at any time.

- Add a component to FG 58-0-1 to define the maximum number of active AI/ML CSI-ReportConfigs across use cases

Proposal 4: Introduce component 5 for 58-3-5 for the occupied number of CPUs for performance monitoring for CSI prediction model

| 58. NR_AIML_Air | 58-3-5 | Performance monitoring for CSI prediction model | 1. Supported of two performance metric SGCS 2. Support of one wideband frequency granularity SGCS per layer 3. Support of one configured time instance for N4>1 4. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 5. supported number of occupied CPU | 58-3-1 | Yes | n/a | Performance monitoring for CSI prediction model is not supported | Per Band and Per BC | N/A | N/A | N/A | Component 4 candidate values: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} Component 5 candidate values: {1,2} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 5: For component 14 of 58-3-1 and component 8 of 58-3-1, support the candidate values {1,2}  for the occupied number of CPUs for performance monitoring for CSI prediction model

| 58. NR_AIML_Air | 58-3-1 | CSI prediction for UE-sided inference when N4=1 | 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) [9. Value for CPU occupation, when P/SP-CSI-RS is configured for CMR] [10. Value for CPU occupation, when A-CSI-RS is configured for CMR] 11. Scaling factor for active resource counting Kp 12. supported value of t for the relaxation of Z and Z’ timeline 13. supported number of occupied CPU 14. supported number of occupied APU 15. Index of the occupied APU pool | 2-35 58-0-1 | yes | n/a | CSI prediction for N4=1 for inference is not supported |  |  |  |  | Component 14: candidate values {1,2,} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_Air | 58-3-2 | CSI prediction for UE-sided inference when N4>1 | 1. Support of CSI prediction for UE-sided inference when N4>1 2. Support for reporting predicted PMI with N4>1 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR 5. Support for the size of DD-basis, N4>1 76. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting 7. Supported number of occupied CPU 8. Supported number of occupied APU 9. Index of the occupied APU pool | 58-3-1 | yes | n/a | CSI prediction for N4>1 for inference is not supported | [Per band and Per BC] | FFS N/A | FFS N/A | N/A | Component 3 candidate values: a. {1,2,4,8} b. {4,8,12,16,24,32} c. {2,3,4 … 64} d. {4, …, 256} Component 6 candidate values: a. {1,2,4,8} b. {4,8,12,16,24,32} c. {4,8,12} d.{4, …, 256} Component 8: candidate values {1,2,} | Optional with capability signalling |

# References

- R1-2506605, Session Notes on 9.1.1, RAN1#122.