3GPP TSG RAN WG1#122bis                                                              R1-2507436

Prague, Czech, Oct 13th – 17th, 2025

Agenda Item:	8.2

Source:	Lenovo

Title:	Maintenance on NR MIMO Phase 5

Document for:	Discussion

Introduction

In this contribution, we provide our views on the remaining issues on NR MIMO Phase 5.

Discussion

Enhancements for UE-initiated/event-driven beam management

L1-RSRP reporting

Regarding the L1-RSRP report format for different events, if RRC parameter enabledCurrentBeamReport is configured for event2 or event7, the L1-RSRP for the current beam is reported by a differential RSRP as specified in TS38.212, which is aligned with the agreement achieved in RAN1 by an assumption that the quality of the current beam doesn’t have the highest L1-RSRP when event2 or event7 is triggered. However, the CSI reference resource for the beam report for UEIBM is determined by the triggering DCI for mode A or by the second PUSCH carrying the beam report for mode B. Given that, determining CSI reference resource for the UE to obtain the L1-RSRP for the beam report may be later than the slot for the first PUCCH transmission to trigger the beam report. Thus, it should be possible that the current beam has the highest L1-RSRP in the determined CSI reference resource in the beam report. The current differential RSRP scheme for current beam cannot be used for this case. The same situation may happen for event1, for which the absolute L1-RSRP for the current beam is directly quantized to a 7-bits value other than a differential value. Thus, we propose to extend this scheme for event2 and event7.

Proposal 1: For the L1-RSRP report for event2 and event7, the L1-RSRP for the current beam is absolute L1-RSRP not a differential RSRP.

DCI for PUSCH scheduling for UEI beam report in mode A

For mode A of a UE initiated beam report, the gNB can send a DCI format 0_1/0_2 if a UE has transmitted a UEIRI in the first PUCCH resource to schedule a PUSCH for the beam report transmission, where the CSI request field indicates a CSI trigger state associated with UEI beam report configuration(s). In Rel-18, DCI format 0_1/0_2 can trigger one or more aperiodic CSI report by using a DCI scrambled by C-RNTI, and can activate one or more semi-persistent CSI reports by using a DCI scrambled by SP-CSI-RNTI. For the prior case, the CSI request field codepoint is mapped with a CSI-AperiodicTriggerState, while for the latter case, the CSI request field codepoint is mapped with SP-CSI triggering state. Either of the DCI can schedule a corresponding PUSCH. During the whole discussion, we assume that the legacy DCI for aperiodic CSI triggering is used in the step 2 of mode A procedure of UEIBM. Thus, it should be restricted that the DCI format 0_1/0_2 in mode A should be scrambled by C-RNTI.

DCI format 0_1/0_2 in step 2 of mode A procedure of UEIBM in response to the UEIRI is scrambled by C-RNTI.

CRI/SSBRI k definition in a CSI report for UEIBM

Reason for change: CRI and SSBRI in CSI report for UEIBM are not defined.

Summary of changes: Introduce the definition of CRI and SSBRI in CSI report for UEIBM.

Consequences if not approved: UE does not know how to determine the value of CRI or SSBRI in a CSI report for UEIBM.

Adopt the following TP for TS38.214V19.1.0

<omitted text> 5.2.1.5.4.1a UE Initiated CSI reporting for event 2 <omitted text> After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission. CRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet configured for newBeamResourceSet. SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet configured for newBeamResourceSet. For each CSI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRP and, when PresenceOfConditionMetIndicator is configured, condition met indicator indicating whether the reference signal indicated by reported CRI or SSBRI triggers the UEIRI transmission, and, when enabledCurrentBeamReport is configured, the differential L1-RSRP corresponding to the reference signal in the indicated TCI state, or to the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state. The UE sends the CSI report <omitted text> 5.2.1.5.4.1b UE Initiated CSI reporting for event 1 <omitted text> After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet. CRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet configured for newBeamResourceSet. SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet configured for newBeamResourceSet. For each CRI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRPs, and, when enabledCurrentBeamReport is configured, the absolute L1-RSRPs, corresponding to the reference signal in the indicated TCI state, or to the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state. The UE sends the CSI report <omitted text> 5.2.1.5.4.1c UE Initiated CSI reporting for event 7 <omitted text> After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212] in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission. CRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet configured for newBeamResourceSet. SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet configured for newBeamResourceSet. For each CRI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRP and, when PresenceOfConditionMetIndicator is configured a condition met indicator indicating whether the reference signal indicated by reported CRI or SSBRI triggers the UEIRI transmission and, when enabledCurrentBeamReport is configured, the differential L1-RSRP corresponding to the reference signal with the valueOfQ highest L1-RSRP out of the activated TCI state reference signals, or to the SS/PBCH block which is QCLed with the reference signal with the valueOfQ highest L1-RSRP out of the activated TCI state reference signals. The UE sends the CSI report <omitted text>

CSI enhancements

CSI enhancements for up to 128 CSI-RS ports

For legacy eType-II Doppler codebook, the pair (Z,Z') corresponding to the timeline is defined as follows:

-   (Z2+14K-1m,Z2'), with (Z2,Z2') of Table 5.4-2 in TS 38.214 [1], if the CSI report is configured with N4=1, codebookType is set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is aperiodic with K CSI-RS resources, or

-	(Z2+w,Z2'), with (Z2,Z2') of Table 5.4-2 in TS 38.214, if the CSI report is configured with N4=1, codebookType is set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is periodic or semi-persistent with a single CSI-RS resource, or

-	(Z2+14K-1m,Z2') or (Z2+14K-1m+Z2',2Z2'), according to UE reported capability, with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4>1, codebookType is set to 'typeII-Doppler-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is aperiodic with K CSI-RS resources, or

-	(Z2+w,Z2') or (Z2+w+Z2',2Z2'), according to UE reported capability, with (Z2,Z2') of table 5.4-2 in TS 38.214, if the CSI   report is configured with N4>1, codebookType is set to 'typeII-Doppler-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is periodic or semi-persistent with a single CSI-RS resource,

Based on legacy design, additional time of 14K-1m OFDM symbols or w (w=14∙Kp-1∙d or w=14∙Kp∙d are introduced for Z corresponding to aperiodic and periodic/semi-persistent CSI-RS transmission respectively, corresponding to no more than 32 ports. In RAN1#117 [2], it was agreed that for Capability 2 of the timeline of Rel-19 Type-I SP and Type-II codebook refinements for 48, 64, and 128 CSI-RS ports via aggregating multiple CSI-RS resources in the CSI-RS resource set as channel measurement resources, the (Z,Z') pair is determined via scaling the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources, based on the following agreement:

Agreement For the Rel-19 Type-I SP and Type-II codebook refinements for 48, 64, and 128 CSI-RS ports via aggregating K>1 CSI-RS resources, regarding timeline, introduce two UE capabilities: - Capability 1: Reuse legacy Z/Z’ values - Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources FFS: CPU occupation and active resource counting Note: - The legacy timeline Z/Z’ for Type-I corresponds to Z1/Z1’ in Table 5.4-2 of TS38.214 for Type-I WB SP-CSI with at most 4 CSI-RS ports in a single resource without CRI, and Z2/Z2’ for other Type-I cases - The legacy timeline Z/Z’ for Type-II corresponds to Z2/Z2’

In our understanding, additional time in case of more than 32 ports is needed for CSI processing time but not CSI-RS transmission time. If scaling is also used for CSI-RS transmission time 14K-1m or w, the timeline will be too loose and CSI feedback delay is too large, leading to degrading the system performance. The scaling factor, i.e. ceil(P/32), is not yet reflected in current specification, and hence the timeline for capability 2 will be not correct without it. Thus, we propose the following TP to include scaling factor for timeline of capability 2.

Support the TP in Table 1 for Capability 2 timeline for eType-II Doppler codebook using more than 32 ports, via the scaling factor, i.e. P32, in the timeline formula.

Adopt the following TP for TS38.214V19.1.0 for UE capability 2 timeline

- 5.4 UE CSI computation time … Z=maxm=0,…,M-1(Z(m))⁡ and Z'=maxm=0,…,M-1(Z'(m)), where M is the number of updated CSI report(s) according to Clause 5.2.1.6, (Z(m),Z'(m)) corresponds to the m-th updated CSI report and is defined as - … - (Z2+14K-1m,Z2'), with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4=1, codebookType is set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is aperiodic with K CSI-RS resources, or - (Z2+w,Z2'), with (Z2,Z2') of table 5.4-2, where w=56⋅(KP –1) or 56⋅KP symbols, according to the reported UE capability, where the value of KP ∈{1,2,4} is indicated by UE capability, if the CSI report is configured with N4=1, codebookType is set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is periodic or semi-persistent with a single CSI-RS resource, or - (Z2+14K-1m,Z2') or (Z2+14K-1m+Z2',2Z2'), according to UE reported capability, with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4>1, codebookType is set to 'typeII-Doppler-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is aperiodic with K CSI-RS resources, or - (Z2+w,Z2') or (Z2+w+Z2',2Z2'), according to UE reported capability, with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4>1, codebookType is set to 'typeII-Doppler-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is periodic or semi-persistent with a single CSI-RS resource, or - (Z2,Z2') or Z2⋅P32,Z2'⋅P32, according to UE reported capability 1 or 2, respectively, with (Z2,Z2') of table 5.4-2, if codebookType is set to 'typeI-SinglePanel-r19', 'typeI-MultiPanel-r19', 'eTypeII-r19' or 'typeII-FePortSelection-r19', where P is the total number of CSI-RS ports across the K configured CSI-RS resources for channel measurement, or - Z2+14KDOPP-1m,Z2' or Z2⋅P32+14KDOPP-1m,Z2'⋅P32,according to UE reported capability 1 or 2, respectively, with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4=1, codebookType is set to 'typeII-Doppler-r19' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is aperiodic with KDOPP CSI-RS resource groups, where P is the total number of CSI-RS ports across the K configured CSI-RS resources in one CSI-RS resource group, or - Z2+w,Z2' or Z2⋅P32+w,Z2'⋅P32 according to UE reported capability 1 or 2, respectively, with (Z2,Z2') of table 5.4-2, where w=56⋅(KP –1) or 56⋅KP symbols, according to the reported UE capability, where the value of KP ∈{1,2,4} is indicated by UE capability, if the CSI report is configured with N4=1, codebookType is set to 'typeII-Doppler-r19' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is periodic or semi-persistent, and P is the total number of CSI-RS ports across the K configured CSI-RS resources in one CSI-RS resource group, or - (Z2+14KDOPP-1m,Z2') or (Z2⋅P32+14KDOPP-1m+Z2'⋅P32,2Z2'⋅P32), according to the reported UE capability, with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4>1, codebookType is set to 'typeII-Doppler-r19' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is aperiodic with KDOPP CSI-RS resource groups, where P is the total number of CSI-RS ports across the K configured CSI-RS resources in one CSI-RS resource group, or - (Z2+w,Z2') or (Z2⋅P32+w+Z2'⋅P32,2Z2'⋅P32), according to the reported UE capability, with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4>1, codebookType is set to 'typeII-Doppler-r19' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is periodic or semi-persistent, where P is the total number of CSI-RS ports across the K configured CSI-RS resources in one CSI-RS resource group, or - … - (Z2,Z2') of table 5.4-2 otherwise. - µ of table 5.4-1 and table 5.4-2 corresponds to the min (µPDCCH, µCSI-RS, µUL) where the µPDCCH corresponds to the subcarrier spacing of the PDCCH with which the DCI was transmitted and µUL corresponds to the subcarrier spacing of the PUSCH with which the CSI report is to be transmitted and µCSI-RS corresponds to the minimum subcarrier spacing of the aperiodic CSI-RS triggered by the DCI Table 5.4-1: CSI computation delay requirement 1 Z1 [symbols]. Z1; Z'1. 0; 10; 8. 1; 13; 11. 2; 25; 21. 3; 43; 36. Table 5.4-2: CSI computation delay requirement 2 Z1 [symbols]; Z2 [symbols]; Z3 [symbols]. Z1; Z'1; Z2; Z'2; Z3; Z'3. 0; 22; 16; 40; 37; 22; X0. 1; 33; 30; 72; 69; 33; X1. 2; 44; 42; 141; 140; min(44, X3X2+ KB1); X2. 3; 97; 85; 152; 140; min(97, X3+ KB2); X3. 5; 388; 340; 608; 560; min(388, X5+ KB3); X5. 6; 776; 680; 1216; 1120; min(776, X6+ KB4); X6.

CJT enhancements under non-ideal synchronization

- In RAN1#119, it was agreed that when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, the 1-bit indicator per CSI trigger state would apply to all, e.g., NR, Rel-18 Type-II CJT CSI reports, linked with the CJTC Dd report. In our understanding, as long as the NR CSI reports corresponding to Rel-18 Type-II CJT codebook are associated with the same CSI report configuration, a common trigger suffices.

For separate triggering of Rel-19 CJTC Dd report type with Rel-18 Type-II CJT report type, the linkage of the two report types spans NR CSI reports corresponding to the same CSI report configuration.

- Finally, it was also discussed to extend this framework to Rel-15 Type-I MP CB. In our opinion, this is not needed, since in multi-panel deployments the panels are expected to be co-located with close to ideal backhaul, and the PDP of the channel corresponding to each of the panels should be strongly correlated.

CSI calculation based on Rel-15 Type-I MP CB based on CJTC DO reporting is not supported.

Conclusion

In this contribution, we provide the following proposals to discuss.

Enhancements for UE-initiated/event-driven beam management

Proposal 1: For the L1-RSRP report for event2 and event7, the L1-RSRP for current beam is absolute L1-RSRP not a differential RSRP.

Proposal 2: DCI format 0_1/0_2 in step 2 of mode A procedure of UEIBM in response to the UEIRI is scrambled by C-RNTI.

Proposal 3: Adopt the following TP for TS38.214 v19.1.0 for UEIBM

<omitted text> 5.2.1.5.4.1a UE Initiated CSI reporting for event 2 <omitted text> After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission. CRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet configured for newBeamResourceSet. SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet configured for newBeamResourceSet. For each CSI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRP and, when PresenceOfConditionMetIndicator is configured, condition met indicator indicating whether the reference signal indicated by reported CRI or SSBRI triggers the UEIRI transmission, and, when enabledCurrentBeamReport is configured, the differential L1-RSRP corresponding to the reference signal in the indicated TCI state, or to the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state. The UE sends the CSI report <omitted text> 5.2.1.5.4.1b UE Initiated CSI reporting for event 1 <omitted text> After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet. CRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet configured for newBeamResourceSet. SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet configured for newBeamResourceSet. For each CRI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRPs, and, when enabledCurrentBeamReport is configured, the absolute L1-RSRPs, corresponding to the reference signal in the indicated TCI state, or to the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state. The UE sends the CSI report <omitted text> 5.2.1.5.4.1c UE Initiated CSI reporting for event 7 <omitted text> After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212] in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission. CRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet configured for newBeamResourceSet. SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet configured for newBeamResourceSet. For each CRI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRP and, when PresenceOfConditionMetIndicator is configured a condition met indicator indicating whether the reference signal indicated by reported CRI or SSBRI triggers the UEIRI transmission and, when enabledCurrentBeamReport is configured, the differential L1-RSRP corresponding to the reference signal with the valueOfQ highest L1-RSRP out of the activated TCI state reference signals, or to the SS/PBCH block which is QCLed with the reference signal with the valueOfQ highest L1-RSRP out of the activated TCI state reference signals. The UE sends the CSI report <omitted text>

CSI enhancements

Proposal 4: Support the TP in Table 1 for Capability 2 timeline for eType-II Doppler codebook using more than 32 ports, via the scaling factor, i.e. P32, in the timeline formula.

Proposal 5: For separate triggering of Rel-19 CJTC Dd report type with Rel-18 Type-II CJT report type, the linkage of the two report types spans NR CSI reports corresponding to the same CSI report configuration.

Proposal 6: CSI calculation based on Rel-15 Type-I MP CB based on CJTC DO reporting is not supported.

Proposal 7: Adopt the following TP for TS38.214V19.1.0 for UE capability 2 timeline.

- 5.4 UE CSI computation time … Z=maxm=0,…,M-1(Z(m))⁡ and Z'=maxm=0,…,M-1(Z'(m)), where M is the number of updated CSI report(s) according to Clause 5.2.1.6, (Z(m),Z'(m)) corresponds to the m-th updated CSI report and is defined as - … - (Z2+14K-1m,Z2'), with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4=1, codebookType is set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is aperiodic with K CSI-RS resources, or - (Z2+w,Z2'), with (Z2,Z2') of table 5.4-2, where w=56⋅(KP –1) or 56⋅KP symbols, according to the reported UE capability, where the value of KP ∈{1,2,4} is indicated by UE capability, if the CSI report is configured with N4=1, codebookType is set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is periodic or semi-persistent with a single CSI-RS resource, or - (Z2+14K-1m,Z2') or (Z2+14K-1m+Z2',2Z2'), according to UE reported capability, with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4>1, codebookType is set to 'typeII-Doppler-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is aperiodic with K CSI-RS resources, or - (Z2+w,Z2') or (Z2+w+Z2',2Z2'), according to UE reported capability, with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4>1, codebookType is set to 'typeII-Doppler-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is periodic or semi-persistent with a single CSI-RS resource, or - (Z2,Z2') or Z2⋅P32,Z2'⋅P32, according to UE reported capability 1 or 2, respectively, with (Z2,Z2') of table 5.4-2, if codebookType is set to 'typeI-SinglePanel-r19', 'typeI-MultiPanel-r19', 'eTypeII-r19' or 'typeII-FePortSelection-r19', where P is the total number of CSI-RS ports across the K configured CSI-RS resources for channel measurement, or - Z2+14KDOPP-1m,Z2' or Z2⋅P32+14KDOPP-1m,Z2'⋅P32,according to UE reported capability 1 or 2, respectively, with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4=1, codebookType is set to 'typeII-Doppler-r19' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is aperiodic with KDOPP CSI-RS resource groups, where P is the total number of CSI-RS ports across the K configured CSI-RS resources in one CSI-RS resource group, or - Z2+w,Z2' or Z2⋅P32+w,Z2'⋅P32 according to UE reported capability 1 or 2, respectively, with (Z2,Z2') of table 5.4-2, where w=56⋅(KP –1) or 56⋅KP symbols, according to the reported UE capability, where the value of KP ∈{1,2,4} is indicated by UE capability, if the CSI report is configured with N4=1, codebookType is set to 'typeII-Doppler-r19' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is periodic or semi-persistent, and P is the total number of CSI-RS ports across the K configured CSI-RS resources in one CSI-RS resource group, or - (Z2+14KDOPP-1m,Z2') or (Z2⋅P32+14KDOPP-1m+Z2'⋅P32,2Z2'⋅P32), according to the reported UE capability, with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4>1, codebookType is set to 'typeII-Doppler-r19' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is aperiodic with KDOPP CSI-RS resource groups, where P is the total number of CSI-RS ports across the K configured CSI-RS resources in one CSI-RS resource group, or - (Z2+w,Z2') or (Z2⋅P32+w+Z2'⋅P32,2Z2'⋅P32), according to the reported UE capability, with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4>1, codebookType is set to 'typeII-Doppler-r19' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is periodic or semi-persistent, where P is the total number of CSI-RS ports across the K configured CSI-RS resources in one CSI-RS resource group, or - … - (Z2,Z2') of table 5.4-2 otherwise. - µ of table 5.4-1 and table 5.4-2 corresponds to the min (µPDCCH, µCSI-RS, µUL) where the µPDCCH corresponds to the subcarrier spacing of the PDCCH with which the DCI was transmitted and µUL corresponds to the subcarrier spacing of the PUSCH with which the CSI report is to be transmitted and µCSI-RS corresponds to the minimum subcarrier spacing of the aperiodic CSI-RS triggered by the DCI Table 5.4-1: CSI computation delay requirement 1 Z1 [symbols]. Z1; Z'1. 0; 10; 8. 1; 13; 11. 2; 25; 21. 3; 43; 36. Table 5.4-2: CSI computation delay requirement 2 Z1 [symbols]; Z2 [symbols]; Z3 [symbols]. Z1; Z'1; Z2; Z'2; Z3; Z'3. 0; 22; 16; 40; 37; 22; X0. 1; 33; 30; 72; 69; 33; X1. 2; 44; 42; 141; 140; min(44, X3X2+ KB1); X2. 3; 97; 85; 152; 140; min(97, X3+ KB2); X3. 5; 388; 340; 608; 560; min(388, X5+ KB3); X5. 6; 776; 680; 1216; 1120; min(776, X6+ KB4); X6.

References

- TR38.214v19.1.0