3GPP TSG RAN WG1 #122bis		R1-2507516

Prague, Czech, Oct 13th – 17th, 2025

Agenda Item:	8.5

Source:	Google

Title:	Maintenance of Network Energy Saving

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide some discussion on the maintenance of network energy saving.

# On-demand SSB SCell operation

## CPU counting for OD-SSB

It has been agreed that the OD-SSB can be configured for L1-RSRP/L1-SINR measurement and report. Currently the CPU occupied time is defined as follows. When OD-SSB is configured, it is unclear whether the “SSB” is all the SSB configured by RRC or the activated SSBs. Different interpretations could result in different CPU occupancy duration. One possible way is to clarify that the SSB for CPU occupancy duration counting is the activated SSB, i.e., SSB being transmitted.

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', or a CSI report with LTM-CSI-ReportConfig, or reportQuantity not set to 'none-bm-r19' or 'none-csi-r19', the CPU(s) (including OCPU,1and/or OCPU,x, for CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', or ‘csi-pai-r19’, or ‘rs-pai-r19’, and CSI reports configured with the higher layer parameter [RRC_name-r19]) are occupied for a number of OFDM symbols as follows: - A periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report, a semi-persistent CSI report on PUSCH configured with reportQuantity set to 'csi-pai-r19', and a semi-persistent CSI report on PUSCH configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18') occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource, or each CSI-RS/CSI-IM resource associated with all configured sub-configurations for periodic CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, or each CSI-RS/CSI-IM resource associated with all activated/triggered sub-configurations for semi-persistent CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, for channel or interference measurement, respective latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the report.

Proposal 1: Endorse the following TP for 38.214

- Reason for change: It is unclear whether the “SSB” used for CPU occupancy time counting is the configured SSB or activated SSB. Different interpretation would result in different CPU occupancy time.

- Summary of the change: Clarify the SSB used for CPU occupancy time counting is the activated SSB.

- Consequence if the change is not endorsed: Unclear CPU occupancy time counting when OD-SSB is configured for L1-RSRP/L1-SINR report

5.2.1.6 CSI processing criteria <omitted text> For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', or a CSI report with LTM-CSI-ReportConfig, or reportQuantity not set to 'none-bm-r19' or 'none-csi-r19', the CPU(s) (including OCPU,1and/or OCPU,x, for CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', or ‘csi-pai-r19’, or ‘rs-pai-r19’, and CSI reports configured with the higher layer parameter [RRC_name-r19]) are occupied for a number of OFDM symbols as follows: - A periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report, a semi-persistent CSI report on PUSCH configured with reportQuantity set to 'csi-pai-r19', and a semi-persistent CSI report on PUSCH configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18') occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource (the SSB is being transmitted), or each CSI-RS/CSI-IM resource associated with all configured sub-configurations for periodic CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, or each CSI-RS/CSI-IM resource associated with all activated/triggered sub-configurations for semi-persistent CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, for channel or interference measurement, respective latest CSI-RS/CSI-IM/SSB occasion (the SSB is being transmitted) no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the report.

## OD-SSB for other CSI report

It has been agreed that the OD-SSB can be configured for L1-RSRP/L1-SINR measurement and report, which is triggered by the NW. It is unclear whether the OD-SSB can be configured for other types of CSI report. Without any clarification, it can be interpreted as the OD-SSB can be configured for other types of CSI report as well.

Proposal 2: Endorse the following TP for 38.214

- Reason for change: It has been agreed that the OD-SSB can be configured for L1-RSRP/L1-SINR report, but current spec fails to clarify whether it can be configured for other types of report.

- Summary of the change: Clarify only Always-on SSB can be configured/measured for other types of report to be aligned with the RAN1 agreement.

- Consequence if the change is not endorsed: It is unclear whether the OD-SSB can be configured for other types of report or not

5.2.1.4 Reporting configurations <omitted text> For a UE configured with od-ssb-config on a SCell and for CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'ssb-Index-RSRP', 'ssb-Index-SINR', 'ssb-Index-RSRP- Index', or 'ssb-Index-SINR- Index ' and not configured with eventType: - if the UE is not provided absoluteFrequencySSB for the SCell, the CSI report configuration is associated with the SS/PBCH block configured by od-ssb-config and the UE reports SSBRI based on SSB-index corresponding to the currently transmitted SS/PBCH block. - if the UE is provided absoluteFrequencySSB for the SCell, the CSI report configuration is associated with both the SS/PBCH block configured by od-ssb-config and the SS/PBCH block provided by absoluteFrequencySSB and the UE reports SSBRI based on SSB-index corresponding to the currently transmitted SS/PBCH block(s) that may be the one configured by od-ssb-config and/or provided by absoluteFrequencySSB based on measurement requirements defined in [11, TS 38.133]. - The UE reports SSBRI based on SSB-index corresponding to the currently transmitted SS/PBCH block, where the SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet. - For other CSI report, if the UE is not provided absoluteFrequencySSB for the SCell, it does not expect the SS/PBCH block configured by od-ssb-config to be configured in the corresponding CSI report configuration, and if the UE is provided absoluteFrequencySSB for the SCell, UE measures the SS/PBCH block provided by absoluteFrequencySSB configured.

# Adaptation of common signal/channel transmissions

## Monitoring of the DCI for SSB adaptation

For SSB adaptation, it is specified that the DCI based on ssbPeriodicityIndication-RNTI can be used to indicate the periodicity for the SSB. It is unclear whether the UE should monitor such DCI during the inactive period for the cell DTX or not. One possible way is to reuse the same mechanism as other types of group-cast DCI, that the UE should not monitor the DCI based on ssbPeriodicityIndication-RNTI during the inactive period of cell DTX.

Proposal 3: Clarify that the UE should not monitor the DCI based on ssbPeriodicityIndication-RNTI during the inactive period for the cell DTX.

## Interaction with SBFD

For SBFD, the NW can also configure additional PRACH resources for SBFD symbols/slots. Then for PRACH adaptation for NES, it should be clarified that whether the additional PRACH resources can be applicable to SBFD symbols/slots. One possible way is to configure two additional PRACH resource sets for NES, where one is not applicable to the SBFD symbols/slots and the other one is applicable to the SBFD symbols/slots, with regard to UEs that support NES only and UEs that support both SBFD and NES. But the first step should be to clarify whether the PRACH adaptation and the SBFD can be configured simultaneously or not.

Proposal 4: Clarify whether the PRACH adaptation and the SBFD can be configured simultaneously or not.

# Conclusion

In this contribution, we provided discussion on adaptation of common signals. Based on the discussion, the following proposals are provided.

OD-SSB

Proposal 1: Endorse the following TP for 38.214

- Reason for change: It is unclear whether the “SSB” used for CPU occupancy time counting is the configured SSB or activated SSB. Different interpretation would result in different CPU occupancy time.

- Summary of the change: Clarify the SSB used for CPU occupancy time counting is the activated SSB.

- Consequence if the change is not endorsed: Unclear CPU occupancy time counting when OD-SSB is configured for L1-RSRP/L1-SINR report

5.2.1.6 CSI processing criteria <omitted text> For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', or a CSI report with LTM-CSI-ReportConfig, or reportQuantity not set to 'none-bm-r19' or 'none-csi-r19', the CPU(s) (including OCPU,1and/or OCPU,x, for CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', or ‘csi-pai-r19’, or ‘rs-pai-r19’, and CSI reports configured with the higher layer parameter [RRC_name-r19]) are occupied for a number of OFDM symbols as follows: - A periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report, a semi-persistent CSI report on PUSCH configured with reportQuantity set to 'csi-pai-r19', and a semi-persistent CSI report on PUSCH configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18') occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource (the SSB is being transmitted), or each CSI-RS/CSI-IM resource associated with all configured sub-configurations for periodic CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, or each CSI-RS/CSI-IM resource associated with all activated/triggered sub-configurations for semi-persistent CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, for channel or interference measurement, respective latest CSI-RS/CSI-IM/SSB occasion (the SSB is being transmitted) no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the report.

Proposal 2: Endorse the following TP for 38.214

- Reason for change: It has been agreed that the OD-SSB can be configured for L1-RSRP/L1-SINR report, but current spec fails to clarify whether it can be configured for other types of report.

- Summary of the change: Clarify only Always-on SSB can be configured/measured for other types of report to be aligned with the RAN1 agreement.

- Consequence if the change is not endorsed: It is unclear whether the OD-SSB can be configured for other types of report or not

5.2.1.4 Reporting configurations <omitted text> For a UE configured with od-ssb-config on a SCell and for CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'ssb-Index-RSRP', 'ssb-Index-SINR', 'ssb-Index-RSRP- Index', or 'ssb-Index-SINR- Index ' and not configured with eventType: - if the UE is not provided absoluteFrequencySSB for the SCell, the CSI report configuration is associated with the SS/PBCH block configured by od-ssb-config and the UE reports SSBRI based on SSB-index corresponding to the currently transmitted SS/PBCH block. - if the UE is provided absoluteFrequencySSB for the SCell, the CSI report configuration is associated with both the SS/PBCH block configured by od-ssb-config and the SS/PBCH block provided by absoluteFrequencySSB and the UE reports SSBRI based on SSB-index corresponding to the currently transmitted SS/PBCH block(s) that may be the one configured by od-ssb-config and/or provided by absoluteFrequencySSB based on measurement requirements defined in [11, TS 38.133]. - The UE reports SSBRI based on SSB-index corresponding to the currently transmitted SS/PBCH block, where the SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet. - For other CSI report, if the UE is not provided absoluteFrequencySSB for the SCell, it does not expect the SS/PBCH block configured by od-ssb-config to be configured in the corresponding CSI report configuration, and if the UE is provided absoluteFrequencySSB for the SCell, UE measures the SS/PBCH block provided by absoluteFrequencySSB configured.

Adaptation of common signals

Proposal 3: Clarify that the UE should not monitor the DCI based on ssbPeriodicityIndication-RNTI during the inactive period for the cell DTX.

Proposal 4: Clarify whether the PRACH adaptation and the SBFD can be configured simultaneously or not.