3GPP TSG-RAN1 Meeting #122bis						            R1-2507571

Prague, Czech Republic, 13 – 17 October 2025

Agenda Item:	   11.2

Source:	ESA, Thales, Viasat, Eutelsat

Title:	Satellite Access Node Characteristics for the Evaluation Assumptions for 6GR air interface

WI code: 	FS_6G_Radio

Release:	Rel-20

Document for:	Discussion

# Introduction

The 6G Radio Study Item approved at RAN#108 (RP-251881) [1] and revised at RAN#109 in RP-252912 [2] highlights the importance of Non-Terrestrial Network (NTN) support in 6G from the outset.

Preliminary discussions on evaluation assumptions for the 6GR air interface were held at RAN1#122 [3]. These discussions covered models, scenarios, parameters and methodologies; evaluation metrics and criteria for technology proposals; energy efficiency; and sensing performance, including potential extensions to the channel model.

In this contribution, we provide inputs on simulation assumptions for NTN, specifically related to the satellite access node characteristics (e.g., EIRP, antenna size, beam diameter, etc.).

# Recap of agreements from RAN #122 meeting

RAN #122 made the following agreements:

Agreement

- The deployment scenarios in TR38.914 should be considered for evaluation assumption

The common evaluation assumptions including the antenna modelling, general system-level simulation assumptions (including the carrier frequency, bandwidth and subcarrier spacing used for link-level simulation) for the deployment scenarios in TR38.914, link budget and traffic models will be discussed in AI 11.2

- Other assumptions including for link-level simulation specific to each technical topic will be separately discussed under each individual agenda.

- Note: Subcarrier spacing decision is up to AI 11.3.2.

Conclusion

  - Template in R1-2506582 is to be used for collecting inputs from companies.

  - - Additional NTN or TN assumptions, if any, or any necessary change of the parameters, are to be incorporated into the updated one of R1-2506582.

Agreement

Study which of the following traffic models are to be used for 6G evaluations, e.g., Full buffer

  - FTP Model 1 (in TR 36.814)

  - FTP Model 2 (in TR 36.814)

  - FTP Model 3 (in TR 36.872)

  - XR Traffic models (in TR 38.838)

  - VoIP model (as in TR 36.814)

  - Instant message (as in TR 38.840)

Study whether to introduce the following traffic models for 6G evaluations considering, e.g., FTP-3 variant with packet delay budget requirement ▪ Details FFS

  - New traffic model considering a mixed/variable packet size and the associated time domain behaviors (e.g., time between adjacent packet arrivals, packet delay budget) ▪ Details FFS

  - New traffic model(s) considering the new use cases or services, e.g., AI/ML services, immersive communication services, etc. ▪ Details FFS

  - Study whether to introduce new/additional approaches that can reflect the impact of bidirectional traffic flows on performance metrics (e.g., impact of UL TCP ACK latency on DL throughput/latency)

Note: Whether/how to consider the combination of traffic model and loading level will be studied under individual agendas.

# Attributes for Non-Terrestrial Network Deployment Scenarios

As outlined in [4], NTN deployment scenarios are defined to allow for the provision of services:

- in vertical space areas (e.g., up to several kilometers altitude) where terrestrial network services are not available (on a permanent or temporary basis), and

- that can be more efficiently supported by Non-Terrestrial Networks.

NTN can overcome terrestrial network coverage limitations, especially in low-density populated areas and maritime areas. Accordingly, NTN will mainly contribute to:

- Achieving ubiquitous connectivity and availability of services in remote and sparsely populated areas,

- Improving overall 6G network resiliency by supporting in priority critical communications and emergency communications (messaging, voice calls). This is essential in situations where disasters disrupt terrestrial networks or damage undersea communication cables.

NTN may support messaging, enhanced Mobile BroadBand (eMBB), voice, video, positioning, sensing, delay tolerant, machine type communications, and other data services. It may also support enablers for navigation (UE in connected mode) and timing services.

Several types of UE may be served by NTN: handheld, IoT devices as well as moving platforms/building mounted terminals or small size antenna aperture terminal (e.g., antenna size smaller than 20 x 20 cm). Moving platforms refer to vehicles, trains, ships, airplanes, drones.

For completeness, these attributes of the NTN deployment scenarios are listed in the Annex (see Table 6).

# General simulation assumptions for NTN

As per RAN1#122 agreement, additional NTN or TN assumptions, if any, or any necessary change of the parameters, are to be incorporated into the updated one of R1-2506582.

In the following, we provide new satellite access node characteristics for the evaluation of NTN based 6GR access. The aim is to update the satellite parameters listed in Table 6.1.1.1-1 from TR 38.821 (Release 16) with the current state-of-the-art space technologies for 6G network deployments.

The next tables are reporting SAN parameters (like EIRP, G/T, antenna size, and beam diameter) for L-, S-, C-, Ku-, Ka-, and Q/V bands.

NOTE-1: The reported beam diameters refer to the Nadir pointing of the satellite. For low elevation angles (e.g., 30 degrees), the beam diameter is approximatively four times larger.

Table 1: Satellite parameters in L- and S- bands.

|  |  | Around 1.5 GHz |  | Around 2 GHz |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | L-band LEO 1200km | L-band LEO 600km | S-band LEO 1200km | S-band LEO 600km | S-band VLEO 300km |
| DL | Equivalent satellite antenna aperture | 2.5 m | 2.5 m | 2.5 m | 2.5 m | 1.5 m |
|  | Satellite EIRP density | 38 dBW/MHz | 38 dBW/MHz | 40 dB/MHz | 40 dB/MHz | 24 dBW/MHz |
|  | Max Tx Antenna Gain | 36 dBi | 36 dBi | 38 dBi | 38 dBi | 27 dBi |
|  | 3dB beamwidth | 4.50 deg | 4.50 deg | 3.6 deg | 3.6 deg | 6.0 deg |
|  | Beam diameter | 95 km | 50 km | 75 km | 40 km | 30 km |
| UL | Equivalent satellite antenna aperture | 4 m | 4 m | 4 m | 4 m | 1.5 |
|  | Max Rx Antenna Gain | 36 dBi | 36 dBi | 38 dBi | 38 dBi | 27 dBi |
|  | G/T | 11 dB/K | 11 dB/K | 13 dB/K | 13 dB/K | 1.5 dB/K |

Table 2: Satellite parameters in C-band.

|  |  | Around 4 GHz |  |
| --- | --- | --- | --- |
|  |  | C-band GEO | C-band LEO 600km |
| DL | Equivalent satellite antenna aperture | 12 m | 2 m |
|  | Satellite EIRP density | 59 dBW/MHz | 34 dBW/MHz |
|  | Max Tx Antenna Gain | 50.5 dBi | 35 dBi |
|  | 3dB beamwidth | 0.42 deg | 2.50 deg |
|  | Beam diameter | 260 km | 30 km |
| UL | Equivalent satellite antenna aperture | 12 m | 2 m |
|  | Max Rx Antenna Gain | 50.5 dBi | 35 dBi |
|  | G/T | 24 dB/K | 9 dB/K |

Table 3: Satellite parameters in Ku-band

|  |  | Around 11/14GHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Ku-band MEO 8000km | Ku-band LEO 1200km | Ku-band LEO 600km |
| DL | Equivalent satellite antenna aperture | 1.50 m | 0.45 m | 0.40 m |
|  | Satellite EIRP density | 27 dBW/MHz | 11 dBW/MHz | 5.5 dB/MHz |
|  | Max Tx Antenna Gain | 42.5 dBi | 32 dBi | 31 dBi |
|  | 3dB beamwidth | 1.00 deg | 3.33 deg | 3.75 deg |
|  | Beam diameter | 140 km | 75 km | 40 km |
| UL | Equivalent satellite antenna aperture | 1.10 m | 0.33 m | 0.30 m |
|  | Max Rx Antenna Gain | 42.5 dBi | 32 dBi | 31 dBi |
|  | G/T | 17 dB/K | 7.2 dB/K | 5.5 dB/K |

Table 4: Satellite parameters in Ka-band

|  |  | Around 20/30GHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Ka-band GEO | Ka-band MEO 8000km | Ka-band LEO1200km | Ka-band LEO 600km | Ka-band LEO 300km |
| DL | Equivalent satellite antenna aperture | 6.0 m | 1.0 m | 0.3 m | 0.25 m | 0.2 m |
|  | Satellite EIRP density | 42 dBW/MHz | 31 dBW/MHz | 15 dBW/MHz | 9 dBW/MHz | 3.5 dBW/MHz |
|  | Max Tx Antenna Gain | 60 dBi | 43 dBi | 33 dBi | 31 dBi | 29 dBi |
|  | 3dB beamwidth | 0.16 deg | 0.95 deg | 3.16 deg | 3.80 deg | 4.75 deg |
|  | Beam diameter | 100 km | 130 km | 70 km | 40 km | 25 km |
| UL | Equivalent satellite antenna aperture | 4.00 m | 0.67 m | 0.2 m | 0.17 m | 0.13 m |
|  | Max Rx Antenna Gain | 60 dBi | 43 dBi | 33 dBi | 31 dBi | 29 dBi |
|  | G/T | 34 dB/K | 17 dB/K | 7.0 dB/K | 4.9 dB/K | 3 dB/K |

Table 5: Satellite parameters in Q/V-band

|  |  | Around 40/50GHz |  |
| --- | --- | --- | --- |
|  |  | Q/V-band GEO | Q/V-band LEO 600km |
| DL | Equivalent satellite antenna aperture | 2.7 m | 0.27 m |
|  | Satellite EIRP density | 45 dBW/MHz | 15 dBW/MHz |
|  | Max Tx Antenna Gain | 60.4 dBi | 40.4 dBi |
|  | 3dB beamwidth | 0.18 deg | 1.88 deg |
|  | Beam diameter | 120 km | 20 km |
| UL | Equivalent satellite antenna aperture | 2.1 m | 0.21 m |
|  | Max Rx Antenna Gain | 60.4 dBi | 40.4 dBi |
|  | G/T | 30 dB/K | 10 dB/K |

Proposal 1: For the evaluation of NTN-based 6GR access, consider the SAN characteristics reported in Tables 1 to 5 of document R1-2507571.

Proposal 2: Add the new SAN characteristics in common TN/NTN evaluation parameters spreadsheet.

# Conclusion

In this contribution, we made the following observations and proposals:

Proposal 1: For the evaluation of NTN-based 6GR access, consider the SAN characteristics reported in Tables 1 to 5 of document R1-2507571.

Proposal 2: Add the new SAN characteristics in common TN/NTN evaluation parameters spreadsheet.

# References

- RP-251881, New SID: Study on 6G Radio, RAN#108

- RP-252912, Revised SID: Study on 6G Radio, RAN#109

- Chairman’s Notes, 3GPP RAN1 #122, Bangalore, India, August 2025

- RP-251935, “Text proposal for TR 38.914 for NTN deployment scenarios”, THALES

# Annex

The attributes of the NTN deployment scenarios discussed in [4] are reported below for information.

Table 6: Attributes for NTN deployment scenarios

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 1.5 GHz Around 2 GHz Around 4 GHz Around 11/13 GHz Around 20/30 GHz Around 40/50 GHz |
| Aggregated system Bandwidth | Up to [60] MHz (DL+UL): Around 1.5 GHz and Around 2 GHz Up to [100] MHz (DL+UL): Around 4 GHz Up to [200] MHz (DL+UL): Around 11/13 GHz Up to [800] MHz (DL+UL): Around 20/30 GHz Up to [1600] MHz (DL+UL): Around 40/50 GHz |
| Orbit types (Note 1) | Any orbit from VLEO (300 to 600 km altitude) to GSO (~35 786 km altitude), including LEO (600 to 2000 km altitude) and MEO (2000 to ~35000 km altitude), |
| Duplex mode | In paired bands (Satellite service allocated bands) and all orbits: FDD (Frequency Division Duplexing) or Half Duplex Frequency Division Duplexing (HD-FDD) mode In unpaired bands (Satellite service allocated bands) with Max RTD < [TBD] ms (e.g. VLEO/LEO only): TDD (Time Division Duplexing) mode (Note 4) |
| Architecture (Note: 2) | Transparent payload: BS on ground (at feeder gateway) Regenerative payload: [architectural impact TBD] |
| Satellite access node’s antenna characteristics | Antenna array of max [TBD] elements [details FFS] |
| Radio Cell type (Note: 3) | Earth fixed (GSO), Quasi Earth fixed (NGSO) or Earth moving (NGSO) |
| User distribution | Below 7.125 GHz: - Case 1 conditions: 100% Outdoor - Case 2 conditions: [TBD%] Outdoor and [TBD%] in adverse propagation conditions (e.g., light indoor, in vehicle) Above 10.7 GHz: Outdoor only: [100%] Outdoor |
| Note 1: The orbit types can be characterised with a set of parameters (e.g. Maximum Round Trip Delay (ms), Maximum Doppler shift (Hz), Max Differential delay (ms), Max Differential Doppler (Hz), Max Doppler rate (Hz/s), Max delay variation (s-2), Channel model type in TR 38.811). Note 2: A transparent payload forward transparently the 3GPP radio protocol between the feeder and the service link and vice versa. Note 3: A radio cell is able to serve UEs on the earth surface as well as in the space above. It may be realised with one or several beams generated by satellites. Only part of the beams generated by a given satellite may be active. Note 4: TDD mode design should consider at least the band around 1.5 GHz. |  |