3GPP TSG RAN WG1 #122bis                                                  R1-2507635

Prague, Czech, Oct 13th – 17th, 2025

Source: Google

Title:	On Evaluation Assumptions for the 6GR air interface

Document for: Discussion and Decision

# Introduction

The new SID for 6G Radio was approved in RAN#108 [1]. This contribution provides further views on evaluation assumptions for the 6G Radio (6GR) air interface study. The focus remains on establishing a simple, efficient, and extensible 6G framework, with a particular emphasis on supporting AI/ML services and practical device configurations.

To ensure a consistent and productive study, it is essential to align on fundamental assumptions for models, scenarios, parameters, and methodologies.

# Discussion

## Traffic Model

In RAN1#122, the following agreement has been reached regarding the traffic models to be used for the 6G evaluations:

Agreement

- Study which of the following traffic models are to be used for 6G evaluations, e.g.,

  - Full buffer

  - FTP Model 1 (in TR 36.814)

  - FTP Model 2 (in TR 36.814)

  - FTP Model 3 (in TR 36.872)

  - XR Traffic models (in TR 38.838)

  - VoIP model (as in TR 36.814)

  - Instant message (as in TR 38.840)

- Study whether to introduce the following traffic models for 6G evaluations considering, e.g.,

  - FTP-3 variant with packet delay budget requirement

    - Details FFS

  - New traffic model considering a mixed/variable packet size and the associated time domain behaviors (e.g., time between adjacent packet arrivals, packet delay budget)

    - Details FFS

  - New traffic model(s) considering the new use cases or services, e.g., AI/ML services, immersive communication services, etc.

    - Details FFS

- Study whether to introduce new/additional approaches that can reflect the impact of bidirectional traffic flows on performance metrics (e.g., impact of UL TCP ACK latency on DL throughput/latency)

Note: Whether/how to consider the combination of traffic model and loading level will be studied under individual agendas.

The agreement confirmed a study for new traffic models, explicitly mentioning: AI/ML services, immersive communication services, etc. This aligns directly with our views for an AI-specific, token-streamlined traffic model.

To support the agreed-upon study for a new traffic model for AI/ML services, we propose to make the token-streamlined traffic model the baseline for evaluating AI-specific use cases. The model should represent the bursty, short-packet nature of token communication inherent in modern AI/ML services.

- Adopt the token-streamlined traffic model as the baseline for evaluating AI-specific use cases

The SA1 agreements [2][3] on the GenAI use cases provide specific characteristics and requirements for three types of GenAI applications: Visual Assistant, Text-to-Image Generation, and Chatbot (see tables agreed in SA1 in Appendix).

- GenAI traffic properties as agreed in SA1/TR 22.870 can be used as baseline for token-streamlined traffic model in RAN1.

Also, because Energy Efficiency is a defined key performance indicator (KPI) for 6G systems, and the energy cost of generating AI traffic is five to ten times that of other traffic types, AI-native applications require stringent reliability to prevent the loss of this energy-intensive data:

- CSI reporting for AI traffic should be assumed to have higher reliability compared to other traffic types. This is critical to ensure the high-reliability requirement for prompts and burst delivery is met.

- A 1% target BLER for CQI reporting in evaluations for AI traffic. This value directly supports the goal of achieving the low-latency and high-reliability required for GenAI bursts.

- Include the max Allowed latency for a burst (e.g., 20 ms) as key target metrics when evaluating PHY layer designs for AI-native services.

- Support 1% target BLER for CQI reporting in evaluations for AI traffic.

- Support a max allowed latency for a burst (e.g., 20 ms) as key target metric for AI traffic.

## UE and Antenna Configurations

To maintain focus on simplicity and practical implementation for GenAI, XR, and other immersive services on mobile devices, we support prioritizing:

- single-panel UE configuration for evaluations in FR2/FR3.

- A variety of antenna configurations, specifically 4Rx in addition to 8Rx/6Rx for DL, and 2Tx in addition to 4Tx for UL. This ensures the study includes typical handheld devices that will run GenAI and immersive applications.

This approach will also ensure that a range of device types and capabilities, from consumer-grade to high-end, are adequately studied.

- Prioritize a single-panel UE configuration for evaluations in FR2/FR3.

- Include 4Rx for DL and 2Tx for UL antenna configurations to accommodate a wider range of device types.

## Multi-TRP Operations

The Feature Lead in RAN1#122 noted that multi-TRP is to be discussed under the MIMO agenda. We continue to propose focusing on single-TRP operations in the initial study and design phase to reduce complexity. Multi-TRP scenarios should be considered as a secondary phase for more advanced features. This ensures that the fundamental air interface for 6GR is simplified and efficient before introducing the complexity of multi-TRP configurations.

## Numerology and Waveform

- Numerology: A consistent numerology is vital for a simplified system design. Therefore, we propose adopting one SCS per frequency range for both data and control channels. However, it can be studied whether a different SCS is necessary for reference signals.

- Waveform (Non-ISAC): For non-ISAC scenarios, both CP-OFDM and DFT-s-OFDM should be included/studied for both UL and DL.

  - For DL, DFT-s-OFDM should be considered for PDSCH.

- Adopt one SCS per frequency range for simplified system design, with a study on the need for different SCS for reference signals.

- Consider using DFT-s-OFDM as additional waveform in DL.

# Conclusion

In this contribution, we proposed evaluation assumptions for the 6G Radio air interface study. The proposals cover key areas including numerology, waveform, coding, modulation, duplexing, initial access, and ISAC, with a focus on simplicity, flexibility, and practical implementation. We believe that these assumptions will serve as a solid foundation for guiding the discussions and evaluations and will facilitate the development of a unified and extensible 6G Radio framework.

- Adopt the token-streamlined traffic model as the baseline for evaluating AI-specific use cases

- GenAI traffic properties as agreed in SA1/TR 22.870 can be used as baseline for token-streamlined traffic model in RAN1.

- Support 1% target BLER for CQI reporting in evaluations for AI traffic.

- Support a max allowed latency for a burst (e.g., 20 ms) as key target metric for AI traffic.

- Prioritize a single-panel UE configuration for evaluations in FR2/FR3.

- Include 4Rx for DL and 2Tx for UL antenna configurations to accommodate a wider range of device types.

- Adopt one SCS per frequency range for simplified system design, with a study on the need for different SCS for reference signals.

- Consider using DFT-s-OFDM as additional waveform in DL.

# References

- RP-251881” New SID: Study on 6G Radio” RAN#108, Prague, Czech Republic, June 9-13, 2025, NTT DOCOMO.

- TR 22.870 Study on 6G Use Cases and Service Requirements

- S1-253593 “Pseudo-CR on update 6.25 Use case on optimizing user experience for GenAI applications”, 3GPP TR 22.870 v0.3.1, Huawei, SA1#111, approved.

# Appendix

KPIs comparison between GenAI application and short video application

| Application | Average packet size (Byte) | Uplink |  |  |  |  |  | Downlink |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Average Data Rate (Mbps) | Packet Percentage | Traffic Percentage | Burst Data Rate (Mbps) | Burst Packet Percentage | Burst Traffic Percentage | Average Data Rate (Mbps) | Burst Data Rate (Mbps) |
| short video | 1144 | 0.007 | 17% | 1% | 0.23 | 17% | 1% | 0.91 | 24 |
| short video live streaming | 921 | 0.034 | 34.7% | 1.6% | 0.23 | 38% | 1.6% | 2 | 13.2 |
| visual assistant | 838 | 1 | 49% | 48% | 67.6 | 51% | 52% | 1.2 | 69.4 |
| text-to-image generation | 987 | 0.03 | 28% | 5.6% | 0.9 | 22% | 2.7% | 0.6 | 34.1 |
| chatbot | 87 | 0.008 | 48% | 22.7% | 0.08 | 48% | 31.8% | 0.11 | 0.14 |
| Note: meanings of KPIs are as follows average (uplink/downlink) data rate: average data rate of uplink/downlink traffic during the whole transmission (uplink) packet percentage: number of uplink packets divided by number of all packets during the whole transmission (uplink) traffic percentage: amount of uplink data traffic divided by amount of all data traffic during the whole transmission burst (uplink/downlink) data rate: average data rate of uplink/downlink traffic within a burst burst (uplink) packet percentage: average number of uplink packets within a burst divided by average number of packets within a burst burst (uplink) traffic percentage: average amount of uplink data traffic within a burst divided by average amount of data traffic within a burst |  |  |  |  |  |  |  |  |  |

Required KPI for GenAI applications

| Traffic Type | Burst size | Max Allowed latency for a burst | Service bit rate: user-experienced data rate | Delay | Average packet size |
| --- | --- | --- | --- | --- | --- |
| Image based GenAI app | 400 KB | 50 ms | 64 Mbps | 20 ms | > 800B |
| Video based GenAI app | 20 MB | 400 ms | 400 Mbps | 20 ms | > 800B |
| chatbot | 0.5 KB | 20 ms | 200 Kbps | 30 ms | < 800B |
| Note: meanings of some KPIs: max allowed latency for a burst: max latency for sending out the whole packets within a burst delay: transmission latency of a packet |  |  |  |  |  |