3GPP TSG RAN WG1 #122bis		             			                           R1-2507748

Prague, Czech Republic, October 13th –17th, 2025

Agenda Item:	11.3.2

Source:	AT&T

Title:	Requirements for 6GR Frame Structure Design

Document for:	Discussion/Decision

# Introduction

At the last RAN1 #122 meeting in Bengaluru, India, the following was agreed.

Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion. Conclusion - Numerologies for sensing is up to sensing agenda discussion. Agreement - 6GR supports normal cyclic prefix, i.e., same as the normal CP defined in NR. - FFS potential need for other CP

These agreements were further revised at the TSG RAN #109 meeting.

RP-252963 Moderator's summary for 6G - system design aspects (numerology, CMCC spectrum) Replaces related to RP-252029, 2055, 2198, 2207, 2328, 2320, 2456, 2466, 2520, 2654, 2657, 2671, 2697 RP-252949 agreements of conclusion section are endorsed Agreement Regarding the RAN1 agreement below, the following is agreed: Keep using “around 7GHz” instead of using detailed frequency range definition in RAN1’s and RAN4’s study. Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion.

In this contribution, we present our views on these and additional aspects of the 6GR frame structure design.

# Requirements for 6GR Frame Structure Design

## Subcarrier Spacings for 6GR

Mixed numerology was supposed to be(come) a key feature of the 5G NR design. In the end, however, deployments with a single numerology per band prevailed. In FR2, one of the key drivers for mixed numerology support was overhead reduction. A larger subcarrier spacing (viz., 240 kHz) shortens the time it takes for beam sweeping with a large number of beams as prevalent in FR2 and allows for more resources for PDSCH transmissions with a smaller subcarrier spacing (viz., 120 kHz). Larger subcarrier spacings also shorten the transmission time interval, assuming a fixed number of OFDM symbols, and multiplexing of numerologies was thus considered beneficial for URLLC, e.g., with preemption. None of these design paradigms, however, have materialized in real-world deployments.

Objective 2(d) of the Study on 6G Radio [1] is to avoid multiple numerologies for the same band / sub-range. RAN1 should codify this agreement with the following proposal:

Proposal 1: The 6GR design allows for at most one sub-carrier spacing per band

Notwithstanding, 3GPP should not preclude the definition of two or more bands that are partially or fully overlapping and define different subcarrier spacings if different subcarrier spacings for the respective frequencies are defined in 3GPP specifications.

To progress the 6GR frame structure design, the following should be agreeable in line with the agreements from both 
RAN1 #122 and RAN #109:

Proposal 2:

  - For sub 6GHz

    - The following subcarrier spacing is at least supported

      - 15kHz SCS for FDD

      - 30kHz SCS for TDD

      - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz

  - For around 7GHz

    - The following subcarrier spacing is at least supported

      - 30kHz

      - FFS: 60kHz

  - FFS: For around 15GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz, 120kHz

  - For between 24.25GHz - 52.6GHz

    - Subcarrier spacing 120kHz is supported

  - FFS whether to allow using additional subcarrier spacing for SSB

  - FFS subcarrier spacing for PRACH and up to initial access discussion.

We are open to studying the benefits of 30kHz SCS for FDD and 60kHz SCS for FR3, e.g., to reduce latency.

## Duplex Types

In our view, more important than the support of dynamic TDD is that UEs support and are tested and certified for a large number of semi-static TDD configurations that are relevant in current or future deployments by mobile network operators.

Observation 1: More important than the support of dynamic TDD is that UEs support and are tested and certified for a large number of semi-static TDD configurations that are relevant in current or future deployments by mobile network operators

Full duplex and half duplex FDD and semi-static TDD must be supported, and we are open to studying gNB semi-static SBFD and dynamic TDD based on the learnings from 5G NR.

Proposal 3: 6GR supports at least the following duplex types:

- FD-FDD

- Semi-static TDD

- HD-FDD on UE side

- FFS: gNB semi-static SBFD

- FFS: Dynamic TDD

## Maximum Channel Bandwidth Options

The following is our proposal for the maximum channel bandwidth of 6GR.

Proposal 4: The maximum channel bandwidth of 6GR is

- 0.410 – 6.425 GHz

  - 100MHz (FDD)

    - FFS whether restrict to [50MHz] for [< 2GHz]

  - 200MHz (TDD)

- 6.425 – 7.125 GHz

  - 400MHz

- 7.125 – X

  - 400MHz

  - FFS: [800MHz]

- X – Y

  - 400MHz

  - FFS: [800MHz]

- 24.25 – 52.6 GHz

  - 800MHz

- FFS: X = 8.4 or 10.5 GHz

- FFS: Y = 15.35 or 24.25 GHz

# Conclusion

In this contribution, we presented our views on aspects of the 6GR frame structure design. The following is observed and proposed:

Observation 1: More important than the support of dynamic TDD is that UEs support and are tested and certified for a large number of semi-static TDD configurations that are relevant in current or future deployments by mobile network operators

Proposal 1: The 6GR design allows for at most one sub-carrier spacing per band

Proposal 2:

  - For sub 6GHz

    - The following subcarrier spacing is at least supported

      - 15kHz SCS for FDD

      - 30kHz SCS for TDD

      - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz

  - For around 7GHz

    - The following subcarrier spacing is at least supported

      - 30kHz

      - FFS: 60kHz

  - FFS: For around 15GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz, 120kHz

  - For between 24.25GHz - 52.6GHz

    - Subcarrier spacing 120kHz is supported

  - FFS whether to allow using additional subcarrier spacing for SSB

  - FFS subcarrier spacing for PRACH and up to initial access discussion.

Proposal 3: 6GR supports at least the following duplex types:

- FD-FDD

- Semi-static TDD

- HD-FDD on UE side

- FFS: gNB semi-static SBFD

- FFS: Dynamic TDD

Proposal 4: The maximum channel bandwidth of 6GR is

- 0.410 – 6.425 GHz

  - 100MHz (FDD)

    - FFS whether restrict to [50MHz] for [< 2GHz]

  - 200MHz (TDD)

- 6.425 – 7.125 GHz

  - 400MHz

- 7.125 – X

  - 400MHz

  - FFS: [800MHz]

- X – Y

  - 400MHz

  - FFS: [800MHz]

- 24.25 – 52.6 GHz

  - 800MHz

- FFS: X = 8.4 or 10.5 GHz

- FFS: Y = 15.35 or 24.25 GHz

# References

- RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone