3GPP TSG RAN WG1 #122bis	R1-2507769

Prague, Czech, Oct 13th – 17th, 2025

Source:	Sharp

Title:	Improvement of SRS capacity and coverage

Agenda Item:	10.2.1

Document for:	Discussion and Decision

Background

In RAN#108, the work item on MIMO Phase 6 was agreed [1].

- On enhancing UL capacity and coverage, specify the following enhancements:

  - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification:

    - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding

      - Note: On phase continuity, the applicable conditions and requirements from the legacy RAN4 spec for DMRS bundling should be retained as much as possible.

    - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set

      - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots

Discussions

  - Intra-repetition hopping for SRS repetition symbols within each SRS frequency hop

Agreement: For SRS configured with RPFS (PF>1) and multiple repetitions (R > 1), support multiple frequency-domain starting positions across SRS repetition symbols within each SRS frequency hop based on the followings: - For SRS repetition symbols within each SRS frequency hop, the starting position patterns across the K different frequency locations are determined by network configuration - The R repetition symbols are equally divided into K subgroups - K is number of starting positions for SRS repetition symbols within each SRS frequency hop - Within each subgroup of R/K symbols, the SRS is transmitted at the same starting position in frequency domain. - Start position pattern for SRS repetition symbols within each SRS frequency hop is the same during the legacy SRS frequency hopping period (for a same value of nSRSb'=bhopBSRSNb') FFS: whether/how to support enabling legacy RPFS start RB index hopping across multiple legacy SRS frequency hopping periods and intra-repetition hopping for SRS repetition symbols within each SRS frequency hop simultaneously. Agreement: For intra-repetition hopping for SRS repetition symbols within each SRS frequency hop, study the following configuration combinations: - PF=2 and K=2 - PF =4 and K=2 - PF =4 and K=4

Combination of PF and K

There are the following combination candidate(s) of a frequency scaling factor PF and number K of RB starting positions:

- PF=2 and K=2

- PF=4 and K=2

- PF=4 and K=4

In our simulation results, MSE is improved by supporting multiple RB starting positions when NapSRS=4, NsymbSRS=4, CSRS=17, BSRS=1, KTC=8, R=2, and PF=4 as shown in Figure 1.

| Case A (Rel-17) | Case B (Rel-20) |
| --- | --- |

Figure 1: MSE for SNR when NapSRS=4, NsymbSRS=4, CSRS=17, BSRS=1, KTC=8, R=2, and PF=4.

Figure 1 shows the MSE performance in a link-level simulation under the CDL-E channel, with 3.5 GHz carrier frequency and 30 kHz SCS. In this result, note that each case has 10logKTCPFNb dB SNR gain caused from sparse frequency resource allocation.

In Case A, the repetition gain is achieved within R repeated symbols, but the interpolation region is larger than Case B. In Case B, the interpolation accuracy is improved, but the repetition gain is not achieved. As results, the improvement of the channel estimation accuracy of 2D-MMSE by Case B is more beneficial according to Figure 1. Therefore, a case PF=4, R=2, and K=2 is more beneficial compared to another case PF=4, R=2, and K=1.

Likewise, we think the combinations of PF=2, R=2, and K=2 outperforms that of PF=2, R=2, and K=1, and the combinations of PF=4, R=4, and K=4 outperforms that of PF=4, R=4, and K=1. This is because the improvement of the interpolation is more beneficial than the repetition gain as shown in Figure 1. For these reasons, we support all the combinations.

Proposal 1: For intra-repetition hopping for SRS repetition symbols within each SRS frequency hop, we support the following combinations:

- PF=2 and K=2

- PF=4 and K=2

- PF=4 and K=4

Furthermore, this result does not depend on time-domain behaviours. Therefore, we can support all SRS types.

Proposal 2: We support intra-repetition hopping for aperiodic, semi-persistent, and periodic SRS.

Consecutive vs non-consecutive R/K symbols

In RAN1#122 meeting, it was agreed that SRS is transmitted at the same starting position within each subgroup of R/K symbols. However, it is not defined whether each subgroup includes consecutive R/K symbols or may include non-consecutive R/K symbols. In our view, non-consecutive R/K symbols are more complicated than consecutive R/K symbols because subgroup switching happens within R symbols. Nevertheless, the repetition gain decreases in non-consecutive R/K symbols and the phase continuity issue potentially happens. For these reasons, we support that each of K subgroups consists of consecutive R/K symbols.

Proposal 3: For intra-repetition hopping, each of K subgroups should consist of consecutive R/K symbols.

In this case that each subgroup consists of consecutive R/K symbols, K subgroups can be switched at every consecutive R/K symbols within an SRS resource.

Proposal 4: For intra-repetition hopping, K subgroups should be switched at every consecutive R/K symbols within an SRS resource.

  - Cross-slot SRS

Agreement: For a P/SP cross-slot SRS resource, the slot offset configured to the SRS resource refers to the first of the two slots spanned by the SRS resource. Agreement: When there is a cross-slot SRS resource in the end of a S slot and the beginning of a U slot in a serving cell, support transmitting PUSCH with a priority index 0 and corresponding DMRS after this cross-slot SRS in the U slot in the serving cell. Agreement: Support at least the following scenarios for cross-slot SRS transmission: - Scenario 1: a periodic, semi-persistent, or aperiodic SRS resource set which includes at least one SRS resource with time-domain resource transmitted across a first S slot and a second consecutive U slot. - FFS whether Scenario 2 below is supported. - Scenario 2: an aperiodic SRS resource set which includes at least one SRS resource with time-domain resource transmitted in a first S slot, and at least one another SRS resource with time-domain resource transmitted in a second consecutive U slot. Agreement: For a given AP-SRS resource set in case of SRS transmission across two adjacent S+U slots, study the following alternatives and down-select one to determine “available slot” in the next meeting: - Alt-0 (per SRS resource set): The available slot(s) can include one slot or two consecutive S and U slots satisfying there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. - Alt-1 (per SRS resource): The available slot(s) can include one slot or two consecutive S and U slots satisfying there are UL or flexible symbol(s) for the time-domain location(s) for at least one of the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. - Alt-2 (per slot): An available slot is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for a subset of SRS resource(s) with same slot offset in the SRS resource set, and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. Agreement: To determine the time-domain location of cross-slot SRS, - One set of time-domain resource allocation related parameters (i.e., startPosition, nrofSymbols, and repetitionFactor) is configured for the SRS resource without restriction on “within a slot”. - For the index of each SRS symbol l', it is the same as legacy spec., i.e., l'=0, 1, …, NsymbSRS-1. - The offset loffset∈0,1,…,13 counts symbols backwards from the end of the starting slot of the resource

Scenario 1 (Cross-slot SRS resource)

In RAN1#122 meeting, Scenario 1, cross-slot SRS resource, was agreed. Regarding the time position, a symbol-level starting position of a SRS resource is determined as l0=NsymbSlot-1-loffset in a slot without the restriction loffset≥NsymbSRS-1. That is, the SRS resource can cross the slot when Nsymbslot is larger than loffset. Furthermore, a slot-level starting position (i.e., first slot) of two S+U consecutive slots is configured by periodicityAndOffset-p/periodicityAndOffset-sp in P/SP SRS resource according or slotOffset in AP SRS resource set which includes at least one SRS resource across two S+U consecutive slots.

As remaining issues, regarding an available slot-level starting position (i.e., available slot offset), there are three alternatives as following:

- Alt 0: “Available slot” includes all SRS resources in one SRS resource set satisfying the conditions.

  - Alt 0-1: “Available slot” is a set of two consecutive slots.

  - Alt 0-2: “Available slot” is one slot of two consecutive slots.

- Alt 1: “Available slot” includes at least one of all SRS resources in one SRS resource set satisfying the conditions.

- Alt 2: “Available slot” includes a subset of SRS resources with the same slot offset in one SRS resource set satisfying the conditions.

In the legacy spec, an available slot is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. Furthermore, Alt 0 is aligned with this legacy spec and has the lowest spec impacts. Therefore, we support Alt 0. Regarding one slot vs two consecutive slots, we prefer to count a set of consecutive S and U slots as “Available slots”. If each slot is counted as available, there is a possibility that t+1 th available slot is not S slot.

Proposal 5: Regarding an available slot offset for cross-slot SRS resource, we support Alt-0 (per SRS resource set). i.e., The available slot(s) include two consecutive S and U slots satisfying there are UL or flexible symbols for all SRS resources in one SRS resource set.

For the number of SRS symbols, we prefer to reuse the legacy value range (i.e., 1-14 symbols). In our understanding, a target of cross-slot SRS resource improves SRS capacity and coverage by efficiently using the legacy time resource, not extending the resource. For example, in Figure 2, SRS capacity and coverage can be improved while avoiding the consumption of PUSCH resources. Therefore, the maximum number of SRS symbols should be limited to 14. Likewise, the maximum number of repetitions should be limited to 14.

| - Legacy | (b) Rel-20 |
| --- | --- |

Figure 2: An example of cross-slot SRS resource.

Proposal 6: For cross-slot SRS resource, the maximum number of SRS symbols should be limited to 14.

Proposal 7: For cross-slot SRS resource, the maximum number of SRS repetitions should be limited to 14.

For antenna switching, multiple SRS resources including y/x SRS resources of xTyR are transmitted in one or different slots. The following cases was raised by OPPO [2]:

Figure 3: Cross-slot antenna switching cases raised by OPPO [2]

In Scenario 1, we think Case 1 and Case 2 are valid because at least one of SRS resource in multiple SRS resource sets cross two consecutive slots and the legacy resource can be efficiently used without extending the legacy resource.

Proposal 8: We support cross-slot SRS resource included in some of multiple SRS resource sets for antenna switching.

In Case 2, SRS resource set1 includes Resource0 across two consecutive slots and Resource1 in the second slot. In this case, it is unclear how to use slotOffset in aperiodic SRS resource set1. As a simple solution, we prefer to introduce per-SRS resource slot offset for each of SRS resources in AP SRS resource sets for antenna switching.

Observation 1: In a case that an aperiodic SRS resource set includes one SRS resource across two consecutive slots and another SRS resource in the second slot of two the consecutive slots, slotOffset in the set cannot be used to indicate different slots for the SRS resources.

Proposal 9: For aperiodic SRS resource sets for antenna switching, per-SRS resource slot offset for each of SRS resources should be introduced.

Scenario 2 (Cross-slot SRS resource set)

Scenario 2, cross-slot SRS resource set, remains as one of FFS points. In our view, Scenario 2 is not needed. For antenna switching, Case 4 of Figure 3 can be considered as Scenario 2. However, this case can be avoided, and the same resource efficiency can be achieved such that SRS resource set0 includes Resource0, and SRS resource set1 includes Resource0 and Resource1 by higher layer parameters. Therefore, we have no strong motivation of supporting Scenario 2.

Proposal 10: We are OK not to support Scenario 2.

Conclusion

In this contribution, we have the following proposal:

Proposal 1: For intra-repetition hopping for SRS repetition symbols within each SRS frequency hop, we support the following combinations:

- PF=2 and K=2

- PF=4 and K=2

- PF=4 and K=4

Proposal 2: We support intra-repetition hopping for aperiodic, semi-persistent, and periodic SRS.

Proposal 3: For intra-repetition hopping, each of K subgroups should consist of consecutive R/K symbols.

Proposal 4: For intra-repetition hopping, K subgroups should be switched at every consecutive R/K symbols within an SRS resource.

Proposal 5: Regarding an available slot offset for cross-slot SRS resource, we support Alt-0 (per SRS resource set). i.e., The available slot(s) include two consecutive S and U slots satisfying there are UL or flexible symbols for all SRS resources in one SRS resource set.

Proposal 6: For cross-slot SRS resource, the maximum number of SRS symbols should be limited to 14.

Proposal 7: For cross-slot SRS resource, the maximum number of SRS repetitions should be limited to 14.

Proposal 8: We support cross-slot SRS resource included in some of multiple SRS resource sets for antenna switching.

Observation 1: In a case that an aperiodic SRS resource set includes one SRS resource across two consecutive slots and another SRS resource in the second slot of two the consecutive slots, slotOffset in the set cannot be used to indicate different slots for the SRS resources.

Proposal 9: For aperiodic SRS resource sets for antenna switching, per-SRS resource slot offset for each of SRS resources should be introduced.

Proposal 10: We are OK not to support Scenario 2.

References

- [1]	RP- 251856, New WID: NR MIMO Phase 6, June, 2025.

- [2]	R1-2506585, Moderator (CATT), Moderator summary on improvement of SRS capacity and coverage: Round 4, Aug., 2025.