3GPP TSG RAN WG1 #122bis			     R1-2507790

Prague, Czech, Oct 13th – 17th, 2025

Source:	NTT DOCOMO, INC.

Title:	Maintenance on NR MIMO Phase 5

Agenda Item:	8.2

Document for: 	Discussion and Decision

- Introduction

In RAN#102 meeting, a new Rel-19 WID of ‘NR MIMO Phase 5’ was approved [1]. In this contribution, we discuss the maintenance on NR MIMO Phase5.

- Enhancements for UE-initiated\event-driven beam management

Trigger-event detection

The following agreements and FL proposal were made for trigger-event-detection.

RAN1#117 Agreement Regarding RS measurement for the new beam for Event 2, at least Option-3a is supported - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured - FFS: Option-3b/3c - Option-3b: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s). Option-3c: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of TCI state(s) in a configured subset of the legacy RRC-configured TCI state list RAN1#118 Agreement Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set. - FFS: The RS in the RS resource set can be updated by MAC-CE RAN1#120 Agreement Regarding triggering event determination for Event 2, at least Candidate #2 is supported for resetting the counting. - Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received; - FFS: whether to reset the counting for all new beams. - FFS: whether to maintain the counting whose new beam is NOT updated. - Candidate#2: [The measured current beam based on] indicated TCI state is updated; - In such case, the UE need to reset the counting for all new beams. - Candidate#3: UEI beam report is transmitted; - FFS: Only reset the counting of new beams fulfilling triggering condition and reported by the UEI beam report - Candidate#4: NW response (e.g., DCI in step-2 of Mode-A) is detected. - Candidate#5: The time window expires - Candidate#6: The threshold for event evaluation is re-configured by RRC signaling - (FFS) Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured. - FFS: RRC parameter(s) - FFS: Other candidates Note: Whether this proposal is captured in RAN1 or RAN2 is a separate discussion point. Proposal 1.2.2: Regarding the triggering event determination for Event-7, when the time window and counter with M are configured, down-select at least one of the following for determining event instance(s) counting: - Option-1: The event instance(s) counting is per new beam (as Event-2) - Option-2: The event instance(s) counting is per a pair of ‘new beam’ and ‘activated TCI state with the Q-th best quality’

There are several open issues. The first issue is related to RS measurement for new beam. In the RAN1#117 meeting, whether to support additional option for RS measurement for new beam was agreed to further study. In our view, remaining options would be optimization, and there would be no clear benefit because it could also be achieved by Option-3a. Thus, we propose

Proposal 1

- On RS measurement for new beam, do not support Option-3b/3c.

  - Option-3b: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s).

  - Option-3c: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of TCI state(s) in a configured subset of the legacy RRC-configured TCI state list.

The next issue is related to RS resource configuration for new beam. In the RAN1#118 meeting, whether to support MAC CE to update RS resources in a RS resource set was agreed to further study. Basically, if a lot of RS resources for measurement/reporting are configured by NW, it would require heavy UE measurement complexity. On the other hand, if small number of RS resources for measurement/reporting are configured by NW, it would require frequent update by RRC depending on UE movement. In the case, RRC message would be not suitable for frequent updates. Thus, MAC CE to update RS resources in a RS resource set can be considered. With this sense, update by MAC CE would be beneficial for NW to configure small number of RS resources in a RS resource set. Thus, we propose

Proposal 2

- For RS resources in a RS resource set for UEIBR, support MAC CE to update RS resource(s) in a RS resource set for UEIBR.

The next issue is related to operation of counter for event-determination. In the RAN1#120 meeting, Candidate#2 was supported among the following candidates, and others were not reached to consensus.

- Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received;

- Candidate#2: [The measured current beam based on] indicated TCI state is updated;

- Candidate#3: UEI beam report is transmitted;

- Candidate#4: NW response (e.g., DCI in step-2 of Mode-A) is detected.

- Candidate#5: The time window expires

- Candidate#6: The threshold for event evaluation is re-configured by RRC signaling

- Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured.

For Candidate#1/#6/#7, those all candidates say that counter should be reset when RRC configuration for UEIBR is reconfigured or updated. So, it would be better to support Candidate#7 with including update by MAC CE since it includes Candidate#1/#6. For Candidate#3/#4, whether they are needed depends overall UEIBR procedure (e.g., retransmission of first PUCCH, acknowledge for each channel, prohibit timer, etc.). So, this should be discussed after the detailed procedure is decided. For Candidate#5, it should be naturally supported as in legacy. Thus, we propose

Proposal 3

- For resetting conditions of counter, support at least following all conditions.

  - Candidate#5: The time window expires;

  - Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured or update by MAC CE (if supported).

The next issue is regarding how to count event instance when counter and timer is configured for Event-7. In the RAN1#120 meeting, the following two options were raised.

- Option-1: The event instance(s) counting is per new beam (as Event-2)

- Option-2: The event instance(s) counting is per a pair of ‘new beam’ and ‘activated TCI state with the Q-th best quality’

In our analysis, Option-2 would be complicated since the activated TCI state with the Q-th best quality would be changed within time window. On the other hand, Option-1 would be simpler. Thus, we propose

Proposal 4

- When the counter and timer is configured for Event-7, support Option-1.

  - Option-1: The event instance(s) counting is per new beam (as Event-2).

UL signal content

The following agreement and FL proposal were made for UL signal content.

RAN1#120 Proposal 2.1.2: On UE-initiated/event-driven beam reporting, revise the following agreement in RAN1#117. - On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, at least Option-3 is supported - Option-3: N ≥ 1 beam(s) are reported in the report instance, - For mode-B, at least one of N reported beam(s) should satisfy the condition of Event-2 - N is configured by gNB - FFS: candidate value of ‘N’. - FFS: RRC can enable or disable whether current beam is always reported in addition to the N beams - FFS: Option-1/1a/1b/2. - Above applies at least for the single CC case RAN1#120bis Agreement On UE-initiated/event-driven beam reporting, support the following interpretation on each of the codepoints of the ‘1-bit’ condition met indicator as follows: - ‘0’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window doesn’t reach the configured number M. - ‘1’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window reach the configured number M. FFS: whether/how to introduce the ‘1-bit’ condition met indicator for Event 7.

There are several open issues. First issue is regarding handling of false alarm case for Mode A. If NW triggers the UEIBR by CSI request field in the DCI when there is no beam satisfying the condition, based on the current agreement, UE will not transmit UEIBR on the second PUSCH. In the case, NW cannot identify whether the reception of second PUSCH/DCI is failed or there is no beam satisfying the condition. In addition, NW needs blind detection of the second PUSCH. Then, to solve the issues, the following condition “at least one of N reported beam(s) should satisfy the condition of Event-2” for report format is not needed for Mode A and should be limited to Mode B. Thus, we propose

Proposal 5

- Regarding report format for event-2, the following condition should be limited to Mode B.

  - “at least one of N reported beam(s) should satisfy the condition of Event-2”.

The next issue is how to identify TCI states to be activated and/or deactivated by NW. As the potential use case of Event-7, updating activated TCI state list was discussed in the past meetings. To achieve the use case, NW needs to identify TCI states to be activated and/or deactivated by UEIBR. Some solutions have been raised for several meetings, but we concluded no consensus of additional report contents for Event-7 in the last meeting. Then, we need to solve the issue by other than additional report contents.

Observation 1

- RAN1 needs to consider the solution (i.e., other than additional report contents) to achieve the use case of Event-7.

Based on current report contents in UEIBR, possible way would be to reuse new beam RS. First, when Event-7 is configured, RSs configured in current activated TCI states should be configured in new beam RS configuration. Second, UE reports X RSs in a single report instance, where X is determined by UE based on measurement results. Then, when N RSs are configured to be reported in a single report instance, (N-X) new beam RSs and X RSs are reported. Here, the X RSs are derived from the worst X current activated TCI states. If this solution is supported, the issue can be solved without any restriction of configuration and additional report contents. Thus, we propose

Proposal 6

- For report format of Event-7, to identify the TCI states to be activated/deactivated by NW, support the following:

  - RSs configured in current activated TCI states should be configured in new beam RS configuration.

  - When N RSs are configured to be reported in a single report instance,

    - (N-X) new beam RSs and X RSs are reported.

    - X is determined by UE based on the measurement results. i.e., up to UE implementation

    - The X RSs are derived from the worst X current activated TCI states.

The next issue includes additional measurement quantity and measurement RS. In current agreement, only L1-RSRP was supported as measurement quantity. On the other hand, based on WID, we should consider measurement quantity assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management. Thus, as in legacy gNB configured beam report, L1-SINR as measurement quantity should be included. Also, for measurement RS, semi-persistent CSI-RS and aperiodic CSI-RS should be supported since it would be beneficial to reduce overhead even for UE-initiated beam report. Thus, we propose

Proposal 7

- Support L1-SINR as measurement quantity.

  - For L1-SINR, RS combination configuration (i.e., CMR only or CMR+ZP-IMR or CMR+NZP-IMR) for Rel-16 L1-SINR is reused.

- Support semi-persistent CSI-RS and aperiodic CSI-RS for measurement RS configuration for UE-initiated beam report.

UL signal medium/container

The following agreements were made for UL signal medium/container.

RAN1#120bis Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, down-select at least one of the following candidates in RAN1#121: - Candidate#1: To introduce a prohibit timer for mode-A and/or mode-B - Option-1A: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH; - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer - Option-1B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH; - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer - If the prohibit timer is running, the first PUCCH is not allowed to be transmitted. - Candidate#2: To introduce a time interval for mode-A and/or mode-B - Option-2A: For a first PUCCH transmission occasion, if there is a transmission of another second PUSCH [corresponding to the CSI report determined by the same triggering beam(s) as the first PUCCH] within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH. - Option-2B: For a first PUCCH transmission occasion, if there is a transmission of another first PUCCH(s) within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH. - Candidate#3: No further enhancement. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, confirming the following working assumption with the following modification. - For Mode-A, the multiple CSI report configurations associated with the same PUCCH resource should be associated with a same CSI-AperiodicTriggerState. - FFS: For Mode-A, the multiple CSI report configurations associated with the same CSI-AperiodicTriggerState should be associated with a same PUCCH resource. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, reuse the intra-UE multiplexing/prioritization rules of PUSCH with SP-CSI for Type-1 CG PUSCH with UEI-BR for Mode B

There are several open issues for each mode.

- Mode A and Mode B

The first issue is how to support prohibit timer of first PUCCH transmission. In the RAN1#120bis meeting, the following options were raised.

- Candidate#1: To introduce a prohibit timer for mode-A and/or mode-B

  - Option-1A: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH;

    - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer

  - Option-1B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH;

    - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer

  - If the prohibit timer is running, the first PUCCH is not allowed to be transmitted.

- Candidate#2: To introduce a time interval for mode-A and/or mode-B

  - Option-2A: For a first PUCCH transmission occasion, if there is a transmission of another second PUSCH [corresponding to the CSI report determined by the same triggering beam(s) as the first PUCCH] within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH.

  - Option-2B: For a first PUCCH transmission occasion, if there is a transmission of another first PUCCH(s) within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH.

- Candidate#3: No further enhancement.

First, to avoid frequent UEIBR, prohibit timer should be supported. Also, considering definition of time window agreed in the last meeting, Candidate#2 would not match. Then, we should focus on Candidate#1.

Observation 2

- Candidate#1 should be focused considering definition of time window agreed in the last meeting.

In our analysis, for Mode B, both Option-1A and Option-1B could work. On the other hand, for Mode A, Option-1A should be supported since large latency would happen by waiting for expiry of prohibit timer when first PUCCH is not successfully received by NW. Thus, we propose

Proposal 8

- Regarding prohibit timer for Mode A and Mode B, support Option-1A.

  - Option-1A: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH;

    - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer

The next issue is regarding acknowledge information with response to each step. First, for second UL channel of Mode A, it would depend on procedure after failure of reception by NW. There could be the following two alternatives.

- Alt1: gNB resends DL to indicate second UL channel again.

- Alt2: acknowledge information for second UL channel is introduced. If no acknowledge information is received at UE, UE will transmit first UL channel again.

For Mode A, second UL channel is dynamically scheduled by UL-grant DCI format, so Alt1 would be reasonable. With the sense, acknowledge information for second UL channel would not be needed.

For first UL channel for Mode B, as we discussed in previous meeting, acknowledge information would be needed to avoid UL collision among UEs. However, it is trade-off between UL resource overhead and latency, so whether to receive acknowledge information can be configured by NW.

For second UL channel for Mode B, if failure of beam reporting happens, UE could retransmit first UL channel after corresponding transmission of second UL channel. Moreover, if preconfigured resource of second UL channel is associated with each resource of first UL channel to have common understanding on the resource for second UL channel between UE and gNB, in case of retransmission of second UL channel, such common understanding would not be ensured. With the sense, we think acknowledge information with response to second UL channel to identify the failure by UE is needed, and if no acknowledge information is received at UE, UE should transmit first UL channel again. Thus, we propose

Proposal 9

- On acknowledge information,

  - Mode A:

    - No need acknowledge information to second UL channel.

  - Mode B:

    - Support acknowledge information to first UL channel.

      - Whether to receive acknowledge information with response to first UL channel of Mode B is configured by NW.

    - Support acknowledge information to second UL channel.

If no acknowledge information is received at UE after corresponding transmission of second UL channel, UE retransmits first UL channel.

Beam application latency reduction

The following agreement and FL proposal were made for beam application latency reduction.

RAN1#119 Agreement Study the following to reduce beam application latency after a UEI/ED beam report is sent - Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report - update TCI state(s) with the reported new beam(s) - activate new beam(s) without additional SSB reception - Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception. - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized) - Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report. - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception - Other alternatives are not precluded RAN1#120bis Proposal 5.1: Regarding the reduction of beam application latency after a UEI/ED beam report is sent, Alt-3 is supported - Alt-3: After sending a UE-initiated beam report [on Event-7], the UE should store the QCL properties of the SSB associated with the reference signal reported in the beam report, subject to UE capability. - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE activating a TCI state associated with the report RS, the UE activates new beam(s) the TCI state without additional SSB reception if all activated TCI states are associated with the SSB(s) corresponding to the reference signal reported in the beam report. - [At the reception of a subsequent reception of RRC signaling to reconfigure the reference signal in TCI-State or TCI-UL-State, the UE activates the TCI state without additional SSB reception if all reconfigured TCI states are associated with the SSB(s) associated with the reference signal reported in the beam report.] Send an LS to RAN4 to capture the above

First, from limited time perspective, minimum enhancements should be considered. With this sense, Alt-1 is not preferred since it would need heavy standard effort. Second, to ensure gain, beam for which additional SSB reception is not needed to activate needs to be always included in the report. With this sense, Alt-2 is not preferred since whether to get gain may be up to UE even if additional field is included in the report. Then, Alt-3 is preferred. Also, in the last meeting, we discussed applicable event type and cases as shown in the FL proposal above. For applicable event type, only Event-7 was focused. However, this restriction should not be introduced since this function would be useful for other events, and there would be no different spec. impact. For applicable cases, “if all activated TCI states are associated with the SSB(s) corresponding to the reference signal reported in the beam report” was introduced as condition. However, this condition would so restrictive. Thus, we propose

Proposal 10

- To reduce beam application latency after a UEIBR is sent, support original Alt-3.

  - Alt3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report.

    - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception

- Enhancements for asymmetric DL sTRP/UL mTRP scenarios

PRACH configuration associated with additional PCI

It was agreed in RAN1#120 meeting that for a UE provided with SSB-MTC-AddtionalPCI and not configured with multi-DCI based mTRP when bit field index 1 of the PRACH association indicator field is indicated the PRACH configuration associated with additional PCI is used. However, it is not captured the latest TS 38.212 v19.1.0.

Agreement For a UE provided with SSB-MTC-AddtionalPCI and not configured with multi-DCI based mTRP, support to reuse the DCI field ‘PRACH association indicator’ in DCI format 1_0 to indicate PL RS for PDCCH-order PRACH: - The bit field index 0 of this field indicates the DL RS that DMRS of PDCCH order DCI is QCLed with is used as PL RS for PRACH; - The bit field index 1 of this field is mapped to the additional PCI associated with the active TCI states and indicates the indicated SSB in this DCI is used as PL RS for PRACH: - In this case, the PRACH configuration associated with addition PCI is used. - This DCI field is present when the corresponding RRC parameter is configured and multi-DCI based mTRP is not configured.

Proposal 11

  - Adopt the following TP for TS 38.212.

    - Reason for change: It was agreed that if a UE is provided with tag2-Id and SSB-MTC-AddtionalPCI and not configured with multi-DCI based mTRP, bit field index 1 of PRACH association indicator field indicates the PRACH configuration associated with additional PCI is used. However, it is not captured in the specification.

    - Summary of change: Clarify that if a UE is provided with tag2-Id and SSB-MTC-AddtionalPCI and not configured with multi-DCI based mTRP, bit field index 1 of PRACH association indicator field indicates PRACH configuration associated with additional PCI is used, and bit field index 0 of PRACH association indicator field indicates PRACH configuration associated with serving cell PCI is used.

    - Consequences if not approved: The specification leads to misunderstanding on whether PRACH configuration associated with serving cell PCI or additional PCI is used if a UE is provided with tag2-Id and SSB-MTC-AddtionalPCI and not configured with multi-DCI based mTRP.

## 7.3.1.2.1 Format 1_0 < Unchanged parts are omitted > - PRACH association indicator - 0 or 1 bit - 1bit if the UE is provided with tag2-Id, and the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs. This field is reserved if the cell indicated by Cell indicator field is a candidate cell. - This field indicates the PCI associated with the PRACH transmission if the UE is provided SSB-MTC-AddtionalPCI. The bit field index 0 of this field is mapped to the PCI of the serving cell, and the bit field index 1 of this field is mapped to the additional PCI associated with active TCI states. - This field indicates the PL-RS for the PRACH transmission if the UE is not provided SSB-MTC-AddtionalPCI. The bit field index 0 of this field is mapped to the DL RS that the DM-RS of the PDCCH order is quasi-collocated with, and the bit field index 1 of this field is mapped to the SS/PBCH indicated by the SS/PBCH index field in this DCI format. - 1bit if the UE is provided with tag2-Id and SSB-MTC-AddtionalPCI, and the UE is not configured with coresetPoolIndex or the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided, and the UE is provided with PrachAssociationIndicator_InDCI_format_1_0, and if the UE is not provided with pl-Offset for any of TCI-State or TCI-UL-State. This field is reserved if the cell indicated by Cell indicator field is a candidate cell. - This field indicates the PCI associated with the PRACH transmission. The bit field index 0 of this field is mapped to the PCI of the serving cell, and the bit field index 1 of this field is mapped to the additional PCI associated with active TCI states. - This field indicates the PL-RS for the PRACH transmission. The bit field index 0 of this field is mapped to the DL RS that the DM-RS of the PDCCH order is quasi-collocated with, and the bit field index 1 of this field is mapped to the SS/PBCH indicated by the SS/PBCH index field in this DCI format. ## - 0 bit otherwise. < Unchanged parts are omitted >

The condition when UE is not configured with multi-DCI based mTRP

The condition when UE is not configured with multi-DCI based mTRP is captured in TS38.212 as below.

7.3.1.2.1 Format 1_0 ## *** Unchanged parts are omitted *** - PRACH association indicator - 0 or 1 bit ## *** Unchanged parts are omitted *** - 1bit if the UE is provided with tag2-Id and SSB-MTC-AddtionalPCI, and the UE is not configured with coresetPoolIndex or the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided, and the UE is provided with PrachAssociationIndicator_InDCI_format_1_0, and the UE is not provided with pl-Offset for any TCI-State in dl-OrJointTCI-StateList or any TCI-UL-State in ul-TCI-State-List. This field is reserved if the cell indicated by Cell indicator field is a candidate cell. ## *** Unchanged parts are omitted ***

In Rel-19 2TA, if UE operates with two TAGs and is not configured with multi-DCI based mTRP, UE can be provided a single value of NTA,offset. It is captured in the latest TS 38.213 v19.1.0 as below. However, the condition representing UE is not configured with multi-DCI based mTRP is not captured accurately. The current specification only captures the condition “if UE is not provided coresetPoolIndex”. The condition “if the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided” is missing.

## 4.2 Transmission timing adjustments ## *** Unchanged parts are omitted *** If a UE operates with two TAGs on a serving cell and is not provided coresetPoolIndex, the UE can be provided a value NTA,offset by n-TimingAdvanceOffset for transmissions with first and second spatial filters associated with first and second joint TCI states provided by dl-OrJointTCI-StateList or with first and second UL TCI states provided by ul-TCI-State-List, respectively. ## *** Unchanged parts are omitted ***

Proposal 12

  - Adopt the following TP for TS 38.213.

    - Reason for change: If a UE operates with two TAGs and is not configured with multi-DCI based mTRP, UE can be provided a single value of NTA,offset. A condition representing UE is not configured with multi-DCI based mTRP, i.e., if the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided, is not captured in spec.

    - Summary of change: Clarify that if a UE operates with two TAGs on a serving cell and if the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided, UE can be provided a value of 𝑁TA,offset by n-TimingAdvanceOffset for transmissions with first and second spatial filters associated with first and second joint TCI states.

    - Consequences if not approved: The specification leads to misunderstanding on how 𝑁TA,offset is provided when if a UE operates with two TAGs on a serving cell and if the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided.

## 4.2 Transmission timing adjustments ## *** Unchanged parts are omitted *** If a UE operates with two TAGs on a serving cell and is not provided coresetPoolIndex or the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided, the UE can be provided a value NTA,offset by n-TimingAdvanceOffset for transmissions with first and second spatial filters associated with first and second joint TCI states provided by dl-OrJointTCI-StateList or with first and second UL TCI states provided by ul-TCI-State-List, respectively. ## *** Unchanged parts are omitted ***

Similar issue as in proposal 2 exists in TS 38.214. If a UE is not configured with multi-DCI based mTRP and if the UE is configured with two TAGs, each TCI-State or TCI-UL-State is associated with a TAG for determining timing adjustment for a corresponding UL transmission. The condition representing UE is not configured with multi-DCI based mTRP is not captured accurately. The current specification only captures the condition “if UE is not provided coresetPoolIndex”. The condition “if the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided” is missing.

Proposal 13

  - Adopt the following TP for TS 38.214.

    - Reason for change: If a UE is configured with two TAGs and is not configured with multi-DCI based mTRP, each TCI-State or TCI-UL-State is associated with a TAG for determining timing adjustment for a corresponding UL transmission. A condition representing UE is not configured with multi-DCI based mTRP, i.e., if the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided, is not captured in spec.

    - Summary of change: Clarify that if a UE configured with two TAGs and if the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided, each TCI-State or TCI-UL-State is associated with a TAG for determining timing adjustment for a corresponding UL transmission.

    - Consequences if not approved: The specification leads to misunderstanding on how to determine timing adjustment for a corresponding UL transmission if a UE is configured with two TAGs and if the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided.

## 6.1 UE procedure for transmitting the physical uplink shared channel ## *** Unchanged parts are omitted *** If a UE is not configured with coresetPoolIndex or the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided for the active BWP of a serving cell, and if the UE is configured with tag2-Id and is configured with dl-OrJointTCI-StateList or TCI-UL-State for a serving cell, each TCI-State or TCI-UL-State is associated with a tag-Id-ptr for determining timing adjustment for a corresponding UL transmission as described in Clause 4.2 of [6, TS 38.213]. *** Unchanged parts are omitted ***

- Conclusion

In this contribution, we discussed the enhancements for UE-initiated/event-driven beam management. Based on the discussion, we made following observations and proposals.

Enhancements for UE-initiated/event-driven beam management

Proposal 1

- On RS measurement for new beam, do not support Option-3b/3c.

  - Option-3b: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s).

  - Option-3c: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of TCI state(s) in a configured subset of the legacy RRC-configured TCI state list.

Proposal 2

- For RS resources in a RS resource set for UEIBR, support MAC CE to update RS resource(s) in a RS resource set for UEIBR.

Proposal 3

- For resetting conditions of counter, support at least following all conditions.

  - Candidate#5: The time window expires;

  - Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured or update by MAC CE (if supported).

Proposal 4

- When the counter and timer is configured for Event-7, support Option-1.

  - Option-1: The event instance(s) counting is per new beam (as Event-2).

Proposal 5

- Regarding report format for event-2, the following condition should be limited to Mode B.

  - “at least one of N reported beam(s) should satisfy the condition of Event-2”.

Observation 1

- RAN1 needs to consider the solution (i.e., other than additional report contents) to achieve the use case of Event-7.

Proposal 6

- For report format of Event-7, to identify the TCI states to be activated/deactivated by NW, support the following:

  - RSs configured in current activated TCI states should be configured in new beam RS configuration.

  - When N RSs are configured to be reported in a single report instance,

    - (N-X) new beam RSs and X RSs are reported.

    - X is determined by UE based on the measurement results. i.e., up to UE implementation

    - The X RSs are derived from the worst X current activated TCI states.

Proposal 7

- Support L1-SINR as measurement quantity.

  - For L1-SINR, RS combination configuration (i.e., CMR only or CMR+ZP-IMR or CMR+NZP-IMR) for Rel-16 L1-SINR is reused.

- Support semi-persistent CSI-RS and aperiodic CSI-RS for measurement RS configuration for UE-initiated beam report.

Observation 2

- Candidate#1 should be focused considering definition of time window agreed in the last meeting.

Proposal 8

- Regarding prohibit timer for Mode A and Mode B, support Option-1A.

  - Option-1A: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH;

    - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer

Proposal 9

- On acknowledge information,

  - Mode A:

    - No need acknowledge information to second UL channel.

  - Mode B:

    - Support acknowledge information to first UL channel.

      - Whether to receive acknowledge information with response to first UL channel of Mode B is configured by NW.

    - Support acknowledge information to second UL channel.

If no acknowledge information is received at UE after corresponding transmission of second UL channel, UE retransmits first UL channel.

Proposal 10

- To reduce beam application latency after a UEIBR is sent, support original Alt-3.

  - Alt3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report.

    - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception

Enhancements for asymmetric DL sTRP/UL mTRP scenarios

Proposal 11

  - Adopt the following TP for TS 38.212.

    - Reason for change: It was agreed that if a UE is provided with tag2-Id and SSB-MTC-AddtionalPCI and not configured with multi-DCI based mTRP bit field index 1 of PRACH association indicator field indicates the PRACH configuration associated with additional PCI is used, but it is not captured in the specification.

    - Summary of change: Clarify that if a UE is provided with tag2-Id and SSB-MTC-AddtionalPCI and not configured with multi-DCI based mTRP, bit field index 1 of PRACH association indicator field indicates PRACH configuration associated with additional PCI is used, and bit field index 0 of PRACH association indicator field indicates PRACH configuration associated with serving cell PCI is used.

    - Consequences if not approved: The specification leads to misunderstanding on whether PRACH configuration associated with serving cell PCI or additional PCI is used if a UE is provided with tag2-Id and SSB-MTC-AddtionalPCI and not configured with multi-DCI based mTRP.

## 7.3.1.2.1 Format 1_0 < Unchanged parts are omitted > - PRACH association indicator - 0 or 1 bit - 1bit if the UE is provided with tag2-Id, and the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs. This field is reserved if the cell indicated by Cell indicator field is a candidate cell. - This field indicates the PCI associated with the PRACH transmission if the UE is provided SSB-MTC-AddtionalPCI. The bit field index 0 of this field is mapped to the PCI of the serving cell, and the bit field index 1 of this field is mapped to the additional PCI associated with active TCI states. - This field indicates the PL-RS for the PRACH transmission if the UE is not provided SSB-MTC-AddtionalPCI. The bit field index 0 of this field is mapped to the DL RS that the DM-RS of the PDCCH order is quasi-collocated with, and the bit field index 1 of this field is mapped to the SS/PBCH indicated by the SS/PBCH index field in this DCI format. - 1bit if the UE is provided with tag2-Id and SSB-MTC-AddtionalPCI, and the UE is not configured with coresetPoolIndex or the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided, and the UE is provided with PrachAssociationIndicator_InDCI_format_1_0, and if the UE is not provided with pl-Offset for any of TCI-State or TCI-UL-State. This field is reserved if the cell indicated by Cell indicator field is a candidate cell. - This field indicates the PCI associated with the PRACH transmission. The bit field index 0 of this field is mapped to the PCI of the serving cell, and the bit field index 1 of this field is mapped to the additional PCI associated with active TCI states. - This field indicates the PL-RS for the PRACH transmission. The bit field index 0 of this field is mapped to the DL RS that the DM-RS of the PDCCH order is quasi-collocated with, and the bit field index 1 of this field is mapped to the SS/PBCH indicated by the SS/PBCH index field in this DCI format. ## - 0 bit otherwise. < Unchanged parts are omitted >

Proposal 12

  - Adopt the following TP for TS 38.213.

    - Reason for change: If a UE operates with two TAGs and is not configured with multi-DCI based mTRP, UE can be provided a single value of NTA,offset. A condition representing UE is not configured with multi-DCI based mTRP, i.e., if the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided, is not captured in spec.

    - Summary of change: Clarify that if a UE operates with two TAGs on a serving cell and if the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided, UE can be provided a value of 𝑁TA,offset by n-TimingAdvanceOffset for transmissions with first and second spatial filters associated with first and second joint TCI states.

    - Consequences if not approved: The specification leads to misunderstanding on how 𝑁TA,offset is provided when if a UE operates with two TAGs on a serving cell and if the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided.

## 4.2 Transmission timing adjustments ## *** Unchanged parts are omitted *** If a UE operates with two TAGs on a serving cell and is not provided coresetPoolIndex or the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided, the UE can be provided a value NTA,offset by n-TimingAdvanceOffset for transmissions with first and second spatial filters associated with first and second joint TCI states provided by dl-OrJointTCI-StateList or with first and second UL TCI states provided by ul-TCI-State-List, respectively. ## *** Unchanged parts are omitted ***

Proposal 13

  - Adopt the following TP for TS 38.214.

    - Reason for change: If a UE is configured with two TAGs and is not configured with multi-DCI based mTRP, each TCI-State or TCI-UL-State is associated with a TAG for determining timing adjustment for a corresponding UL transmission. A condition representing UE is not configured with multi-DCI based mTRP, i.e., if the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided, is not captured in spec.

    - Summary of change: Clarify that if a UE configured with two TAGs and if the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided, each TCI-State or TCI-UL-State is associated with a TAG for determining timing adjustment for a corresponding UL transmission.

    - Consequences if not approved: The specification leads to misunderstanding on how to determine timing adjustment for a corresponding UL transmission if a UE is configured with two TAGs and if the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided.

## 6.1 UE procedure for transmitting the physical uplink shared channel ## *** Unchanged parts are omitted *** If a UE is not configured with coresetPoolIndex or the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided for the active BWP of a serving cell, and if the UE is configured with tag2-Id and is configured with dl-OrJointTCI-StateList or TCI-UL-State for a serving cell, each TCI-State or TCI-UL-State is associated with a tag-Id-ptr for determining timing adjustment for a corresponding UL transmission as described in Clause 4.2 of [6, TS 38.213]. *** Unchanged parts are omitted ***

- Reference

- 3GPP RP-234007, “New WID: NR MIMO Phase 5”, RAN#102, Dec. 2023.