3GPP TSG RAN WG1 #122 bis			R1-2507816

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda item:	11.3.1

Title:	Discussion on Waveform

Source:	NTT DOCOMO, INC.

Document for:	Discussion and Decision

- Introduction

In RAN#109, the updated SID on the Study on 6G Radio has been approved [1].  Waveform study is a part of the SID based on the objective of “Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4]”.

Agreement (first agreement for 6G!!)

CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink

- Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions

- Other OFDM based waveforms are not precluded.

Agreement

CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink

- Enhancements/modifications on CP-OFDM will be studied as potential additions

- DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink

Note: proponents to identify at least the target use cases, signals/channels to use the waveform, and how the proposal is intended (if applicable) to support multiplexing with CP-OFDM, including MRSS, and how multi-user multiplexing is supported, etc.

Note:

Proponents are encouraged to provide more detailed information on their proposals for the next meeting, e.g.:

Proponents to characterize the main motivation for modification/additional waveform proposals:

- Targeted link direction, i.e. DL, UL or both

- Targeted use case (e.g. NTN, specific frequency range, etc.), if any

- Potential motivations metrics used, and quantified gains for a proposal, e.g.

  - Coverage

  - Network energy efficiency

  - UE energy efficiency

  - Spectral efficiency

  - High speed tolerance

  - Scheduling flexibility

  - Integration with ISAC

Proponents provide information on the following aspects, if applicable

- MRSS compatibility

- Target channels/signals, e.g. all channels, PxSCH only, etc.

- MIMO (SU and MU-MIMO) compatibility

- Target modulations, and impact to other modulations, if applicable

- Multi-user multiplexing/scheduling flexibility

- Multiplexing/coexistence with baseline waveforms

- Impact on synchronization and initial access

- Expected specification impact

- Transmitter/receiver complexity and impact to power consumption.

In the previous RAN1#122, several topics were discussed for 6GR waveform, including NR baseline, low peak-to-average power ratio (PAPR) waveform, Doppler-robust waveform, waveform for sensing, etc. The following agreements regarding the basis uplink and downlink waveform of 6GR are as follows [2]. The enhancements and other waveform schemes are still FFS.

In this document, we firstly discuss several waveform candidates proposed in the literature for UL coverage enhancement. We aim at developing a common technical understanding of the pros and cons of the waveform candidates by means of theoretical analysis and initial link-level evaluations. Then, we show our views on other waveform candidates, multi-layer DFT-s-OFDM for UL, DFT-s-OFDM for DL, and OTFS, etc.

- NR waveform status

In this section, the 5G NR waveform status is introduced. 5G NR adopts a multicarrier waveform orthogonal frequency division multiplexing with cyclic prefix (CP-OFDM) with high spectral efficiency as the core waveform for both DL and UL, and supports single carrier waveform discrete Fourier transform spreading OFDM (DFT-s-OFDM) with high power efficiency only for UL, targeting the scenario where uplink coverage is limited [3].

2.1. CP-OFDM

The transmitter block diagram of CP-OFDM is shown in Figure 1. CP-OFDM is widely adopted in 5G NR uplink and downlink because it offers high spectral efficiency, robustness against severe multipath fading in wideband transmissions, and strong compatibility with multiple-input multiple-output (MIMO) systems.

However, when considering the influence of power amplifier (PA) non-linearity, CP-OFDM will exhibit signal distortion and performance degradation due to its high PAPR. Although a crest factor reduction (CFR) module can be used for CP-OFDM to reduce the PAPR by transmitting signal clipping and filtering, the computational complexity of CFR is very high due to the iterative clipping and filtering process[4].

Figure 1. CP-OFDM transmitter block diagram

2.2. DFT-s-OFDM

DFT-s-OFDM is also adopted in the 5G NR UL when coverage is limited due to the low PAPR performance. The transmitter block diagram of DFT-s-OFDM is shown in Figure 2. The time-domain modulated symbols are transformed into the frequency domain based on the DFT spreading module.

According to [5], DFT-s-OFDM can be considered as a scheme that upsamples the modulated symbols by a factor equal to the ratio of the inverse fast Fourier transform (IFFT) and DFT block sizes and applies a circular-shaped pulse before the CP insertion. The circular-shaped pulse of DFT-s-OFDM is the Dirichlet sinc function. However, the heavily overlapped pulse-sidelobes of the Dirichlet sinc pulses can create large peaks in the time domain when combined coherently, which also results in poor PAPR performance.

Figure 2. DFT-s-OFDM transmitter block diagram

2.3. DFT-s-OFDM with FDSS

DFT-s-OFDM with frequency domain spectrum shaping (FDSS) can reduce the PAPR by introducing a well-designed low-complexity frequency-domain filter to change the original time-domain Dirichlet sinc function of DFT-s-OFDM [6]. The transmitter block diagram of DFT-s-OFDM with FDSS is shown in Figure 3.

Figure 3. DFT-s-OFDM with FDSS transmitter block diagram

FDSS combined with the low PAPR modulation pi/2 phase-rotated binary phase-shift keying (pi/2-BPSK) for DFT-s-OFDM has been adopted in NR since R15 for uplink coverage-limited scenarios. In R18, FDSS combined with the quadrature PSK (QPSK) for DFT-s-OFDM has also been supported for NR uplink coverage enhancement [7].

Although DFT-s-OFDM with FDSS can reduce the PAPR, the resulting inter-symbol interference (ISI) will impact the quality of the received signal.

- High-level view on 6GR waveform

In this section, we describe our high-level view on how RAN1 should proceed with 6GR waveform study.

First of all, we believe it is important to follow what 6GR SID describes, that is, “Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals”. In other words, we believe RAN1 can even preclude any waveform candidates which is not OFDM-based, to make the study scope reasonable. Such a limited scope for waveform is important because 1) too long discussions on waveform (or other PHY fundamental aspects) will result in minimization of available discussion time for other important functionalities in 6GR, which should be avoided, and 2) given that 6GR is likely to share a band with 5G NR (i.e., MRSS), high commonality for PHY fundamental aspects between 5G NR and 6GR is strongly preferred.

Then, even for OFDM-based waveforms, we believe a clear justification (e.g., large gain achievable in practical deployment scenarios) would be necessary. During our experience of 5G NR operation in real field, we haven’t identified a significant drawback from 5G NR waveforms. Therefore, at this stage, we believe keeping 5G NR waveforms in 6GR as they are should still be a valid way-forward, instead of focusing how to change.

Another important aspect is maximized unification with non-eMBB scenario. For example, 6GR SID clearly describes that “j)	Aim at a harmonized 6G Radio design for TN and NTN, including their integration”, which we believe is important for 3GPP devices to be more competitive against another NTN system in terms of e.g., minimizing development cost, seamless connectivity with 3GPP TN networks, etc. We believe waveforms for TN and NTN shouldn’t be fragmented, rather, a unified waveform set applicable to (and practically workable in) both TN and NTN scenarios should be pursued.

Having said above, there are a few aspects that we are interested in to analyse during 6GR SI. For example, we think it would be great if 6GR can reduce our deployment cost compared with 5G NR, and for this target, it could be preferred if coverage and spectrum efficiency by a single site is extended further. To address this issue, during the study, we think low PAPR waveform with higher spectrum efficiency performance can be considered as one of the discussion directions for potential new waveform in 6GR. In section 5, we have considered  several waveform schemes that can be considered to improve PAPR performance, such as DFT-s-OFDM with FDSS and spectrum extension (FDSS-SE) and DFT-s-OFDM with FDSS and frequency-domain cyclic extension (FDSS-CE). What’s more, for further PAPR improvement, the candidate modulation scheme with QPSK rotation is also considered. Pi/2-BPSK modulation is adopted in NR for cell-edge UEs due to the low PAPR, however the spectrum efficiency is limited.  In Section 5, we have also considered the waveform schemes that can be considered to improve the spectrum efficiency performance of pi/2-BPSK modulation, such as DFT-s-OFDM with truncated mapping and asymmetric DFT-s-OFDM.

Proposal 1: For 6GR study on waveform,

- Only OFDM-based waveform(s) should be considered (as described in the SID)

- Any new waveform(s), even for OFDM-based, should be justified by clear gain

- Unified design across scenarios/use cases is strongly preferred

- Following the above, RAN1 can carefully assess the need in 6GR to introduce waveform(s) beyond 5G NR, targeting, e.g.,

  - Potential better coverage by better PAPR performance for uplink

  - Spectrum efficiency improvement

- Evaluation metrics

In this section, the evaluation metrics for low PAPR candidate waveforms as well as QPSK rotation modulation are introduced.

4.1. Power efficiency

The lower PAPR candidates improve coverage performance by enhancing the UE maximum transmission power through improving power efficiency. Therefore, the metrics related to power efficiency should be considered.

PAPR

PAPR performance is one direct assessment metric for power efficiency. There are two different PAPR definition methods:

- Method 1: Instantaneous PAPR, which is defined as the ratio between the instantaneous squared envelope and the corresponding average power of the signal, i.e,

PAPRn=xn21/Nl=0N-1xl2,

where xn denotes the complex sample at time-instant n, N is the overall signal length in samples.

- Method 2: The PAPR is defined as the ratio between the maximum instantaneous power and the corresponding average power of the signal, i.e.,

PAPR=max0≤n≤N-1xn21/Nl=0N-1xl2.

We believe it is important to have a common understanding on which of the methods above is applied for PAPR calculation during evaluation works. Considering method 2 is more commonly used in PAPR calculation, we propose method 2 as the baseline PAPR evaluation method.

OBO/MPR

The lowest output back off (OBO) or the maximum power reduction (MPR) that satisfies the RF requirements provides a more comprehensive assessment for power efficiency, which jointly considers both the nonlinear PA impact and the PAPR distribution.

The OBO or MPR is calculated as

OBOdB=PsatdBm-PoutdBm,

where Psat and Pout are the PA saturation power and actual mean output power. The RF requirements such as adjacent channel leakage ratio (ACLR), error vector magnitude (EVM), occupied bandwidth (OBW), in-band emissions (IBE), and so on, should be considered for the OBO/MPR evaluation.

4.2. Link performance

A solution for power efficiency (UE maximum transmission power) improvement may have a drawback about its link-level performance loss. Therefore, such performances, e.g., block error rate (BLER), should also be evaluated.

4.3. Coverage performance

The coverage performance for the low PAPR candidates should jointly consider both the power efficiency and link performance. The net gain metric, as used in 5G NR, is used to evaluate the coverage gain.

Considering there are two power efficiency metrics, i.e., PAPR and OBO(MPR), there are two net gain calculation methods:

- Method A: Net gain related to PAPR

Net gain GAdB=∆PAPRdB+∆SNRdB

- Method B: Net gain related to OBO

Net gain GBdB=∆OBOdB+∆SNRdB

where GA  and GB are the net gain compared with the baseline, e.g., 5G DFT-s-OFDM.  ∆PAPR  is the PAPR gain for a given complementary cumulative distribution function (CCDF) value (e.g.,  0.01% CCDF), ∆SNR is the SNR gain for a given BLER requirement (e.g., 0.1), and ∆OBO  is the OBO gain under the RF reuqirements. For differernt low PAPR candicate waveforms, the PAPR/SNR/OBO performance should be evaluated under a given required spectral efficiency with the same occupied time and frequency domain resources..

Observation 1: Both the PAPR and net gain under a given required spectral efficiency, with the same occupied time and frequency domain resources, should be considered for low PAPR waveform evaluation.

- Low PAPR candidates for 6GR UL

  - Motivation for UL PAPR performance improvement

Now that mobile communication has become an important infrastructure for a lot of people, MNO’s responsibility is getting more and more important to ensure connectivity in wider area. To achieve this, we have deployed a bunch number of BSs, spending a lot of cost and effort. It would always be important to reduce the sources of such a cost and effort. Extension of site coverage is a promising approach since per-area BSs can be reduced without having a coverage hole. Note that, to practically achieve this, it is essential to make sure that such an extended coverage is supported from the perspective of any UE, not only NW.

A typical approach to extend coverage is repetition of UL signal/channel, which have widely been implemented and operated in real field already. While we believe this is still promising in 6GR (and so we are proposing to extend the specification support of repetition from 6G Day 1), there is indeed a drawback, e.g., UL resource efficiency. Especially for TDD band, since it isn’t easy (even not practical) to increase UL ratio, this drawback might be a significant bottleneck for the whole UL performance.

To satisfy the following 6G requirements as listed in 6GR SID [1]:

- Enhanced overall coverage, focus on cell-edge performance and UL coverage.

- Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

- Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

Low PAPR enhancement based on NR waveforms with better coverage may be necessary for 6GR, especially for UL. Meanwhile, we believe it should be carefully discussed together with other perspectives, such as the achievable net gains, link budget, and system-wise performance.

Observation 2: Wider coverage may be needed for 6G

- For 6GR waveform, PAPR performance improvement for better coverage, especially for the uplink may be necessary for 6GR

- It should be noted that several other aspects should be considered together with PAPR performance, such as achievable net gains, link budget, and system-wise performance when it is deployed

  - Candidate waveforms for PAPR reduction based on DFT-s-OFDM

DFT-s-OFDM-based enhancements with spectrum extension (SE) can increase the time separation between adjacent time-domain pulse shapes, thereby reducing ISI. Furthermore, the PAPR is further reduced by the SE operation. Both symmetric SE and asymmetric SE can be considered.

    - FDSS-SE

FDSS-SE refers to the DFT–s–OFDM enhancement with symmetric SE. It was first proposed in [8]. The transmitter block diagram of FDSS-SE is shown in Figure 4.

Figure 4. FDSS-SE transmitter block diagram

The symmetric SE is achieved by introducing the SE module that follows the DFT spreading module.  Within the allocated M frequency-domain subcarriers, there are only Q≤M subcarriers carrying data. The other M-Q subcarriers are the excess band to realize symmetric SE, and α=MQ  refers to the SE factor.

According to the analysis in [5], the time separation between time-domain shaped pulses of DFT-s-OFDM increases based on the symmetric SE, thereby reducing ISI. Additionally, the PAPR is further reduced as the adjacent pulses have larger time separation. However, an equivalent ejπα-1m phase rotation is also introduced for the m-th time-domain shaped-pulses by FDSS-SE, which will affect the PAPR performance. Moreover, when an odd number of RBs is allocated for SE, FDSS-SE imposes a fractional-RB extension on each side.  Another potential issue for FDSS-SE could be that the double-sided SE increases implementation complexity.

    - FDSS-CE

FDSS-CE refers to the DFT-s-OFDM enhancement with asymmetric SE (i.e., cyclic extension in an asymmetric manner). It was first proposed in [9]. The transmitter block diagram of FDSS-CE is shown in Figure 5.

Figure 5. FDSS-CE transmitter block diagram

The asymmetric SE is achieved by introducing a frequency-domain cyclic extension (FD-CE) module that follows the DFT spreading module.  Similar to FDSS-SE, within the allocated M frequency-domain subcarriers, there are only Q≤M subcarriers carrying data. The other M-Q subcarriers are the excess band to realize asymmetric SE.

Like FDSS-SE, the time separation between time-domain shaped pulses of DFT-s-OFDM increases based on the asymmetric SE, thereby reducing ISI. Additionally, the PAPR is further reduced as the adjacent pulses have larger time separation [9]. The key distinction is that in FDSS-CE with asymmetric SE, no additional phase rotation is applied, leading to better PAPR performance compared to FDSS-SE. Another potential advantage of FDSS-CE is that it can do asymmetric SE straightforwardly whenever the number of RBs allocated for SE is odd or even. Furthermore, the serial implementation method (similar to the operation of adding CP) for asymmetric SE requires lower implementation complexity.

5.2.3. Consideration of FDSS-SE and FDSS-CE

The considerations of FDSS-SE and FDSS-CE on the following aspects are provided:

- MRSS compatibility

  - The same MRSS compatibility as the NR baseline waveform DFT-s-OFDM. FDSS-SE and FDSS-CE can degrade to legacy DFT-s-OFDM by setting the SE factor to 1

- Target channels/signals, e.g. all channels, PxSCH only, etc.

  - Target channel is PUSCH, and if necessary, FDSS-SE/FDSS-CE can also be applied to other UL channels/signals

- MIMO (SU and MU-MIMO) compatibility

  - The same MIMO (SU and MU-MIMO) compatibility as the NR baseline waveform DFT-s-OFDM

- Target modulations and impact on other modulations, if applicable

  - All modulations. FDSS-SE and FDSS-CE can be further combined with QPSK rotation, as described in section 4.3, to achieve additional PAPR reduction

- Multi-user multiplexing/scheduling flexibility

  - The same flexibility as the NR baseline waveform DFT-s-OFDM

- Multiplexing/coexistence with baseline waveforms

  - Can easily multiplex/coexist with baseline waveforms

- Impact on synchronization and initial access

  - No impact

- Expected specification impact

- Decoupling DFT precoding size and allocated RB number, FDRA indicator, etc.

- Transmitter/receiver complexity and impact to power consumption.

  - No additional transmitter and receiver complexity compared to the NR baseline waveform DFT-s-OFDM

  - Lower PA back-off with higher PA efficiency to reduce power consumption for a given transmission power and power class

Observation 3: DFT-s-OFDM enhancement with SE, including FDSS-SE and FDSS-CE, can reduce PAPR performance to achieve better coverage with a small impact, which can be considered the waveform candidates for 6G uplink

Observation 4: FDSS-CE has no requirement for the number of RBs used for SE, and requires lower implementation complexity than FDSS-SE based on the serial implementation method for asymmetric SE

  - QPSK enhancement with constellation rotation for PAPR reduction

5G NR supports pi/2 phase-rotated binary phase-shift keying (pi/2-BPSK) in UL when DFT-s-OFDM is employed for the scenarios with coverage limitations. The pi/2-BPSK is shown in Figure 6. When pi/2-BPSK is employed, the first symbol is mapped to the original BPSK constellation point, while the second is rotated by pi/2. In DFT-s-OFDM, this rotation reduces the probability that the time-domain pulse shapes of two consecutive symbols are in phase; consequently, the PAPR is lower than with conventional BPSK.

Original BPSK points       BPSK points with pi/2 phase rotation.

Figure 6. pi/2-BPSK.

Like pi/2-BPSK, the QPSK rotation can also reduce the PAPR of DFT-s-OFDM-based waveforms by reducing the in-phase probability between the two consecutive time-domain shaped pulses [5], with a potential small impact on implementation. When QPSK with phase rotation ϕ is employed, i.e.,ϕ-QPSK, the first symbol is mapped to the original QPSK constellation point, while the second symbol is rotated by phase ϕ, then the third symbol is rotated by phase 2ϕ, …, etc. Compared to pi/2-BPSK, QPSK rotation can achieve a higher data rate.   Legacy uniform QPSK demodulation algorithms can be reused for QPSK rotation, so there is no extra demodulation complexity.

Observation 5: QPSK rotation can reduce the PAPR of the DFT-s-OFDM waveform while achieving a higher data rate than pi/2-BPSK.

QPSK rotation can be combined with the DFT-s-OFDM-based enhancement waveform scheme, such as FDSS-SE and FDSS-CE, for further PAPR reduction. Different optimal phase-rotated angles for QPSK are required for different DFT-s-OFDM-based enhancement schemes.

- When combined with DFT-s-OFDM, DFT-s-OFDM with FDSS, and DFT-s-OFDM with symmetric SE enhancement, i.e., FDSS-SE, the optimal QPSK phase rotation angle is pi/4, i.e., pi/4-QPSK as shown in Figure 7(a).

- When combined with DFT-s-OFDM with asymmetric SE enhancement, i.e., FDSS-CE, the optimal QPSK phase rotation angle should depend on the SE factor α, which can be calculated based on mod{𝛼𝜋-𝜋/4, 𝜋/2}, i.e., mod{𝛼𝜋-𝜋/4, 𝜋/2}-QPSK. For example, when α=43, pi/12-QPSK can achieve the optimal PAPR performance for FDSS-CE, as shown in Figure 7 (b).

(a) pi/4-QPSK

(b) pi/12-QPSK with α=4/3

Original QPSK points      QPSK points with phase rotation

Figure 7. Examples for QPSK rotation.

Similar to pi/2-BPSK, the legacy uniform QPSK demodulation algorithm can be reused for QPSK rotation without additional complexity. The expected spec. impacts include (1) Predefine one and/or several QPSK rotation constellations and/or phase rotation angles at transmitter and/or receiver; (2) updating the MCS table with QPSK rotation for low PAPR conditions.

Observation 6: QPSK rotation can be combined with DFT-s-OFDM, DFT-s-OFDM with FDSS and/or SE enhancement for further PAPR reduction.

  - Candidate waveforms for pi/2-BPSK spectrum efficiency improvement

DFT-s-OFDM based on pi/2-BPSK can achieve better coverage than based on QPSK due to the lower PAPR. However, the spectrum efficiency for pi/2-BPSK is lower. The spectrum efficiency of pi/2-BPSK can be improved based on frequency-domain truncation (FDT). This is thanks to a certain frequency domain symmetry property of DFT-s-OFDM waveforms with pi/2-BPSK, which can be expressed as:

- For the BPSK modulation based on the assumption of sBPSKm∈1, -1, there is

Sk =SL2-k*,                 0≤k≤L2S3L2-k*,L2+1≤k≤L-1

- For the BPSK modulation based on the assumption of sBPSKm∈121+j, -1-j, there is

Sk =ejπ2SL2-k*,            0≤k≤L2ejπ2S3L2-k*, L2+1≤k≤L-1

Where Sk are the frequency domain samples and L is the DFT size.

So within in L DFT output frequency domain samples, up to L2-1 DFT samples can be dropped based on FDT to increase the spectrum efficiency of pi/2-BPSK without any loss of information. However, the PAPR performance loss is introduced due to the FDT. The larger the FDT truncation size/ratio, the larger the PAPR performance loss [11]. Therefore, FDT truncation size/ratio should be designed carefully to achieve a tunable trade-off between PAPR and spectrum efficiency. There are two methods to achieve FDT, as described as follows.

    - DFT-s-OFDM with truncated mapping

DFT-s-OFDM with truncated mapping realizes FDT based on the frequency-domain truncated mapping after asymmetric DFT spreading, as shown in Figure 8. The symmetric DFT spreading requires higher computational complexity because L-to-L point DFT spreading is required.

Figure 8. DFT-s-OFDM with truncated mapping

    - Asymmetric DFT-s-OFDM

Asymmetric DFT-s-OFDM realizes FDT by replacing the symmetric DFT spreading with asymmetric DFT spreading whose output size K is smaller than the input size L, as shown in Figure 9.

Figure 9. Asymmetric DFT-s-OFDM

The element fk,l in the k-th row and l-th column of the asymmetric DFT matrix can be represented as

fk,l=e-j2πlk+EL, 0≤k≤K-1, ≤l≤L-1

Where E is the FDT size at the head of frequency-domain samples. To avoid information loss, the value of E should satisfy E≤L4. Thanks to the L-to-K point asymmetric DFT spreading, the computational complexity is lower than that of DFT-s-OFDM with truncated mapping.

Observation 7: DFT-s-OFDM with frequency domain truncation (FDT) can achieve a tunable trade-off between PAPR and spectrum efficiency

  - Initial evaluation results

This section evaluates and compares candidate waveforms for PAPR reduction based on DFT-s-OFDM, including FDSS-SE and FDSS-CE, with the NR baseline waveform (DFT-s-OFDM with and without FDSS) under different assumptions, using various evaluation metrics. The general simulation parameters for different evaluation assumptions are given in Table A- 1 in the Appendix.

    - Power efficiency performance

Case 1: PAPR performance under different SE factors

To analyse the relationship between the SE factor α of FDSS-SE/FDSS-CE and achievable PAPR gain compared to the NR baseline DFT-s-OFDM, the PAPR performance under different SE factors is evaluated based on the assumption of same spectrum efficiency, same #PRBs.

The specific simulation parameters for this ccase are given in Table A- 2 in the Appendix. The complementary cumulative distribution function (CCDF) of PAPR for different waveforms under various SE factors is shown in Figure 10 (a) for QPSK modulation and in Figure 10 (b) for QPSK rotation, respectively. The PAPR gain compared to NR baseline DFT-s-OFDM with QPSK at the 0.01% CCDF point are shown in Table 1.

- QPSK

- QPSK rotation

Figure 10. PAPR performance with different SE factors

Table 1. PAPR gain vs. DFT-s-OFDM with QPSK

| alpha Waveform |  | 1 | 6/5 | 5/4 | 4/3 | 8/5 | 5/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM with FDSS | QPSK | 3.41dB |  |  |  |  |  |  |
| FDSS-SE |  |  | 4.26dB | 4.42dB | 4.56dB | 5.50dB |  | 5.54dB |
| FDSS-CE |  |  | 4.68dB | 4.90dB | 4.81dB | 5.56dB |  | 5.62dB |
| DFT-s-OFDM with FDSS | QPSK rotation | 3.92dB |  |  |  |  |  |  |
| FDSS-SE |  |  | 4.77dB | 4.93dB | 5.12dB | 5.67dB |  | 5.57dB |
| FDSS-CE |  |  | 4.77dB | 4.93dB | 5.12dB | 5.67dB |  | 5.57dB |

Compared to DFT-s-OFDM with QPSK under QPSK modulation, it can be found that

- DFT-s-OFDM with FDSS achieves 3.41dB PAPR gain

- FDSS-SE and FDSS-CE achieve larger PAPR gain than DFT-s-OFDM with FDSS

  - The PAPR gain increases with the increase of alpha.

  - When alpha = 5/3, the maximum PAPR gain is 5.54dB and 5.62dB, respectively.

- FDSS-CE achieves a larger PAPR gain than FDSS-SE for a given SE factor

  - The maximum PAPR gain vs. FDSS-SE is 0.5dB at alpha = 5/4.

When further combined with QPSK rotation, it can be found that

- DFT-s-OFDM with FDSS achieves 3.92dB PAPR gain, which is 0.5dB larger than QPSK

- FDSS-SE and FDSS-CE achieve the same PAPR performance under QPSK rotation.

  - The PAPR first increases, then decreases, and finally increases again with the increase of alpha

  - When alpha = 8/5, the maximum PAPR gain is 5.57dB

Case 2: PAPR performance under different bandwidths

To analyse the relationship between the exact bandwidths of FDSS-SE/FDSS-CE and achievable PAPR gain compared to the NR baseline DFT-s-OFDM, the PAPR performance under different exact bandwidths is evaluated.

The evaluation is based on the assumption of the same spectrum efficiency, the same SE factor with different #PRBs. The specific simulation parameters are given in Table A- 3 in the Appendix. The PAPR gain compared to DFT-s-OFDM with QPSK at 0.01% CCDF point for alpha =6/5 and alpha=5/3 are shown in Table 2 (a) and Table 2 (b), respectively.

Table 2. PAPR gain vs. DFT-s-OFDM with QPSK under different exact bandwidths

- alpha=6/5

| Bandwidth Waveform |  | 6PRB | 12PRB | 30PRB |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM with FDSS | QPSK | 2.5dB | 2.7dB | 3.2dB |
| FDSS-SE (alpha=6/5) |  | 3.0dB | 3.8dB | 4.0dB |
| FDSS-CE(alpha=6/5) |  | 3.5dB | 4.0dB | 4.4dB |
| DFT-s-OFDM with FDSS | QPSK rotation | 2.7dB | 3.3dB | 2.7dB |
| FDSS-SE (alpha=6/5) |  | 3.65dB | 4.1dB | 4.5dB |
| FDSS-CE (alpha=6/5) |  | 3.65dB | 4.1dB | 4.5dB |

- alpha=5/3

| Bandwidth Waveform |  | 5PRB | 15PRB | 30PRB |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM with FDSS | QPSK | 2.3dB | 2.9dB | 3.1dB |
| FDSS-SE (alpha=5/3) |  | 5.2dB | 5.2dB | 5.4dB |
| FDSS-CE (alpha=5/3) |  | 5.3dB | 5.3dB | 5.5dB |
| DFT-s-OFDM with FDSS | QPSK rotation | 2.8dB | 3.3dB | 3.7dB |
| FDSS-SE (alpha=5/3) |  | 5.3dB | 5.3dB | 5.5dB |
| FDSS-CE (alpha=5/3) |  | 5.3dB | 5.3dB | 5.5dB |

Compared to DFT-s-OFDM with QPSK under QPSK modulation, it can be found that

- For DFT-s-OFDM with FDSS, the PAPR gain is no less than 2.3dB for any bandwidth

- For FDSS-SE, the PAPR gain is no less than 3.0dB and 5.2dB for any bandwidth for alpha=6/5 and alpha=5/3, respectively

- For FDSS-CE, the PAPR gain is no less than 3.5dB and 5.3dB for any bandwidth for alpha=6/5 and alpha=5/3, respectively

When further combined with QPSK rotation, it can be found that

- For DFT-s-OFDM with FDSS, the PAPR gain is no less than 2.7dB for any bandwidth

- For FDSS-SE/FDSS-CE, the PAPR gain is no less than 3.65dB and 5.3dB for any bandwidth for alpha=6/5 and alpha=5/3, respectively

Observation 8: FDSS-SE, FDSS-CE can achieve a larger PAPR gain than DFT-s-OFDM with FDSS with any given SE factor and/or any given bandwidth for both QPSK and QPSK rotation

Observation 9: FDSS-CE achieves a larger PAPR gain than FDSS-SE with the same SE factor under QPSK. If further combining with QPSK rotation, FDSS-SE and FDSS-CE achieve the same PAPR performance.

Observation 10: FDSS-CE under QPSK without rotation achieves near-optimal PAPR performance

    - Link performance

Case 1: Link performance under different SE factors

To analyse the relationship between the SE factor α of FDSS-SE/FDSS-CE and the link performance loss compared to the NR baseline DFT-s-OFDM, the BLER performance under different SE factors is evaluated based on the assumption of same spectrum efficiency, same #PRBs.

The specific simulation parameters for this case are given in Table A- 2 in the Appendix. The required SNR for 10% BLER with different waveforms under different SE factors for QPSK modulation and QPSK rotation is shown in Figure 11. The SNR loss compared to the NR baseline DFT-s-OFDM under QPSK is shown in Table 3.

Figure 11. Required SNR for 10% BLER with different SE factors

Table 3. SNR loss vs. NR baseline of DFT-s-OFDM under QPSK

| alpha Waveform |  | 1 | 6/5 | 5/4 |  | 4/3 | 8/5 | 5/3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM with FDSS | QPSK | 1.3dB |  |  |  |  |  |  |
| FDSS-SE |  |  | 0.75dB |  | 0.62dB | 0.45dB | 0.3dB | 0.3dB |
| FDSS-CE |  |  | 0.65dB |  | 0.62dB | 0.50dB | 0.4dB | 0.4dB |
| DFT-s-OFDM with FDSS | QPSK rotation | 1.2dB |  |  |  |  |  |  |
| FDSS-SE |  |  | 0.7dB |  | 0.55dB | 0.45dB | 0.4dB | 0.35dB |
| FDSS-CE |  |  | 0.7dB |  | 0.55dB | 0.45dB | 0.4dB | 0.35dB |

Compared to DFT-s-OFDM with QPSK under QPSK modulation, it can be found that

- DFT-s-OFDM with FDSS results in 1.3dB SNR loss due to the ISI introduced by FDSS

- FDSS-SE and FDSS-CE result in smaller SNR loss due to the ISI being reduced based on spectrum extension, as well as the spectrum extension is utilized at the receiver side for detection

  - The maximum SNR loss is 0.75dB and 0.65dB at alpha = 6/5, respectively

- The SNR loss difference between FDSS-SE and FDSS-CE is no larger than 0.1dB for any SE factor

When further combined with QPSK rotation, it can be found that

- QPSK rotation has very little impact on link performance

- SNR loss difference between QPSK rotation and QPSK is no larger than 0.1dB for any waveform under any SE factor

Case 2: Link performance under different bandwidths

To analyse the relationship between the exact bandwidths of FDSS-SE/FDSS-CE and the link performance loss compared to the NR baseline DFT-s-OFDM, the BLER performance under different exact bandwidths is evaluated.

The evaluation is performed based on the assumption of same spectrum efficiency, same SE factor with different #PRBs. The specific simulation parameters are given in Table A- 3 in the Appendix. The SNR loss compared to the NR baseline DFT-s-OFDM under QPSK for 10% BLER for  alpha=6/5 and alpha=5/3 is shown in Table 3 (a) and Table 3 (b), respectively.

Table 4. SNR loss vs. NR baseline of DFT-s-OFDM under QPSK

- alpha=6/5

| Bandwidth Waveform |  | 6PRB | 12PRB | 30PRB |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM with FDSS | QPSK | 1.1dB | 1.4dB | 1.4dB |
| FDSS-SE (alpha=6/5) |  | 0.8dB | 0.85dB | 0.9dB |
| FDSS-CE(alpha=6/5) |  | 0.8dB | 0.95dB | 0.8dB |
| DFT-s-OFDM with FDSS | QPSK rotation | 1.1dB | 1.3dB | 1.4dB |
| FDSS-SE(alpha=6/5) |  | 0.7dB | 0.95dB | 0.95dB |
| FDSS-CE(alpha=6/5) |  | 0.7dB | 0.95dB | 0.95dB |

- alpha=5/3

| Bandwidth Waveform |  | 5PRB | 15PRB | 30PRB |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM with FDSS | QPSK | 0.95dB | 1.55dB | 1.4dB |
| FDSS-SE (alpha=6/5) |  | 0.4dB | 1.1dB | 0.65dB |
| FDSS-CE(alpha=6/5) |  | 0.5dB | 1.05dB | 0.7dB |
| DFT-s-OFDM with FDSS | QPSK rotation | 0.95dB | 1.45dB | 1.4dB |
| FDSS-SE(alpha=6/5) |  | 0.45dB | 1.1dB | 0.7dB |
| FDSS-CE(alpha=6/5) |  | 0.45dB | 1.1dB | 0.7dB |

Compared to DFT-s-OFDM with QPSK under QPSK modulation, it can be found that

- For DFT-s-OFDM with FDSS, the SNR loss is no smaller than 0.95dB for any bandwidth

- For FDSS-SE, the SNR loss is no larger than 0.9dB and 1.1dB for any bandwidth for alpha=6/5 and alpha=5/3, respectively

- For FDSS-CE, the SNR loss is no larger than 0.95dB and 1.01dB for any bandwidth for alpha=6/5 and alpha=5/3, respectively

When further combined with QPSK rotation, it can be found that

- QPSK rotation has very little impact on link performance

- SNR loss difference between QPSK rotation and QPSK is no larger than 0.1dB for any waveform under any bandwidth

Observation 11: FDSS-SE and FDSS-CE result in smaller SNR loss compared to DFT-s-OFDM with FDSS. The different SE methods, as well as QPSK rotation, have very little impact on link performance

    - Net gain

The net gain performance related to PAPR is evaluated and analyzed in this section.

Case 1: Net gain under different SE factors

The net gain of FDSS-SE/FDSS-CE compared to the NR baseline DFT-s-OFDM under different SE factors is evaluated based on the same spectrum efficiency, same #PRBs evaluation assumption. The specific simulation parameters for this case are given in Table A- 2 in the Appendix. The net gain with different waveforms under different SE factors under QPSK modulation and QPSK rotation is shown in Figure 12. The corresponding value is shown in Table 5.

Figure 12. Net gain related to PAPR vs. NR baseline of DFT-s-OFDM under QPSK

Table 5. Net gain related to PAPR vs. NR baseline of DFT-s-OFDM under QPSK

| alpha Waveform |  | 1 | 6/5 | 5/4 | 4/3 | 8/5 | 5/3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM with FDSS | QPSK | 2.11dB |  |  |  |  |  |
| FDSS-SE |  |  | 3.52dB | 3.81dB | 4.09dB | 5.2dB | 5.22dB |
| FDSS-CE |  |  | 4.04dB | 4.29dB | 4.32dB | 5.2dB | 5.22dB |
| DFT-s-OFDM with FDSS | QPSK rotation | 2.72dB |  |  |  |  |  |
| FDSS-SE |  |  | 4.08dB | 4.38dB | 4.67dB | 5.27dB | 5.23dB |
| FDSS-CE |  |  | 4.08dB | 4.38dB | 4.67dB | 5.27dB | 5.23dB |

Compared to DFT-s-OFDM with QPSK under QPSK modulation, it can be found that

- DFT-s-OFDM with FDSS can achieve 2.11dB net gain

- FDSS-SE and FDSS-CE achieve a larger net gain than DFT-s-OFDM with FDSS.

- FDSS-CE achieves a larger net gain than FDSS-SE for a given SE factor smaller than 8/5.

  - The maximum net gain for FDSS-SE and FDSS-CE is  5.22dB at alpha = 5/3

When further combined with QPSK rotation, it can be found that

- DFT-s-OFDM with FDSS achieves 2.72dB PAPR gain, which is 0.62dB larger than QPSK

- FDSS-SE and FDSS-CE achieve the same net gain under QPSK rotation.

  - When alpha = 8/5, the maximum net gain is 5.27dB

Case 2: Net gain under different bandwidths

The net gain of FDSS-SE/FDSS-CE compared to the NR baseline DFT-s-OFDM under different bandwidths is evaluated based on the same spectrum efficiency, same SE factor, and different #PRBs evaluation assumptions. The specific simulation parameters for this case are given in Table A- 3 in the Appendix. The net gain compared to the NR baseline DFT-s-OFDM under QPSK for alpha =6/5 and alpha=5/3 is shown in Table 6 (a) and Table 6 (b), respectively.

Table 6. Net gain related to PAPR vs. NR baseline of DFT-s-OFDM under QPSK

- alpha=6/5

| Bandwidth Waveform |  | 6PRB | 12PRB | 30PRB |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM with FDSS | QPSK | 1.4dB | 1.3dB | 1.8dB |
| FDSS-SE (alpha=6/5) |  | 2.2dB | 2.95dB | 3.05dB |
| FDSS-CE(alpha=6/5) |  | 2.7dB | 3.055dB | 3.6dB |
| DFT-s-OFDM with FDSS | QPSK rotation | 1.6dB | 2.0dB | 1.3dB |
| FDSS-SE(alpha=6/5) |  | 2.95dB | 3.15dB | 3.55dB |
| FDSS-CE(alpha=6/5) |  | 2.95dB | 3.15dB | 3.55dB |

- alpha=5/3

| Bandwidth Waveform |  | 5PRB | 15PRB | 30PRB |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM with FDSS | QPSK | 1.35dB | 1.35dB | 1.7dB |
| FDSS-SE (alpha=6/5) |  | 4.8dB | 4.1dB | 4.75dB |
| FDSS-CE(alpha=6/5) |  | 4.8dB | 4.25dB | 4.8dB |
| DFT-s-OFDM with FDSS | QPSK rotation | 1.85dB | 1.85dB | 2.3dB |
| FDSS-SE (alpha=6/5) |  | 4.85dB | 4.2dB | 4.8dB |
| FDSS-CE (alpha=6/5) |  | 4.85dB | 4.2dB | 4.8dB |

Under QPSK, it can be found that

- For DFT-s-OFDM with FDSS, the net gain is no less than 1.3dB for any bandwidth

- For FDSS-SE, the net gain is no less than 2.2dB and 4.1dB for any bandwidth for alpha=6/5 and alpha=5/3, respectively

- For FDSS-CE, the net gain is no less than 2.7dB and 4.25dB for any bandwidth for alpha=6/5 and alpha=5/3, respectively

When further combined with QPSK rotation, it can be found that

- For DFT-s-OFDM with FDSS, the net gain is no less than 1.3dB for any bandwidth

- For FDSS-SE/FDSS-CE, the net gain is no less than 2.95dB and 4.85dB for any bandwidth for alpha=6/5 and alpha=5/3, respectively

Case 3: Net gain under different PRB allocations

To further analyse the impact of different PRB allocations on the net gain, the net gain of PRBs located inside and outside the system bandwidth are evaluated and compared separately. The specific simulation parameters for this case are given in Table A- 4 in the Appendix. The net gain compared to the NR baseline DFT-s-OFDM under QPSK for alpha =4/3 for the inner PRB allocation and outer PRB allocation is shown in Figure 13 (a) and Figure 13 (b), respectively.

- Inner PRB allocation

- Outer PRB allocation

Figure 13. Net gain related to PAPR vs. NR baseline of DFT-s-OFDM under QPSK under different PRB allocations

It can be found that FDSS-SE and FDSS-CE achieve a larger net gain than DFT-s-OFDM with FDSS under both the inner PRB allocation and outer PRB allocation

Observation 12: FDSS-SE, FDSS-CE achieve a larger net gain compared to DFT-s-OFDM with FDSS with both QPSK and QPSK rotation whenever inner PRB allocation or outer PRB allocation

Observation 13: The net gain of FDSS-CE is no smaller than FDSS-SE for a given SE factor

    - Link budget comparison between different waveforms

The link budget comparison between different waveforms with the same payload size is evaluated in this subsection.

Firstly, the block error rate (BLER) performance for different waveforms is evaluated. The evaluation results are presented in Figure 14, and the SNR required to achieve 10% BLER for different waveforms is listed in Table 7.

Figure 14. BLER performance with QPSK under different SE factors

Table 7. Required SNR to achieve 10% BLER

| alpha Waveform | 1 | 6/5 | 5/4 | 4/3 | 8/5 | 5/3 |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM with FDSS | 7dB | - | - | - | - | - |
| FDSS-SE | - | 5.35dB | 4.3dB | 3.7dB | 1.65dB | 1.32dB |
| FDSS-CE | - | 4.2dB | 4.2dB | 3.4dB | 1.6dB | 1.3dB |

Then, the link budget performance related to Maximum isotropic loss (MIL) as used in R17 coverage enhancement is calculated and compared based on the parameters given in Table A- 5. The definition of MIL is given in [10]:

MIL = Total transmit power - Receiver sensitivity + gNB antenna gain (component 2 + 3 + 4) + UE antenna gain

The MIL gain of FDSS-SE/FDSS-CE compared to DFT-s-OFDM with FDSS under different SE factors is given in Table 8.

Table 8. MIL gain vs. DFT-s-OFDM with FDSS with QPSK

| alpha Waveform | 6/5 | 5/4 | 4/3 | 8/5 | 5/3 |
| --- | --- | --- | --- | --- | --- |
| FDSS-SE | 0.85dB | 1.7dB | 2.05dB | 3.3dB | 3.46dB |
| FDSS-CE | 2.0dB | 2.3dB | 2.35dB | 3.4dB | 3.48dB |

It can be found that:

- The MIL gain of FDSS-SE/FDSS-CE compared to NR DFT-s-OFDM increases with the increase of alpha. When alpha = 5/3, the maximum MIL gain is 3.46dB and 3.48dB for FDSS-SE and FDSS-CE, respectively.

- For FDSS-CE, the MIL gain compared to FDSS-SE decreases with the increase of alpha. When alpha = 6/5, the maximum MIL gain is 1.15dB.

For further analysis in the link-budget comparison among the different waveforms, a smaller payload size is considered. For both FDSS-SE and FDSS-CE, a larger SE factor yields a higher PAPR gain but also higher spectral efficiency, whereas a smaller SE factor yields a lower PAPR gain and lower spectral efficiency.  To achieve a larger PAPR gain, the larger SE factor of 5/3 with 5 PRB is selected for further evaluation, while the smaller SE factor of 6/5 with 6 PRB is also considered to limit spectrum efficiency loss.

The MIL gain of FDSS-SE/FDSS-CE compared to DFT-s-OFDM with FDSS is given in Table 9.

Table 9. MIL gain vs. DFT-s-OFDM with FDSS with QPSK under a smaller payload

| alpha Waveform | 6/5 | 5/3 |
| --- | --- | --- |
| FDSS-SE | 1.56dB | 4.08dB |
| FDSS-CE | 1.56dB | 4.53dB |

It can be found that:

The MIL gain of FDSS-SE/FDSS-CE over NR DFT-s-OFDM is always present, regardless of the payload size.

    - Spectrum efficiency comparison between different waveforms

In this subsection, the spectral efficiency of different waveforms is analysed and compared under the assumption of the same payload size.

The spectrum efficiency is calculated based on the following definition:

spectrum efficiency=payload/(T * BW).

If the payload is the same, the actual bandwidth occupied by FDSS-SE/FDSS-CE is alpha times that of  DFT-s-OFDM with FDSS, i.e., the spectrum efficiency of FDSS-SE/FDSS-CE is 1/alpha of DFT-s-OFDM with FDSS.

The spectrum efficiency loss of FDSS-SE/FDSS-CE compared to DFT-s-OFDM with FDSS under different SE factors is given in Table 10.

Table 10. Spectrum efficiency loss vs. DFT-s-OFDM with FDSS with QPSK

| alpha Waveform | 6/5 | 5/4 | 4/3 | 8/5 | 5/3 |
| --- | --- | --- | --- | --- | --- |
| FDSS-SE | 16.67% | 20% | 25% | 37.5% | 40% |
| FDSS-CE | 16.67% | 20% | 25% | 37.5% | 40% |

It can be found that the spectrum efficiency loss of FDSS-SE/FDSS-CE compared to NR DFT-s-OFDM increases with the increase of alpha. When alpha = 5/3, the maximum spectrum efficiency loss is 40%.

- Discussions on other waveform candidates

Multi-layer DFT-s-OFDM for UL spectrum efficiency improvement can be one of the promising directions for the 6GR waveform.  Multi-layer DFT-s-OFDM (e.g., rank-2) could have the capability to improve both the cell-edge and cell-average throughput. However, the multi-layer DFT-s-OFDM may affect the PAPR performance, which should be carefully considered. What’s more, the potential codebook design needs to be considered for multi-layer DFT-s-OFDM.

Observation 14: UL multi-layer DFT-s-OFDM (e.g., rank-2) could achieve higher spectrum efficiency, but the impact on PAPR needs to be carefully considered

DFT-s-OFDM for downlink coverage enhancement and energy saving is also proposed for the 6GR waveform. However, DL DFT-s-OFDM may have significant impacts on actual DL operation, e.g., potential scheduler limitation caused by the FDRA limitation, potential MU possibility limitation, and MU-MIMO performance degradation caused by low PAPR DMRS pattern and sequence. Moreover, the practical gain (or the case where we can enjoy the gain) and specification impact should be clearly justified.

Observation 15: DL DFT-s-OFDM may have a significant impact on DL operation, so the practical gain (or the case where we can enjoy the gain) and specification impact should be clearly justified

OTFS-based waveforms for high-speed scenarios (e.g., NTN) and ISAC are another research direction for the 6GR waveform. OTFS-based waveforms improve the Doppler robustness by transforming the time-frequency domain modulated symbols into the delay-Doppler domain using a new specific transform, e.g., symplectic finite Fourier transform (SFFT), Zak Transform, etc.  However, their processing latency may be increased due to the block operation for the new specific transform. Moreover, the introduction of a new specific transform may cause high transceiver complexity and large specification impacts.

Observation 16: OTFS-based waveforms cause additional complexity and cost due to low compatibility with OFDM-based waveforms

- Conclusion

In this contribution, we present our general views on the waveform design for the 6GR. This includes comprehensive theoretical analysis, numerical evaluations, and comparisons of the waveform candidates for UL coverage enhancement. The following observations and proposals are made based on the discussion.

Observation 1: Both the PAPR and net gain under a given required spectral efficiency, with the same occupied time and frequency domain resources, should be considered for low PAPR waveform evaluation.

Observation 2: Wider coverage may be needed for 6G

- For 6GR waveform, PAPR performance improvement for better coverage, especially for the uplink may be necessary for 6GR

- It should be noted that several other aspects should be considered together with PAPR performance, such as achievable net gains, link budget, and system-wise performance when it is deployed

Observation 3: DFT-s-OFDM enhancement with SE, including FDSS-SE and FDSS-CE, can reduce PAPR performance to achieve better coverage with a small impact, which can be considered the waveform candidates for 6G uplink

Observation 4: FDSS-CE has no requirement for the number of RBs used for SE, and requires lower implementation complexity than FDSS-SE based on the serial implementation method for asymmetric SE

Observation 5: QPSK rotation can reduce the PAPR of the DFT-s-OFDM waveform while achieving a higher data rate than pi/2-BPSK.

Observation 6: QPSK rotation can be combined with DFT-s-OFDM, DFT-s-OFDM with FDSS and/or SE enhancement for further PAPR reduction.

Observation 7: DFT-s-OFDM with frequency domain truncation (FDT) can achieve a tunable trade-off between PAPR and spectrum efficiency

Observation 8: FDSS-SE, FDSS-CE can achieve a larger PAPR gain than DFT-s-OFDM with FDSS with any given SE factor and/or any given bandwidth for both QPSK and QPSK rotation

Observation 9: FDSS-CE achieves a larger PAPR gain than FDSS-SE with the same SE factor under QPSK. If further combining with QPSK rotation, FDSS-SE and FDSS-CE achieve the same PAPR performance.

Observation 10: FDSS-CE under QPSK without rotation achieves near-optimal PAPR performance

Observation 11: FDSS-SE and FDSS-CE result in smaller SNR loss compared to DFT-s-OFDM with FDSS. The different SE methods, as well as QPSK rotation, have very little impact on link performance

Observation 12: FDSS-SE, FDSS-CE achieve a larger net gain compared to DFT-s-OFDM with FDSS with both QPSK and QPSK rotation whenever inner PRB allocation or outer PRB allocation

Observation 13: The net gain of FDSS-CE is no smaller than FDSS-SE for a given SE factor

Observation 14: UL multi-layer DFT-s-OFDM (e.g., rank-2) could achieve higher spectrum efficiency, but the impact on PAPR needs to be carefully considered

Observation 15: DL DFT-s-OFDM may have a significant impact on DL operation, so the practical gain (or the case where we can enjoy the gain) and specification impact should be clearly justified

Observation 16: OTFS-based waveforms cause additional complexity and cost due to low compatibility with OFDM-based waveforms

Proposal 1: For 6GR study on waveform,

- Only OFDM-based waveform(s) should be considered (as described in the SID)

- Any new waveform(s), even for OFDM-based, should be justified by clear gain

- Unified design across scenarios/use cases is strongly preferred

- Following the above, RAN1 can carefully assess the need in 6GR to introduce waveform(s) beyond 5G NR, targeting, e.g.,

  - Potential better coverage by better PAPR performance for uplink

  - Spectrum efficiency improvement

- References

- 3GPP, RP-252912, NTT DOCOMO, INC, “Revised SID: Study on 6G Radio”, Jun. 2025.

- RAN1 Chairmans’s note, 3GPP TSG RAN WG1 Meeting#122, Bengaluru, India, Aug, 2025

- 3GPP TS 38.300, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; NR and NG-RAN Overall Description; Stage 2 (Release 18),” Mar. 2024.

- Y. Rahmatallah and S. Mohan, “Peak-to-average power ratio reduction in OFDM systems: A survey and taxonomy,” IEEE Commun. Surveys Tut., vol.15, no.4, pp.1567–1592, 2013.

- J. Liu, X. Hou, W. Liu, C. Li, and L. Chen, “Low-PAPR 6G Waveform and Modulation Design with 5G NR Backward Compatibility,” IEICE Trans. on Commun., vol. E109-B, no. 2, Feb. 2026.

- 3GPP, R1-050702, NTT DOCOMO, NEC, SHARP, “DFT-spread OFDM with pulse shaping filter in frequency domain in evolved UTRA uplink”, Sep. 2005.

- 3GPP, R4-2314910, Nokia, Nokia Shanghai Bell, “WF for NR_cov_enh2_part2”, Aug. 2023.

- I.P. Nasarre, T. Levanen, K. Pajukoski, A. Lehti, E. Tiirola, and M. Valkama, “Enhanced uplink coverage for 5G NR: frequency-domain spectral shaping with spectral extension,” IEEE Open J. Commun. Soc., vol.2, pp.1188–1204, 2021.

- J. Liu, X. Hou, W. Liu, and L. Chen, “Low-PAPR and Low-Complexity DFT-s-OFDM Enhancement for 6G,” accepted by the 2025 IEEE Asia-Pacific Conference on Communications (APCC).

- 3GPP, RAN1#102e, RAN1 Chairman’s Notes, Sep. 2020.

- 3GPP, R1-2506218, Qualcomm Incorporated, “Waveforms for 6GR”, 3GPP TSG-RAN WG1 #122, 25th – 29th August 2025, Bengaluru, India.

- Appendix

In this appendix, we provide the general simulation parameters for different evaluation assumptions and the specific simulation parameters for a specific valuation assumption considered in section5.5.

Table A- 1.  General simulation parameters for different evaluation assumptions

| Parameter | Simulation Assumption |
| --- | --- |
| Carrier frequency | 4GHz |
| Subcarrier spacing | 30kHz |
| FFT size | 4096 |
| Allocated RB number | See Table A-2 to Table A-4 |
| System RB number | 276 (by default) |
| Channel model | TDL-A |
| Dealy spread | 30ns |
| UE mobility | 3km/h |
| CP length | 288 |
| Channel Coding | LDPC |
| Channel Estimation | Realistic estimation |
| Antenna configuration | 1Tx/1Rx |
| Phase Noise model | w/o PN |
| Slot Number | 1000 |

Table A- 2. Specific simulation parameters for case 1 in section 5.5.1/5.5.2/5.5.3

| Waveform | FDSS type | alpha | PRB | Modulation | Target code Rate |
| --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | NONE | 1 | 60 | QPSK, pi/4-QPSK | 120/1024 |
| DFT-s-OFDM with FDSS | 3-Tap FDSS with [0.335,1,0.335] | 1 | 60 | QPSK, pi/4-QPSK | 120/1024 |
| FDSS - SE | 3-Tap FDSS with [0.335,1,0.335] | 2 | 60 | QPSK, pi/4-QPSK | 240/1024 |
|  |  | 5/3 | 60 | QPSK, pi/4-QPSK | 200/1024 |
|  |  | 4/3 | 60 | QPSK, pi/4-QPSK | 160/1024 |
|  |  | 6/5 | 60 | QPSK, pi/4-QPSK | 144/1024 |
| FDSS-CE | 3-Tap FDSS with [0.335,1,0.335] | 2 | 60 | QPSK, pi/4-QPSK | 240/1024 |
|  |  | 5/3 | 60 | QPSK, 11pi/12-QPSK | 200/1024 |
|  |  | 4/3 | 60 | QPSK, pi/12-QPSK | 160/1024 |
|  |  | 6/5 | 60 | QPSK, 9pi/20-QPSK | 144/1024 |

Table A- 3. Specific simulation parameters for evaluation case 2 in section 5.5.1/5.5.2/5.5.3

| Waveform | FDSS type | alpha | PRB | Modulation | Target code Rate |
| --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | NONE | 1 | 5, 6, 12, 15, 30 | QPSK, pi/4-QPSK | 120/1024 |
| DFT-s-OFDM with FDSS | 3-Tap FDSS with [0.335,1,0.335] | 1 | 5, 6, 12, 15, 30 | QPSK, pi/4-QPSK | 120/1024 |
| FDSS - SE | 3-Tap FDSS with [0.335,1,0.335] | 5/3 | 5, 15, 30 | QPSK, pi/4-QPSK | 200/1024 |
|  |  | 6/5 | 6, 12, 30 | QPSK, pi/4-QPSK | 144/1024 |
| FDSS-CE | 3-Tap FDSS with [0.335,1,0.335] | 5/3 | 5, 15, 30 | QPSK, 11pi/12-QPSK | 200/1024 |
|  |  | 6/5 | 6, 12, 30 | QPSK, 9pi/20-QPSK | 144/1024 |

Table A-4. Specific simulation parameters for evaluation case 3 in section 5.5.3

| Waveform | FDSS type | alpha | PRB | Modulation | Target code Rate |
| --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | NONE | 1 | 2,4,8,12,20,30,60 | QPSK, pi/4-QPSK | 120/1024 |
| DFT-s-OFDM with FDSS | 3-Tap FDSS with [0.335,1,0.335] | 1 | 2,4,8,12,20,30,60 | QPSK, pi/4-QPSK | 120/1024 |
| FDSS - SE | 3-Tap FDSS with [0.335,1,0.335] | 4/3 | 2,4,8,12,20,30,60 | QPSK, pi/4-QPSK | 160/1024 |
| FDSS-CE | 3-Tap FDSS with [0.335,1,0.335] | 4/3 | 2,4,8,12,20,30,60 | QPSK, pi/12-QPSK | 160/1024 |

Table A-5. Specific simulation parameters for link budget calculation in section 5.5.4

| Parameter | Simulation cases |
| --- | --- |
| Carrier frequency | 4GHz |
| Subcarrier spacing | 30kHz |
| Number of transmit antenna elements | 1 |
| Gain of antenna element | 0 dBi |
| Total transmitter power | 23dBm |
| Total antenna gain at antenna gain component 3 & antenna gain component 4 of receiver | 8.7dB |
| Total antenna gain at antenna gain component 2 of receiver | 12.04dB |
| Receiver noise figure | 5dB |
| Thermal noise density | -174 dBm/Hz |
| Receiver interference density | -172 dBm/Hz |
| Total noise plus interference density | -167.24 dBm/Hz |
| Occupied bandwidth | Allocated PRBs * subcarrier number per PRB * Subcarrier spacing |
| Effective noise power | Total noise plus interference density +10log(Occupied bandwidth)(dBm) |
| Required SNR at 10% BLER |  |
| Receiver sensitivity | Effective noise power +Required SNR |
| Hardware link budget, a.k.a. MIL | Total transmit power - Receiver sensitivity + gNB antenna gain (component 2 + 3 + 4) + UE antenna gain |