3GPP TSG-RAN WG1 Meeting #122bis        				 	R1-2507828

Prague, Czech Republic, 13rd - 17th October, 2025

Agenda Item:     10.1.2

Source:	Beijing University of Posts and Telecommunications (BUPT), ZGC Institute of Ubiquitous-X Innovation and Application, Pengcheng Laboratory

Title:	      Views on inter-vendor training collaboration for two-sided AI/ML models

Document for:	Discussion

Introduction

In RAN1#118bis meeting, some agreements have been reached regarding two-sided model alignment and the complexity of inter-vendor collaboration for two-sided CSI compression.

Agreement For issues listed for inter-vendor collaboration direction B, further study the overhead in Direction B based on the size of the encoder (i.e., number of parameters and quantization level), the number of encoders, and how often the parameters need to be transferred. Agreement For Directions of addressing inter-vendor collaboration complexity for two-sided CSI compression, - For Direction A: - Further study the following options with potential down-selection - 4-1: dataset is {target CSI, CSI feedback}, - 3a-1: encoder parameter exchange, with {target CSI} - Note: Additional information beyond what’s listed above may need to be exchanged if necessary. Agreement - For Direction C: - It is clarified that, the purpose of a fully specified CSI generation (reconstruction) part is to enable development/training of compatible CSI reconstruction (generation) parts at the NW (UE) side. - Study whether the fully specified reference models or their compatible models may provide reasonable performance for deployment at NW and UE - Including performance evaluation of training/finetuning the models by modeling data distribution mismatch between synthetic and field data, and/or by using actual field data - Including performance evaluation of training/finetuning the models by modelling data distribution mismatch between NW-side field data and UE-side field data, and/or by using actual field data from both NW-side and UE-side - Note: Training/finetuning is done at either or both sides without inter-vendor collaboration. - FFS whether the reference model is the encoder (1-1) or decoder (1-2) or both (1-3) - FFS whether the reference model is the same as RAN4 specified model - FFS whether a single or multiple reference (and how many) models need to be specified Agreement For the evaluation studies of Direction A, - For encoder parameter exchange from NW-side to UE-side of 3a-1, - Alt. 1: UE-side first trains a nominal decoder - Step 1: UE-side develops a nominal decoder against the exchanged encoder. - Step 2: UE-side develops actual encoder against the nominal decoder - Alt. 2: UE-side directly develops and trains the actual encoder. - Note: Target CSI is available at UE side - For dataset exchange from NW-side to UE-side {target CSI, CSI feedback} of 4-1, - Alt. 1: UE-side first trains a nominal decoder - Step 1: UE-side develops a nominal decoder using the exchanged dataset. - Step 2: UE-side develops an actual encoder against the nominal decoder. - Alt. 2: UE-side directly develops and trains the actual encoder. Agreement (further amended on Friday as shown in red) For discussion on performance degradation due to UE-side / NW-side data distribution mismatch with respect to UE side additional condition (issue 4 and 6), consider - NW-side data (dataset A) and UE-side data (dataset B) are mismatched in terms of UE-side additional condition (e.g., SVD phase normalization method, UE antenna virtualization, antenna imbalance, antenna spacing / layout, etc) - Note: dataset A may or may not contain dataset B - Note: if dataset A includes dataset B, this means NW timely collect data from the UE - Note: Dataset A and dataset B are aligned in terms of NW-side additional conditions. - Case 1: An encoder-decoder pair is trained on dataset B. This serves as an upper-bound. - Case 2 (for direction A): - For direction A: NW-side trains a decoder on dataset A, and UE-side trains an encoder based on either dataset A, dataset B, or both (where the choice depends on the inter-vendor collaboration Options (4-1 or 3a-1), the training alternatives (Alt 1 or Alt 2) within the Options, and the choice of target CSI for training) - Case 3 (for direction B): NW-side train an encoder-decoder on dataset A and transfers the encoder parameters to UE. - Study issue 4 and 6 by evaluating the inference performance for Case 2 / 3 on test dataset B, comparing to the inference performance for case 1 on test dataset B. - Note: Evaluations are assumed to perform without common reference model structure among companies. Agreement - For the evaluation study of offline engineering for Direction C for Issue 9: - Case 1: The AI/ML model is trained based on training Dataset B, and then the AI/ML model performs inference/test on a dataset from test Dataset B. - Note: This serves as the upper-bound performance. - Case 2: The AI/ML model is trained based on training Dataset S, and then the AI/ML model performs inference/test on test Dataset B. - Note: This represents the performance of Direction C when not using field data. - Case 2A: - Step 1: The AI/ML model is trained based on training Dataset S - Step 2: - NW-side and/or UE-side may separately train new CSI reconstruction part and/or CSI generation part compatible to the AI/ML model from Step 1 based on training Dataset B, as agreed in Note 1-1, 1-2, and 1-3 as agreed in RAN1#118. - Dataset from Step 1 may be additionally used. - Step3: The CSI generation part and the CSI reconstruction part from Step 2 performs inference/test on test Dataset B - Note: This represents the performance of Direction C when the AI/ML models are updated based on field data. - Note: Dataset S is meant to represent the synthetic data used for fully specified model development, and Dataset B is meant to represent the field data. - Examples for Dataset S and B: - Modeling via TxRU mapping, UMi vs. UMa, UMa vs. InH, etc. - It is acknowledged that the scenario/configuration modeling between S and B may not be complete representative of the difference between synthetic and field data distribution mismatch. - Synthetic vs. actual field data (if available) - Companies are encouraged to provide details on the scenarios/configurations underlying the field data - Note: Case 1 and Case 2 have been studied during Rel-18 generalization study and conclusions have been captured in the TR. - Note: For Case 2A, companies may additionally evaluate UE-side / NW-side data distribution mismatch with respect to UE side additional condition. This can be done, in Step 2, by treating the training Dataset B as representing UE-side data distribution and considering a training Dataset A representing NW-side data distribution. - Note: Evaluations are assumed to perform without common reference model among companies.

In RAN1#122 meeting, For Option 4-1 under Direction A in AI/ML-based CSI compression, exchanging quantization codebooks together with datasets is approved to support model training at the UE side and enable two-sided model alignment.

Agreement: For Option 4-1 under Direction A in AI/ML based CSI compression, if the quantization codebook for CSI feedback is not fixed by the specification, support exchange of quantization codebook along with dataset exchange, - FFS: Quantization codebook exchange method (e.g, look up table, , or how to exchange quantization codebook related information, or other methods) - FFS: Common or different quantization codebook for different CSI payload size Note: leveraging the discussion in agenda item 10.1.1.1 when applied

In this section, we would focus on the further study inter-vendor training collaboration for two-side AI/ML models

2 Additional information for two-sided AI/ML model finetuning

In the feasible directions of sub-option 4-1 and 3a-1 previously discussed, after the NW side exchanges datasets or model parameters with the UE side, the UE trains its own model based on the exchanged information rather than directly using the exchanged model parameters for inference. We reconsider that, when a two-sided model is already operating normally, performance degradation caused by mobility, data drift, or other factors can be mitigated through the transmission of additional information. The additional information, generated by training on one side, is applied by the other side as model update information without requiring retraining. This enables better alignment and consistency between the two-sided models.

In AI/ML-based CSI compression frameworks, a critical stage is the efficient quantization of the high-dimensional latent vector produced by the UE-side encoder. Compared to Scalar Quantization (SQ), which quantizes each element independently and thus ignores inherent statistical correlations, Vector Quantization (VQ) encodes the entire latent vector as a single entity. This approach fully leverages the strong correlations inherent in CSI across the antenna, delay, and frequency domains. By employing an optimized Vector Codebook, VQ can achieve a significantly lower quantization error for the same bit overhead. Furthermore, VQ is naturally aligned with the structure of AI models, allowing the quantizer to be jointly optimized end-to-end with the UE-side encoder and the NW-side decoder. This facilitates a globally optimal mapping from CSI feature extraction to codebook indexes, and ultimately to the reconstructed CSI.

Current discussions on VQ for CSI feedback have largely focused on its quantization performance gains, i.e., the CSI compression and reconstruction performance under specific data distributions. However, these discussions have mostly overlooked the pivotal role of the codebook as the core of the “feature-to-index” mapping, and its inherent potential and flexibility in adapting to diverse data distributions and dynamic uplink transmission environments. We believe that to comprehensively evaluate and standardize AI/ML CSI feedback solutions, a systematic study of vector codebook implementation forms is necessary.

The VQ codebook, serving as a mapping of feature distributions, effectively guides encoding and decoding. Therefore, when data distribution changes lead to two-sided AI/ML model mismatch, updating and transmitting the VQ codebook from one side enables correction of the model at the other side.

Proposal 1: During AI/ML model deployment, when performance degradation occurs, finetuning shall be supported by exchanging additional information (e.g., partial parameters or vector quantization codebooks) to align AI/ML models between the UE and NW sides.

Proposal 2: Under Direction A in AI/ML based CSI compression 3a-1, it is proposed to support the exchange of quantization codebooks together with parameters.

- 3 Conclusion

In this contribution, we provide our opinions on additional information exchange for two-sided AI/ML model training and finetuning.

Proposal 1: During AI/ML model deployment, when performance degradation occurs, finetuning shall be supported by exchanging additional information (e.g., partial parameters or vector quantization codebooks) to align AI/ML models between the UE and NW sides.

Proposal 2: Under Direction A in AI/ML based CSI compression 3a-1, it is proposed to support the exchange of quantization codebooks together with parameters.

- 4 Reference

- 3GPP R1-2506455, Moderator summary #1 on AI/ML for 6GR

- 3GPP R1-2506702, Final Report of 3GPP TSG RAN WG1 #122 v1.0.0

- 3GPP R1-2506109, Discussion on Inter-Vendor Training Collaboration for Two-Side AI/ML Models