3GPP TSG RAN WG1 #122bis	R1-2507867

# Prague, Czech, Oct 13th – 17th, 2025

Agenda Item:	10.4.1

Source: 	ASUSTeK

Title:  	Discussion on coverage enhancement

Document for:	Discussion and Decision

- Introduction

In RP-251862, objective of phase three coverage enhancement are quoted below:

The detailed objectives for coverage enhancement of this work item include: - Specify following PRACH coverage enhancements [RAN1, RAN2] - Multiple PRACH transmissions with different Tx beams for 4-step RACH procedure. - UE may receive UL beam information after transmission of MSG1 - UL beam information is to assist the UE decision on Msg3 beam selection. - Note 1: “different Tx beams” is for the purpose of future RAN1 discussions - Note 2: The enhancements of PRACH are targeting for FR2 and can also apply to FR1 when applicable. - Note 3: The enhancements of PRACH are targeting short PRACH formats and can also apply to other formats when applicable. - Note 4: The PRACH repetitions are transmitted over ROs associated with the same SSB. - Note 5: The procedure for repetitions of a PRACH transmission is as in Rel-18 - Specify enhancements to support PUSCH repetition scheduled by DCI 0_0 with C-RNTI [RAN1, RAN2] - Specify enhancements to improve PUSCH coverage for higher uplink data rate by extending pi/2-BPSK to more MCS entries in MCS tables [RAN1]

In this contribution, we provide some aspects on coverage enhancement.

- Discussion

In Rel-20, one scope of WID targets on Msg1 repetition with different TX beams. According to WID, after transmitting Msg1, UE may receive UL beam information to assist UE decision on Msg3 beam selection. Typically, the UL beam information could indicate qualified or best UL beam, and UE will determine Msg3 beam based on the indicated UL beam. Thus, this mechanism could enhance coverage UE’s Msg3 transmission.

The first point needs to discuss may be which type of RA could apply Msg1 repetition with different TX beams. RAN1 could discuss whether to support CFRA, CBRA or both for Msg1 repetition with different TX beams.

The second point needs to discuss may be whether to have RACH partition for Msg1 repetition with different TX beams. From our perspective, it may be beneficial for UE to use different set of ROs (for Rel-20 Msg1 repetition with different TX beams), and thus gNB can determine whether to provide UL beam information in the consequent signal.

The third point needs to discuss may be size of the set of ROs for Msg1 repetition with different TX beams and how UE determines which set of ROs. Typically, {2, 4, 8} could be considered as baseline. Back to Msg1 repetition with same TX beam in Rel-18, DL radio condition (e.g., RSRP threshold) is leveraged to determine the repetition number and corresponding set of ROs which we think could be baseline for determining set of ROs in Rel-20. Besides, with introduction of two mechanisms for repetition of Msg1, how UE determines to select which set of ROs may need further study. Some detailed issues may be whether UE could apply intra-TX beam repetition alongside with different TX beam repetition for Msg1.

The fourth point needs to discuss may be how to signal the UL beam information. From our perspective, PDCCH with CRC scrambled by RA-RNTI and RAR could be leveraged to indicate the UL beam information. More specifically, size of the UL beam information could be based on size of set of ROs for Msg1 repetition with TX beams.

The fifth point needs to discuss may be after RA procedure is completed, which TX beam is served as default beam for UL transmission before receiving TCI state indication.

The sixth point needs to discuss may be whether different TX beams for Msg1 repetition should be constrained to one selected SSB. In WID, the relationship between ROs and SSB is referred. The relationship between different TX beams and SSB should be firstly checked for confirming scenario.

The seventh point needs to discuss may be how many TX beams are indicated in the UL beam information. Compared to Msg1 repetition with the same TX beams, which provides additional gain so that the preamble can be detected in gNB side, Msg1 repetition with different TX beam assists in selecting the TX beam with best quality. The best quality may mean over a threshold and meet a desired value. However, in some scenarios, e.g., high moving speed UE or deep fading, a TX beam with best quality may be hard to be decided via only Msg1 repetition. Hence, indicating more than one TX beam in UL beam information may be considered.

Proposal 1: RAN1 study following aspects for Msg1 repetition with different TX beams:

- Which type RA UE could apply Msg1 repetition with different TX beams (e.g., CFRA, CBRA, or both)

- Whether to support RACH partition for Msg1 repetition with different TX beams

- In which condition, the UE is allowed to apply Msg1 repetition with different TX beams (e.g., UE determines based on RSRP, or gNB indicates) and how UE determines Rel-18 or Rel-20 Msg1 repetition scheme for RA procedure

- How gNB provides the UL beam information (e.g., via PDCCH or RAR)

- After RA procedure is completed, which TX beam is used as default beam for UL transmission before receiving TCI state indication.

- The different TX beams for Msg1 repetition are constrained to a selected one SSB.

- Study the number of TX beams indicated in UL beam information.

Another scope of WID targets on PUSCH repetition scheduled by DCI format 0_0 with C-RNTI. As mentioned in WID, PUSCH repetition scheduled by DCI format 0_0 with C-RNTI is referred in order to overcome the bottleneck during initial access. With such design, it needs study on when to apply the feature of “PUSCH repetition scheduled by DCI format 0_0 with C-RNTI”. In current NR design, PUSCH repetition is supported in DCI format 0_1. Compared to DCI format 0_1, DCI format 0_0 size is smaller and more robust. RAN1 can study whether the feature of “PUSCH repetition scheduled by DCI format 0_0 with C-RNTI” can be applied other than Msg5.

Proposal 2: RAN1 study following aspect for PUSCH repetition scheduled by DCI 0_0 with C-RNTI:

- In which condition, the UE is allowed to apply “PUSCH repetition scheduled by DCI format 0_0 with C-RNTI.

- Conclusion

In this contribution, we have following proposals:

Proposal 1: RAN1 study following aspects for Msg1 repetition with different TX beams:

- Which type RA UE could apply Msg1 repetition with different TX beams (e.g., CFRA, CBRA, or both)

- Whether to support RACH partition for Msg1 repetition with different TX beams

- In which condition, the UE is allowed to apply Msg1 repetition with different TX beams (e.g., UE determines based on RSRP, or gNB indicates) and how UE determines Rel-18 or Rel-20 Msg1 repetition scheme for RA procedure

- How gNB provides the UL beam information (e.g., via PDCCH or RAR)

- After RA procedure is completed, which TX beam is used as default beam for UL transmission before receiving TCI state indication.

- The different TX beams for Msg1 repetition are constrained to a selected one SSB.

- Study the number of TX beams indicated in UL beam information.

Proposal 2: RAN1 study following aspect for PUSCH repetition scheduled by DCI 0_0 with C-RNTI:

- In which condition, the UE is allowed to apply “PUSCH repetition scheduled by DCI format 0_0 with C-RNTI.