3GPP TSG RAN WG1 #122bis																		   		R1-2507904

Prague, CZ, Oct-13th to Oct- 17th, 2025

Agenda item:	         11.4.1

Source:	CEWiT

Title:	Channel coding for control and data channels in 6G

Document for:	     Discussion and Decision

# Introduction

In RAN plenary #108, study item 6G study objectives are finalised [1]. The study aims to develop one non-backward compatible radio access technology to meet a broad range of use cases, and requirements as defined during the RAN requirements study. The study objectives which is relevant to this paper is states as follow,

Physical Layer structure for 6GR, - .. - .. - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - .. - ..

In this document, we provide our views on channel coding related aspects of data and control channels.

# Discussion

## Data channel

Channel coding is a fundamental technique used to enhance the reliability of data transmission by introducing controlled redundancy into the transmitted message. This redundancy allows the receiver to detect and, in many cases, correct errors without the need for retransmission. Channel coding plays a pivotal role in meeting stringent requirements for throughput, latency, and reliability. The selection of a specific coding scheme is influenced by factors such as target error performance, computational complexity, and decoding latency.

5G NR adopts two distinct channel coding schemes, LDPC codes and Polar codes, each optimized for a specific type of data. LDPC codes are chosen for data channels such as PDSCH/PUSCH chosen to enable highly reliable transmission of larger transport blocks and to support incremental redundancy during retransmissions. Polar codes are chosen for control channels like PDCCH, PUCCH and PBCH. Polar codes offer near-capacity performance at short block lengths. Channel coding in 5G NR operates along with rate-matching, interleaving, and modulation.

We feel that the channel coding schemes in NR can be treated as a baseline for 6G as the NR coding schemes went through extensive study and provide proven reliability. The usage of the coding schemes for different channels should be studied in the forthcoming meeting for example, polar codes can possibly be used for data channels such as PDSCH/PUSCH in case of smaller transport block sizes. As a starting point, consider NR coding schemes as the baseline for 6G.

Proposal 1: Consider NR coding schemes (Polar and LDPC) as baseline for channel coding schemes in 6G

In NR, codeword to layer mapping enables the transmission of different codewords across different layers. NR supports the transmission of up to two codewords wherein the mapping of a CW to a given layer is dependent on the rank of the transmission. One codeword can be transmitted using up to 4 layers. Two codeword transmission can be done only when number of layers is greater than 4 where the number of layers used for each codeword can differ by at most 1. In NR, different modulation and coding can be used across different codewords. The NR CW-to-Layer mapping schemes are can be considered as the baseline for 6G.

Proposal 2: Consider NR CW-to-Layer mapping schemes as baseline in 6G.

In 6GR, FWA devices are targeted as the key driving factor for 6GR marketization. FWA devices are mainly characterized by a stationery and rank deficit channel and higher number of Tx/Rx antennas. Currently an FWA device operating in FR1 band can support up to 8 Tx/Rx antennas. With the introduction of new frequencies in FR3 band (7-24GHz), the number of Tx/Rx antennas can increase up to 4 times i.e., up to 32 Tx/Rx antennas can be supported. Similarly for hand held devices, the number of Tx/Rx antennas can increase by 4x in 6G. The maximum number of layers in NR in UL/DL is limited by the number of Tx/Rx antennas at the UE respectively. With the increase in number of Tx/Rx antennas at the UE, there is a need to enhance the maximum supported number of layers for both DL and UL. Hence, the need to support higher number of codewords in 6G and appropriate extension to CW-to-Layer mapping schemes for additional codewords should be studied.

Proposal 3: The need to support higher number of codewords in 6G and appropriate extension to CW-to-Layer mapping schemes for additional codewords should be studied.

FWA devices are also characterized by a stationery and rank deficit channel. The rank deficiency of the channel arises because of the LoS nature of the FWA channel. Hence, for the FWA devices, there will be a huge difference in the capacities of multiple layers for example, the layers that maximizes the LoS path will have very large capacity compared to the layers that doesn’t. Hence, it is required to enhance the existing CW to layer mapping such that the overall capacity of the channel is not affected.

Proposal 4: Enhance the existing CW-to-Layer mapping schemes to accommodate multiple device types

## Control channel

In 5G NR, CRC aided polar codes are extensively studied and agreed to be supported as one of the channel coding scheme for PDCCH and PUCCH. In addition to this, distributed CRC is supported for PDCCH in order to support early termination of blind decoding. We feel that polar codes should be retained at least for the payload sizes for which polar coding is supported in NR.

Proposal 5: Support distributed CRC aided polar codes as one of the coding scheme for 6G control channels

In NR, the DCI payload size supported is varies approximately between 30 to 140 bits. Polar coding can be used as the coding scheme for PDCCH when the payload size of PDCCH falls within the range that is supported in NR. Although, in 6G the supported range for DCI payload size may be different from that of NR. Before deciding on any additional coding scheme for PDCCH besides polar coding, the requirement for a larger/smaller payload size for PDCCH/PUCCH should be studied.

Proposal 6: The requirement for a larger/smaller payload size for PDCCH/PUCCH should be studied.

# Conclusion

Proposal 1: Consider NR coding schemes (Polar and LDPC) as baseline for channel coding schemes in 6G

Proposal 2: Consider NR CW-to-Layer mapping schemes as baseline in 6G.

Proposal 3: The need to support higher number of codewords in 6G and appropriate extension to CW-to-Layer mapping schemes for additional codewords should be studied.

Proposal 4: Enhance the existing CW-to-Layer mapping schemes to accommodate multiple device types

Proposal 5: Support distributed CRC aided polar codes as one of the coding scheme for 6G control channels

Proposal 6: The requirement for a larger/smaller payload size for PDCCH/PUCCH should be studied.

# References

- RP-251881, Study on 6GR Radio, 3GPP TSG RAN Meeting #108.