3GPP TSG-RAN WG1 Meeting #122bis	 R1-2507934

Prague, Czech Republic, October 13-17, 2025

Agenda Item:	7

Source:	Huawei, HiSilicon

Title:	Discussion on the lowest numbered resource block for PRACH baseband signal generation

Document for:	Discussion and Decision

# Discussion

### 5.3.2 OFDM baseband signal generation for PRACH The time-continuous signal sl(p,μ)t on antenna port p for PRACH is defined by sl(p,μ)t=k=0LRA-1ak(p,RA)ej2πk+Kk1+kΔfRAt-NCP,lRATc-tstartRA K=ΔfΔfRA k1=k0μ+NBWP,istart-Ngridstart,μNscRB-Ngridsize,μNscRB2+nRAstartNscRB+nRANRBRANscRBif LRA∈139, 839nRANRBRANscRBif LRA∈571, 1151 in FR2-2NRB,UL,n0+nRAstart,μ-NRB,UL,n0start,μNscRBif LRA∈571, 1151 in FR1 k0μ=Ngridstart,μ+Ngridsize,μ2NscRB-Ngridstart,μ0+Ngridsize,μ02NscRB2μ0-μ where and - is given by clause 6.3.3; - Δf is the subcarrier spacing of the initial uplink bandwidth part during initial access. If the PRACH transmission is for a candidate cell Δf is provided by ltm-PRACH-SubcarrierSpacing in EarlyUL-SyncConfig. If the PRACH transmission is for SIB1 request, Δf is provided by msg1-SubcarrierSpacing in SIB1-RequestConfig. Otherwise, is the subcarrier spacing of the active uplink bandwidth part; - μ0 is the largest μ value among the subcarrier spacing configurations by the higher-layer parameter scs-SpecificCarrierList; - NBWP,istart is the lowest numbered resource block of the initial uplink bandwidth part and is derived by the higher-layer parameter initialUplinkBWP or initialUplinkBWP-RedCap during initial access and from the higher-layer parameters bwp-GenericParameters in EarlyUL-SyncConfig if the PRACH transmission is for a candidate cell and from the higher-layer parameters locationAndBandwidth in SIB1-RequestConfig, if the PRACH transmission is for SIB1 request. Otherwise, NBWP,istart is the lowest numbered resource block of the active uplink bandwidth part and is derived by the higher-layer parameter BWP-Uplink;

For OFDM baseband signal generation of PRACH, the value of NBWP,istart is derived by

- the higher-layer parameter initialUplinkBWP or initialUplinkBWP-RedCap if the PRACH transmission is for initial access, or

- the higher-layer parameter bwp-GenericParameters in EarlyUL-SyncConfig if the PRACH transmission is for a candidate cell, or

- the higher-layer parameter locationAndBandwidth in SIB1-RequestConfig, if the PRACH transmission is for SIB1 request, or

- the higher-layer parameter BWP-Uplink corresponding to the active uplink bandwidth part, otherwise.

In two TA operation under inter-cell mTRP scenario, UE needs to transmit PRACH to an addition PCI. In this case, how to determine the value of NBWP,istart is unclear since the current spec does not cover such case. To fix this issue, one simple way is to follow the design of Rel-18 LTM, i.e., to use a higher-layer parameter bwp-GenericParameters to determine the value of NBWP,istart. However, in PRACH configuration for additional PCI (i.e., RACH-ConfigTwoTA), there is no such parameter. Therefore, RAN1 needs to send an LS to RAN2 to ask for adding such parameter in RACH-ConfigTwoTA. An example of RRC change is given in Table 1.

Proposal 1: Send an LS to RAN2 to ask for adding a new RRC parameter bwp-GenericParameters in RACH-ConfigTwoTA-r18 to determine the value of NBWP,istart for the case where the PRACH is transmitted to an additional PCI.

Table 1

RACH-ConfigTwoTA-r18 ::= SEQUENCE { additionalPCI-andRACH-Index-r18 AdditionalPCIIndex-r17, rach-ConfigGeneric-r18 RACH-ConfigGeneric, ssb-perRACH-Occasion-r18 ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen} OPTIONAL, -- Need M prach-RootSequenceIndex-r18 CHOICE { l839 INTEGER (0..837), l139 INTEGER (0..137), l571 INTEGER (0..569), l1151 INTEGER (0..1149) }, msg1-SubcarrierSpacing-r18 SubcarrierSpacing OPTIONAL, -- Cond L139 twoTA-restrictedSetConfig-r18 ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB} OPTIONAL -- Need R bwp-GenericParameters-r18 BWP } BWP ::= SEQUENCE { locationAndBandwidth INTEGER (0..37949), subcarrierSpacing SubcarrierSpacing, cyclicPrefix ENUMERATED {extended} OPTIONAL -- Need R }

With such new parameter, RAN1 spec can be updated according to the CR given in our companion paper R1-2507935 [1] to fix the above issue.

Proposal 2: Adopt the CR in R1-2507935.

# Summary and conclusion

In this contribution, we have the following proposals.

Proposal 1: Send an LS to RAN2 to ask for adding a new RRC parameter bwp-GenericParameters in RACH-ConfigTwoTA-r18 to determine the value of NBWP,istart for the case where the PRACH is transmitted to an additional PCI.

Proposal 2: Adopt the CR in R1-2507935.

# Summary and conclusion

- R1-2507935, “Correction on the lowest numbered resource block for PRACH baseband signal generation in 38.211”, RAN1 122bis, October 13-17, Huawei, Hisilicon.