3GPP TSG RAN WG1 #122bis	R1-2508169

# Prague, Czech, Oct 13th – 17th, 2025

Title:	[Draft] LS on the LO determination based on lpwus-LoFrameOffsetList for LP-WUS operation in IDLE/INACTIVE mode

Response to:

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	[Moderator (Apple)]

To:	RAN2

Cc:

Contact person:	Sigen Ye, sigen_ye@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

In RAN1#122bis, the following clarification was made regarding the LO determination based on lpwus-LoFrameOffsetList:

Agreement

The LO determination based on lpwus-LoFrameOffsetList is clarified as follows:

- If the number of POs associated with a LO is less than Ns (the number of POs per PF), the UE uses the (i_sNPOWO+1)-th value in offsetForLongerWakeUpDelay (if configured) and the (i_sNPOWO+1)-th value in offsetForShorterWakeUpDelay (if configured) provided by lpwus-LoFrameOffsetList.

- For the case when both offsetForLongerWakeUpDelay and offsetForShorterWakeUpDelay are configured,

  - If the gap between the end of the last LP-WUS MO the UE would monitor in the LO associated with the value in offsetForShorterWakeUpDelay and the start of the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO associated with the value in offsetForShorterWakeUpDelay;

  - Otherwise if the gap between the end of the last LP-WUS MO the UE would monitor in the LO associated with the value in offsetForLongerWakeUpDelay and the start of the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO associated with the value in offsetForLongerWakeUpDelay;

  - Otherwise, the UE monitors legacy PO.

- For the case when only one of offsetForLongerWakeUpDelay and offsetForShorterWakeUpDelay is configured,

  - If the gap between the end of the last LP-WUS MO the UE would monitor in the LO associated with the value in offsetForLongerWakeUpDelay or offsetForShorterWakeUpDelay (whichever is configured) and the start of the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO;

  - Otherwise, the UE monitors legacy PO.

RAN1 respectfully asks RAN2 to capture the above agreement in TS 38.304.

# 2	Actions

To RAN2:

ACTION: 	RAN1 respectfully asks RAN2 to capture the above agreement in TS 38.304.

# 3	Dates of next TSG-RAN WG1 meetings

TSG-RAN WG1 Meeting #123 	17 November – 21 November 2025		Dallas, USA

TSG-RAN WG1 Meeting #124 	9 February – 13 February 2026     		Gothenburg, SE