3GPP TSG RAN WG1 #123			R1-2508448

Dallas, USA, Nov 17th – 21st, 2025

Source: 	CMCC

Title:	Discussion on R2D signals, channels, waveform and procedures

Agenda item:	10.3.2.1

Document for:	Discussion/Decision

- Introduction

RAN#108 has agreed R20 SI to study enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices [1]. RAN1 related objectives are as followings,

Objectives - Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]: - Deployment scenario 4 with topology 1 with no external carrier wave. - Traffic types DO-DTT, DT, DO-A. - Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. - Assumption of the same air interface for Device 2b and Device C. - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 - Positioning for outdoor scenarios is under objective 2. This objective begins after RAN#108 (June 2025) RAN#111 (March 2026) will make a decision on whether to include outdoor scenarios in Rel-20 normative work. This objective includes: RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios - Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C. - While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m - Maximum transmit power between -3 dBm and +5 dBm for device C - RAN1 to recommend feasible values within this range - Maximum transmit power of -20 dBm and -10 dBm for device 2b Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources. - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations. - Define link budget calculation for coverage.

During RAN1#122bis meeting, significant progress has been made [2]. In this contribution, we continually provide our views on R2D related aspects based on the outcomes.

- Waveform and modulation

For R19 R2D modulation, OOK is adopted for passive device with simple envelope detection. In R20, despite the enhanced device capability, the demodulation remains non-coherent (no CFO/channel estimation). In this context, OOK modulation based on DFT-s-OFDM waveform from R19 can be a starting point for R20 R2D. The benefits of OOK modulation utilizing the DFT-s-OFDM waveform include: (1) Improved spectral efficiency (compared to pure OOK); (2) Single-carrier characteristics (low PAPR, suitable for power-constrained devices); (3) OOK mapped to DFT-s-OFDM symbols maintains non-coherent detection (no CFO/channel estimation needed), which only need low-power envelope detection.

Proposal 1: For R20 R2D, OOK modulation based on DFT-s-OFDM waveform is supported.

In RAN1#122, the following was agreed for R2D M value:

Agreement For R2D, study necessity of addition/removal of M values from Rel-19 M value set for Device 2b and for Device C for at least the following M values: - Addition of M=1 - Addition of M=32 - Removal of M=24

A smaller M value, indicating fewer chips per OFDM symbol, leads to a higher energy per chip due to the longer chip duration. With higher energy per chip, the receiver can more reliably detect the signal even at very low signal-to-noise ratios (SNR). Consequently, the operational range of the link is thereby expanded, as the signal remains decodable at greater distances under poor SNR conditions.

Proposal 2: For R2D modulation, M = 1 could be considered to improve the coverage performance.

For R19 OOK, variable values of M can be considered to support different downlink data rates. In TR 38.769, the following Table was agreed as the start point for determining M values and associated minimum Btx,R2D:

Table 6.1.1.4-1 is a starting point for study of M values and the associated minimum Btx,R2D value. The reader can use any transmission bandwidth greater than or equal to the minimum Btx,R2D value. Note: Depending on further study, the maximum value of M may be less than 32. Note: The performance can be better when transmission bandwidth greater than the minimum Btx,R2D, depending on device processing and transmit power constraint. Table 6.1.1.4-1: Starting point for M values and the associated minimum Btx,R2D value M; Minimum Btx,R2D # of PRBs. 1; 1. 2; 1. 4; 1. 6; 1. 8; 2. 12; 2. 16; 2. 24; 3. 32; 4.

In TR 38.769, Table 6.1.1.4-1 is derived from the R2D double-sideband (DSB) modulation assumption. But actually, single-sideband (SSB) modulation is a viable option for R2D communication, enabling 50% bandwidth reduction compared to DSB modulation. That is, for 1 PRB, the use of SSB modulation would theoretically allow the maximum M value in Table 6.1.1.4-1 to be doubled from 6 to 12. To verify this, we conducted simulations using an OOK-4 waveform generation method based on SSB modulation, which involves the following steps:

Step 1: Generation of the time-domain signals

- Given M chips per OFDM symbol, the time-domain signal is y0, y1,y2, …, yM-1.

- Resample the time-domain signal to N points z0, z1,z2, …, zN-1, where N is the FFT size.

Step 2: Transform to frequency domain

- DFT transform of the time-domain signal z={z0, z1,z2, …, zN-1} to obtain the frequency-domain sequence x={x0, x1,x2, …, xN-1}. The DFT output is naturally ordered starting with the DC component (index 0), followed by positive frequencies up to the Nyquist frequency, and then by the negative frequencies.

- FFT shift of x into x=x0, x1,x2, …, xN-1. This operation centers the spectrum by moving the DC component to the middle of the sequence (index N/2). The resulting order is: negative frequencies (in decreasing order of their absolute value), the DC component, and then positive frequencies (in increasing order).

Step 3: Remove the components that outside the transmission bandwidth

- Assuming the transmission bandwidth is K FFT points (e.g., BW/15kHz), nulling all elements (set to zero) of x except xN2, xN2+1, …, xN2+K-1. This retains the DC component and the adjacent K−1 positive frequency components.

- Cyclic shift K/2 points to the left, so that the DC is at the center of the N/2 to N/2+K-1 points. The out put signal is written as x'

- FFT shift of the x', the output is written as x''

Step 4: generate time-domain transmitted OOK-4 signal

- IFFT of x''. The output is written as z'

- Insert CP for signal z'

- Adding Tx Filter (optional) so that the CP burst (if exist) can be suppressed.

Following the above steps, the BLER performance of the PRDCH with respect to the M value and transmission bandwidth is evaluated, and the results are shown in Fig. 1. For a 10% BLER target with 1 PRB, M=12 fails to meet the target while M=8 succeeds. This implies that the optimization potential of SSB modulation in achieving the target BLER with M=8 and a minimal bandwidth of 1 PRB.

Figure 1: Decoding performance of SSB for M=8 and M=12

Observation 1: For R2D, SSB modulation shows optimization potential in achieving the 10% BLER target with M=8 using a minimal bandwidth of only 1 PRB.

Proposal 3: For R20 R2D, SSB modulation could be considered to reduce bandwidth occupancy and support larger M value for one RB.

- R2D line coding, FEC, and repetition

3.1 Line coding

In R19 R2D, line coding is implemented to serve two critical functions: (1) it prevents the consecutive transmission of bit “1” or bit “0”, and (2) it embeds clock synchronization information through deliberate voltage transitions. The rising or falling edges in each codeword helps the device to track chip-level timing in scenarios with large sampling frequency offsets (SFOs).

For R20 Device 2b/C, however, the improved local oscillator precision reduces the need for such timing assistance. Furthermore, the elimination of line coding provides a substantial throughput advantage - since line coding typically imposes a 50% rate penalty, its removal in R20 directly translates to higher peak data rates.

Proposal 4: For R20 R2D, line coding may not be reused for high peak data rates.

3.2 FEC

In RAN1#122b, the following was agreed for R2D FEC:

Agreement For the study of necessity and feasibility of FEC for R2D for Device 2b and for Device C, further study the following aspects: - Coding schemes - LTE TBCC - Tailed convolutional code with the same polynomial as LTE TBCC - RM code - FEC code rates: 1/2, 1/3, 1/4

For RM codes, decoding complexity grows rapidly with code length, making them particularly suitable for short-code applications. In the 5G NR, RM codes support only a limited number of information bits, i.e., in the range of 3 to 11 bits. Therefore, due to the restricted operational range, RM codes are deprioritized in A-IoT scenarios where more flexible and longer codes are often required.

Proposal 5: For R20 R2D FEC, RM code is deprioritized due to the restricted operational range.

For LTE TBCC and tailed CC, we analyze their pros and cons by combing the following agreement in RAN1#122b.

Agreement For R2D and D2R, study the order of channel coding (including interleaver if supported) and block-level repetition (if supported) for complexity reduction, considering necessity and feasibility.

The primary distinction between LTE TBCC and tailed CC lies in the initialization and termination of the encoder state. For LTE TBCC, the encoder's starting state is identical to its ending state. This circular encoding structure makes TBCC insensitive to the order of encoding and repetition, simplifying transceiver design. In contrast, the tailed CC uses zero-padding to initialize and terminate the encoder state. Thus, tailed CC requires per-block state reset, which is sensitive to the order of encoding and repetition. Moreover, tailed CC suffers from code rate reduction.

Proposal 6: For R20 R2D FEC, LTE TBCC can be considered.

If LTE TBCC is supported, especially the LTE bit collection, rate matching is inherently enabled. This inherent support allows for flexible adjustment of the code rate. Given that device capabilities are limited, a down-selection from the available code rates should be considered. From our perspective, 1/3 code rate is supported when interleaving is not applied.

Proposal 7: For R20 R2D FEC, code rate 1/3 is supported when interleaving is not applied.

3.3 Repetition

For R19 R2D, repetition was not considered, not only because the transmit power is sufficient for indoor coverage, but also because of the limited block combining capability of the device. For R20 R2D, given the increased outdoor coverage target as well as enhanced device capabilities, repetition could be considered, and the following agreement was reached in RAN1#122:

Agreement Study necessity and feasibility of R2D repetitions for R2D for Device 2b and for Device C considering the following repetition types: - Block-level repetitions - Bit-level Type-2 repetitions - Chip-level repetitions (in case of M=1 with FEC if applied)

TR 38.769 defines repetition types:

The study defines repetition types as follows: - Block level: All the bits received from higher layers and/or physical layer (according to what is present) after CRC attachment (if used) are blockwise repeated Rblock times - Bit level type 1: Each bit after CRC attachment (if used) is repeated Rbit times - Bit level type 2: Each bit after both CRC attachment (if used) and FEC (if used) is repeated Rbit times - Chip level: Each chip after line coding (if used) or after square wave modulation (if used) is repeated Rchip time. NOTE: This is equivalent to extending the duration of each chip by Rchip times.

To enhance coverage, block-level repetition is theoretically the most effective technique. It allows the receiver to combine multiple copies of the entire data block, maximizing time diversity gain and effectively mitigating deep fades. However, it requires the device to allocate memory to store all repeated copies for combining, posing a challenge for memory-constrained devices. Furthermore, it introduces considerable processing latency as decoding cannot begin until all repetitions are received. Considering the enhanced capabilities and increased storage of device 2b/C, the block-level repetition can be an option.

Proposal 8: To achieve the outdoor coverage design target for R20 R2D, block-level repetition could be considered to enhance link reliability.

- R2D signal and channel

4.1 R2D synchronization signal/channel

Periodic synchronization signal

In the last RAN1 meeting, an R2D synchronization signal for at least frequency synchronization, frequency acquisition, and timing synchronization/tracking was studied and the following agreement has been made:

Agreement Study R2D synchronization signal at least for the functionality of frequency synchronization (including frequency acquisition) and for the functionality of timing synchronization/tracking. - Alt1: periodic synchronization signal - Alt2: aperiodic synchronization signal

During the discussion, views on whether the synchronization signal should be periodic or aperiodic were controversial, and should be further studied.

Rel-19 A-IoT system in indoor scenarios can be treated as an “asynchronized” system, there is no periodic synchronization signal/channel considering the following reasons:

- The initial SFO for Device 1 is up to 105 ppm, it is not feasible for Device 1 to keep time synchronization with the reader.

- Device 1 is RF-ED receiver, and there is no need for Device 1 to search a signal transmitted for frequency synchronization and acquisition for R2D reception.

- Rel-19 A-IoT does not consider mobility, and the device does not need to perform measurement for cell selection/re-selection-like function.

- The traffic type of Rel-19 A-IoT only considers DO-DTT and DT, where all D2R transmissions are triggered by a corresponding R2D message. The device can acquire timing information from the R2D transmission.

In contrast, for Device 2b/C, designs should be re-visited considering the following characteristics:

- In RAN1#122bis meeting, the assumptions for SFO after calibration have been confirmed. For Device 2b, the SFO after calibration is 100 ~ 1000 ppm, and for Device C, it is 10 ppm. In this case, assuming 10~100 ppm SFO after calibration, the timing error is only 1~10 us with accumulation of 100 ms. Hence, it is feasible for Device 2b/C to maintain time synchronization.

- Device 2b/C is IF-ED or ZIF receiver, the device needs to search the carrier frequency to receive R2D transmission.

- Considering typical use cases such as logistics for outdoor scenarios, it is reasonable for the device to support cell-selection/re-selection-like mobility function.

- Rel-20 A-IoT should support DO-A traffic, where a device should acquire time synchronization to autonomously send the very first D2R transmission without any R2D message.

Observation 2: The Rel-20 A-IoT R2D design should be applicable to the following characteristics:

- The SFO after calibration for Device 2b/C is much smaller, e.g., tens or hundreds of ppm, which is feasible to maintain time synchronization.

- Device 2b/C with IF-ED or ZIF receiver needs to search the carrier frequency to receive R2D transmission;

- Rel-20 A-IoT for outdoor wide area deployment is reasonable to support cell-selection/re-selection-like mobility function.

- Rel-20 A-IoT should support DO-A traffic, a device should acquire time synchronization to autonomously send the very first D2R transmission

From the analysis, to support the above characteristics of Device 2b/C, if the synchronization signal is aperiodic, or there is no default assumption of a periodicity from a device perspective to monitor the synchronization signal, a device has to keep monitoring. With a peak power consumption of hundreds of uW or several mW, it is not feasible for a device to keep monitoring R2D transmissions. Therefore, a periodic synchronization signal is required.

Observation 3: If the synchronization signal is aperiodic, or there is no default assumption of a periodicity from a device perspective to monitor the synchronization signal, a device has to keep monitoring, which is not feasible for Device 2b/C with much larger peak power consumption than Device 1.

Proposal 9: For the R2D synchronization signal at least for the functionality of frequency synchronization (including frequency acquisition) and for the functionality of timing synchronization/tracking, support Alt. 1, i.e.,

- Alt. 1: periodic synchronization signal.

Periodicity of the periodic synchronization signal

To provide at least the functionality of frequency synchronization including frequency acquisition and timing synchronization/tracking, the periodicity of the synchronization signal should consider the following,

- The overall frequency acquisition and reader identification latency is not too long to avoid consuming too much energy storage from device perspective. Note that for A-IoT system, as the R2D uses OOK waveform, which is sensitive to interference, most likely, the deployment of adjacent readers/cells will use different carriers located in different frequency locations (i.e., located on different synchronization raster).

- The timing drift due to SFO within the periodicity of the synchronization signal. As we calculated, assuming 10~100 ppm SFO after calibration, the timing error with accumulation of 100 ms is only 1~10 us, which is able to maintain the timing synchronization.

- The overhead of synchronization signal/channel within the periodicity is reasonable.

From the above analysis, the periodicity of the synchronization signal can be hundreds of ms. For example, 160 ms, can be considered as reasonable, and used as a starting point.

Proposal 10: The periodicity of the synchronization signal can be hundreds of ms, e.g., 160 ms, as a starting point.

Periodic broadcast information

In existing systems such as LTE/NR and NB-IoT, the broadcast information that provides necessary system information for cell search and random access includes MIB and SIB1. For Rel-20 A-IoT, to support the above-mentioned characteristics, no matter the broadcast information that provides necessary system information is split into two parts or not, the transmission should be periodic to avoid the device keep monitoring the broadcast information.

In addition, to support DO-A traffic, we think that the necessary information to provide the occasions of the very first D2R transmission can also be conveyed by the SIB1-like broadcast information, since the DO-A traffic arrives randomly, the broadcast information to provide first D2R transmission of DO-A should also be sent periodically.

Proposal 11: The transmission of broadcast information, including the necessary information for the very first D2R transmission of DO-A should be periodic.

We then discuss the information included in the broadcast information. In the existing systems such as NR and NB-IoT, upon reception of SSB (or PSS/SSS and PBCH) and SIB1, a UE can obtain the physical cell identity (PCI), system frame number (SFN), cell-specific configuration parameters (e.g., PDCCH monitoring configurations, RO configurations), cell selection related information, etc. Since the overall payload of the system information is large, it is more efficient to split the system into different parts.

In Rel-20 A-IoT, at least the following information is necessary to be provided for initial frequency acquisition, random access, and DO-A traffic:

- Cell/reader ID: Rel-20 A-IoT system is for outdoor scenarios, therefore, from the perspective of network deployment and device identification/differentiation of cell/reader, cell ID is needed. In current mechanisms, the PCI is delivered by synchronization signal. Upon blind detection of PSS and/or SSS, a UE can identify the PCI of a cell. The PCI is then used to generate the scrambling sequence of physical channels (e.g., PBCH) or reference signals, to randomize interferences from adjacent cells. In Rel-20 A-IoT, however, due to the limited complexity of Device 2b/C, how to carry the cell/reader ID should be further studied. RAN1 can further study the following options:

  - Option 1: Cell/reader ID is carried in MIB-like broadcast information.

  - Option 2: Cell/reader ID is carried by synchronization signal.

  - Option 3: Combination of Option 1 and Option 2.

Observation 4: Rel-20 A-IoT system is for outdoor scenarios, therefore, from the perspective of network deployment and device identification/differentiation of cell/reader, cell/reader ID is needed.

- Timing information (e.g., SFN-like information): In existing systems, SFN information is provided so that a UE can acquire the time domain granularity and timeline information of a cell. In Rel-20 A-IoT, similar information is also needed for Device 2b/C. For example, to support DO-A traffic, the configuration information of first D2R transmissions (e.g., the sequence-based Msg1 transmission) should be provided. To determine the time domain occasions including the periodicity and durations of the first D2R transmission, a time domain scheduling granularity and timeline information is required for a device to count and determine. In addition, based on the study of the device (un)availability in Rel-19, it is a common understanding that for Device 2b/C, it is not feasible for a device to keep monitoring due to a much larger power consumption, and a DRX-like sleeping pattern is required. A device only needs to wake up and monitor R2D messages (e.g., paging) during the ON state. In this sense, an SFN-like timing information is also required to configure the DRX-like pattern.

Observation 5: In Rel-20 A-IoT, to support characteristics such as DO-A traffic and DRX, timing information (i.e., a time domain granularity and timeline information) is necessary for a device to determine the time domain occasions of first D2R transmission for DO-A, and to determine the DRX configurations.

- Configurations of cell/reader-common L1 R2D control information: According to our discussions in the following sub-section, in Rel-20 A-IoT system, there exist cases that a physical channel carriers L1 R2D control information only, and hence, a more efficient solution is to carry L1 R2D control information in an independent physical channel. In this sense, configurations of cell/reader-common L1 R2D control information should be provided by the broadcast information.

- Necessary information of first D2R transmission for DO-A: To support DO-A traffic, the necessary information of the very first D2R transmission should be carried in broadcast information. Once a DO-A traffic arrives, upon reception of the broadcast information, a device can obtain the transmission configurations.

- DRX-related configuration for paging monitoring: Unlike device 1/2a, device 2b/C are expected to work outdoors under macro-coverage, so the received signal level will be lower. Then R19 SIP-based detection can no longer applicable for start indication of R2D channel, which means detecting arriving of R2D channel will consume more energy. So it will be impractical for device to continuously monitor possible paging message due to its limited stored energy. DRX cycle for paging monitoring should be studied that allows the device to wake up only at paging occasions. The broadcast information shall therefore supply the DRX parameters for paging (e.g. cycle, offset) so that the device can decide exactly when to detect paging.

From the discussion, it is noted that the payload to be carried in the broadcast information is expected to be large, to reduce the overhead and improve the efficiency, it is more reasonable to split the broadcast information into two parts, i.e., a MIB-like part and a SIB1-like part. For the MIB-like broadcast information, it can carry the required information for initial frequency acquisition, cell/reader identification/differentiation, and timing information. The other necessary information can then be carried in the SIB1-like information.

Observation 6: The payload to be carried in the broadcast information is expected to be large, it is more reasonable to split the broadcast information into two parts, i.e., a MIB-like part and a SIB1-like part.

Observation 7: For the MIB-like broadcast information, it can carry the required information for initial frequency acquisition, cell/reader identification/differentiation, and timing information. The other necessary information can then be carried in the SIB1-like information.

Proposal 12: In Rel-20 A-IoT, at least the following information should be studied to be included in the broadcast information:

- Cell/reader ID;

- Timing information (e.g., time domain granularity and timeline information);

- Configurations of cell/reader-common L1 R2D control information;

- Necessary information of first D2R transmission for DO-A;

- DRX configuration for paging monitoring.

Physical channel of the broadcast information

In NR, MIB is carried via PBCH and SIB1 is carried via PDSCH. As discussed above, we think that it is more efficient to split the broadcast information into two parts, i.e., a MIB-like part and a SIB1-like part. For the MIB-like part, RAN1 should further study whether a new physical broadcast channel is needed. One option is to reuse the mechanism in NR that to define a physical broadcast channel for A-IoT to carry MIB-like broadcast information. As the time/frequency resources and MCS for this channel is pre-defined, without any prior scheduling information, a device can successfully decode the MIB-like broadcast information. The other option is to use PRDCH to carry such information. In such a case, scheduling information including time/frequency resources with respect to the periodic synchronization signal and MCS is pre-defined for the PRDCH format to carry the MIB-like broadcast information.

Proposal 13: In Rel-20 A-IoT, RAN1 should study the following two options to carry the MIB-like broadcast information:

- Option 1: A new physical broadcast channel is defined.

- Option 2: Use PRDCH. The scheduling information including time/frequency resources with respect to periodic synchronization signal and MCS is pre-defined for the PRDCH format.

Time domain structure of the periodic synchronization signal/channel

In RAN1#122bis meeting, the following agreement has been made to further study the time relationship of the broadcast information to the synchronization signal.

Agreement For R2D, study at least the following aspects for the study of broadcast information: - Information included in the broadcast information - Whether to use PRDCH to carry the broadcast information - Transmission of broadcast information: - Option 1: periodic - Option 2: aperiodic - Time and frequency resources for transmission of the broadcast information, e.g. in relation to the synchronization signal.

According to the discussion above, the time resources for the broadcast information are separately considered for MIB-like part and SIB1-like part.

For SIB1-like part, as it can be carried in a PRDCH, its time resources can be indicated by MIB-like part or scheduled by the corresponding L1 R2D control information.

Proposal 14: For the SIB1-like broadcast information, the time resources can be indicated by MIB-like part or scheduled by the corresponding L1 R2D control information.

For the time resources of the MIB-like part, the following two options can be studied:

- Option 1: The physical channel carrying MIB-like part is transmitted separately with the synchronization signal. The periodicity and time resources are pre-defined.

- Option 2: The physical channel carrying MIB-like part and the synchronization signal are transmitted as a block, and the synchronization signal is preceding the MIB-like part.

Figure 2: Time domain structure of periodic synchronization signal and periodic channel carrying system information.

Proposal 15: For the SIB1-like broadcast information, the following two options of time resources can be studied:

- Option 1: The physical channel carrying MIB-like part is transmitted separately with the synchronization signal. The periodicity and time resources are pre-defined.

- Option 2: The physical channel carrying MIB-like part and the synchronization signal are transmitted as a block, and the synchronization signal is preceding the MIB-like part.

Sequence of the synchronization signal

As the main functionality of the periodic synchronization signal includes time synchronization and frequency acquisition, a binary sequence with good auto-correlation property, e.g., m-sequence, can be considered as one feasible solution. The Device 2b/C can perform sequence correlation to determine the timing information and the frequency for R2D reception.

Proposal 16: For the synchronization signal, a binary sequence (e.g., m-sequence) with good auto-correlation property should be studied.

In addition, as discussed in previous sub-sections, the sequence design of the synchronization signal is also related to how to carry the cell/reader ID. To be specific,

- Option 1: Cell/reader ID is carried in MIB-like broadcast information. In this option, unlike NR or NB-IoT, the cell/reader ID is not carried by the synchronization signal, which is friendly to device blind detection complexity, therefore, only one sequence is required for the synchronization signal.

- Option 2: Cell/reader ID is carried by synchronization signal. In this option, similar as NR or NB-IoT, the cell/reader ID is carried by the synchronization signal. It should be noted that due to the limited device complexity, it may be difficult for a device to blind detect multiple sequences without any timing synchronization. One possible solution is to consider a two-part synchronization signal, part-1 synchronization signal (similar as PSS) has only one sequence, which does not require the device to blind detect multiple sequences, and part-2 synchronization signal (similar as SSS) has multiple sequences to carry the cell/reader ID. RAN1 can further study use CFO calibration signal as the type-2 synchronization signal and how to carry the cell/reader ID.

- Option 3: Combination of Option 1 and Option 2.

Proposal 17: In Rel-20 A-IoT, RAN1 studies whether and how to use synchronization signal to carry cell/reader ID:

- Option 1: Cell/reader ID is not carried by the synchronization signal. One sequence is required for the synchronization signal.

- Option 2: Cell/reader ID or part of cell/reader ID is carried by the synchronization signal. RAN1 can study a two-part synchronization signal to reduce the device blind detection complexity, where part-1 synchronization signal uses one sequence, and part-2 synchronization signal uses multiple sequences to carry cell/reader ID.

4.2 CFO calibration signal

In the last RAN1 meeting, the following agreement has been made on CFO calibration signal:

Agreement For the functionality of CFO estimation/LO calibration at the device side, study CFO calibration signal with the following aspects: - Whether the signal is - Option a: unmodulated sinusoid single tone - Option b: unmodulated multiple sinusoid single tones - Option c: modulated multiple tones - Whether the CFO calibration signal is the same or different from the synchronization signal (if supported) - Whether the CFO calibration signal is transmitted periodically and/or as needed (aperiodically) - The time location(s) of CFO calibration signal. - Option 1: in relation to the L1 R2D control information - Option 2: in relation to the data payload of the PRDCH - Option 3: in relation to the PDRCH - Option 4: in relation to the synchronization signal (if supported) - Option 5: in relation to broadcast information (if supported) - Combination of Options is not precluded - Other details (e.g., time duration, frequency location(s) of the CFO calibration signal)

Design of the CFO calibration signal

During the discussion in the last RAN1 meeting, companies’ views on the CFO calibration signal is diverse and three options were listed for further study. For Option a and Option b, the CFO calibration signal is unmodulated sinusoid tone(s). For Option c, the CFO calibration signal is modulated multiple tones.

First of all, Option c is not feasible for Device 2b/C for CFO calibration. The reason is that for R2D, Device 2b/C is non-coherent reception and the modulated tones at the baseband only contains amplitude information and no phase information.

Observation 8: For Device 2b/C, the R2D reception is non-coherent, the modulated tones at the baseband only contains amplitude information and no phase information.

Observation 9: It is not feasible to use modulated multiple tones for CFO calibration.

CFO calibration can only be implemented at RF or IF domain, the feasible solution is to use an unmodulated sinusoid single tone for calibration, i.e., Option a is feasible. To be specific, the reader transmits the CFO calibration signal with a pre-defined duration, the device mixes the CFO calibration signal with its internally generated wave, can then count the number of periodicities of the residual frequency within the duration to estimate and calibrate the CFO.

Observation 10: It is feasible to use an unmodulated sinusoid single tone to calibrate CFO at RF or IF domain.

The following table shows the performance using an unmodulated single-tone as the CFO calibration signal. The initial CFO is assumed to be 100 ppm. TDL-C channel with RMS delay of 300 ns and SNR = 12 dB is assumed in the evaluation.

| Duration of CFO calibration signal | 1 ODFM symbol | 4 OFDM symbol | 10 OFDM symbol |
| --- | --- | --- | --- |
| CFO after calibration | 8.67 ppm | 6.39 ppm | 5.02 ppm |

It is observed that with the increase of the duration of the CFO calibration signal, the CFO after calibration reduces as expected. With a duration of 1 OFDM symbol, the CFO after calibration can be reduced to less than 10 ppm.

Observation 11: With the increase of the duration of the CFO calibration signal, the CFO after calibration reduces.

Observation 12: With a duration of 1 OFDM symbol, the CFO after calibration can be reduced from the initial 100 ppm to less than 10 ppm.

Proposal 18: For the CFO calibration signal, support Option a, i.e.,

- Option a: the signal is an unmodulated sinusoid single tone.

Periodic CFO calibration signal

Similar as the synchronization signal, for the CFO calibration signal, RAN1 should further study whether the CFO calibration signal is transmitted periodically and/or as needed (aperiodically).

Different from Device 1, the D2R transmission of Device 2b/C is internally generated. Therefore, for D2R transmission, CFO calibration is required to reduce the potential guard interval. As mentioned in previous sections, Rel-20 A-IoT should support DO-A traffic types, a device should monitor CFO calibration signal and perform CFO calibration before sending the very first D2R transmission. Hence, at least periodic CFO calibration signal is required to avoid a device from keep monitoring.

Observation 13: To support DO-A traffic, periodic CFO calibration signal is required to avoid a device from keep monitoring the CFO calibration signal before transmitting the very first D2R transmission.

In addition, regarding whether aperiodic CFO calibration signal is required, with a periodicity of hundreds of ms of the CFO calibration signal, the CFO drifting is very trivial and therefore no need to introduce aperiodic CFO calibration signal. More details are in the following sub-sections.

Observation 14: With a periodicity of hundreds of ms of the CFO calibration signal, the CFO drifting is very trivial and therefore no need to introduce aperiodic CFO calibration signal.

Proposal 19: CFO calibration signal is transmitted periodically.

Periodicity of the CFO calibration signal

In the study item phase of Rel-19, the drift rate of CFO for Device 2b has been assumed in the evaluation, which is 0.1 or 1 ppm/s, and up to companies to report. Even if 1 ppm/s is considered, the drifting is not drastic, the frequency offset error is only 900 Hz with accumulation of 1 s, and the LO after calibration can be considered as stable. In this sense, the periodicity of the CFO calibration signal can be the same as the periodic synchronization signal, which is a reasonable trade-off to provide sufficient CFO calibration and stabilization performance and to avoid large overhead for CFO calibration.

Observation 15: The drift rate of CFO is 0.1~1 ppm/s, the frequency drifting is only 900 Hz with accumulation of 1 s, which is not drastic.

Proposal 20: The periodicity of the CFO calibration signal can be the same as the periodic synchronization signal, i.e., use 160 ms as a starting point.

Time locations of the CFO calibration signal

For the time location(s) of the CFO calibration signal, a series of options and potential combinations were listed in the last meeting for further study.

First, it should be noted that for Device 2b/C, the R2D is still OOK waveform and the reception is non-coherent reception, and hence it is robust to CFO. From this perspective, the CFO calibration signal is not related to R2D reception, and therefore the time domain location does not require to be in relation to R2D channels carrying L1 R2D control or data payload.

Observation 16: The R2D is OOK waveform and based on non-coherent reception, which is robust to CFO. The CFO calibration signal is not related to R2D reception and the location does not require to be in relation to R2D channels carrying L1 R2D control or data payload.

Second, as mentioned above, the CFO drift rate is 0.1 or 1 ppm/s, after CFO calibration, the drifting is very slow. From this perspective, the CFO calibration does not require to be in relation to, e.g., close to, D2R transmissions.

Observation 17: The CFO drift rate is 0.1 or 1 ppm/s, after CFO calibration, the drifting is very slow. The CFO calibration does not require to be in relation to, e.g., close to, D2R transmissions.

With the assumption that the periodicity of CFO calibration signal and synchronization signal is the same, it is straightforward that the transmission of the CFO calibration signal can be in relation to the synchronization signal, i.e., Option 4. In such a case, the periodic signal for CFO calibration can be transmitted immediately after the periodic synchronization signal. For a device to acquire time/frequency synchronization at the first place, as the device performs non-coherent reception for R2D OOK waveform, it is robust to CFO and it can first search the periodic synchronization signal for time synchronization and frequency acquisition. The device can then use the periodic signal for CFO calibration to facilitate D2R transmission.

Furthermore, as analyzed that the broadcast information (e.g., MIB-like information) can be transmitted together with the synchronization signal, the CFO calibration signal can also be in relation to the broadcast information, i.e., Option 5. In such a case, the synchronization signal, CFO calibration signal, and MIB-like broadcast information can be transmitted as a block.

Figure 3: Time domain structure of periodic signal for CFO calibration and periodic synchronization signal.

Proposal 21: For the time location(s) of the CFO calibration signal, the following two options can be studied:

- Option 4: in relation to the synchronization signal (if supported).

  - The CFO calibration signal can be transmitted immediately after the synchronization signal.

- Option 5: in relation to broadcast information (if supported).

  - The synchronization signal, CFO calibration signal, and MIB-like broadcast information can be transmitted as a block. The CFO calibration signal can be transmitted immediately after the synchronization signal, or immediately after the MIB-like broadcast information.

4.3 R2D signal to indicate the start of the R2D transmission

In Rel-19 A-IoT, for each PRDCH transmission, an R2D TAS is transmitted immediately preceding the PRDCH for timing acquisition. A SIP part is included in the R-TAS to indicate the start of R2D.

During the last meeting, the following agreement has been achieved regarding the potential change to the Rel-19 SIP:

Agreement For the functionality of indicating the start of the R2D transmission that contains PRDCH, study at least the following options if the functionality relies on SIP: - Option 1: binary sequence-based SIP (e.g M-sequence or Golay sequence). - Option 2: Reuse the Rel-19 SIP.

Based on the evaluation assumption, SFO after calibration for Device 2b is 100 ppm (M) and for Device C is 10 ppm. According to our discussion on the periodicity of the periodic synchronization signal, which can be 160 ms, taking Device C as an example, the timing drift is 1.6 us in such a case. Note that at device side, it will have a synchronization error of ~ 1/4 chip duration of the synchronization signal, then for M ≥ 6, the total timing error may be larger than 1/4 chip, and it may deteriorate the PRDCH reception performance, if no additional R2D preamble preceding the PRDCH for timing synchronization. However, for smaller M values smaller than 6, the R2D preamble preceding each PRDCH transmission may not be needed.

Observation 18: With a periodic synchronization signal in a periodicity of hundreds of ms, e.g., 160 ms, for Device C, the timing drift is 1.6 us. For M ≥ 6, the total timing error may be larger than 1/4 chip, and it may deteriorate the PRDCH reception performance, if no additional R2D preamble preceding the PRDCH for timing synchronization. For M < 6, the R2D preamble preceding each PRDCH transmission may not be needed.

If a SIP part is used preceding the PRDCH transmission, besides the functionality to indicate the start of the R2D transmission, it should also be able to allow a device to acquire the timing synchronization. From this perspective, Option 1, i.e., a sequence-based SIP, should be supported. A device can perform sequence correlation to achieve more accurate time synchronization.

Observation 19: A sequence-based SIP part can allow a device to perform correlation operation to achieve more accurate time synchronization.

Proposal 22: If a SIP part preceding a PRDCH is used for timing synchronization, support Option 1, i.e.,

- Option 1: binary sequence-based SIP (e.g M-sequence or Golay sequence)

4.4 R2D signal to indicate the chip duration of PRDCH and L1 R2D control

In Rel-19 A-IoT, for each PRDCH transmission, a CAP part is defined to determine the chip duration of the subsequence PRDCH.

During the last RAN1 meeting, on how to determine the chip duration of L1 R2D control part and the data payload in the PRDCH for Rel-20 A-IoT, the following agreements have been reached:

Agreement For the functionality of chip duration determination of the data payload in the PRDCH, study the following options: - Option 1: same as the chip duration of the corresponding L1 R2D control information - Option 2: indicated by the corresponding L1 R2D control information Agreement For the functionality of chip duration determination of the L1 R2D control information, study at least the following options: - Option 1: Fixed chip duration. No use of CAP for chip duration determination. - Option 2: Blind detection from a predefined set of the chip durations of L1 R2D control information. No use of CAP for chip duration determination. - Option 3: Use CAP - Alt.3-1 Rel-19 CAP pattern - Alt.3-2 Enhanced CAP - Alt.3-2a: Rel-19 CAP pattern with repetition - Alt.3-2b: CAP pattern and the association between chip durations for CAP and chip durations of L1 R2D control information - Option 4: Use a set of different binary sequences with fixed or variable length - Alt.4-1: using binary sequences for SIP. No use of CAP for chip duration determination. - Alt.4-2: using binary sequences for CAP - Option 5: Use broadcast information (if any) to indicate the chip duration

Indication of the chip duration of data payload of PRDCH

To indicate the chip duration of the data payload of PRDCH, Option 2 should be supported. Option 1 mandates that the chip duration of L1 R2D control information is the same as the scheduled PRDCH, however, the transmission reliability of L1 R2D control may not be the same as the data payload. Similar as LTE/NR, the BLER is 1% for PDCCH and 10% for PDSCH. From this perspective, it is not reasonable to restrict the chip duration of L1 R2D control and the corresponding PRDCH. In addition, from efficiency perspective, link adaptation of PRDCH should be allowed. Therefore, the chip duration of the data payload of PRDCH should be flexibly indicated by the corresponding L1 R2D control.

Observation 20: The transmission reliability of L1 R2D control may not be the same as the data payload.

Observation 21: Using the same chip duration of data payload of PRDCH and the corresponding L1 R2D control information precludes the link adaptation of PRDCH.

Proposal 23: For the functionality of chip duration determination of the data payload in the PRDCH, support Option 2, i.e., indicated by the corresponding L1 R2D control information.

Indication of the chip duration of L1 R2D control

For the functionality of chip duration determination of the L1 R2D control information, five different options were listed for further study in RAN1 and it is open for introducing other options. In the following, we provide our observations on different options.

Regarding Option 1, where a fixed chip duration is considered for the L1 R2D control information, it is quite inefficient for the Rel-20 outdoor scenarios. It should be noted that in outdoor scenarios with large coverage, the devices are with different coverage levels, apparently the chip duration of L1 R2D control information targeting for cell-center, cell-medium, and cell-edge devices can be different. If fixed chip duration is assumed for all L1 R2D control information, the link adaptation is precluded and will harm the system efficiency.

Observation 22: Option 1 is inefficient for Rel-20 A-IoT system. If fixed chip duration is assumed for all L1 R2D control information, the link adaptation is precluded and will harm the system efficiency.

Regarding Option 3, different alternatives under this option were captured. The solution either reuses the Rel-19 CAP, or enhances the Rel-19 CAP patterns and the association between chip durations. Essentially, the feasibility of Option 3 is based on the prerequisite that the device will perform edge detection to determine the chip duration based on CAP. However, in outdoor scenarios, to achieve a MCL ~144 dB for co-site deployment, the required CNR/SNR at the device side is lower than 10 dB. Under this low CNR/SNR region, the edge detection is not feasible.

Observation 23: The feasibility of Option 3 is based on the prerequisite that the device will perform edge detection to determine the chip duration based on CAP. However, in outdoor scenarios, the required CNR/SNR at the device side is lower than 10 dB, which is not feasible for performing edge detection.

Besides Option 1 and Option 3, the other options do not rely on edge detection-based CAP to indicate the chip duration, which are applicable for outdoor scenarios. According to our discussion in the following sub-section, we think that for Rel-20 A-IoT system, unlike Rel-19 that the L1 control part is always transmitted together with data payload, there will be L1 control information only transmission. Therefore, the transmission of L1 R2D control information is basically via an independent physical channel. From this perspective, the transmission parameters including chip duration of the L1 R2D control information can be provided by broadcast information or by configuration signaling, and hence, Option 2, Option 5, and the newly proposed Option 6 (see below) are more reasonable solutions than Option 4.

First, Option 5 is preferred for L1 R2D control information common for devices, for example, the L1 R2D control information can be used for scheduling paging message. The concept is similar as the common search space in NR.

In addition, an additional Option 6 is proposed, i.e., use device-specific configuration signaling to indicate the chip duration. In such a case, the L1 R2D control information can be used for scheduling a device specific command message. The concept is similar as the UE-specific search space in NR. Furthermore, to provide more flexibility, the device-specific configuration signaling can provide a set of chip durations for device to perform blind detection. As long as the candidate chip durations are limited, the device blind detection complexity can be acceptable.

Proposal 24: For the functionality of chip duration determination of the L1 R2D control information, the following options should be studied:

- Option 5: Use broadcast information (if any) to indicate the chip duration

- Option 6: Use device-specific configuration signaling to indicate the chip duration.

- Option 2: Blind detection from a predefined set of the chip duration of L1 R2D control information.

  - Option 2 can be combined with Option 6.

4.5 L1 control and physical control channel

In the last RAN1 meeting, the following agreements have been made regarding L1 R2D control information.

Agreement Study L1 R2D control information with separate CRC for Device 2b and for Device C considering the following aspects - Necessary information to be included in the L1 R2D control - Details on how the L1 R2D control information is transmitted Agreement For the L1 R2D control information preceding and scheduling the data payload of PRDCH, study the following aspects: - Time location of L1 R2D control information: - Option 1: L1 R2D control information and data payload of PRDCH are contiguous in time. - Option 2: L1 R2D control information and data payload of PRDCH are not contiguous in time. - Study frequency resource of L1 R2D control information - Whether L1 R2D control information is included in the PRDCH that carries that data payload, or in another PRDCH, or in another channel

Based on the agreement, RAN1 should further study how the L1 R2D control information is transmitted, including whether it should be always transmitted together with the data payload, and which physical channel is used to carry the L1 control information.

Physical channel and time location of L1 R2D control information

In Rel-19, only a single channel PRDCH is supported for R2D. The channel carries R2D block originating from A-IoT higher layer and L1 control part. It is neither necessary nor feasible for Rel-19 A-IoT Device 1 to support physical control channel to convey control information such as DCI in NR. For Rel-20 A-IoT, on the other hand, corresponding designs can be re-visited and further studied.

In Rel-20 A-IoT system, the L1 R2D control information is not necessarily to be always transmitted together with data payload of the PRDCH. For example, from a device perspective, it is not the case that every paging message is targeted to the device itself. If the L1 control information is always transmitted with the data payload, the device has to decode the whole paging message, which is not efficient. In addition, to support the DO-A traffic, it is reasonable to support re-transmissions of the PDRCH. In such a case, only L1 R2D control information carrying re-transmission indication is sent, without any data payload. Therefore, in Rel-20 A-IoT, there exist cases that a physical channel carriers L1 R2D control information only.

Observation 24: In Rel-20 A-IoT, cases that a physical channel carriers L1 R2D control information only exit, e.g., L1 R2D control information to schedule paging, or to schedule D2R re-transmission.

According to the above analysis, introduction of a new physical channel or a new PRDCH format to carry L1 R2D control information only is a more efficient design. However, due to the low complexity and power consumption for Device 2b/C, designs regarding CORESET and search space for control channel monitoring, blind detection and RNTI scrambling, etc., should be largely simplified.

Observation 25: Defining a new physical channel or a new PRDCH format to carry L1 R2D control information only is more efficient for scheduling and resource utilization.

Proposal 25: For the L1 R2D control information preceding and scheduling the data payload of PRDCH, RAN1 should study a R2D control channel carrying control information to schedule PRDCH.

- Designs regarding CORESET and search space for PDCCH monitoring, PDCCH blind detection and RNTI scrambling, etc., should be largely simplified.

With a separate physical channel to transmit L1 R2D control information, it is apparent that the L1 R2D control information and data payload of PRDCH are not contiguous in time.

Proposal 26: For the L1 R2D control information preceding and scheduling the data payload of PRDCH, support Option 2 for the time location of L1 R2D control information, i.e.,

- Option 2: L1 R2D control information and data payload of PRDCH are not contiguous in time.

Necessary information to be included in the L1 R2D control

The content that L1 R2D control needs to carry is directly related to its purpose. As analyzed above, L1 control information can be used either to indicate a paging message or to schedule a retransmission of Msg3. Other PRDCH transmission can also be scheduled through L1 control.

- For R2D scheduling

When it is used for paging, similar to NR, the L1 control message may carry,

- Short message indicator like information, telling devices on SI update notification and/or paging arriving.

- Scheduling information for following paging PRDCH, if any, such as TBS, FEC code-rate, number of repetitions, M value, and the time-offset between the L1 control physical channel and the scheduled PRDCH.

For other PRDCH scheduling the only difference is the absence of the paging/SI update indication field, and all remaining scheduling fields are similar to those used for paging.

- For D2R scheduling

If a PDRCH transmission is accompanied by a preceding PRDCH, the scheduling information for that PDRCH can still be carried in the higher-layer payload (for instance, Msg2 can schedule Msg3). If the D2R transmission occurs alone, e.g., a Msg3 retransmission, then a separate physical channel of L1 control is required. The scheduling fields in this case include,

- Frequency resource for D2R channel, so that device can generate indicated carrier frequency.

- Bit duration Tb. This combines with code rate can determine the D2R data rate.

- TBS, if the D2R payload is variable.

- Time resource, at least slot offset between R2D scheduling channel and scheduled D2R channel can be indicated.

- FEC code rate and repetition information. Since the coverage will be larger for active devices, the code rate can be adjusted by the Reader and indicate to device through control information, to improve efficiency.

Proposal 27: For R2D scheduling, the necessary information to be included in L1 control includes TBS, FEC code-rate, number of repetitions, M value, and the time-offset between the L1 control physical channel and the scheduled PRDCH.

Proposal 28: For R2D scheduling of paging message, the necessary information to be included in L1 control additionally includes short message indication like information.

Proposal 29: For D2R scheduling, the necessary information to be included in L1 control includes frequency resource, bit duration Tb, TBS, time resource, FEC code rate and repetition information, while some of them can be omitted if predetermined.

- Bandwidth and multiplexing

During previous meeting, the following agreements about bandwidth and R2D FDM were made,

Agreement The minimum R2D transmission bandwidth of 1 PRB is the baseline for Rel-20. Agreement Study necessity and feasibility of R2D FDM from the same reader for Device 2b and for Device C considering at least the following aspects: - complexity of device to support FDM - how a device determines the frequency resource for receiving one of the FDMed R2D transmissions

Although 1 PRB is agreed to be baseline for minimum R2D bandwidth of R20, to support higher data rates or larger capacity, A-IoT system design should be scalable. There are two approaches to achieve this,

- Approach 1: Multiple carriers each with single RB

Each carrier supports only one RB. Additional capacity is achieved by configuring additional carriers, akin to non-anchor carriers of NB-IoT.

Pros: The device always operates on a single carrier and does not need to dynamically adjust the bandwidth of its band-pass or low-pass filters.

Cons: The device can only work on one carrier at a time, so the data rate is limited to a few tens of kbps.

- Approach 2: A single carrier with configurable number of RBs

Only one carrier is used, but the number of RBs is configurable (e.g., via system information).

Pros: Enables high-rate transmission for devices in good channel conditions and it can supports FDM of R2D and D2R for different devices within the same carrier.

Cons: The device have to support variable R2D filter bandwidth. For example, if SI is transmitted in 1 RB but the carrier is configured for 3 RBs, the device may need to receive PRDCH on 1, 2, or 3 RBs depending on the frequency bandwidth allocation if FDM is supported. This requires dynamic adjustment of filter bandwidth and IF center frequency.

Therefore, further study is needed to evaluate the feasibility of both sapproaches.

Proposal 30: Two approaches can be studied to support high data rate or high capacity,

- Approach 1: Multiple carriers each with single RB (similar to NB-IoT non-anchor carrier)

- Approach 2: A single carrier with configurable number of RBs

As discussed above, if FDM is supported, dynamic adjustment of filter bandwidth and IF center frequency is required for device, which increases the complexity, while the benefit of FDM is not such clear. So we propose not to support FDM within a single carrier.

Proposal 31: FDM is not support for R2D within a single carrier.

- Conclusion

In this contribution, we discuss the necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios and the following observation and proposals are made:

Observation 1: For R2D, SSB modulation shows optimization potential in achieving the 10% BLER target with M=8 using a minimal bandwidth of only 1 PRB.

Observation 2: The Rel-20 A-IoT R2D design should be applicable to the following characteristics:

- The SFO after calibration for Device 2b/C is much smaller, e.g., tens or hundreds of ppm, which is feasible to maintain time synchronization.

- Device 2b/C with IF-ED or ZIF receiver needs to search the carrier frequency to receive R2D transmission;

- Rel-20 A-IoT for outdoor wide area deployment is reasonable to support cell-selection/re-selection-like mobility function.

- Rel-20 A-IoT should support DO-A traffic, a device should acquire time synchronization to autonomously send the very first D2R transmission

Observation 3: If the synchronization signal is aperiodic, or there is no default assumption of a periodicity from a device perspective to monitor the synchronization signal, a device has to keep monitoring, which is not feasible for Device 2b/C with much larger peak power consumption than Device 1.

Observation 4: Rel-20 A-IoT system is for outdoor scenarios, therefore, from the perspective of network deployment and device identification/differentiation of cell/reader, cell/reader ID is needed.

Observation 5: In Rel-20 A-IoT, to support characteristics such as DO-A traffic and DRX, timing information (i.e., a time domain granularity and timeline information) is necessary for a device to determine the time domain occasions of first D2R transmission for DO-A, and to determine the DRX configurations.

Observation 6: The payload to be carried in the broadcast information is expected to be large, it is more reasonable to split the broadcast information into two parts, i.e., a MIB-like part and a SIB1-like part.

Observation 7: For the MIB-like broadcast information, it can carry the required information for initial frequency acquisition, cell/reader identification/differentiation, and timing information. The other necessary information can then be carried in the SIB1-like information.

Observation 8: For Device 2b/C, the R2D reception is non-coherent, the modulated tones at the baseband only contains amplitude information and no phase information.

Observation 9: It is not feasible to use modulated multiple tones for CFO calibration.

Observation 10: It is feasible to use an unmodulated sinusoid single tone to calibrate CFO at RF or IF domain.

Observation 11: With the increase of the duration of the CFO calibration signal, the CFO after calibration reduces.

Observation 12: With a duration of 1 OFDM symbol, the CFO after calibration can be reduced from the initial 100 ppm to less than 10 ppm.

Observation 13: To support DO-A traffic, periodic CFO calibration signal is required to avoid a device from keep monitoring the CFO calibration signal before transmitting the very first D2R transmission.

Observation 14: With a periodicity of hundreds of ms of the CFO calibration signal, the CFO drifting is very trivial and therefore no need to introduce aperiodic CFO calibration signal.

Observation 15: The drift rate of CFO is 0.1~1 ppm/s, the frequency drifting is only 900 Hz with accumulation of 1 s, which is not drastic.

Observation 16: The R2D is OOK waveform and based on non-coherent reception, which is robust to CFO. The CFO calibration signal is not related to R2D reception and the location does not require to be in relation to R2D channels carrying L1 R2D control or data payload.

Observation 17: The CFO drift rate is 0.1 or 1 ppm/s, after CFO calibration, the drifting is very slow. The CFO calibration does not require to be in relation to, e.g., close to, D2R transmissions.

Observation 18: With a periodic synchronization signal in a periodicity of hundreds of ms, e.g., 160 ms, for Device C, the timing drift is 1.6 us. For M ≥ 6, the total timing error may be larger than 1/4 chip, and it may deteriorate the PRDCH reception performance, if no additional R2D preamble preceding the PRDCH for timing synchronization. For M < 6, the R2D preamble preceding each PRDCH transmission may not be needed.

Observation 19: A sequence-based SIP part can allow a device to perform correlation operation to achieve more accurate time synchronization.

Observation 20: The transmission reliability of L1 R2D control may not be the same as the data payload.

Observation 21: Using the same chip duration of data payload of PRDCH and the corresponding L1 R2D control information precludes the link adaptation of PRDCH.

Observation 22: Option 1 is inefficient for Rel-20 A-IoT system. If fixed chip duration is assumed for all L1 R2D control information, the link adaptation is precluded and will harm the system efficiency.

Observation 23: The feasibility of Option 3 is based on the prerequisite that the device will perform edge detection to determine the chip duration based on CAP. However, in outdoor scenarios, the required CNR/SNR at the device side is lower than 10 dB, which is not feasible for performing edge detection.

Observation 24: In Rel-20 A-IoT, cases that a physical channel carriers L1 R2D control information only exit, e.g., L1 R2D control information to schedule paging, or to schedule D2R re-transmission.

Observation 25: Defining a new physical channel or a new PRDCH format to carry L1 R2D control information only is more efficient for scheduling and resource utilization.

Proposal 1: For R20 R2D, OOK modulation based on DFT-s-OFDM waveform is supported.

Proposal 2: For R2D modulation, M = 1 could be considered to improve the coverage performance.

Proposal 3: For R20 R2D, SSB modulation could be considered to reduce bandwidth occupancy and support larger M value for one RB.

Proposal 4: For R20 R2D, line coding may not be reused for high peak data rates.

Proposal 5: For R20 R2D FEC, RM code is deprioritized due to the restricted operational range.

Proposal 6: For R20 R2D FEC, LTE TBCC can be considered.

Proposal 7: For R20 R2D FEC, code rate 1/3 is supported when interleaving is not applied.

Proposal 8: To achieve the outdoor coverage design target for R20 R2D, block-level repetition could be considered to enhance link reliability.

Proposal 9: For the R2D synchronization signal at least for the functionality of frequency synchronization (including frequency acquisition) and for the functionality of timing synchronization/tracking, support Alt. 1, i.e.,

- Alt. 1: periodic synchronization signal.

Proposal 10: The periodicity of the synchronization signal can be hundreds of ms, e.g., 160 ms, as a starting point.

Proposal 11: The transmission of broadcast information, including the necessary information for the very first D2R transmission of DO-A should be periodic.

Proposal 12: In Rel-20 A-IoT, at least the following information should be studied to be included in the broadcast information:

- Cell/reader ID;

- Timing information (e.g., time domain granularity and timeline information);

- Configurations of cell/reader-common L1 R2D control information;

- Necessary information of first D2R transmission for DO-A;

- DRX configuration for paging monitoring.

Proposal 13: In Rel-20 A-IoT, RAN1 should study the following two options to carry the MIB-like broadcast information:

- Option 1: A new physical broadcast channel is defined.

- Option 2: Use PRDCH. The scheduling information including time/frequency resources with respect to periodic synchronization signal and MCS is pre-defined for the PRDCH format.

Proposal 14: For the SIB1-like broadcast information, the time resources can be indicated by MIB-like part or scheduled by the corresponding L1 R2D control information.

Proposal 15: For the SIB1-like broadcast information, the following two options of time resources can be studied:

- Option 1: The physical channel carrying MIB-like part is transmitted separately with the synchronization signal. The periodicity and time resources are pre-defined.

- Option 2: The physical channel carrying MIB-like part and the synchronization signal are transmitted as a block, and the synchronization signal is preceding the MIB-like part.

Proposal 16: For the synchronization signal, a binary sequence (e.g., m-sequence) with good auto-correlation property should be studied.

Proposal 17: In Rel-20 A-IoT, RAN1 studies whether and how to use synchronization signal to carry cell/reader ID:

- Option 1: Cell/reader ID is not carried by the synchronization signal. One sequence is required for the synchronization signal.

- Option 2: Cell/reader ID or part of cell/reader ID is carried by the synchronization signal. RAN1 can study a two-part synchronization signal to reduce the device blind detection complexity, where part-1 synchronization signal uses one sequence, and part-2 synchronization signal uses multiple sequences to carry cell/reader ID.

Proposal 18: For the CFO calibration signal, support Option a, i.e.,

- Option a: the signal is an unmodulated sinusoid single tone.

Proposal 19: CFO calibration signal is transmitted periodically.

Proposal 20: The periodicity of the CFO calibration signal can be the same as the periodic synchronization signal, i.e., use 160 ms as a starting point.

Proposal 21: For the time location(s) of the CFO calibration signal, the following two options can be studied:

- Option 4: in relation to the synchronization signal (if supported).

  - The CFO calibration signal can be transmitted immediately after the synchronization signal.

- Option 5: in relation to broadcast information (if supported).

  - The synchronization signal, CFO calibration signal, and MIB-like broadcast information can be transmitted as a block. The CFO calibration signal can be transmitted immediately after the synchronization signal, or immediately after the MIB-like broadcast information.

Proposal 22: If a SIP part preceding a PRDCH is used for timing synchronization, support Option 1, i.e.,

- Option 1: binary sequence-based SIP (e.g M-sequence or Golay sequence)

Proposal 23: For the functionality of chip duration determination of the data payload in the PRDCH, support Option 2, i.e., indicated by the corresponding L1 R2D control information.

Proposal 24: For the functionality of chip duration determination of the L1 R2D control information, the following options should be studied:

- Option 5: Use broadcast information (if any) to indicate the chip duration

- Option 6: Use device-specific configuration signaling to indicate the chip duration.

- Option 2: Blind detection from a predefined set of the chip duration of L1 R2D control information.

  - Option 2 can be combined with Option 6.

Proposal 25: For the L1 R2D control information preceding and scheduling the data payload of PRDCH, RAN1 should study a R2D control channel carrying control information to schedule PRDCH.

- Designs regarding CORESET and search space for PDCCH monitoring, PDCCH blind detection and RNTI scrambling, etc., should be largely simplified.

Proposal 26: For the L1 R2D control information preceding and scheduling the data payload of PRDCH, support Option 2 for the time location of L1 R2D control information, i.e.,

- Option 2: L1 R2D control information and data payload of PRDCH are not contiguous in time.

Proposal 27: For R2D scheduling, the necessary information to be included in L1 control includes TBS, FEC code-rate, number of repetitions, M value, and the time-offset between the L1 control physical channel and the scheduled PRDCH.

Proposal 28: For R2D scheduling of paging message, the necessary information to be included in L1 control additionally includes short message indication like information.

Proposal 29: For D2R scheduling, the necessary information to be included in L1 control includes frequency resource, bit duration Tb, TBS, time resource, FEC code rate and repetition information, while some of them can be omitted if predetermined.

Proposal 30: Two approaches can be studied to support high data rate or high capacity,

- Approach 1: Multiple carriers each with single RB (similar to NB-IoT non-anchor carrier)

- Approach 2: A single carrier with configurable number of RBs

Proposal 31: FDM is not support for R2D within a single carrier.

- Reference

- RP-251884, New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices, 3GPP TSG RAN Meeting #108.

- Chair nots RAN1#122b eom2, 3GPP TSG RAN WG1 #122bis.