3GPP TSG RAN WG1 #123                                                   	 R1-2508724

Dallas, USA, Nov 17th - 21st, 2025

Source:	OPPO

Title:	Discussion on NR-NTN GNSS resilience

Agenda Item:	10.6.1

Document for:	Discussion and Decision

- Introduction

The agreements made in previous RAN1 meetings for NR-NTN GNSS resilience are provided in Appendix. In this contribution, we discuss the potential issues on NR-NTN GNSS-resilient operation.

- Discussion on PRACH tolerance

For GNSS resilience operation in both transparent and regenerative payloads, how to pre-compensate time offset and frequency offset on service link is the key.

  - Evaluation results for differential one-way delay and doppler

In RAN1 meeting #122, it was agreed to study scenario 1 and scenario 2 for GNSS-resilient operation, and the corresponding evaluation assumptions were agreed. In RAN1 meeting #122bis, the definitions of differential one-way delay and differential one-way doppler reached consensus, and two alternatives to calculate the differential one-way delay and the differential one-way doppler were provided. In our evaluation, Alt 1 is considered.

It should be noted that Alt 1 calculation methodology only applies to satellite along orbit case. For comprehensive analysis, we also considered the differential one-way doppler in satellite across orbit case. For the non-nadir beam across the orbit plane, the maximum one-way differential delay and doppler happen at the same beam edge points, so the calculation methodology in Alt 1 can be used. However, for the non-nadir beam across the orbit plane, the maximum one-way differential delay and doppler happen at different beam edge points, so the opening angle β in the slant plane is used to calculate the maximum one-way doppler, as depicted in Figure 1.

Figure 1. Illustration of satellite across orbit

The following formulas are used to calculate the differential one-way doppler in this case:

dcenter=RE+hue2sin2α+hs-hue2+2hs-hueRE+hue-RE+huesinα

β=Rcdcenter

fdref=vssinβ+vuecosαcfc

∆fd=2fdfar

The calculated values for differential one-way delay and differential one-way doppler on service link are provided in Table 1 and Table 2 for different satellite scenarios and for 2 GHz and 30 GHz respectively.

Table 1. Differential one-way delay and doppler on service link for 2 GHz band

| Scenario | Propagation delay (in ms) and doppler shift (in kHz)* |  |  | Differential one-way delay (in us) | Differential one-way doppler (in kHz) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Near-UE | Center-UE | Far-UE |  | Along orbit | Cross orbit |
| LEO600set1@90° | 2.0019ms -2.1kHz | 2ms 0kHz | 2.0019ms 2.1kHz | 1.9us | 4.20kHz (2.10ppm) |  |
| LEO600set1@30° | 3.44ms 39.13kHz | 3.51ms 39.53kHz | 3.58ms 39.90kHz | 143.0us | 0.76kHz (0.38ppm) | 2.39kHz (1.19ppm) |
| LEO1200set1@90° | 4.0033ms 1.78kHz | 4ms 0kHz | 4.0033ms 1.78kHz | 3.3us | 3.56kHz(1.78ppm) |  |
| LEO1200set1@30° | 6.40ms 33.82kHz | 6.53ms 34.22kHz | 6.66ms 34.60kHz | 259us | 0.78kHz (0.39ppm) | 2.17kHz (1.09ppm) |
| GEOset1@90° | 119.292ms -0.07kHz | 119.287ms 0kHz | 119.292ms 0.07kHz | 4.8us | 0.14kHz (0.07ppm) |  |
| GEOset1@12.5° | 133.58ms 2.98kHz | 133.98ms 3.00kHz | 134.39ms 3.01kHz | 809.28us | 0.03kHz (0.015ppm) | 0.13kHz (0.06ppm) |
| LEO600set2@90° | 2.0061ms -3.77kHz | 2ms 0kHz | 2.0061ms 3.77kHz | 6.15us | 7.54kHz (3.77ppm) |  |
| LEO600set2@30° | 3.33ms 38.45kHz | 3.45ms 39.21kHz | 3.58ms 39.90kHz | 255us | 1.45kHz (0.72ppm) | 4.37kHz (2.18ppm) |
| LEO1200set2@90° | 4.015ms -3.74kHz | 4ms 0kHz | 4.015ms 3.74kHz | 14.9us | 7.49kHz (3.74ppm) |  |
| LEO1200set2@30° | 6.13ms 32.83kHz | 6.39ms 33.77kHz | 6.66ms 34.60kHz | 535us | 1.77kHz (0.89ppm) | 4.70kHz (2.35ppm) |
| GEOset2@90° | 119.302ms -0.13kHz | 119.287ms 0kHz | 119.302ms 0.13kHz | 15.59us | 0.26kHz (0.13ppm) |  |
| GEOset2@12.5° | 132.94ms 2.95kHz | 133.66ms 2.98kHz | 134.39ms 3.01kHz | 1490us | 0.06kHz (0.03ppm) | 0.23kHz (0.11ppm) |
| Note*: Only the doppler shift of the beam footprint along the orbit plane is provided. |  |  |  |  |  |  |

Table 2. Differential one-way delay and doppler on service link for 30 GHz band

| Scenario | Propagation delay (in ms) and doppler shift (in kHz)* |  |  | Differential one-way delay (in us) | Differential one-way doppler (in kHz) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Near-UE | Center-UE | Far-UE |  | Along orbit | Cross orbit |
| LEO600set1@90° | 2.0003ms -12.60kHz | 2ms 0kHz | 2.0003ms 12.60kHz | 0.3us | 25.20kHz (0.84ppm) |  |
| LEO600set1@30° | 3.53ms 594.03kHz | 3.55ms 596.26kHz | 3.58ms 598.44kHz | 57.5us | 4.41kHz (0.15ppm) | 14.18kHz (0.47ppm) |
| LEO1200set1@90° | 4.0007ms -11.87kHz | 4ms 0kHz | 4.0007ms 11.87kHz | 0.7us | 23.73kHz (0.79ppm) |  |
| LEO1200set1@30° | 6.55ms 513.93kHz | 6.61ms 516.48kHz | 6.66ms 518.95kHz | 114.9us | 5.02kHz (0.17ppm) | 14.37kHz (0.48ppm) |
| GEOset1@90° | 119.2876ms -0.47kHz | 119.2867ms 0kHz | 119.2876ms 0.47kHz | 0.93us | 0.94kHz (0.03ppm) |  |
| GEOset1@12.5° | 134.03ms 44.98kHz | 134.21ms 45.07kHz | 134.39ms 45.16kHz | 357.2us | 0.19kHz (0.006ppm) | 0.84kHz (0.028ppm) |
| LEO600set2@90° | 2.0019ms -31.47kHz | 2ms 0kHz | 2.0019ms 31.47kHz | 1.9us | 62.95kHz (2.10ppm) |  |
| LEO600set2@30° | 3.44ms 586.99kHz | 3.51ms 592.89kHz | 3.58ms 598.44kHz | 143us | 11.45kHz (0.38ppm) | 35.87kHz (1.20ppm) |
| LEO1200set2@90° | 4.0033ms -2.67kHz | 4ms 0kHz | 4.0033ms 2.67kHz | 3.3us | 53.36kHz (1.78ppm) |  |
| LEO1200set2@30° | 6.41ms 507.25kHz | 6.53ms 513.28kHz | 6.66ms 518.95kHz | 256.9us | 11.70kHz (0.39ppm) | 32.68kHz (1.09ppm) |
| GEOset2@90° | 119.293ms -1.20kHz | 119.287ms 0kHz | 119.283ms 1.20kHz | 6us | 2.39kHz (0.08ppm) |  |
| GEOset2@12.5° | 133.48ms 44.66kHz | 133.93ms 44.92kHz | 134.39ms 45.16kHz | 905.7us | 0.5kHz (0.017ppm) | 2.12kHz (0.07ppm) |
| Note*: Only the doppler shift of the beam footprint along the orbit plane is provided. |  |  |  |  |  |  |

  - PRACH tolerance for differential RTT

To evaluate PRACH tolerance, the following PRACH formats are considered:

- For FR1 (2 GHz):

  - PRACH format 0, 1, 2, 3

  - PRACH format A1, A2, A3, B1, B2, B3, B4, C0, C2 with 15 kHz SCS

- For FR2 (30 GHz):

  - PRACH format A1, A2, A3, B1, B2, B3, B4, C0, C2 with 60 kHz SCS

According to the agreements in RAN1 meeting #122bis, PRACH tolerance is exceeded if

min⁡(NCP,Sequence duration×maxNCSL)-2Te≤Differential RTT

In the above formula, differential RTT is 2 time of one-way delay, and Te is an additional timing error to be reported by company. According to TS 38.104, the time error tolerance for PRACH estimation for AWGN and TDLC300-100 is given in Table 8.4.2.1-1. In our view, this time error represents the TA estimation error with PRACH, and can be considered as 2Te. Besides, high doppler shift/spread induced by high-speed satellite movement makes NTN evaluation more like TDLC300-100 case, so the time error tolerance for TDLC300-100 is reused here. Note that the PRACH formats with the time error tolerance in Table 8.4.2.1-1 are not aligned with our evaluated PRACH formats. For PRACH format with 5 kHz SCS, the time error tolerance of PRACH format with 1.25 kHz SCS is reused. For PRACH formats with 60 kHz SCS for FR2, a time error tolerance of (3/4) * minCP is considered, where minCP is the minimum CP duration among all the PRACH formats with 60 kHz SCS, i.e., B1 (1.76 us). Table 3 gives the 2Te values for the evaluated PRACH formats.

Table 3. Additional time error for different PRACH formats

| PRACH format | PRACH SCS (kHz) | Additional time error (2Te) |
| --- | --- | --- |
| 0, 1, 2 | 1.25 | 2.55 us |
| 3 | 5 | 2.55 us |
| A1, A2, A3, B1, B2, B3, B4, C0, C2 | 15 | 2.03 us |
|  | 60 | 1.32 us |

The maximum Ncs values and the corresponding durations for evaluated PRACH formats are provided in Table 4. The CP duration, maximum Ncs duration, and maximum tolerable differential RTT are provided in Table 5 and Table 6 for PRACH format 0/1/2/3 and PRACH format A1/A2/A3/B1/B2/B3/B4/C0/C2 respectively. Note that the maximum tolerable differential RTT is the upper bound of RTT that PRACH tolerance is fulfilled.

Table 4. Maximum Ncs and the corresponding duration for different PRACH formats

| Sequence length L | PRACH SCS (kHz) | Sequence duration (us) | Max Ncs value |  |  | Sequence duration* MaxNcs / L (us) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Unrestricted set | Restricted set type A | Restricted set type B | Unrestricted set | Restricted set type A | Restricted set type B |
| 839 | 1.25 | 800 | 0 | 237 | 137 | 800 | 225.98 | 130.63 |
| 839 | 5 | 200 | 0 | 237 | 137 | 200 | 56.50 | 32.66 |
| 139 | 15 | 66.67 | 0 | - | - | 66.67 | - | - |
| 139 | 60 | 16.67 | 0 | - | - | 16.67 | - | - |

Table 5. CP duration, maximum Ncs duration, and maximum tolerable differential RTT for PRACH format 0/1/2/3

| PRACH format | Ncp (us) | Sequence duration* MaxNcs / L (us) |  |  | Max tolerable differential RTT (us) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Unrestricted set | Restricted set type A | Restricted set type B | Unrestricted set | Restricted set type A | Restricted set type B |
| 0 | 103.12 | 800 | 225.98 | 130.63 | 100.57 | 100.57 | 100.57 |
| 1 | 684.37 | 800 | 225.98 | 130.63 | 681.82 | 223.43 | 128.08 |
| 2 | 152.60 | 800 | 225.98 | 130.63 | 150.05 | 150.05 | 128.08 |
| 3 | 103.12 | 200 | 56.50 | 32.66 | 100.57 | 53.95 | 30.11 |

Table 6. CP duration, maximum Ncs duration, and maximum tolerable differential RTT for PRACH format A1/A2/A3/B1/B2/B3/B4/C0/C2

| PRACH format | Ncp (us) |  | Sequence duration* MaxNcs / L (us) |  | Max tolerable differential RTT (us) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 15 kHz | 60 kHz | 15 kHz | 60 kHz | 15 kHz | 60 kHz |
| A1 | 9.37 | 2.34 | 66.67 | 16.67 | 7.34 | 1.02 |
| A2 | 18.75 | 4.69 | 66.67 | 16.67 | 16.72 | 3.37 |
| A3 | 28.13 | 7.03 | 66.67 | 16.67 | 26.10 | 5.71 |
| B1 | 7.03 | 1.76 | 66.67 | 16.67 | 5.00 | 0.44 |
| B2 | 11.71 | 2.93 | 66.67 | 16.67 | 9.68 | 1.61 |
| B3 | 16.41 | 4.10 | 66.67 | 16.67 | 14.38 | 2.78 |
| B4 | 30.47 | 7.62 | 66.67 | 16.67 | 28.44 | 6.30 |
| C0 | 40.36 | 10.09 | 66.67 | 16.67 | 38.33 | 8.77 |
| C2 | 66.67 | 16.67 | 66.67 | 16.67 | 64.64 | 15.35 |

  - PRACH tolerance for differential doppler offset

According to the agreements in RAN1 meeting #122bis, PRACH tolerance is exceeded if

scaling factor ×Differential Doppler+2×fe×fc≥(1+γ)∆fRA

In the above formula, for restricted set A, γ=2. For restricted set B, γ=4. For non-restricted set, γ=0. The scaling factor*Differential Doppler is equal to the UL differential doppler/frequency offset, and the scaling factor is to be reported by company. In our view, the UL differential doppler offset should be the sum of DL differential one-way doppler and UL differential one-way doppler. The ratio of DL to UL differential one-way doppler equals to the ratio of DL to UL carrier frequency. In previous agreement, S-band for FR1 and Ka-band for FR2 are assumed for evaluation, thus:

- For FR1 (2 GHz for UL, 2 GHz for DL):

  - UL differential doppler: 2 time of UL differential one-way doppler, and the scaling factor is 2.

  - Additional frequency offset: 2×fe×fc=2×0.1 ppm×2 GHz=0.4 kHz

- For FR2 (30 GHz for UL, 20 GHz for DL):

  - UL differential doppler: UL differential one-way doppler + (20/30) *UL differential one-way doppler = 1.67*UL differential one-way doppler, and the scaling factor is 1.67.

  - Additional frequency offset: 2×fe×fc=2×0.1 ppm×30 GHz=6 kHz

The maximum tolerable UL differential one-way doppler for different PRACH SCSs are provided in Table 7 for evaluated PRACH formats. Note that the maximum tolerable UL differential one-way doppler offset is the upper bound of doppler offset that PRACH tolerance is fulfilled.

Table 7. Maximum tolerable UL differential one-way doppler for different PRACH SCSs

| PRACH SCS (kHz) | fc (GHz) | (1+γ) ∆fRA (kHz) |  |  | Max tolerable UL differential one-way doppler (kHz) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Unrestricted set | Restricted set type A | Restricted set type B | Unrestricted set | Restricted set type A | Restricted set type B |
| 1.25 | 2 | 1.25 | 3.75 | 6.25 | 0.425 | 1.675 | 2.925 |
| 5 | 2 | 5 | 15 | 25 | 2.3 | 7.3 | 12.3 |
| 15 | 2 | 15 | - | - | 7.3 | - | - |
| 60 | 30 | 60 | - | - | 32.4 | - | - |

  - PRACH tolerance for evaluated satellite scenarios

Based on above discussion, PRACH tolerances for format 0/1/2/3 and format A1/A2/A3/B1/B2/B3/B4/C0/C2 are provided in Table 8 and Table 9 respectively.

Table 8. PRACH tolerance for format 0/1/2/3

| PRACH format | Max tolerable differential RTT and UL differential one-way doppler |  |  |
| --- | --- | --- | --- |
|  | Unrestricted set | Restricted set type A | Restricted set type B |
| 0 | 100.57 us | 100.57 us | 100.57 us |
|  | 0.425 kHz | 1.675 kHz | 2.925 kHz |
| 1 | 681.82 us | 223.43 us | 128.08 us |
|  | 0.425 kHz | 1.675 kHz | 2.925 kHz |
| 2 | 150.05 us | 150.05 us | 128.08 us |
|  | 0.425 kHz | 1.675 kHz | 2.925 kHz |
| 3 | 100.57 us | 53.95 us | 30.11 us |
|  | 2.3 kHz | 7.3 kHz | 12.3 kHz |

Table 9. PRACH tolerance for format A1/A2/A3/B1/B2/B3/B4/C0/C2

| PRACH format | Max tolerable differential RTT and UL differential one-way doppler |  |
| --- | --- | --- |
|  | 15 kHz | 60 kHz |
| A1 | 7.34 us | 1.02 us |
|  | 7.3 kHz | 32.4 kHz |
| A2 | 16.72 us | 3.37 us |
|  | 7.3 kHz | 32.4 kHz |
| A3 | 26.10 us | 5.71 us |
|  | 7.3 kHz | 32.4 kHz |
| B1 | 5.00 us | 0.44 us |
|  | 7.3 kHz | 32.4 kHz |
| B2 | 9.68 us | 1.61 us |
|  | 7.3 kHz | 32.4 kHz |
| B3 | 14.38 us | 2.78 us |
|  | 7.3 kHz | 32.4 kHz |
| B4 | 28.44 us | 6.30 us |
|  | 7.3 kHz | 32.4 kHz |
| C0 | 38.33 us | 8.77 us |
|  | 7.3 kHz | 32.4 kHz |
| C2 | 64.64 us | 15.35 us |
|  | 7.3 kHz | 32.4 kHz |

For 2 GHz evaluation, PRACH format 0, 1, 2, 3, and PRACH format A1, A2, A3, B1, B2, B3, B4, C0, C2 with 15 kHz SCS are candidate PRACH formats. By comparing Table 1, Table 8 and Table 9, Table 10 gives the results whether current PRACH formats can support the evaluated scenarios.

Table 10. Candidate PRACH formats for evaluated scenarios for 2 GHz band

| Scenario | Differential RTT (us) | Differential one-way doppler (kHz) | PRACH format | Differential one-way doppler (kHz) | PRACH format |
| --- | --- | --- | --- | --- | --- |
|  |  | Along orbit | Along orbit | Cross orbit | Cross orbit |
| LEO600set1@90° | 3.8 | 4.2 | 3/A1/A2/A3/ B1/B2/B3/B4/C0/C2 | 4.2 | 3/A1/A2/A3/ B1/B2/B3/B4/C0/C2 |
| LEO600set1@30° | 286 | 0.76 | N/A | 2.39 | N/A |
| LEO1200set1@90° | 6.6 | 3.56 | 3/A1/A2/A3/ B2/B3/B4/C0/C2 | 3.56 | 3/A1/A2/A3/ B2/B3/B4/C0/C2 |
| LEO1200set1@30° | 518 | 0.78 | N/A | 2.17 | N/A |
| GEOset1@90° | 9.6 | 0.14 | 0/1/2/3/A2/A3/ B2/B3/B4/C0/C2 | 0.14 | 0/1/2/3/A2/A3/ B2/B3/B4/C0/C2 |
| GEOset1@12.5° | 1618.56 | 0.03 | N/A | 0.13 | N/A |
| LEO600set2@90° | 12.3 | 7.54 | 3 | 7.54 | 3 |
| LEO600set2@30° | 510 | 1.45 | N/A | 4.37 | N/A |
| LEO1200set2@90° | 29.8 | 7.49 | 3 | 7.49 | 3 |
| LEO1200set2@30° | 1070 | 1.77 | N/A | 4.7 | N/A |
| GEOset2@90° | 31.18 | 0.26 | 0/1/2/3/C0/C2 | 0.26 | 0/1/2/3/C0/C2 |
| GEOset2@12.5° | 2980 | 0.06 | N/A | 0.23 | N/A |

For 30 GHz evaluation, PRACH format A1, A2, A3, B1, B2, B3, B4, C0, C2 with 60 kHz SCS are candidate PRACH formats. By comparing Table 2 and Table 9, Table 11 gives the results whether current PRACH formats can support the evaluated scenarios.

Table 11. Candidate PRACH formats for evaluated scenarios for 30 GHz band

| Scenario | Differential RTT (us) | Differential one-way doppler (kHz) | PRACH format | Differential one-way doppler (kHz) | PRACH format |
| --- | --- | --- | --- | --- | --- |
|  |  | Along orbit | Along orbit | Cross orbit | Cross orbit |
| LEO600set1@90° | 0.6 | 25.20 | A1/A2/A3/ B2/B3/B4/C0/C2 | 25.20 | A1/A2/A3/ B2/B3/B4/C0/C2 |
| LEO600set1@30° | 115 | 4.41 | N/A | 14.18 | N/A |
| LEO1200set1@90° | 1.4 | 23.73 | A2/A3/ B2/B3/B4/C0/C2 | 23.73 | A2/A3/ B2/B3/B4/C0/C2 |
| LEO1200set1@30° | 229.8 | 5.02 | N/A | 14.37 | N/A |
| GEOset1@90° | 1.86 | 0.94 | A2/A3/ B3/B4/C0/C2 | 0.94 | A2/A3/ B3/B4/C0/C2 |
| GEOset1@12.5° | 714.4 | 0.19 | N/A | 0.84 | N/A |
| LEO600set2@90° | 3.8 | 62.95 | N/A | 62.95 | N/A |
| LEO600set2@30° | 286 | 11.45 | N/A | 35.87 | N/A |
| LEO1200set2@90° | 6.6 | 53.36 | N/A | 53.36 | N/A |
| LEO1200set2@30° | 513.8 | 11.70 | N/A | 32.68 | N/A |
| GEOset2@90° | 12 | 2.39 | C2 | 2.39 | C2 |
| GEOset2@12.5° | 1811.4 | 0.5 | N/A | 2.12 | N/A |

From Table 10 and 11, it is observed that

- For LEO600set2@90° and LEO1200set2@90° in 30 GHz band, the requirements for differential one-way doppler offset cannot be fulfilled by PRACH format with 60 kHz SCS. However, if PRACH format C2 with 120 kHz SCS is considered, the maximum tolerable differential RTT would be 7.68 us and the maximum tolerable UL differential one-way doppler would be 68.4 kHz, then the requirements of these two scenarios can be fulfilled.

- For other evaluated satellite scenarios that cannot be supported by current PRACH formats, the reason is that the duration of the tolerable maximum differential RTT is smaller than the observed maximum different RTT in satellite deployment.

To solve the issue that the requirement for differential RTT cannot be fulfilled, gNB implementation can be considered, e.g., assuming the duration gap between the tolerable maximum differential RTT and the observed maximum different RTT in satellite deployment is T duration, the gNB can extend the PRACH reception window for the selected PRACH format to cover the T duration, as illustrated in Figure 2.

Figure 2. Illustration of PRACH reception window

To reduce the extended PRACH processing window, a PRACH format which can address the differential one-way doppler offset and has the maximum tolerable differential RTT should be selected for a certain satellite scenario.

Based on above discussion, Table 10 and 11 can be updated by Table 12 and 13.

Table 12. Candidate PRACH formats for evaluated scenarios for 2 GHz band

| Scenario | Differential RTT (us) | Differential one-way doppler (kHz) | PRACH format | Differential one-way doppler (kHz) | PRACH format |
| --- | --- | --- | --- | --- | --- |
|  |  | Along orbit | Along orbit | Cross orbit | Cross orbit |
| LEO600set1@90° | 3.8 | 4.2 | 3/A1/A2/A3/ B1/B2/B3/B4/C0/C2 | 4.2 | 3/A1/A2/A3/ B1/B2/B3/B4/C0/C2 |
| LEO600set1@30° | 286 | 0.76 | 1(restricted type A) with gNB implementation | 2.39 | 1/2(restricted type B) with gNB implementation |
| LEO1200set1@90° | 6.6 | 3.56 | 3/A1/A2/A3/ B2/B3/B4/C0/C2 | 3.56 | 3/A1/A2/A3/ B2/B3/B4/C0/C2 |
| LEO1200set1@30° | 518 | 0.78 | 1(restricted type A) with gNB implementation | 2.17 | 1/2(restricted type B) with gNB implementation |
| GEOset1@90° | 9.6 | 0.14 | 0/1/2/3/A2/A3/ B2/B3/B4/C0/C2 | 0.14 | 0/1/2/3/A2/A3/ B2/B3/B4/C0/C2 |
| GEOset1@12.5° | 1618.56 | 0.03 | 1(unrestricted) with gNB implementation | 0.13 | 1(unrestricted) with gNB implementation |
| LEO600set2@90° | 12.3 | 7.54 | 3 | 7.54 | 3 |
| LEO600set2@30° | 510 | 1.45 | 1(restricted type A) with gNB implementation | 4.37 | C2(15kHz) with gNB implementation |
| LEO1200set2@90° | 29.8 | 7.49 | 3 | 7.49 | 3 |
| LEO1200set2@30° | 1070 | 1.77 | 1/2(restricted type B) with gNB implementation | 4.7 | C2(15kHz) with gNB implementation |
| GEOset2@90° | 31.18 | 0.26 | 0/1/2/3/C0/C2 | 0.26 | 0/1/2/3/C0/C2 |
| GEOset2@12.5° | 2980 | 0.06 | 1(unrestricted) with gNB implementation | 0.23 | 1(unrestricted) with gNB implementation |

Table 13. Candidate PRACH formats for evaluated scenarios for 30 GHz band

| Scenario | Differential RTT (us) | Differential one-way doppler (kHz) | PRACH format | Differential one-way doppler (kHz) | PRACH format |
| --- | --- | --- | --- | --- | --- |
|  |  | Along orbit | Along orbit | Cross orbit | Cross orbit |
| LEO600set1@90° | 0.6 | 25.20 | A1/A2/A3/ B2/B3/B4/C0/C2 | 25.20 | A1/A2/A3/ B2/B3/B4/C0/C2 |
| LEO600set1@30° | 115 | 4.41 | C2(60kHz) with gNB implementation | 14.18 | C2(60kHz) with gNB implementation |
| LEO1200set1@90° | 1.4 | 23.73 | A2/A3/ B2/B3/B4/C0/C2 | 23.73 | A2/A3/ B2/B3/B4/C0/C2 |
| LEO1200set1@30° | 229.8 | 5.02 | C2(60kHz) with gNB implementation | 14.37 | C2(60kHz) with gNB implementation |
| GEOset1@90° | 1.86 | 0.94 | A2/A3/ B3/B4/C0/C2 | 0.94 | A2/A3/ B3/B4/C0/C2 |
| GEOset1@12.5° | 714.4 | 0.19 | C2(60kHz) with gNB implementation | 0.84 | C2(60kHz) with gNB implementation |
| LEO600set2@90° | 3.8 | 62.95 | C2(120kHz) | 62.95 | C2(120kHz) |
| LEO600set2@30° | 286 | 11.45 | C2(60kHz) with gNB implementation | 35.87 | C2(120kHz) with gNB implementation |
| LEO1200set2@90° | 6.6 | 53.36 | C2(120kHz) | 53.36 | C2(120kHz) |
| LEO1200set2@30° | 513.8 | 11.70 | C2(60kHz) with gNB implementation | 32.68 | C2(120kHz) with gNB implementation |
| GEOset2@90° | 12 | 2.39 | C2 | 2.39 | C2 |
| GEOset2@12.5° | 1811.4 | 0.5 | C2(60kHz) with gNB implementation | 2.12 | C2(60kHz) with gNB implementation |

Observation 1: For GNSS resilience operation with pre-compensation of common time offset and frequency offset on service link, proper PRACH formats with necessary gNB implementation can be used to address the observed maximum differential RTT and maximum differential one-way doppler for all the evaluated satellite scenarios.

Proposal 1: For GNSS resilience operation, RAN1 does not pursue PRACH enhancement in Rel-20.

- UL synchronization in connected mode

In this section, we discuss the impacts of GNSS resilience operation on UL time synchronization in RRC connected mode.

  - UL time synchronization in connected mode

For UE in RRC connected mode, legacy close-loop TA update procedure can be a starting point to study the impact of GNSS unavailable on UL time synchronization. In general, if autonomous UE-specific TA adjustment in RRC connected mode cannot be performed, the UL time synchronization can still be maintained by more frequent TA updates from network indication through TA command. Since the GNSS capability is temporarily unavailable, the signaling overhead of TA command seems acceptable. However, it should be studied how to make the gNB be aware of GNSS unavailable at UE side, so it can increase the frequency of TA command transmission for the UE.

Proposal 2: For UL time synchronization in connected mode, reuse legacy close-loop TA update procedure, and study the mechanism how to make gNB be aware of temporarily unavailable of GNSS capability at UE side.

  - UL frequency synchronization in connected mode

For UE in RRC connected mode, considering the GNSS capability is temporarily unavailable, there is no need to introduce new mechanism for compensation of residual UE-specific frequency offset. The impact of the residual UE-specific frequency offset on demodulation performance of PUSCH temporarily can be enhanced by PUSCH retransmission or repetition.

Proposal 3: For UL frequency synchronization in connected mode, enhancements on compensation of residual UE-specific frequency offset are not needed.

- NR positioning

In our previous contribution [2], the result that idle UE performs DL TDOA positioning using legacy SSB signals from multiple satellites is provided. In this section, we further evaluate the positioning performance of DL TDOA using PRS. Detailed evaluation assumptions are provided in Table 14.

Table 14. Evaluation assumptions

| Scenarios | Assumptions |
| --- | --- |
| Scenarios | Rural, LOS |
| Satellite Orbit | 600km |
| Satellite parameters | Reuse Set-1satellite parameters as in table 6.1.1.1-1 of TR38.821 |
| Channel model/ Delay spread | Based on section 6.7.2 of TR 38.811 |
| FR/Carrier frequency | FR1: 2GHz, S-band (n256). |
| BW | 5MHz |
| Subcarrier spacing, kHz | 15 for FR1 |
| Number of satellites in view | 3 satellite orbits, 4 occasions are used per positioning estimation |
| Orbit inclination | 53 degree inclination |
| UE type | Handheld terminal |
| UE related parameters | Handheld UE characteristics as in Table 6.1.1.1-3 of TR38.821 |
| Positioning signals | PRS |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | Comb-2 |
| RS type of sequence/number of ports | Gold sequence |
| Number of symbols used per occasion | 6 |
| Interference modelling (ideal muting, or other) | Ideal muting |
| Reference Signal Transmission Bandwidth | 5MHz |
| Reference point for timing measurement | Satellite |
| UE speed | 3km/h |
| Performance metrics | Horizontal accuracy (UE 2D position accuracy) |
| Network synchronization error | Not considered |
| UE/gNB TX/RX calibration error | Not considered |

In our simulation assumptions, four satellite positions are considered for DL TDOA calculation with two satellite positions on the orbit 1 and two other satellite positions on the orbit 2 and orbit 3 respectively, as shown in Figure 3. All the three orbits are modelized at 600 km and can be seen by UE.

The positioning performance of DL TDOA with the aid of 4 satellite positions is provided in Figure 4 and summarized in Table 15. It is observed that the positioning accuracy error is within 173 m at 95% CDF point, and the corresponding one-way delay/doppler uncertainty on service link for 2 GHz band is provided in Table 16. This ensures the timing offset is within CP length and the frequency error is within 0.1 ppm at 2 GHz carrier frequency. With this accuracy, NR positioning for idle UE with PRS can be an alternative for acquiring UE location, and can be used in GNSS-resilient scenario.

Table 15. Positioning accuracy

|  | Number of occasions per positioning estimation | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- |
| Location uncertainty X (m) | 4 | 48.17 m | 72.03 m | 113.1 m | 173 m |

Table 16. One-way delay/doppler uncertainty on service link for 2 GHz band

| Scenario | Location uncertainty X (m) | One-way delay uncertainty | One-way doppler uncertainty (kHz) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Along orbit | Cross orbit |
| LEO600set1@90° | 173 | 0.1 ns | 0.029 |  |
| LEO600set1@30° |  | 1 us | 0.005 | 0.016 |

Figure 3. Illustration of orbit 0, orbit 1 and orbit 2

Figure 4. Positioning error CDF

Observation 2: Using existing NR positioning, the positioning accuracy error is within 173 m at 95% CDF point. This ensures the timing offset is within CP length and the frequency error is within 0.1 ppm at 2GHz carrier frequency.

Proposal 4: For GNSS resilience operation, RAN1 considers UE using existing NR positioning to obtain UE location in Rel-20.

- Backward compatible issues

For Rel-20 NR-NTN, network may need to provide services to GNSS capable and GNSS unavailable UEs in a same beam footprint. For GNSS capable UEs, legacy NR-NTN UE behaviours are assumed to maintain UL time and frequency synchronization. For GNSS unavailable UEs, based on above discussion, proper PRACH format should be selected according to the corresponding NTN deployment scenario to reduce the impacts of residual TA error and residual frequency offset error after the common TA and common frequency offset compensation. Besides, the performance of PRACH transmitted with the common TA and common frequency offset compensation may be worse that of PRACH transmitted with UE-specific TA and frequency offset compensation derived from GNSS location, due to lack of UL synchronization accuracy.

Therefore, for cells supporting GNSS resilience operation, it is beneficial to separate PRACH configurations for GNSS capable and GNSS unavailable UEs, including PRACH formats and PRACH occasions, to achieve backward compatible between legacy UE and Rel-20 UE in NR-NTN.

Proposal 5: Separate PRACH configurations including PRACH formats and PRACH occasions for GNSS capable and GNSS unavailable UEs.

- Conclusion

In this contribution, we discuss the potential issues on NR-NTN GNSS-resilient operation.

The following observations are provided.

Observation 1: For GNSS resilience operation with pre-compensation of common time offset and frequency offset on service link, proper PRACH formats with necessary gNB implementation can be used to address the observed maximum differential RTT and maximum differential one-way doppler for all the evaluated satellite scenarios.

Observation 2: Using existing NR positioning, the positioning accuracy error is within 173 m at 95% CDF point. This ensures the timing offset is within CP length and the frequency error is within 0.1 ppm at 2GHz carrier frequency.

The following proposals are provided.

Proposal 1: For GNSS resilience operation, RAN1 does not pursue PRACH enhancement in Rel-20.

Proposal 2: For UL time synchronization in connected mode, reuse legacy close-loop TA update procedure, and study the mechanism how to make gNB be aware of temporarily unavailable of GNSS capability at UE side.

Proposal 3: For UL frequency synchronization in connected mode, enhancements on compensation of residual UE-specific frequency offset are not needed.

Proposal 4: For GNSS resilience operation, RAN1 considers UE using existing NR positioning to obtain UE location in Rel-20.

Proposal 5: Separate PRACH configurations including PRACH formats and PRACH occasions for GNSS capable and GNSS unavailable UEs.

- Reference

- RP-251863, Study on GNSS resilient NR-NTN operation, Thales (Moderator).

- R1-2507174, Discussion on NR-NTN GNSS resilience, OPPO.

# Appendix

## Agreements in RAN1 #122bis

Agreement:

For the evaluation of GNSS resilient NR-NTN operation, consider the following assumptions:

- For scenario 1 and 2

  - The following time/frequency differences should be evaluated:

    - The differential one-way delay (in µs) as calculated between the largest UE-Sat delay and smallest UE sat delay within the uncertainty area.

      - For UL performance evaluation 2 times differential one-way delay should be considered.

    - The differential one-way Doppler/frequency offset (in ppm) is calculated between the largest UE-sat Doppler/frequency offset and smallest UE-sat Doppler/frequency offset within the uncertainty area.

      - For UL performance evaluation, scaled one-way differential Doppler/frequency offset to determine UL differential Doppler/frequency offset should be considered. Scaling factor should be reported.

Agreement:

For PRACH performance evaluation for existing PRACH preamble formats based on analytical characterization, adopt the following criteria:

- PRACH tolerance is exceeded if

  - min (Ncp, Sequence duration ×maxNCSL) - 2 Te≤ Differential RTT, where;

    - Ncp is cyclic prefix duration

    - max Ncs is given by Tables 6.3.3.1-5, 6.3.3.1-6 and 6.3.3.1-7 of TS 38.211

    - L is the ZC sequence length

    - Te is an additional timing error.

      - Companies to report how Te is derived.

    - Differential RTT is 2 time one-way delay.

    - Note: the delay spread is neglected

    - GP can be considered and reported for the case of consecutive RO.

  - Or (scaling factor*(Differential Doppler) +2* fe×fc) ≥ (1+γ) ∆fRA

    - For restricted set A, γ=2. For restricted set B, γ=4. For non-restricted set, γ=0

      - Note: In practice, the detection performance degrades at frequency offsets exceeding 1 SCS and 2 SCS with restricted set A and B, respectively.

      - Othe values of γ can be used and reported with justification

    - fe=0.1 ppm

    - fc is the uplink carrier frequency

    - Note: The scaling factor*Differential Doppler is equal to the UL differential Doppler/frequency offset.

    - The same scaling factor as per the previous agreement is to be reported

Note: the above is applicable to single PRACH transmission.

Agreement:

RAN1 to assess the impacts on connected mode during NR NTN GNSS-resilient operation, including at least:

- Impact on signalling overhead.

Observation:

For the study on GNSS resilient NR-NTN operation, the following solutions for initial access to increase PRACH tolerance and reducing time/frequency uncertainty are listed based on inputs/discussions in RAN1#122bis:

- Solution 1: To improve robustness:

  - Solutions based on modified random access procedures, using existing PRACH formats:

    - Solution 1A: Multiple PRACH transmissions (e.g. with different roots or different formats or with different time/frequency pre-compensation using multiple reference locations within the uncertainty area) using existing PRACH formats

    - Solution 1B: New PRACH restricted sets.

    - Solution 1C: Single PRACH transmission with multiple TAC/FAC commands in RAR

    - Solution 1D: Signalling enhancements for Msg2 (e.g. enhanced TA command, frequency adjustment command, reference point adjustment command)

    - Solution 1E: Selection of a set of existing PRACH root

    - Solution 1F: Using TA margin and RO masking.

    - Other solutions are not precluded

- Solution 2: Solutions based on reducing time-frequency uncertainty

  - Solution 2A: Single/multi-satellite DL-TDOA based on current specifications.

  - Solution 2B: multiple PRACH attempts based on different time/frequency pre-compensation hypotheses (e.g. based on multiple reference points within the uncertainty area)

  - Solution 2C: Solutions based on broadcasting DL timestamp(s)

  - Solution 2D: UE side time/frequency pre-compensation based on reference location.

  - Solution 2E: service link time/frequency pre-compensation based on last acquired GNSS position

  - Solution 2F: UE side time/frequency pre-compensation based on reference signal.

  - Other solutions are not precluded

- Solution 3: Implementation-based techniques e.g. using a long enough PRACH processing window and multiple timing hypotheses for PRACH preamble reception with large max differential delay.

- Other Solutions are not precluded

Agreement:

In RAN1#123, based on the following alternatives representing two calculation methodologies, select a calculation methodology with potential adjustments, including correction of equations:

Alt 1:

For the calculation of the differential one-way delay and the differential one-way Doppler/frequency offset the following could be considered:

Geometry model and equations could be considered. For the variables in the equations, refer to Figure below.

For non-nadir beam

dfar=RE+hue2sin2α+hS-hue2+2hs-hueRE+hue-RE+huesinα

φ1=φ-2RcRE

drefnear=RE+hue2+RE+hs2-2RE+hueRE+hscosφ1

φ=sin-1(dfarRE+hscosα)

β=π2-(α+φ)

β1=sin-1RE+huedrefnearsinφ1

γ=cos-1RE+hsdrefnearsinφ1

fdestfar=(vssinβ+vuecosα)cfc

fdrefnear=vssinβ1+vuecosγcfc

∆t=d-drefdfar-dnearc

∆fd=fdref-fdestfdfar-fdnear

For nadir beam

dnear=hs-hue

φ=RcRE

dfar=RE+hue2+RE+hs2-2RE+hueRE+hscosφ

β=sin-1RE+huedfarsinφ

α=π2-(β+φ)

fdfar=(vssinβ+vuecosα)cfc

∆t=dfar-dnearc

∆fd=2fdfar

Note 1: UE is at same height hue as the reference point.

Note 2: For the purpose of evaluation, fixed size of uncertainty area is used within the whole satellite coverage footprint is a baseline. Other size of uncertainty area is not precluded.

FFS: Whether the minimum elevation is at farthest w.r.t satellite point of the uncertainty area or at the center of the uncertainty area

FFS: How to take into account the UE altitude uncertainty.

Alt 2:

For the calculation of one-way differential delay and one-way differential Doppler, the following geometry model is considered. For the variables in the equations, refer to Figure-X.

Considering satellite location at s=0,0,RE+h

One-way delay between a point u and satellite:
done-wayu=|u-s|2c

One-way Doppler between u and satellite (assuming satellite is moving along x=1,0,0 with speed v and UE moving along  k=cosα,sinα,0 with speed vue:

dopplerone-wayu=vcfc<u-s,x>done-wayu+vuecfc<s-u,k>done-wayu

Let R be the location uncertainty area.

Differential one-way delay = maxu∈R done-wayu-minu∈Rdone-wayu

Differential one-way doppler = maxu∈Rdopplerone-wayu-minua∈Rdopplerone-way(u)

For evaluation purpose, R can be a circle on surface of earth (height can be considered optionally), placed such that the furthest point in the uncertainty area can view the satellite at an elevation angle e and is located at an azimuth angle ∅ from the orbital plane (x-y plane).

Note: Assume satellite and UE moves parallel to the x-y plane. <⋅,⋅> is the vector inner product, ||.|​2 is the vector magnitude.

## Agreements in RAN1 #122

Agreement

For the study on GNSS resilient operation, scenarios should be considered where:

- Scenario 1: The UE cannot rely on its GNSS for timing and frequency compensation on the service link.

- Scenario 2: There is a previously acquired GNSS based position

  - UE has not received GNSS information for time duration T from the last acquired GNSS position and hence, the GNSS accuracy is degraded

    - FFS: value of T

    - FFS: GNSS based UE location validity duration

    - FFS: GNSS accuracy

- Note: Two scenarios above belong to description as in SID.

Agreement

For the study on GNSS resilient operation, the following scenarios are considered for evaluation purposes:

| Satellite orbit | GSO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- |
| Satellite altitude | 35786 km | 1200 km | 600 km |
| Satellite scenario parameters | At least Table 6.1.1.1-1 in TR 38.821, Parameters in Table 6.1.1.1-2 in TR 38.821could be considered. |  |  |
| Beam size (note 3) | At least values captured in Table 6.1.1.1-1/2 are considered, |  |  |
| Minimum elevation angle | 30° (LEO), 12.5° (GSO) |  |  |
| Frequency ranges/bands | FR1 NTN (2GHz/Ku: 14 GHz) and FR2 NTN (30 GHz) |  |  |
| UE type | Handheld (L/S band), VSAT (Ku/Ka) |  |  |
| UE speed (note 1) | L/S band: 3 km/h, 120km/h, 1500 km/h, Ku/Ka: 3 km/h , 120 km/h, 1500 km/h, (note 2) |  |  |
| Beam/cell type (note 4) | (Quasi)-Earth-fixed beams/cells and Earth-moving |  |  |
| Note 1: UE altitude is considered Note 2: 10 km altitude in case of aircraft scenario. 1500 km/h is for aircraft scenario. Note 3: Same values can be reused for other elevation angle than nadir. Beam size of the edge beam can be reported by companies. Note 4: Footprint of the beam could be: case 1: within the orbital plane, case 2: at 90° with respect to orbital plane. Companies to report the elevation angle of the beam. Note 5: Other parameters can be reported by companies. |  |  |  |

Note: The cyan highlighted parameters are prioritized for evaluation purposes.

Agreement:

For PRACH performance evaluation for existing PRACH formats, adopt the following methods:

- Baseline: Analytical characterization of performance based on e.g. properties of ZC sequences and values of differential Delay / Doppler.

- Optional: Link level evaluations

  - The following table is proposed for LLS parameters

| Configurations | S/L-band | Ka/Ku-band |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | For Ka: UL 30GHz, for Ku: UL 14 GHz |
| Channel Model | Baseline NTN TDL-C rural model in TR38.811 |  |
| Antenna Configuration at the TRP (satellite) | 1 Rx | 1 Rx |
| Antenna Configuration at the UE | A single omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter |
| PRACH format | To be reported by companies. | To be reported by companies |
| PRACH configuration | To be reported by companies. |  |
| Metric | PRACH detection rate, FAR (Based on the preamble pool size is not less than 64), CDF of estimation error for frequency/timing |  |
| Receiver | Companies to report the receiver for PRACH detection. |  |

Agreement:

For evaluation regarding GNSS-resilient operation, based on GNSS information or the lack of it, it is assumed that the UE has a horizontal location uncertainty within an area

- Case a: the location uncertainty area is the area served by the cell or beam.

- Case b: the location uncertainty area is a circle of radius X km

  - X to be reported

UE altitude error should be taken into account.

- UE altitude error to be reported.

For both Case a and Case b above, RAN1 to evaluate at least:

- Differential delay values

- Differential frequency offset/Doppler values