3GPP TSG RAN WG1 #123	R1-2509155

Dallas, US, Nov 17th – 21st, 2025

Source: 	MediaTek Inc.

Title:	Other procedures for Ambient IoT

Agenda item:	10.3.2.3

Document for:	Discussion/Decision

- Introduction

In RAN #109, the revised SID on enhancements for solutions for Ambient IoT in NR outdoor for active devices was described in the [1]:

Objectives - Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]: - Deployment scenario 4 with topology 1 with no external carrier wave. - Traffic types DO-DTT, DT, DO-A. - Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. - Assumption of the same air interface for Device 2b and Device C. - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 - Positioning for outdoor scenarios is under objective 2. This objective begins after RAN#108 (June 2025) RAN#111 (March 2026) will make a decision on whether to include outdoor scenarios in Rel-20 normative work. This objective includes: RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios - Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C. - While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m - Maximum transmit power between -3 dBm and +5 dBm for device C - RAN1 to recommend feasible values within this range - Maximum transmit power of -20 dBm and -10 dBm for device 2b Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources. - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations. - Define link budget calculation for coverage. - Identify D2R RSRP-like measurement, and the involved A-IoT signal(s)/channel(s), which are feasible for Device 2b/Device C for more accurate outdoor Device localization than based on Reader-ID [RAN1]. - RAN1 is not expected to conclude on the positioning accuracy as part of the identification of feasible D2R RSRP-like measurement - Note: Device localization based on Reader ID as specified in Rel-19 can be supported for active device(s) in Rel-20 with no additional work. - The design of D2R signal(s)/channel(s) for active devices is not impacted by the identification of the D2R RSRP-like measurement - This objective does not include the study of dedicated Device procedures for Device localization - The objective assumes no dedicated positioning architecture will be specified in Rel-20 - The objective assumes no specification of inter-Reader coordination for Device localization in Rel-20 - This objective will start at RAN1#124

In this contribution, we discuss the necessary and feasible changes to the Rel-19 A-IoT specifications, including necessary and feasible change to procedures such as for initial frequency acquisition and broadcast information acquisition, random access, DO-A, and power control to support A-IoT active devices in outdoor scenarios.

- Initial frequency acquisition

For active devices, such as device 2b and device c, there is no prior knowledge of the system’s frequency or timing information, nor of the D2R transmission schedule. As a result, it is essential to implement an initial frequency acquisition procedure, particularly from the perspective of transmitting the first DO-A message. For ambient IoT devices with limited capabilities, it is preferable to avoid a “cell search” type procedure, where the device continuously scans for timing and frequency points to identify periodic or multiple periodic signals. This is especially relevant when using correlation-based or coherent-based detection methods, as such procedures can significantly increase device complexity and power consumption.

Observation 1: Implementing a cell search-like procedure would lead to increased complexity for ambient IoT devices.

Proposal 1: Avoid a cell search-like procedure that requires searching for both timing and frequency points

- Broadcast information acquisition and random access

Once the device has successfully acquired the necessary frequency and timing information, the next step is for the device to obtain details about the D2R random access or Msg1 resource allocation, such as the specific time and/or frequency resources available for transmission. To facilitate this process, a random access-like procedure is required for the DO-A operation. In order for the device to receive this essential resource information, it is necessary to transmit broadcast information that clearly specifies the relevant time and frequency resources for D2R random access or Msg1 transmission.

Ideally, this broadcast information should be made available to the device immediately following the initial frequency acquisition. By ensuring that the broadcast information is received promptly after synchronization, the device can efficiently proceed with the random access procedure without unnecessary delay or additional searching.

Proposal 2: The broadcast information is expected to be received right after the initial frequency acquisition

In the last R122bis meeting, the following agreement is made:

Agreement For the content of the first D2R transmission for DO-A for Device 2b and for Device C, study the following options: - Option 1: Msg1-like content - Option 2: DO-A data payload - FFS: with or without device ID - Option 3: DO-A scheduling request

In Rel-19 A-IoT, there is no defined signal for uplink initial access. The random access procedure is specified using slot-ALOHA as a baseline, such as the transmission of Msg1. However, the initial access capacity for devices in Rel-20 A-IoT systems, particularly those targeting outdoor scenarios, is relatively limited. To address this limitation and enhance the capacity for random access, it is necessary to explore code domain multiplexing techniques. By employing code domain multiplexing, multiple devices can simultaneously access the system, thereby increasing the overall random access capacity.

Figure-1 DO-A procedure

To support this approach, a sequence-based NR preamble-like D2R DO-A request signal is required. This signal enables devices to initiate random access in a manner similar to NR preambles, facilitating efficient multiplexing and identification. An example of this procedure is illustrated in Figure 1. To reduce the complexity associated with multiplexing, it is proposed that only the sequence-based portion of the signal be transmitted during the first D2R transmission for DO-A. The actual DO-A data payload should be transmitted only after the random access procedure has been successfully completed. This separation ensures that the initial transmission remains simple and efficient, while the data payload is sent in a more controlled manner.

Observation 2: Incorporating code domain multiplexing for the DO-A data payload would increase system complexity and negatively impact decoding performance.

Proposal 3: The first D2R transmission for DO-A, i.e., a DO-A request signal, should include the random ID or device ID.

- Other aspects

It is essential to further discuss and clarify the specific characteristics of DO-A traffic. In particular, the transmission interval and message payload size for DO-A traffic are critical factors that are closely tied to the design of the DO-A procedure, and by extension, to the detailed physical layer design. For instance, the physical layer requirements and design considerations may differ significantly between periodic DO-A transmissions and event-triggered DO-A transmissions. Furthermore, if both periodic and event-triggered DO-A traffic types are to be supported simultaneously, additional complexity may arise in the physical layer design to accommodate the differing requirements of each traffic type.

Therefore, we recommend a comprehensive analysis of the DO-A traffic characteristics, including typical transmission intervals and payload sizes and etc., to inform the development of efficient and robust physical layer solutions. This will ensure that the system can effectively support the diverse requirements of DO-A traffic in future releases.

Observation 3: The DO-A procedure design is highly related with the DO-A traffic characteristics.

Proposal 4: Support further study of DO-A traffic characteristics, e.g., DO-A interval, etc.

- Conclusion

In this contribution, the following observation and proposals are made:

Observation 1: Implementing a cell search-like procedure would lead to increased complexity for ambient IoT devices.

Observation 2: Incorporating code domain multiplexing for the DO-A data payload would increase system complexity and negatively impact decoding performance.

Observation 3: The DO-A procedure design is highly related with the DO-A traffic characteristics.

Proposal 1: Avoid a cell search-like procedure that requires searching for both timing and frequency points.

Proposal 2: The broadcast information is expected to be received right after the initial frequency acquisition

Proposal 3: The first D2R transmission for DO-A, i.e., a DO-A request signal, should include the random ID or device ID.

Proposal 4: Support further study of DO-A traffic characteristics, e.g., DO-A interval, etc.

- Reference

- RP-252964, Revised SID: Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices, LG Electronics, RAN #109, September 15-18, 2025.

- R1-2508175, FL summary #9 for 10.3.2 “Study of air interface for Device 2b/C”, Moderator (LG Electronics), RAN1#122bis, Oct 13th – 17th, 2025