3GPP TSG RAN WG1 Meeting #82bis	R1-156100

Malmö, Sweden, 5th - 9th October 2015

Agenda item:	7.2.4.3.2

Source: 	Samsung

Title: 	Performance Evaluation of Class B CSI Reporting

Document for:	Discussion and Decision

# Introduction

In RAN1#82, it was agreed that two CSI reporting classes are supported for Rel.13 EB/FD-MIMO: class A (aimed for non-precoded CSI-RS) and class B (aimed for beamformed CSI-RS). For class B, four alternatives were identified [1]:

- Alt.1: Indicator for beam selection and L-port CQI/PMI/RI for the selected beam. Total configured number of ports across all CSI-RS resources in the CSI process is larger than L.

- Alt.2: L-port precoder from a codebook reflecting both beam selection(s) and co-phasing across two polarizations jointly. Total configured number of ports in the CSI process is L.

- Alt.3: Codebook reflecting beam selection and L-port CSI for the selected beam. Total configured number of ports across all CSI-RS resources in the CSI process is larger than L.

- Alt.4: L-port CQI/PMI/RI. Total configured number of ports in the CSI process is L. (if CSI measurement restriction is supported, it is always configured)

- Note: A “beam selection” (whenever applicable) constitutes either a selection of a subset of antenna ports within a single CSI-RS resource or a selection of a CSI-RS resource from a set of resources

- Note: The reported CSI may be an extension of Rel.12 L-port CSI

- Details such as possible values of L are FFS

- Further down-selection/merging of the four alternatives is FFS

In this contribution, system level simulation results are presented for the class B scheme described in [2] and [3]. This scheme enables UE-specific beamformed (BF) CSI-RS operation with K=1 NZP CSI-RS resource. In particular, the eNodeB performs wideband

# Description of scheme

The simulated class B scheme can be summarized as follows (see also [2] and [3]):

- The eNodeB configures the UE with two CSI processes. This is termed the “hybrid” CSI-RS scheme in [4].

  - Process 1: 16-port class A

  - Process 2: NP–port class B with K=1

- The eNodeB performs UE-specific wideband beamforming on a 16-port NZP CSI-RS resource. This beamforming operation reduces the number of CSI-RS ports that the UE shall measure from 16 to NP. The eNodeB derives the necessary beamforming vector/matrix by utilizing the UE feedback associated with CSI process 1. The 16-port codebook used in this simulation is taken from [5]. This UE feedback is reported at a low rate.

- Upon measuring the NP CSI-RS ports (which are associated with NP/2 “micro-beams” [2]), the UE calculates and reports CSI (for process 2) where its corresponding PMI represents micro-beam selection(s) (1 or 2 out of NP /2, depending on the transmission rank) and co-phasing. This can be implemented by reusing the W2 components of the existing Rel.12 8-port codebook or the Rel.13 16-port class A codebook [3]. This CSI reporting is done at a higher rate.

# Simulation results

Throughput metrics including average user throughput, 5% user throughput, 50% user throughput, and average served load are compared for different schemes. ‘Class A only’ is used as a reference. As described in Section 2, the class B K=1 scheme simulated in this contribution utilizes non-precoded CSI-RS (and hence class A) and thus labelled ‘Class B K=1 + Class A’. The following transmission periodicity (and hence UE measurement periodicity) is assumed:

- ‘Class A only’: 5ms

- ‘Class B K=1 + class A’: 5ms for class B (UE-specific BF CSI-RS) and 50ms for class A (non-precoded CSI-RS)

Table 1 depicts the results for RU=20% with NP=4 and 2-Rx UEs. In this case, the maximum transmission rank is 2. Some notable performance gain in user TP is observed over the reference scheme (‘class A only’). This net gain comes from a combination of reduced CSI-RS overhead and improved CSI-RS coverage due to UE-specific beamforming. This notable gain comes together with reduced UE measurement complexity. That is, instead of measuring performing channel measurement and CSI computation associated with 16 CSI-RS ports every 5ms, the UE performs channel measurement and CSI computation associated with 4 CSI-RS ports every 5ms (except for once every 50ms).

Table 1: Throughput performance NP=4 and 2-Rx UEs, max rank = 2: RU=20%

| Channel model | Scheme | Throughput (Mbps) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Average user TP | 5% user TP | 50% user TP | Average served load |
| UMa-200m | Class A only (reference) Mbps | 37.542 | 14.679 | 36.197 | 5.990 |
|  | Class B K=1 + Class A Mbps | 39.461 | 15.043 | 36.658 | 5.990 |
|  | Class B K=1 + Class A relative to reference (%) | 105 % | 103 % | 102% | 100% |
| UMi-2GHz | Class A only (reference) Mbps | 38.269 | 15.669 | 36.480 | 6.217 |
|  | Class B K=1 + Class A Mbps | 40.609 | 15.925 | 38.090 | 6.212 |
|  | Class B K=1 + Class A relative to reference (%) | 106 % | 102 % | 104 % | 100 % |

Tables 2A and 2B depict the results for RU=20% with NP=4 and 4-Rx UEs. In this case, the maximum transmission rank is either 2 (Table 2A) or 4 (Table 2B). A similar trend for the relative gain over ‘class A only’ is observed. In addition, setting the maximum rank to 4 offers some substantial gain over maximum rank of 2. Comparing Table 2A with 2B, it is evident that the average user throughput increases by 29-34% while the other throughput metrics remain comparable. This implies that introducing rank 3 and 4 in this low-load scenario results in more UEs serviced with rank-3 and 4 (hence higher throughput) transmission.

Table 2A: Throughput performance NP=4 and 4-Rx UEs, max rank = 2: RU=20%

| Channel model | Scheme | Throughput (Mbps) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Average user TP | 5% user TP | 50% user TP | Average served load |
| UMa-200m | Class A only (reference) Mbps | 49.353 | 23.796 | 56.672 | 5.888 |
|  | Class B K=1 + Class A Mbps | 53.425 | 25.099 | 61.152 | 5.904 |
|  | Class B K=1 + Class A relative to reference (%) | 108 % | 106 % | 108 % | 100 % |
| UMi-2GHz | Class A only (reference) Mbps | 49.169 | 24.73 | 56.672 | 6.248 |
|  | Class B K=1 + Class A Mbps | 53.061 | 25.503 | 61.152 | 6.262 |
|  | Class B K=1 + Class A relative to reference (%) | 108 % | 103 % | 108 % | 100 % |

Table 2B: Throughput performance NP=4 and 4-Rx UEs, max rank = 4: RU=20%

| Channel model | Scheme | Throughput (Mbps) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Average user TP | 5% user TP | 50% user TP | Average served load |
| UMa-200m | Class A only (reference) Mbps | 67.943 | 23.015 | 60.909 | 5.800 |
|  | Class B K=1 + Class A Mbps | 71.627 | 23.225 | 63.439 | 5.754 |
|  | Class B K=1 + Class A relative to reference (%) | 105 % | 101 % | 104 % | 99 % |
| UMi-2GHz | Class A only (reference) Mbps | 65.748 | 21.330 | 57.341 | 6.170 |
|  | Class B K=1 + Class A Mbps | 68.593 | 22.756 | 59.457 | 6.129 |
|  | Class B K=1 + Class A relative to reference (%) | 104 % | 107 % | 104 % | 99 % |

# Conclusion

In this contribution, system level simulation results are presented for class B K=1 scheme described in [2] and [3]. It is observed that some notable gain can be obtained from class B K=1 scheme (used in conjunction with a low duty cycle 16-port non-precoded CSI-RS) over its class A reference – while at the same time significantly reducing UE channel measurement complexity. Furthermore, it is observed that increasing the maximum rank from 2 to 4 (with 4-Rx UEs) for class B K=1 scheme results in substantial gain in average user throughput.

# Appendix: Simulation assumptions

Table 5: Simulation Parameters

| Parameters | Values |
| --- | --- |
| Simulation Type | Non-full-buffer FTP, low load (20% RU) |
| Channel model | UMi-2GHz, UMa-200m |
| Number of BS (H,V) antenna elements | (8,8), x-polarized, subarray partition |
| (N1,N2, P) for class A | (4,2,2), 16 ports |
| Number of ports NP for class B (K=1) | 4 |
| BS (H,V) antenna spacing | (0.5, 0.8)λ |
| BS and MS antenna polarizations | BS: (+45°,-45°); MS: (0°, 90°) |
| Number of UE antennas | 2, 4 |
| MU pre-coding | SLNR |
| Scheduling | SU, Proportional fair |
| Channel estimation | Modeled |
| Transmission rank | Rank adaptation:1, 2, 3, 4 |
| Receiver | MMSE-IRC |

# References

RAN1#82, Chairman Notes

R1-155499, Samsung, “Schemes for Class B CSI Reporting for Rel.13” 3GPP TSG RAN1#82bis

R1-155501, Samsung, “Codebook design with W2-only feedback for class B CSI reporting” 3GPP TSG RAN1#82bis

3GPP, TR36.897, Technical report of the study on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE

R1-155490, Samsung, “Rank 1-2 codebook for Class A CSI reporting” 3GPP TSG RAN1#82bis

3GPP, TS36.213