3GPP TSG RAN WG1 #83	R1- 156489

November 15th  – 22nd, 2015

Anaheim, USA

Agenda item:	6.2.3.1

Source: 	Cisco Systems

Title: 	Views on Energy Detection Threshold Adaptation

Document for:	Discussion and Decision

# Introduction

Following RAN1 #82, it was agreed that RAN1 should identify rules for adaptively lowering the energy detection threshold at LAA eNB so as to ensure co-existence with other RATs including Wi-Fi and good performance of LAA.  In [1], we provided our initial views on how the threshold adaptation should occur. This contribution discusses our updated views on this important topic.

Energy Detection Threshold Adaptation Agreements in RAN1 #82

RAN1 shall identify adaptation rules for LAA to adaptively lower the maximum energy detection threshold to ensure co-existence with other RATs including Wi-Fi and good performance of LAA

  - Technologies that ensure co-existence with other RATs including Wi-Fi, using alternative means not requiring lowering of the maximum energy detection threshold, are not precluded.

- At least the following shall be considered in defining the adaptation rules of the maximum energy detection threshold:

  - Antenna gain and number of transmit antennas

  - Coexistence with LAA in absence of other RATs including Wi-Fi

  - The maximum rated EIRP of the LAA transmission point within unlicensed band

  - The maximum EIRP within the transmission burst following the LBT procedure

  - The transmission bandwidth

  - Measured ambient noise floor

  - Deployment scenario: Indoor, outdoor

  - Estimated Load on the operating channel

  - Feasibility of the co-existence test

  - Single global solution

- In regions and bands/sub-bands without regulations, define an upper bound for the maximum energy detection threshold taking into consideration

  - May be based on bandwidth and presence of other RATs

# Relevant LAA Functionalities

In [2][3], we have presented proposals regarding the ED threshold adaptation procedure taking into consideration two classes of LAA eNB functionalities. In the following sections, we shall elaborate on the adaptation procedure for each of the identified functionalities.

Preamble Detection Functionality: The eNB is capable of detecting and attempts to detect, for LBT, the presence (including start and duration) of 802.11a/n/ac transmission on the same channel that the LAA eNB intends to transmit.

Energy Detection Functionality: This corresponds to the following 2 classes of LAA eNB functionalities:

- An LAA eNB is capable of detecting presence of a 802.11 a/n/ac BSS but not capable of detecting the start and duration of a 802.11 a/n/ac transmission, on the same channel that the LAA eNB intends to transmit.

- An LAA eNB is not capable of detecting whether there is a co-channel 802.11 a/n/ac transmission on the same channel that the LAA eNB intends to transmit on, and the absence of the co-channel 802.11 a/n/ac transmission cannot be guaranteed.

# Preamble Detection

For preamble detection capable LAA eNB, the preamble detection threshold for detecting the presence (including start and duration) of legacy 802.11a/n/ac transmission should equal -82 dBm in the absence of interference. The energy detection threshold can be simply set to equal that of Wi-Fi namely -62 dBm. Upon detecting the presence of an ongoing 802.11a/n/ac transmission, the LAA eNB should regard the channel as busy for the purpose of LBT for the duration of the 802.11-a/n/ac transmissions and thereafter perform an extended CCA backoff.

Proposal: For preamble detection based LAA functionality

- Thresholds for preamble detection and energy detection are set to -82 dBm and -62 dBm respectively.

- Upon detecting the presence of an ongoing 802.11 a/n/ac transmission, the LAA eNB should regard the channel as busy for the duration of the transmission.

- Following completion of the 802.11a/n/ac transmission the LAA eNB should perform an extended CCA backoff prior to starting its TXOP.

We clarify that the above proposal is to target Rel-13 LAA devices to ensure co-existence with existing 802.11 technologies. This proposal can be revised beyond Rel-13 taking into account future technologies’ capabilities, as and when their specification work is complete. For example, 802.11ax has introduced a BSS coloring concept. Each BSS transmits a certain color as part of its header;  a device shall use the existing CCA threshold of -82 dBm if it sees identical color as its own in the detected .11ax preamble. For example all devices in the same BSS employ identical color, as a result, the low spatial reuse (high SINR) prevents unsynchronized operation among intra-BSS devices. On the other hand, if a device sees a different color in the detected .11ax preamble, for example, from an OBSS transmission, that device can employ a higher CCA-PD threshold. The high CCA-PD threshold, in effect, encourages higher spatial reuse, since the device abandons the PPDU reception procedure and can start transmitting.

Similar to BSS coloring, Rel-14 LAA operation can employ a higher preamble and energy detection threshold if it is aware that its neighbour devices are .11ax capable. Much like .11ax devices will defer to legacy devices and transmit legacy preambles, Rel-14 LAA can still back off at lower ED threshold to legacy Wi-Fi while employing higher ED/CCA-PD thresholds to .11ax devices.

Observation: To ensure co-existence with future technologies, Rel-14 LAA operation can employ higher ED/CCA-PD thresholds upon detecting .11ax preambles and reuse Rel-13 thresholds (CCA-PD = -82 dBm, CCA-ED = -62 dBm).

# Energy Detection

For LAA eNB capable of only energy detection, per [3], the maximum energy detection threshold T is proposed to be set per the following procedure:

T= min (Tmax, Tmax-Y+PH-PTX+g(Tmax-Y+PH-PTX,Nmeas))

In the above equation,

Tmax=-75 dBm/MHz+10×log10BW designates the upper limit on the maximum ED threshold over a reference bandwidth BW. For example, it equals -62 dBm over 20 MHz.

PH is a reference EIRP (e.g. 23 dBm).

PTX  shall be chosen from either of the following alternatives:

- Maximum rated EIRP of the Device Under Test (DUT) measured in dBm

- Maximum transmit power (set on a long-term basis) for the configured carrier

Nmeas is the measured noise floor for the carrier in dBm.

Y is an offset (in dB). Based on prior studies [4][5][6], it was determined that for any LAA ED threshold value above -77 dBm, performance of both best effort and delay sensitive applications at the non-replaced Wi-Fi networks suffers. This suggests that the choice Y=15 dB is optimal assuming PHand PTX respectively equal 23 dBm.

gTmax-Y+PH-PTX,Nmeas is a function (to be determined) which adjusts the maximum ED threshold based on background noise floor measurements.

In practice, there can be wide variation in background noise floor measurement due to different noise floor measurement algorithms and different receiver noise figures. In addition, the dynamic nature of the background interference in 5 GHz band implies that the measured background noise can be highly non-stationary. As an example, even when there is no on-going transmission, clients may frequently send probe requests on different channels during the association to their AP. As a result, the background noise estimate may be high even in absence of traffic.

In 3GPP, no analysis has been performed on either how an LAA device estimates background noise floor or what are the system-level coexistence impacts of adapting the ED threshold based on the estimated background noise floor. We suggest that 3GPP study the background noise measurement procedure as well as the ensuing system level impacts due to adapting ED threshold based on the noise floor estimate. It is also important that the noise floor adjustment be based on the long-term background noise rather than short term interference fluctuations which can be quite common e.g. presence of an nearby interferer transmitting traffic in a bursty manner. Note anyway that short-term interference can be resolved via the LAA eNB sharing the medium in a TDM-like manner with the neighbour transmitter device.

In view of the lack of evidence-based analysis of the impact of noise floor adjustment, our preference is to have as baseline, g(.) = 0 dB.

Proposal:  For background noise floor adjustment, the following options shall be considered:

- Baseline: gTmax-Y+PH-PTX,Nmeas=0 dB

- Other alternatives: If the function gTmax-Y+PH-PTX,Nmeas is non-zero

  - Background noise measurement procedure should be clearly identified in RAN1/RAN4.

  - System-level coexistence impacts of adapting ED threshold based on background noise floor estimate should be further studied.

# Absence of Wi-Fi is Known to LAA

Proposal: In scenarios where the Wi-Fi is known to be absent on the unlicensed band where LAA is operating, or when an LAA eNB can determine the absence of Wi-Fi, the maximum single channel ED threshold can be set equal to -62 dBm or even higher, subject to region-specific regulation.

# Summary

Proposal 1: For preamble detection based LAA functionality:

- Thresholds for preamble detection and energy detection are set to -82 dBm and -62 dBm respectively.

- Upon detecting the presence of an ongoing 802.11 a/n/ac transmission, the LAA eNB should regard the channel as busy for the duration of the transmission.

- Following completion of the 802.11a/n/ac transmission the LAA eNB should perform an extended CCA backoff prior to starting its TXOP.

Proposal 2: For ED detection based LAA functionality

T= min (Tmax, Tmax-Y+PH-PTX+g(Tmax-Y+PH-PTX,Nmeas))

In the above equation,

Tmax=-75 dBm/MHz+10×log10BW designates the upper limit on the maximum ED threshold, equalling -62 dBm over 20 MHz.

PH is a reference EIRP (e.g. 23 dBm).

PTX  shall be chosen from either of the following alternatives:

- Maximum rated EIRP of the Device Under Test (DUT) measured in dBm

- Maximum transmit power (set on a long-term basis) for the configured carrier

Nmeas is the measured noise floor for the carrier in dBm.

Y is an offset (in dB) suggested to equal 15 dB.

gTmax-Y+PH-PTX,Nmeas is a function (to be determined) which adjusts the maximum ED threshold based on background noise floor measurements.

Proposal 3:  For background noise floor adjustment, the following options shall be considered:

- Baseline: gTmax-Y+PH-PTX,Nmeas=0 dB

- Other alternatives: If the function gTmax-Y+PH-PTX,Nmeas is non-zero

  - Background noise measurement procedure should be clearly identified in RAN1/RAN4.

  - System-level coexistence impacts of adapting ED threshold based on background noise floor estimate should be further studied.

Proposal 4: In scenarios where the Wi-Fi is known to be absent on the unlicensed band where LAA is operating, or when an LAA eNB can determine the absence of Wi-Fi, the maximum single channel ED threshold can be set equal to -62 dBm or even higher, subject to region-specific regulation.

# References

- R1-155150, Remaining Details of DL LBT Procedure for Rel-13 LAA, Cisco Systems.

- R1-156360, Proposed Harmonized WF on LAA DL LBT ED Threshold Adaptation Rule (Functionality of Detecting On-going 802.11 a/n/ac tx), Cisco Systems, Broadcom.

- R1-156361, Proposed Harmonized WF on LAA DL LBT ED Threshold Adaptation Rule (Remaining Functionalities), Cisco Systems, Broadcom.

- R1-152936, Coexistence Evaluation Results using LBT Category 4 for Wi-Fi DL and LAA DL Only Scenario, Broadcom.

- R1-152937, Coexistence Evaluation Results using LBT Category 4 for Wi-Fi DL + UL and LAA DL only Scenario, Broadcom.

- R1-155310, Energy Detection Threshold for LAA, Intel Corporation.