3GPP TSG RAN WG1 Meeting #83		R1-156792

Anaheim, USA, 15th - 22th November 2015

Agenda item:	6.2.4.3.1

Source: 	Samsung

Title: 	Codebook design for class B

Document for:	Discussion and Decision

Introduction

In [1] and [2], CSI reporting for class B and K=1 was agreed as follows:

- For K=1

  - A value N1={1, 2, 4, 8} is configured as one Rel.12 NZP CSI-RS resource

    - eNodeB signals the number of ports N1 via NZP CSI-RS resource configuration

  - CSI reporting with PMI-feedback-only based on W2-only feedback for N1 ports

    - Using all/components of W2 in Rel.13 class A codebook configuration 4

      - DFT vectors are replaced by column vectors of identity matrix

  - In addition, legacy (Rel.12) CSI reporting is also supported with MR functionality

Based on the above agreement, rank-1 codebook design for class B K=1 was also agreed in [3] by using agreed rank-1 codebook for class A in [1]. In this contribution, we discuss on rank-2 to rank-8 codebook design for class B when K=1.

Class B Codebook Design

In [4] and [5], we have proposed rank-2 to rank-8 codebook for class A. Based on the agreement in [2], i.e., codebook design for class B using all/components of W2 in Rel.13 class A codebook configuration 4, and our proposal for class A, we can obtain codebook for class B.

Rank-2 class B codebook

Rank-2 class A codebook was proposed in [4]. 2/4/8 ports Rank-2 class B codebook which uses components of W2 of the class A codebook can derived with below assumptions

- Configuration 4 of the rank-2 codebook for class A

- i11=i12=0

- vm1=Q2emod(m1,Np/2)Np/2, um2=1 where eiN is a length-N selection vector

With above derivation, PMI definition for rank-2 class B codebook is given in Table 1

Table 1. Rank-2 class B codebook and PMI definition

| NP | Single PMI i2 (Rel.13) | Precoder |  |
| --- | --- | --- | --- |
|  |  | Precoder description based on class A codebook (reuse of W2) | Resulting precoder |
| 2 | 0, 1 | i11=i12=0, i2'=0, 1 | 11jk-jk, k=0,1 |
| 4 | 0, 1, …, 7 | i11=i12=0, i2'=0, 1, 2, 3, 8, 9, 10, 11 | eieijkei-jkei, k=0,1; i=0,1 and e0e1jke0-jke1, e1e0jke1-jke0, k=0,1 |
| 8 | 0, 1, …, 15 | i11=i12=0, i2'=0, 1, …, 15 | eieijkei-jkei, k=0,1; i=0,1,2,3 and eiei+1jkei-jkei+1, k=0,1; i=0,1 and eie3jkei-jke3, k=0,1; i=0,1 |

Proposal 1: Adopt Table 1 and corresponding derivations for rank-2 class B codebook design.

Rank-3 to Rank-4 class B codebook

Rank-3 and rank-4 class A codebooks were proposed in [5]. 4/8 ports rank-3 and rank-4 class B codebooks which use components of W2 of the class A codebook can derived with below assumptions

- Configuration 4 of the rank-3 and rank-4 codebook for class A, and δ1=1, p1=1

- i11=i12=0

- vm1=Q2emod(m1,Np/2)Np/2, um2=1 where eiN is a length-N selection vector

With above derivation, PMI definitions for rank-3 and rank-4 class B codebooks are given in Table 2 and Table 3, respectively.

Table 2. Rank-3 class B codebook and PMI definition

| NP | Single PMI i2 (Rel.13) | Precoder |  |
| --- | --- | --- | --- |
|  |  | Precoder description based on class A codebook (reuse of W2) | Resulting precoder |
| 4 | 0, 1, 2, 3 | i11=i12=0, i2'=0, 1, 2, 3 | e0e0e1e0-e0-e1, e1e0e1e1-e0-e1, e0e1e1e0e1-e1, e1e0e0e1e0-e0 |
| 8 | 0, 1, …, 15 | i11=i12=0, i2'=0, 1, …, 15 | eieiei+1ei-ei-ei+1, ei+1eiei+1ei+1-ei-ei+1, eiei+1ei+1eiei+1-ei+1, ei+1eieiei+1ei-ei, i=0,1,2 and e3e3e0e3-e3-e0, e0e3e0e0-e3-e0, e3e0e0e3e0-e0, e0e3e3e0e3-e3 |

Table 3. Rank-4 class B codebook and PMI definition

| NP | Single PMI i2 (Rel.13) | Precoder |  |
| --- | --- | --- | --- |
|  |  | Precoder description based on class A CB (reuse of W2) | Resulting precoder |
| 4 | 0, 1 | i11=i12=0, i2'=0, 1 | e0e1e0e1jke0jke1-jke0-jke1, k=0,1 |
| 8 | 0, 1, …, 7 | i11=i12=0, i2'=0, 1, …, 7 | eiei+1eiei+1jkeijkei+1-jkei-jkei+1, k=0,1; i=0,1,2 and e3e0e3e0jke3jke0-jke0-jke0, k=0,1 |

Proposal 2: Adopt Table 2 and corresponding derivations for rank-3 and rank-4 class B codebook design

Rank-5 to Rank-8 class B codebook

Rank-5 to rank-8 class A codebooks were proposed in [5]. Compared to rank-1 to rank-4 class A codebooks which have W2 components in the codebook, rank-5 to rank-8 codebooks do not have. In this sense, based on the agreement in [2], we propose a single codeword for rank-5 to rank-8 class B codebook. 8 ports rank-5 to rank-8 class B codebooks which has a single component can derived with below assumptions

- Rank-5 to rank-8 codebook for class A, and δ1,1=1, δ1,2=2, δ1,3=3

- i11=i12=0

- vm1=Q2em1Np/2, um2=1 where eiN is a length-N selection vector

With above derivation, PMI definitions for rank-5 to rank-8 class B codebooks are given in Table 4.

Table 4. Rank-5 to rank-8 class B codebook and PMI definition

| NP | Rank ν | Single PMI i2 (Rel.13) | Precoder |  |
| --- | --- | --- | --- | --- |
|  |  |  | Precoder description based on class A CB (reuse of W2) | Resulting precoder |
| 8 | 5 | 0 | i11=i12=0 | e0e0 e0-e0 e1e1 e1-e1 e2e2 |
|  | 6 | 0 | i11=i12=0 | e0e0 e0-e0 e1e1 e1-e1 e2e2 e2-e2 |
|  | 7 | 0 | i11=i12=0 | e0e0 e0-e0 e1e1 e1-e1 e2e2 e2-e2 e3e3 |
|  | 8 | 0 | i11=i12=0 | e0e0 e0-e0 e1e1 e1-e1 e2e2 e2-e2 e3e3 e3-e3 |

Proposal 3: Adopt Table 4 and corresponding derivations for rank-5 to rank-8 class B codebook design

Relation between Class A codebook proposals and Class B codebook

Since class B codebook is based on configuration 4 of class A codebook, class B codebook may be changed according to agreed class A codebook. In [6], there have been various proposals for rank-2 to rank-8 codebook design, in particular, class A codebook design. In our companion contribution [7], we tried to summarize submitted proposals. In the summary, we observed that there is a clear majority in rank-2 codebook configuration 4 and W1-only codebook design for rank-5 to rank-8 codebook which are the same as what we assumed in this contribution. Therefore, we can conclude following observation.

Observation 1: As long as majority of class A rank-2 codebook proposals in [6] is used, class B codebook proposal in this contribution applies.

Discussion and Conclusion

This contribution has discussed W2-only feedback based rank-2 to rank-8 class B codebook design and we made following proposals and observation.

Proposal 1: Adopt Table 1 and corresponding derivations for rank-2 class B codebook design.

Proposal 2: Adopt Table 2 and corresponding derivations for rank-3 and rank-4 class B codebook design

Proposal 3: Adopt Table 4 and corresponding derivations for rank-5 to rank-8 class B codebook design

Observation 1: As long as majority of class A rank-2 codebook proposals in [6] is used, class B codebook proposal in this contribution applies.

References

- 3GPP TSG RAN1#82b, Chairman’s note.

- R1-156165, WF on class A and class B CSI reporting for Rel.13 EB/FD-MIMO, AT&T, Beijing Xinwei Telecom Tech., CATR, CATT, CHTTL, CMCC, Deutsche Telekom, Ericsson, ETRI, Huawei, HiSilicon, ITRI, Kathrein-Werke KG, KDDI, KT Corporation, Nokia Networks, NTT DOCOMO, Samsung, Sony Corporation

- R1-156280, WF on PMI feedback for class B K=1, Samsung, Ericsson, NTT Docomo, AT&T

- R1-156381, Joint Proposal on Rank 2 codebook for Class A and B CSI reporting, Samsung, Ericsson, NTT DOCOMO, CATT

- R1-156390, Joint proposal on rank 3-8codebook, Samsung, Ericsson, NTT DOCOMO, CATT

- Email discussion, [82b-13] Codebook proposals for EB/FD-MIMO - higher ranks

- R1-156789, Finalizing Rel.13 Rank>1 Codebook Design, Samsung