3GPP TSG RAN WG1 Meeting #85	R1-164251

Nanjing, China 23rd - 27th May 2016

Source:	CATT

Title:	Performance evaluation of binary turbo codes with low complexity decoding algorithm

Agenda Item:	7.1.5.1

Document for:	Discussion/Decision

Introduction

In RAN1#84bits, the following has been agreed on the support of channel coding design for 5G new radio [1].

Agreements:

- Candidates for 5G new RAT data transmission are identified as the following

  - LDPC code

  - Polar code

  - Convolutional code (LTE and/or enhanced convolutional coding)

  - Turbo code (LTE and/or enhanced turbo coding)

  - Note: It is RAN1 common understanding that combination of above codes is not precluded

  - Note: Outer erasure code is not precluded

- Selection of 5G new RAT channel coding scheme(s) will consider,

  - Performance

  - Implementation complexity

  - Latency (Decoding/Encoding)

  - Flexibility (e.g., variable code length, code rate, HARQ (as applicable for particular scenario(s)))

To identify channel coding schemes for each usage scenario, the evaluation methodology was discussed with the agreement of using AWGN channel for the initial evaluation and calibration [1].

In this contribution, we introduces a new low complexity decoding algorithm of Turbo codes. The performance results of binary Turbo codes of the proposed decoding algorithm and the reference Max-log-MAP and log-MAP algorithms on AWGN channel are shown.  The complexities of different Turbo decoding algorithms along with LDPC and Polar decoding algorithms are also analyzed.

Discussion

The major scenarios for 5G new radio are eMBB (enhanced Mobile Broadband), mMTC (massive Machine Type Communications) and URLLC (Ultra-Reliable and Low Latency Communications). The target of eMBB communication is to provide peak throughput at least 20Gbps. The channel codes with the long code block (such as more than 10000 bits) and high speed decoding capacity are the best choice for eMBB. In mMTC and URLLC, selection criterions of channel codes are good performance for small code block with low energy consumption and low cost and very low latency.

The existing LTE Turbo coding scheme will face a great challenge to meet the higher requirements of NR. The high complexity and memory consumption of the traditional decoders, e.g., Log-MAP, Max-Log-MAP and SOVA [2] [3] [4], are the major obstacle of Turbo codes to be applied in NR.

  - Trimming Soft-Input Soft-Output Viterbi Algorithm

In this contribution, we provide an effective decoding algorithm, called Trimming Soft-Input Soft-Output Viterbi Algorithm (TSOVA), to reduce the complexity of traditional decoding schemes of Turbo codes. It is well-known that the complexity of the Soft-input Soft-output Viterbi Algorithm (SOVA) [4] largely depends on the number of backtracking operations or the number of metric differences. The proposed TSOVA decoding algorithm reduces the complexity of SOVA by trimming metric differences M times. By picking up M smallest ones among the metric differences, TSOVA only conducts backtracking operations on the related competitive paths of these metric differences. To minimize the degradation of LLRs due to trimming the backtracking operations, the proposed TSOVA algorithm selects important metric differences and estimates LLRs from intrinsic information and neighboring LLRs. Hence, TSOVA maintains good LLR quality with complexity reduction at most at 1/M backtracking operations of SOVA.

  - Simulation results

- eMBB applications

For eMBB applications, we compare the performance of the Log-MAP, Max-Log-MAP and TSOVA decoding algorithms for two long code block Turbo codes with several code rates R (e.g., 1/3 and 1/2).

The code block lengths and parameters of the Turbo code are as follows:

- Code 1 is a LTE-Turbo code with information block lengthbits.

- Code 2 is a LTE-Turbo code with information block lengthbits.

The simulations are conducted over the BI-AWGN channel with QPSK modulation. The maximum iteration number is set to 8 for all decoding algorithms. The trimming factor of the TSOVA algorithm is set to 8. Simulation results are shown in Figure 1 and Figure 2.

Figure 1: Performance of LTE-Turbo code with infomationblock length of 2016 bits and  code rates ½ and 1/3.

Figure2: Performance of LTE-Turbo code with informationblock length of 4032 bits and code rates ½ and 1/3.

It is shown from Figure 1 and Figure 2 that the TSOVA algorithm can outperform the Max-Log-MAP algorithm at the BLER of 10-4 for different code block lengths and code rates. The performance gap between the TSOVA and Log-MAP algorithms is within 0.2 dB for Code2 and even within 0.01 dB for Code1.

Observation 1: TheTSOVA decoding algorithm has slightly performance edge over  the Max-Log-MAP algorithm for the large block size. contribution.

- URLLC and mMTC applications

For URLLC and mMTC applications, we also compare the performance of the Log-MAP, Max-Log-MAP and TSOVA decoding algorithms for two short code block LTE-Turbo codes with code rates R= 1/3.

The parameters of the LTE-Turbo codes are as follows:

- Code 3 is a LTE-Turbo code with information block length bits.

- Code 4 is a LTE-Turbo code with information block length bits.

The simulations are conducted over the BI-AWGN channel with QPSK modulation and 16QAM, respectively. The maximum iteration number is set to 8 for all decoding algorithms. The trimming factor of the TSOVA algorithm is set to 8. Simulation results are shown in the Figure 3-Figure 6.

Figure3: Performance of LTE-Turbo code Info. block length of 200 bits with QPSK modulation.

Figure4: Performance of LTE-Turbo code with infomation block length of 200 bits and 16QAM modulation

Figure5: Performance of LTE-Turbo code with information block length of 576 bits and QPSK modulation

Figure 6: Performance of LTE-Turbo code with infomation block length of 576 bits and 16QAM modulation.

The performance results in Figure 3-Figure 6 show that the TSOVA algorithm can outperform the Max-Log-MAP algorithm at the BLER of 10-4 for different code block lengths and modulation. The performance gaps between the TSOVA and Log-MAP algorithms are very small for Code3 and Code4 with QPSK modulation and about 0.2 dB for Code3 and Code4 with 16QAM modulation.

Observation 2: For the short block size Turbo codes considered in this contribution, TSOVA outperforms Max-Log-MAP with both QPSP and 16QAM modulations.

  - Complexity analysis of Turbo decoding algorithms

Computational complexity is an important index in the evaluation of the decoding algorithms.  We compare the computational complexities of several decoding algorithms for LTE-Turbo codes and present in Table 1.

Table 1: Decoding complexity per iteration for binary Turbo codes

|  | Log-MAP | Max-Log-MAP | TSOVA |
| --- | --- | --- | --- |
| Additions |  |  |  |
| MAX process |  |  |  |
| Look-up-table operations |  | NA | NA |

Notations:  for information block length,  for memory length of component code of Turbo code, for backtracking length, for trimming factor. There are two component codes for the LTE-Turbo code. Number of multiplication is included within additions by considering log domain processing.

The computational complexity of the Code1~Code4 evaluated in Section 2.1 are presented in Table 2~Table 5, respectively. As shown in Table 2-Table 5, the total computational complexity of TSOVA for LTE- Turbo codes is about 43% compared with the Max-Log-MAP algorithm and 13% compared with the Log-MAP algorithm. Thus, the TSOVA has distinct advantage of the decodingcomplexity over both Max-Log-MAP and Log-MAP decoding algorithms.

Table 2: Decoding complexity analysis per iteration for Code1 in eMBB

|  | Log-MAP | Max-Log-MAP | TSOVA |
| --- | --- | --- | --- |
|  | Code 1 | Code 1 | Code 1 |
| Additions | 387072 | 258048 | 131040 |
| MAX process | 129024 | 129024 | 34272 |
| Look-up-table operations | 774144 | 0 | 0 |
| Total | 1290240 | 387072 | 165312 |

Table 3: Decoding complexity analysis per iteration for Code2 in eMBB

|  | Log-MAP | Max-Log-MAP | TSOVA |
| --- | --- | --- | --- |
|  | Code 2 | Code 2 | Code 2 |
| Additions | 774144 | 516096 | 262080 |
| MAX process | 258048 | 258048 | 68544 |
| Look-up-table operations | 1548288 | 0 | 0 |
| Total | 2580480 | 774144 | 330624 |

Table4: Decoding complexity analysis per iteration for Code3 in URLLC and mMTC

|  | Log-MAP | Max-Log-MAP | TSOVA |
| --- | --- | --- | --- |
|  | Code 3 | Code 3 | Code 3 |
| Additions | 38400 | 25600 | 13000 |
| MAX process | 12800 | 12800 | 3400 |
| Look-up-table operations | 12800 | 0 | 0 |
| Total | 64000 | 38400 | 16400 |

Table5: Decoding complexity analysis per iteration for Code4 in URLLC and mMTC

|  | Log-MAP | Max-Log-MAP | TSOVA |
| --- | --- | --- | --- |
|  | Code 4 | Code 4 | Code 4 |
| Additions | 110592 | 73728 | 37440 |
| MAX process | 36864 | 36864 | 9792 |
| Look-up-table operations | 36864 | 0 | 0 |
| Total | 184320 | 110592 | 47232 |

Notations: Memory length of component code of each Turbo code .Assuming the computation costs of Addition: MAX: Look-Up-Table = 1:1:6,  and.

Observation 3: The computational complexity of TSOVA is much less than those of Log-MAP and Max-Log-MAP for the long block size and short block size Turbo codes considered in this contribution.

  - Complexity Analysis of LDPC and Polar Decoding Algorithms

We make the same exercise of computational complexity analysis of LDPC and polar codes decoding algorithms. The outputs of the exercise are used for the comparison of the implementation complexity of the considered decoding algorithms for LTE-Turbo codes.

The analysis of the decoding complexitiesofTurbo, LDPC and polar codes is shown  in Table 6 in reference  to [6]. The example in [6] is used here.

Example: The parameters of complexities analysis are as follows,  the code block  length N=1944 for Turbo and LDPC codes, N = 2048 for polar code, R=1/2, m=3, Imax = 8 for Turbo code, Imax = 15, dv = 3.58 and dc = 7.17 for LDPC code, L = 32 for polar code. Assuming the computation costs of Addition: MAX: Look-Up-Table = 1:1:6, M0=8, δ=8. The complexity of the codes can be calculated as in Table 7. As shown in Table 7, the TSOVA for Turbo codes has the least computational complexity.

Table 6: Decoding complexity for Turbo, LDPC and polar codes

|  | Turbo (Max-Log-MAP) | Turbo (TSOVA) | LDPC (BP) [7] | Polar (SCL) [8] |
| --- | --- | --- | --- | --- |
| Additions |  |  |  |  |
| MAX process |  |  | NA | NA |
| Look-up-table | NA | NA |  | NA |

Notations:  for maximum iteration number,  for information block length, for code length,  for number of parity bits, for average variable degree of LDPC parity check matrix (PCM), for average check degree of LDPC PCM,  for size of list of polar code,  for memory length of component code of Turbo code, for backtracking length of TSOVA, for trimming factor of TSOVA. Assume two component codes for the Turbo code.

Table 7: Example complexity analysis for Turbo, LDPC, and polar codes

|  | Turbo (Max-Log-MAP) | Turbo (TSOVA) | LDPC (BP) | Polar (SCL) |
| --- | --- | --- | --- | --- |
| Additions | 995328 | 513216 | 403282 | 720896 |
| MAX processes | 497664 | 139968 | NA | NA |
| Look-up-table processes | NA | NA | 627231 | NA |
| Total | 1492992 | 653184 | 1030513 | 720896 |

In addition, we compare the implementation complexity between the Max-Log-MAP algorithm and TSOVA for LTE-Turbo codes. TSOVA requires less storage for intermediate quantity and less number of addressing operation compared with the Max-Log-MAP algorithm.  On the other hand, simulation results show that TSOVA requires less quantization bits (about 1 bit) compared with the Max-Log-MAP algorithm since TSOVA only requires minimum backtracking operations. Therefore, not only the implementation complexity of TSOVA is lower but also the throughput of TSOVA is higher.

Furthermore, TSOVA is suitable for high throughput ASIC implementation. The complexity of TSOVA is dominated by its first stage, which is Viterbi algorithm. It is shown that Viterbi decoders can achieve a throughput of 40Gbps [9].

Conclusion

In this contribution, we provide a Trimming Soft-Input Soft-Output Viterbi Algorithm (TSOVA) and compare the performance and complexity of TSOVA with different decoding schemes of Turbo codes in eMBB, URLLC and mMTC, respectively. It shows that the TSOVA algorithm can provide better trade-offs between performance and complexity and has higher throughput. Moreover, TSOVA can has similar convergence behavior as the Log-MAP algorithm.

By only changing the value of trimming factor M, TSOVA matches the different requirements of different usage scenarios. Hence, the Turbo codes with the TSOVA decoding scheme can be considered as one of the most promising candidates for different usage scenarios to fulfil higher requirements of 5G New Radio.

The above discussion is summarized with following observations:

Observation 1: There is a slight gap between TSOVA decoding algorithm has slightly performance edge over and the Max-Log-MAP algorithm for the large block size. Turbo codes considered in this contribution.

Observation 2: For the short block size Turbo codes considered in this contribution, TSOVA outperforms Max-Log-MAP with both QPSP and 16QAM different modulations.

Observation 3: The  computational complexity of TSOVA is much less than those of Log-MAP and Max-Log-MAP for the long block size and short block size Turbo codes considered in this contribution.

Observation 4: The TSOVA algorithm is robust to the quantization and can support  20 Gbps throughput which is required by eMBB services.

- References

[1].	RAN1 chairman notes in #84bits

[2].	L. Bahl, J. Cocke, F. Jelinek, and J. Raviv, “Optimal decoding of linear codes for minimizing symbol error rate,” IEEE Trans. Inf. Theory, vol.20, no. 2, pp. 284–287, Mar 1974.

[3].	M.P.C. Fossorier, F. Burkert, S. Lin, and J. Hagenauer, “On the equivalence between SOVA and Log-Map-map decodings,” IEEE Commun. Lett., vol. 2, no. 5, pp. 137–139,May 1998.

[4].	J. Hagenauer and P. Hoeher, “A Viterbi algorithm with soft-decision outputs and its applications,” in Proc. 1989 IEEE Global Commun.Conf, Nov 1989, pp. 1680–1686 vol.3.

[5].	J. Feldman, I. Abou-Faycal, and M. Frigo, “A fast maximum-likelihood decoder for convolutional codes,” in Proc. 2002 IEEE Vehicular Tech. Conf. -Fall, vol. 1, pp. 371–375.

[6].	R1-162897 “Performance and complexity of Turbo, LDPC and Polar codes”

[7].	W. E. Ryan, S. Lin, “Channel Codes: Classical and Modern”, New York, NY: Cambridge University Press, 2009

[8].	K. Niu, K. Chen, J. Lin, and Q. T. Zhang “Polar Codes: Primary Concepts and Practical Decoding Algorithms” IEEE Communication Magazine, vol. 52, no. 7, pp. 192-203, Jul. 2014.

[9].	T. Veigel, et al., “A Viterbi Equalizer Chip for 40 Gb/s optical communication links,” Microwave Integrated Circuits Conference (EuMIC), European IEEE, 2013.