3GPP TSG RAN WG1 Meeting #86                                 	           R1-166735

# Gothenburg, Sweden 22nd - 26th August 2016

# Source:        Samsung

Title:           LC based explicit CSI feedback and simulation results

Agenda Item:   7.2.4.2.3

Document for:  Discussion and Decision

- Introduction

In RAN1#85, the following agreement about advanced CSI was made [1].

Agreements:

- Advanced CSI

- Specify enhancement on CSI reporting to improve eNB precoding. The specified enhancement is to be selected from the following categories:

  - Enhancements to Rel-13 feedback codebooks (FFS which numbers of antenna ports from the set {8, 12, 16, 20, 24, 28, 32}) that increase CSI resolution through improved beam selection / construction in W1 and/or improved beam/port selection / combining / weighting mechanisms in W2

  - Parameters representing channel coefficients, or some reduced space representation thereof including beam combining / weighting with coefficient quantisation or channel quantisation or channel covariance matrix quantisation

    - Uplink physical channel enhancements to carry the representation of channel coefficients can be included if selected

- Also, interference measurement enhancement can be considered

This contribution proposes an enhancement on CSI reporting based on explicit feedback, which belongs to the 2nd category (highlighted) in the agreement. In particular, linear combination (LC) based explicit feedback framework is proposed to report a form of DL channel explicitly. Simulation results are provided to show performance of the proposed explicit feedback scheme.

- Linear Combination Based Explicit Feedback

Figure 1: LC based explicit feedback framework

In Figure 1, an LC based explicit CSI feedback framework is proposed in which a form of the DL channel is reported using a double codebook for explicit feedback: W = W1W2, where

- W1 codebook is for WB and long-term basis vectors {bl:l=0,1,…,L-1} feedback,

- W2 codebook is for SB and short-term LC coefficients {cl:l=0,1,…,L-1} feedback, and

- L is the size of the basis vector set.

A few examples of the forms of DL channel and their LC representations are as follows:

- Channel: For a subcarrier k in SB f, the DL channel is represented as Hk≈l=0L-1clbl;

- Covariance matrix: For SB f, the covariance matrix is represented as K=1|f|k∈fHkHkH≈l=0L-1clblblH; and

- Eigenvector: For SB f, the dominant eigenvector of the covariance matrix K is represented as e≈l=0L-1clbl.

For W1 codebook, the Rel. 13 Class A codebook framework can be re-used with the following two alternatives for the basis vectors (as shown in Table 1):

- (Alt 0) L = 4: Config = 2, 3, and 4 of Rel. 13 Class A codebook; and

- (Alt 1) L = 8: (4, 2), where 1st dimension is the longer dimension.

Table 1: Beam groups

| Number of beams (L) | Config | Beam group |
| --- | --- | --- |
| 4 (Alt0) | 2 |  |
|  | 3 |  |
|  | 4 |  |
| 8 (Alt1) | - |  |

The W2 codebook has two components:

- Co-phase φll=0L-1: co-phase belonging to {1,j,-1,-j} for each of for L beams.

- Coefficients cll=0L-1: The unquantized LC coefficients can be obtained as the least-square solution to minimize the squared error. For example, for eigenvector feedback, the unquantized coefficients are cll=0L-1=argmincll=0L-1e-l=0L-1clblφlbl2. The resultant solution is obtained by pre-multiplying the dominant eigenvector e with the pseudo-inverse of the basis vector set, i.e., cll=0L-1=B*B-1B*e, where B=[b0, b1,…,bL-1] is basis matrix in which columns are basis vectors.

Proposal 1: For advanced CSI reporting, consider explicit reporting of a form of DL channel using a LC framework, where

- the form of DL channel can be DL channel itself or covariance matrix or eigenvectors; and

- the LC framework corresponds to a double codebook for explicit feedback: W1W2

  - W1 codebook is for the WB selection of L basis vectors and is based on Rel. 13 Class A codebook framework with the following two alternatives for the L basis vectors (Table 1)

    - (Alt 0) L = 4: Config = 2, 3, and 4 of Rel. 13 Class A codebook; and

    - (Alt 1) L = 8: (4, 2), where 1st dimension is the longer dimension.

  - W2 codebook is for SB co-phase and coefficient selection for the selected L basis vectors, where

    - Co-phase values are selected from {1,j,-1,-j}.

    - Coefficients are selected to quantize LC coefficients, e.g., to minimize squared error.

The proposed LC based explicit feedback scheme can also be applied to enhance Class B, K ≥ 1 CSI reporting. For instance, for K = 1 and P beam-formed ports, the UE can report explicit CSI, which represents P-port beam-formed channel using the proposed scheme. Similarly, the proposal is applicable to enhance hybrid CSI reporting scheme in which two types of CSI-RS resources, for example non-precoded and beamformed CSI-RS, are associated with two eMIMO-Types. One or both of the two reported CSIs can be the proposed LC based explicit CSI.

Observation 1: Proposal 1 can be extended to Class B and hybrid CSI enhancement in which at least one of the reported CSIs indicates LC based explicit CSI.

In the proposed explicit CSI feedback scheme, the reported CSI comprises of the following components:

- 1st PMI (i1,1, i1,2): indicating L beams or basis vectors, similar to Rel. 13 Class A codebook;

- 2nd PMI i2: indicating

  - co-phase for the L selected beams from the QPSK codebook {i,j,-1,-j}, and

  - LC coefficients to combine the beams;

- (Optional) CQI: indicating dominant eigenvalues; and

- (Optional) RI: indicating the preferred rank.

Proposal 2: The explicit CSI report includes the following components:

- 1st PMI (i1,1, i1,2) indicating L beams or basis vectors, similar to Rel. 13 Class A codebook;

- 2nd PMI i2 indicating QPSK co-phase and coefficients for the L selected beams; and

- (optional) CQI and RI.

- Simulation Results

To demonstrate the performance of the proposed LC based explicit feedback scheme, simulation-level simulation results are provided for the following alternatives to represent the dominant eigenvector of the DL channel:

- L = 4: Config 2

- L = 4: Config 3

- L = 8: (4,2) beam group.

For co-phase, QPSK codebook is assumed and results are provided for unquantized coefficients. The first PMI (i1,1, i1,2) reporting overhead is the same for all schemes. The second PMI i2 reporting overhead is according to Table 2 where K is the number of bits to represent each coefficient.

Table 2: i2 reporting overhead

| L | Beam group | Co-phase (#bits) | Coefficients (#bits) | Total (#bits) |
| --- | --- | --- | --- | --- |
| 4 | Config 2 | 8 | 4K | 8 + 4K |
|  | Config 3 |  |  |  |
| 8 | (4,2) | 16 | 8K | 16 + 8K |

The non-full-buffer system-level evaluation is carried out for UMa-200m and UMi-2GHz channel models in heavy (70% target RU) traffic loading scenarios. The detailed results can be found in Table 4 and Table 5 in the Appendix. The results are provided for 16 and 32 antenna ports with (N1,N2) = (4,2) and (N1,N2) = (4,4), respectively. Here, we assume that the first dimension is horizontal and the second dimension is vertical. The downtilt angles in the elevation domain are chosen according to [2]. In these simulations, full-port non-precoded CSI-RS is used for CSI estimation, and the corresponding CSI-RS overhead is taken into account in the final throughput calculation. Cell association antenna pattern is approximated by one-TXRU pattern, and proportional fair scheduling (max 4 layers per time-frequency resource) have been used. For MU-MIMO, SLNR precoding is considered. The relevant simulation parameters are enlisted in Table 3. The rest of the simulation assumption is according to [2]. For comparison, performance results for Class A, Config 2, 3, and 4 based implicit CSI feedback, LC codebook proposed in [3], and ideal CSI feedback in which dominant eigenvector is assumed to be known at eNB are also provided. The performance gains with “Class A Config 2 based implicit scheme” as reference are summarized in Figure 2 - Figure 5.

Figure 2: (N1,N2) = (2,4), UMa-200m

Figure 3: (N1,N2) = (2,4), UMi-2GHz

Figure 4: (N1,N2) = (4,4), UMa-200m

Figure 5: (N1,N2) = (4,4), UMi-2GHz

From the SLS results, we can make the following observation.

Observation 2: From the SLS results, we can make the following observation:

- Proposed explicit feedback scheme shows significant gain over implicit scheme and reduces gap towards ideal CSI: ~3-4% gap in avg. UPT; and ~10-15% gap in 5% UPT

- L = 8 shows significant performance gain over L = 4.

- Performance gain is maintained on increasing the number of ports from 16 to 32.

- Conclusion

This document proposes an LC based explicit CSI feedback. The proposals and observations made can be summarized as follows.

Proposal 1: For advanced CSI reporting, consider explicit reporting of a form of DL channel using a LC framework, where

- the form of DL channel can be DL channel itself or covariance matrix or eigenvectors; and

- the LC framework corresponds to a double codebook for explicit feedback: W1W2

  - W1 codebook is for the WB selection of L basis vectors and is based on Rel. 13 Class A codebook framework with the following two alternatives for the L basis vectors (Table 1)

    - (Alt 0) L = 4: Config = 2, 3, and 4 of Rel. 13 Class A codebook; and

    - (Alt 1) L = 8: (4, 2), where 1st dimension is the longer dimension.

  - W2 codebook is for SB co-phase and coefficient selection for the selected L basis vectors, where

    - Co-phase values are selected from {1,j,-1,-j}.

    - Coefficients are selected to quantize LC coefficients, e.g., to minimize squared error.

Observation 1: Proposal 1 can be extended to Class B and hybrid CSI enhancement in which at least one of the reported CSIs indicates LC based explicit CSI.

Proposal 2: The explicit CSI report includes the following components:

- 1st PMI (i1,1, i1,2) indicating L beams or basis vectors, similar to Rel. 13 Class A codebook;

- 2nd PMI i2 indicating QPSK co-phase and coefficients for the L selected beams; and

- (optional) CQI and RI.

Observation 2: From the SLS results, we can make the following observation:

- Proposed explicit feedback scheme shows significant gain over implicit scheme and reduces gap towards ideal CSI: ~3-4% gap in avg. UPT; and ~10-15% gap in 5% UPT

- L = 8 shows significant performance gain over L = 4.

- Performance gain is maintained on increasing the number of ports from 16 to 32.

References

[1] RAN1#85, Chairman’s notes

[2] 3GPP, TR36.897v1.0.1, “Study on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE”

[3] R1-166734, “Linear combination (LC) codebook based CSI reporting and simulation results,” Samsung

Appendix: Simulation Assumptions and Results

Table 3: Simulation Parameters

| Parameters | Values |
| --- | --- |
| Simulation Type | Non-full-buffer (Heavy load 70% Target RU, Lambda = 4) |
| Channel model | UMi-2GHz, UMa-200m |
| Number of BS (H,V) antenna elements | (8,8), x-polarized, subarray partition |
| (N1,N2, P) | 16, 32 ports: (4,2,2), (4,4,2) |
| BS (H,V) antenna spacing | (0.5, 0.8)λ |
| BS and MS antenna polarizations | BS: (+45°,-45°); MS: (0°, 90°) |
| Number of UE antennas | 2 |
| SU/MU pre-coding | SLNR |
| Scheduling | MU, Proportional fair, up to 4 layers |
| Channel estimation | Ideal |
| Transmission rank | 1 |
| Receiver | MMSE-IRC |
| Explicit CSI schemes | LC: Rel. 13 Class A W1 codebook as framework 4 beams: Config 2,3 8 beams: (4,2) beam group Co-phase: QPSK Coefficient: Unquantized For comparison: ideal CSI, Class A, LC Codebook [3] |

Table 4: Non-full buffer simulation results for 16 ports

| Channel | Scheme | RU | Avg UPT | 50% UPT | 5% UPT | Avg UPT gain | 50% UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UMa-200m | Class A, Config 2 | 57.4% | 17.29 | 17.43 | 5.78 | 100.0% | 100.0% | 100.0% |
|  | Class A, Config 3 | 57.6% | 17.20 | 17.24 | 5.78 | 99.5% | 98.9% | 99.9% |
|  | LC CB, Config 2 | 53.2% | 19.42 | 20.00 | 8.00 | 112.3% | 114.8% | 138.4% |
|  | LC CB, Config 3 | 53.2% | 19.41 | 20.20 | 8.10 | 112.3% | 115.9% | 140.1% |
|  | LC exp, Config 2 | 51.9% | 20.17 | 21.20 | 8.85 | 116.6% | 121.6% | 153.1% |
|  | LC exp, Config 3 | 51.8% | 20.17 | 21.33 | 8.93 | 116.7% | 122.4% | 154.5% |
|  | LC exp, (4,2) beam group | 50.3% | 21.09 | 22.47 | 10.06 | 122.0% | 128.9% | 173.9% |
|  | Ideal | 49.8% | 21.59 | 22.99 | 10.97 | 124.9% | 131.9% | 189.8% |
| UMi-2GHz | Class A, Config 2 | 58.6% | 17.46 | 17.58 | 6.53 | 100.0% | 100.0% | 100.0% |
|  | Class A, Config 3 | 58.9% | 17.37 | 17.49 | 6.49 | 99.5% | 99.5% | 99.4% |
|  | LC CB, Config 2 | 54.6% | 19.64 | 20.62 | 8.19 | 112.5% | 117.3% | 125.5% |
|  | LC CB, Config 3 | 54.6% | 19.64 | 20.62 | 8.29 | 112.5% | 117.3% | 127.1% |
|  | LC exp, Config 2 | 52.9% | 20.51 | 21.86 | 9.17 | 117.4% | 124.3% | 140.5% |
|  | LC exp, Config 3 | 52.8% | 20.53 | 21.72 | 9.37 | 117.5% | 123.6% | 143.5% |
|  | LC exp, (4,2) beam group | 51.3% | 21.41 | 22.86 | 10.41 | 122.6% | 130.0% | 159.4% |
|  | Ideal | 50.6% | 21.95 | 23.53 | 11.05 | 125.7% | 133.8% | 169.4% |

Table 5: Non-full buffer simulation results for 32 ports

| Channel | Scheme | RU | Avg UPT | 50% UPT | 5% UPT | Avg UPT gain | 50% UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UMa-200m | Class A, Config 2 | 55.8% | 18.40 | 18.78 | 7.04 | 100.0% | 100.0% | 100.0% |
|  | Class A, Config 3 | 55.7% | 18.35 | 18.69 | 7.12 | 99.7% | 99.5% | 101.1% |
|  | LC CB, Config 2 | 52.2% | 20.23 | 21.28 | 9.01 | 110.0% | 113.3% | 127.9% |
|  | LC CB, Config 3 | 52.1% | 20.21 | 21.28 | 8.88 | 109.8% | 113.3% | 126.1% |
|  | LC exp, Config 2 | 50.9% | 20.91 | 22.22 | 10.03 | 113.7% | 118.3% | 142.4% |
|  | LC exp, Config 3 | 50.8% | 20.92 | 22.10 | 10.15 | 113.7% | 117.7% | 144.2% |
|  | LC exp, (4,2) beam group | 49.6% | 21.63 | 23.12 | 10.99 | 117.6% | 123.1% | 156.1% |
|  | Ideal | 48.6% | 22.30 | 24.10 | 11.94 | 121.2% | 128.3% | 169.5% |
| UMi-2GHz | Class A, Config 2 | 55.4% | 19.03 | 19.75 | 7.89 | 100.0% | 100.0% | 100.0% |
|  | Class A, Config 3 | 55.4% | 19.00 | 19.70 | 8.05 | 99.9% | 99.8% | 102.0% |
|  | LC CB, Config 2 | 52.3% | 20.85 | 22.10 | 9.96 | 109.6% | 111.9% | 126.3% |
|  | LC CB, Config 3 | 52.2% | 20.85 | 22.22 | 9.99 | 109.6% | 112.5% | 126.6% |
|  | LC exp, Config 2 | 51.1% | 21.52 | 22.92 | 10.72 | 113.1% | 116.1% | 136.0% |
|  | LC exp, Config 3 | 51.0% | 21.57 | 22.99 | 10.79 | 113.3% | 116.4% | 136.9% |
|  | LC exp, (4,2) beam group | 49.9% | 22.24 | 23.67 | 11.80 | 116.9% | 119.8% | 149.6% |
|  | Ideal | 49.0% | 22.95 | 24.92 | 12.90 | 120.6% | 126.2% | 163.6% |