3GPP TSG RAN WG1 Meeting #87                                                                         R1-1611345

Reno, USA, November 14th-18th, 2016

Source:	CATT

Title:	Remaining issues on frequency domain CSI-RS overhead reduction

Agenda Item:	6.2.2.4.1

Document for:	Discussion and Decision

Introduction

In RAN#71, the work item on further full-dimension MIMO (FD-MIMO) enhancement has been approved [1]. The objective of reference signal enhancement for non-precoded CSI-RS is:

- Non-precoded CSI-RS, extending the existing numbers {1, 2, 4, 8, 12, 16} of CSI-RS antenna ports for support of {20, 24, 28, 32} CSI-RS ports with mechanism for reducing the overhead for CSI-RS transmission

The following agreements about density reduction for CSI-RS have been achieved in RAN1#86bis:

Agreement:

- For Class B NZP CSI-RS:

  - Support CSI-RS density of d ∈ {1,1/2} RE/RB/port

  - Additional values of d=1/3 and/or 1/8 are not precluded for Class B if significant benefit can be shown at RAN1#87.

- Transmission and/or measurement comb offsets are 0..1/d-1

  - For d=1/2 (i.e. for Class A & Class B), comb offsets ∈ {0, 1} can be configured for CSI-RS transmission and/or measurement in the n-th PRB, where n mod 2 = comb offset

  - For d=1/3 (i.e. at least for Class A), comb offsets ∈ {0, 1, 2} can be configured for CSI-RS transmission and/or measurement in the n-th PRB, where n mod 3 = comb offset

- Working Assumption: For Class A NZP CSI-RS with more than 16 CSI-RS port, in a given CSI-RS resource all CSI-RS configurations have the same values of d

  - Will be confirmed automatically at RAN1#87 unless there is consensus to support different values of d per CSI-RS configuration

- For Class A NZP CSI-RS with more than 16 CSI-RS ports each CSI-RS configuration can have different values of comb offset

- FFS the detailed signalling design

In addition, CDM-8 has been agreed to be supported for at least {24,32} ports. The corresponding agreements are shown as follows:

Agreement:

- CDM-8 design for {24, 32} ports:

  - At least for 32-port CSI-RS, at least one CDM-8 pattern exists which uses REs from more than 2 OFDM symbols

  - FFS until RAN1#87:

    - which CDM-8 patterns are useful, and

    - downselection between:

      - Alt 1: CDM-8 patterns are configured by aggregation of CDM-2/4 patterns

      - Alt 2: CDM-8 patterns are defined in the specifications without explicit aggregation

  - Overhead reduction for CDM-8 is FFS

  - Note: CDM-8 can be supported in both normal and special subframes

In this contribution, we discuss the potential issues about the configurations of CSI-RS density reduction.

Discussion

According to the above agreement, there are two more RRC parameters employed for CSI-RS configuration, i.e. CSI-RS density d and comb offset. For {20,24,28,32} ports CSI-RS, a CSI-RS resource is aggregated by K CSI-RS configurations. Each CSI-RS configuration would have a CSI-RS density value and a comb offset value. As port sharing with Rel-12/13 UEs is not supported for {20,28} ports CSI-RS, it seems no need to configure different densities for different CSI-RS ports. Therefore, we prefer to use the same value of d for all CSI-RS configurations in a CSI-RS resource, in order to guarantee the same density for all the CSI-RS ports.

Proposal:

- Use the same value of d for all CSI-RS configurations in a CSI-RS resource.

In the case of comb offset, for {20,24,28,32} ports CSI-RS, each CSI-RS configuration can have a different value. It is feasible for CDM-2 and CDM-4, because their CDM patterns are mapped within a single CSI-RS configuration. However for CDM-8, one CDM-8 pattern may use REs from more than 2 OFDM symbols. This implies the CDM-8 pattern would be mapped across multiple CSI-RS configurations. Figure 1 depicts a candidate 32 port CSI-RS with CDM-8, which is aggregated by (M,K)=(8,4). The REs with the same color correspond to an 8-port CSI-RS configuration, and the REs labeled with the same alphabet denote a CDM-8 group. If different comb offset values are used for different configurations, one CDM-8 group may be mapped to different PRBs, which will deteriorate the orthogonality within the CDM group. Therefore, for CDM-8, the possible combinations of the K comb offset values in a CSI-RS resource needs to be constrained. The same comb offset value for each CSI-RS configuration is preferred.

Figure 1: Candidate 32-port CSI-RS with CDM-8

Proposal:

- For CDM-8, the same comb offset value should be configured for all CSI-RS configurations in a CSI-RS resource.

Conclusions

In this contribution, we discuss the CSI-RS density reduction configuration issues, where special concern is required for CDM-8. We have the following proposal and observation:

Proposals:

- Use the same value of d for all CSI-RS configurations in a CSI-RS resource.

- For CDM-8, the same comb offset value should be configured for all CSI-RS configurations in a CSI-RS resource.

- References

- RP-160623, “New WID proposal: Enhancement of Full-Dimension (FD) MIMO for LTE,” Gothenburg, Sweden, March 7-10, 2016