3GPP TSG-RAN WG1 #87	                 	        R1-1611546

Reno, USA,

Nov. 14th-18th, 2016

Agenda Item:  7.1.3.3

Source:	Sony

Title:	Beam Training Procedure for NR MIMO in FDD

Document for:	Discussion and consideration

Introduction

During RAN1 #86 meeting, three fundamental procedures of beam management have been defined and agreed [1].

Agreements:

- The following DL L1/L2 beam management procedures are supported within one or multiple TRPs:

  - P-1: is used to enable UE measurement on different TRP Tx beams to support selection of TRP Tx beams/UE Rx beam(s)

    - For beamforming at TRP, it typically includes a intra/inter-TRP Tx beam sweep from a set of different beams

    - For beamforming at UE, it typically includes a UE Rx beam sweep from a set of different beams

    - FFS: TRP Tx beam and UE Rx beam can be determined jointly or sequentially

  - P-2: is used to enable UE measurement on different TRP Tx beams to possibly change inter/intra-TRP Tx beam(s)

    - From  a possibly smaller set of beams for beam refinement than in P-1

    - Note: P-2 can be a special case of P-1

  - P-3: is used to enable UE measurement on the same TRP Tx beam to change UE Rx beam in the case UE uses beamforming

In this contribution, we present our view of hybrid beamforming with special focus on the analog part. Then we discuss the fundamental procedure of beam management with partial Channel Reciprocity (CR) or no CR.

Overview of Hybrid Beamforming

Full-Connection Model for Hybrid Beamforming

Assuming the full-connection model of hybrid beamforming in Figure 1, we next present the mathematical model when taking only the k-th UE (1≤k≤K) into consideration. Note that the sub-connection model can be expressed in a similar way. However, the procedure of beam management can be different between full-connection and sub-connection model due to physical antenna structure.

Specifically, the serving TRP and each out of K UEs is equipped with M and N antenna elements respectively. Compared with traditional full digital precoding at TRP which requires M RF chains, the hybrid beamforming architecture enjoys a much lower hardware complexity with only marginal performance loss. For simplicity, we assume K RF chains at TRP and single RF chain at each UE.

On the DL direction, the TRP applies a digital precoder B=b1 b2…bK∈CK×K followed by an analog precoder F=f1 f2…fK∈CM×K. Therefore, the transmitted signal at TRP is given as s=FBx where x∈CK×1 are data streams for K UEs. Then the received signal of the k-th UE can be expressed as

yk=wkTHkFBx+wkTnk,  1≤k≤K

where Hk∈CN×M is the channel between TRP and the k-th UE, nk∈CN×1 is the AWGN and wk is the analog Rx beam adopted by the k-th UE.

Moreover, the analog Tx-Rx beams are generally chosen from pre-defined codebooks, denoted as F and W at TRP and UE, respectively. The purpose of beam training is to find the optimal beam pair which satisfies the maximum RSRP condition, but not limited to

wk,opt, fk,opt=argmax ||wkTHkfk||,    s.t. wk∈ W, fk∈F

After the analog Tx-Rx beam pair is selected, we have the equivalent baseband channel established between TRP and the k-th UE as Heqk=wk,optTHkFopt . Finally, by collecting all the equivalent channel of K UEs, the digital precoder B is then obtained at TRP, using classic precoding criteria, such as Zero-Forcing (ZF), Minimum Mean Square Error (MMSE) and Signal Leakage Noise Ratio (SLNR).

Figure 1. Hybrid Beamforming Architecture

Categories of Beam Training

Speaking of beam training in this contribution, one may consider it as a more general concept when compared with beam sweeping. In other words, beam sweeping can be viewed as a specific procedure to implement beam training. From the perspective of beam training between TRP(s) and UE(s), one may classify the training procedure in different ways. To the best of our knowledge, we list the categories, but not limited to

- Channel reciprocity (CR), partial CR or non-CR assisted beam training

- Joint or separate Tx-Rx beam training

- Single stage or multiple stage Tx-Rx beam training

- Control or data channel Tx-Rx beam training

- Single or multiple TRP(s) beam training

- Single or multiple Tx-Rx beam pair(s)

Some of the categories mentioned above could be further combined, thereby resulting in numerous types of beam training for us to consider and compare in the forthcoming meetings.

Issues of Beam Training

Besides the state-of-the-art beam management technologies, we still face a few of issues which may have strong impact over performance. Likewise, to the best of our knowledge, we make an issue listed below, but not limited to

- Huge overhead burden considering MU beam training

- Link blockage and its quick recovery

- UE rotation / fast-moving behaviors

- Gap of beamforming gain between control and data channel

As the NR standard progress moves forwardly, we may identify more beam management related issues to be handled. For more discussion of beam management issues, please kindly find our paper [2]. Currently, assuming the sequential/joint beam sweeping as a baseline, we next discuss the overhead reduction of beam training procedure and then propose potential solutions.

Discussion on Beam Training

In this section, we briefly discuss the beam training procedures for FDD system assuming partial Channel Reciprocity (CR) or no CR.

Partial Channel Reciprocity Assisted Beam Training

In FDD mode, there exists partial channel reciprocity which may be leveraged as assistant information to beam training procedure. The rationale behind this statement lies in the fact that the signal paths of DL and UL in spatial domain may somehow overlap in some scenarios of mmWave band. More specifically, with the aid of beam training results in DL/UL, it is possible to further reduce the beam training overhead in the converse direction, i.e. UL/DL.

Figure 2. Tx-Rx Connectivity of All Beam Combinations

Let us take the example above for illustration. First, one may consider the beam sweeping in DL with joint beam training. Then we may have a table as shown in Figure 2 which presents the connectivity of all the Tx-Rx beam combinations. Specifically, for illustration purpose we assume 4 by 4 Tx-Rx beam combinations with columns and rows representing Tx and Rx beams respectively. Given one set of 4 Rx beams, one may find a strong Tx beam, and with 4 sets of Rx beams one may have 4 strong Tx beams which can be overlapped. Then one may obtain the first 2 strongest Tx beams colored as yellow in Figure 3; similarly given 4 sets of Tx beams, one may have the first 2 strongest Rx beams colored as blue in Figure 3. Afterwards, these combinations of the selected 2 strongest Tx and 2 strongest Rx beams in DL could be the beam training set for UL beam training. Therefore, the UL overhead can be significantly reduced from search space of 16 (4 by 4) to 4 (2 by 2).

Observation 1: For FDD mode, DL/UL beam sweeping results could be assistant information for UL/DL to greatly reduce the overhead of beam training.

Proposal 1: For FDD NR, we need to study and design the beam training procedure based on partial channel reciprocity.

Beam Training without Channel Reciprocity

For certain scenarios of FDD, the channel reciprocity between DL and UL channel completely perishes. It is not easy to avoid the heavy overhead and feedback for beam sweeping in DL direction. Nevertheless, thanks to the inspiring codebook design in ongoing LTE, it is possible to apply the DFT-based vector as analog Tx/Rx codebook for beam training procedure. The basic idea is utilizing the Kronecker-Product (KP) structure of codebook to decouple the Tx/Rx beam at TRP into horizontal and vertical codebook. In addition, if it is necessary, it is also feasible to decouple analog codebook at UE side for the same reason.

In particular, for a Tx/Rx beam vector in the codebook, for instance f ∈F, then it can be expressed as the KP of a horizontal beam vector and a vertical beam vector as f=fh⨂fv where we have fh∈Fh and fv∈Fv where Fh and Fv denote the horizontal and vertical codebook respectively with the property that F=Fh⨂Fv. Consequently, by dropping the subscript of UE k for simplicity, we may re-write the beam training criterion as

wopt, fh,opt,fv,opt=argmax ||wTH(fh⨂fv)||,    s.t. w∈W, fh∈Fh, fv∈Fv

Therefore, we have the chance to reduce the overhead of beam training by separately sweeping beams in horizontal and vertical domain at TRP as shown in Figure 3.

Particularly, in the 1st step, TRP transmits omni-direction beam reference signals to train UE Rx beam. In reality, the omni-direction beam should be replaced by a set of coarse beams with sufficient cell coverage. In the 2nd step, TRP only transmits beams in vertical domain while UE uses the previously selected Rx beam. In the last step, TRP only transmits beams in horizontal domain while UE uses the same Rx beam as Step 2. It is worthy to note that the training procedure above-mentioned can be switched to facilitate beam training procedure. Moreover, in the 1st step, TRP transmits omni-direction beam, thereby resulting in small beamforming gain. This may somehow affect the beam training at UE side. Here, we use the omni antenna pattern just for illustration purpose. In reality, two solutions may work. One is to train TRP beam first, and then go to UE beam training. The other is to decouple the omni antenna pattern into directional coarse beams with sufficient beamforming gain.

Figure 3 Beam Training Procedure in Horizontal and Vertical Domain

Observation 2: Tx/Rx beam training/sweeping at TRP can be separately carried out in horizontal and vertical domain to reduce overhead and UE complexity.

Proposal 2: RAN1 needs to study the necessary mechanism of beam management by decoupling 3D beam into both horizontal and vertical beams.

Conclusions

Finally, allow us to summary our observations and proposals as below

Observation 1: For FDD mode, DL/UL beam sweeping results could be assistant information for UL/DL to greatly reduce the overhead of beam training.

Observation 2: Tx/Rx beam training/sweeping at TRP can be separately carried out in horizontal and vertical domain to reduce overhead and UE complexity.

Proposal 3: For FDD NR, we need to study and design the beam training procedure based on partial channel reciprocity.

Proposal 4: RAN1 needs to study the necessary mechanism of beam management by decoupling 3D beam into both horizontal and vertical beams.

References

[1] R1-168278, “WF on DL beam management”, RAN1 #86.

[2] R1-1608934, “Views on beam management for NR”, RAN1 #86bis.