3GPP TSG-RAN WG1 #87	R1-1612663

Reno, USA November 14-18, 2016

Source:	Ericsson

Title:	Wideband vs. Subband Amplitude Feedback

Agenda Item:	6.2.2.1.1

Document for:	Discussion and Decision

# Introduction

In RAN1#86, the following was agreed regarding advanced CSI reporting:

- Specify CSI feedback enhancement with the following advanced CSI feedback framework:

  - Reduced space (eigenvectors)/W1 is constructed based on one of the following alternatives (TBD RAN1#86bis):

    - Alt1. Orthogonal basis (e.g. orthogonal DFT matrix)

    - Alt2. Non-orthogonal basis (e.g. Rel.13 Class A W1 for rank-1 and/or 2)

  - Reduced space representation/W2 is to further combine selected beams

  - Granularity of weighting(phase and/or amplitude) can be either wideband only or wideband/subband, and is constructed based on one of the following alternatives (TBD RAN1#86bis):

    - Alt1. Phase and amplitude

    - Alt2. Phase-only weighting

  - How the enhanced framework can be applicable for Class A and/or Class B eMIMO-Types is FFS

  - FFS: How to handle the relationship between advanced CSI feedback and legacy CSI feedback framework

In our contribution for RAN1#86bis [1], we presented our view on the need for both amplitude and phase weighting. In this contribution, we further investigate the performance impact of different granularities of beam amplitude weighting.

# Amplitude quantization in Advanced CSI codebook

As further elaborated in [2], an advanced CSI rank-2 precoder based on linear combination of beams can be expressed as

W(2)=W1W2(2)=w0,0w0,1w1,0w1,1

wr,l=i=0Nbeams-1bIS(i)⋅pr,l,i⋅cr,l,i, r=0,1,l=0,1,

where  bIS(i) is a 2D DFT beam with index i and pr,l,i and cr,l,i are gain and phase combining coefficients for beam i on polarization r, for layer l.

In this contribution, we have investigated the following alternatives for quantizing the gain combining coefficients pr,l,i:

- Wideband: A single power coefficient is used for both layers and polarizations for a beam and is selected on a wideband level

  - i.e. p0,0,if=p1,0,if=p0,1,if=p1,1,if=pi, ∀f

- Subband:  A single power coefficient is used for both layers and polarizations for a beam and is selected on a subband level

  - i.e. p0,0,if=p1,0,if=p0,1,if=p1,1,if=pif

- Subband per polarization and layer: The beam power is set individually for each polarization, layer and subband

  - i.e. p0,0,if≠ p1,0,if≠ p0,1,if≠p1,1,if≠pif in the general case

- Subband differential:  Same as Subband, but the beam power coefficients are differentially encoded with respect to a wideband beam power coefficient

- Wideband per layer:  A wideband power level is selected separately for each layer (but is the same for both polarizations)

  - i.e. p0,0,if=p1,0,if=pi,0,∀f  and p0,1,if=p1,1,if=pi,1, ∀f

The associated feedback payload for beam amplitude quantization, assuming combining of Nbeams beams and Nbits,power bits to quantize each coefficient pr,l,i is presented in Table 1 below.

Table 1: Feedback payload for amplitude quantization

| Amplitude Quantization scheme | Amplitude Feedback Payload [bits] |  |
| --- | --- | --- |
|  | General expression | Assuming Nbeams=3 and Nbits,power=3 and NSB=9 and Nbits,power,diff=1 |
| Wideband | (Nbeams-1) ⋅Nbits,power | 6 bits |
| Subband | NSB⋅(Nbeams-1)⋅Nbits,power | 54 bits |
| Subband per polarization and layer | 4⋅NSB⋅(Nbeams-1)⋅Nbits,power | 216 bits |
| Subband differential | Nbeams-1⋅Nbits,power +NSB⋅(Nbeams-1)⋅Nbits,power,diff | 24 bits |
| Wideband per layer | 2⋅(Nbeams-1) ⋅Nbits,power | 12 bits |

For the Wideband amplitude quantization scheme, only (Nbeams-1) amplitude coefficients have to be quantized as one can assert that the strongest beam is the one with index i=0 and set p0=1. In the subband cases this is not necessarily so, as the strongest beam can vary over the subbands (assuming a wideband beam selection). However, to keep overhead down, we assume quantization relative to the strongest wideband beam in the subband cases as well for 8-32 antenna ports. For 4TX however, this constraint was found to be too restrictive and the power levels for both beams are quantized.

# Evaluation results

We have evaluated the different amplitude quantization schemes for advanced CSI codebook presented in the previous section, using a codebook based on unrestricted orthogonal beam selection of Nbeams=3 beams in W1 (except for the 4TX case which uses 2 beams) and 8-PSK beam phase combining in W2, according to our proposed codebook presented in [2]. The schemes are evaluated for 32, 16, 8 and 4 antenna ports using an 8x4 antenna array (except for 4TX which uses a 4x4 array) in the 3GPP 3D UMi scenario using the FTP-1 traffic model. MU-MIMO based on SLNR is used for all simulated systems, and other parameters are according to the Appendix.

The results are presented in Figures 1-4 below. For all number of ports, the Wideband, Subband, Subband differential and Wideband per layer amplitude quantization schemes perform approximately equal. As the feedback payload is only 6 bits for the Wideband amplitude quantization scheme while it is 54 bits for the Subband amplitude quantization scheme, it is thus obvious that the former scheme should be preferred.  To get an additional increase in performance over the Wideband scheme, one must go as far as to quantize the amplitude for each layer, polarization and subband independently, resulting in an “explicit feedback”-like quantization of the reduced space eigenvectors. By doing so, a significant performance gain can be achieved, going from 32% to 54% cell edge gain over Rel-13 in the 32TX case. However, the increase in feedback payload is 210 bits, which is a quite massive increase from the 6 bits feedback payload. Allowing for such a large payload for amplitude quantization must be well motivated, especially since the marginal utility of increasing the payload from 6 bits is around zero until one reaches a sufficiently large payload so that the amplitude can be set per subband, layer and polarization.

No performance difference between wideband and subband beam amplitude quantization when the same power coefficients are used across layers and polarizations

Independent beam amplitude quantization for subband, layer and polarization gives a significant gain over wideband beam amplitude quantization, but with substantially larger overhead

Based on these observations we propose the following:

For advanced CSI codebook, beam amplitude coefficients are selected on a wideband basis and is the same for both polarizations and all layers of a beam

Figure 1: 32TX advanced CSI codebook performance with different amplitude quantization granularity

Figure 2: 16TX advanced CSI codebook performance with different amplitude quantization granularity

Figure 3: 8TX advanced CSI codebook performance with different amplitude quantization granularity

Figure 4: 4TX advanced CSI codebook performance with different amplitude quantization granularity

# Conclusions

In this contribution, the required granularity of beam amplitude quantization for advanced CSI codebooks have been investigated. The following observations have been made:

Observation 1	No performance difference between wideband and subband beam amplitude quantization when the same power coefficients are used across layers and polarizations

Observation 2	Independent beam amplitude quantization for subband, layer and polarization gives a significant gain over wideband beam amplitude quantization, but with substantially larger overhead

Based on these observations, we have made the following proposals:

Proposal 1	For advanced CSI codebook, beam amplitude coefficients are selected on a wideband basis and is the same for both polarizations and all layers of a beam

# References

R1-1609855, “Phase and amplitude vs. phase only beam combining for advanced CSI reporting”, Ericsson, RAN1#86bis Lisbon

R1-1612661 “Advanced CSI Codebook structure”, Ericsson, RAN1#87 Reno

# Appendix

## Simulation parameters

| Simulation Parameters |  |
| --- | --- |
| Carrier frequency | 2 GHz |
| Bandwidth | 10 MHz |
| Scenarios | 3D UMi 200m ISD |
| Antenna Configurations | 32 TX: 8x4 with 2x1 virt., UMi (130° tilt) 16 TX: 8x4 with 4x1 virt., UMi (108° tilt) 8 TX: 8x4 with 4x2 virt., UMi (108° tilt) 4TX: 4x4 with 4x2 virt., UMi (108° tilt) |
| Cell layout | 57 homogeneous cells |
| Wrapping | Radio distance based |
| UE receiver | MMSE-IRC |
| CSI periodicity | 5 ms |
| CSI delay | 5 ms |
| CSI mode | PUSCH Mode 3-2 |
| Advanced CSI codebook (when used) | Number of beams: 3 or 2 ( for 4TX) Beam space rotation hypotheses per dimension: 4 Beam power: As indicated Co-phasing: 8-PSK |
| Outer loop Link Adaptation | Yes, 10% BLER target |
| UE noise figure | 9 dB |
| eNB Tx power | 41 dBm (UMi) |
| Traffic model | FTP Model 1, 100 kB packet size |
| UE speed | 3 km/h |
| Scheduling | Proportional fair in time and frequency Max 8 MU layers |
| DMRS overhead | 2 DMRS ports |
| CSI-RS | Overhead accounted for. Channel estimation error modeled. |
| HARQ | Max 5 retransmissions |
| Antenna spacing | 0.8 lambda in vertical, 0.5 lambda in horizontal |
| Handover margin | 3 dB |
| Transmission Mode | TM10, with non-shifted CRS |